3GPP TSG RAN WG1 #111                                                                          R1-2300937

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	UL precoding indication for multi-panel transmission

Agenda item:	9.1.4.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss UL transmission enhancement to precoding indication for multi-panel transmission for Rel-18 NR MIMO WI

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

# UE Panel Association for STxMP

In previous RAN1 meetings, the following agreements have been made.

108e Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - Each UE capability value [set] comprises the max supported number of SRS ports - Any two capability values [sets] are different - Whether the UE capability value [set] can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session

Panel-ID reporting:

| Figure 1: UL STxMP Tx - L1-RSRP with panel selection (2Rx) | Figure 2: UL single TRP Tx - L1-RSRP using all panels (4Rx) |
| --- | --- |

As listed above, different STxMP schemes and transmission scenarios have been identified to study. In order to decide whether to schedule STxMP UL transmission or single-panel UL transmission and also to decide pairs of TCI-states that are compatible for STxMP, the gNB should be informed about the association between the UE panels and the gNB transmission beams and the corresponding L1-RSRPs. This can be done by the following steps.

- Step-0: gNB transmits SSBs/CSI-RSs in different CSI resource sets using different TRPs.

- Step-1: In a groupBasedBeamReporting setting (Figure 1), the UE measures the SSB/CSI-RS received from the best selected panel. Then, the UE reports L1-RSRPs in a CSI report, where each L1-RSRP is associated with a panel-ID indicating which panel is used for L1-RSRP measurement. In a non- groupBasedBeamReporting setting (Figure 2), the UE measures the received the SSB/CSI-RS from all panels. Then, the UE reports L1-RSRP in a CSI report

- Step-2: According to the L1-RSRP value pairs with panel IDs, the gNB can determine pairs of TCI states that are compatible for STxMP. Comparing L1-RSRP reports for the same Tx beam with groupBasedBeamReporting switched on or off, it can decide whether to schedule a STxMP transmission or a single-panel UL transmission.

Furthermore, as agreed in Rel-17 unified TCI framework discussion, a list of UE capability value sets can be reported by the UE to facilitate UE-initiated panel activation and selection, where each UE capability value set comprises the max supported number of SRS ports and any two capability values sets are different. To be specific, for L1-RSRP reporting, when the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRPCapability[Set]Index' or 'ssb-Index-RSRP-Capability[Set]Index' an index of UE capability value set, indicating the maximal supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-RSRP. And L1-SINR reporting follows the same mechanism.

This mechanism can be enhanced to support the UE panel indication for STxMP transmission. For example, different UE capability indices can be mapped to different SRS port groups which may have the same or different maximal supported number of SRS antenna ports. And such a UE capability index pair can be reported along with a pair of SSBRI/CRI and L1-RSRP.

Proposal 1. To support STxMP operation with panel-id reporting, the Rel-17 UE capability value sets can be extended to the case of same maximal supported number of SRS ports (per panel).

# Single-DCI Based STxMP UL Transmission

## Single-DCI based STxMP PUSCH

### SRS Resource Set Configuration

111 Agreement For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH) 110be Agreement For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB. - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. - FFS: For codebook-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports

As listed above, configuring two SRS resource sets for SDM and SFN schemes of single-DCI based STxMP PUSCH has been agreed in the previous RAN1 meeting.

In Rel-17 muti-TRP PUSCH repetition, the two configured SRS resource sets should have the same number of SRS resources for the convenience of repetition transmission and SRI/TPMI indication. However, in Rel-18 SDM STxMP PUSCH transmission, different panels can transmit different number of layers and the two SRIs/TPMIs are indicated independently. Moreover, configuring different number of SRS resources in the two SRS resource sets allows the gNB to configure different number of antenna ports, e.g., 2Tx, 4Tx for the different panels for NCB PUSCH transmission, which is similar to supporting different number of SRS ports for the CB case. However, we note that there is no fixed mapping between an SRS resource set and a physical UE panel because the physical UE panel can rotate according to UE orientation and only for some static scenarios asymmetric UE panels may motivate different number of SRS resources.

One aspect to note is that for codebook based full-power transmission Mode-2, it may be meaningful for the network to configure SRS resources with different number of ports for a UE.

Proposal 2. For sDCI based STxMP PUSCH SDM scheme, it is not critical to support different number of SRS resources in different SRS resource sets.

Proposal 3. For sDCI based STxMP PUSCH SDM scheme, the two SRS resources indicated by the two SRI fields can have different number of SRS ports for CB PUSCH (from full-power Mode-2 operation point of view).

Proposal 4. For sDCI based STxMP PUSCH SFN scheme, follow the decision for SDM STxMP.

Proposal 5. For sDCI based STxMP PUSCH SFN scheme, follow the decision for SDM STxMP.

### Dynamic Switching Between Different Transmission Modes

111 Agreement - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission. - (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme 110be Agreement For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB. - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. - FFS: For codebook-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports

Table 1. STxMP and sTRP mode SRI field(s) length comparison for dynamic switching (separate SRI1 and SRI2 field design.)

|  | sTRP, NSRS,1 or NSRS,2=4 Lmax; SRI length, codepoints(bits). 1; 4 (2). 2/3/4; 10/14/15 (4). | sTRP, NSRS,1 or NSRS,2=3 Lmax; SRI length, codepoints(bits). 1; 3 (2). 2/3/4; 6/7/7 (3). | sTRP, NSRS,1 or NSRS,2=2 Lmax; SRI length, codepoints(bits). 1; 2 (1). 2/3/4; 3/3/3 (2). |
| --- | --- | --- | --- |
| STxMP, NSRS,1=4, NSRS,2=4 Lmax,1; Lmax,2; SRI1+SRI2 length, codepoints(bits). 1; 1; 4(2) + 4(2) = (4). 1; 2; 4(2) + 10(4) = (6). 2; 1; 10(4) + 4(2) = (6). 2; 2; 10(4) + 10(4) = (8). | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |  |  |
| STxMP, NSRS,1=4, NSRS,2=3 Lmax,1; Lmax,2; SRI1+SRI2 length. 1; 1; 4(2) + 3(2) = (4). 1; 2; 4(2) + 6(3) = (5). 2; 1; 10(4) + 3(2) = (6). 2; 2; 10(4) + 6(3) = (7). | using only one SRI field in sTRP mode may not be enough for some max number of layers values. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |  |
| STxMP, NSRS,1=4, NSRS,2=2 Lmax,1; Lmax,2; SRI1+SRI2 length. 1; 1; 4(2) + 2(1) = (3). 1; 2; 4(2) + 3(2) = (4). 2; 1; 10(4) + 2(1) = (5). 2; 2; 10(4) + 3(2) = (6). | using only one SRI field in sTRP mode may not be enough for some max number of layers values. If switching from layer comb {1+1} to sTRP mode rank-2 transmission with SRS resource set 1, 4 bits are needed for SRI indication, but there are only 3 bits |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |
| STxMP, NSRS,1=3, NSRS,2=3 Lmax,1; Lmax,2; SRI1+SRI2 length. 1; 1; 3(2) + 3(2) = (4). 1; 2; 3(2) + 6(3) = (5). 2; 1; 6(3) + 3(2) = (5). 2; 2; 6(3) + 6(3) = (6). |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |  |
| STxMP, NSRS,1=3, NSRS,2=2 Lmax,1; Lmax,2; SRI1+SRI2 length. 1; 1; 3(2) + 2(1) = (3). 1; 2; 3(2) + 3(2) = (4). 2; 1; 6(3) + 2(1) = (4). 2; 2; 6(3) + 3(2) = (5). |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |
| STxMP, NSRS,1=2, NSRS,2=2 Lmax,1; Lmax,2; SRI1+SRI2 length. 1; 1; 2(1) + 2(1) = (2). 1; 2; 2(1) + 3(2) = (3). 2; 1; 3(2) + 2(1) = (3). 2; 2; 3(2) + 3(2) = (4). |  |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |

(Notes: For the convenience of comparison, Lmax,1 and Lmax,2 represent the configured maximal number of layers applied to the 1st and 2nd SRS resource set respectively in STxMP mode, and Lmax represents the configured maximal number of layers applied to single-TRP mode. NSRS,1 and NSRS,2 represents the number of SRS sources in the 1st and 2nd SRS resource sets respectively in STxMP mode.)

In previous RAN1 meetings, it has been agreed that a 2-bit DCI field is used for dynamic switching between SDM/SFN STxMP PUSCH transmission mode and single-TRP PUSCH transmission mode. As shown in Table 1, the SRI field(s) lengths for STxMP mode and sTRP mode are compared with different maximal number of layers and number of SRS resources configurations. For many configurations, using only one SRI field in single-TRP mode are not enough.

For example, when (NSRS,1=4,NSRS,2=4, Lmax,1=1,  Lmax,2=1,  Lmax>1) or (NSRS,1=4, NSRS,2=3, Lmax,1=1,  Lmax,2=1 or 2,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 4 SRS resources, which needs 4 bits for the SRI field.

When (NSRS,1=3, NSRS,2=3, Lmax,1=1,  Lmax,2=1,  Lmax>1) or (NSRS,1=3, NSRS,2=2, Lmax,1=1,  Lmax,2=1 or 2,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 3 SRS resources, which needs 3 bits for the SRI field.

When (NSRS,1=2, NSRS,2=2, Lmax,1=1,  Lmax,2=1,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 2 SRS resources, which needs 2 bits for the SRI field.  But using both the 1st SRI field and the 2nd SRI field to indicate the single SRI for single-TRP mode will not have this issue.

Thus, we have the following proposal.

Proposal 6. For sDCI based STxMP PUSCH SDM scheme, propose using both the 1st SRI field and the 2nd SRI field to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 7. For sDCI based STxMP PUSCH SFN scheme, both the 1st SRI field and the 2nd SRI field can be used to indicate the single SRI when switching to single-TRP mode by DCI.

Furthermore, there is one special case, i.e., (NSRS,1=4,NSRS,2=2, Lmax,1=  Lmax,2=1,  Lmax>1),  that even using both SRI fields for single-TRP SRI indication is not enough to indicate the SRI for single-TRP mode, which are highlighted in red in Table 1. In this case, the total number of bits for the two SRI fields is 3, but 4 bits are needed to indicate the SRI for single-TRP mode.

To solve the problem. We can consider several methods. In one method, an additional bit can be added only for the specific configuration, i.e., (NSRS,1=4, NSRS,2=2, Lmax,1=  Lmax,2=1,  Lmax>1). This indicates the maximal number of layers for single-TRP mode and STxMP should be distinguishable.

Alternatively, we can reduce the resolution of SRI table when the total number of codepoints is not enough for single-TRP SRI. For example, two codepoints in the SRI table can be removed for (NSRS,1=4,NSRS,2=2, Lmax,1= Lmax,2=1,  Lmax=2).

Overall, we prefer the second method.

Proposal 8. For sDCI based STxMP PUSCH SDM/SFN scheme, reduce the resolution of SRS resource indication for single-TRP mode, if 4 and 2 SRS resources are configured in the two SRS resource sets for STxMP mode and the maximal number of layers values for both SRS resource sets in STxMP mode is 1.

Table 2. TPMI field length comparison

| Configurations | Length of TPMI field for codebookSubset = (FullyAnd…, PartialAnd…, NonCoherent) | Reference in TS38.212 |
| --- | --- | --- |
| 4 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= absent/fullpowerMode2/(or fullpower) | [28, 12, 4] codepoints (5, 4, 2) bits | Table 7.3.1.1.2-3 (1st and 2nd TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 2/3/4, ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [62, 32, 12] codepoints (6, 5, 4) bits | Table 7.3.1.1.2-2 (1st TPMI) |
| 4 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= fullpowerMode1 | [NA, 16, 5] codepoints (NA, 4, 3) bits | Table 7.3.1.1.2-3A (1st and 2nd TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= fullpowerMode1 | [NA, 30, 12] codepoints (NA, 5, 4) bits | Table 7.3.1.1.2-2A (1st TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 3/4, ul-FullPowerTransmission= fullpowerMode1 | [NA, 36, 15] codepoints (NA, 6, 4) bits | Table 7.3.1.1.2-2B (1st TPMI) |
| 2 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [6, NA, 2] codepoints (3, NA, 1) bits | Table 7.3.1.1.2-5 (1st and 2nd TPMI) |
| 2 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [9, NA, 3] codepoints (4, NA, 2) bits | Table 7.3.1.1.2-4 (1st TPMI) |
| 2 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= fullpowerMode1 | [NA, NA, 3] codepoints (NA, NA, 2) bits | Table 7.3.1.1.2-5A (1st and 2nd TPMI) |
| 2 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= fullpowerMode1 | [NA, NA, 4] codepoints (NA, NA, 2) bits | Table 7.3.1.1.2-4A (1st TPMI) |

As agreed in previous meeting, for CB-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. There is no dependence between the lengths of the two TPMI fields in SDM STxMP transmission. Thus, the length of the TPMI field for single-TRP mode in current spec is a good reference for the SDM STxMP TPMI field design. The lengths of single-TRP TPMI field for different configurations are shown in Table 2. Similar to what have been observed in SRI field design, Table 2 shows that if only one TPMI field is used for the single TPMI indication when switching from SDM STxMP to single-TRP mode, either the 1st or 2nd TPMI field may not have the enough bits for the scenario that the single-TRP transmission has a larger ‘maxRank’. For example, as shown in the first 2 rows in green in the table, if ‘maxRank1’ = ‘maxRank2’ = 1, ‘maxRank’=2/3/4, ‘fullpowerMode2’ is configured, ‘codebookSubset = fullyAndPartialAndNonCoherent’, and there are 4 antenna ports in both panels, when switching from layer combination {1+1} to single-TRP mode with rank-2 transmission, 6 bits are needed for single-TRP TPMI indication. However, each TPMI field only has 5 bits. On the other hand, the total number of bits is 5+5=10, which is enough for the single-TRP mode TPMI indication with rank 2 and more.

(Notes: For the convenience of comparison, ‘maxRank1’ and ‘maxRank2’ represent the configured maximal ranks applied to the 1st and 2nd SRS resource set respectively in STxMP mode, and ‘maxRank’ is the configured maximal rank applied to single-TRP mode.)

Proposal 9. For sDCI based STxMP PUSCH SDM/SFN scheme, propose using both the 1st TPMI field and the 2nd TPMI field to indicate the single TPMI when switching to single-TRP mode by DCI.

111 Agreement For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. - FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

Next, regarding the two options for the maximal number of layers for sTRP transmission in the above agreement, our understanding is that option1 indicates that all the maxRank values from 1 to 4 should be supported in sTRP mode of SDM STxMP configuration, while option2 means we can down select certain maxRank values. Alt1 to Alt4 provide different ways to configure the maximal number of layers for STxMP mode, which are about whether the two different values of maximal number of layers can be configured for STxMP and about the relation among the legacy Lmax and the maximal number of layers for STxMP and single-TRP mode.

From our perspective, we support two configurations for maximum number of layers for sTRP operation, a) sTRP and STxMP maximum number of layers are equal, or b) sTRP maximum number of layers is less than that for STxMP. Thus, we prefer Option1. Regarding the maximal number of layers of SDM transmission, we are fine with Alt1.

Proposal 10. For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission, Option2+Alt1 is preferred.

Table 3. Dynamic switching between sDCI SDM STxMP and sTRP transmission and SRI/TPMI field mapping when sDCI SDM STxMP is configured

| Codepoint | Transmission mode/SRS resource set | SRI (for both CB and NCB)/TPMI (CB only) field(s) to use |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 01 | s-TRP mode with 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 10 | SDM STxMP mode with (TRP1, TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | Reserved | Reserved |

Table 4. Dynamic switching between sDCI SFN STxMP and sTRP transmission and SRI/TPMI field mapping when sDCI SFN STxMP is configured

| Codepoint | Transmission scheme/mode/SRS resource set | SRI (for both CB and NCB)/TPMI (CB only) field(s) to use |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 01 | s-TRP mode with 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 10 | SFN STxMP mode with (TRP1, TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields (the length of the 2nd SRI/TPMI field depends on the number of layers indicated by the 1st SRI/TPMI field) |
| 11 | Reserved | Reserved |

Next, the codepoint design for dynamic switching when SDM STxMP and SFN STxMP are configured may follow Table 3 and Table 4, respectively. In particular, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with different panel. The third codepoint ‘10’ can be used to indicate SDM or SFN STxMP mode and the last codepoint ‘11’ can be reserved.

Proposal 11. For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with the 1st and 2nd SRS resource sets respectively. The third codepoint ‘10’ can be used to indicate SDM/SFN STxMP mode and the last codepoint ‘11’ can be reserved.

### DMRS Ports Indication

111 Agreement For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2: - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups. 111 Agreement For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively: - The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field - The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field - Support at least one of the following options for indication of layer combination {1+2}: - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}. - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field. - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers - Other options are not precluded

In Rel-17, one DCI field of antenna ports is used to indicate the DMRS port(s) for single-TRP PUSCH transmission and multi-TRP PUSCH repetition. For single-DCI based STxMP PUSCH SDM scheme, different layer combinations such as {1+1, 1+2, 2+1, 2+2} can be supported. Although the existing antenna port(s) tables corresponding to the DCI field of antenna ports can indicate different number of DMRS ports with different CDM groups, not all the layer combination scenarios are supported. For example, the indication of DMRS ports such as (0,2,3) or (2,0,1) corresponding to layer combination 1+2 is not listed in current antenna port(s) tables.

In previous RAN1 meeting, three options have been listed to indicate the DMRS port for layer combination {1+2} in SDM STxMP PUSCH transmission. In option 1, by adding a new entry such as (0,2,3) or (2,0,1) to the DMRS port table, the total length of the DMRS ports indication field will not increase. Moreover, this allows the transmission through different panels using different CDM groups, which benefits the interference reduction and aligns with the Rel-16 DL transmission.

Proposal 12. For SDM scheme single-DCI based STxMP transmission when layer combination {1+2} is transmitted, a new entry such as (0,2,3) or (2,0,1) can be added to DMRS table.

### PTRS-DMRS Association

110be Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme

To compensate phase noise, PTRS is transmitted from PTRS ports and embedded in the PUSCH transmission with DMRS. The DMRS ports corresponds to the layers of the PUSCH transmission. Then the PTRS port(s) needs to be associated with DMRS ports. In Rel-17, there are at most two PTRS ports to be associated in a PUSCH transmission occasion for a panel and the association is indicated in the DCI fields of PTRS-DMRS association and the second PTRS-DMRS association. For STxMP PUSCH transmission, the association between PTRS and DMRS should be specified. In our view, associating 1 PTRS port for each panel can be sufficient.

Proposal 13. For single-DCI based STxMP PUSCH transmission, one PTRS port is assigned for each UE panel.

Furthermore, different layer combinations can be scheduled or configured for sDCI based STxMP PUSCH transmission. And each layer should be associated with a DMRS port. Thus, the PTRS-DMRS association field length depends on the layer combinations. For all the layer combinations, a 2-bit PTRS-DMRS association field is enough. For example, the MSB and LSB can be used for the PTRS-DMRS association for the first and second panel respectively if layer combination {2+2} is transmitted. And if 1 layer is transmitted in a panel, the corresponding bit field can be reserved. If 3 layers are transmitted in a panel, both the MSB and LSB can be used to indicate the PTRS-DMRS association for the panel.

Proposal 14. For single-DCI based STxMP PUSCH transmission, a 2-bit PTRS-DMRS association field can be used, where

- the MSB and LSB are used to indicate the PTRS-DMRS association for the first and second panel respectively for the layer combination {2+2},

- the MSB or LSB can be reserved if only one layer is transmitted through the panel.

### STxMP for CSI

105e Agreement For s-DCI based multi-TRP PUSCH repetition Type A and B, support transmitting A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH. - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions. - The UE is expected to follow the above operation for transmitting A-CSI on two PUSCH repetitions only if - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation). - For PUSCH repetition Type A and B, UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE transmits A-CSI only on the first PUSCH repetition similar to Rel. 15/16. - Note: The scheduling offset for the first A-CSI should meet the Z and Z’ requirement

In Rel-17, A/SP-CSI multiplexing on multi-TRP PUSCH repetition is supported, as shown in the above agreements. Similarly, for STxMP PUSCH transmission, an A/SP-CSI being multiplexed on one of the PUSCHs without TB. Specifically, if one A/SP-CSI is scheduled, it could be multiplexed on the PUSCH that is without TB and targets to the same TRP with the A/SP-CSI. Furthermore, two A/SP-CSIs targeting different TRPs can be transmitted simultaneously through two panels, where each A/SP-CSI is multiplexed on the corresponding PUSCH that is without TB and targeting the corresponding TRP.

Proposal 15. For STxMP PUSCH transmission with A/SP-CSI,

- one A/SP-CSI can be multiplexed on the PUSCH that is without TB and targets to the same TRP with the A/SP-CSI,

- two A/SP-CSIs targeting different TRPs can be transmitted through two panels simultaneously, where each A/SP-CSI should be multiplexed on the PUSCH that is without TB and targets to the same TRP with the A/SP-CSI.

## Switching Between STxMP PUCCH Transmission Schemes

111 Agreement for UTCI On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design

In last RAN1 meeting, it was agreed that ‘on unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group’. It seems the agreement can be applied to both multi-TRP PUCCH repetition and STxMP PUCCH. Then how to distinguish the two schemes and switching between them as well as the single-TRP PUCCH should be studied.

Proposal 16. For single-DCI based STxMP SFN PUCCH transmission, study how to switch among STxMP transmission, multi-TRP repetition, and single-TRP transmission schemes.

# Multi-DCI Based STxMP UL Transmission

## Multi-DCI based STxMP PUSCH Transmission

111 Agreement - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

As listed above, the two SRS resource sets in STxMP PUSCH+PUSCH transmission are associated with different coresetPoolIndex values. For the FFS, it is nature that the first SRS resource set is the set with lower set ID.

Next, the maxRank and the maximal number of layers configured for multi-DCI based STxMP PUSCH+PUSCH should be considered. Similar to what have been discussing for single-DCI based STxMP, the questions are 1) whether different maxRanks should be configured for the two PUSCH transmissions respectively in multi-DCI based STxMP transmission and 2) whether different maxRanks should be configured for multi-DCI based STxMP transmission and single-TRP transmission.

## Multi-DCI based STxMP PUCCH Transmission

111 Conclusion There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system WID #94e for MIMO Evolution for Downlink and Uplink, Objective 6 Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In last RAN1 meeting, a conclusion was made that there is no consensus on the support of STxMP PUCCH+PUCCH in multi-DCI based mTRP system. However, this conclusion contradicts the Objective 6 in the WID for MIMO evolution which states that for the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. As discussed in last RAN1 meeting, the major concern is the UCI multiplexing issue when multi-DCI based STxMP PUCCH + PUCCH is supported. However, we think applying the legacy collision handling rule to each panel could handle most of the scenarios. Thus, we prefer to support the multi-DCI STxMP PUCCH+PUCCH transmission.

Proposal 17. Support multi-DCI based STxMP PUCCH+PUCCH transmission.

# Collision Handling for STxMP PUCCH/PUSCH Transmission

In Rel-17, the collision handling between multi-TRP PUCCH repetition and other channels/signals is concluded to use Rel-15/16 collision handling rules. For example, in Rel-15, when single-slot PUCCH carrying dynamic HARQ-ACK/SR and single-slot PUCCH carrying CSI overlap in a slot, and if the timeline requirement is satisfied, one PUCCH resource is determined to carry a combined payload of dynamic HARQ-ACK/SR and CSI,  as shown in Figure 3. Note that if time-domain STxMP repetition is supported, more STxMP collision scenarios can be expected.

Figure 3. Rel-15 dynamic HARQ-ACK and CSI multiplexing: a new PUCCH resource is selected based on combined payload size and PRI

In previous RAN1 meeting, SFN scheme for single-DCI based STxMP PUCCH transmission has been supported. Similar to the Rel-15/16 collision scenarios, a single or STxMP PUCCH transmission can collide with a single or STxMP PUCCH transmission (i.e., single PUCCH + STxMP PUCCH, or STxMP PUCCH + STxMP PUCCH). Different from Rel-17 that the collision happens among PUCCHs transmitting to the same TRP, the STxMP PUCCH collision can happen among PUCCHs transmitting to different TRPs. In order to minimize the spec change, the collisions among PUCCHs can be handled for each TRP/panel separately.

Proposal 18. If collision happens between PUCCH and PUCCH when STxMP is enabled, e.g., single PUCCH + STxMP PUCCH collision, and STxMP PUCCH + STxMP PUCCH collision, the legacy collision handling rule for single TRP can be applied for each TRP/panel separately.

Furthermore, when STxMP PUSCH/PUCCH transmission schemes are supported, there are other PUCCH and PUSCH collision scenarios. More specifically, collision scenarios can be identified as follows. First, a single PUSCH transmission can collide a STxMP PUCCH transmission (single PUSCH + STxMP PUCCH). Second, a single PUCCH transmission can collide with a STxMP PUSCH transmission (single PUCCH + STxMP PUSCH). Third, a STxMP PUCCH transmission can collide with a STxMP PUSCH transmission (STxMP PUCCH + STxMP PUSCH).

However, as mentioned above, the Rel-15/16 collision handling mechanism cannot clearly describe the UE behaviour for all the collision scenarios in Rel-18. Thus, new solutions should be considered.

Proposal 19. Study new collision handling rules for the following scenarios, single PUSCH + STxMP PUCCH, single PUCCH + STxMP PUSCH, STxMP PUCCH + STxMP PUSCH.

# Power Headroom Report for STxMP

106e Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. 106e Agreement For option 4, support the following: - When PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, second PHR value is determined as, - If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value, select Alt. 2A - Alt.2A: Is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual. - If there are multiple repetitions associated with the other TRP in slot n, the earliest one in slot n is selected. - If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), select Alt. 1B - Alt1B: a second PHR value is reported as virtual PHR. - If the first PHR value is virtual, select Alt. 1C - Alt1C: a second PHR value is reported as virtual PHR. - Note: It was agreed that when second PHR is virtual, it is calculated based on a set of default power control parameters defined for the other TRP (that is not associated with the first PHR) - Note: It was agreed that the above is applicable to both single entry and multi-entry PHR reports

In Rel-17 multi-TRP PUSCH repetition, two PHRs associated with the two TRPs are reported to the BS if the UE has such capability. Similarly, for STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs should be reported. In other words, the PHR report for STxMP PUSCH transmission should be panel-specific.

Proposal 20. The PHR report for STxMP PUSCH transmission should be panel specific.

In current specification, the PHR report can be triggered by different events such as PHR timer expires, path loss changing more than threshold, reconfiguration, etc. And the triggering condition is not TRP specific. For example, there is only one phr-PeriodicTimer and one phr-ProhibitTimer.

In multi-DCI based STxMP PUSCH transmission, the time domain overlap scenario can be different if partial overlapping PUSCH is supported. PHR reporting can be triggered in different scenarios, such as 1) only one panel is scheduled for STxMP transmission, and 2) both panels are scheduled for STxMP transmission. For the second scenario, two PHRs should be transmitted. While for the first scenario, there can be two options, one option is reporting one PHR of the scheduled transmission, the other option is reporting one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission. In our view, the second option is more preferrable since the PHR triggering is not panel/TRP specific and this option is more aligned with the Rel-17 PHR reporting for multi-TRP repetition.

Proposal 21. For multi-DCI based STxMP PUSCH transmission, when PHR report is triggered,

- if two panels are scheduled for STxMP transmission, two actual PHRs corresponding to the two panels/TRPs can be reported,

- if only one panel is scheduled for STxMP transmission, one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission can be reported.

For single-DCI based STxMP PUSCH transmission, the two PUSCHs from two panels are always fully overlapped in time domain. Thus, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

Proposal 22. For single-DCI based STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

# Conclusion

The main proposals in this paper are outlined as follows.

Proposal 1. To support STxMP operation with panel-id reporting, the Rel-17 UE capability value sets can be extended to the case of same maximal supported number of SRS ports (per panel).

Proposal 2. For sDCI based STxMP PUSCH SDM scheme, it is not critical to support different number of SRS resources in different SRS resource sets.

Proposal 3. For sDCI based STxMP PUSCH SDM scheme, the two SRS resources indicated by the two SRI fields can have different number of SRS ports for CB PUSCH (from full-power Mode-2 operation point of view).

Proposal 4. For sDCI based STxMP PUSCH SFN scheme, follow the decision for SDM STxMP.

Proposal 5. For sDCI based STxMP PUSCH SFN scheme, follow the decision for SDM STxMP.

Proposal 6. For sDCI based STxMP PUSCH SDM scheme, propose using both the 1st SRI field and the 2nd SRI field to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 7. For sDCI based STxMP PUSCH SFN scheme, both the 1st SRI field and the 2nd SRI field can be used to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 8. For sDCI based STxMP PUSCH SDM/SFN scheme, reduce the resolution of SRS resource indication for single-TRP mode, if 4 and 2 SRS resources are configured in the two SRS resource sets for STxMP mode and the maximal number of layers values for both SRS resource sets in STxMP mode is 1.

Proposal 9. For sDCI based STxMP PUSCH SDM/SFN scheme, propose using both the 1st TPMI field and the 2nd TPMI field to indicate the single TPMI when switching to single-TRP mode by DCI.

Proposal 10. For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission, Option2+Alt1 is preferred.

Proposal 11. For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with the 1st and 2nd SRS resource sets respectively. The third codepoint ‘10’ can be used to indicate SDM/SFN STxMP mode and the last codepoint ‘11’ can be reserved.

Proposal 12. For SDM scheme single-DCI based STxMP transmission when layer combination {1+2} is transmitted, a new entry such as (0,2,3) or (2,0,1) can be added to DMRS table.

Proposal 13. For single-DCI based STxMP PUSCH transmission, one PTRS port is assigned for each UE panel.

Proposal 14. For single-DCI based STxMP PUSCH transmission, a 2-bit PTRS-DMRS association field can be used, where

- the MSB and LSB are used to indicate the PTRS-DMRS association for the first and second panel respectively for the layer combination {2+2},

- the MSB or LSB can be reserved if only one layer is transmitted through the panel.

Proposal 15. For STxMP PUSCH transmission with A/SP-CSI,

- one A/SP-CSI can be multiplexed on the PUSCH that is without TB and targets to the same TRP with the A/SP-CSI,

- two A/SP-CSIs targeting different TRPs can be transmitted through two panels simultaneously, where each A/SP-CSI should be multiplexed on the PUSCH that is without TB and targets to the same TRP with the A/SP-CSI.

Proposal 16. For single-DCI based STxMP SFN PUCCH transmission, study how to switch among STxMP transmission, multi-TRP repetition, and single-TRP transmission schemes.

Proposal 17. Support multi-DCI based STxMP PUCCH+PUCCH transmission.

Proposal 18. If collision happens between PUCCH and PUCCH when STxMP is enabled, e.g., single PUCCH + STxMP PUCCH collision, and STxMP PUCCH + STxMP PUCCH collision, the legacy collision handling rule for single TRP can be applied for each TRP/panel separately.

Proposal 19. Study new collision handling rules for the following scenarios, single PUSCH + STxMP PUCCH, single PUCCH + STxMP PUSCH, STxMP PUCCH + STxMP PUSCH.

Proposal 20. The PHR report for STxMP PUSCH transmission should be panel specific.

Proposal 21. For multi-DCI based STxMP PUSCH transmission, when PHR report is triggered,

- if two panels are scheduled for STxMP transmission, two actual PHRs corresponding to the two panels/TRPs can be reported,

- if only one panel is scheduled for STxMP transmission, one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission can be reported.

Proposal 22. For single-DCI based STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

# References

- 3GPP RAN1, Chairman’s Note, 3GPP RAN1 #111, November 2022.

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e, Samsung