3GPP TSG RAN WG1 #112			      R1-2300942

Athens, Greece, February 27th – March 3rd, 2023

Source: 	Intel Corporation

Title: 	On other aspects on AI/ML for CSI feedback

Agenda item:	9.2.2.2

Document for: 	Discussion and Decision

# Introduction

The new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes the analysis of solutions for CSI feedback enhancements:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project

In this contribution, we express our views on potential specification impact focusing on spatial-frequency domain CSI compression using two-sided AI model sub-use case and CSI prediction using one-sided AI model at the UE side sub-use case.

Discussion

## Spatial-frequency domain CSI compression using two-sided AI model

### Model performance monitoring and data collection

Enhanced Type II PMI codebook supports multiple parameter combinations {L, pv, beta}, where each parameter combination corresponds to different overhead and performance for a given channel. Depending on the channel statistics (e.g. LoS/NLoS, number of clusters, delay spread etc.) the optimal parameter combination can be selected for a UE which allow to optimize CSI reporting overhead. Performance for the parameter combination corresponding to larger L, pv and beta is generally better comparing to the parameter combinations with lower parameter values. Performance of AI-ML model for CSI also depends on many aspects including pre/post-processing, alignment of training dataset and actual channel realisation for CSI calculation/inference, neural network structure and quantization, etc. Thus, the proper AI-ML model should be configured for a UE to achieve the best performance and overhead tradeoff.

In order to select the proper AI-ML model for CSI reporting, model performance monitoring can be used. At the RAN1#110b-e meeting the following agreements were made on the model performance monitoring.

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring. Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

As it can be seen from the above agreements, at least eventual KPI and intermediate KPI are considered as monitoring metrics for AI-ML model performance monitoring. However, it is not clear how this metrics is calculated. If the metrics for model performance monitoring is calculated based on one model configured for a UE, then it is not possible to decide whether to re-configure the AI-ML model or continue to use the same AI-ML model. In order to select the AI-ML model with the best performance for CSI, testing of multiple AI-ML models with the measured channel can be considered.

Observation 1:

- Model performance monitoring based on intermediate KPI or eventual KPI calculated based on one AI-ML model is not giving enough information for proper configuration of AI-ML Model

Proposal 1:

- Testing of different AI-ML models with the measured channel should be considered for model performance monitoring

Considering that testing of different AI-ML models at the UE side may be complex and will lead to significant power consumption, network-based approach should be used. Network-based performance monitoring in this case can be done by using SRS measurements. If UL and DL channels have significant difference which doesn’t allow to identify proper AI-ML model for DL CSI from UL channel measurements, reporting of ground-truth CSI can be used. The reporting of ground-truth CSI is already considered by RAN1 with details covered in the below agreement.

Agreement For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options - High resolution scalar quantization, e.g., Float32, Float16, etc. - FFS select one of the scalar quantization resolutions as baseline - High resolution codebook quantization, e.g., R16 Type II-like method with new parameters - FFS new parameters - Other quantization methods are not precluded

Thus, we propose to consider network-based AI-ML model performance monitoring based on SRS or ground-truth CSI reporting.

Proposal 2:

- The following approaches for channel measurements/reporting are considered for the network-based AI-ML model performance monitoring

  - Network-based model performance monitoring based on SRS measurements

  - Network-based model performance monitoring based on ground truth CSI reporting

The following agreement was made at the RAN1#110b-e meeting for the co-existence of legacy PMI codebooks and AI-ML based CSI reporting.

Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

In our view the new AI-ML based CSI reporting can be fitted to the existing CSI framework as new codebook type or as new CSI quantity. In this case co-existence and fallback can be done via the existing features. For example, two CSI Report Config parameters can be configured with one CSI Report Config corresponding to CSI report with a PMI codebook and another corresponding to AI-ML based CSI feedback. If CSI with PMI codebook outperforms the CSI with AI-ML model based on network performance monitoring procedure, CSI report with PMI codebook can be triggered/activated/used.

Proposal 3:

- Co-existence and fallback mechanism between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode should be based on existing CSI framework

One of the aspects which is related to the training of the neural network is the data collection. In our view, at least for initial study of the AI/ML CSI sub use cases it can be assumed that existing NR features are considered for data collection (e.g. SRS, CSI-RS, CSI reporting).

Proposal 4:

- Consider existing NR features as baseline for data collection (e.g. SRS, CSI-RS, CSI reporting)

### AI/ML model input and inference

At the RAN1#110 meeting the following agreements were made w.r.t. AI/ML model input and inference.

Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination Agreement In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least - Model output type/dimension/configuration and potential post processing

In our view the parameters for the model input should be defined similar to parameters L and M for the Enhanced Type II PMI codebook, where parameter L controls the number of inputs for spatial domain and parameter M controls the number of inputs for frequency domain, so the dimension of input matrix is L x M. Also, it should be clarified that input for the AI/ML model corresponds to the input for neural network after pre-processing.

Proposal 5:

- The dimensions of the input are defined by parameters similar to parameters L/M parameters for Enhanced Type II PMI codebook (considering that input corresponds to the neural network input after pre-processing)

Since the neural network is trained assuming a particular algorithm for pre-processing, it doesn’t make sense to have separate configuration/specification of pre-processing algorithm and neural network (including neural network weights considering that the same neural network may be trained with different pre-processing algorithms). Thus, if an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded, transferred etc.). Since the pre-processing and neural network are highly related it is important to provide clear definition of pre-processing algorithm. To describe pre-processing at least linear transforms (DFT across different dimensions), downselection of matrix elements and normalization operations should be defined.

Proposal 6:

It is expected that AI/ML model is trained assuming a particular pre/post processing

- If an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded etc.)

- Pre/post-processing may include at least linear transforms (DFT across different dimensions), downselection of matrix elements and normalization

### CQI and RI reporting

For spatial-frequency domain CSI compression using two-sided AI model, PMI calculation approach is redesigned. However, for CQI and RI calculation it is still possible to use legacy principles. Thus, RI can be used to indicate the number of layers recommended for the PDSCH transmission and CQI can be used to indicate the spectral efficiency of the effective channel (i.e., recommended modulation and coding scheme).

Proposal 7:

- Consider existing principles for RI and CQI for spatial-frequency domain CSI compression using two-sided AI model sub-use case

The RI determination procedure is not defined in the specification, and it is subject for UE implementation with verification of performance via RAN4 tests. To align assumptions for CQI calculation at the UE side and CQI usage at the gNB side, it is specified in TS 38.214 how to calculate the effective channel based on measured channel and precoding matrix from reported PMI. Thus, precoding matrix used for CQI calculation at the UE is reconstructed based on PMI and it is aligned with the precoding matrix assumed for PDSCH transmission at the gNB.

For spatial-frequency domain CSI compression using two-sided AI model, the reconstruction of precoding matrix based on PMI bits is possible only if the decoder NN with weights are known at the UE side. If decoder NN with weights are not known at the gNB, approximation of precoding matrix should be used. For example, Type I single-panel PMI codebook can be used to calculate CQI. Or eigenvectors of the channel matrix can be used as precoding matrix for effective channel calculation. In this case, to model the degradation of performance due to non-ideal accuracy of PMI, random noise can be added to the eigenvectors.

Proposal 8:

- Assumptions for CQI determination (e.g. applied precoding matrix for effective channel calculation) shall be defined in specification to avoid ambiguity at the gNB side

  - If trained decoder is known at the UE side, output precoding matrix can be used at the UE effective channel calculation

  - If trained decoder is not known at the UE side, approximation of precoding matrix can be used for effective channel calculation (e.g. based on Type I PMI codebook or channel eigenvectors)

## CSI prediction using one-sided AI model at the UE side

At the last RAN1 meeting the following agreement was made for CSI prediction using one-sided AI model at the UE side sub-use case.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

There is a parallel discussion on CSI prediction in Rel-18 MIMO WI, where UE-side channel prediction support is enabled for new PMI codebooks, where codebook enhancements as well as assistance signalling and CQI determination mechanism are specified. Considering the above, CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility to avoid redundant work on similar issues.

Proposal 9:

- CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility

Conclusion

In this contribution, we have provided our views on sub use cases for AI/ML CSI including potential specification impact. The following proposals and observations were made.

Observation 1:

- Model performance monitoring based on intermediate KPI or eventual KPI calculated based on one AI-ML model is not giving enough information for proper configuration of AI-ML Model

Proposal 1:

- Testing of different AI-ML models with the measured channel should be considered for model performance monitoring

Proposal 2:

- The following approaches for channel measurements/reporting are considered for the network-based AI-ML model performance monitoring

  - Network-based model performance monitoring based on SRS measurements

  - Network-based model performance monitoring based on ground truth CSI reporting

Proposal 3:

- Co-existence and fallback mechanism between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode should be based on existing CSI framework

Proposal 4:

- Consider existing NR features as baseline for data collection (e.g. SRS, CSI-RS, CSI reporting)

Proposal 5:

- The dimensions of the input are defined by parameters similar to parameters L/M parameters for Enhanced Type II PMI codebook (considering that input corresponds to the neural network input after pre-processing)

Proposal 6:

It is expected that AI/ML model is trained assuming a particular pre/post processing

- If an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded etc.)

- Pre/post-processing may include at least linear transforms (DFT across different dimensions), downselection of matrix elements and normalization

Proposal 7:

- Consider existing definitions for RI and CQI for spatial-frequency domain CSI compression using two-sided AI model sub-use case

Proposal 8:

- Assumptions for CQI determination (e.g. applied precoding matrix) shall be defined in specification to avoid ambiguity at the gNB side

  - If trained decoder is known at the UE side, output precoding matrix can be used at the UE for accurate CQI calculation

  - If trained decoder is not known at the UE side, approximation of precoding matrix can be used (e.g. based on Type I PMI codebook or channel eigenvectors)

Proposal 9:

- CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility

References

- RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021.