3GPP TSG RAN WG1 #112		    				            R1-2300969

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	Evaluations on LP WUS

Agenda item:	9.13.1

Document for:	Discussion and Decision

# Introduction

In last RAN1 meetings, the assumptions for the evaluation of power consumption/latency and link level simulation were agreed. Therefore, in this contribution, we provide the preliminary evaluation results.

# Power consumption / latency

# Power consumption model

In the existing power consumption model, micro/light/deep sleep were introduced for different level of power saving [4][5]. Additionally, ultra-deep sleep is agreed for further power saving of main radio. In this mode, the local oscillator may be completely off. The control processor and DDR memory may be off too. It needs clarification whether all or some information in DDR memory are lost. As a result, longer transition time and larger additional transition energy are necessary for switching to/from the ultra-deep sleep. The agreed assumption for LPHAP, which also adopts an ultra-deep sleep mode for a NR receiver, can be used as reference to derive the parameters for ultra-deep sleep mode of main radio. Therefore, the proposed values are summarized below.

- The relative power can be confirmed as 0.015

- The proposed value range is 2000 – 40000 in RAN1#110b-e meeting. It may be too challenging for 2000 units. On the other hand, 40000 is too conservative which may underestimate the gain of LP-WUS. We prefer to reuse same value as LPHAP, i.e., the additional transition energy is 10000 units with optional 5000 units

- The proposed ramp-up time is 400 or 100ms in RAN1#110b-e meeting. As comparison, the total transition time agreed for LPHAP is 400ms. The ramp-down time is not necessary in the determination of latency for the wake-up. However, ramp-down time is still useful to check if UE is practical to turn into ultra-deep sleep. For example, UE can switch to ultra-deep sleep only if the gap between the two adjacent channels/signals are larger than the total transition time of ramp-up and ramp-down. To be conservative, the ramp-up time of 400ms can be used in the evaluation.

- The time/power consumption for sync/re-sync was agreed to be separately modeled. Since LP-WUS can act as a timing reference with granularity smaller than a MC-OOK/FSK symbol, UE can know the suitable timing for SSB detection in main radio. Consequently, the time for sync/re-sync may not different much from the PEI based paging procedure. We prefer to model the sync/re-sync time based on the number of SSBs that should be detected according to the SNR at UE.

Table 1: Proposed Power consumption and transition time for deeper sleep mode

| Power State | Relative Power | Ramp-up and down transition energy | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Deeper Sleep | 0.015 | 10000, optional 5000 | 400 ms | 1,2,3 SSBs |

Proposal 1: On power consumption of main radio

- For the ultra-deep sleep mode, the relative power, Ramp-up and down transition energy and ramp-up time can be respectively assumed as 0.015, 10000 and 400ms in the evaluation.

- The sync/re-sync time based on 1,2,3 SSBs that should be detected according to the SNR at UE

Multiple relative power values of LP-WUR had been agreed in last RAN1 meetings. One FFS point is transition time/energy for LP-WUR. If the relative power is assumed to be rather small for the ON state of LP-WUR, the transition time would be very small too. A transmission time of 1 ms can be assumed if it is not zero. On the other hand, when the relative power becomes higher, e.g., up to 4, the transition energy may occupy large part of power consumption of LP-WUR. A transition time of 10ms can be used.

Proposal 2: On power consumption of LP-WUR

- The transition time can be modeled as 1ms for a low relative power consumption of LP-WUR

- The transition time can be modeled as 10ms for low relative power consumption of LP-WUR from 1 to 4

In idle/inactive state, DCI format 2_7 was introduced in Rel-17 which indicates paging early indication (PEI). Instead of waking up at the paging occasion and monitoring PDCCH in Type2 CSS, a UE can wake up early and detect PEI PDCCH. Then, the UE may only detect PDCCH and associated paging PDSCH when PEI indicates the sub-group for the UE is currently paged. To reduce the time for UE to do fine time/frequency synchronization, TRS/CSI-RS can be configured in idle/inactive mode. However, the UE still need to perform SSB-based RRM.

In idle/inactive state, both basic IDRX operation and enhanced power saving schemes in Rel-16/17 can be considered as baseline to verify the potential gain of LP-WUS based operation. Specifically, There are four combinations for the reference cases: 1) IDRX without PEI; 2) IDRX with PEI; 1) eDRX without PEI; 2) eDRX with PEI. LP-WUS can be introduced to the existing IDRX operations in following three options. The relaxation on RRM by main radio or the LP-WUS to support RRM or related RRM should be studied.

- Option 1: LP-WUS serves as one bit indication to start IDRX procedure without PEI

- Option 2: LP-WUS serves as one bit indication to start IDRX procedure with PEI

- Option 3: LP-WUS carries paging subgroup information which can be same as PEI PDCCH.

Two operation mode for LP-WUS detection can be considered in the above 3 options, i.e., LP-WUS can be always ON or with a duty cycle. We provide preliminary results on power consumption for LP-WUS based operation compared to the 4 reference cases and the 3 options for LP-WUS with the two operation modes. It assumes IDRX cycle of 1.28s and eDRX cycle 61.44s. For duty-cycle based operation, it assumes one slot in every 100 slots is available for LP-WUS detection. A fixed paging arrival rate is assumed to be 0.001 per UE per second. It is assumed there are 10 UEs in the paging group. It assumes the number of paging subgroup is 8 for options based on PEI and subgroup indication by LP-WUS.

# Power consumption w/o consideration of RRM

In this section, we perform evaluations for power consumption/latency based on LP-WUS, without consideration of the RRM measurement by main radio. The results are applicable if LP-WUS can support RRM measurement with sufficient accuracy. Otherwise, the results can be seen as an upper bound for the benefit that can be achieved by LP-WUS.

- Figure 1A to 1D provide power consumption with different relative power for LP-WUR ON with total transition energy 10000 units. Significant benefit on power saving in LP-WUS based operation are observed except that LP-WUS is always ON with ON power of 4 units. The issue can be simply solved by duty-cycle based LP-WUS detection.

- Figure 1E provides power consumption with relative power 0.5 unit for LP-WUR ON with extremely high total transition energy 40000 units. Large power saving gain is still observed.

- Figure 1F is the performance of latency which applies irrespective the setting of relative power. As expected, LP-WUS enables a latency slightly higher than IDRX mainly due to the 400ms ramp-up time. LP-WUS achieve quite obvious gain in term of latency compared with eDRX.

- From Figure 1A to 1E, comparing the 3 options for LP-WUS based operation, Option 3 provides the lowest power consumption. Since paging subgroup is indicated by LP-WUS, it avoids unnecessary waking up the main radio if there is no paging message to the UE.

- (B)

(C)                                                  		(D)

(E)                                                  		(F)

Figure 1: Power consumption/latency in idle/inactive mode without RRM by MR

Observation 1: For idle/inactive mode, without consideration on RRM by main radio

- Significant benefit on power saving in LP-WUS based operation are observed except that LP-WUS is always ON with ON power of e.g., 4 units. The issue can be simply solved by duty-cycle based LP-WUS detection.

- Large power saving gain is still observed even when extremely high total transition energy 40000 units.

- LP-WUS enables a latency slightly higher than IDRX and significantly lower than eDRX.

- The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

# Power consumption with RRM

For a UE in idle/inactive mode, it is important for the UE to do RRM measurement for potential cell reselection. The RRM measurement by main radio in every paging cycle may consume large power. Therefore, relaxed RRM measurement by main radio was proposed in early RAN1 meetings. The RRM based on LP-WUS is further proposed which may compensate the performance on RRM measurement and cell reselection.

In this section, we perform evaluations for power consumption/latency based on LP-WUS considering relaxed RRM measurement by main radio.

- Figure 2A to 2D provide power consumption with different relative power for LP-WUR ON with total transition energy 10000 units. Though the benefit for power saving reduces a lot compared to the corresponding case in Figure 1A-1D, still large gains are observed.

- Figure 2E provides power consumption with relative power 0.5 unit for LP-WUR ON with extremely high total transition energy 40000 units. A loss is observed on power consumption due to the transition energy for RRM measurement.

- Figure 2F is the performance of latency which is similar to Figure 1F. That is, LP-WUS enables a latency slightly higher than IDRX mainly due to the 400ms ramp-up time. LP-WUS achieves quite obvious gain in terms of latency compared with eDRX.

- From Figure 2A to 2E, comparing the 3 options for LP-WUS based operation, Option 3 still provides the lowest power consumption. However, the gain is reduced since RRM measurement occupies a large share of the power consumption.

(A)                                                  		 (B)

(C)                                                  	 	(D)

(E)                                                  		(F)

Figure 2: Power consumption/latency in idle/inactive mode with relaxed RRM by MR

Observation 2: For idle/inactive mode with relaxed RRM by main radio

- Large gain on power saving in LP-WUS based operation are observed. For the case LP-WUS ON power of 4 units, power saving gain can be achieved in duty-cycle based LP-WUS detection.

- There is a loss for extremely high total transition energy 40000 units.

- LP-WUS enables a latency slightly higher than IDRX and significantly lower than eDRX.

- The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

If the exact UE ID and/or the cause for waking up, e.g. SI update is directly indicated by LP-WUS, the UE can avoid detection of PO after waking up. Though we didn’t simulate such cases without PO monitoring, the results in Figure 1 can be a good reference, since the operations of SSB based time/frequency synchronization and/or RRM are common to different cases.

Proposal 3: For idle/inactive mode

- Further study the cases that LP-WUS can provide an indication on paging group, paging subgroup or UE ID

# Link level simulation

# Simulation assumptions

False alarm ratio defines the possibility that UE considers a LP-WUS is detected if gNB doesn’t transmit LP-WUS to the UE. False alarm of LP-WUS results waste of power due to useless waking up the main radio. FAR of 10% is not desired since it on average results in UE morning one PO in every 10 paging cycles. However, assuming per group paging rate of 1%, there is only about one valid paging message to the UE every 100 paging cycles. Mis-detection of LP-WUS is for the case that UE doesn’t consider a LP-WUS is detected but gNB actually transmit the LP-WUS. Mis-detection, if happens, results in missing the traffic to the UE or fails to update network status which causes longer latency.

As discussed in [3], the LP-WUS may contain two parts: Preamble part is for AGC and carries a sequence for synchronization, while message part may be used to carry multi-bit information for wake-up. Both parts of LP-WUS needs to be detected for correct wake-up indication. The link performance of the two parts may not be same. For example, the preamble part is normally more robust since it has the function to do synchronization too. The final performance of LP-WUS detection needs to consider the two parts jointly. Further, the UE needs to correctly detect the LP-WUS and the PEI PDCCH and/or the paging PDCCH to receive the associated paging PDSCH. The actual FAR or MDR for the operation should jointly consider the detection of the two parts of LP-WUS, PEI PDCCH and/or paging PDCCH. The CRC of the message part of LP-WUS helps to reduce the overall false alarm rate.

Proposal 4:

- FAR of 10% is not desired since it wastes UE power due to unwanted waking up the main radio

- The actual FAR or MDR for the operation should jointly consider the detection of LP-WUS, PEI PDCCH and/or paging PDCCH.

Based on the agreement in last RAN1 meetings, we provide our detailed simulation assumptions in Table 2.

Table 2: Simulation assumptions

| Attributes | Assumptions |
| --- | --- |
| Carrier frequency | 2.6GHz |
| Channel structure | Preamble + data + 8-bit CRC |
| Coding scheme | Manchester coding |
| Waveform | OOK |
| Payload size | 32 bits |
| WUS waveform | Option 1: DFT-S-OFDM based, SCS 30kHz Option 2: One OOK per OFDM symbol, SCS 120/240kHz |
| SCS | Option 1: 30kHz Option 2: 120/240kHz |
| gNB Channel BW | 20MHz |
| WUS Bandwidth | 12/8/4/2 PRBs with 30kHz |
| Guard band | 0/1/2/3/6 PRBs |
| ACI | Interference to signal power ratio 0/3/6/10 dB |
| Filter | 3rd or 5th order Butterworth LP filter |
| Sampling Rate | Down-sampling factor 4 |
| Frequency error/drifts | 0/50/100/200 ppm |
| ADC | 1/2/3/4, ideal |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |

# Simulation results

According to the agreed simulation assumptions, link level simulations were performed to check multiple design aspects for LP-WUS, including

- The proper BW of LP-WUS

- The necessary guard band to mitigate adjacent subcarrier interference in the same cell

- The number of MC-OOK symbols that can be multiplexed in an OFDM symbol of main radio

- The impact of Manchester coding with spreading factor 2 or 4

- The impact of Rx filter. Specifically, Butterworth filter with order 3 and 5 are evaluated

- The ADC bit-width

- The impact of adjacent subcarrier interference

- The impact of time/frequency error

All the above listed factors impact the performance for LP-WUS detection. However, it would be possible to consider the multiple factors step by step. In a first step, we assume an OOK symbol duration of 33.3/8=4.17us which is similar to the OOK symbol duration adopted in 802.11ba. We assume the adjacent subcarriers for other NR channels/signals has the same Tx power as the subcarriers for LP-WUS. This should reflect a typical case or even worst case since LP-WUS, being a common signal, normally has larger Tx power than other unicast control/data channels in the same cell. The 3rd order Butterworth LP filter is modeled according to the example in the agreed link level simulation assumption. A guard band of 2 PRBs is assumed (from the results in Figure 4, 2 PRBs are sufficient to avoid the degradation due to the interference from the adjacent subcarriers).

The link performance for LP-WUS with different BW is provided in Figure 3. A larger BW always has better link performance. One reason is that the absolute transmission power is linearly increased with the BW. Therefore, to see the net effect of BW size, the SNR for the curves in left figure is normalized to a SNR of BW 12PRBs. Consequently, from the right figure, a low BW results in worse performance since a low BW cannot overcome the negative impact of frequency selective fading. A high BW only provides a marginal gain which also results in low frequency resource efficiency. In summary, a BW of 12PRB achieves a good balance of link performance and resource efficiency, which is assumed in further study.

Figure 3: BLER for LP-WUS with different BW

To evaluate a proper size for the guard band with a given order of Rx filter, the power of adjacent subcarriers of other NR channels/signals are increased to 6dB higher than a subcarrier of LP-WUS, which should be able to reflect the worst case. From Figure 4, the performance degradation is marginal with a guard band of 2 PRBs under 6dB adjacent subcarrier interference. This result confirms that guard band of 2 PRBs is reasonable assumption and is used in the following simulations.

Figure 4: BLER for LP-WUS with different guard band and interference power

The number of OOK symbols in an OFDM symbol, or equivalently the OOK symbol duration is another key factor for LP-WUS design. The spreading factor by Manchester coding is another related factor. Therefore, the number of OOK symbols per OFDM symbol (M) and the spreading factor (SF) are jointly simulated. The results are provided in Figure 5. Similar to Figure 3, the SNR in right figure is normalized for same Tx power as M8/SF4. From the results, a larger OOK symbol duration (low M) is better to overcome the multiple delay of the channel. On the other hand, larger OOK symbol duration (low M or high SF) may lose the power gain due to non-coherent detection (about 2dB performance gain is observed when OOK symbol duration is doubled, i.e., 3dB power increase). From the results, M=4 or 8 is better candidate for channel of TDL-C 300ns.

Figure 5: BLER for LP-WUS with different number of OOK symbols per OFDM symbol

In the next step, we evaluation the impact of bit-width of ADC. The link performance is provided in Figure 6. From the results, ADC bit-width of 3 bits or more achieves similar good performance. Thus, 3 bits can be good choice with reasonable cost and performance.

Figure 6: BLER for LP-WUS with different bit-width of ADC

Due to the non-coherent detection and the protection of guard band, the performance of LP-WUS is not sensitive to frequency error. On the other hand, if the interference from adjacent NR channels/signals are increased, certain degradation is still observed for the high frequency error.

Figure 7: BLER for LP-WUS with frequency error

The timing error has significant impact on OOK based transmission if the time error is not corrected. In this simulation, timing error is modeled as consequence of frequency drift of 1 second (about the paging cycle in power consumption evaluation). The related results are provided in Figure 8. Without any correction as shown in left figure, it is completely failed for frequency drift of 200/20/2 ppm. In fact, a large degradation is observed even for frequency drift 1 ppm. However, such negative impact can be easily removed by timing correction as shown in right figure. Note: a preamble-based exhausted search for best timing is modeled in the simulation..

Figure 8: BLER for LP-WUS with different timing error

Observation 3: Based on the link level simulations

- A BW of 12PRB for LP-WUS achieves a good balance of link performance and resource efficiency

- A guard band of 2 PRBs can be sufficient to reduce interference from the adjacent subcarriers.

- At least for channel TDL-C 300ns, the number of OOK symbols per OFDM symbol can be 8 or 4.

- ADC of 3 bits more achieves similar good performance.

- The performance of LP-WUS is not sensitive to frequency error. Degradation is observed for the high frequency error if interference from adjacent subcarriers is high

- The timing error has significant impact on OOK based transmission if the time error is not corrected. The degradation is marginal if time error is corrected.

# Preliminary MIL for link budget

As shown in the left figure in Figure 5, the required SNR for 1% BLER for LP-WUS detection ranges from -1.5 ~ 5.5dB, depending on the number of OOK symbols per OFDM symbol and the spreading factor of Manchester coding, with an assumption of payload size 32bits + 8-bit CRC. This range of required SNR can be used to derive some preliminary insight on the link budget MIL for LP-WUS.

Table 3: Link budget MIL

| System configuration | Common PDCCH | PUSCH (1Mbps) | LP-WUS |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 2.6 |  |  |
| Pathloss model | TDL-C, 300ns |  |  |
| UE speed (km/h) | 3 |  |  |
| Number of transmit chains | 4 | 1 | 4 |
| Number of receive antennas | 1 | 4 | 1 |
| PSD | 33 |  | 33 |
| Occupied BW (PRB) | 48 | 10 | 12 |
| Transmit power for occupied BW | 45.38 | 23 | 39.35 |
| Data channel EIRP | 62.86 | 22 | 56.84 |
| Receiver noise figure (dB) | 7.00 | 5.00 | 15.00 |
| Required SNR (dB) | -4.50 | -5.25 | -1.5 – 5.5 |
| Receiver sensitivity | -97.12 | -99.61 | -93.95 |
| Link budget (MIL) | -158.98 | 140.35 | -147.98 - 140.98 |

From the above calculation, MIL for LP-WUS can be better than PUSCH. However, there is a big gap between the MIL for LP-WUS and common PDCCH. Note: the MIL of unicast PUSCH largely depends on the assumed data rate for the UL traffic.

Observation 4:

- A preliminary calculation shows that the MIL for LP-WUS can be better than PUSCH but is much worse than common PDCCH.

# Conclusions

In this contribution, we presented our views on methodology and performance metric for the evaluations for LP-WUS/WUR. We made the following observations and proposals

Proposal 1: On power consumption of main radio

- For the ultra-deep sleep mode, the relative power, Ramp-up and down transition energy and ramp-up time can be respectively assumed as 0.015, 10000 and 400ms in the evaluation.

- The sync/re-sync time based on 1,2,3 SSBs that should be detected according to the SNR at UE

Proposal 2: On power consumption of LP-WUR

- The transition time can be modeled as 1ms for a low relative power consumption of LP-WUR

- The transition time can be modeled as 10ms for low relative power consumption of LP-WUR from 1 to 4

Observation 1: For idle/inactive mode, without consideration on RRM by main radio

- Significant benefit on power saving in LP-WUS based operation are observed except that LP-WUS is always ON with ON power of e.g., 4 units. The issue can be simply solved by duty-cycle based LP-WUS detection.

- Large power saving gain is still observed even when extremely high total transition energy 40000 units.

- LP-WUS enables a latency slightly higher than IDRX and significantly lower than eDRX.

- The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

Observation 2: For idle/inactive mode with relaxed RRM by main radio

- Large gain on power saving in LP-WUS based operation are observed. For the case LP-WUS ON power of 4 units, power saving gain can be achieved in duty-cycle based LP-WUS detection.

- There is a loss for extremely high total transition energy 40000 units.

- LP-WUS enables a latency slightly higher than IDRX and significantly lower than eDRX.

- The lowest power consumption is observed when paging subgroup is indicated by LP-WUS

Proposal 3: For idle/inactive mode

- Further study the cases that LP-WUS can provide an indication on paging group, paging subgroup or UE ID

Proposal 4:

- FAR of 10% is not desired since it wastes UE power due to unwanted waking up the main radio

- The actual FAR or MDR for the operation should jointly consider the detection of LP-WUS, PEI PDCCH and/or paging PDCCH.

Observation 3: Based on the link level simulations

- A BW of 12PRB for LP-WUS achieves a good balance of link performance and resource efficiency

- A guard band of 2 PRBs can be sufficient to reduce interference from the adjacent subcarriers.

- At least for channel TDL-C 300ns, the number of OOK symbols per OFDM symbol can be 8 or 4.

- ADC of 3 bits more achieves similar good performance.

- The performance of LP-WUS is not sensitive to frequency error. Degradation is observed for the high frequency error if interference from adjacent subcarriers is high

- The timing error has significant impact on OOK based transmission if the time error is not corrected. The degradation is marginal if time error is corrected.

Observation 4:

- A preliminary calculation shows that the MIL for LP-WUS can be better than PUSCH but is much worse than common PDCCH.

# References

[1] RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

[2] R1-2300970, Discussion on LP-WUS receiver architecture, Intel

[3] R1-2300971, Discussions on L1 signal design and procedure for low power WUS, Intel

[4] TS38.840, Study on User Equipment (UE) power saving in NR

[5] TS38.875, Study on support of reduced capability NR devices

[6] TS38.838, Study on XR (Extended Reality) Evaluations for NR