3GPP TSG RAN WG1 #112		    													                     R1-2300972

Athens, Greece, February 27th – March 3rd, 2023

Source: 	Intel Corporation

Title:	Discussions on PRACH coverage enhancement

Agenda item:	9.14.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#111 meeting, the following working assumptions and agreements were made regarding PRACH coverage enhancement [1]:

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

- Study at least the following case for multiple PRACH transmissions with different Tx beams.

  - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

  - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

  - Note: not related to decision on CFRA

- Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

Working Assumption

Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

    - Focus on FR2.

      - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

    - Performance metric: 0.1% false alarm, 1% miss-detection

    - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

  - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

  - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In the contribution, we present our views on PRACH coverage enhancement. Our views on power domain enhancement and dynamic switching between DFT-S-OFDM and CP-OFDM waveform are described in our companion contributions [2] and [3], respectively.

# Multiple PRACH transmissions with same beams

## Differentiation of multiple PRACH transmissions

During Rel-17 NR coverage enhancement study item phase, PRACH was identified as one of the bottleneck channels that need coverage enhancement [4]. Given that PRACH transmission is the very first step during RACH procedure, PRACH performance is critical for the system operation, especially when considering initial access and beam failure recovery. In order to improve the PRACH coverage performance during 4-step RACH procedure, one straightforward approach is to employ repetitions with the same Tx beams for the transmission of PRACH.

Note that the PRACH coverage enhancement applies to short PRACH formats and can also apply to other formats when applicable. Considering that only one PRACH sequence is included for PRACH format 0, PRACH repetition may also apply for the transmission of PRACH format 0 to improve the coverage for FR1. Hence, in our view, PRACH repetition is applied for all short PRACH formats and PRACH format 0.

Figure 1 illustrates PRACH detection performance with repetitions. In the simulations, 0.1% false alarm probability was assumed for PRACH format 0 and 700 MHz carrier frequency. The detailed simulation assumptions are provided in Table 2 in the Appendix. From the figure, it can be observed that ~2.1dB performance gain can be achieved by doubling the repetition levels for PRACH transmission.

Figure 1. PRACH detection performance with repetitions

Observation 1

- ~2.1dB performance gain can be achieved for PRACH transmission when repetition level is doubled.

Proposal 1

- PRACH repetition is applied for all short PRACH formats and PRACH format 0.

At the RAN1#111 meeting, several options were identified to differentiate single PRACH transmission and multiple PRACH transmissions as follows:

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

Similar to the request of Msg3 repetition as defined in Rel-17 coverage enhancement, either Option 1 with shared PRACH occasions with separate PRACH preambles or Option 2 with separate PRACH occasions can be supported to differentiate between single PRACH transmission and multiple PRACH transmissions. For Option 3, part of multiple PRACHs are transmitted with separate preamble on shared PRACH occasions while remaining multiple PRACHs are transmitted on separate PRACH occasions, which can be viewed as a mixed version of Option 1 and Option 2. Given the fact that gNB may combine multiple PRACH detection statistics to improve the performance, this option may introduce unbalanced detection performance for multiple PRACH transmissions, especially when considering the loading conditions for cell edge and cell center UEs may be different. In our view, only Option 1 and Option 2 are supported to differentiate single PRACH transmission and multiple PRACH transmissions.

Figure 2 illustrates PRACH resource partitioning when shared PRACH occasions are configured for PRACH with and without repetitions. In the figure, different PRACH preambles associated with the same SSB index can be allocated for PRACH transmission with different repetition levels.

Figure 2. Preambles allocated for PRACH with different repetition levels

Proposal 2

- Option 1 and Option 2 are supported to differentiate single PRACH transmission and multiple PRACH transmissions

  - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

  - Option 2: Multiple PRACH are transmitted on separate ROs.

Further, at the RAN1#111 meeting, the following two options were considered for down-selection [1]:

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions

While it is evident that Option 1 may simplify system operation, it may not be desirable from perspective of efficient resource management, especially when considering that different UEs may have different coverage status. More specifically, if gNB only configures one value for the number of multiple PRACH transmissions by assuming the worst coverage scenario in the network, large PRACH resource waste may be expected given the fact that UE with relatively poor channel conditions may not need such high number of PRACH repetitions for coverage enhancement. In order to allow flexible system operation and efficient PRACH resource management, in our view, it is more desirable to allow gNB to configure more than one values for the number of multiple PRACH transmissions.

In case multiple number of multiple PRACH transmissions are configured, similar to differentiation of single PRACH transmission and multiple PRACH transmissions, PRACH resource partitioning can be employed to differentiate different number of repetitions for multiple PRACH transmissions. Based on PRACH resource partitioning, each PRACH resource may correspond to one repetition level. In this regard, after successful detection of PRACH preambles in the configured resources, gNB may combine multiple PRACH detection statistics to improve the performance. Note that in order to avoid fragmented PRACH resources and minimize the impact on the legacy system, it is preferable to limit the set of repetitions for multiple PRACH transmissions or only allow separate preamble on shared PRACH resources to differentiate different number of repetitions for multiple PRACH transmissions.

Proposal 3

- For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

At the RAN1#111 meeting, it was agreed that SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt [1]. In this case, when the measured SSB-RSRP is lower than an RSPR threshold, UE may trigger multiple PRACH transmissions with same Tx beam. Further, if multiple repetition levels are configured for multiple PRACH transmissions, multiple thresholds may need to be defined accordingly for proper operation.

For UEs that support both PRACH repetition and Msg3 PUSCH repetitions, a common SS-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions. In this case, after successfully detecting the PRACH with repetitions, gNB may have good knowledge of channel condition of UE and can inform UE to transmit Msg3 PUSCH with appropriate number of repetitions.

Proposal 4

- A common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions.

## Frequency hopping for PRACH repetitions

At the RAN1#110b-e meeting, it was agreed that at least ROs located at different time instances can be utilized for the multiple PRACH transmissions with same Tx beam [5]. It is FFS whether the starting RB of ROs can be different at different time instances for multiple PRACH transmissions and multiple PRACH transmissions can be located in the same time instance, e.g., for UEs with multiple Tx chains.

In our view, for multiple PRACH transmissions with same Tx beam, frequency hopping can be employed in conjunction with repetitions for PRACH transmission in order to exploit the benefit of frequency diversity and hence further improve the detection performance. Depending on the number of time domain PRACH occasions in a PRACH slot, frequency hopping within or across the PRACH slot may be employed for PRACH repetitions.

Figure 3 illustrates PRACH detection performance with repetitions and frequency hopping. From the figure, it can be observed that in case of 2 and 4 repetitions, an additional 2.5dB performance gain can be achieved when frequency hopping is employed for PRACH transmission.

Figure 3. PRACH detection performance with repetitions and frequency hopping

Observation 2

- For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping.

Further, it was discussed in the last meeting whether simultaneous PRACH transmissions using multiple Tx chains can be supported for multiple PRACH transmission with same Tx beam. This highly depends on UE capability on whether UE is equipped with multiple Tx chains and without power sharing. However, during initial access, gNB may not know the UE capability and hence may not be able to perform coherent combining on the PRACH detection statistic. In this regard, performance degradation may be expected compared to the case when multiple PRACH transmissions are multiplexed in the TDM manner. In our view, multiple PRACH transmissions are not located in the same time instances.

Proposal 5

- For multiple PRACH transmission with same Tx beam,

  - Starting RBs of ROs can be different at different time instance for the multiple PRACH transmissions.

  - Multiple PRACH transmission are not located in the same time instances.

## Association between SSB and PRACH repetitions

For PRACH repetitions with same Tx beam, multiple PRACH transmissions in the valid PRACH occasions need to be associated with the same SSB. Further, a PRACH repetition window can be defined to ensure a common understanding between gNB and UE on the potential starting and ending position of the corresponding PRACH occasions used for PRACH repetitions.

At the RAN1#110b-e meeting, it was agreed that same PRACH preamble during multiple PRACH transmissions in one RACH attempt [5]. The main motivation is to allow gNB to perform combining on multiple detection statistics for better performance. It is for further study on whether different preambles can be used during multiple PRACH transmissions. Note that at least for CBRA procedure, if supported, cell specific PRACH sequence pattern across different transmissions needs to be aligned between gNB and UE side so as to ensure the PRACH coverage enhancement. In this regard, if two UEs select a same preamble index in the first PRACH transmission, consistent preamble collision will still happen for multiple PRACH transmissions. In our view, considering the additional specification impact and no clear benefit compared to same preamble, different preambles during multiple PRACH transmission in one attempt is not supported.

Proposal 6

- A PRACH repetition window can be defined to map PRACH repetitions into an SSB.

- Different preambles during multiple PRACH transmission in one attempt is not supported.

Further, it is more desirable to reuse the existing mapping rule between SSB and PRACH occasions supporting PRACH repetition with same Tx beam in order to minimize the specification impact. In case of shared PRACH occasions for PRACH with and without repetitions, the PRACH occasions used for the PRACH repetitions can be the ones mapped to the same SSB after PRACH occasions are associated with all the SSBs. Depending on the configuration of the number of PRACH occasions associated with an SSB, PRACH repetitions may be transmitted in non-consecutive slots. Figure 4 illustrates one example of association between and PRACH occasions for PRACH repetitions.

Figure 4. Association between SSB and PRACH occasions for PRACH repetitions

Proposal 7

- Existing mapping rule between SSB and PRACH occasions is reused for multiple PRACH transmission with same Tx beam.

- In case of shared PRACH occasions for single and multiple PRACH transmissions, PRACH occasions used for PRACH repetitions can be the ones mapped to a same SSB after PRACH occasions are associated with all the SSBs.

## RAR monitoring window and RA-RNTI determination

During 4-step RACH procedure, UE monitors PDCCH for scheduling Msg2 transmission in a RAR monitoring window, which starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS, that is at least one symbol after the last symbol of the PRACH occasion corresponding to the PRACH transmission. When multiple PRACH transmissions with same Tx beam are employed during 4-step RACH procedure, starting positioning of the RAR monitoring window needs to be determined accordingly.

At the RAN1#110b-e meeting, two options were considered for RAR monitoring of multiple PRACH transmission with same Tx beam [5]:

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design

- Option 2: Only one RAR window for all of the multiple PRACH transmissions

For these two options, it is evident that for Option 1 with one RAR window per each PRACH transmission, access latency may be reduced for 4-step RACH procedure. This option, however, may require gNB to perform opportunistic PRACH detection by attempting to detect the PRACH repetition in each PRACH transmission occasion. Further, additional specification impact may be expected on how to handle the overlapping RAR window.

For Option 2, UE monitors the PDCCH for scheduling Msg2 after the last symbol of the last PRACH occasion corresponding to the PRACH repetitions. In this case, gNB can perform combining for detection statistics on all PRACH transmission occasions so as to ensure the PRACH detection performance in case of the PRACH repetitions. However, this mechanism may prolong the latency for 4-step RACH procedure as UE has to wait for the last PRACH occasion to start RAR monitoring window.

Based on the discussions above, considering the detection performance and implementation complexity, it would be more desirable to support Option 2 for RAR window determination, i.e., only one RAR window for all of the multiple PRACH transmissions. Further, UE monitors the PDCCH for scheduling Msg2 after the last symbol of the last PRACH occasion corresponding to the PRACH repetitions.

Proposal 8

- For RAR window determination, Option 2 is supported, i.e., only one RAR window for all of the multiple PRACH transmissions.

As defined in TS 38.321, RA-RNTI for a single PRACH transmission is determined as follows [6]:

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

For multiple PRACH transmission with same Tx beam, RA-RNTI determination may need to be updated to avoid ambiguity between gNB and UE on Msg2. As mentioned above, for RAR monitoring window, it is more desirable to consider one RAR window for all of the multiple PRACH transmissions. To follow same design principle, RA-RNTI can be determined based on the last RO corresponding to the multiple PRACH transmissions.

Proposal 9

- For multiple PRACH transmissions with same Tx beam, RA-RNTI is determined based on the last RO corresponding to the multiple PRACH transmissions.

# Multiple PRACH transmissions with different beams

At the RAN1#111 meeting, it was agreed to further study UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS [1]. Note that for multiple PRACH transmission with different beams, the following two scenarios can be considered:

- Scenario 1: SSB is transmitted with a wider beamwidth, while multiple PRACH can be transmitted with narrower beam sweeping.

- Scenario 2: during initial access, UE that is not capable of full beam correspondence may perform PRACH with beam sweeping.

For the Scenario 1, a hierarchical beam structure can be employed at gNB for system operating at mmWave bands, where SSB may be transmitted using a wider beamwidth to cover a larger area and CSI-RS can be transmitted using a narrower beam based on beam refinement procedure for higher data rate. When UE attempts to access the network, the UE will select one SSB with RSRP above threshold and transmit the PRACH using the Tx beam based on association between SSB and PRACH occasion.

According to current NR specification, UE only transmits a PRACH with single Tx beam and attempts to monitor RAR within a monitoring window. If the PRACH is not detected by gNB, the UE needs to wait for next PRACH opportunity for PRACH transmission with same or different Tx beams after the RAR monitoring window. To reduce the access latency and improve the reliability of the PRACH detection during 4-step RACH procedure, the UE may transmit multiple PRACH repetitions with narrower beam sweeping.

Figure 5 illustrates PRACH transmissions with narrow beam sweeping during initial access. Note that all these narrower beams may be linked to the same SSB with wider beamwidth. In this case, same procedure as defined for multiple PRACH transmission with same Tx beams can be applied.

Figure 5. PRACH transmissions with narrow beam sweeping

Observation 3

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection.

For the Scenario 2, as defined in UE feature list and illustrated in Table 1, UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 [7]. This indicates that during initial access, UE may not support full beam correspondence.

Table 1. UE feature group for beam correspondence

| 2-20 | Beam correspondence | Support Beam correspondence | beamCorrespondenceWithoutUL-BeamSweeping | MIMO-ParametersPerBand | No | Applicable only to FR2 | Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports. | Mandatory with capability signalling - UE that fulfils the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

When UE is not capable of full beam correspondence, Tx and Rx beam are not fully aligned, which indicates that the selected Rx beam for the corresponding SSB may not be the most suitable Tx beam for PRACH transmission. If UE utilizes the single Tx beam for PRACH transmission, gNB may not be able to detect the PRACH preamble due to Tx and Rx beam mis-alignment. In this case, it may be more beneficial to employ the PRACH beam sweeping at UE side before the start of RAR monitoring window to improve the detection performance and reduce the access latency, especially when considering the scenario that narrow beam is applied for SSB transmission and PRACH detection.

Observation 4

- When UE is not capable of full beam correspondence, it may be more beneficial to employ the PRACH beam sweeping at UE side to improve the detection performance and reduce the access latency.

Note that for multiple PRACH transmission with different beams, two options can be further considered, which depends on whether the ROs used for the multiple PRACH transmissions are associated with same or different SSBs. It is evident that when multiple PRACH transmissions are associated with different SSBs, substantial specification impact, e.g., the SSB and RO association, determination of QCL assumption for RAR monitoring, etc., can be expected. On the contrary, in case of multiple PRACH transmissions on ROs associated with same SSB, the design for multiple PRACH transmissions with same Tx beam can be largely reused, which can help minimize specification effort.

Based on the discussions above, considering the specification impact, and the benefits of access latency reduction and improved detection performance, in our view, at least the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported for multiple PRACH transmissions with different Tx beams in Rel-18.

Proposal 10:

- For multiple PRACH transmissions with different Tx beams, at least the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported in Rel-18.

# Conclusions

In this contribution, we presented our views on PRACH coverage enhancement. Further, we summarize the observations and proposals as follows:

Observation 1

- ~2.1dB performance gain can be achieved for PRACH transmission when repetition level is doubled.

Observation 2

- For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping.

Observation 3

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection.

Observation 4

- When UE is not capable of full beam correspondence, it may be more beneficial to employ the PRACH beam sweeping at UE side to improve the detection performance and reduce the access latency.

Proposal 1

- PRACH repetition is applied for all short PRACH formats and PRACH format 0.

Proposal 2

- Option 1 and Option 2 are supported to differentiate single PRACH transmission and multiple PRACH transmissions

  - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

  - Option 2: Multiple PRACH are transmitted on separate ROs.

Proposal 3

- For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

Proposal 4

- A common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions.

Proposal 5

- For multiple PRACH transmission with same Tx beam,

  - Starting RBs of ROs can be different at different time instance for the multiple PRACH transmissions.

  - Multiple PRACH transmission are not located in the same time instances.

Proposal 6

- A PRACH repetition window can be defined to map PRACH repetitions into an SSB.

- Different preambles during multiple PRACH transmission in one attempt is not supported.

Proposal 7

- Existing mapping rule between SSB and PRACH occasions is reused for multiple PRACH transmission with same Tx beam.

- In case of shared PRACH occasions for single and multiple PRACH transmissions, PRACH occasions used for PRACH repetitions can be the ones mapped to a same SSB after PRACH occasions are associated with all the SSBs.

Proposal 8

- For RAR window determination, Option 2 is supported, i.e., only one RAR window for all of the multiple PRACH transmissions.

Proposal 9

- For multiple PRACH transmissions with same Tx beam, RA-RNTI is determined based on the last RO corresponding to the multiple PRACH transmissions.

Proposal 10:

- For multiple PRACH transmissions with different Tx beams, at least the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported in Rel-18.

# References

- Chairman’s notes, RAN1#111 Meeting, November 2022

- R1-2300973, “Discussions on power domain enhancement”, Intel Corporation, February 2023

- R1-2300974, “Dynamic switching between DFT-S-OFDM and CP-OFDM”, Intel Corporation, February 2023

- TR 38.830, “Study on NR coverage enhancements”.

- Chairman’s notes, RAN1#110b-e Meeting, October 2022

- TS 38.321, “NR: Medium Access Control (MAC) protocol specification”

- TS 38.322, “NR: User Equipment (UE) feature list”.

# Appendix

Table 2. Simulation assumption for PRACH repetition

| Parameters | Values |
| --- | --- |
| Carrier frequency | 700MHz |
| PRACH formats | Format 0 |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Channel model | TDL-C 300ns with 3km/h |
| Timing offset | 0 |
| Frequency offset | 0 |
| Performance metrics | 1% PRACH misdetection probability 0.1% false alarm probability |