3GPP TSG-RAN WG1 #112	R1-2301034

Athens, Greece, 27th Feb – 3rd May 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on XR-specific capacity enhancements

Document for:        	Discussion and decision

Agenda Item:         	9.10.1		XR-specific capacity enhancements

- Introduction

In RAN#98-e, new WID on XR enhancements was approved [1]. From RAN1 perspective, the following objectives were specified for XR-specific capacity improvements.

Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

In this contribution, we provide the discussion on the potential capacity enhancements for XR.

- Discussion

Compared to dynamic grant (DG), configured grant (CG) is effective in reducing the delay. CG provides a UE with periodic resource allocations without having to transmit scheduling request (SR) and wait for scheduling DCI. CG is useful for minimizing the transmission delay to meet the packet delay budget (PDB) of XR services. CG has another characteristic that it is suitable for transmitting small and fixed size packets. Therefore, for UL scene/video traffic with large packets, one PUSCH transmission occasion within one CG period may not be enough. In this case, additional dynamic scheduling is required, which causes additional delay. In order to solve this problem, it was agreed in the previous meeting to support multiple PUSCH transmission occasions within one CG period. In addition, it was also agreed that the UE can dynamically indicate the unused CG PUSCH occasion(s) to adapt to scene/video traffic whose packet size varies significantly. The indication by the UE allows the gNB to reallocate unused occasion(s) for other transmissions. This enhancement could help to avoid wasting resources when the UE is allocated more CG PUSCH occasions than it needs.

Observation1	: The following enhancements are beneficial for XR traffic.

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration.

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE.

To allocate multiple PUSCH occasions within one CG period, it can be considered to reuse the framework for scheduling multiple PUSCHs by a single DCI. In the current specification, for DG, multiple PUSCH occasions can be scheduled by DCI indicating the row index containing multiple SLIV entries in the TDRA table. A similar mechanism can be enabled in the CG configurations for simplicity of specification. It can be indicated to schedule one or more PUSCH occasions within one CG period by the TDRA field included in higher layer configurations for CG type 1 or an activation DCI for CG type 2. Additionally, it may also be necessary to study whether HARQ process IDs can be assigned separately for each of multiple PUSCH occasions. If the same approach as DG is introduced, HARQ process IDs incremented by 1 in scheduled order may be used for each PUSCH occasion.

Proposal 1:	The framework for scheduling multiple PUSCHs by a single DCI can be reused for CG enhancements.

Regarding the dynamic indication of unused occasion(s), two options were discussed: CG-UCI or a new UCI. When introducing a new UCI, the specification impact may be larger. On the other hand, with CG-UCI, the existing framework can be reused by simply introducing new fields to indicate unused occasion(s). For example, the current multiplexing scheme of CG-UCI on PUSCH can be reused. Therefore, it could be preferred to reuse CG-UCI than to introduce a new UCI for the dynamic indication of the unused CG PUSCH occasion(s).

Proposal 2:	For the dynamic indication of the unused CG PUSCH occasion(s), it could be preferred to reuse CG-UCI than to introduce a new UCI.

Then how UCI indicates unused occasion(s) needs to be discussed. One possible way is to introduce a bitmap where each bit indicates whether the corresponding PUSCH occasion is unused or not. For example, UCI can include the indication for all PUSCH occasions within the CG period when the UCI is transmitted. In this case, the required bitmap size is equal to the number of PUSCH occasions. Another way to reduce the UCI size is to indicate the number of PUSCH occasions that the UE will use. For example, after receiving UCI, the gNB can implicitly assume that the indicated number of consecutive PUSCH occasions are used and the remaining ones are unused. However, it may not have as much flexibility as the indication using a bitmap.

Figure 1: Example of how UCI indicates unused occasion(s) (Option 1 and 2)

Alternatively, UCI can be multiplexed on the last used occasion within one CG period and indicates that the remaining PUSCH occasions are unused. Only one bit is sufficient for this indication, thus further reducing the UCI size. However, the delay in receiving the indication by the gNB may limit the time available for reallocation of unused occasions. Therefore, this approach may not work well for reducing wasted resources and increasing capacity.

Figure 2: Example of how UCI indicates unused occasion(s) (Option 3)

Proposal 3:	RAN1 to study the following options for how UCI indicates the unused occasion(s).

- Option.1: Introduce a bitmap indicating whether each PUSCH occasion is unused.

- Option.2: Indicate the number of PUSCH occasions that the UE will use

- Option.3: Multiplexed on the last used occasion within one CG period to indicate that the remaining PUSCH occasions are unused.

In terms of signalling overhead, it can also be considered that the UCI indication is applied over multiple CG periods. Frequent UCI transmissions can be prevented at the expense of increasing the UCI size. However, if a long CG period is configured, it may be necessary to transmit UCI before the UE determines which occasion to use in the subsequent CG periods. Therefore, it would be better to apply the UCI indication only within the CG period when the UCI is transmitted.

Proposal 4:	The UCI indication can be applied only within the CG period when the UCI is transmitted.

- Summary and proposal

In this contribution, we provided the following observation and proposals.

Observation1	: The following enhancements are beneficial for XR traffic.

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration.

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE.

Proposal 1:	The framework for scheduling multiple PUSCHs by a single DCI can be reused for CG enhancements.

Proposal 2:	For the dynamic indication of the unused CG PUSCH occasion(s), it could be preferred to reuse CG-UCI than to introduce a new UCI.

Proposal 3:	RAN1 to study the following options for how UCI indicates the unused occasion(s).

- Option.1: Introduce a bitmap indicating whether each PUSCH occasion is unused.

- Option.2: Indicate the number of PUSCH occasions that the UE will use

- Option.3: Multiplexed on the last used occasion within one CG period to indicate that the remaining PUSCH occasions are unused.

Proposal 4:	The UCI indication can be applied only within the CG period when the UCI is transmitted.

- References

[1]	RP-223502, New WID on XR Enhancements for NR, Nokia, Qualcomm