3GPP TSG RAN WG1 #112			R1- 2301068

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.5.4

Source: 	LG Electronics

Title: 	Discussion on Bandwidth aggregation for positioning measurements

Document for:	Discussion and decision

- Introduction

In RAN#98 meeting a work item on Expanded and Improved NR Positioning was approved [1]. For positioning accuracy improvement, it was agreed to specify PRS/SRS bandwidth aggregation for positioning measurements across multiple carriers as follows:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

During the study item phase, RAN4 studied several aspects of the feasibility of PRS/SRS bandwidth aggregation support [2]. As a conclusion, it was agreed that PRS/SRS bandwidth aggregation for intra-band contiguous carriers is feasible considering timing error, phase coherency, frequency errors, power imbalance, etc.

In this contribution, we discuss aspects that would be required to support PRS/SRS bandwidth aggregation from RAN1 perspective.

- Discussion

DL PRS bandwidth aggregation

RAN4 studied feasibility of PRS/SRS bandwidth aggregation for intra-band contiguous carrier toward for higher accuracy positioning. From RF aspects, it was concluded that PRS/SRS bandwidth aggregation for intra-band contiguous carriers is feasible for single chain Tx/Rx architectures at both the UE and gNB. In terms of RRM, RAN4 also concluded bandwidth aggregation is feasible when a common numerology is assumed across all intra-band contiguous PFL transmitted by the same TRP. To utilize DL PRS bandwidth aggregation, not only the TRPs but also the UEs who measure the DL PRS have to know which set of resources would be transmitted with same Tx antenna chain.

In NR, maximum 64 TRP ID (i.e. dl-prs-ID) can be configured for a PFL and up to 4 PFLs can be configured. As discussed above, a set of DL PRS resources from different PFLs transmitted by the same TRP is required for bandwidth aggregation. However, the current specification does not provide a way for the UE to assume the same TRP for TRPs belonging to different PFLs. Since the baseline assumption for bandwidth aggregation requires gNB to transmit DL PRSs of different PFLs with single Tx chain, it would be necessary to introduce information for same TRP assumption. Alternatively, configuring information regarding DL PRS bandwidth aggregation per PRS resource or resource set could also be a considered.

Proposal 1: Discuss signalling method for UE to assume single Tx chain for PRS resources from different PFLs

For NR positioning in Release 16/17, the positioning measurement reports include information about the measured PRS resources, such as TRP ID, PRS resource (Set) ID, time stamp, and other data, which could be helpful to LMF in achieving more accurate positioning. Since multiple PRS resources from different PFLs are used to obtain a measurement for PRS bandwidth aggregation, it is necessary to discuss how to report information on the PRS resources used

Proposal 2: Discuss the necessary information to be included in positioning measurement reports when utilizing DL PRS bandwidth aggregation

UL SRS

To take advantage of UL SRS bandwidth aggregation, gNBs which receive the SRS shall be able to assume UE behavior in advance. Since the UL SRS is configured, activated and triggered by gNB, it can configure a UE to perform SRS bandwidth aggregation for a set of SRS resources. For example, SRS resource (set) configuration contains association information for other SRS resources (sets) could be introduced to UE determining bandwidth aggregation behavior. Details how to design the association information regarding SRS bandwidth aggregation should be discussed further.

Currently, semi-persistent/aperiodic SRS transmission can be activated/triggered for a carrier. The motivation of the bandwidth aggregation is to utilize multiple SRS resources that are transmitted simultaneously. To reduce the signaling overhead, a method of activating/triggering more than one SRS resource sets transmitted by different carriers using single indication may be considered.

Proposal 3: gNB should configure sets of SRS resources for the UE to transmit with a single Tx chain in order to support SRS bandwidth aggregation

Proposal 4: Discuss method for activating/triggering SRS on multiple carriers through a single indication.

- Conclusion

In this contribution, we discussed on potential enhancements on PRS/SRS bandwidth aggregation for positioning measurements. From the discussion, we obtained following proposals:

Proposal 1: Discuss signalling method for UE to assume single Tx chain for PRS resources from different PFLs

Proposal 2: Discuss the necessary information to be included in positioning measurement reports when utilizing DL PRS bandwidth aggregation

Proposal 3: gNB should configure sets of SRS resources for the UE to transmit with a single Tx chain in order to support SRS bandwidth aggregation

Proposal 4: Discuss method for activating/triggering SRS on multiple carriers through a single indication.

- Reference

- RP-223549: "New WID on Expanded and Improved NR Positioning".

- 3GPP TR 38.859: “Study on expanded and improved NR positioning”.