3GPP TSG RAN WG1 #112   	                              R1-2301073

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		9.11.2

Source:		LG Electronics

Title:			Discussion on network verified UE location for NR NTN

Document for:	Discussion and Decision

Introduction

In RAN1#111 meeting, followings were concluded for network verified UE location. Also, other observations are listed in the Annex.

Conclusion: For network verification of UE location in NR NTN with single satellite in view with multi-RTT positioning: - From RAN1 perspective, if the UE’s Rx-Tx time difference measurements report can be assumed to be trusted, multi-RTT positioning method using Rx-Tx time difference measurements can meet the accuracy requirement of less than 10km with 90% confidence, in case of: - At least LEO600 based deployment - Earth fixed cells - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RTT measurements - Note: the required over-the-air latency reported in evaluations ranged from less than 10s up to 180s Conclusion For network verification of UE location in NR NTN with single satellite in view with DL-TDOA positioning: From RAN1 perspective, if the UE’s RSTD measurements report can be assumed to be trusted, DL-TDOA positioning method can meet the accuracy requirement of less than 10km with 90% confidence, in case of: - At least LEO600 based deployment - Earth fixed cells - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RSTD measurements Note 1: the above is based on evaluation results that didn’t account for UE Clock drift Note 2: the required over-the-air latency reported in evaluations ranged from less than 20s up to 180s Note 3: The requirements of Network verification of UE location may not be met if realistic assumption on UE clock drift is considered. Conclusion For network verification of UE location in NR NTN based on multi-RTT using UE RX-TX time difference report, if the UE reports needed to perform multi-RTT can be assumed to be trusted, existing multi-RTT framework may be reused with potential enhancements to adapt it to NTN context. This may include, but not limited to: - If justified: NTN-specific definition of UE RX-TX time difference, including as an example, potential modifications to UE Rx – Tx time difference to enable network verification of UE location without introducing any additional measurements at the UE (with respect to Rel-17 NTN) - The following is not precluded: the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX, where TUE-RX – TUE-TX is directly derived from the timing advance TTA applied by the UE at a given subframe. - Above does not imply that the relevant work is prioritized. - Other assistance data (e.g. ephemeris) to be transferred from gNB to the LMF. - If justified: Other assistance data (e.g. to resolve ambiguity on mirror position issue) to be transferred from UE to LMF - If justified: Adaptations enabling Rx-TX measurements for Multi-RTT involving multiple cells within the same satellite For network verification of UE location in NR NTN based on DL-TDOA positioning, if the UE reports needed to perform DL-TDOA positioning can be assumed to be trusted, existing DL-TDOA positioning framework may be reused with potential enhancements to adapt it to NTN context.

In RAN#98-e, it was agreed to start normative phase on network verified UE location [1] and detailed work scope was captured in [2] as below.

4.1.3 Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

This contribution discusses and provides our view on network verified UE location for NR NTN.

Discussions

Potential specification impact for multi-RTT

As captured above, multi-RTT assuming a single satellite in view was agreed to prioritize in normative phase. Illustration of multi-RTT with a single satellite is depicted in Figure 1.

Figure 1. Illustration of multi-RTT with a single satellite

For the single satellite based multi-RTT technique, we need to firstly discuss how to determine RTT. In this context, following options were discussed during previous meetings.

- Option1: The multi-RTT positioning method makes use of the UE Rx-Tx time difference measurements of downlink signals (i.e. PRS) received from the satellite, measured by the UE and reported to the gNB and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS).

- Option 2: RTT measurement can be done in NTN by UE transmitting an UL reference signal followed by a report of the total TA applied to the reference signal. The network determines the RTT as the sum of the reported total TA and the timing error of the UL reference signal, i.e., RTT=Te+TA

Option 1 has the same principle of RTT determination as in TN. So, it is natural extension and is preferred. On the other hand, option 2 relies on the total TA and its error term as TA is used to compensate the delay due to round trip delay. Regarding option2, it was also discussed whether TA value reported by UE is trustful or not. Unfortunately, it was failed to achieve consensus. In our view, considering the fact that UE-specific TA is based on GNSS measurements which may be faked by a malicious UE, TA information reported by UE is not appropriate for the NW verified UE location unless there is clear agreement that TA information is trustful. Therefore, option 1 is preferred. Also, in order to conclude this issue, LS to SA3/SA3-LI to ask whether reported TA value is trustful or not is also needed.

Proposal #1: For RTT determination, option 1 is preferred.

- Option 1: The multi-RTT positioning method makes use of the UE Rx-Tx time difference measurements of downlink signals (i.e. PRS) received from the satellite, measured by the UE and reported to the gNB and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS).

Proposal #2: Send an SA3/SA3-LI to ask whether TA value reported by UE is trustful or not.

If option 1 is supported, following issues need to be further considered. One issue is how to reflect timing error / delay due to processing time in satellite and movement of satellite and/or UE. One way is to let LMF know the ephemeris information, UE movement, etc. In this way, LMF can handle delay/error caused by satellite and/or UE movement. Also, feeder link delay (e.g., Kmac and/or common TA) needs to be signaled to LMF for the RTT calculation.

Another issue is whether some enhancement on configuration of DL-PRS and SRS for the multiple measurements of UE Rx-Tx time difference and reporting of multiple UE Rx-Tx time difference are needed for multi-RTT based NW verified UE positioning. For example, one or more DL-PRS resources with the same or different ids can be associated for multiple measurements. In that case, some enhancement on configuring PRS/SRS and reporting multiple measurements with different time stamp may be needed.

The last issue is mirror image ambiguity for multi-RTT. As discussed in the last meeting, this issue can be resolved by gNB implementation, e.g., beamforming. With this method, it has a benefit that the specification impact can be minimized. Thus, discussions on mirror image ambiguity issue can be deprioritized in this normative phase.

Proposal #3: For multi-RTT, consider at least following assistant information to LMF

- Ephemeris information

- Feeder link delay (e.g., Kmac and/or common TA)

Conclusion

In this contribution, we discussed on network verified UE location for NR NTN. Based on the above discussion, we have the following proposals.

Proposal #1: For RTT determination, option 1 is preferred.

- Option 1: The multi-RTT positioning method makes use of the UE Rx-Tx time difference measurements of downlink signals (i.e. PRS) received from the satellite, measured by the UE and reported to the gNB and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS).

Proposal #2: Send an SA3/SA3-LI to ask whether TA value reported by UE is trustful or not.

Proposal #3: For multi-RTT, consider at least following assistant information to LMF

- Ephemeris information

- Feeder link delay (e.g., Kmac and/or common TA)

- Reference

[1] RP-223552, “Moderator’s summary of discussion [94e-24-R18-NR-NTN]”, Thales

[2] RP-223534, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- Annex

Observation

For network verified UE location based on multi-RTT positioning method using Rx-Tx time difference measurements with single satellite, assuming the ambiguity of the mirror image position is resolved, if the UE reports needed to perform multi-RTT can be assumed to be trusted:

- Five sources observed that multi-RTT positioning method can meet the NTN UE location verification accuracy requirement for LEO 600km:

- Four sources observed that the positioning horizontal accuracy of less than 10km can be achieved with few seconds over-the-air latency (less or equal to 10s) with 95-percentile confidence level.

  - Regarding the above observation, the following inputs were reported by companies:

  - One source reported that the timing measurement error of SRS is smaller than 232 ns with 95% probability. And the timing measurement error of PRS can be smaller than 13ns and 16ns with 95% probability under the bandwidth of 8.64 MHz and 4.5 MHz, respectively. This source, reported also that satellite’s movement between TX and RX measurements is taken into account in the evaluation.

  - One source reported that the timing measurement error is around 11ns for PRS detection with PRS bandwidth of 9.36 MHz. While for the SRS measurement, the maximum timing error is around 50ns with SRS bandwidth of 9.36 MHz. Further, this source, proposed that the RTT estimation error due to the movement of the satellite should be taken into account.

  - Note: this source provided results using 2D positioning method.

  - One source considered the maximum timing measurement error: 30ns, 50ns, 100ns, 200ns and uniform distribution of timing measurement error.

  - One source reported that the timing measurement error of 95 percentile is equal to 8ns and 12.6ns for PRS and SRS respectively with an oversampling of 8. To take into account satellite movement between TX and RX measurements, for RTT calculation this source observed that the RTT between a satellite and a UE at time t0 can be approximated by the sum of the one-way delay at t0-T and the one-way delay at t0+T when T is small, e.g., less than 200 ms.

- One source observed that the positioning horizontal accuracy of less than 10km can be achieved with 180 seconds latency for earth fixed beam with 90-percentile confidence level

  - This source reported that the timing measurement error of SRS and PRS can be smaller than 26.7ns and 6.1ns respectively with 95% probability under 30 degree elevation angle for LEO-600. Further, it observed that the satellite movement is taken into consideration when calculating the RTT.

- Two sources observed that multi-RTT positioning method require latency larger than 60 seconds for UE located nearby the orbital plane of a satellite during a certain time duration.

Note 1: Some companies observed that when 2D positioning method is used (e.g. when UE altitude is known to the network) better positioning latency/accuracy can be achieved compared to 3D positioning method.

Observation

For network verified UE location based on DL-TDOA positioning method with single satellite:

Eight companies commented on the suitability of the method: Assuming the ambiguity of the mirror image position is resolved and if the UE reports needed to perform DL-TDOA can be assumed to be trusted:

- Five sources observed that DL-TDOA positioning method can meet the NTN UE location verification accuracy requirement for LEO 600km without considering UE Clock drift:

  - Four sources observed that the positioning horizontal accuracy of less than 10km can be achieved with 30 seconds or less:

    - One of these 4 sources observed that horizontal positioning error is equal to 2.5km with 95% probability.

    - This source reported that the timing measurement error is around 11ns for PRS detection with PRS bandwidth of 9.36 MHz

      - Note 1: this source provided results using 2D positioning method.

    - One of these 4 sources observed that horizontal positioning error of DL-TDOA via PRS with 3 RSTDs and a latency of 24s is equal to 5.33km with 90% probability and 8.92km with 95% probability.

    - This source reported that the timing measurement error of PRS can be smaller than 13ns and 16ns with 95% probability under the bandwidth of 8.64 MHz and 4.5 MHz, respectively.

    - This source observed that existing CSI RS can be used to meet the requirement with comparable latency

    - One of these 4 sources observed that horizontal positioning accuracy for a latency of 30s with SNR of 5dB and with 90% probability is equal to 9.44km.

    - This source observed that the maximum timing measurement error that can be allowed to meet the accuracy requirement of 10km is about 80ns.

    - One of these 4 sources observed the horizontal positioning accuracy of less than 10km can be achieved for 90% of UEs with 12 seconds latency and for 95% of UEs with 20 seconds latency.

    - The maximum time measurement error considered by this source is equal to 6ns

  - One source observed that the horizontal positioning error of DL-TDOA method can be smaller than 10 km with over 80% probability with 180 seconds latency.

    - This source reported that the timing measurement error of PRS can be smaller than 6.1ns with 95%

- One source observed that the geometry of UE location relative to the satellite orbit will impact the positioning performance in DL-TDOA method e.g. for UE’s location at 200km away from the orbital plane, the NTN UE location verification accuracy requirement can be met and the positioning error of DL-TDOA method can be smaller than 10 km with 95% probability (for UE’s location at 200km away from the orbital plane) and a latency of 220 seconds in case of LEO600km and 342 seconds in case of LEO1200km. For UE located under the satellite orbit, NTN UE location verification accuracy requirement can be meet only with 30% probability.

  - Note 2: This source considered 10 ns UE Clock drift for all time measurement window.

Note 3: Position accuracy requirements may not be met if realistic assumption on UE clock drift is considered.

Observation

For network verified UE location based on UL-TDOA positioning method with single satellite:

Two companies commented on the suitability of the method: Assuming the ambiguity of the mirror image position is resolved and if the measurements needed to perform UL-TDOA can be assumed to be trusted:

- One source observed that UL-TDOA cannot meet the target requirement for both earth fixed beam and earth moving beam. With 180s latency, positioning error performance that can be achieved is 34 km, CDF=90% and 13km, CDF=80%.

  - This source reported that the timing measurement error of SRS can be smaller than 26.7ns with 95% probability under 30 degree elevation angle for LEO-600 set-1, rural LOS S-band scenario.

- One source observed that the geometry of UE location relative to the satellite orbit will impact the positioning performance in UL-TDOA method e.g. for UE’s location at 200km away from the orbital plane, the NTN UE location verification accuracy requirement can be met and the positioning error of UL-TDOA method can be smaller than 10 km with 95% probability (for UE’s location at 200km away from the orbital plane) and a latency of 220 seconds in case of LEO600km and 342 seconds in case of LEO1200km. For UE located under the satellite orbit, NTN UE location verification accuracy requirement can be meet only with 30% probability.