3GPP TSG RAN WG1 #112	                               R1-2301274

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.5.5

Source:	Samsung

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In the newly agreed WID [1], the following objective is included regarding the support of positioning for RedCap UEs

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

This contribution discusses the aspects related to the specification support of positioning for RedCap UEs.

# Hopping pattern design consideration

Based on the instruction from WID, the target of the Redcap positioning is to design the frequency hopping operation. As we discussed in the previous meetings, for the redcap UE, it cannot jump over the different BW as operating in small sub-BW within a larger BWP and without switching time gap.

Following legacy FH principle, the different PRS FH part should be consecutive in time domain. This could be beneficial for combining the different PRS in different OFDM symbols assuming the same/similar channel response within a limited time. Nonetheless, for the feature of Redcap UEs, in which the FH part is operating based BWP, e.g., different FH part switching needs to consider the TimeGap for BWP switching. As shown in following figure, there could be two types of the FH indication.

Type 1: BWP based manner. The first one as shown case (a) in following figure, the PRS/SRS is configured as per BWP manner, such that the PRS/SRS might be confined with each BWP and the UE will follow the BWP operation rule to precede the PRS reception or SRS transmission.

Type 2: RB set based manner. This type can allow gNB configures the resource (especially the PRS resource) in a complete part (e.g., in one BWP basis), but the UE handles the PRS reception based on RB set with consideration of constraints of the BW switching. For example, the PRS could be configured outside BWP and refers to point A, and the reception could apply the time line requirement of BWP switching at UE side.

Case (a)                                              Case (b)

Fig.1. illustration of two frequency hopping manners

Proposal 1: both BWP based or RB set based FH type to design FH pattern can be considered.

The size of the time gap between hops may also have issue on the coherence time duration (see Annex), e.g., if the gap has surpassed the coherence time, the benefits to have FH could be reduced. Thus the candidate value of the time gap and the applicable condition should also be studied.

Proposal 2: a time gap configuration should be considered between hops, FFS the candidate value and applicable condition.

On the other hand, for the applicability of using FH based method to improve the Redcap positioning with smaller bandwidth, the impact of channel timing varying should be handled. For the simulated impact of UE speed, for the given simulation has been submitted, some claims the benefits of having overlapped REs to estimate and compensate the phase offset.

Fig.2 illustration of overlapped F-domain between hops

We think the benefits could be expected from the simulation in case the phase error/offset was added on and without considering not much about another time varying channel property, e.g., amplitude change, which is hard to compensate. Especially when the time gap between two hops is large. Each RE may experience different frequency selective fading, the method of using the overlap REs for phase offset estimation and compensation need more study to check the feasibility and effectiveness.

Proposal 3: FFS how to compensate the impacts on phase offset and/or amplitude change, from the time varying channel property.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: both BWP based or RB set based FH type to design FH pattern can be considered.

Proposal 2: a time gap configuration should be considered between hops, FFS the candidate value and applicable condition.

Proposal 3: FFS how to compensate the impacts on phase offset and/or amplitude change, from the time varying channel property.

# Reference

[1] RP-223549, New WID on Expanded and Improved NR Positioning, Intel

# Annex – coherence calculation

The coherence time and Doppler spread are inversely related, i.e.

Coherence Time ≈ 14×Doppler spread

Table 1 lists the values for the Doppler spread and associated channel coherence time for two frequency bands evaluated for RedCap UE positioning.

Table 1: Doppler Spreads and Coherence Times for RedCap UE positioning

| Carrier Frequency | UE speed | Max Doppler Spread | Channel Coherence Time |
| --- | --- | --- | --- |
| 3.5GHz | 3 km/h | 9.72Hz | 25.7ms |
| 3.5GHz | 30 km/h | 97.2Hz | 2.57ms |
| 3.5GHz | 60km/h | 194.4 Hz | 1.285ms |
| 28GHz | 3 km/h | 77.8 Hz | 3.21 ms |
| 28GHz | 30 km/h | 777.8 Hz | 0.321 ms |
| 28GHz | 60km/h | 1555.6 Hz | 0.161 ms |