3GPP TSG RAN WG1 #112			R1-2301347

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.4.1.2

Source:	Apple

Title:		Discussion on Sidelink Physical Channel Design Framework for Unlicensed Spectrum

Document for:	Discussion/Decision

# Introduction

The physical channel design framework for sidelink on FR1 unlicensed spectrum has been extensively discussed in the past several RAN1 meetings [1]-[5] .

In this contribution, we provide our views on the sidelink physical channel design framework on FR1 unlicensed spectrum.

# Discussion

## Resource pool

RAN1 #109-e Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

It was agreed that at least supporting one sidelink resource pool including integer number of RB sets. It is open whether to support one sidelink resource pool including subset of PRBs of one RB set.

If a resource pool includes a subset of PRBs of one RB set, then the PSCCH/PSSCH transmission may be via part of the RB set, which may violate the OCB requirement where the transmission needs to occupy at least 80% of the LBT channel bandwidth. Hence, it is preferred not to support that one sidelink resource pool is (pre)configured to include a subset of RB set.

Proposal 1: Do not support that one sidelink resource pool is (pre)configured to include a subset of RB set.

The time domain configuration of a resource pool was discussed in RAN1 #109-e meeting. It is open which slots belong to resource pool.

The time resources in the same carrier may be shared by multiple sidelink resource pools. For example, one resource pool occupies a part of slots in a DFN cycle (i.e., 10240 ms), while another resource pool occupies another part of slots in a DFN cycle. Hence, it is necessary to use a bitmap to indicate the time resources of a resource pool, in the same way as in NR sidelink on licensed spectrum.

Proposal 2: Use a bitmap to indicate the time domain resources of a resource pool.

### Intra-cell guard band

RAN1 #110b-e Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

It was agreed that PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets. It was further agreed that PRBs within intra-cell guard band of two adjacent RB sets can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels in multi-channel case and the UE uses both RB sets for PSSCH transmission. The details are open, including handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets.

In our view, the PRBs within intra-cell guard band can be allocated to the interlaces in one of the two adjacent RB sets, e.g., the RB set with lower frequency. These PRBs can be treated as an extension of the RB set when associating them with the corresponding interlaces. Figure 1 shows an example of the association of these PRBs with interlaces. There are 3 PRBs in the intra-cell guard band in the figure, which are allocated to RB set 1. There are 5 interlaces in RB set 1, where the last PRB (i.e., before intra-cell guard band) in RB set 1 belongs to interlace 1 (block box). Then the first PRB in the intra-cell guard band belongs to interlace 2 of RB set 1 (red box), the second PRB in the intra-cell guard band belongs to interlace 3 of RB set 1 (blue box), and the third PRB in the intra-cell guard band belongs to interlace 4 of RB set 1 (green box).

Figure 1: Illustration of intra-cell guard band

Proposal 3: If a resource pool contains two adjacent RB sets, all the PRBs within intra-cell guard band of two adjacent RB sets are associated with the interlaces in one of the two adjacent RB sets.

- These PRBs are treated as an extension of the allocated RB set when associating them with the corresponding interlaces.

When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs within intra-cell guard band. It is worth examining whether the rate matched bits for SCI stage 2 are also mapped to these PRBs within intra-cell guard band.

Proposal 4: When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs.

- FFS whether SCI stage 2 rate matched bits are mapped to these PRBs.

The association of PRBs within intra-cell guard band with interlaces causes certain interlaces having more PRBs than other interlaces. This leads to unequal sub-channel size. To avoid the inconsistent TBS determination, it is preferred that resource pool (pre)configures a unified number of PRBs per interlace in calculating sidelink TBS.

Proposal 5: To avoid inconsistent TBS determination, resource pool (pre)configures a single number of PRBs per interlace in calculating sidelink TBS.

It is open whether the PRBs within intra-cell guard band are used for PSFCH transmission. In our view, each PSFCH transmission is restricted to a single RB set, and hence, the PRBs within intra-cell guard band are not used for PSFCH transmission.

Proposal 6: The PRBs within intra-cell guard band are not used for PSFCH transmission.

## Slot structure for 2 candidate starting symbols

RAN1 #110b-e Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH RAN1 #111 Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 RAN1 #111 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured RAN1 #111 Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot RAN1 #111 Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

It was agreed that the slot based PSCCH/PSSCH transmission is supported. This implies the starting symbol of PSCCH/PSSCH transmission is the first symbol of a slot for sidelink transmission. It was working assumption to support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

The main motivation to support additional starting symbols is to allow a UE to occupy unlicensed channels in a timely manner once it has LBT success. A UE does not need to wait till the beginning of the next slot to use the channel, when LBT is successful in the middle of a first slot. This also reduces the chance of collision for PSCCH/PSSCH transmission.

If an additional starting symbol within a slot for the PSCCH/PSSCH transmission is supported, then a receiver UE needs to perform PSCCH decoding more than once in a slot. This dramatically increases UE implementation complexity. Considering the UE implementation complexity, we prefer to keep the flexibility via resource pool (pre)configuration. Specifically, resource pool (pre)configures the slots supporting 2 starting symbols and the slots not supporting 2 starting symbols. For example, the periodicity of slots supporting 2 starting symbols can be configured by resource pool. This is aimed to balance between channel access success probability and UE’s PSCCH blind decoding complexity.

Proposal 7: Resource pool (pre)configures the slots supporting 2 starting symbols and the slots not supporting 2 starting symbols.

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, it is open on the positions of the first and the second candidate starting symbols.

In our view, the second candidate starting symbol position should be always set as the middle of sidelink symbols in a slot, so that UE has the same amount of delay to access the channel no matter whether LBT success is in the first half of sidelink symbols or in the second half of sidelink symbols of a slot.

In the case where resource sharing (in a slot) between Uu link and sidelink is not considered, a whole slot is used for sidelink transmissions if the slot belongs to a resource pool. Here, the first candidate starting symbol position is fixed as symbol #0, and the second candidate starting symbol position is fixed as symbol #7 in a slot.

In the case where resource sharing (in a slot) between Uu link and sidelink is considered, the first candidate starting symbol position is indicated by sl-StartSymbol as in Rel-16 NR sidelink. The second candidate starting symbol position is at the middle of sidelink symbols in a slot. For example, if sl-StartSymbol is (pre)configured as symbol #2, then the second candidate starting symbol position is set as symbol #8. In this way, the second candidate starting symbol position is derived from sl-StartSymbol and there is no need to explicitly (pre)configure the location of the second candidate starting symbol.

Proposal 8: For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the second candidate starting symbol position is always set as the middle of sidelink symbols in a slot.

- In case the first candidate starting symbol position is fixed as symbol #0, then the second candidate starting symbol position is fixed as symbol #7.

It is working assumption that the candidate starting symbol(s) are intended for AGC purpose. It is open whether a second candidate starting symbol is for AGC purpose if PSCCH/PSSCH transmission starting from the first candidate starting symbol.

If a PSCCH/PSSCH transmission starts from the first candidate starting symbol, then the chance that another PSCCH/PSSCH transmission starts from the second candidate starting symbol in the same slot is low since the LBT for the latter PSCCH/PSSCH transmission will likely fail.

However, this situation may not be fully avoided, e.g., in hidden node case. Suppose UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol. It is possible that UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol in the same slot, in case UE1 is a hidden node to UE2 and UE2’s LBT is successful without detecting UE1’s transmission. UE2’s transmission will lead to AGC issue to other UEs’ reception if the second candidate starting symbol is not used for AGC purpose.

Besides hidden node case, the situation where UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol while UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol may also occur if a resource pool contains more than 1 RB set.

Suppose UE1 and UE2 make PSCCH/PSSCH transmissions on two different RB sets of a resource pool. UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol and UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol of the same slot. This is possible since UE2’s LBT operation is performed only on the second RB set and it does not detect UE1’s PSCCH/PSSCH transmission on the first RB set. Considering a receiver UE’s AGC operation is performed per resource pool or per sidelink BWP, UE2’s transmission will lead to AGC issue to other UEs’ reception if the second candidate starting symbol is not used for AGC purpose. This is illustrated in Figure 2.

Figure 2: Illustration of AGC issue over different RB sets

To avoid this AGC issue which occurs only if a resource pool has more than 1 RB set, we propose that for a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot in case a resource pool contains 1 RB set. Two AGC symbols are applied for PSCCH/PSSCH transmission in a slot in case a resource pool contains more than 1 RB set, where the second candidate starting symbol is used as the second AGC symbol. The second AGC symbol is the copy of its following symbol.

Proposal 9: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then

- a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains 1 RB set;

- two AGC symbols are applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains more than 1 RB set.

  - The second AGC symbol is the second candidate starting symbol, with contents being the copy of its following symbol.

It is possible that a PSCCH/PSSCH initial transmission starts from the first starting symbol position, while a PSCCH/PSSCH retransmission starts from the second starting symbol position. If the sidelink TBS determination relies on the actual starting symbol position of PSCCH/PSSCH, then the sidelink TBS calculation will be different at UE receiving the initial transmission and at UE receiving only the retransmission. Hence, it is preferred that sidelink TBS determination is always based on the number of OFDM symbols for PSCCH/PSSCH transmission starting from the first starting symbol.

Proposal 10: Sidelink TBS determination is always based on the number of OFDM symbols for PSCCH/PSSCH transmission starting from the first candidate starting symbol.

In Rel-16/17 NR sidelink, the PSFCH occasion is configured to be at least 2 or 3 slots after PSCCH/PSSCH transmission. If PSCCH/PSSCH transmission starts from the second candidate starting symbol position, then the processing time constraints need to be examined. In this case, PSCCH decoding can only start after the second candidate starting symbol position, which will delay the corresponding PSSCH decoding, as well as the subsequent preparation of PSFCH transmission. To ensure there are enough processing time, it is preferred that if a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

Proposal 11: If a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

## PSCCH and PSSCH

### Sub-channel in interlace RB-based resource pool

RAN1 #110 Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool RAN1 #110b-e Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets RAN1 #110b-e Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details RAN1 #111 Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs

It was agreed that for interlace RB-based PSCCH/PSSCH transmission, frequency domain resource allocation granularity is one sub-channel, where one sub-channel equals K interlaces. The details are open.

One interlace in an RB set of 20 MHz is generally composed of 10 or 11 PRBs. On the other hand, a sub-channel in Rel-16/17 NR sidelink may occupy a configurable number of PRBs, range from 10 PRBs to 100 PRBs. The configurable sub-channel size is beneficial in supporting different types of applications and can be re-used in SL-U. To avoid wasting of resources, it is preferrable that the total number of interlaces in a resource pool is divisible by the value K.

Proposal 12: For interlace RB-based transmission of PSCCH/PSSCH, a sub-channel equals K interlaces, where K is configurable by resource pool such that the total number of interlaces in a resource pool is divisible by K.

To indicate the sub-channel usage/reservation status, the sub-channels in a resource pool needs to be indexed. The mapping between interlace and sub-channel has been discussed extensively. Five options have been identified for further study.

To obtain the mapping between interlace and sub-channel, we first obtain the indices of all the interlaces within a resource pool. This system wide (or resource pool wide) interlace index is obtained based on interlace index within an RB set and RB set index. Here, we apply the interlace index within an RB set first, RB set index second rule. Suppose each RB set has A interlaces and a resource pool contains B RB sets. Then, the xth interlace in the yth RB set has the system wide interlace index as A*y+x, where 0≤x≤A-1 and 0≤y≤B-1. This is illustrated in Figure 3.

Consider a sub-channel is composed of K interlaces. Each sub-channel is composed of K consecutive system wide interlaces following a natural order. Specifically, the ith sub-channel is composed of system wide interlaces with indices A⋅BK*i, A⋅BK*i+1-1.

Consider an example that a resource pool contains 2 RB sets and each RB set has 5 interlaces. If a sub-channel is composed of 1 interlace, then there is a one-to-one mapping between sub-channel index to system wide interlace index. This is illustrated in Figure 3. If a sub-channel is composed of 2 interlaces, then there is a one-to-two mapping between sub-channel index to system wide interlace index. This is illustrated in Figure 4.

It is possible that a sub-channel spans 2 adjacent RB sets belonging to a resource pool. As illustrated in Figure 4, sub-channel #2 spans 2 RB sets belonging to a resource pool.

Proposal 13: For interlace RB-based transmission of PSCCH/PSSCH, a sub-channel is defined within 1 RB set or 2 adjacent RB sets and is incrementally indexed first over interlaces within an RB set, and then across different RB sets within the resource pool (i.e., Option 4).

It was agreed that frequency domain resource allocation granularity is one sub-channel for PSSCH transmission. In Rel-16/17 NR sidelink, PSCCH contains a field to indicate the frequency domain resources for a PSSCH transmission. Specifically, the frequency resource indicator vector (FRIV) value in SCI stage 1 indicates the starting sub-channel index of retransmissions and the number of sub-channels for a PSSCH transmission. We think the similar mechanism is applicable to SL-U.

In Rel-16/17 NR sidelink, a PSCCH/PSSCH transmission may occupy more than 1 sub-channel with contiguous sub-channel indices. This scheme facilitates the PSSCH transmissions, as well the SCI signaling of the reserved sub-channels. This can be applicable to SL-U. Specifically, a PSCCH/PSSCH transmission occupies one or more sub-channels with continuous indices. The FRIV field in SCI stage 1 is used to indicate the reserved sub-channels for the PSCCH/PSSCH transmission, in a similar way as in Rel-16/17 NR sidelink.

Proposal 14: A PSCCH/PSSCH transmission occupies one or more sub-channels with continuous indices. SCI stage 1 indicates FRIV in a similar way as in Rel-16/17 NR sidelink.

It is open when more than one RB set is used for PSCCH/PSSCH transmission, whether the used interlace indices in different RB sets are always the same or can be different. Since Rel-16/17 NR sidelink design of FRIV indication of sub-channels is reused, it is natural that the used interlace indices in different RB sets can be different.

Proposal 15: When more than one RB set is used for PSCCH/PSSCH transmission, the used interlace indices in different RB sets can be different.

Figure 3: Illustration of sub-channel indexing

Figure 4: Illustration of sub-channel indexing where a sub-channel spans over 2 consecutive RB sets

In Rel-16/17 NR sidelink, PSCCH is restricted in a single sub-channel even if PSSCH transmissions occupy more than one sub-channel. This design is to facilitate PSCCH decoding. It was agreed that PSCCH is transmitted within a single sub-channel for SL-U.

Furthermore, the number of PRBs for PSCCH is configurable per resource pool to support different types of applications. Note that a sub-channel may occupy K interlaces and hence, a PSCCH may occupy more than 10 PRBs, similar to Rel-16/17 NR sidelink.

Proposal 16: The number of PRBs for PSCCH is configurable per resource pool.

In SL-U, it is possible that each interlace may contain a different number of PRBs. For example, one interlace is composed of 10 PRBs and another interlace is composed of 11 PRBs. If initial transmission and retransmission use two interlaces with different numbers of PRBs, then the sidelink TBS calculation based on the actual number of PRBs of an interlace will lead to TBS misalignment between initial transmission and retransmission. To avoid this issue, a reference number of PRBs per interlace is used for the sidelink TBS determination. This reference number is (pre)configured by resource pool.

Proposal 17: In calculating sidelink TBS, a reference number of PRBs per interlace is used, where this number is (pre)configured by resource pool.

### Sub-channel in contiguous RB-based resource pool

RAN1 #111 Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband

It was agreed that both contiguous RB-based PSCCH/PSSCH transmissions and interlace RB-based PSCCH/PSSCH transmissions are supported. For contiguous RB-based PSCCH/PSSCH transmission, the mapping between sub-channel and PRBs were discussed, and three options were identified.

In Option 1, sub-channel aligns with resource pool boundary. It is possible that the intra-cell guard band PRBs can be at every location of a sub-channel. In general, PSCCH occupies the lowest PRBs of a sub-channel. If the lowest PRBs of a sub-channel are the intra-cell guard band PRBs, then these intra-cell guard band PRBs may be used for PSCCH transmission. This is against the agreement that intra-cell guard band PRBs are not used for PSCCH transmission. Hence, this option is not supported.

In Option 2, sub-channel aligns with RB set boundary, but a sub-channel is always refined within an RB set. This option may lead to significant resources waste if the sub-channel size is not properly configured.

In Option 3, sub-channel aligns with RB set boundary, but a sub-channel may span over guard band. This option is more resource efficient than Option 2. It also avoids the case where the lowest PRBs of a sub-channel is within intra-cell guard band. Hence, Option 3 is preferred. Furthermore, to avoid PSCCH transmissions in intra-cell guard band, we need to ensure that the total number of PRBs of any sub-channel in an RB set is not smaller than the (pre)configured number of PRBs for PSCCH.

Proposal 18: For contiguous RB-based PSCCH/PSSCH transmission, in each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guard band PRBs according to the sub-channel size, until the remaining PRBs in the RB set is less than the (pre)configured number of PRBs for PSCCH.

### Sub-channel for 60 kHz SCS

The sub-channel definition for 60 kHz SCS was discussed. It was proposed by FL [2] that for PSCCH/PSSCH/PSFCH/S-SSB, RAN1 to further study how to meet OCB and PSD requirement under 60 kHz SCS. An RB set is of 20 MHz, which contains only 24 PRBs under 60 kHz SCS. If an interlace is composed of 10 or 11 PRBs, like 15 kHz or 30 kHz SCS, then an RB set contains 2 interlaces. Subsequently, the bandwidth of unused PRBs in an RB set could be large under 60 kHz SCS.

To mitigate frequency resource waste, it is proposed that each interlace under 60 kHz SCS contains more PRBs than an interlace under 15 kHz or 30 kHz SCS.

Proposal 19: To meet OCB and PSD requirement under 60 kHz SCS, RB-based interlace is supported, where each interlace includes more PRBs than an interlace under 15 kHz or 30 kHz SCS.

### Multi-consecutive slot transmission

RAN1 #110 Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details RAN1 #110 Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

For PSCCH and PSSCH resource indication in time/frequency domain, it was agreed that Rel-16 NR sidelink TRIV is reused as baseline. It is open whether any enhancement is needed if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission. Additionally, it was agreed that multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U.

In Rel-16 NR sidelink, TRIV is determined by two time gap variables, t1 and t2, where t1 is the time gap between the initial transmission slot and the first retransmission slot, t2 is the time gap between the initial transmission slot and the second retransmission slot. Since the resource reservation window by SCI is 32 slots, the value range of t1 is 1≤t1≤31 and the value range of t2 is t1<t2≤31. With MCSt, if Rel-16 NR sidelink TRIV is reused, then the value ranges of t1 and t2 need to be redefined such that there is no time overlap between initial transmission and retransmissions.

Proposal 20: In supporting multi-consecutive slots transmission, the value ranges of t1 and t2 in deriving TRIV are redefined to avoid time overlap between initial transmission and retransmissions.

## PSFCH

### Time domain

RAN1 #110 Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded FFS details of above alternatives RAN1 #110b-e Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives RAN1 #111 Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity

In Rel-16/17 NR sidelink, PSFCH occasion is either 2 or 3 slots (by configuration) after the PSSCH transmission. In other words, the timing of PSFCH is associated with the timing of the corresponding PSSCH transmission. If the similar timing mapping rule is applied in SL-U, it is possible that PSFCH occasion may or may not be in the same COT as for PSSCH transmission. If PSFCH occasion is outside the COT duration for PSSCH transmission, then type 1 LBT is required before PSFCH transmission. It is likely that PSFCH transmission is dropped due to LBT failure. Even if PSFCH occasion is inside the COT duration for PSSCH transmission, it is possible that the COT is lost when there is no contiguous PSSCH transmissions. Then type 1 LBT is required before PSFCH transmission.

Since sidelink HARQ-ACK feedback is important for transmission reliability, it seems necessary to address the issue of PSFCH transmission dropping due to LBT failure. A straightforward scheme is to introduce multiple PSFCH occasions for a PSCCH/PSSCH. For example, we could introduce the PSFCH retransmission occasion(s) which are after the PSFCH initial transmission, where the PSFCH initial transmission is 2 or 3 slots after PSCCH/PSSCH transmission as in the legacy way. The time gap between PSFCH retransmission and PSFCH initial transmission can be (pre)configured per resource pool. Different frequency domain PSFCH resources (i.e., interlace) or code domain PSFCH resources (i.e., cyclic shift pairs) can be used for PSFCH retransmission. This is illustrated in Figure 5.

Figure 5: Illustration of PSFCH retransmission

Proposal 21: To address PSFCH transmission dropping due to LBT failure, introduce multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission (i.e., Alt. 1).

- PSFCH re-transmission time occasions are (pre)configured by resource pool.

It is open on the HARQ-ACK timeline for the case of supporting more than 1 PSFCH occasion per PSCCH/PSSCH transmission. In Rel-16/17 NR sidelink, when a UE selects the resources for a single TB transmission, it ensures a minimum time gap Z = a +b between any two selected resources of a TB where a HARQ feedback for the first resource is expected, where a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception and b is the time required for PSFCH reception and processing plus PSSCH retransmission preparation and is determined by UE implementation.

In the case of supporting multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission, when a UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected, where c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSSCH transmission. This is illustrated in Figure 6.

Figure 6 Illustration of HARQ-ACK timeline

Proposal 22: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, when a transmitter UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected,

- a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the first corresponding PSFCH reception,

- b is the time required for PSFCH reception and processing plus PSCCH/PSSCH retransmission preparation,

- c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSCCH/PSSCH transmission.

If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, then the receiver UE’s behavior of using multiple PSFCH transmission occasions needs to be specified. In our view, a receiver UE only needs to make a single PSFCH transmission among multiple PSFCH transmission occasions for a TB’s (re)transmission. If a receiver UE fails transmitting PSFCH on any precedent PSFCH occasion corresponding to the same TB due to LBT failure, then it attempts to make a PSFCH transmission.

Consider an example of groupcast with ACK/NACK feedback. If there are two PSFCH transmission occasions for PSCCH/PSSCH initial transmission and there are two PSFCH transmission occasions for PSCCH/PSSCH retransmission (i.e., same TB), then a receiver UE only needs to make a single PSFCH transmission. Specifically, it attempts to make a PSFCH transmission in the first PSFCH transmission occasion for PSCCH/PSSCH initial transmission. If it succeeds, then it stops sending PSFCH in the remaining 3 PSFCH transmission occasions. Otherwise, the receive UE attempts to make a PSFCH transmission in the second PSFCH transmission occasion for PSCCH/PSSCH initial transmission, and so on.

Proposal 23: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, a receiver UE attempts to make a PSFCH transmission if it has not transmitted on any precedent PSFCH occasion corresponding to the same TB.

### Frequency domain

RAN1 #111 Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. RAN1 #111 Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details

It was agreed that at least RB-based interlace PSFCH transmission is supported to meet OCB and PSD requirement. If each PSFCH transmission occupies one interlace (e.g., 10 PRBs), then the frequency domain resources of PSFCH will be limited. Note that in Rel-16/17 NR sidelink, each PSFCH transmission occupies only a single PRB.

The details about RB-based interlace PSFCH transmission are open, where three alternatives are identified to avoid too small PSFCH capacity. In the first alternative, each PSFCH transmission occupies a common interlace and K3 dedicated PRBs. In the second alternative, each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., FD-OCC or RB-level cyclic shift. In the third alternative, each PSFCH transmission occupies one dedicated PRB and some common PRBs.

In our view, the third alternative can meet the OCB requirements and can largely reuse the existing design of PSFCH transmission. In the third alternative, a resource pool has a bitmap to indicate the locations of a dedicated PRB set for PSFCH, in a similar way as in Rel-16/17 NR sidelink PSFCH resource configuration. The dedicated PRB set is used to deliver the actual sidelink ACK/NACK information. The frequency resource mapping from PSSCH to PSFCH PRB in the dedicated PRB set could reuse Rel-16/17 NR sidelink design.

Additionally, a common PRB set is configured as a subset of PRBs of multiple interlaces. The common PRB set is used to meet the OCB requirements and do not deliver actual ACK/NACK information. For example, a dedicated PRB set is configured in the middle of an RB set, and a common PRB set is configured to be a subset of PRBs of multiple interlaces near the edges of the RB set. This is illustrated in Figure 7.

For a PSSCH transmission on a slot in a sub-channel, the corresponding PRBs in a dedicated PRB set are determined for PSFCH transmission. Also, the mapping of PSSCH transmission to a PSFCH interlace is determined. A receiver UE transmits PSFCH on a single PRB with a cyclic shift pair in the dedicated PRB set and additionally transmits on the PRBs in the common PRB set corresponding to the PSFCH interlace.

For ACK/NACK based groupcast feedback, all receiver UEs share the same common PRBs and each receiver UE uses a dedicated PRB and a dedicated cyclic shift pair in the dedicated PRB set for the transmission of PSFCH.

Figure 7: Illustration of PSFCH frequency resources for groupcast with ACK/NACK feedback

Proposal 24: Each PSFCH transmission occupies a single PRB in a dedicated PRB set and a subset of PRBs of an interlace in a common PRB set.

- Dedicated PRB set is configured to deliver actual sidelink ACK/NACK information.

- Common PRB set is configured to meet the OCB requirement.

The PSFCH transmission under 60 kHz SCS was discussed and two alternatives are identified for further study. We think the similar PSFCH transmission as for 15 kHz or 30 kHz SCS could be applied to 60 kHz SCS as well. Specifically, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where the value K is (pre)configured per resource pool and two common PRBs are at the two edges of an RB set.

Proposal 25: For 60 kHz SCS, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where

- the value K is (pre)configured per resource pool,

- two common PRBs are at the two edges of an RB set.

## S-SSB

### Time domain

RAN1 #110b-e Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer RAN1 #110b-e Agreement Regarding additional candidate S-SSB occasions: - Their number and time domain locations are (pre-)configured or pre-defined RAN1 #111 Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number RAN1 #111 Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details RAN1 #111 Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets.

It was agreed that if LBT is performed for S-SSB, in addition to the S-SSB occasions in Rel-16/17 NR sidelink design, additional candidate S-SSB occasions are supported.

In our view, if a UE successfully transmits S-SSB on the initial S-SSB occasion, then the UE does not use the additional candidate S-SSB occasions. Even in this case, the additional candidate S-SSB occasions should not be reused for PSCCH/PSSCH transmission. Specifically, the slots containing additional candidate S-SSB occasions are excluded from sidelink resource pool. This semi-static configuration of resource pool largely simplifies UE’s operations on PSCCH/PSSCH transmission. Since otherwise, UE needs to dynamically determine which slots belong to sidelink resource pool, depending on whether initial S-SSB transmission is successful or not.

Proposal 26: If additional candidate S-SSB occasions are supported, then the slots containing additional candidate S-SSB occasions are excluded from resource pool.

It was agreed that the number and time domain locations of additional candidate S-SSB occasions are (pre)configured or pre-defined. Furthermore, there are five identified options on the number and locations of additional candidate S-SSB occasions.

In our view, the number of additional candidate S-SSB occasions is (pre)configured as part of the IE SL-SyncConfig to keep the flexibility. The locations of additional candidate S-SSB occasions are (pre)configured by a time offset to the initial S-SSB occasion. This is illustrated in Figure 8.

Figure 8: Illustration of S-SSB occasions

Proposal 27: Regarding the number and location(s) of additional candidate S-SSB occasions, each Rel-16/17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion (i.e., Option 2), where

- the value of K is (pre)configured,

- the gap between additional candidate S-SSB occasions and Rel-16/17 NR SL S-SSB slots is (pre)configured.

A UE attempts to transmit S-SSB on a Rel-16/17 NR S-SSB slot. If the UE successfully transmits S-SSB in a Rel-16/17 NR S-SSB slot, then it does not need to make any additional S-SSB transmissions until the next Rel-16/17 NR S-SSB slot. Otherwise, the UE attempts to transmit S-SSB on the subsequent additional candidate S-SSB occasion. If a UE successfully transmits S-SSB in an additional candidate S-SSB occasion, then it does not make any additional S-SSB transmissions until the next Rel-16/17 NR S-SSB slot. Otherwise, the UE attempts to transmit S-SSB on the subsequent additional candidate S-SSB occasion.

Proposal 28: A UE attempts to transmit on an additional candidate S-SSB occasion, if it has not transmitted on the precedent Rel-16/17 NR SL S-SSB slot and its other corresponding additional candidate S-SSB occasions.

If the additional candidate S-SSB occasions are not counted in resource pool configuration, then the slots for PSCCH/PSSCH transmissions may be largely reduced. For high spectrum efficiency, the S-SSB structure could be in 4 symbols (plus AGC symbol) such that more than 1 candidate S-SSB occasion lies in a single slot.

Proposal 29: Consider 4-symbol S-SSB structure (plus AGC symbol) in sidelink operations in unlicensed spectrum.

### Frequency domain

RAN1 #111 Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it

In Rel-16/17 NR sidelink, each S-SSB occupies 11 contiguous PRBs. This design does not meet OCB and PSD requirement for S-SSB transmission. Four options were agreed in RAN1 #109-e meeting [5] on the frequency allocation of S-SSB. It was further agreed [3] to down select between the first option and the third option. In the first option, the interlaced RB transmission is applied to S-SSB, in a similar way as for PSCCH/PSSCH transmission. In the third option, S-SSB is repeated in frequency domain such that it occupies enough bandwidth.

Besides these two options, it is open whether the temporary exemption of OCB requirement is applicable for S-SSB transmission. The following limits are copied from regulation [6]:

Figure 9: Limitation of occupied channel bandwidth [6]

During a COT, UE may operate temporarily with an OCB of less than 80% of its nominal channel bandwidth with a minimum of 2 MHz. The periodicity of S-SSB transmission is 160 ms, which implies an infrequent channel occupancy. If the temporary exemption of OCB requirement is applicable for S-SSB transmission, there is no need to modify the legacy S-SSB structure for the operations on unlicensed spectrum.

In RAN1 #111 meeting [1], the sub-options of the first option and the third option were identified and the option of applying OCB exemption to S-SSB was also listed.

In our view, the simplest way of designing S-SSB for SL-U is to apply OCB exemption to S-SSB (i.e., Option A). The legacy S-SSB occupies 11 PRBs in frequency, which is equal to 1.98 MHz bandwidth for sub-carrier spacing (SCS) of 15 kHz or 3.96 MHz bandwidth for sub-carrier spacing of 30 kHz.

It is open how to meet the minimum of 2 MHz requirement under 15 kHz SCS. One way to meet the temporary exemption of OCB requirement is to append an additional PRB to the legacy S-SSB structure, on the symbols of PSBCH. This is illustrated in Figure 10. The contents of the additional PRB can be identical to its neighbor PRB in the legacy PSBCH. In this way, the legacy S-SSB structure is largely reused and there is no impact on the reception of S-SSB.

Figure 10: Temporary exemption of OCB requirements for S-SSB transmission

Proposal 30: For sidelink operations on unlicensed spectrum to meet OCB and PSD requirement, apply temporary exemption of OCB requirement for S-SSB transmission (i.e., Option A).

- For 15 kHz SCS, an additional PRB is appended to legacy S-SSB structure on the symbols of PSBCH.

In case this temporary exemption of OCB requirement is not applicable for the whole S-SSB transmission, then we prefer Option 1-2. Specifically, the interlace RB-based transmission is only applied to PSBCH, while the S-PSS and S-SSS are remained unchanged. Note that this applies to either full-slot S-SSB transmission or 4-symbol SSB transmission, as respectively shown in the left side or in the right side of Figure 11.

Figure 11: Interlaced RB-based PSBCH transmission

Proposal 31: If the temporary exemption of OCB requirement is not applicable to the whole S-SSB transmission, then interlace RB-based transmission is applied to PSBCH only and apply OCB exemption to S-PSS and S-SSS (i.e., Option 1-2).

# Conclusion

In this contribution, we provided our views on physical channel design framework for sidelink on FR1 unlicensed spectrum. Our proposals are as follows:

Proposal 1: Do not support that one sidelink resource pool is (pre)configured to include a subset of RB set.

Proposal 2: Use a bitmap to indicate the time domain resources of a resource pool.

Proposal 3: If a resource pool contains two adjacent RB sets, all the PRBs within intra-cell guard band of two adjacent RB sets are associated with the interlaces in one of the two adjacent RB sets.

- These PRBs are treated as an extension of the allocated RB set when associating them with the corresponding interlaces.

Proposal 4: When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs.

- FFS whether SCI stage 2 rate matched bits are mapped to these PRBs.

Proposal 5: To avoid inconsistent TBS determination, resource pool (pre)configures a single number of PRBs per interlace in calculating sidelink TBS.

Proposal 6: The PRBs within intra-cell guard band are not used for PSFCH transmission.

Proposal 7: Resource pool (pre)configures the slots supporting 2 starting symbols and the slots not supporting 2 starting symbols.

Proposal 8: For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the second candidate starting symbol position is always set as the middle of sidelink symbols in a slot.

- In case the first candidate starting symbol position is fixed as symbol #0, then the second candidate starting symbol position is fixed as symbol #7.

Proposal 9: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then

- a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains 1 RB set;

- two AGC symbols are applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains more than 1 RB set.

  - The second AGC symbol is the second candidate starting symbol, with contents being the copy of its following symbol.

Proposal 10: Sidelink TBS determination is always based on the number of OFDM symbols for PSCCH/PSSCH transmission starting from the first candidate starting symbol.

Proposal 11: If a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

Proposal 12: For interlace RB-based transmission of PSCCH/PSSCH, a sub-channel equals K interlaces, where K is configurable by resource pool such that the total number of interlaces in a resource pool is divisible by K.

Proposal 13: For interlace RB-based transmission of PSCCH/PSSCH, a sub-channel is defined within 1 RB set or 2 adjacent RB sets and is incrementally indexed first over interlaces within an RB set, and then across different RB sets within the resource pool (i.e., Option 4).

Proposal 14: A PSCCH/PSSCH transmission occupies one or more sub-channels with continuous indices. SCI stage 1 indicates FRIV in a similar way as in Rel-16/17 NR sidelink.

Proposal 15: When more than one RB set is used for PSCCH/PSSCH transmission, the used interlace indices in different RB sets can be different.

Proposal 16: The number of PRBs for PSCCH is configurable per resource pool.

Proposal 17: In calculating sidelink TBS, a reference number of PRBs per interlace is used, where this number is (pre)configured by resource pool.

Proposal 18: For contiguous RB-based PSCCH/PSSCH transmission, in each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guard band PRBs according to the sub-channel size, until the remaining PRBs in the RB set is less than the (pre)configured number of PRBs for PSCCH.

Proposal 19: To meet OCB and PSD requirement under 60 kHz SCS, RB-based interlace is supported, where each interlace includes more PRBs than an interlace under 15 kHz or 30 kHz SCS.

Proposal 20: In supporting multi-consecutive slots transmission, the value ranges of t1 and t2 in deriving TRIV are redefined to avoid time overlap between initial transmission and retransmissions.

Proposal 21: To address PSFCH transmission dropping due to LBT failure, introduce multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission (i.e., Alt. 1).

- PSFCH re-transmission time occasions are (pre)configured by resource pool.

Proposal 22: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, when a transmitter UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected,

- a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the first corresponding PSFCH reception,

- b is the time required for PSFCH reception and processing plus PSCCH/PSSCH retransmission preparation,

- c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSCCH/PSSCH transmission.

Proposal 23: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, a receiver UE attempts to make a PSFCH transmission if it has not transmitted on any precedent PSFCH occasion corresponding to the same TB.

Proposal 24: Each PSFCH transmission occupies a single PRB in a dedicated PRB set and a subset of PRBs of an interlace in a common PRB set.

- Dedicated PRB set is configured to deliver actual sidelink ACK/NACK information.

- Common PRB set is configured to meet the OCB requirement.

Proposal 25: For 60 kHz SCS, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where

- the value K is (pre)configured per resource pool,

- two common PRBs are at the two edges of an RB set.

Proposal 26: If additional candidate S-SSB occasions are supported, then the slots containing additional candidate S-SSB occasions are excluded from resource pool.

Proposal 27: Regarding the number and location(s) of additional candidate S-SSB occasions, each Rel-16/17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion (i.e., Option 2), where

- the value of K is (pre)configured,

- the gap between additional candidate S-SSB occasions and Rel-16/17 NR SL S-SSB slots is (pre)configured.

Proposal 28: A UE attempts to transmit on an additional candidate S-SSB occasion, if it has not transmitted on the precedent Rel-16/17 NR SL S-SSB slot and its other corresponding additional candidate S-SSB occasions.

Proposal 29: Consider 4-symbol S-SSB structure (plus AGC symbol) in sidelink operations in unlicensed spectrum.

Proposal 30: For sidelink operations on unlicensed spectrum to meet OCB and PSD requirement, apply temporary exemption of OCB requirement for S-SSB transmission (i.e., Option A).

- For 15 kHz SCS, an additional PRB is appended to legacy S-SSB structure on the symbols of PSBCH.

Proposal 31: If the temporary exemption of OCB requirement is not applicable to the whole S-SSB transmission, then interlace RB-based transmission is applied to PSBCH only and apply OCB exemption to S-PSS and S-SSS (i.e., Option 1-2).

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 #111 Meeting, Nov. 2022.

- R1-2212649, “FL summary#5 for AI 9.4.1.2 SL-U physical channel design framework,” Nov. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #110bis-e Meeting, Oct. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #110 Meeting, Aug. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #109-e Meeting, May 2022.

- ETSI EN 301 893, 5 GHz RLAN; Harmonized standard covering the essential requirements of article 3.2 of directive 2014/53/EU, v2.1.1, May 2017.