3GPP TSG RAN WG1 #112			R1-2301374

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.14.1

Source:	Apple Inc.

Title:	Discussion on PRACH coverage enhancement

Document for:	Discussion/Decision

# Introduction

A new Rel-18 WI on further coverage enhancement [1] was approved in RAN#94e. one of the objectivce of this WI is to improve PRACH coverage.

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

In RAN1#111 meeting, the following agreements were made on PRACH coverage enhancements [2].

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

- Study at least the following case for multiple PRACH transmissions with different Tx beams.

  - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

  - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

  - Note: not related to decision on CFRA

- Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

- Working Assumption

- Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

    - Focus on FR2.

      - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

    - Performance metric: 0.1% false alarm, 1% miss-detection

    - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

  - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

  - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In this contribution, we discuss the aspects of PRACH coverage enhancement and give our proposals.

# Discussion on PRACH repetition

According to study outcome from Rel-17 coverage enhancement SI in [3], it was identified the PRACH format B4 is the UL coverage bottleneck channel for both FR1 and FR2. One of techniques to improve the PRACH coverage is multiple PRACH transmissions with the same beam.

For the short PRACH format, each PRACH channel will occupy several OFDM symbols, i.e., 2, 4, 6, 12 symbols. However, for long PRACH format, i.e., format 0/1/2/3, the duration is 1ms/3ms/3.5ms/1ms respectively. Thus, the time domain RO numbers in a PRACH configuration period are different for each PRACH format. If multiple PRACH transmissions are associated with the same beam, the PRACH transmission delays are different. According to current specification, a number of PRACH configuration indexes are associated with a PRACH format for FDD and TDD. The time domain RO numbers are quite different for different PRACH configuration index. To support PRACH repetition and reduce the transmission delay, the network should configure the PRACH configuration index with more time domain ROs. There is no need to introduce new PRACH configuration with more time domain ROs for repetition.

Proposal 1: No new PRACH configurations are to be introduced for each PRACH format.

For the PRACH repetition number, as indicated in [3], 4 PRACH repetitions could provide 3.7-5.2 dB coverage gain. In addition, considering the PRACH coverage enhancement could apply to NTN as well, it was observed the coverage gap could be 1.2 to 11.9dB for PRACH format B4 for LEO-1200 from NTN study. Thus, the maximum 8 PRACH repetitions is suitable number and could fulfil the NTN coverage enhancement requirement. The PRACH repetition number can be indicated by SIB1, the value range is {1, 2, 4, 8}.

Observation [4] For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS, - One source observed that the existing specification can meet the performance requirement - Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS, - Ten sources observed that the existing specification can meet the performance requirement - Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS, - Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Proposal 2: SIB1 indicates a PRACH repetition number from the candidate value of {1, 2, 4, 8}.

Regarding how to differentiate the multiple PRACH transmission with single PRACH transmission, three options were discussed in last RAN1 meeting. Since Rel-15 and Rel-16, the PRACH was used to differentiate different features, such as the CBRA vs. CFRA, 4-step RACH vs. 2-step RACH. In Rel-17, PRACH was still used to separate feature of SDT, Msg3 PUSCH repetition, RedCap, and slicing, thus RAN2 defined the RACH Indication and Partitioning framework. With this framework, each feature can configure the RACH resource independently with AddtionalRACH-ConfigList-r17, it’s up to gNB implementation to configure the shared RO or separated RO for each feature. In addition, the framework is forward compatible, the spare bits are reserved in FeatureCombination-r17. Therefore, the Rel-18 PRACH repetition could re-use this framework directly with minor additional standard work. From this perspective, the Option 1 and Option 2 are supported already by RACH partition framework. For option 3, according to our understanding, the share ROs are from the PRACH configuration. The separate ROs are new ROs and configured from legacy PUSCH resources, the separate RO is following the shared RO in time domain to reduce the PRACH repetition transmission delay. More standard work would be required for this option, such as defining new RO, collision handling between new RO and PUSCH.

Proposal 3: According RAN2 RACH Indication and Partitioning framework, Option 1 and Option 2 can be supported to differentiate the PRACH repetition with single PRACH transmission.

With the introduction of PRACH repetition, the starting subframe or slot for the first PRACH transmission should be specified to make the base station and UE having the same understanding on the repetitions of PRACH. LTE eMTC PRACH repetition was specified in Rel-13, similar design could be applied to NR PRACH repetition. For eMTC PRACH repetition, the starting subframe periodicity is defined, which is considering the prach-ConfigurationIndex and PRACH repetition number. The starting subframe offset could be derived by UE for PRACH repetition.

For NR, there are multiple downlink beams, thus SSB-to-RO association was specified within association period and association pattern period. The starting RO for different SSB maybe not at the same time instance according to ssb-perRACH-OccasionAndCB-PreamblesPerSSB configuration, and the SSB-to-RO mapping cycles could be different in different association period due to RO validation rule. But anyway, the SSB-to-RO mapping will repeat the same association from one association pattern period to another one. So, if multiple PRACH transmissions can be finished within an association pattern period, then the first PRACH transmission will always start at the beginning of association pattern period. This makes gNB and UE implementation easier to identify the first PRACH transmission. In this way, the first PRACH transmission or starting RO offset can be derived from association pattern period, the offset is relative to SFN=0 and could be zero or multiple of association pattern period. To save the overhead for PRACH repetition, the starting RO periodicity can be configured with multiple association pattern periods. An example of PRACH repetition configuration is showing in Figure 1.

Figure1: PRACH repetition configuration

Proposal 4: Multiple PRACH transmissions with same Tx beam are complete within an association pattern period.

Proposal 5: eMTC defined PRACH repetition mechanism can be the starting point for NR multiple PRACH transmissions with same beam, i.e., defining the periodicity of starting RO and the starting RO offset.

If the PRACH repetition is supported, then PRACH frequency hopping can be supported as well to get the frequency domain diversity gain, and the required PRACH repetition number can be reduced in some extend. The PRACH frequency can be cell specific enabled/disabled. The frequency hopping interval can be subframe level or radio frame level, which can be configured according to the PRACH format. The frequency offset can be configured in the initial BWP as well.

Proposal 6: RAN1 to consider supporting PRACH frequency hopping.

According to last meeting agreements, one remaining issue is whether allow gNB configure multiple PRACH repetition numbers, i.e., for Option 1, gNB can only configure one repetition number; for Option 2, gNB can configure one or multiple repetition numbers. As discussed above, the maximum PRACH repetition number could be up to 8. If only one repetition number is configured by considering the UE with the worst coverage, but the RSRP threshold for PRACH repetition is configured according to cell planning, i.e., considering the UE in the cell edge. This mismatch between maximum repetition number and RSRP threshold will cause all the coverage extension UEs transmission with the maximum repetition. It’s unnecessary increasing the UE power consumption. If multiple PRACH coverage enhancement levels are supported, the UE could select the right PRACH repetition numbers according to configured RSRP measurement threshold for each repetition level. In addition, the Option 2 provides the flexibility to network, it’s up to gNB to configured single PRACH repetition level or multiple PRACH repetition levels.

Proposal 7: Multiple PRACH coverage enhancement levels, i.e., Option 2, are supported. SSB-RSRP threshold is configured for each PRACH repetition level.

Regarding the RAR monitoring window, two options were discussed in last RAN1 meeting. The benefit of Option 1 is the PRACH transmission could be early terminated if PDCCH is correctly detected with the specific RA-RNTIs. The drawback is UE would monitor each RAR window associated with the transmitted PRACH, UE would monitor several RAR windows at the same time instance. For Option 2, UE only start to monitor RAR after the last PRACH transmission. From UE implementation perspective, Option 2 is simpler. As for the latency concern, gNB can configure the PRACH configuration with more time domain ROs to reduce the latency.

In addition, if only one RAR window is monitored by the UE and the ra-ResponseWindow is not extended from 10ms, the ra-ResponseWindow should start at the first PDCCH occasion from the end of the last random access preamble transmission in the multiple PRACH transmissions. The RA-RNTI is determined according to the last preamble transmission of the multiple PRACH transmission.

Proposal 8: UE only monitors one RAR window for all of the multiple PRACH transmission in one RACH attempt.

Proposal 9: RA-RNTI is determined according to last preamble transmission of the multiple PRACH transmissions.

Msg3 PUSCH repetition was specified in Rel-17 coverage enhancement WI, and PUCCH repetition for Msg4 HARQ-ACK is to be supported in Rel-18 for NTN. During the random access procedure, three UL channels, i.e., PRACH, Msg3 and PUCCH for Msg4 HARQ-ACK, could have the coverage issue. Whether the UL transmission repetition is applied depends on the configured SSB RSRP threshold. As SSB RSRP threshold had already defined for Msg3 PUSCH repetition, i.e., rsrp-ThresholdMsg3-r17, this threshold can be re-used by PRACH and PUCCH repetition. In this way, the UL coverage of three channels can be considered jointly.

Proposal 10: Msg3 RSRP threshold is re-used to determine the PRACH repetition and PUCCH repetition for Msg4 HARQ-ACK.

In last RAN1 meeting, RO group/RO bundle was extensively discussed. According to our understanding, a RO group includes consecutive TDMed ROs for an associated SSB to reduce the random access delay. RO is replaced with RO group to perform the association with SSB. Otherwise, there is no strong motivation to introduce a new term, as SSB-to-RO association period and association pattern period were clearly defined. The PRACH repetition number and repetition alignment could be easier derived by gNB and UE.

Proposal 11: RO group is defined as consecutive TDMed ROs for an associated SSB.

# Summary

In this contribution, we discuss the aspects on PRACH coverage enhancement and have the following proposals:

Proposal 1: No new PRACH configurations are to be introduced for each PRACH format.

Proposal 2: SIB1 indicates a PRACH repetition number from the candidate value of {1, 2, 4, 8}.

Proposal 3: According RAN2 RACH Indication and Partitioning framework, Option 1 and Option 2 can be supported to differentiate the PRACH repetition with single PRACH transmission.

Proposal 4: Multiple PRACH transmissions with same Tx beam are complete within an association pattern period.

Proposal 5: eMTC defined PRACH repetition mechanism can be the starting point for NR multiple PRACH transmissions with same beam, i.e., defining the periodicity of starting RO and the starting RO offset.

Proposal 6: RAN1 to consider supporting PRACH frequency hopping.

Proposal 7: Multiple PRACH coverage enhancement levels, i.e., Option 2, are supported. SSB-RSRP threshold is configured for each PRACH repetition level.

Proposal 8: UE only monitors one RAR window for all of the multiple PRACH transmission in one RACH attempt.

Proposal 9: RA-RNTI is determined according to last preamble transmission of the multiple PRACH transmissions.

Proposal 10: Msg3 RSRP threshold is re-used to determine the PRACH repetition and PUCCH repetition for Msg4 HARQ-ACK.

Proposal 11: RO group is defined as consecutive TDMed ROs for an associated SSB.

# References

- RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom

- R1-2300001, “Report of RAN1#111 meeting”, ETSI MCC

- TR38.830, “Study on NR coverage enhancements”

- R1-2208321, “Report of RAN1#110 meeting”, MCC