3GPP TSG RAN WG1 #112			R1-2301433

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.11.2

Source: 	Qualcomm Incorporated

Title: 	Network verified UE location for NR NTN

Document for:	Discussion/Decision

# Introduction

In RAN#98e, the objective of Rel-17 work item NR_NTN_enh for network verified UE location was agreed as the following [1]:

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied.

Note 1: Enhancements assume reuse of the RAT dependent positioning framework

Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance

Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity)

Note 4 : Multiple satellite in view by the UE may be considered if time allows

Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved

Note 6 : The enhancements should take into account the mirror-image ambiguity

Note 7 : Network verified UE location is an optional UE feature

In this contribution, we discuss enhancements to support single-sat RTT, the feasibility of single-sat DTOA, and the advantages of using multiple satellites for network verification of UE location in NTN. In addition, we discuss measurements in NTN and present simulation results for both single and multiple satellite cases.

# Single-Sat Multi-RTT

RTT can be measured by UE reporting the RX-TX time difference between a DL reference signal and an UL reference signal as shown in Figure 1. In Figure 1(a), UE sends an SRS after receiving the DL RS from the network and reports the time difference between the DL RS and the SRS. In such case, an RTT measurement, RTT_m is calculated as  T2-T1.    In Figure 1 (b), the SRS is sent before a DL RS and the time difference between the SRS and the DL RS is reported. In such case, RTT_m is calculated as T1-T2.

Figure 1. RTT measurement based on RX-TX time differences at UE and gNB.

In the above discussion, the RTT_m is obtained as the sum of two propagation delays at different times, i.e., t0 and t1. For verification of UE location, the so-obtained RTT_m can be an approximate of the RTT at time (t0+t1)/2, i.e.,

RTTt0+t12≈RTTm=Propagation Delay t0+Propagation Delay t1,  when t1-t0   is small

The error, |RTT(|RTTt0+t12-RTTm|, by considering both satellite movement and earth rotation is shown to be less than 3 ns when the t1-t0 is less than 200 ms [2].

In terrestrial network where slot durations at UE is constant, signaling of the above time gaps (i.e., T1 and T2) to LMF can be supported by using UE RX-TX time difference and gNB RX-TX time difference and appropriate time stamps.

In the existing specification, the UE RX-TX time difference, TUE_RX- TX , is defined as [3]:

For UE RX-TX time difference in NTN, we have the following remarks:

- In TN, the UE received timing, TUE-RX, of a downlink subframe #j, can be determined by using multiple DL RS such as PRS and CSI-RS. This is not desirable in NTN as the observed subframe duration can be time varying at UE due to satellite movement. Hence, when measuring a UE RX-TX time difference, the arrival time should be determined using the involved DL RS only without considering the arrival times of other DL RS.

- Although UE TA in NTN is intended to compensate RTT, it does not meet the accuracy requirement for location verification in NTN.  In NR, the downlink synchronization and UL transmit timing errors allowed can be a few hundred ns. For instance, the UL transmit timing error for 15 kHz SCS can be 29*64*Tc, or about 944 ns. Such a large error does not allow location verification with 10 km accuracy in NTN as will be clear in the subsequent simulation results. Hence, both DL RS and UL SRS are needed in NTN for accurate RTT measurement.

Proposal 1: For RTT measurement in NTN, support UE report of UE RX-TX time difference that indicates the time difference between the arrival time of a DL RS and the transmit time of an SRS.

- FFS: details of report.

In the existing specification, the gNB RX-TX time difference, TgNB_RX- TX , is defined as [3]:

Similarly, only one SRS resource should be used to determine the arrival time of SRS.

Proposal 2: For RTT measurement in NTN, support gNB report of gNB RX-TX time as defined in 38.215 with the following change:

- Only the SRS resource starting within a subframe can be used to determine the start of the subframe.

To resolve the mirror-point ambiguity issue, other information can be useful. For instance, if the footprint of the connected beam/cell is on one side of the satellite orbit, the mirror point that lies at the other side of the orbit can be ruled out. In addition, RSRP measurements of neighboring beams/cells can be used to determine which of the two mirror points are more likely.  To make use of the RSRP measurements of neighboring cells/beams, their footprints or antenna patterns are also needed.

Proposal 3: For network verification of UE location, support the report of the following from RAN to LMF:

- Footprints of the connected beam/cell and of the neighboring cells/beams if their RSRP measurements are reported.

# Single-Sat TDOA

DL-TDOA with a single satellite has already been studied. It has shown that DL-TDOA has a worse geometry as compared to RTT for UE location verification:

- Single-sat DL-TDOA is more sensitive to measurement error.

- Single-sat DL-TDOA requires much longer measurement time.

In addition, the feasibility of single-sat DL-TDOA is questionable due to the following two issues

- UE clock drift

- Dependency of UE time measurement on UE assumed GNSS

If UE measures the time with a clock that runs independently with the DL signal, clock drift needs to be considered. Assuming a 0.01 ppm drifting rate, the measurement error could be up to 1 ms for a time duration of 10 s.  Such a large measurement error will lead to UE location error beyond 10 km range. In Figure 3, the achievable location accuracy based on DL-TDOA with a single satellite is shown. As can be seen from the figure, even with a 0.01 ppm clock drift and without any other measurement errors, the location error can be more than 10 km.

Figure 3. Location contour using DL-TDOA measurements.

If the clock used for time measurement is synchronized to DL signal. The resultant time measurement depends on the GNSS location. Let’s assume that the carrier frequency be fc and, for simplicity, the Doppler in a duration of two consecutive DL PRS transmissions be constant. If the clock is synchronized to the DL signal, UE will see exactly fc DL signal ticks during the duration of two PRS arrivals that are transmitted exactly 1 second apart. Consequently, the UE measure the time gap of two PRS arrivals as

T=fcfc(1+D)

where D is the Doppler that UE assumes. Since the UE derives the Doppler based on GNSS, the above says that the DL time difference of arrival measurement will be consistent with the GNSS location that UE assumes. Using the above measurements of TDOA, one will conclude that the UE location is the same as the GNSS location that UE assumes, correct or not.

Observation 1: If a clock that is synchronized with the DL signal is used to measure TDOA of DL signals from a LEO satellite, the TDOA measurements will be consistent with whatever GNSS location that UE assumes.

Clearly, if a UE uses DL synchronized clock to measure the time gap, DL-TDOA cannot be used to verify the GNSS location that a UE assumes. Note that the above is not a big issue in single-sat multi-RTT because the RTT is not solely determined by UE RX-TX time difference measurement although an independent clock is still preferred in the measurement.

In summary, the feasibility of DL-TDOA needs further study, particularly with respect to the above two issues.

# Multi-sat cases

With a LEO constellation with global coverage, a UE will certainly be able to receive from multiple satellites due to the imbalance between UL and DL coverage [2]. Hence, it is desirable to consider UE location verification with multiple satellites to take the advantage of better geometry in triangulation.

Proposal 4: Support network verification of UE location using multiple satellites with the following assumptions:

- UE is connected to one satellite

- UE can detect DL RS from multiple satellites

- The involved satellites are synchronized at uplink synchronization reference points

# Simulation results

## Performance of PRS and SRS measurements

We evaluate the timing measurement performance for DL-PRS and UL-SRS in the NTN case. The UE antenna gain is assumed to be -5.5 dB as updated in RAN#97e. Simulation assumptions are listed in Table 1 below.

Table 1. Simulation assumptions

| Parameter | Description/Value |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.811 LEO-600, S band, Set-1, Rural LOS |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 15 kHz |
| Reference Signal Transmission Bandwidth | DL: 10 MHz UL: 10 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-1 UL: Comb -12 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of symbols used per occasion | DL: 1 UL: 12 |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Delay between symbol boundary and first path | DL,UL: Uniform in [0,1000]ns |

For cases with elevation angle >300, the CDFs of timing measurement error for PRS and SRS are given in Figures 4 and 5, respectively; and the 90-percentile errors as a function of oversampling rate are summarized in Table 2. As can be seen from the figures, with NTN rural LOS channels the timing measurement performance improves as the  oversampling rate increases.

Figure 4. Timing measurement Error for PRS

Figure 5. Timing Measurement Error for SRS

Table 2. Timing measurement error of 90 and 95 percentile.

| Measurement error (ns) | PRS |  | SRS |  |
| --- | --- | --- | --- | --- |
| Oversampling | 90% | 95% | 90% | 95% |
| 1 | 47.9 | 50.8 | 48.6 | 52.2 |
| 2 | 24.2 | 25.9 | 25.5 | 28.2 |
| 4 | 12.5 | 13.6 | 14.5 | 17.1 |
| 8 | 6.9 | 8.0 | 9.8 | 12.6 |
| 16 | 4.6 | 5.8 | 8.4 | 11.2 |

Note that in the NTN Rural LOS case, the resulting channel is LOS dominant and results in good timing performance with a large oversampling rate.

We also compare the measurement error of PRS arrival time with different elevation angles in Figure 6. Despite the decreasing SNR as elevation angle decreases, the timing errors do not deteriorate significantly due to larger K factors at lower elevation angles.

Figure 6. PRS Timing Measurement vs Elevation angle at UE.

## Performance with single-satellite multi-RTT

Below we consider the error in determining the horizontal 2-D UE location assuming UE altitude is known to the network. For simplicity we assume that UE altitude is 0 and the Earth is a perfect sphere. The error in RTT for each RTT measurement is the sum of PRS timing error, SRS timing error and other errors. PRS and SRS timing errors are sampled from the previous distributions. Other errors are modelled as a uniform error in the range [-5,5]ns.

The simulation assumptions are given in the Table below.

Table 3. Simulation assumptions for single-satellite RTT.

| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | RTT – Single Sat |
| --- | --- |
| Number of measurement occasions | 3 |
| Time of measurement occasions | -x,0,x -> x=[1,2,4,8] |
| Satellite location at t = 0 | 0N 0E |
| Satellite orbit | 90 degree inclination |
| Earth rotation | Ignored(Negligible compared to satellite motion) |
| UE altitude | 0 m (Surface of earth) and known |

We generate a grid over the coverage area (elevation angle>30 deg at t=0) and drop UEs at each grid point. For each UE, we perform RTT measurements at time [-x,0,x]s with error sampled as described above with x chosen from [1,2,4,8]. Then using these measurements, we estimate the UE location under the assumption that UE is on the surface of the Earth. Below we plot the 90 percentile position error at each grid point.

Note that the two points that are symmetric with respect to the orbit plane will have the same RTT values and cannot be distinguished using timing. Other information can be used to resolve this ambiguity. Here we ignore this ambiguity and assume we know if the UE is to the left or right of the orbit.

Assuming the altitude of the UE is known within a [-50, 50]m range, the CDF of horizontal errors are shown in Figure 7 and summarized in Table 4 for different measurement time gaps. As can be seen, even in the case of 1s gap in measurements, we are able to resolve the UE position within 2km for UE locations. As shown in [2], the accuracy also depends on the UE location with respect to the satellite orbit: the closer is the UE to the orbital plan, the worse is the accuracy of location.  In practice, one can choose a shorter measurement time span for UEs in farther beams and a longer measurement time for UEs in beams closer to the orbit.

Figure 7. CDF of Horizontal position error with 3 RTT measurements and altitude uncertainty uniformly distributed in [-50 50] m.

Table 4 Horizontal error with 3 RTT measurements and altitude uncertainty uniformly distributed in [-50 50] m.

| Error in m | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| [-1,0,1] | 155 | 239 | 370 | 715 | 1928 |
| [-2,0,2] | 90 | 133 | 206 | 408 | 1107 |
| [-4,0,4] | 62 | 86 | 132 | 275 | 757 |
| [-8,0,8] | 51 | 68 | 104 | 226 | 611 |

## Performance in multi-satellite case

For multi-satellite case, we consider a 3-satellite case where UE reports the UE RX-TX time difference to the serving cell so that the RTT of the serving satellite can be measured and also reports the TDOAs of two neighbor satellites with respect to the serving satellite. The simulation assumptions are given in Table 5.

Table 5. Simulation assumptions for 3-satellite case

| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | RTT with Serving Sat + TDOA of neighbor satellite wrt Serving Sat Single Instant |
| --- | --- |
| Max error | RTT: 200ns, TDOA:100ns |
| Serving Satellite location | 0N 0E |
| Neighbor Satellite 1 | 0N 8E |
| Neighbor Satellite 2 | 6N 4E |
| UE altitude | 0m(Surface of earth) and known |

With the above assumptions, the maximal error along the NS direction and along the EW direction are given in the figures below.

Figure 8. Maximal location error along NS direction as a function of UE location with 3 satellites.

Figure 9. Maximal location error along EW direction as a function of UE location with 3 satellites.

As can be seen from the above figures, with 3 satellites a UE location can be determined with about 100 m uncertainty along latitude or longitude.

# Conclusions

In this contribution, we have discussed enhancements to support single-satellite multi-RTT based network verification of UE location, the feasibility issues regarding single-satellite DL-TDOA method, and the advantages of using multiple satellites. Observations and proposals are summarized below.

Observation 1: If a clock that is synchronized with the DL signal is used to measure TDOA of DL signals from a LEO satellite, the TDOA measurements will be consistent with whatever GNSS location that UE assumes.

Proposal 1: For RTT measurement in NTN, support UE report of UE RX-TX time difference that indicates the time difference between the arrival time of a DL RS and the transmit time of an SRS.

- FFS: details of report.

Proposal 2: For RTT measurement in NTN, support gNB report of gNB RX-TX time as defined in 38.215 with the following change:

- Only the SRS resource starting within a subframe can be used to determine the start of the subframe.

Proposal 3: For network verification of UE location, support the report of the following from RAN to LMF:

- Footprints of the connected beam/cell and of the neighboring cells/beams if their RSRP measurements are reported.

Proposal 4: Support network verification of UE location using multiple satellites with the following assumptions:

- UE is connected to one satellite

- UE can detect DL RS from multiple satellites

- The involved satellites are synchronized at uplink synchronization reference points

# References

- RP-223534, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, 3GPP RAN#98-e.

- RP-2212137, “Network verified UE location for NR NTN”, Qualcomm, 3GPP RAN1#111.

- 3GPP TS 38.215, “Physical layer measurements,” v 17.2.0.