3GPP TSG RAN WG1 #112	R1-2301469

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on multi-slot PDSCH/PUSCH repetition in unified TCI

Agenda Item:	8.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss UE behaviour of Rel.17 unified TCI, if indicated joint/DL/UL TCI state is updated at middle of multi slot PDSCH/PUSCH repetition.

- Discussion

In AI 8.2.5 in RAN1#109e, it was pointed out that the existing text of PDSCH repetition for indicated TCI state is not applied to Rel.17 unified TCI state, because the previous sentence is only applied to Rel.15 TCI state [1].

5.1.5 Antenna ports quasi co-location […] The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s).

In AI 8.2.5 in RAN1#111, the following conclusion was made based on the common understanding that the above text is only applicable to Rel.15/16 TCI framework. Hence, the above text is not applied to Rel.17 unified TCI framework.

Conclusion For Rel-17 unified TCI framework, the applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH/PUSCH. - No additional specification support is needed.

For multi slot PDSCH/PUSCH repetition in Rel.17 unified TCI framework, based on the current specification, the following beam application timing (BAT) is strictly applied, because there is no exception for multi-slot PDSCH/PUSCH repetition.

5.1.5 Antenna ports quasi co-location […] When a UE configured with dl-OrJoint-TCIStateList would transmit a PUCCH with HARQ-ACK information or a PUSCH with HARQ-ACK information corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated TCI-State and/or TCI-UL-State should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State or TCI-UL-State that are active at the end of the PUCCH or the PUSCH carrying the HARQ-ACK information.

In order to avoid potential beam miss-understanding for multi-slot PDSCH/PUSCH repetition between UEs and gNB, we propose to conclude as the following.

Proposal for conclusion:

  - For multi-slot PDSCH/PUSCH repetition, if DLorJointTCIState or UL-TCI-State is configured, the indicated joint/DL/UL TCI state on each slot is applied to the scheduled PDSCH/PUSCH on each slot.

Conclusion

In this contribution, we discussed UE behaviour of Rel.17 unified TCI, if indicated joint/DL/UL TCI state is updated at middle of multi slot PDSCH/PUSCH repetition, and we made the following proposal.

Proposal for conclusion:

  - For multi-slot PDSCH/PUSCH repetition, if DLorJointTCIState or UL-TCI-State is configured, the indicated joint/DL/UL TCI state on each slot is applied to the scheduled PDSCH/PUSCH on each slot.

- References

- R1-2204114, “Remaining issue for beam management”, RAN#109e, Ericsson, May. 2022.