3GPP TSG RAN WG1 #112                                                       　  R1-2301516

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation methodology for low power WUS

Agenda Item:	9.13.1

Document for: 	Discussion and decision

- Introduction

In RAN1#111, RAN1 made the agreement on the methodology and assumption for further LP-WUR/WUR evaluation. In this contribution, we provide our views on the remaining details of evaluation methodology for LP-WUS.

- Discussion

  - Use cases

During RAN1#111-e discussion, the following possible proposal was discussed. However, it was not reached the consensus.

#### [H] Proposals 2A-v1: The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - latency is required within e.g., [the order of seconds], [latency in-sensitive IoT cases can also work] - primary for small form devices - power-sensitive, e.g., the battery should last at least few years. - E.g., targeting for limited data activity - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - latency is required within, e.g., the order of [seconds] - primary for small form devices, - power-sensitive, the battery should last multiple days (up to 1-2 weeks). - E.g., targeting for limited data activity - low/medium speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - latency is required within e.g., the order of [milliseconds] - devices form is various and not restricted - provide even higher power saving gains compared to the legacy solutions with acceptable latency impact of some typical NR services - E.g., targeting for intensive traffic arrival with delay requirements (e.g., XR for RRC connected mode) - low/medium speed Note: other use cases are not precluded if any.

NR devices, which may be in the form of RedCap/eRedCap, generally needs to provide longer battery life. In the current specification, various types of power saving features were already supported. By utilizing such existing features, UE power saving requirements may be fulfilled in most cases. However, in some cases (e.g., XR devices), good balance between latency and power consumption may be necessary and LP-WUS/WUR would be a good match for such cases.

In the possible proposals as above, three use cases are identified, i.e., IoT case, wearable case, and eMBB cases. According to SID description, IoT/wearables were already supported. However, it is still unclear to support eMBB cases. Since this is SI phase for new WUS design, we are open to discuss potential benefits from LP-WUS/WUR even for eMBB use cases.

Proposal 1: Study eMBB use cases for LP-WUS/WUR study in addition to IoT/wearable use cases.

  - Target coverage

In the RAN1#110b-e, the following agreement was made [1].

Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

As stated above, the target coverage of LP-WUS is still FFS. Since different LP-WUR architectures and corresponding signal design likely provide different coverage/sensitivity performance, and they should be selected for offering appropriate LP-WUS coverage performance. Hence, at first, it is better to clarify the target coverage with specific NR channels.

One possible way is to aim at the same coverage performance as one for NR. In Rel-17 SI on NR coverage enhancements, potential bottleneck channels were identified for different deployment scenarios. According to the study, in most of deployment scenarios, PUSCH is the potential bottleneck. Therefore, it is most straightforward to assume PUSCH as LP-WUS target coverage (Alt.1).

On the other hand, since LP-WUS/WUR is motivated for ultra-low power consumption, LP-WUS coverage performance would be quite limited. In the sense, Alt.1 may be too challenging, and target coverage needs to be carefully studied based on the study outcome of LP-WUR architectures. If target coverage of LP-WUS is too severe, power saving gain from this SI would be restricted as well. Thus, another alternative is to assume limited LP-WUS coverage compared with NR coverage. In addition, it is still not clear how to support HO and/or cell reselection by LP-WUR. If no any mobility support is provided in Ultra-Deep Sleep mode, LP-WUS/WUR can be more optimised for stationary and non-cell-edge UEs with some fallback mechanisms for cell-edge UEs.

Proposal 2: RAN1 consider the target coverage of LP-WUS with the following alternatives

- Alt.1: NR bottleneck channel, i.e., PUSCH, should be baseline for the target coverage of LP-WUS

- Alt.2: Limited coverage compared with NR should be assumed for LP-WUS/WUR

  - Assumptions for coverage evaluation

Although RAN1 has already agreed that methodology and assumptions in TR38.830/TR38.875 are reused as baseline for coverage evaluation, LP-WUS/WUR specific assumptions should also be clarified. In RAN1#110bis-e, an agreement was made for LP-WUS coverage evaluation on noise figure of LP-WUR however it is still necessary to discuss the remaining details on the assumption for LP-WUS coverage evaluation.

Agreement - For LP-WUS coverage evaluation, the noise figure of LP-WUR is - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies - FFS: how to determine the NF option. - The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

For further evaluation, possible assumptions are as follows. In our view, many of the followings would be highly depended on the specific architecture of LP-WUR, so it is better to wait the progress in AI 9.13.2. In the sense, at this stage, it may be enough to define evaluation assumptions just to be reported from individual companies. However, if the specific values of these assumption keep to be FFS, the evaluation results are difficult to align among companies encouraged to provide coverage evaluation. Therefore, RAN1 should continue the investigation on the more specific assumption to be recommend for coverage evaluation.

- Waveform (e.g., OOK and FSK)

- Channel design and structure (e.g., X-bits preamble + Y-bits payloads + Z-bits CRC)

- Coding scheme (e.g., Manchester coding)

- SCS and Bandwidth (e.g., 20MHz with 30kHz SCS)

- Guard band (e.g., X RBs on each side of LP-WUS)

- Filter and co-channel interference (X-th order Butterworth low-pass filter with cutoff frequency at Y MHz)

- Sampling rate

- Initial frequency error (e.g., 200 ppm with Uniform distribution)

- ADC bit-width

- Assumed FAR

Proposal 3: RAN1 should study the LP-WUS/WUR specific assumption for coverage evaluation with the following assumptions.

- Waveform (e.g., OOK and FSK)

- Channel design and structure (e.g., X-bits preamble + Y-bits payloads + Z-bits CRC)

- Coding scheme (e.g., Manchester coding)

- SCS and Bandwidth (e.g., 20MHz with 30kHz SCS)

- Guard band (e.g., X RBs on each side of LP-WUS)

- Filter and co-channel interference (X-th order Butterworth low-pass filter with cutoff frequency at Y MHz)

- Sampling rate

- Initial frequency error (e.g., 200 ppm with Uniform distribution)

- ADC bit-width

- Assumed FAR

- Conclusion

In this contribution, use cases, evaluation methodology, and assumptions on LP-WUS/ WUR was discussed. Based on the discussion, the following proposals were made:

Proposal 1: Study eMBB use cases for LP-WUS/WUR study in addition to IoT/wearable use cases.

Proposal 2: RAN1 consider the target coverage of LP-WUS with the following alternatives

- Alt.1: NR bottleneck channel, i.e., PUSCH, should be baseline for the target coverage of LP-WUS

- Alt.2: Limited coverage compared with NR should be assumed for LP-WUS/WUR

Proposal 3: RAN1 should study the LP-WUS/WUR specific assumption for coverage evaluation with the following assumptions.

- Waveform (e.g., OOK and FSK)

- Channel design and structure (e.g., X-bits preamble + Y-bits payloads + Z-bits CRC)

- Coding scheme (e.g., Manchester coding)

- SCS and Bandwidth (e.g., 20MHz with 30kHz SCS)

- Guard band (e.g., X RBs on each side of LP-WUS)

- Filter and co-channel interference (X-th order Butterworth low-pass filter with cutoff frequency at Y MHz)

- Sampling rate

- Initial frequency error (e.g., 200 ppm with Uniform distribution)

- ADC bit-width

- Assumed FAR

References

- RAN1 Chair’s Notes, RAN1#110b-e, October 2022.