3GPPRAN1#112                	R1-2301583

# Athens, 27th Feb. – 3th Mar., 2023

Agenda item:		9.12.2

Source:	MediaTek Inc.

Title:	UL timing management to reduce handover latency

Document for:		Discussion

# 1	Introduction

In RAN1 #111 meeting, following agreements regarding Rel-18 mobility enhancement TA management had been achieved:[1]:

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

In this contribution, we discuss the details of PDCCH ordered RACH to support TA acquisition of candidate cell(s) before the cell switch command is received in L1/L2-triggered mobility (LTM).

# 2	Timing advance management to reduce latency

According to the latest RAN1 agreement, UE need to acquire TA of the candidate cell(s) in Rel-18 LTM. As shown in Figure 1, UE might switch to the beams of neighboring cell and need to transmit PRACH toward candidate beams of multiple cells before the cell switch command. However, requiring UE to transmit PRACH toward each TCI state is a waste of UE power and RO occasions. In fact, the TA of different beams under the same cell will not vary too much, so we think it should be enough to transmit PRACH only toward one of the TCI states in a cell. In the meanwhile, considering that “UE might not be able to receive PDCCH during the PRACH transmission,” how many TAGs that UE can manage simultaneously depends on UE capability. We suggest that

Proposal 1: For TA management in L1/L2 based mobility, 1 TA/TAG per candidate cell can be acquired

- The number of TAs/TAGs among all candidate cells can be larger than 1

- The maximum number of TA/TAGs among all candidate cells that UE can manage simultaneously shall be a UE capability

Figure 1: Activate asynchronous TCI states of different cells before the cell switch command

The next issue to be studied is how to associate TA/TAG? In RAN1 #110Bis meeting, 2 alternatives of associating TA/TAG to candidate cell are provided.

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

  - the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

  - the association is provided as a part of candidate cell(s) configuration

  - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

Considering that ICBM has already been agreed as one scenario of the LTM, UE can obtain the cell ID 
through the additional PCI listed in the TCI state. It is intuitive to “associate TA/TAG to TCI-state/spatial relation” for LTM, so that we won’t have different mechanism for different LTM scenarios.

Observation 1: Association between Cell id and TCI-states have been given in the ICBM, it is intuitive to further associate TA/TAG to TCI-states if 1TA/TAG per candidate cell is assumed for LTM

As we known, even though UE would perform L1-RSRP measurement upon all the candidate cells configured in the RRC reference/delta configurations, the DL synchronization will only be done during the TCI activation stage. Therefore, if gNB trigger PRACH transmission upon beams/cells not listed in the TCI-states, UE has to simultaneously perform DL synchronization upon all the TCI states and additional beams/cells that gNB trigger PRACH transmission upon, that would introduce further limitation for the UE design and also introduce extra latency for the overall RACH procedure. To simplify the LTM procedure, we suggest associating the PRACH transmission to the activated TCI-states

Observation 2: If gNB trigger PRACH transmission upon beams/cells not listed in the TCI-states, UE may not be able to perform the DL synchronization before the PRACH transmission

Proposal 2: Associate PRACH transmission to the activated TCI-states

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

Regarding how to trigger the PRACH transmission in the DCI, in last meeting RAN1 agree to introduce indication of candidate cell in DCI. As shown below:

However, as we explained, gNB needs to trigger PRACH transmissions toward multiple cells in LTM. If the triggered beams/cell is not listed in the TCI states, DL synchronization can’t be guaranteed, and UE can only transmit PRACH based on rough frequency-time tracking results. Therefore, instead of providing the cell ID, which need 11 bits, it is a simpler approach to provide the association between the PRACH transmission and the activated TCI-states in the DCI. One possible solution is introducing a 3 bits Bitmap in DCI format 1_0 (PDCCH order) to indicate which TCI-state that UE needs to perform UL synchronization upon. As shown in Figure 2, the gNB determines the triggered PRACH transmission based on L1-RSRP reporting of activated DL TCI-states, that means DL synchronization can be guaranteed and the waste of UL transmission power is relatively small. In addition to reduce the number of bits required for the indication, introducing Bitmap can also trigger multiple PRACH transmissions through only one DCI.

Observation 3: Introducing Bitmap of the TCI-states in DCI 1_0 to trigger PRACH transmission can reduce the number of indication bits

Proposal 3: Trigger PRACH transmissions to multiple activated TCI-states by the same PDCCH order

Figure 2: Trigger PRACH transmissions to multiple activated TCI states

In last meeting, companies already agreed that configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order. According to the procedure shown in Figure 2, the corresponding RACH resource configuration of each beam/cell can be provided in advance in the RRC reference/delta configuration. In this way, when gNB indicates a certain TCI-state through the DCI, UE can find the corresponding RACH resource by tracking back the RRC configuration.

Proposal 4: RACH resource configuration for candidate cell/beam is configured in the RRC
reference/delta configuration

The remaining issue is whether and how to transmit RAR. In last meeting, companies agreed to focus on following options:

Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

  - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

  - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

Here, the Alt 2 is newly proposed in LTM, and the merit of this option is “ it will not cause interference to the serving cell reception.” However, due to the lack of feedback RAR information, the biggest problem with Alt 2 is that it is hard to control the power ramping mechanism. By contrast, Alt 1 can resolve the power control issue, but the RA-window needs to be further extended to cover the delay caused by the RAR transmission from neighboring cell to serving cell. Both methods have their own applicable scenarios, so we suggest that

Proposal 5: One indication is introduced in the PDCCH order to inform UE whether RAR is needed

- If reception of RAR is indicated, UE expect msg2 will be transmitted and RAR is received from serving cell

- If reception of RAR is not configured/indicated, UE expect no msg2 will be transmitted and TA value of candidate cell will be given in cell switch command

# 3	Summary

In this contribution, we have following observations

Observation 1: Association between Cell id and TCI-states have been given in the ICBM, it is intuitive to further associate TA/TAG to TCI-states if 1TA/TAG per candidate cell is assumed for LTM

Observation 2: If gNB trigger PRACH transmission upon beams/cells not listed in the TCI-states, UE may not be able to perform the DL synchronization

Observation 3: Introducing Bitmap of the TCI-states in DCI 1_0 to trigger PRACH transmission can reduce the number of indication bits

and we propose

Proposal 1: For TA management in L1/L2 based mobility, 1 TA/TAG per candidate cell can be acquired

- The number of TAs/TAGs among all candidate cells can be larger than 1

- The maximum number of TA/TAGs among all candidate cells that UE can manage simultaneously shall be a UE capability

Proposal 2: Associate PRACH transmission to the activated TCI-states

Proposal 3: Trigger PRACH transmissions to multiple activated TCI-states by the same PDCCH order

Proposal 4: RACH resource configuration for candidate cell/beam is configured in the RRC
reference/delta configuration

Proposal 5: One indication is introduced in the PDCCH order to inform UE whether RAR is needed

- If reception of RAR is indicated, UE expect msg2 will be transmitted and RAR is received from serving cell

- If reception of RAR is not configured/indicated, UE expect no msg2 will be transmitted and TA value of candidate cell will be given in cell switch command

# Reference

- 3GPP TSG-RAN WG1 Meeting #111 Chairman Notes, Nov., 2022.