3GPP TSG RAN WG1 #112		R1-2301666

Athens, Greece, February 27th – March 5th, 2023

Agenda Item:	9.2.2

Source:	Indian Institute of Tech (M), CEWiT, IIT Kanpur

Title:	Discussion on AI/ML based CSI Feedback Enhancement

Document for:	Discussion

# Introduction

In RAN1 #111 [1], the following agreements on evaluation of AI/ML based CSI Feedback Enhancement have been reached.

Working Assumption The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification - FFS the description and results for generalization/scalability may need a separate table - FFS the value or range of payload size X/Y/Z - FFS the description and results for different training types/cases may need a separate table - FFS: training related overhead Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case] Source 1; …. CSI generation part; AL/ML model backbone. Pre-processing. Post-processing. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. CSI reconstruction part; AL/ML model backbone. [Pre-processing]. [Post-processing]. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. Common description; Input type. Output type. Quantization /dequantization method. Dataset description; Train/k. Test/k. Ground-truth CSI quantization method. [Other assumptions/settings agreed to be reported]. Benchmark. Intermediate KPI I#1 of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Intermediate KPI I#1 of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI I#1, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI#1, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. Intermediate KPI I#2 of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Intermediate KPI I#2 of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI I#2, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI#2, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. Gain for Mean UPT; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for 5% UPT; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. FFS others. Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1. - Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn. - Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. - Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison. Agreement For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies: - Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training - Fixed/pre-configured quantization method/parameters is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2: Quantization aware training, where quantization/dequantization is involved in the training process - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase - Companies to report how to update the quantization method/parameters during the training - Note: the above cases apply for training Type 1/2/3 - Others are not precluded. Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2 - For NW-first training - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - For UE-first training - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. - Quantization behaviour, e.g., whether the shared input of the UE side CSI reconstruction part is before or after quantization.

# Discussion

In this contribution, we provide a discussion on evaluation of AI/ML based CSI Feedback Enhancement, particularly the CSI Compression for Rel 18. Specifically, this contribution focusses on prior agreements related to model switching, collaboration between NW and UE, performance monitoring and overheads.

As background we offer the following scenario in which an AI/ML based CSI Compression model would operate.

- Step 1: NW to UE – Transmit CSI-RS

- Step 2: At the UE -  Perform channel estimation to obtain the Channel State Information. Then compress the CSI using an encoder AI/ML Model

- Step 3: UE to NW  - Transmit compressed CSI on the PUCCH or PUSCH

- Step 4: At NW – Reconstruct the CSI using the corresponding decoder AI/ML Model

- Step 5: NW to UE – Transmit precoded PDSCH using reconstructed CSI, where the precoder is obtained using the CSI from Step 4

Proposal 1: A Study on Switching between AI/ML Models for various Compression Ratios should be considered.

There could be a study to determine whether there is a need for multiple AI/ML models for CSI Compression, each with a different compression ratio. The study should also investigate, criteria for switching between models of different compression ratios. We propose SNR (estimated from the CSI-RS reception) as one of those criteria. Another criterion to select a model with a particular compression ratio, could be the number of antennas available at the UE.

Proposal 2: A Study on exchange of model configuration parameters, such as the encoder output size, between NW and UE should be conducted.

In the case of multiple AI/ML models, the UE first selects an encoder model which has a pre-defined output size. The NW has to select the corresponding decoder model in order to reconstruct the CSI. A study could be done to develop a procedure by which the UE can convey the model configuration parameters, particularly the size of the encoder output, to the NW. This will aid the NW in selecting the correct decoder model. One way is to transmit the encoder output size along with the compressed CSI, as part of the UCI payload (on PUCCH or PUSCH).

Proposal 3: A detailed study on the overheads for exchanging training data in Type 3 Compression is required.

In RAN1 #110 [2] it was agreed that three types of AI/ML model training collaboration frameworks will be studied. Our comments are specifically for Type 3 which we reproduce as follows:

Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

For the type of separate training to occur, we believe that there needs to be an exchange of several instances of encoder outputs between the UE and the NW. It is our recommendation that further study is needed on the feasibility of transferring such large datasets over the air or otherwise. Constraints such as limited bandwidth, latency requirements, etc., should be considered in this study.

# Conclusion

In this contribution, we provided a discussion on the evaluation of AI/ML based CSI Compression for Rel 18. The discussion has resulted in the following proposals.

Proposal 1: A Study on Switching between AI/ML Models for various Compression Ratios should be considered.

Proposal 2: A Study on exchange of model configuration parameters, such as the encoder output size, between NW and UE should be conducted.

Proposal 3: A detailed study on the overheads for exchanging training data in Type 3 Compression is required.

# References

[1] Chair’s Notes, RAN1 #111, November 2022

[2] Chair’s Notes, RAN1 #110, August 2022