3GPP TSG RAN WG1 #112		R1- 2301688

Athens, Greece, 27 February – 03 March 2023

Agenda item:	9.2.2.1

Source:		CEWiT

Title:			Evaluation of AI and ML for CSI feedback enhancement

Document for:	Discussion

# 1 Introduction

In RAN#111, the following agreements, conclusions, and working assumptions were adopted regarding CSI feedback enhancement.  For general CSI enhancement evaluation, the following items apply:

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

Working Assumption

For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification.

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | UE speed |  |  |
|  | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Benchmark 1 |  |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 1 |  |  |  |
| Intermediate KPI #2 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT |  |  |
|  | 5% UPT |  |  |

Agreement

Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement.

# 2  CSI Prediction

Channel State Information (CSI) is a mechanism that a UE measure various radio channel quality and report to the base station. Due to time varying nature of channel and delays in computation, the channel varies between when it is learned at the base station and when it is used. This results in aging of channel. Thus CSI prediction can be performed and the main goal of it is to achieve a good prediction range into the future with minimal prediction error.

In this section, we discuss the evaluation methodology for AI based CSI prediction in time domain , including type of model and preliminary simulation results.

2.1 Evaluation Methodology

The AI-based CSI prediction is considered a regression problem where a range of output values is predicted. The model uses historical CSI data to forecast future CSI values within a prediction window. The input to the model is the historical CSI data, and the output is the predicted future CSI data. The CSI signals are assumed to have a time interval of 5ms and cover the entire RB.

For the prediction sub-use case, a time-series model was created based on a SISO-OFDM channel. The input to the model consisted of down link channel estimates up to a certain time instant. The model was used to make single-step predictions with varying input lengths, and the impact of different observation window sizes on the accuracy of the predictions was analyzed.

2.2 Simulation Parameters

In this section, we provide some simulation parameters for the task. The data is generated using Broadband Wireless Simulator(BWSim), and channel is modeled based on TR 38.901 for UMa scenario.

Table-1:  System Level Simulation Assumptions

| Paramter | Value |
| --- | --- |
| Channel model | According to TR 38.901 |
| Scenario | UMa (7 macrocell sites and 3 sectors per site) |
| Carrier frequency | 4Ghz |
| Subcarrier spacing | 15Khz 14 OFDM symbol slot |
| UE Speeds | 3 kmph , 30 kmph |
| CSI Periodicity | 5ms |
| Antenna configuration [Mg Ng M N P] | [1 1 1 1 1] |

The parameters of the AI/ML model has also been presented. 100K samples of data were considered  which is split in the ratio 90-10 for training and validation. Adam optimizer was used and NMSE was taken as loss function.

Table-2 : Neural Network model parameters

| Parameter | Value |
| --- | --- |
| Batch Size | 256 |
| Number of epochs | 50 |
| Optimizer | Adam |
| Learning rate | 0.0001 |
| Training / validation split | 90% /10% |
| Algorithm | RNN / LSTM |
| Loss Function | Normalised Mean Squared Error |
| Metric | Absolute Difference |
| Input | Estimated downlink channel H, upto kth time instances |
| Number of parameters | 1.64M |

2.2 Preliminary results

Real and imaginary parts of the channel coefficient's were taken separately for prediction.  Data was pre-processed before training the model. Currently, single step prediction was performed and multi-step forecasting is currently in progress.

Fig 1: CDF of the NMSE with channel prediction 5 ms ahead for a UE velocity of 3kmph (Pedestrain)

Fig 2: CDF of the NMSE with channel prediction 5 ms ahead for a UE velocity of 30kmph

3. Conclusion:

Observation 1: For CSI prediction, the number of past observations are vital for prediction accuracy.  The result shows that with more number of past instances the prediction seems to perform better.

Observation 2: For CSI prediction, the user mobility and  coherence time are important factors for AI/ML model’s prediction accuracy.

Observation 3: For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds.

Observation 4: When the coherence time is less than the CSI-RS periodicity, the CSI prediction performance will degrade rapidly.

Observation 5: For AI/ML-based CSI prediction, an LSTM-based AI/ML model can be applied   	     for training.

Proposal 1: The solution to improve the generalization capability of AI model across different configurations/ scenarios could be further studied.

Proposal 2: Time domain based prediction can be considered as  the data will be obtained for slot basis in real time scenario.

Proposal 3: To study the trade-off between observation length and prediction length for different configurations.

Proposal 4: To  study the input and output types of the AI/ML based CSI prediction model.

Proposal 5: The generalization of AI/ML -based CSI prediction across various UE speeds should be studied.

4. Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2212673, Summary#5 of [111-R18-AI/ML], November 2022.

- 3GPP RAN1#111, RAN1 Chairman’s Notes, November 2022

- 3GPP RAN1#110bis-e, RAN1 Chairman’s Notes, October 2022.

- 3GPP RAN1#110, RAN1 Chairman’s Notes, August 2022.