3GPP TSG RAN WG1 #112			R1-2301893

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.12.2

Source: 	Moderator (CATT)

Title: 	Moderator summary on Timing advance management for LTM: Round 1

Document for:	Discussion and Decision

- Introduction

In RAN #94e, the Rel-18 WID of Further NR mobility enhancements are approved [1]. In the approved WID, Timing Advance management is a part of RAN1 objectives,

To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

This summary includes the following:

- Summary of companies’ views on each of open issues raised by interested companies

- Observations and recommended proposals based on the summary of companies’ views

- Issue 1 – Initial TA acquisition for RACH-based solutions

Open issues on RACH-based solutions for TA acquisition of candidate target cell(s) and company views are summarized below.

Table 1. Summary of views on Issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | For RACH-based solution, how to configure PRACH resources for candidate cell(s)? | PRACH configuration associated with each candidate cell: Google, Lenovo, MTK(RACH resource configuration for candidate cell/beam is configured in the RRC reference/delta configuration), Spreadtrum, NTTDoCoMo, OPPO, ZTE(associated with RACH-ConfigCommon for the candidate cell as legacy), CATT, Huawei(RACH configuration for candidate cell should be separate from CellGroupConfig for candidate cell) |
| 1.2 | For RACH-based solution, how to indicate PRACH resources for a candidate cell to perform TA acquisition? | Alt1: Use a new field in DCI format 1_0 for PDCCH order to carry the index of a candidate cell or a serving cell - Huawei, Vivo, NTTDoCoMo, CATT, Spreadtrum, OPPO, Lenovo, Google, Apple, QC Alt2: Introduce bitmap of the TCI-states in DCI format 1_0 to trigger PRACH transmission - MTK |
| 1.3 | CFRA or CBRA based RACH | CFRA: Huawei, CATT, ZTE, Samsung, Qualcomm, Vivo, Nokia, MTK, NTTdocomo CBRA: Samsung(lower priority) Both CBRA & CFRA: KDDI, Intel |
| 1.4 | Whether to receive RAR after triggered PRACH transmission? | Alt1: RAR is needed: ZTE, Futurewei, KDDI Alt2: RAR is not needed: Ericsson, Qualcomm, Spreadtrum, OPPO, Intel, LG, Xiaomi, Interdigital, KDDI(for CFRA) Alt3: whether RAR is needed can be configured: Huawei, Samsung, vivo, Nokia, MTK(indicated in PDCCH order), Xiaomi, Lenovo, Google, Apple, NTTDoCoMo. |
| 1.5 | If reception of RAR is indicated/configured, whether RAR is received from serving cell or candidate cell? | From serving cell: Samsung, NTT, DoCoMo(from Spcell), MTK From candidate cell: ZTE, Nokia, Futurewei, Apple |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P1-1

Proposal 1-1: For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured in each candidate cell configuration.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | OK in principle |
| QC | Not support. Configuring under candidate cell will require UE to validate/parse all candidate cells even they are later not selected for switch. We are fine for either configured under serving cell or under separate IE outside any cell (preferred), so UE only needs to parse the candidate cell config when it is selected for switch |
| Nokia | We can support this proposal once we confirm with RAN2 that the UE can read and process a candidate cell configuration before the cell switch. Also, we agree with QC, but since this is a RAN2 issue, this needs to be confirmed with them. |
| Lenovo | OK in principle, however, we have the same concern with QC, whether the PRACH configuration for candidate cell is configured under serving cell or under separate IE outside any cell should be further discussed. |
| Futurewei | We have similar view as QC. For PDCCH ordered early RACH, RACH configuration for the candidates is better to be under serving cell or a separate IE so that when an early RACH is triggered, the UE only needs to apply the RACH configuration instead of applying all configurations of the candidate cell. |
| Xiaomi | RAN1 can decide what information about RACH of candidate cell(s) should be configured. The configuration signaling can be determined by RAN2. Proposal 1-1: For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices for each candidate cell should be pre-configured. - The configuration signaling is decided by RAN2 |
| Mod | Thanks for the comments and suggestions. I tend to agree with you that RRC signaling design is up to RAN2 decision. And then, from RAN1 perspective, we can just let them to know what is the necessary information for PDCCH-ordered RACH. To that end, P1-1 is revised as follows. Proposal 1-1: For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. |
| vivo | RACH configuration is RAN2’s issue and prefer to make decision after RAN2’s confirm. |
| MTK | Okay to the updated proposal |
| NTTdocomo | Support. |
| Ericsson | We are fine with the updated proposal |
| FGI | We are fine with the updated proposal. Indeed, RRC signalling design is RAN2’s work. RAN1 just needs to decide what is the necessary information we need for PDCCH ordered RACH. |
| Huawei, HiSilicon | We are fine with the updated proposal 1-1 by FL. The configuration should be per candidate cell. However, the IE carrying the configuration should be determined in RAN2. At least we do not think such configuration should be in cellgroupconfig for candidate cell as it may not be parsed before CSC according to RAN2 discussion. |

For the 1st offline discussion:

Proposal 1-1: For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

P1-2

Proposal 1-2: The PDCCH order from the source cell contains the indication of candidate cell (e.g., a logic identity of a candidate cell or a serving cell).

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | OK in principle. |
| QC | Support. Prefer to add the following FFS for detailed PDCCH order PRACH implementation issue. The root issue is that UE needs to prepare/store separate RF configuration for each candidate cell not part of UL CA. So the number of such candidate cells should be UE capability. Also, the triggering timeline between DCI and PRACH can be longer due to RF config switch, which will also interrupt the UL Tx of serving cells. The issue is similar to SRS carrier switch described in 38.214->6.2.1.3 Proposal 1-2: The PDCCH order from the source cell contains the indication of candidate cell (e.g., a logic identity of a candidate cell or a serving cell). - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity - FFS: For PDCCH order based PRACH on candidate cell without PUCCH/PUSCH, corresponding UE capability, timeline, interruption/prioritization rule |
| Nokia | Support. |
| Lenovo | Support. |
| Futurewei | Fine with the FL proposal. |
| Xiaomi | Support |
| vivo | Support. |
| NTTdocomo | Support. |
| Ericsson | Support |
| FGI | Support. |
| Huawei, HiSilicon | Support proposal 1-2 by FL |

For the 1st offline discussion:

Proposal 1-2: The PDCCH order from the source cell contains the indication of candidate cell (e.g., a logic identity of a candidate cell or a serving cell).

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Proposal 1-2-1:

- FFS: For PDCCH order based PRACH on candidate cell without PUCCH/PUSCH, corresponding UE capability, timeline, interruption/prioritization rule

P1-3

Proposal 1-3: For PDCCH-order based RACH on candidate cell, only CFRA is supported.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | Support. |
| QC | Support. CFRA is sufficient. CBRA needs Msg3 for contention resolution |
| Nokia | Support. |
| Lenovo | Support. |
| Futurewei | OK with FL proposal. CFRA with Msg2 from the candidate cell seems better than CBRA which require Msg3, 4 to resolve contention which introduces more delay. The cost of CFRA is that the dedicated preamble resources need to be pre-configured for all the candidate cells. The resource maybe locked for long time. Anyway, it is a known low resource efficiency issue with PDCCH ordered early RACH. |
| Xiaomi | Support |
| vivo | Support. |
| MTK | Support |
| NTTdocomo | Support. |
| Ericsson | Whether to use CFRA or CBRA can be left for network implementation. With no RAR; there is no need for the UE to know if this is CBRA or CFRA. |
| Huawei, HiSilicon | Support |

For the 1st offline discussion:

Proposal 1-3: For PDCCH-order based RACH on candidate cell, only CFRA is supported.

P1-4

Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated

- If reception of RAR is not configured/indicated

  - TA value of candidate cell is indicated in cell switch command

  - UE should not retransmit PRACH with power ramping

- FFS: if reception of RAR is configured/indicated, whether RAR is received from serving cell or target cell

  - If RAR is received from the target cell, Type1-PDCCH CSS of the target cell is configured to the UE

- FFS: signaling for configuration/indication of whether RAR needs to be received

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | Selecting either option of Alt 1(RAR is needed) or Alt 2(RAR is not needed) is more preferred. As we can see in sub-bullets, both options need further spec efforts from legacy. Prefer to focus on either option considering limited TU of this agenda. |
| QC | We prefer support no RAR reception first. The procedure is clear. It is not clear on the benefit for RAR reception, and the corresponding procedure is also new, e.g. there is no PRACH and RAR on different cells today, while transmitting Msg1 + receiving Msg2 on candidate cell seems violate the HO purpose. But we are open to discuss this option further. Proposal 1-4: For PDCCH ordered-RACH, at least no RAR reception is supported - […] |
| Nokia | Support. |
| Lenovo | Support. |
| Futurewei | We do not support Proposal 1-4. We think no-RAR from the candidate cell will cause the more problems than benefit: - Introduced additional backhaul delay on top of RACH delay for the UE to receive the target TA. Note: RAN2/3 already agreed for LTM there is no source DU and target DU direct connection/interface to be developed. The target TA needed to be delivered from target-DU ->CU-> source-DU-> UE. - Introduced additional delay for the source DU/cell before issuing the cell switch command. This will increase the LTM cell switch failure rate. - Without RAR from the candidate DU, existing RACH preamble retransmission process is broken. If only one preamble transmission is allowed, the reliability of RACH is compromised. If we rely on the source cell to trigger the retransmission, a new RA response time window needs to be added and the awareness of the Msg1 failure is largely delayed. - Non RAR introduce changes at the DU/CU interface, and has more impact to RAN2 and RAN3. Unless the non-RAR is only used in intra-DU LTM scenario. The benefit of no-RAR is only reduce some DL interruption in case DCCA is not enabled. Comparing with the problems introduced by no-RAR, it is not worth to support the no-RAR option. Even make it configurable, UE still need to support both. Increase of complexity cannot be avoided. Since RAR of RACH is part of MAC which is handled by RAN2, and whether to have RAR has more impact to RAN2 and RAN3, we would suggest to let RAN2/3 to discuss it and make the decision on this issue. |
| Xiaomi | Support |
| vivo | Support. |
| MediaTek | Support the compromise proposal |
| NTTdocomo | We support in principle. No RAR is certainly beneficial for candidate cell. So, we are open to support no RAR for candidate cell which is not current serving cell. However, we think RAR is needed for at least candidate cell which is current serving cell since current serving cell should follow legacy. For second sub-bullet, we do not understand why PRACH retransmission with power ramping is not needed. When PRACH is not received on candidate cell and TA of the candidate cell should be acquired, (i.e., next PRACH is transmitted on the same candidate cell, e.g., indicating the same beam), how could PRACH be received on candidate cell if power ramping is not supported? Thus, we think PRACH with power ramping should be considered in some cases. Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated - If reception of RAR is not configured/indicated - TA value of candidate cell is indicated in cell switch command - UE can not transmit PRACH with power ramping in some conditions. FFS the conditions (e.g., if UE receives PDCCH order again indicating the same candidate cell with the same beam index). […] |
| Ericsson | We support |
| FGI | We support FL’s proposal. |
| Mod | Based on comments from Docomo, the following update is made. Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed for candidate target cell(s) can be configured/indicated - If reception of RAR is not configured/indicated - TA value of candidate cell is indicated in cell switch command - FFS: whether UE can should not retransmit PRACH with power ramping or not - FFS: if reception of RAR is configured/indicated, whether RAR is received from serving cell or target cell - If RAR is received from the target cell, Type1-PDCCH CSS of the target cell is configured to the UE - FFS: signaling for configuration/indication of whether RAR needs to be received |
| Huawei, HiSilicon | For the case RAR reception is not configured, the current sentence “UE should not retransmit PRACH with power ramping” is clear. It may imply the retransmission of PRACH without power ramping is still possible. We think the intention is UE do not retransmit PRACH if RAR is not received, no matter power ramping is used or not. Moreover, as UE does not know whether PRACH is received or not through RAR reception and failure may be due to low initial transmit power at UE, it would be beneficial to introduce TPC bits in PDCCH order to help UE determine its transmit power for the subsequent PRACH transmission triggered by PDCCH order. For the case RAR reception is configured, the similar hierarchical structure of proposal should be kept as those for the case of RAR reception is not configured. We suggest update proposal 1-4 as follows Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated - If reception of RAR is not configured/indicated - TA value of candidate cell is indicated in cell switch command - UE should not retransmit PRACH when RAR is not received - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - if reception of RAR is configured/indicated, - FFS: whether RAR is received from serving cell or target cell - If RAR is received from the target cell, Type1-PDCCH CSS of the target cell is configured to the UE - FFS: signaling for configuration/indication of whether RAR needs to be received |

For the 1st offline discussion:

Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed for candidate target cell(s) can be configured/indicated

- If reception of RAR is not configured/indicated

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE can should not retransmit PRACH with power ramping or not

- FFS: if reception of RAR is configured/indicated, whether RAR is received from serving cell or target cell

  - If RAR is received from the target cell, Type1-PDCCH CSS of the target cell is configured to the UE

- FFS: signaling for configuration/indication of whether RAR needs to be received

Revised Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated

- If reception of RAR is not configured/indicated

  - TA value of candidate cell is indicated in cell switch command

  - FFS: UE should not retransmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- if reception of RAR is configured/indicated,

  - FFS: whether RAR is received from serving cell or target cell

  - FFS: content of RAR

  - FFS: If RAR is received from the target cell, Type1-PDCCH CSS of the target cell is configured to the UE

- FFS: signaling for configuration/indication of whether RAR needs to be received

P1-5

Proposal 1-5: For PDCCH order based PRACH on candidate cell without PUCCH/PUSCH, further study the corresponding UE capability, timeline, interruption/prioritization rule, etc.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |

- Issue 2 – Initial TA acquisition for RACH-less solutions

Open issues on TA acquisition of the candidate target cell(s) for RACH-less solutions and company views are summarized below.

Table 2. Summary of views on Issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | Initial TA acquisition for RACH-less solutions | Alt1: SRS based TA acquisition - Support: Huawei(SRS+RSTD), Vivo,OPPO, CATT, Qualcomm(UE reports RX timing difference) - Not support: Futurewei, Nokia, Spreadtrum, Lenovo, NTTdocomo Alt2: RACH-less mechanism as in LTE - Support: ZTE, Samsung, Spreadtrum, Intel, Apple, Nokia Alt3: UE based TA measurement(UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA command for the current serving cell) Support: Ericsson, Qualcomm, Vivo, Nokia(with lower priority), NTTDoCoMo(UE optional), OPPO, Intel, Xiaomi, Google, CMCC, Futurewei Not support: Lenovo |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P2-1

Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA=0 or the latest TA value of the source cell.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Fine for the conclusion. |
| Nokia | Fine with the conclusion, but we need to finalize other details, where/how will the source cell indicate the TA, e.g., in the cell switch command, in a dedicated TA field? |
| Lenovo | Ok with the conclusion. |
| Futurewei | We agree with the observation. But we would like to point out that the two very special case is included in Alt3 via corresponding configurations and DL timing measurement without separate effort. |
| Xiaomi | OK |
| vivo | Fine with the conclusion and RACH-less mechanism can be achieved by Alt3. |
| NTTdocomo | Support. |
| Ericsson | We do not want to limit to the LTE case but have the possibility to also signal other values to the UE. With the TA in the cell switch command, the UE does not need to how the network has found the new TA. |
| FGI | Support. |
| Mod | @Futurewei: I agree with you that if Alt-3 is supported, the two values can actually be included. Also, I’d like to explain that with this proposed conclusion, we do not intend to introduce any extra spec effort in addition to R-18 LTM. Based on Ericsson’s comment, the following note is added for further discussion. Proposal 2-1 (proposed conclusion): RACH-less mechanism as in LTE can be supported by indicating TA=0 or the latest TA value of the source cell. - Note: this doesn’t mean to preclude TA values other than 0 and the latest TA value of the source cell. |
| Huawei, HiSilicon | support |

P2-2

Proposal 2-2: TBD

| Company | Input |
| --- | --- |
| Mod | Details/proposals on Alt1 and 3 can be presented in this table. Depending on the opinion of majority companies and progress of our discussion, FL proposal can be drafted later. |
| QC | We think the benefit of Alt3 is clear on reducing overhead, interference, implementation issue due to PRACH on candidate cell. To our understanding, good knowledge on DL Tx timing difference is feasible at least in intra-DU case. Many solutions are introduced in IEEE 1588/802.1AS |
| Futurewei | We think to address the latency issue in fast cell switch scenarios of LTM, RACH-less solution is required. Due to the merits of Alt3, we think it is a good base line RACH-less solution. We see the benefits and we are open to the possibility of converged Alt1 and Alt3 solution, i.e., the Alt3 determined initial TA can be used for the initial UE UL transmission of SRS to the target cell. We think at least the UE initial UL transmission to the target cell can be SRS and UL sync inaccuracy tolerable short data block. SRS can be used by the target cell for further fine TA tunning. |
| vivo | We share similar view with QC and Alt3 should also be supported. And, only spec impact is introducing a new UE capability. |

- Issue 3 – Association between TA/TAG and candidate target cell

Open issues on TA indication and company views are summarized below.

Table 3. Summary for Issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1 | Association between TA and candidate cell | Opt1: implicit association between TA/TAG and candidate target cell: Interdigital - Opt 1.1: The association between TA/TAG and TCI states can be configured: Samsung, MTK Opt2: explicit association between TA/TAG and candidate target cell ID: ZTE, Spreadtrum, Lenovo, Intel, OPPO, CMCC, Nokia, LG - Opt 2.1: the association is provided as a part of candidate cell(s) configuration: vivo, Apple, Qualcomm, NTT DoCoMo - Opt 2.2: the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration: Huawei Opt3: Do not support associating the TA with a candidate cell: Google |

P3-1

Proposal 3-1:  Explicit association of TA/TAG and candidate cell is supported, the following solutions can be further discussed.

- The association is provided as a part of candidate cell(s) configuration

- The association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cells(s) configuration

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | Support Opt 2.1. |
| QC | Fine for Proposal 3-1, and prefer Opt 2.1. |
| Nokia | Before finalizing one of the options, we should first finalize the details of PDCCH order RACH. In case, the TA is sent in the cell switch command, which would also contain the target cell identity, do we need any additional association (TAG/TA ID)? Or, if the TA is sent in a RAR message from the target cell – do we need any additional association (TAG/TA ID)? It is not clear when this kind of association is needed. In legacy TA acquisition, such association is needed when a TAC is sent, but we haven’t agreed on the need of such TAC type message yet. We may need such association for CA or mTRP in the target cell, but lets clarify the scenarios first. |
| Lenovo | Support Opt 2.1. |
| Futurewei | Support Proposal 3-1, we support Opt 2.2 and think Opt. 2.2 is the details for Opt. 2.1. The configuration should be down to the beam level to facilitate target beam handling for mobility. An example would be the target cell has mTRPs, in order to support subsequent cell switches, the UE operations with TAC in the new serving should be supported without new RRC configuration in the new serving cell. |
| Xiaomi | Fine with proposal 3-1. Support option 2.1 |
| vivo | Support Opt 2.1. |
| NTTdocomo | Support Opt 2.1. |
| Ericsson | Agree with Nokia that this can be discussed once the details of the TA handling is agreed |
| FGI | We are fine with FL’s proposal. |
| Huawei, HiSilicon | The association should be explicitly with candidate cell, e.g. with SSB/TRS. The detail IE can be left for FFS. The “as a part of candidate cells(s) configuration” should be further clarified whether it implies cellgroupconfig of candidate cell or other IE. |

- Issue 4 - TA updating

Table 4. Summary for Issue 4

| # | Issue | Companies’ views |
| --- | --- | --- |
| 4.1 | Condition to trigger TA updating for candidate cell(s) | Opt1: UE-based updating(e.g. TAT maintained at UE side) Opt2: NW based updating(the network maintains the TA values for the candidate cells and signals the value for the target cell during cell switch command) Ericsson, Interdigital, Xiaomi, Nokia(based on UL signal quality etc), Qualcomm(cell switch command can trigger SRS to refine the TA for the new cell), NTTdocomo |

P4-1

Proposal 4-1(proposed conclusion): The condition triggering TA updating/re-acquisition for candidate cell is up to NW.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above issue. |
| QC | We think it is better to restrict to PDCCH-order based scheme for now. Other schemes are still unclear. The condition triggering TA updating/re-acquisition for candidate cell is up to NW at least for PDCCH-order based TA acquisition. |
| Nokia | Support QC’s proposal. |
| Lenovo | Support QC’s proposal. |
| Futurewei | We agree with that for now early TA acquisition/update is triggered by NW. |
| Xiaomi | Agree with QC’s modification. And there is another issue we are worried about. The TA may not be valid if UE does not receive the triggering signaling, that is PDCCH order, for a long time. An FFS should added to deal with this issue. Proposal 4-1(proposed conclusion): The condition triggering TA updating/re-acquisition for candidate cell is up to NW at least for PDCCH-order based TA acquisition. - FFS: What if UE does not receive the triggering signaling for a long time |
| Mod | Based on comments above, the proposal is revised accordingly. Proposal 4-1(proposed conclusion): At least for PDCCH-order based TA acquisition, the condition triggering TA updating/re-acquisition for candidate cell is up to NW. @Xiaomi: as what we have in this proposal, the condition triggering TA updating/re-acquisition for candidate cell is up to NW. So, it’s up to NW whether/when to trigger the updating. |
| vivo | We think this issue is related to the approach of initial TA acquisition and the procedure of TA maintenance, it should be discussed after agreements/conclusions on the above issues are achieved. |
| NTTdocomo | Support |
| Ericsson | Did we not agree this last time? |
| FGI | We are fine with FL’s revised proposal. |
| Mod | @vivo: at least we agreed to support TA acquisition solution based on PDCCH ordered RACH. So, the part in red is added. @Ericsson: in the last meeting we agreed to use the same triggering mechanism as in initial TA acquisition. In this proposal, what we’re trying to conclude is the condition triggering TA update. |
| Huawei, HiSilicon | Fine with the moderate’s updated proposal 4-1. |

- Other issues

Table 5. Summary for other issues

| # | Issue | Companies’ views |
| --- | --- | --- |
| 5.1 | Number of TAs associated with candidate cell(s) can be handled by UE | At least one TA associated with candidate cell(s) can be handled by UE Huawei, CATT, ZTE, Spreadtrum The number of TA associated with candidate cell(s) can be handled by UE depends on UE capability vivo, MTK,CMCC, CATT, UE, NTTdocomo |
| 5.2 | TA acquisition of the candidate cell before cell switch command when it is deactivated SCell | Support: Qualcomm, NTTDoCoMo, Vivo |
| 5.3 | Whether two TA(s)/TAG(s) for a candidate cell can be configured | Yes: Nokia/Google(up to two), Huawei No: LG(up to one) |
| 5.4 | In case acquisition of TA of candidate cell cannot be performed before cell switch, it needs to be triggered following reception of the cell switch command. Different alternatives can be envisioned for this: - Alt. 1: UE gets explicit indication to initiate RACH in cell switch command. - Alt. 2: UE triggers RACH if it does not have valid TA for the candidate cell (TCI state), based on the TAG associated to this cell or TCI state. | Support: Interdigital |
| 5.5 | RAN1 should investigate the interruption/prioritization of UL Tx on serving cell due to PRACH on candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - The interruption/prioritization rule due to SRS transmitted on serving cell without PUCCH/PUSCH configured can serve as starting point, i.e. 38.214->6.2.1.3 | Support: Qualcomm |

P5-1

Proposal 5-1:  For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA other than the TA used for the serving cell

- The maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| LGE | OK in principle. |
| QC | The main bullet may not be needed. In case of small cells, the propagation delay can be ignored with TA=0. So UE not supporting TA acquisition can still work for that scenario. |
| Lenovo | OK in principle. |
| Futurewei | In principle, Proposal 5-1 is fine. This will ensure good mobility performance when the target cell has multiple TRPs. |
| Mod | As commented by QC, TA could be 0 in some cases. So, P5-1 is revised as follows. Proposal 5-1: For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA - The maximal number of TA associated with candidate cell(s) can be handled by UE is up to UE capability |
| vivo | Fine with Proposal 5-1. |
| NTTdocomo | Support in principle. |
| Ericsson | No need to discuss at this time |
| Huawei, HiSilicon | support |

P5-2

Proposal 5-2: TA acquisition before cell switch command can be applied when candidate cell is deactivated SCell.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | It would be beneficial to generalize the solution to deactivated SCell. Otherwise, additional latency will be introduced. Btw, the deactivated SCell is defined in 38.321->5.9 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; |
| Futurewei | We are fine with proposal 5-2. |
| NTTdocomo | Support. |

- Issues for online discussion/confirmation

7.1 Round 1

Offline agreement (Proposal 1-1): For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

Offline agreement (Proposal 1-2): The PDCCH order from the source cell contains the indication of candidate cell (e.g., a logic identity of a candidate cell or a serving cell).

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Revised Proposal 1-4: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated

- If reception of RAR is not configured/indicated

  - TA value of candidate cell is indicated in cell switch command

  - FFS: UE should not retransmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated, FFS

  - whether RAR is received from serving cell or target cell

  - content of RAR

  - if RAR is received from the target cell, whether Type1-PDCCH CSS of the target cell is configured to the UE

- FFS: signaling for configuration/indication of whether RAR needs to be received

Proposal 1-3: For PDCCH-order based RACH on candidate cell, only CFRA is supported.

Proposal 1-5: For PDCCH order based PRACH on candidate cell without PUCCH/PUSCH, further study the corresponding UE capability, timeline, interruption/prioritization rule, etc.

- References

- RP-222332	Revised WID on Further NR mobility enhancements		                MediaTek (Moderator)

- R1-2300057	Discussion of the merits of UE based  RACH-less TA acquisition for LTM 	FUTUREWEI

- R1-2300129	Timing advance management to reduce latency				Huawei, HiSilicon

- R1-2300189	Enhancements on TA management to reduce latency			ZTE

- R1-2300240	Discussion on timing advance management to reduce latency		Spreadtrum Communications

- R1-2300312	Discussions on Timing Advance Management				OPPO

- R1-2300385	Timing Advance Management for L1/L2-triggered Mobility			Nokia, Nokia Shanghai Bell

- R1-2300475	Discussion on TA management for L1L2 mobility				vivo

- R1-2300490	Discussion on TA management for LTM					FGI

- R1-2300518	Timing advancement management for L1L2 mobility			Lenovo

- R1-2300537	Enhancements on TA management for mobility				LGE

- R1-2300558	Discussion on Timing advance management				Xiaomi

- R1-2300663	Discussion on time advance management to reduce latency			CATT

- R1-2300968	On Timing Advance Management					Intel Corporation

- R1-2301025 	Discussion on timing advance management to reduce latency		CMCC

- R1-2301035	Discussion on TA management to reduce latency				KDDI Corporation

- R1-2301095	Timing advance management for L1/L2 Mobility				Ericsson

- R1-2301168	Discussion on timing advance management to reduce latency		Google

- R1-2301170	Timing advance management to reduce latency				InterDigital Communications

- R1-2301288	Candidate cell TA acquisition for NR L1/L2 mobility enhancement		Samsung

- R1-2301370	Timing advance management for L1/L2 Mobility				Apple

- R1-2301437	TA management to reduce latency for L1/L2 based mobility			Qualcomm Incorporated

- R1-2301515	Timing advance enhancement for inter-cell mobility			NTT DOCOMO, INC.

- R1-2301583	UL Timing management to reduce handover latency			MediaTek Inc.

- Previous agreements

RAN1 #110bis-e

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

•         RACH-based solutions

e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

•         RACH-less solutions

e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- the association is provided as a part of candidate cell(s) configuration

- the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

RAN1 #111

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

Agreement (Made in RAN1#110b-e)

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

    - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell