3GPP TSG-RAN WG1 Meeting #112	R1-2301920

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT]CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type-1 HARQ-ACK codebook multiplexing for unicast and multicast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), [ZTE] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If UE is configured with unicast DCI formats and multicast DCI formats for PDSCH reception and UE is provided fdmed-ReceptionMulticast, there are three cases for the DCI receptions from the UE side: - 1st case, only the unicast DCI format is received; - 2nd case, only the multicast DCI format is received; - 3rd case, both the unicast DCI format and the multicast DCI format are received. Only the 3rd case is specified currently. UE behaviors for the other two cases are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | the UE generates - a Type-1 HARQ-ACK codebook for the set SU of serving cells by setting NcellsDL=NcellsDL,U in the following pseudo-code, if only the unicast DCI format is received from the set SU of serving cells, or - a Type-1 HARQ-ACK codebook for the set SM of serving cells by setting NcellsDL=NcellsDL,M in the following pseudo-code, if only the multicast DCI format is received from the set SM of serving cells, or - a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code, if both the unicast DCI format is received from the set SU and the multicast DCI format is received from the set SM of serving cells, respectively. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviors are unclear for the two cases when only the unicast DCI format is received or only the multicast DCI format is received for multiplexing Type-1 HARQ-ACK codebook when UE is provided fdmed-ReceptionMulticast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged parts are omitted>

If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats

-	a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates

- a Type-1 HARQ-ACK codebook for the set SU of serving cells by setting NcellsDL=NcellsDL,U in the following pseudo-code, if only the unicast DCI format is received from the set SU of serving cells, or

- a Type-1 HARQ-ACK codebook for the set SM of serving cells by setting NcellsDL=NcellsDL,M in the following pseudo-code, if only the multicast DCI format is received from the set  SM of serving cells, or

- a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code, if both the unicast DCI format is received from the set SU and the multicast DCI format is received from the set SM of serving cells, respectively. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.

<Unchanged parts are omitted>