3GPP TSG-RAN WG1 Meeting #112  	R1-2301982

# Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft 38.213 CR on parameter name alignment for unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spreadtrum Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-core |  |  |  |  |  | Date: |  |  | 2023/02/17 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The follwing conclusion was captured in RAN1#110, For alignment TS38.213 CR: Text proposal provided in R1-2207898 Correction on RRC parameters for SRS and power control with unified TCI states is endorsed for the editorial corrections.. which was from Moderator alignment CR R1-2207898 based on vivo contribution R1-2206729. - Change “TCI-State_r17” to “TCIState in dl-OrJoint-TCIStateList-r17 or UL-TCIstate” in section 6; - Change “p0-Alpha-CLID-PUSCH-Set” to “p0AlphaSetforPUSCH”, change “p0-Alpha-CLID-PUCCH-Set” to “p0AlphaSetforPUCCH”, and “p0-Alpha-CLID-SRS-Set” to “p0AlphaSetforSRS” in section 6 and 7; - Change “useIndicatedTCIState” to “followUnifiedTCIstateSRS” in section 7.. However, some related alignment corrections are missed in sections 6 & 7.7.1 in the newest updated TS38.213. - The higher layer parameters followUnifiedTCIstate, followUnifiedTCIstateSRS and dl-OrJoint-TCIStateList in sections 7 & 10.1 in TS38.213 are not aligned with TS 38.331. |
| Summary of change: |  | Section 6: change “TCI-State_r17” to “dl-OrJointTCI-StateList or TCI-UL-State”. Section 6 & 7.7.1: change “p0-Alpha-CLID-PUSCH-Set” to “p0AlphaSetforPUSCH”, change “p0-Alpha-CLID-PUCCH-Set” to “p0AlphaSetforPUCCH”, change “p0-Alpha-CLID-SRS-Set” to “p0AlphaSetforSRS” . Section 10.1: change “followUnifiedTCIstate” to “followUnifiedTCI-State”. Section 7: change “followUnifiedTCIstateSRS” to “followUnifiedTCI-StateSRS”. Section 7 & 10.1: change “dl-OrJoint-TCIStateList” to “dl-OrJointTCI-StateList”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7, 7.7.1, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6	Link recovery procedures

< Unchanged parts are omitted >

If a UE is provided dl-OrJointTCI-StateList or TCI-UL-State indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE

-	if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and the PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

< Unchanged parts are omitted >

7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State and for an indicated TCI-State or TCI-UL-State as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS

-	in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, and 11.1.1.

7.7.1	Type 1 PH report

< Unchanged parts are omitted >

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State.

< Unchanged parts are omitted >

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

For a CORESET with index 0,

-	if the UE is provided TCI-State and if followUnifiedTCI-State = 'enabled' for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214]

-	else, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command, μ is the SCS configuration for the PUCCH in the slot when the activation command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided.

If a UE is provided TCI-State in dl-OrJointTCI-StateList, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State [6, TS 38.214].

If a UE is provided followUnifiedTCI-State for a CORESET, other than a CORESET with index 0, associated at least with CSS sets other than Type3-PDCCH CSS sets, and if followUnifiedTCI-State is set as enabled, a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State.

< Unchanged parts are omitted >