3GPP TSG RAN WG1 Meeting #112	R1-2302147

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 36.211 | CR |  | rev | 0 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Draft CR on correction of UE capability parameter name in 36.211 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Moderator (MediaTek), Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2023-Mar-01 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | The parameter name used for the UE capability related to use of segmented precompensation is not aligned between TS 36.331, where it is “ntn-SegmentedPrecompensationGaps-r17”, and TS 36.211, where it is “ue-CE-NeedSegmentedPrecompensationGaps” for eMTC in sections 5.3.4, 5.4.3 and “ue-NBIOT-NeedSegmentedPrecompensationGaps” for NB-IoT in section 10.1.3.6. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The parameter name is changed to “ntn-SegmentedPrecompensationGaps-r17” in sections 5.3.4, 5.4.3 and 10.1.3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 36.211 and TS 36.331 parameter names may cause confusion.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.3.4 Mapping to physical resources

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ntn-SegmentedPrecompensationGaps-r17, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is configured by higher layers based on the UE capability, if signalled.

#### 5.4.3 Mapping to physical resources

For BL/CE UEs communicating over NTN, for PUCCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the single UE capability ntn-SegmentedPrecompensationGaps-r17, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is configured by higher layers based on the UE capability, if signalled.

#### 10.1.3.6 Mapping to physical resources

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 , as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled.