3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302369

E-meeting, 17-26 April, 2023

Agenda Item:	9.10.2

Source:	Huawei, HiSilicon

Title:	Timing advance management to reduce latency

Document for:	Discussion and Decision

Introduction

In this paper, we share our views on the timing advance (TA) management in LTM. The rest of this paper are structured as follows. Potential TA acquisition mechanisms before cell switch command are discussed in section 2. TA indication methods are discussed in section 3. Options of TA association are discussed in section 4. Finally, conclusions are made in section 5.

TA acquisition mechanisms

To acquire the TA of candidate cell before cell switch command is received, candidate solutions are categorized as: RACH-based solutions and RACH-less solutions. Both solutions are still under discussion and we will share our further views on details of both solutions in this section.

On RACH-based solution

During RAN1#112 meeting, the PDCCH ordered RACH for TA acquisition was agreed. Several issues were still FFS including RAR related issue as following:

Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement On whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

In legacy RACH procedure, after sending preamble, UE will monitor the PDCCH to receive the RAR based on current specified procedures. If no RAR is received, UE may retransmit the preamble according the configuration. In LTM when RAR reception is not configured (without RAR), UE cannot determine whether Msg1 transmission is success based on RAR reception. Autonomous re-transmission of PRACH by UE cause waste of transmission power and UL resources.

Proposal 1: When reception of RAR is not configured, UE autonomous re-transmission of PRACH should not be allowed (Alt 1).

However, it is still possible that the first preamble transmission failed. To successfully acquire TA, Network can trigger UE to transmit the preamble again. In detail, serving cell could know whether candidate cell detected the preamble correctly, based on inter-cell interface. This could be achieved easily for intra-DU case while some high layer signalling may be needed for inter-DU case. Then the serving cell can use PDCCH to order a RACH Msg1 transmission again towards to the same candidate cell. UE can apply power ramping according to legacy procedure if the PDCCH-order indicate the following is the retransmission of PRACH.

Proposal 2: The re-transmission of PRACH to candidate cell should be indicated explicitly in PDCCH order to help UE determine whether power ramping should be applied.

If reception of RAR is configured, whether RAR is received from serving cell or candidate cell is still FFS. In mTA mTRP (intra DU) which is under discussion under AI 9.1.1.2, it was concluded not to introduce additional type 1 CSS configuration per additional PCI. In other words, UE will receive RAR for candidate cell from serving cell in mTA mTRP scenario. In mTA CA, according to TS 38.213, RAR for the SCell associate to a TAG different from PCell is scheduled by PDCCH in PCell and is transmitted in PCell as well. In Rel-18 LTM, when candidate cell is a SCell in mTA CA or a TRP configured with additional PCI in mTA mTRP, the TA acquired before cell switch command can be received from serving cell.

Proposal 3: When the candidate cell is the SCell associate to a TAG different from PCell in mTA CA, or a TRP configured with the additional PCI in mTA mTRP, RAR is received from serving cell.

In Rel-18 LTM, both intra-DU and inter-DU cell switch are considered. In inter-DU case, it may be challenging for source cell to transmit RAR is transmit with existing RAR window, which can start as early as one symbol after the last symbol of the PRACH occasion and last as long as 10ms in licensed band. In inter-DU case, high layer signalling is required to feedback the preamble reception results from candidate cell DU to serving cell DU and this high layer signalling could be carried by F1-AP from candidate cell DU to CU, and then from CU to serving cell DU. These procedures may require a large delay about 10ms.

Thus, at least for the inter-DU scenario, RAR received from candidate cell can be considered if TA/RAR coordination between candidate cell DU and serving cell DU is not feasible from the perspective of higher layer signalling delay.

Proposal 4: At least for the inter-DU scenario, RAR received from candidate cell should be supported.

In RAN1#112, RAN1 agreed to further study the following issues on PDCCH order based PRACH for candidate cell on which there is that is no PUCCH/PUSCH configured for serving cell.

Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

According to TS38.213, the minimum gap between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission should be larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, in which NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1, ∆BWPSwitching is the BWP switch delay defined in 38.133, ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2, and Tswitchuplink switching gap NTx1-Tx2. For a PRACH transmission for candidate cell, additional delay should be considered.

For the inter-frequency case where the RO for candidate cell is configured on the BW there is no UL transmission of serving cell configured, UE may need more time for RF re-tuning after UE received DCI triggered PRACH and switch to target frequency to transmit the preamble. Meanwhile the term ∆BWPSwitching in the formula could be zero, since the RF retuning time may already cover the BWP switching time. The detail value depends on RAN4’s decision.

In addition, for the intra-frequency cell switch case, even if the DL synchronization with candidate cell can be acquired by UE in advance, it may still take time for UE to adjust DL timing from the serving cell to candidate cell before PRACH transmission, especially in the scenario when the RTD exceed the CP length. Hence, based on the formula of minimum gap between PDCCH order and RO in TS 38.213, , a new delay factor related DL synchronization adjustment should be considered, which requires RAN4 input.

Proposal 5: The minimum gap between PDCCH order and the first valid RO for candidate cell should be extended in both intra frequency scenario (RTD exceed CP length) and inter frequency scenario (no serving cell UL configured). The detail values depend on RAN4’s decision.

For UE with limited number of RF chains, if the RO for candidate is in the different BW from those for the UL in serving cell, the UL interruption on the serving cell should be expected when UE is triggered to transmit PRACH for candidate cell. In addition to PRACH transmission itself, the RF retuning time before and after the transmission should also be counted. As the requirement of existing mechanism is in RAN4’s spec, the issue related to LTM should also be discussed in RAN4.

Observation 1: The interruption on UL TX of serving CCs due to PRACH Tx for candidate cell should be discussed in RAN4.

On RACH-less solutions

In last meeting, there was a working assumption on the UE based TA measurement solution:

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

When UE performing the L1 measurement in LTM, the reference signal timing difference (RSTD) may be derived by the UE based on the measurement on SSBs of serving cell and candidate cell. UE may determine the TA of candidate cell from the serving cell TA and RSTD, and apply to the UL transmission after cell switch directly. There are three sources contributing to TA estimation error for the candidate cell. Firstly, the UE may have DL synchronization errors for both serving cell and candidate cells. In addition, the RSTD defined in positioning is based on PRS which have larger bandwidth to provide better estimation accuracy compare to SSB. Secondly, the estimation of DL Tx timing between serving cell and candidate cells may be difficult. It can be as long as 3 us according to RAN4 spec even for synchronous deployment. It is usually difficult to measure such error by gNB itself. Thirdly, the TA for the candidate cell is derived from the serving cell TA, which has estimation error itself. Thus, further evaluation is required to see whether the accuracy of TA estimation from RSTD can meet the RAN4 requirement on UL synchronization.

Observation 2: Regarding the UE-based TA measurement mechanism, the accuracy of TA estimation by UE should be evaluated/studied considering the UE derived TA will be used for the potential UL data transmission in the new serving cell after cell switch.

TA indication

There are some other scenarios also considering LTM, i.e. mTA CA/mTRP. As agreed in RAN2, the candidate cell may be activated SCell of the serving cell. In case the SCell belongs to another TAG in a cell group, then the TA value is already acquired by UE by TA management in the serving cell. Similarly, in the scenario of mTA mTRP, the TA of TRP is configured as a separate TAG and TA is already indicated to UE. Thus, the TA indication in cell switch command is not necessary anymore.

Observation 3: In mTA CA/mTRP scenarios, if UE maintained TA for 2nd TAG associated to candidate/target cell, the separate TA indication for LTM (in RAR or cell switch command) is not necessary.

TA association

Regarding the TA association, there were two candidate solutions listed for further study.

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

In Alt 1, TA/TAG is associated with candidate cell implicitly. In the sub-bullet, one example is given that TA/TAG may associate to TCI states. In current release, TCI state is configured per CC/BWP. As the TA acquired for a candidate cell may be an inter-frequency cell, so the solution does not work in such scenario.

In Alt 2, the association between TA/TAG and candidate cell is explicitly via SSB(s)/TRS(s), which can support the inter-frequency cases. Furthermore, the TA acquired before the cell switch command is used by UE for UL transmission timing after cell switch. The source reference signal for UL TCI state and even Pathloss RS used in UL transmission are usually bonded with SSB(s)/TRS(s). Such design is straightforward and can be applied for both FR1 and FR2. Hence, we propose to support Alt 2 in agreements in RAN1#110b-e, i.e. associate TA/TAG and candidate cell explicitly for TA association in Rel-18 LTM.

Furthermore, since RAN2 has agreed in RAN2#119-bis that the candidate cell can be current SCell:

For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

When candidate cell is an activated SCell, the TA/TAG is configured for that SCell as 2nd TAG in this cell group. In this case, UE can acquire the TA of 2nd TAG via PRACH and the RO is associate with SSB for this SCell. It means that if the candidate cell is an activated SCell, TA/TAG is already associate to the candidate cell explicitly by TAG configuration. We believe that TA association design should coexistence with the case that activated SCell is the candidate cell. Thus, we support to associate TA/TAG and candidate cell explicitly.

Proposal 6: Support associating TA/TAG explicitly with candidate cell(s) (Alt 2).

In the mobility scenarios, it is possible that UE may have multiple candidate cells. To reduce the latency for UL synchronization, at least one TA associated with one of the candidate cells should be acquired. The gNB can make the decision on which one of the candidate cells UE should acquire the TA. Regarding the maximum number of TAs a UE can acquire for candidate cells before cell switch command, it could be based on UE capability.

RAN2 has following agreement in RAN2#119e:

R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell.

If the UE support mTA CA, then the UE may be configured with two TAGs after cell switch. For the case that UE operates with mTA CA and performs LTM, it is reasonable to support UE acquires two TAs for the two CCs after cell switch.

Proposal 7: For LTM, UE can at least acquire one TA for a candidate cell before cell switch. The maximum number of TAs UE can acquire and maintain before cell switch could be based on UE capability.

In addition, mTA mTRP is under discussion parallel in Rel-18 MIMO, it is also possible that UE may configure with two TAGs for TRPs before/after cell switch. For the case that UE operates with mTA mTRP and performs LTM, it is reasonable to support UE acquires two TAs for the two TRPs after cell switch.

Proposal 8: Support two acquired TAs associated with two TRPs of one candidate cell.

Summary and conclusion

In this contribution, we observe and propose the following:

Observation 1: The interruption on UL TX of serving CCs due to PRACH Tx for candidate cell should be discussed in RAN4.

Observation 2: Regarding the UE-based TA measurement mechanism, the accuracy of TA estimation by UE should be evaluated/studied considering the UE derived TA will be used for the potential UL data transmission in the new serving cell after cell switch.

Observation 3: In mTA CA/mTRP scenarios, if UE maintained TA for 2nd TAG associated to candidate/target cell, the separate TA indication for LTM (in RAR or cell switch command) is not necessary.

Proposal 1: When reception of RAR is not configured, UE autonomous re-transmission of PRACH should not be allowed (Alt 1).

Proposal 2: The re-transmission of PRACH to candidate cell should be indicated explicitly in PDCCH order to help UE determine whether power ramping should be applied.

Proposal 3: When the candidate cell is the SCell associate to a TAG different from PCell in mTA CA, or a TRP configured with the additional PCI in mTA mTRP, RAR is received from serving cell.

Proposal 4: At least for the inter-DU scenario, RAR received from candidate cell should be supported.

Proposal 5: The minimum gap between PDCCH order and the first valid RO for candidate cell should be extended in both intra frequency scenario (RTD exceed CP length) and inter frequency scenario (no serving cell UL configured). The detail values depend on RAN4’s decision.

Proposal 6: Support associating TA/TAG explicitly with candidate cell(s) (Alt 2).

Proposal 7: For LTM, UE can at least acquire one TA for a candidate cell before cell switch. The maximum number of TAs UE can acquire and maintain before cell switch could be based on UE capability.

Proposal 8: Support two acquired TAs associated with two TRPs of one candidate cell.