3GPP TSG RAN WG1 #112bis-e						         R1-2302486

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Channel access mechanism for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.1

Document for:	Discussion and Decision

- Introduction

The revised WID for NR sidelink evolution has been approved in RAN#94e [1]. The channel access mechanisms and the physical channel design framework for sidelink on unlicensed spectrum were agreed for further investigation:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss the channel access mechanisms for sidelink on unlicensed spectrum.

- LBE

Load based Equipment (LBE) is a very flexible scheme that enable devices to access the channel fairly with different priorities and different MCOTs. It is widely adopted in many regions in sub-7GHz as channel access mechanism. LBE performs LBT with backoff mechanism, i.e., Type 1 channel access procedure. Device can transmit whenever the channel is sensed as idle. When the channel is sensed as busy during the LBT procedure, device can continuously perform CCA until the selected backoff counter reaches zero. LBE has no fixed frame structure. The channel occupancy time depends on the channel access priority class. In the following sections, the remaining details of LBE when operating in SL-U are discussed.

  - LBT procedure

109 Agreement Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel. - FFS conditions for the actual channel access type(s) used for each SL channel and signal transmitted, and based on COT sharing conditions (if supported) - FFS whether UL CAPC or DL CAPC or both should be used as the baseline, - FFS how the channel access priority classes apply to each SL channel and signal - FFS sidelink priority levels (PQI or L1 priority), channel and signal mapping to the 4 channel access priority classes. The discussion may involve other WGs. 110 Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs - CW adjustment - NR-U DL CW adjustment mechanism is used as the baseline for SL-U when SL-HARQ feedback is enabled in SCI for unicast - FFS any necessary update for SL-U operation - FFS: how to determine CW size when SL-HARQ feedback is disabled in SCI - FFS the case of groupcast option 1 (NACK-only) and groupcast option 2 110bis Agreement - Type 1 SL channel access procedure is applicable to the following transmissions by a UE: - PSSCH/PSCCH transmission(s) scheduled or configured by a gNB in SL Mode 1 resource allocation. - PSSCH/PSCCH transmission(s) from the UE in SL Mode 2 resource allocation. - Other SL transmissions including S-SSB and PSFCH transmissions from a UE - FFS: how to set CAPC for S-SSB and PSFCH - Note: Type 1 can be used to initiate a COT - A UE uses a channel access priority class applicable to the sidelink user plane data multiplexed in PSSCH for performing the Type 1 channel access procedures to transmit transmission(s) including PSSCH with user plane data and its associated PSCCH. - Note: how to set CAPC for MAC CE multiplexed in PSSCH is up to RAN2 - A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds the maximum COT duration where the channel access procedures are performed based on a channel access priority class p associated with the UE transmissions, as given in CAPC table for SL. 110bis Agreement - RAN1 is to study the definition of a “SL reference duration” following the NR-U principle and RAN1 is to agree on the definition before down-selection to an option for CW adjustment for SL HARQ-ACK feedback enabled/disabled and each cast type - In Type 1 SL channel access procedure, further study the following cases and options. Other options are not precluded. - CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled): - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value. - Option 5: If a collision indicator is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - FFS: the (pre-)configuration ratio values - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p; otherwise CWp is increased. - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: CWp is reset to CWmin,p for every priority class p∈1,2,3,4. - Option B: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p, otherwise CWp is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then CWp=CWmin,p, otherwise CWp is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p ; otherwise CWp is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration). 111 Agreement Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure 112 Agreement Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

It has been agreed that when Type 1 channel access is applied to PSFCH, p=1 is used for PSFCH. To further enhance channel access probability for important signaling with limited transmission time, Type 2A channel access procedure can be considered. For example, for PSFCH transmission, which contains only one bit HARQ feedback information and with only 2 symbols, Type 2A channel access can be considered to enhance the channel access possibility. When Type 2A is also supported for PSFCH, the duty cycle, the total transmission duration, and the maximum number of transmissions within the observation period should be jointly considered with S-SSB. For those S-SSBs and/or PSFCHs which do not meet the regulation requirement, Type 1 LBT should be applied. Therefore, we need to define some criteria to select the LBT type for S-SSB and PSFCH. Followings are some of the potential candidates:

Alt 1: only the S-SSB and/or PSFCH in certain candidate positions can use Type 2A LBT for transmission

Alt 2: only the S-SSB and/or PSFCH with certain SCS can use Type 2A LBT for transmission

Alt 3: introducing separate limitation for S-SSB and PSFCH respectively (e.g, no more than N1 or X1% of the S-SSB and/or no more than N2 or X2% of the PFSCH can use Type 2A LBT for transmission)

Alt 4: only the S-SSB and/or PSFCH with certain priority can use Type 2A LBT for transmission

The observation period of the above transmissions can be the same as that in ETSI regulation, i.e., 50ms. However, to adapt more flexibly to the SL transmission, the observation period can be defined as other values, e.g., the periodicity of the S-SSB, 160ms.

As agreed in RAN1#110 meeting, Type 2A/2B/2C channel access can be considered when sharing a COT from another UE for transmission. When the gap between the SL transmission from responding UE and that from the COT initiating UE is 16 us, both Type 2B and Type 2C channel access can be applied. The choosing of the channel access type depends on the SL transmission duration of the responding UE. If the SL transmission duration is larger than 584us, Type 2B channel access must be applied. If the SL transmission duration is less than or equal to 584us, Type 2C or Type 2B channel access can be applied.

Proposal 1: Support Type 2A channel access for PSFCH transmission.

Proposal 2: The criterion of applying LBT type-2 for S-SSB and PSFCH should be determined.

Proposal 3: The UE chooses to apply Type 2B or Type 2C channel access depending on the SL transmission duration.

Regarding the reference duration, option 1a has been agreed while the CWS for groupcast option 1 and MCSt is still FFS. The reference duration for multi-consecutive slots transmission is not different than that for single slot transmission. The ending time for the reference duration of MCSt can be the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted. The rest of the transmissions should not be included in the reference duration. The other transmissions such as PSFCH and S-SSB, if they fall into the reference duration, cannot be used for CWS adjustment. When the transmissions within the COT are only groupcast option 1 NACK-only transmission and/or transmissions with SL-HARQ feedback disabled, reference duration is not applicable.

Proposal 4: Reference duration is not defined when the COT contains only groupcast option1 transmission and/or transmissions with SL-HARQ feedback disabled.

Proposal 5: The ending time of reference duration for MCSt is the same as that for single slot transmission.

Proposal 6: The PSFCH or S-SSB within the reference duration cannot be used for CWS adjustment.

By applying the above reference duration definition, when all transmissions within the latest COT have SL-HARQ feedback disabled, it is preferable to retain the CWS, which is the option 1 from the agreement. Transmissions with SL-HARQ feedback disabled cannot be effectively used for CWS adjustment, and blind retransmission does not necessarily imply a transmission failure, therefore, it is better to retain the CWS.

Groupcast option 1 should also be supported for SLU, which is not fundamentally different from other SL transmissions except for the feedback. Since there is no reference duration defined when all transmissions within the latest COT are groupcast option 1 NACK-only, the CWS should not be adjusted due to the lack of effective feedback.  Therefore, it is better to use the latest CWp, i.e., the option 1 from the agreement.

When all the transmissions within the latest SL reference duration include only groupcast option 2 PSSCH, option 1 from the agreement is preferred. UE can adjust the CWS based on a (pre-)configurable ratio of the received SL HARQ-ACK feedbacks in the latest SL reference duration. The ratio can be defined as the ratio of received ACK to all received feedbacks, and the threshold of the ratio can be (pre-)configured.

When the reference duration includes different cast types of transmissions, UE can determine the CWS based on the effectiveness of the feedback. For example, the effectiveness of the feedback for SL transmissions can be ordered as: unicast PSSCH with SL-HARQ feedback enabled > groupcast option 2 PSSCH > other transmissions. If the reference duration includes more than one type of the SL transmission, UE always uses the SL transmission with feedback of the highest effectiveness to adjust the CWS.

Proposal 7: When all the transmissions within the COT are PSSCH with HARQ feedback disabled or PSSCH of groupcast option 1, UE uses the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

Proposal 8: When all the transmissions within the COT are groupcast option 2, UE adjusts the CWS based on the ratio of the received ACK to all the received feedbacks.

Proposal 9: When the transmissions within the reference duration includes different cast types, UE adjusts the CWS based on the transmission with ACK/NACK HARQ-ACK enabled, where the unicast transmission are treated with the highest priority.

  - Type 1 LBT blocking

Proposal 6 (I): - To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail. Further study at least the following solution options for down-selection in a future meeting. Other options are not precluded. - Option 1: - UE avoid selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the CAT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. - Option 4: LBT duration is determined firstly, then resource selection takes into account of the LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection.

In RAN1#112 meeting, type 1 LBT blocking issue was discussed and several solutions have been proposed as in [3] and cited above.

Option 1:

- Enhancement 1: UE avoids selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource.

- Enhancement 2: UE avoids selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.

For option 1, both of the enhancements are beneficial to avoid type 1 LBT failure. However several essential issues should be addressed when introducing the enhancements. For the first enhancement, the method to acquire the type 1 LBT duration of the reserved resource should be specified, e.g., via a predefined type 1 LBT duration, or indicated by SCI, etc. Furthermore, for both of the enhancements, the method to determine LBT blocking between the reserved resource and a selected resource should be discussed, e.g., when the energy measurement result associated with the reserved resource is above a energy threshold, the reserved resource may block a later selected resource.

Proposal 10: To resolve the Type 1 LBT blocking issue, the following solution is supported:
- UE avoids selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource.
- UE avoids selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.
- FFS how to acquire the type 1 LBT duration of the reserved resource.
- FFS how to determine LBT blocking between the reserved resource and a selected resource.

Option 2:

- Enhancement 1: UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource).

- Enhancement 2: UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the COT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource).

For option 2, the enhancemetns work only if the source/destination ID and CAPC for the transmissions on the reserved resource and the selected resource meet the COT sharing condition. However, the UE can know the source/destination ID and CAPC for the transmission on the reserved resource only when retransmission is performed on the reserved resource. For now, how to map the initial transmission and retransmission on the reserved resource have not been discussed for SLU, so the option 2 should be discussed later.

For the other options cited above (i.e., option 3/4/5), it seems that the options are not straightforward solutions to address type 1 LBT blocking issue. Hence, option 3/4/5 are not preferred to be discussed in this topic.

Besides the option 1 and option 2, another straightforward option is that, when SL UE detects another SL UE’s transmission in type 1 LBT procedure, the UE can still deem channel free. More specifically, a UE can obtain the RSRP of SL transmission based on the existing mode-2 sensing and RSRP measurmenet procedure, according to the received SCI format 1-A. Then, when the UE performs LBT sensing in slot n, the energy detection result can be derived by the total received energy (e.g., RSSI) excluding the energy contributed by SL transmission (e.g., deduced from SL RSRP) in the slot, and then is compared with the EDT for LBT CCA checking. For example, if the EDT is -72 dBm, the sensed total power is -70 dBm, while the detected SL transmission is -71 dBm, the UE can still considered the sensing slot is idle because the remaining power is (-70 dBm) – (-71 dBm) = -76 dBm which does not exceed the EDT.

Some preliminary evaluations are performed to show effectiveness of the above solution to address the type 1 LBT blocking issue. In the simulation, SL UE deems LBT failure only if the UE detects WIFI transmission during the LBT duration. From Figure 1, it can be observed that, when the solution is used, the performance of LBE based channel access increase significantly.

Figure 1 System performance for enhanced LBT

Proposal 11: To resolve the Type 1 LBT blocking issue, SL UE deems channel busy only if the UE detects transmission other than SL transmission occupying the channel (e.g., exceeding the energy detection threshold), i.e., the energy detection for EDT checking in LBT procedure does not take into account the energy from SL transmissions.

  - Mode 2 resource allocation enhancement

109 Agreement - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2 Proposal 7-1 (II): To address the potential issue of insufficient time for a UE to perform Type 1 LBT before a selected resource due to LBT sensing time can be longer than T1 of resource selection window, further study at least the following options for down-selection in a future meeting. Other options are not precluded. - Option 1: A time offset is added to T1 of resource selection window. FFS the time offset length. - Option 2: MAC layer takes into account of a potential Type 1 LBT sensing duration and selects resources accordingly. - Option 3: Type 1 LBT sensing duration is determined firstly, then resource selection takes into account of the LBT duration is performed. - Option 4: Resource is selected firstly, then Type 1 LBT sensing duration is adjusted based on timing of the selected resource. - Option 5: Do nothing; Drop the SL transmission due to LBT failure, perform resource re-selection and attempt to access the channel for the next selected resource. - Option 6: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection. - Option 7: UE prioritizes resource in a detected shared COT.

In RAN1#109 e-meeting, it has been agreed that existing SL mode 2 resource allocation are supported as baseline for sidelink operation in a shared carrier, and the enhancements due to shared spectrum channel access should be further studied. In RAN1#112 meeting, several enhanced aspects were discussed as in [3] and cited above.

The SL resource allocation should be enhanced considering the potential LBT failure. There are several reasons for which the LBT failure may occur, e.g.,

- There is no sufficient absolute time for the UE to finish the LBT procedure before the transmission resource.

- There is a time gap between the end of LBT procedure and the start of the SL transmission resource

Regarding the insufficient type 1 LBT duration, the mode 2 resource selection can be enhanced to guarantee the time gap between slot n (where resource selection is triggered) and the slot of transmission resource, or between any two transmission resources, is larger than the minimal/remaining LBT duration for the UE, which can be determined based on, e.g., the value of the LBT counter.

Another solution to cope with type 1 LBT duration is to use COT sharing mechanism, then UE use type 2 LBT instead of type 1 LBT. In order to fully and quickly use the shared COT, the enhancement of resource selection in shared COT needs further study. For example, the resource selection window should be selected within the shared COT. Or, the RSRP measurement requirement should be relaxed for the resources within the shared COT compared with that of the legacy resource exclusion procedure. As a result, the UE can have a higher possibility to select the resources in the shared COT.

Proposal 12: The mode 2 resource selection should be enhanced to guarantee sufficient LBT duration before the SL transmission resource(s).

Proposal 13: The enhancement of resource selection mechanism in shared COT needs further study. E.g., the resource selection window is selected within the shared COT or the RSRP measurement is relaxed for the resource within the shared COT.

Regarding the gap between the end of LBT procedure and the start of the SL transmission resource, UE may either fill the gap by dummy (or valid) transmission or delay the LBT procedure until the start of the transmission resource. Considering the resource efficiency, it is preferred to delay the LBT procedure, i.e., apply a 25us deferred LBT before the SL transmission resource, similarly as that in NR-U. Then, SL UE can still perform transmission on the resource if the deferred LBT succeed. However, the deferred LBT may fail as well, then UE has to hold on the TB transmission until next selected/reserved PSCCH/PSSCH.

Proposal 14: When UE detects the gap between the end of LBT procedure and the start of the SL transmission resource, the UE apply a 25us deferred LBT before the SL transmission resource.

To increase the TB transmission reliability, it can be assumed that the transmission resource is selected as early as possible to approach the end of the LBT procedure, so that if the deferred LBT failed, the UE has sufficient time to continue another LBT procedure to transmit the TB within its PDB.

Proposal 15: For mode 2, the transmission resource should be selected as early as possible to approach the end of the LBT procedure.

  - Mode 1 resource allocation enhancement

109 Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2 Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

The first aspect that needs to be discussed is whether the LBT type as well as the priority class is indicated by the gNB or determined by the scheduled UE. In R16 SL, gNB has no information on the priority of data transmitted in the scheduled resources, and UE determines the data mapped on the resource based on the LCP configuration. Similarly, if it is up to the UE to select the data to transmit as well as the LBT mechanism to use in SL-U, the UE should at least ensure that the COT obtained after access is long enough to contain the scheduled resource. Further, in addition to the scheduled resources, the COT may contain additional time resources. It should be first clarified if these additional time resource can be used by the mode-1 UE for transmission or sharing with peer UE.

If this is not allowed, the consumed LBT effort and obtained transmission opportunity are somehow wasted. If it is allowed, it seems that this may be somehow not aligned with the behavior of traditional mode 1, as the UE uses resources not explicitly scheduled by the gNB. Further, in this case, the exact frequency resources used for transmission on the additional time occasions should be determined. The simplest way would be to use the same frequency resources as the ones being scheduled for further transmission or sharing.

Proposal 16: For mode-1 UE, LBT type for the scheduled resources/configured grant as well as the priority class is decided by UE.

Proposal 17: For mode-1 UE, if the obtained COT includes additional time resources other than the scheduled resources/configured grant, the UE is allowed to exploit these resources or to share them with its peer UE(s).

If there is strong interference in the vicinity of the scheduled UE, the UE may fail to access the scheduled resources. In order to increase the possibility of channel access, gNB may schedule a same set of resources for more than one UE. UEs with less severe interference have a higher possibility to access the resources, and they can share the obtained COT with other UEs in the group.

Proposal 18: For mode-1 UE, gNB can indicate a set of resources to a group of UE by a group common DCI or RRC, UEs in the group can perform LBT for the scheduled resources/configured grant and possibly share the COT with other UEs if LBT succeeds.

A mode-1 UE can only access the scheduled resources and perform transmission successfully once LBT succeeds (i.e., the measured signal strength has been lower than the EDT for a certain period), so the UE is unlikely to suffer strong interference from other UEs when transmitting on the scheduled resources. If the UE reports a NACK after transmission, this is more likely due to a TX/RX cancellation, or PSSCH decoding failure of the peer UE. Thus, the UE can report the LBT result to help gNB to better understand the status of the SL scheduling. A straightforward approach is that if the LBT fails, a NACK is reported. However, a disadvantage of this approach is that gNB may misinterpret the NACK as an outcome of de-prioritization of PSSCH transmission/PSFCH reception, or PSSCH decoding failure of the peer UE, which then leads to inappropriate or ineffective retransmission scheduling. Moreover, when the NACK-only based HARQ-ACK scheme is used, if the peer UE fails to access the PSFCH, according to the existing spec, the scheduled UE will report ACK as no PSFCH is detected.

An alternative solution is to indicate LBT result and SL HARQ-ACK separately. For example, a dedicated resource or separate codebook is used for LBT result indication in addition to SL HARQ-ACK reporting. This also helps the gNB to determine exactly how many times a TB has been sent so as not to exceed the maximum number of (re)transmissions maintained by the gNB for DG or configured for CG to UE.

In addition to LBT results, COT-related information can also be reported to gNB. Due to the uncertainty of LBT and that LBT type and CAPC are determined by UE itself, gNB does not know where the COT starts/ends. With the COT information from UE, gNB can schedule other UE dynamically to share the COT, thus save LBT effort.

Proposal 19: For mode-1 UE, separate reporting of LBT results/COT information and HARQ-ACK for the scheduled resources are supported.

In R16, the time location of PUCCH for SL HARQ-ACK is determined based on the PSFCH corresponding to the last scheduled resource by a SL DCI. In R18, due to the uncertainty of channel access, multiple PSFCH candidates should be supported for a PSSCH/PSCCH transmission as discussed in [2]. The time location of UL resources for reporting can be determined in the following two ways:

Option1. For each PSFCH candidate, there is one candidate UL resource. The candidate UL resource associated with the successfully accessed PSFCH candidate, or the last PSFCH candidate if all LBT fails, is used for reporting.

Option2. There is one UL resource for reporting, and its time location is derived from the last PSFCH candidate.

Proposal 20: For a mode-1 grant, the time location of the corresponding UL resource for SL HARQ-ACK or LBT results/COT information reporting is determined based on one of the following options:
- Option1. For each PSFCH candidate, there is one associated candidate UL resource. The candidate UL resource associated with the successfully accessed PSFCH candidate, or the candidate UL resource associated with the last PSFCH candidate if all LBT fails, is used for reporting.
- Option2. There is one UL resource for reporting, and its time location is derived from the last PSFCH candidate.

  - COT sharing mechanism

109 Agreement - UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U). - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access and any restrictions (e.g. whether the COT can be shared with a single UE or multiple UEs) - FFS all other details in compliance with the regulatory requirements 110 Agreement - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiver - FFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s) - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA 111 Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE. 112 Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated 112 Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. 112 Agreement For UE-to-UE COT sharing, When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. 112 Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

According to the agreements in previous meetings, there are still some SL-U COT sharing issues need to be addressed.

- FFS: details of the channel type of the COT initiating UE’s transmission

When a UE initiates a COT and transmits at least PSCCH/PSSCH, it has been agreed that the UE can share the COT to its responding UE(s). It was also agreed that a responding UE is a UE with a PSCCH/PSSCH transmission containing a destination ID or source ID that matches a ID from the COT initiator UE. Additionally, the responding UE can use the shared COT to transmit S-SSB. However, for the case where an initiator UE initialized a COT only for S-SSB or PSFCH transmission and has no companion PSCCH transmission containing any UE ID in the COT, it it should be clarified whether another UE still can share the COT.

It was agreed that the CAPC priority for S-SSB is fixed as 1, when a UE launched Type-1 channel access for S-SSB transmission only, the MCOT length of COT is fixed as 2ms, which consists of 1/2/4 slots for 15/30/60 kHz respectively. If another UE is not allowed to share the S-SSB-only COT, then the UE has to perform Type-1 channel access for the following S-SSB occasion(s). As shown in Figure 2, two sync resources are configured on two consecutive slots. When the SCS is set to 30/60 kHz, the GP symbol of the preceding S-SSB is too short to accommodate Type 1 channel access, which requires at least 43μs. If the LBT starts earlier, as indicated by the blue line, the initiating UE will need to drop the end of its S-SSB transmission. Alternatively, if the LBT starts later, as shown by the red line, there is no sufficient time to complete Type-1 LBT unless the AGC of the subsequent S-SSB transmission is dropped. Overall, either approach results in S-SSB performance degradation.

Figure 2 Example of Type-1 LBT for two consecutive S-SSB occasions

One may argue that UE should directly perform Type-2A for S-SSB without a shared channel occupancy in this case. However, it is worth noting that to perform Type-2A LBT without a shared channel occupancy, the duty cycle must not exceed 1/20, allowing a very limited number of transmission opportunities (i.e., a maximum of 8ms transmissions within a 160ms period, which are shared among 3 sync resources). If Type-2A LBT without a shared channel occupancy can also be applied directly to PSFCH, the number of S-SSB transmission opportunities will be further reduced, potentially degrading S-SSB coverage. On the other hand, S-SSB is a broadcast channel, any UE that detects S-SSB and performs the following S-SSB transmission can also be considered as a responding UE to the transmitting UE,  thus COT sharing for S-SSB only case should be supported. More specifically, for a UE (e.g., UE2 using sync resource2 or UE3 using sync resource1), if it didn’t detect any SCI indicating a COT overlapped with sync resource(s) spanning a set of consecutive slots, but detected S-SSB(s) over one or more consecutive slots, it can assume that the channel is accessed by some UEs(e.g., UE1) based on Type-1 channel access for S-SSB transmission purposes only, and the COT lasts 2 ms and starts from the first detected S-SSB among the set of slots. Thereby, UE2/UE3 can share the COT for S-SSB transmission if the COT covers the its S-SSB transmission occasion.

Figure 3 Example of S-SSB only COT sharing

As an example shown above in Figure 3, the sync resources for S-SSB transmissions for UE1/UE3 and UE2 are sync resource1 and sync resource2 respectively. Assuming that UE1 only had S-SSB to transmit and performed Type-1 LBT to access sync resource1, UE2 and UE3 did not detect any SCI indicating a COT which overlaps with sync resource1. Sync resource2 is right after sync resource1. In case1, UE3 did not access S-SSB1 in sync resource1 but it detected S-SSB1 on sync resource1, while UE2 also detected S-SSB1, thus UE2 and UE3 can assume that some UE has obtained the COT for S-SSB transmission only, and the COT starting from S-SSB1. As the COT is also overlapped with S-SSB2 in sync resource1 and S-SSBs in sync resource2, UE2/UE3 can perform Type-2 LBT (e.g, Type-2A/B/C for 15kHz, Type-2B/2C for 30kHz/60kHz) and transmit S-SSB in the COT if Type-2 LBT succeed. In case2, UE1/3 did not access S-SSB1 in sync resource1 but UE1 accessed S-SSB2, while UE2 also detected S-SSB2, thus UE2 can assume that some UE has obtained the COT for S-SSB transmission only, and the COT starting from S-SSB2. As the COT is also overlapped with sync resource2, UE2 can perform Type-2 LBT (e.g, Type-2A/B/C for 15kHz, Type-2B/2C for 30kHz/60kHz) and transmit S-SSB in the COT if Type-2 LBT succeed.

Additionally, if a UE successfully accesses the channel for S-SSB transmission purposes only, it should set the reserved bits in the MasterInformationBlockSidelink to indicate that this COT is for S-SSB only cases and/or COT information such as the starting of the COT. It is worth noting that the S-SSB only COT sharing occurs only when all UEs using the same sync resource have no data to transmit. If one of them has companion data transmission, the CAPC priority will be determined based on the data instead, which is equal to or lower than S-SSB As a result, the correspondingLBT requires longer detection period and may block S-SSB only UE. Therefore, the MasterInformationBlockSidelink transmitted in the SFN manner should carry the same information.

MasterInformationBlockSidelink ::=           SEQUENCE {

sl-TDD-Config-r16                            BIT STRING (SIZE (12)),

inCoverage-r16                               BOOLEAN,

directFrameNumber-r16                        BIT STRING (SIZE (10)),

slotIndex-r16                                BIT STRING (SIZE (7)),

reservedBits-r16                             BIT STRING (SIZE (2))

}

Proposal 21: For a set of consecutive slots configured for R16 S-SSB occasion, if UE intends to transmit S-SSB on one or multiple of the slot, and it detects no SCI indicating a shared COT that is overlapped with the one or multiple slots but detects S-SSB on at least one slot in the set of consecutive slots.
- It assumes that there is a 2ms shared COT starting from the first detected S-SSB
- If the assumed COT is overlapped with its occasion for S-SSB transmission, it can share the COT to transmit S-SSB.

Proposal 22: When a UE only has S-SSB to transmit and it accesses the R16 S-SSB occasion with Type-1 LBT successfully, the reserved bits in MasterInformationBlockSidelink is used to indicate the shared COT information, e.g., whether the COT can be shared for S-SSB, and/or the start of the COT.

- FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator

In NR-U, the gNB is allowed to transmit control/broadcast signals/channels for any UEs in a UE-initiated COT as long as the transmission contains transmissions for the UE that initiated the channel occupancy and/or DL signals/channels (PDSCH, PDCCH, reference signals) meant for the UE that initiated the channel occupancy. Same COT sharing principle should be applied in SL-U. In order to save the PSFCH overhead, a responding UE should be allowed to transmit PSFCH(s) to UE(s) other than the COT initiator if the responding UE also transmits PSFCH(s) to the initiating UE simultaneously.

Proposal 23: A responding UE can transmit PSFCH(s) to UE(s) other than the COT initiator if the responding UE also transmits PSFCH(s) to the initiating UE simultaneously.

- FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

  - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE.

It has been agreed that a responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when the ID is matched for unicast or groupcast. Whether the additional ID is supported is not converged. If additional ID in the COT sharing information is supported, only the source/destination ID related to an initiating UE is possible to be included. For example, if an initiating UE carries an additional ID of a destination ID for groupcast when transmitting a unicast, the responding UE can transmit groupcast PSCCH/PSSCH to the UE(s) including the COT iniating UE of which the destination ID matches to the additional ID. However, an additional ID with at least 8 bits causes high overhead but achieves limited gain in the scenario where an initiating UE and  a responding UE have different cast types. Therefore, additional ID included in the COT sharing information is not preferred.

Proposal 24: Additional ID included in the COT sharing information is not preferred. The case is not supported if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

- Content in COT information

This issue has been discussed for several meetings, CAPC level and RB set(s) and remaining COT duration, L1 ID are mostly supported. In our view, CAPC level and remaining COT duration should be supported in COT information since it has been agreed that a responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Meanwhile, the L1 ID (source ID, destination ID) should also be included since it has been agreed to match the L1 ID.

Regarding the RB set indication, UE can derive the RB set of the initiating COT by frequency domain resource indication in the SCI. Thus, the RB set indication is not needed additionally. The channel access type is not necessary to be indicated since the channel access type within the shared COT can be determined by the responding UE based on the gap between transmissions of the COT initiating UE and responding UE. The CPE indication is proposed by some companies to resolve the collision problem. However, the problem is that initiating UE may not know all the reserved resource for responding UE(s), so that it cannot adjust the proper CPE value for the responding UEs.

Proposal 25: The CAPC level, remaining COT duration and L1 ID are included in COT information.

Proposal 26: The channel access type within the shared COT is determined by the responding UE based on the gap between transmissions of the COT initiating UE and responding UE.

Proposal 27: The RB set indication and CPE indication in COT information are not necessary.

- COT information container

The COT information is preferred to be contained in SCI than in MAC CE for less latency. In additionally, whether it is in existing 1st stage SCI or 2nd stage SCI, or any other new SCI needs further study. The maximum value of reserved bits for SCI format 1-A is 4 which is not enough to contain the COT information. So, a new 1st SCI format is required for SL-U if COT information is contained in the 1st stage SCI. Noted that it is also beneficial from power saving perspective (eliminating the need of 2nd SCI decoding), which is very important to commercial UE.

Proposal 28: The COT information is preferred to be contained in SCI, and a new 1st SCI format is required for SL-U if COT information is contained in the 1st stage SCI.

- Forwarding/relaying information

According to the regulation, an initiating device is allowed to grant an authorization to one or more associated responding devices to transmit on the current operating channel within the current COT. If UE needs receive  forwarding/relaying information about a COT initiated from another UE, it may go against the regulation since the authorization is not from the initiating device.

Proposal 29: UE forwarding/relaying information about a COT initiated by another UE is not supported.

Last but not least, if more than one COT are identified by a responding UE, the responding UE should determine which COT to share. There are some alternative principles for the UE to select according to the COT sharing information. For example, the shared COT with earliest starting time, the more time-frequency resource of the shared COT, or the lowest CAPC level.

Proposal 30: If more than one COT is identified by a COT sharing UE, the responding UE should determine which COT to share according to the COT sharing information.

Besides, since in SL, the initiating device and responding device are all UEs with similar level of transmit power, there is no need to further enhance the UE-to-UE COT sharing ED threshold as that in NRU UE-to-gNB COT sharing case (i.e., sharing a COT to another node having higher transmit power). If very low power UE is considered, the typical scenario is that the very low power UE shares the COT of another very low power UE or the COT of a standard power UE, but not vice versa. In other words, sharing a COT to another node having higher transmit power is not supported. Thus, the EDT adjustment based on the transmission power of the responding device is unnecessary.

Proposal 31: No enhancement on the UE-to-UE ED threshold is needed.

  - Multi-consecutive slots transmission

Mode 2 resource allocation enhancement for the multi-consecutive slot transmission (MCSt) has been discussed in last RAN1 meeting, and the following agreements have been reached. In this section, the mode 2 enhancement will be further discussed.

RAN1 110b-e Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

Since Multi-consecutive slots transmission (MCSt) is supported for SL-U. Three alternatives can be considered for the TB mapping on multi-consecutive slots.

- Alt 1: one TB processing over multiple slots

- Alt 2: one TB repetition over multiple slots

- Alt 3: multi-TBs mapping over multiple slots

Alt 1 is generally beneficial for coverage enhancement in the poor coverage scenarios, which is not the typical scenario for SL operation. Meanwhile, it may lead to large specification impacts on the current physical channel structure. Thus, alt 1 is not preferred. Alt 2 and alt 3 are beneficial to increase the sidelink data rate and to improve the reliability of sidelink transmissions with less spec impact. Thus, alt 2 and alt 3 are preferred as the scenarios for the multi-consecutive slot transmission in SL-U.

Proposal 32: One TB repetition and multi-TBs mapping over multiple slots are preferred for the scenarios of the multi-consecutive slot transmission in SL-U.

For the resource selection for MCSt, the PHY layer aspects have been discussed, including resource selection parameters provided to PHY layer, and PHY layer procedure to determine the candidate resource for MCSt.

Regarding the resource selection parameters provided to PHY layer, it was discussed that, when selecting resource for the MCSt, whether one set of transmission parameters or multiple sets of transmission parameters are provided to L1. In our view, if there are multiple parameters provided to L1, the resource exclusion procedure would become complicated, such as how to determine the candidate resource related to multiple LsubCH values, or how to determine the RSRP threshold values according to multiple priority values, etc. On the other hand, the main motivation of providing multiple set of parameters is to transmit TBs with different QoS requirements in different slots of the MCSt. But it can also be achieved by triggering multiple selection procedures from L2. Thus, it is preferred to support option 1, i.e., only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Proposal 33: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Regarding the PHY layer resource selection procedure, three options have been proposed, i.e., option A, option B and option C.

For option A, the reported candidate multi-slot resource can be generated by the following options:

- Option A-1: The concept of candidate multi-slot resource is introduced at step 7) in 8.1.4 of TS 38.214. For example, firstly, the UE determines the candidate single-slot resource set according to the legacy procedure, then it determines the candidate multi-slot resource. Noted that it is possible to select a slot with severe interference (i.e., not a ‘good slot’ in the SA according to the candidate single-slot resource selection procedure) to form a candidate multi-slot resource, in order to ensure at least a number of candidate multi-slot resources are reported.

- Option A-2: The concept of candidate multi-slot resource is introduced from step 1) in 8.1.4 of TS 38.214. For example, the UE determines the candidate multi-slot resources by applying the Rel-16 resource exclusion procedure on every N consecutive slots. More specifically, the resource exclusion procedure can be updated by extending the granularity of single-slot to N consecutive slot:

  - A candidate multi-slot resource is defined as any set of LsubCH contiguous sub-channels included in the corresponding resource pool with any set of N consecutive slots within the time interval [𝑛+𝑇1, 𝑛+𝑇2].

  - The RSRP measurement is performed for these candidate multi-slot resources, and the average RSRP can be achieved to be compared with the threshold.

  - When the number of the remaining multi-slot resources satisfies requirement of the proportion of the total candidate multi-slot resources, the procedure is finished.

Option B and Option C have less spec impact, as the current RSRP measurement and resource selection procedure in PHY layer is mostly reused. However, Option B would be problematic in some case. For example, for Option B, there may not be any consecutive slots at all in (SA) as in Rel-16, leading to invalid report to L2. On the other hand, option C is similar to Option A-1, which would guarantee a number of candidate consecutive single-slot resources for multi-slot resource selection in L2. Option A-2 require to modify the existing candidate resource selection mechanism, leading to more spec impacts, so as to guarantee that at least a suitable number of candidate consecutive slots is reported. Noted that the interference level would be quite diverse among slots of the multi-slot resource reported by Option A-2.

Some preliminary simulations were performed to evaluate the performance between these options. (Note: Case 0 represent the Rel-16 resource exclusion procedure.)

- Case 1 assumes N is flexible and the single-slot resource within any candidate multi-slot resource are included in the candidate single-slot resource set (SA) as in Rel-16 (Option A-1 or Option C).

- Case 2 assumes the physical layer determine the candidate multi-slot resources by updated resource exclusion procedure (Option A-2).

As shown in Figure 4, Case 1 and Case 2 both have performance gain compared with single-slot case. Moreover, Case 1 has better performance than Case 2 owing to the less interference and lower retransmission latency.

Observation 1: The mode 2 enhancements for multi-slot transmission resource selection (either on PHY layer or MAC layer) improve the UPT performance compared with that of the legacy mode 2 resource selection.

Proposal 34: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time. (Option A)
-  FFS at which step in 8.1.4 of TS 38.214 the concept of candidate multi-slot resource is applied.

Proposal 35: Additional ending loop condition in resource selection step 7) is required to ensure consecutive single-slot resources reported to higher layer.

Figure 4 System performance for multi-slot transmission

  - Flexible channel access timing

Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

In order to increase the chance for SL UE to get the transmission opportunities, more frequent channel access instants are beneficial for the SL system. One approach to achieve such purpose is to introduce multiple starting symbols for SL transmission, e.g., SL mini-slot, then SL UE can start transmission at any of the starting symbols.

According to the working assumption in RAN1#110bis-e e-meeting, up to two candidate starting symbols within a slot can be used for a PSCCH/PSSCH transmission. The simulation results in Figure 7 shows that introducing two starting symbols per slot can improve the throughput by 11% already. Therefore, the working assumption to support maximum 2 candidate starting symbols within a slot should be confirmed, and larger number of candidate starting symbols is not needed.

Figure 5 System performance for mini-slot transmission

Proposal 36: Confirm the working assumption to support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

Although one more PSCCH/PSSCH transmission occasion is introduced per slot, resource allocation procedure is not expected to be performed independently for the transmission starting from the 2nd starting symbol. To minimize the specification effort, a unified slot-based resource allocation procedure (e.g., mode 2) should be reused regardless of whether the PSCCH/PSSCH transmission starts from the 1st or 2nd starting symbol within a slot.

Proposal 37: The existing mode-2 resource allocation mechanism is used regardless of whether the PSCCH/PSSCH transmission starts from the 1st or 2nd starting symbol within a slot.

In mode 2 resource selection procedure, the sensing-based resource exclusion is based on decoding of PSCCH. Such principle can still be applied for the PSCCH transmitting from the 2nd starting symbol. When the PSCCH transmitted from the 2nd starting symbol is decoded by the UE, the associated resource reservation information can be also used in the mode 2 procedure. However, if UE is mandated to decode PSCCH transmitted from the 2nd starting symbol, the implementation complexity for PSCCH decoding will be doubled compared with Rel-16 UE. To maintain a reasonable UE complexity, it is preferred that the decoding of 2nd PSCCH in a slot is not mandatory for all the UEs.

Proposal 38: Information decoded from the PSCCH transmitted from the 2nd starting symbol is used for resource selection procedure.

Proposal 39: Decoding of PSCCH transmitted from the 2nd starting symbol of a slot is not mandatory for all the UEs.

  - CPE

In RAN1#111 meeting, it was agreed to support both single CPE or multiple CPE starting positions. Further details on the use of CPE will be discussed in this section.

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

- Candidate CPE starting positions

It has been agreed in the previous meeting that the CPE starting position is located within one or two symbol before the next AGC symbol. However, it has also been agreed that the starting symbol and the number of symbols for SL slot is configurable. Consequently, the ‘symbol’ before the AGC symbol may actually not a SL symbol (e.g., a DL, UL, or flexible symbol). It should be clarified whether the CPE starting position can be transmitted in SL symbol only, or in any symbol. As example is shown below. The contradiction is that, if CPE can only be transmitted in SL symbol, given that the the AGC symbol and the prior SL symbol may not be adjacent, the COT would be broken, which makes the whole design of CPE becomes meanlingless.

Proposal 40: RAN1 should clarify whether the CPE starting position can be transmitted in SL symbol only, or in any symbol before the next AGC symbol.

To determine the candidate CPE starting positions, the NRU CPE mechanism of configured grant based PUSCH transmission can be reused as much as possible. At least for 15kHz SCS and for 30kHz SCS with 2-symbol CPE, the candidate CPE starting positions should exactly be the same as that in NRU, i.e., 7 candidate CPE postions based on Table 5.3.1-2 TS 38.214. However, for 30kHz SCS with 1-symbol CPE or for 60kHz SCS with 1-symbol/2-symbol CPE, due to insufficient time duration, only a subset of CPE is selected from the 7 candidate CPE postions.

Based on agreement from RAN1#112 meeting, there are two options for the length of the candidate CPEs.

- Option 1: within the symbol just before the next AGC symbol

- Option 2:

  - within the symbol just before the next AGC symbol for 15 kHz SCS

  - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS

In our option, there is no need to introduce or define option 1 and 2 in the specification, nor applicable condition for each option. The number/index of CPEs as well as the CPE duration (within one or two symbols) can be (pre-)configured per resource pool.

Proposal 41: For 15kHz SCS and for 30kHz SCS with 2-symbol CPE, the 7 candidate CPE postions based on Table 5.3.1-2 TS 38.214 is reused; for 30kHz SCS with 1-symbol CPE or for 60kHz SCS with 1-symbol/2-symbol CPE, a subset of candidate CPE postions is selected from the 7 candidate CPE postions.

Proposal 42: Regarding the length of the candidate CPEs, there is no need to introduce or define option 1 and 2 in the specification, nor applicable condition for each option. The number/index of CPEs as well as the CPE duration (within one or two symbols) can be (pre-)configured per resource pool.

Figure 6 Defer duration Td for CAPC=1 (mp=2)

One issue raised by some companies is the Tx/Rx switching time. When a UE finishes a SL transmission in a previous slot (e.g., PSFCH) and starts LBT for the next COT in the next slot (e.g., PSSCH), the UE have to first perform a Tx/Rx switching after the first transmission, then excuting the LBT sensing, and finally a Rx/Tx switching for the second transmission if LBT succeeds. The question is that whether the Tx/Rx and Rx/Tx switching time should be taken into counted when designing the CPE length. According to the spec, the LBT procedure starts with a defer time duration Td, which consists of a Tf followed by mp consecutive Tsl as shown in Figure 6. The 16us Tf includes a Tsl at the start of the Tf, and energy dection within the Tsl should be at least 4us. Therefore, by implementation, UE can perform the energy detection at the end of the first Tsl, and use the first 5us for Tx/Rx switching. The rest of the Tsl (7us) can be used for processing. In other words, the Tx/Rx switching time and the Rx/Tx switching time can be absorbed by the time gap defined as NRU.

Proposal 43: The Tx/Rx and Rx/Tx switching time can be absorbed by the time gap for CPE based on the Table 5.3.1-2 in TS 38.214.

- Single CPE starting position for PSCCH/PSSCH

In our view, when single CPE starting position for PSCCH/PSSCH is configured for the resource pool, the CPE can be used to fill the gap(s) between slots to formulate consecutive transmissions in a COT. For example, the gaps between PSSCH or gaps between PSSCH and PSFCH from the same SL UE can be filled to meet the transmission burst requirement as shown in Figure 7. Another example is that the gap between PSSCH transmission of COT sharing UE and PSSCH transmission of the shared COT UE can be filled with CPE as well.

The gap between two neighboring transmissions can be filled with CPE as in NRU, and it should be ensured that the remaining gap between the CPE and the prior transmission is less than or equal to 16 us, and the CPE ends until ACG symbol of the later transmission, so that the neighboring transmissions can be treated as a transmission burst and be sent without additional LBT for the later SL transmissions.

Figure 7 Continuous transmission

Proposal 44: When single CPE starting positions is configured for the resource pool, the CPE is at least used to fill the gap(s) between the consecutive SL transmissions in the same COT.

Proposal 45: When single CPE starting positions is configured for the resource pool, the CPE starts within 16us after the prior transmission, and ends until the ACG symbol of a following transmission.

- Multiple CPE starting positions for PSCCH/PSSCH

When multiple CPE starting positions for PSCCH/PSSCH are configured for the resource pool, different transmission starting points can be used to avoid resource collision, i.e., if two UE selects overlapped resources, UE selecting the earlier CPE starting point can block the other one to avoid resource collision. Although mode 2 resource selection procedure can also avoid resource collision, but for initial transmission of two UE or for the resource selection in high congestion scenario, resource collision may also occur. Hence, use of multiple CPE starting positions can complement mode 2 procedure in the sense of resource collision avoidance.

Although use of multiple CPE starting positions can help to avoid resource collision, if there are FDMed/SDMed transmissions in a given RB set, use of flexible starting positions may suffer from inter-UE blocking issue, i.e., the UEs with earlier starting position will block the later transmissions. To mitigate the inter-UE blocking issue, the mechanism of CPE determination for partial RB set transmission should consider the FDMed transmissions, i.e., for partial RB set transmission, if the UE detects potential inter-UE blocking with another FDMed transmission, a (pre-)configured default CPE starting position is used for the partial RB set transmission. Regarding the detection of inter-UE blocking with a FDMed transmission, the resource location acquired from the resource reservation information can be used to determine the FDMed relationship, and the energy measurement resource associated with the reserved resource can be used to determine whether there will be inter-UE blocking or not.

For the selection of CPE starting position from the multiple positions, selection of CPE based on transmission priority should be supported to protect high priority transmission. Neverthelss, a UE can use a CPE starting position not earlier than the selected one. For example, if candidate CPEs are located within 2 symbols before the next AGC symbol and the there is only one gap symbol between adjacent slots, when the UE selects a earliest CPE starting position based on priority, it can further decide whether to use this earliest CPE starting positin or use a later CPE starting position, e.g., when there is no SL transmission in prior slot, UE use this selected ealiest CPE starting position, otherwise UE uses a CPE located within 1 symbol before the next AGC symbol.

Proposal 46: For partial RB set transmission, if the UE detects another reserved FDMed resource and the energy measurement assocated with the reserved resource is higher than EDT, (pre-)configured default CPE starting position is used; Otherwise, one CPE starting position is selected from the multiple CPE starting positions based on priority (CAPC) , UE can use a CPE starting not earlier than the selected CPE starting position.

Moreover, as discussed above, for the purpose of gap filling for multi-consecutive slots transmission or for COT sharing, the CPE with earliest starting position can always be used to make the remaining gap after filling the CPE to be less than 16us. Such rule should always be applied even multiple CPE are used for the resource pool to guarantee higher channel access probability.

Proposal 47: Even if multiple CPE starting position are used in a resource pool, a fixed CPE starting position (e.g., earliest CPE starting position in the gap) is used to fill the gap(s) between the consecutive SL transmissions in the same COT.

- CPE for S-SSB

Regarding the LBT for the R16 S-SSB occasion (for a single S-SSB occasion or for the 1st S-SSB occasion of a burst of consecutive S-SSB occasions), at least a unified CPE is agreed. Considering that the S-SSB occasion may be right after a PSSCH/PSCCH reception, this CPE should be short enough to avoid interruption to the prior PSSCH/PSCCH reception, and to ensure enough time for executing LBT sensing.

For R18 additional S-SSB occasions within the resource pool, the following options can be considered.

- Option 1: Use the same CPE as the R16 S-SSB occasion.

- Option 2: Determine the CPE in the same way as PSSCH/PSCCH. e.g., both PSSCH/PSCCH CPE and S-SSB CPE are determined based on their priorities, or by random selection. Alternatively, both S-SSB and PSSCH/PSCCH use a default CPE (pre-)configured for the resource pool. The benefit of option2 is to provide a unified CPE determination method for both S-SSB and PSCCH/PSSCH in the resource pool. However, it should be noted that with the priority-based CPE determination, S-SSB transmission on a R18 S-SSB occasion still may be blocked by other UE's PSSCH/PSCCHs with higher priority when performing type-2 channel access.

- Option 3: The CPE is related to whether the UE has successfully transmitted S-SSB on the corresponding R16 S-SSB occasions. For example, if the LBT for the prior R16 S-SSB occasions failed, it means that the R18 S-SSB occasion has to be used for S-SSB transmission. Accurate synchronization is essential to maintain subsequent communication, to avoid blocking from other UEs selecting the same R18 S-SSB occasion for data transmission, the longest CPE or the CPE associated with the highest priority should be used for LBT for R18 S-SSB in this case.

- Option 4: The CPE is configurable. The advantage of having a configurable CPE for the additional S-SSB transmission is that it allows UE to achieve a tradeoff between S-SSB coverage and throughput. As analyzed in our companion paper[2], successful LBT for PSSCH on additional S-SSB occasions can result in up to 1.875% extra PSSCH resources being available to improve throughput per 160ms period. On the contrary, if LBT for S-SSB succeeds, S-SSB coverage can be guaranteed. Therefore, if the NW/OEM wishes to protect potential PSCCH/PSSCH transmission on additional S-SSB occasions, a short CPE can be (pre-)configured for S-SSB, increasing the likelihood of data transmission and ensuring high throughput. Conversely, if the NW/OEM places greater value on synchronization coverage, a longer CPE for S-SSB can be (pre-)configured.

Figure 8．Example of CPE for R16 S-SSB occasion

Figure 9．Example of CPE for R18 S-SSB occasion (option3)

In addition, in the case of multiple S-SSB occasions spanning a set of consecutive time slots, the UE needs to insert a CPE between consecutive S-SSBs transmissions to maintain occupancy of the channel. The length of this CPE should be long enough to ensure that the channel will not be recognized as idle by other devices. For example, the CPE starts within 16us after the prior S-SSB transmission and ends until the ACG symbol of a following S-SSB transmission.

Proposal 48: For LBT for Rel-18 S-SSB occasion within a resource pool, the following options can be considered for CPE determination：
- Option 1: the CPE is the same as the R16 S-SSB occasion.
- Option 2: the CPE is determined in the same way as PSSCH/PSCCH in the same resource pool.
- Option 3: If the LBT for the corresponding R16 S-SSB occasions failed, the longest CPE, or the CPE associated with the highest priority should be used for S-SSB transmission. 
- Option 4: the CPE is configured for the resource pool.

Proposal 49: For consecutive S-SSB transmissions in a COT, a short enough CPE is used for the leading S-SSB to avoid being blocked by prior transmission, and a long enough CPE is used for other S-SSB to occupy the channel.

- FBE

FBE based operation is suitable for controlled scenario, e.g., factory, game center, etc., especially where LBE equipment are absent. It has very short sensing time and fixed transmission starting position. The aligned Fixed Frame Periods (FFPs) among devices are beneficial to increase the spectrum efficiency by simultaneous transmissions from different devices. Considering that the applicable scenarios of FBE and LBE are different, FBE should also be supported besides LBE, which is already supported in NR-U.

Observation 2: FBE based operation is suitable for controlled scenario, e.g., factory, game center, etc., and is beneficial to increase the spectrum efficiency.

Frame Based Equipment as shown in Figure 10 performs transmission/reception based on a frame structure that has a periodic timing with a periodicity equal to FFP, which is selected from {1ms, 2ms, 2.5ms, 4ms, 5ms, 10ms}. The FFP includes Observation Slot, Channel Occupancy Time, and Idle Period. The Observation Slot shall have a duration of not less than 9 μs. The Channel Occupancy Time shall not be greater than 95 % of the FFP and shall be followed by an Idle Period until the start of the next FFP such that the Idle Period is at least 5% of the Channel Occupancy Time, with a minimum of 100 μs.

Figure 10 FBE frame structure

Support of FBE based operation by SL UE requires much less design and specification efforts than LBE, thanks to the basic frame structure design of SL. For the SL operation based on FFP, the SL transmission should start from the beginning of a FFP, and SL transmission is not permitted in the end of a FFP. To follow such restriction, it can be assumed that the FFP is aligned with SL slot, i.e., assuming 1ms FFP length and 1ms SL slot length, thus all the PSCCH/PSSCH transmissions will start from the beginning of FFP, and gap symbol of a slot can meet the 5% idle period requirement. By this way the existing SL structure can be directly reused without changes. Moreover, the existing mode-2 resource allocation procedure can also be easily reused with a one-shot CCA during the gap symbol for channel access.

Observation 3: Support of FBE requires much less design and specification efforts than LBE, Rel-16 SL PHY structure/procedure and resource allocation mechanism can be maximumly reused.

Proposal 50: For FBE based SL operation, the FFP can be aligned with SL slot.

The comparison between FBE and LBE is simulated, assuming that only one of the FBE and LBE operation is used by SL UE (in mode 2 with single slot resource allocation). Noted that other system (i.e., WiFi) is present in both cases in the simulation. From the simulation results in Figure 11, it can be observed that FBE has a 16% improvement in throughput compared to LBE.

Figure 11 System level performance comparison between FBE and LBE

Observation 4: FBE-based operation improves UPT by 10%, 18%, and 8%, respectively, compared to LBE-based operation for different BO load cases , assuming mode 2 with single slot resource allocation and WiFi is present.

Proposal 51: Support FBE based operations for SL transmission on unlicensed band.

- Wideband operation

In  RAN1 111 meeting, the following agreement has been approved.

Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

Figure 12: The coexistence problem between different capability UEs

In SL unlicensed band, UE with different capabilities should be able to operate within the same resource pool. For example, in a typical commercial use case, a smartwatch should be able to communicate with the paired smartphone in the same unlicensed band. Noted that a watch is typically supporting smaller bandwidth compared to a smartphone, i.e., only one RB set bandwidth (20MHz). When the smartphone indicates a wideband transmission with the SCI transmitted in the lowest RB set of PSSCH, the watch cannot receive the transmission if it locates in other RB sets. Furthermore, this may cause the resource collision problem because the watch cannot identify the resources reserved by the SCI in the other RB sets as Figure 12. To support this use case, the UE should transmit SCI in every allocated RB set, and avoid to reserve resources in RB set other than the RB sets of the receiver. Thus, UE supporting smaller bandwidth can be aware of resources indicated by other UEs.

Proposal 52: The design of wideband operation in SL-U should support direct communication between a UE operating in multiple RB sets and another UE can only operate in one or subset of the RB sets.

Proposal 53: The SL UE transmits SCI in every allocated RB set and avoid to reserve resources in RB set other than the RB sets of the receiver.

In the previous meeting, it has been agreed that NR-U UL channel access procedure is considered as the baseline for transmission on multiple channels. Given that the NR-U UL multi-channel access procedure is an all-or-nothing mechanism, the transmission possibility is affected by the number of allocated RB sets, i.e., the more the RB sets are selected, the lower the successful channel access probability can be expected. Thus, in order to reduce the impact of LBT failure on transmission, the mode 2 resource selection in the frequency domain should be enhanced.

Since the transmission cannot be performed as long as there is one unsuccessful LBT detection in all the channels, the number of the allocated RB sets should be limited as much as possible, especially when the TB size is small.

Proposal 54：In mode 2 resource selection, the number of the allocated RB sets should be limited as much as possible, especially when the TB size is small.

In addition to TB size, there are some other information, e.g. priority and CBR, should be considered to determine whether to perform wideband transmission and the number of RB sets in wideband transmission. For example, if the priority is higher, the reserved resources would more likely be maintained (as the lower priority transmission shall not preempt them). In this case, wideband transmission is preferred as the success rate of access may be high. In addition, when the CBR measurement is high (e.g., higher than a threshold), it means there are a lot of transmission in the resource pool or in the RB set. So the transmission is more likely to be blocked and the channel access possibility of wideband transmission would be affected heavily. Thus the wideband transmission is not preferred in this condition.

Proposal 55: When determine whether to perform wideband transmission and the number of selected channels in wideband transmission, the information other than TB size, e.g. transmission priority or CBR measurement result, should be considered

It has agreed that the NR-U DL multi-channel access procedure can be used as the baseline for PSFCH transmission in case of dynamic channel access mode with multi-channels. A remaining issue is whether Type A or Type B or both can be supported for this case for PSFCH transmission. Type A multi-channel access requires to perform Type 1 LBT on each channel it intends to transmit, while Type B multi-channel access requires to perform Type 1 LBT only on the primary channel and sense the remaining channel for at least 25us before transmission. Type B has a demerit that once the type-1 LBT fails, all the PSFCHs are droped, while Type A only drops the PSFCH of the LBT channel that LBT fails. Therefore, Type A is preferred.

Proposal 56: Type A multi-channel access procedures is supporeted  for PSFCH transmission in the case of dynamic channel access mode with multi-channels.

In addition, the bandwidth-limited UE can only receive and transmit synchronization signals in their supported RB set. Hence, to achieve synchronization among bandwidth-limited UEs and other UEs, S-SSB should be transmitted in every supported RB set. Meanwhile, considering S-SSB is a fundamental feature of the SL system, if S-SSB can be transmitted in more than one RB set, the DL channel access should be used as the baseline for S-SSB transmission in the case of dynamic channel access mode with multi-channels. Otherwise the synchronization between UEs might be affected heavily.

Proposal 57: The DL channel access is used as the baseline for S-SSB transmission in the case of dynamic channel access mode with multi-channels.

- Conclusion

This contribution focus on the channel access mechanisms for sidelink on unlicensed spectrum, and providing the following observations and proposals:

Observation 1: The mode 2 enhancements for multi-slot transmission resource selection (either on PHY layer or MAC layer) improve the UPT performance compared with that of the legacy mode 2 resource selection.

Observation 2: FBE based operation is suitable for controlled scenario, e.g., factory, game center, etc., and is beneficial to increase the spectrum efficiency.

Observation 3: Support of FBE requires much less design and specification efforts than LBE, Rel-16 SL PHY structure/procedure and resource allocation mechanism can be maximumly reused.

Observation 4: FBE-based operation improves UPT by 10%, 18%, and 8%, respectively, compared to LBE-based operation for different BO load cases , assuming mode 2 with single slot resource allocation and WiFi is present.

Proposal 1: Support Type 2A channel access for PSFCH transmission.

Proposal 2: The criterion of applying LBT type-2 for S-SSB and PSFCH should be determined.

Proposal 3: The UE chooses to apply Type 2B or Type 2C channel access depending on the SL transmission duration.

Proposal 4: Reference duration is not defined when the COT contains only groupcast option1 transmission and/or transmissions with SL-HARQ feedback disabled.

Proposal 5: The ending time of reference duration for MCSt is the same as that for single slot transmission.

Proposal 6: The PSFCH or S-SSB within the reference duration cannot be used for CWS adjustment.

Proposal 7: When all the transmissions within the COT are PSSCH with HARQ feedback disabled or PSSCH of groupcast option 1, UE uses the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

Proposal 8: When all the transmissions within the COT are groupcast option 2, UE adjusts the CWS based on the ratio of the received ACK to all the received feedbacks.

Proposal 9: When the transmissions within the reference duration includes different cast types, UE adjusts the CWS based on the transmission with ACK/NACK HARQ-ACK enabled, where the unicast transmission are treated with the highest priority

Proposal 10: To resolve the Type 1 LBT blocking issue, the following solution is supported:
- UE avoids selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource.
- UE avoids selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.
- FFS how to acquire the type 1 LBT duration of the reserved resource.
- FFS how to determine LBT blocking between the reserved resource and a selected resource.

Proposal 11: To resolve the Type 1 LBT blocking issue, SL UE deems channel busy only if the UE detects transmission other than SL transmission occupying the channel (e.g., exceeding the energy detection threshold), i.e., the energy detection for EDT checking in LBT procedure does not take into account the energy from SL transmissions.

Proposal 12: The mode 2 resource selection should be enhanced to guarantee sufficient LBT duration before the SL transmission resource(s).

Proposal 13: The enhancement of resource selection mechanism in shared COT needs further study. E.g., the resource selection window is selected within the shared COT or the RSRP measurement is relaxed for the resource within the shared COT.

Proposal 14: When UE detects the gap between the end of LBT procedure and the start of the SL transmission resource, the UE apply a 25us deferred LBT before the SL transmission resource.

Proposal 15: For mode 2, the transmission resource should be selected as early as possible to approach the end of the LBT procedure.

Proposal 16: For mode-1 UE, LBT type for the scheduled resources/configured grant as well as the priority class is decided by UE.

Proposal 17: For mode-1 UE, if the obtained COT includes additional time resources other than the scheduled resources/configured grant, the UE is allowed to exploit these resources or to share them with its peer UE(s).

Proposal 18: For mode-1 UE, gNB can indicate a set of resources to a group of UE by a group common DCI or RRC, UEs in the group can perform LBT for the scheduled resources/configured grant and possibly share the COT with other UEs if LBT succeeds.

Proposal 19: For mode-1 UE, separate reporting of LBT results/COT information and HARQ-ACK for the scheduled resources are supported.

Proposal 20: For a mode-1 grant, the time location of the corresponding UL resource for SL HARQ-ACK or LBT results/COT information reporting is determined based on one of the following options:
- Option1. For each PSFCH candidate, there is one associated candidate UL resource. The candidate UL resource associated with the successfully accessed PSFCH candidate, or the candidate UL resource associated with the last PSFCH candidate if all LBT fails, is used for reporting.
- Option2. There is one UL resource for reporting, and its time location is derived from the last PSFCH candidate.

Proposal 21: For a set of consecutive slots configured for R16 S-SSB occasion, if UE intends to transmit S-SSB on one or multiple of the slot, and it detects no SCI indicating a shared COT that is overlapped with the one or multiple slots but detects S-SSB on at least one slot in the set of consecutive slots.
- It assumes that there is a 2ms shared COT starting from the first detected S-SSB
- If the assumed COT is overlapped with its occasion for S-SSB transmission, it can share the COT to transmit S-SSB.

Proposal 22: When a UE only has S-SSB to transmit and it accesses the R16 S-SSB occasion with Type-1 LBT successfully, the reserved bits in MasterInformationBlockSidelink is used to indicate the shared COT information, e.g., whether the COT can be shared for S-SSB, and/or the start of the COT.

Proposal 23: A responding UE can transmit PSFCH(s) to UE(s) other than the COT initiator if the responding UE also transmits PSFCH(s) to the initiating UE simultaneously.

Proposal 24: Additional ID included in the COT sharing information is not preferred. The case is not supported if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

Proposal 25: The CAPC level, remaining COT duration and L1 ID are included in COT information.

Proposal 26: The channel access type within the shared COT is determined by the responding UE based on the gap between transmissions of the COT initiating UE and responding UE.

Proposal 27: The RB set indication and CPE indication in COT information are not necessary.

Proposal 28: The COT information is preferred to be contained in SCI, and a new 1st SCI format is required for SL-U if COT information is contained in the 1st stage SCI.

Proposal 29: UE forwarding/relaying information about a COT initiated by another UE is not supported.

Proposal 30: If more than one COT is identified by a COT sharing UE, the responding UE should determine which COT to share according to the COT sharing information.

Proposal 31: No enhancement on the UE-to-UE ED threshold is needed.

Proposal 32: One TB repetition and multi-TBs mapping over multiple slots are preferred for the scenarios of the multi-consecutive slot transmission in SL-U.

Proposal 33: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Proposal 34: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time. (Option A)
-  FFS at which step in 8.1.4 of TS 38.214 the concept of candidate multi-slot resource is applied.

Proposal 35: Additional ending loop condition in resource selection step 7) is required to ensure consecutive single-slot resources reported to higher layer.

Proposal 36: Confirm the working assumption to support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

Proposal 37: The existing mode-2 resource allocation mechanism is used regardless of whether the PSCCH/PSSCH transmission starts from the 1st or 2nd starting symbol within a slot.

Proposal 38: Information decoded from the PSCCH transmitted from the 2nd starting symbol is used for resource selection procedure.

Proposal 39: Decoding of PSCCH transmitted from the 2nd starting symbol of a slot is not mandatory for all the UEs.

Proposal 40: RAN1 should clarify whether the CPE starting position can be transmitted in SL symbol only, or in any symbol before the next AGC symbol.

Proposal 41: For 15kHz SCS and for 30kHz SCS with 2-symbol CPE, the 7 candidate CPE postions based on Table 5.3.1-2 TS 38.214 is reused; for 30kHz SCS with 1-symbol CPE or for 60kHz SCS with 1-symbol/2-symbol CPE, a subset of candidate CPE postions is selected from the 7 candidate CPE postions.

Proposal 42: Regarding the length of the candidate CPEs, there is no need to introduce or define option 1 and 2 in the specification, nor applicable condition for each option. The number/index of CPEs as well as the CPE duration (within one or two symbols) can be (pre-)configured per resource pool.

Proposal 43: The Tx/Rx and Rx/Tx switching time can be absorbed by the time gap for CPE based on the Table 5.3.1-2 in TS 38.214.

Proposal 44: When single CPE starting positions is configured for the resource pool, the CPE is at least used to fill the gap(s) between the consecutive SL transmissions in the same COT.

Proposal 45: When single CPE starting positions is configured for the resource pool, the CPE starts within 16us after the prior transmission, and ends until the ACG symbol of a following transmission.

Proposal 46: For partial RB set transmission, if the UE detects another reserved FDMed resource and the energy measurement assocated with the reserved resource is higher than EDT, (pre-)configured default CPE starting position is used; Otherwise, one CPE starting position is selected from the multiple CPE starting positions based on priority (CAPC) , UE can use a CPE starting not earlier than the selected CPE starting position.

Proposal 47: Even if multiple CPE starting position are used in a resource pool, a fixed CPE starting position (e.g., earliest CPE starting position in the gap) is used to fill the gap(s) between the consecutive SL transmissions in the same COT.

Proposal 48: For LBT for Rel-18 S-SSB occasion within a resource pool, the following options can be considered for CPE determination：
- Option 1: the CPE is the same as the R16 S-SSB occasion.
- Option 2: the CPE is determined in the same way as PSSCH/PSCCH in the same resource pool.
- Option 3: If the LBT for the corresponding R16 S-SSB occasions failed, the longest CPE, or the CPE associated with the highest priority should be used for S-SSB transmission.

Proposal 49: For consecutive S-SSB transmissions in a COT, a short enough CPE is used for the leading S-SSB to avoid being blocked by prior transmission, and a long enough CPE is used for other S-SSB to occupy the channel.

Proposal 50: For FBE based SL operation, the FFP can be aligned with SL slot.

Proposal 51: Support FBE based operations for SL transmission on unlicensed band.

Proposal 52: The design of wideband operation in SL-U should support direct communication between a UE operating in multiple RB sets and another UE can only operate in one or subset of the RB sets.

Proposal 53: The SL UE transmits SCI in every allocated RB set and avoid to reserve resources in RB set other than the RB sets of the receiver.

Proposal 54：In mode 2 resource selection, the number of the allocated RB sets should be limited as much as possible, especially when the TB size is small.

Proposal 55: When determine whether to perform wideband transmission and the number of selected channels in wideband transmission, the information other than TB size, e.g. transmission priority or CBR measurement result, should be considered.

Proposal 56: Type A multi-channel access procedures is supporeted  for PSFCH transmission in the case of dynamic channel access mode with multi-channels.

Proposal 57: The DL channel access is used as the baseline for S-SSB transmission in the case of dynamic channel access mode with multi-channels.

- References

- RP-213678, WID on NR sidelink evolution, RAN #94e, Electronic Meeting, Dec. 6-17, 2021.

- R1-2300454, Physical channel design framework for sidelink on unlicensed spectrum, vivo, RAN1 #112.

- R1-2301797, FL summary for AI 9.4.1.1: SL-U channel access mechanism (EOM), OPPO, RAN1 #112.

Annex I

Table 1 System level simulation assumption

| Parameter | Value |
| --- | --- |
| Deployment | Indoor |
| Channel model | P2P link defined in TR37.885 |
| Communication type | Unicast and groupcast |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15KHz |
| Link adaptive | Enable |
| PSFCH LBT | enble |
| Traffic model for SL | FTP model 3 with arrival rate: •BO Low load: 10%~25% •BO Mid load: 35%~50% •BO High load: above 55% |
| Traffic model for AP | Same traffic load as SL |
| Channel access | FBE, LBE |
| Tx Power | 23 dBm |
| Antenna number | [M N P Mg Ng, Mp Np] = [1 1 2 1 1, 1 1] for SL-UE [M N P Mg Ng, Mp Np] = [1 1 2 1 1, 1 1] for AP |
| UE deploy | For unicast: 6 SL-U pairs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz For group cast: 6 SL-U group and ach group has five UEs, 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz |

Figure 13 unicast UE distribution

Figure 14 groupcast UE distribution

Figure 15 WIFI system throughput

Figure 16 WIFI system throughput