3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302519

e-Meeting, April 17th – April 26th, 2023

Source: 	National Spectrum Consortium

Title: 	Sidelink Channel Access Mechanisms

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

# Background

The WI on NR sidelink evolution [1] includes an objective to study and specify support for sidelink operation in the unlicensed bands with focus on the FR1 unlicensed bands, n46 and n96/n102. The agreements related to SL-U channel access mechanism at the 3GPP RAN WG1 #112 meeting are listed below [2].

Agreement

The CAPC level that should be used for S-SSB transmissions:

- Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission

Agreement

The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission

Agreement

The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows:

- Option 1a

  - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted

  - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT

  - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed

  - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

Agreement

A CPE can be transmitted from a CPE starting position before SL transmission for the following two options:

- Option 1: within the symbol just before the next AGC symbol

- Option 2:

  - within the symbol just before the next AGC symbol for 15 kHz SCS

  - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS

- FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

Agreement

- A responding UE over a shared COT can be:

  - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator

    - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE

    - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE

  - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator

    - FFS Limitations on what additional IDs may be included and how they may be indicated

Agreement

A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information.

Agreement

A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when,

- In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported)

- In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported)

- FFS: all other details and additional restrictions

In this contribution we present our views on open and unaddressed issues on this topic.

# Channel Access

## Semi-static Channel Access

In RAN1#112, there were discussions to down-prioritize semi-static channel access. For example, the following agreement was proposed:

•	Specification work to support channel access procedures for semi-static channel occupancy is supported for SL-U is lower priority in Rel-18.

o	Channel access procedures for semi-static channel occupancy from NR-U should be taken as the starting pointbaseline

o	If all essential details of channel access procedures for semi-static channel occupancy are not finalized by RAN1 August 2023 meeting, channel access procedures for semi-static channel occupancy will not be specified for SL-U in Rel-18.

We believe introducing semi-static channel access to SL-U is less complex than dynamic channel access, since the principles of UE-initiated semi-static channel access introduced in Rel-17 can be applied to sidelink. The essence of Rel-17 UE-initiated semi-static channel access is captured below from TS 37.213:

A UE initiates a channel occupancy in a period of duration Tu if the UE transmits a UL transmission burst starting at the beginning of the period immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9usTsl and ends the transmission of the UL transmission burst before the start of the idle duration of that period. When the UE initiates a channel occupancy in that period, the UE shall not transmit any transmission(s) within the idle duration of that period.

Translating this scheme to the sidelink case implies that a UE with at least PSSCH/PSCCH transmissions can initiate a semi-static channel occupancy, and no SL-U transmissions are allowed within the idle duration of that period. Then within the channel occupancy, the UE-to-UE COT sharing schemes designed to date in Rel-18 are also applicable.

Proposal 1: Do not de-prioritize channel access procedures for semi-static channel occupancy. UE-initiated semi-static channel access introduced in Rel-17 should be taken as the baseline.

## CPE and Starting Positions

The essence of CPE is to bridge time gaps between transmissions of different nodes within a shared CO and obviate the need for Type 1 LBT. Four levels of CP extension were defined for PUSCH in NR-U for this reason. CPE is best suited for responding PSSCH/PSCCH/PSFCH transmissions within a shared CO, and the utility of CPE for S-SSB is dubious when Type 2A LBT is available for S-SSB transmission outside a shared CO. Two CPE options, Option 1 and Option 2 have been agreed in the last meeting.

Proposal 2:

- Only Type 2 LBT is applicable for Option 2 CPE within at most 1, 2 symbols just before the next AGC symbol for 15, 30 and 60 kHz SCS, respectively.

- A single CPE starting position is pre-defined for PSFCH.

- At most one CPE starting position for S-SSB that is pre-defined.

- One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH

  - When multiple CPE starting positions are (pre-)configured, the criterion for selecting the value is determined based on indication from the COT initiating UE.

## COT Sharing

For UE-UE COT sharing, it has been agreed that a responding UE sends PSSCH/PSCCH to the CO initiating UE when the source and destination IDs are symmetric. For PSFCH in response to a CO-initiating UE, there is an open issue regarding whether the responding UE can transmit PSFCH(s) to UE(s) other than the initiator. Not allowing this operation would severely limit the efficient utilization of SL-U shared spectrum. For example, a responding UE may have pending HARQ feedback for the initiator as well as for a second UE for which it could not send HARQ feedback in a previous CO due to processing time limits. Forcing PSFCH to be transmitted only in a pair-wise manner will result in many additional CO initiations simply to send PSFCH, and unnecessary delays in CWS adjustment by UEs that have not received pending HARQ feedback.

Proposal 3: For UE-UE COT sharing, a responding UE with PSFCH transmission can transmit PSFCH(s) to UE(s) other than the initiator.

Similar logic holds for COT sharing when performing PSSCH/PSCCH transmission(s). A responding UE may have a pending PSSCH carrying a CSI report for another UE as well as a PSSCH that is scheduled by the initiator UE.

Proposal 4: For UE-UE COT sharing, support the case where a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

## Multi-channel transmission

For multiple PSFCH transmissions on multiple channels, the NR-U DL multi-channel transmission scheme is to be used and each PSFCH is confined to one LBT channel. Agile channel access requires that we allow frequency non-contiguous transmission for the PSFCHs, as is allowed in NR-U when intra-carrier guard bands are configured. Whether Type A or Type B is used can be left to implementation.

Proposal 5: For multiple PSFCH transmissions on multiple channels, multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is not limited to contiguous RB sets. Support both Type A and Type LBT.

# Summary of Proposals

This contribution presented the following proposals for R18 SL-U channel access.

Proposal 1: Do not de-prioritize channel access procedures for semi-static channel occupancy. UE-initiated semi-static channel access introduced in Rel-17 should be taken as the baseline.

Proposal 2:

- Only Type 2 LBT is applicable for Option 2 CPE within at most 1, 2 symbols just before the next AGC symbol for 15, 30 and 60 kHz SCS, respectively.

- A single CPE starting position is pre-defined for PSFCH.

- At most one CPE starting position for S-SSB that is pre-defined.

- One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH

  - When multiple CPE starting positions are (pre-)configured, the criterion for selecting the value is determined based on indication from the COT initiating UE.

Proposal 3: For UE-UE COT sharing, a responding UE with PSFCH transmission can transmit PSFCH(s) to UE(s) other than the initiator.

Proposal 4: For UE-UE COT sharing, support the case where a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

Proposal 5: For multiple PSFCH transmissions on multiple channels, multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is not limited to contiguous RB sets. Support both Type A and Type LBT.

# References

- 3GPP RP-221938: "WID revision: NR sidelink evolution ".

- Chairman’s Notes, RAN WG1 #112

- 3GPP TS 37.213 V17.3.0 (2022-09), Physical layer procedures for shared spectrum channel access (Release 17)