3GPP TSG RAN WG1#112bis-e	 	 	R1-2303720

e-Meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on positioning for RedCap UEs

Agenda Item:	9.5.5

Document for: 	Discussion and Decision

- Introduction

At the RAN#98-e meeting, new WID on Rel-18 NR positioning enhancement was agreed [1]. The work item includes objectives related to positioning support for RedCap UEs as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

In this contribution, we present our views on potential enhancements of positioning support for RedCap UEs for Rel-18 NR positioning.

- Discussions on DL PRS frequency hopping

  - MG based and gapless measurements

Regarding potential enhancements for RedCap UEs, the following agreements were made at the last meeting [2].

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

RAN1 discussed the evaluation of PRS/SRS frequency hopping (FH) and concluded that the introduction of PRS/SRS frequency hopping is one possible solution to compensate for the issues associated with narrower bandwidth in SI phase. Reference signal frequency hopping can virtually enhance the bandwidth, which contributes to both obtaining more frequency diversity gain and improving the resolution of the timing-based measurement. In WI phase, RAN1 started to discuss the details of FH.

Regarding the measurement method, RAN1 agreed to support at least measurements on DL PRS with Rx frequency hopping using a measurement gap. One FFS issue is whether the UE uses a single or multiple instances of a MG to measure DL PRS with frequency hopping. Figure 2.1-1 shows an example of single and multiple MG instance(s) configuration. In a single MG case, UE uses a single MG instance where all PRS FH resources are measured by retuning Rx frequencies within the MG instance. In multiple MG instances case, each hop of PRS resources is measured within separate MG instance and retuning of Rx frequencies is performed for each MG instance.

Figure 2.1-1 Single and multiple MG instance(s) configuration

The current specification supports 20 ms and 6 ms maximum measurement gap lengths for each normal UE and RedCap UE, respectively. Considering that RAN1 has agreed that UE hops within a DL PRS resource, these gap lengths are sufficiently long to accommodate a series of hopping and measurement procedures in a single MG instance. Furthermore, the minimum measurement gap repetition periods of both normal and RedCap UEs are 20 ms, and hence it is too long to use multiple MG instances to measure a DL PRS resource. Thus, we propose to support a single instance of a MG for DL PRS FH.

RAN1 also agreed to send an LS to RAN4 asking the feasible values for the switching time between hops. Based on values to be in RAN4 reply LS, RAN1 should check whether current MG length can accommodate all necessary components within a single MG instance to measure DL PRS with FH, such as duration of measuring each hop of PRS resources, switching time between hops and at beginning/ending of measurement. If some enhancement on MG is identified as necessary, RAN1 should ask RAN4 to discuss it.

Proposal 1:

- RAN1 should support a single instance of a MG for DL PRS FH.

Regarding the gap-less measurements for DL PRS FH, the use of PPW is also an FFS issue. RAN1 has agreed that UE hops within a DL PRS resource for MG-based measurement, which means UE doesn’t expect to switch BWP for DL PRS measurement. In Rel-17, PPW was introduced to support low latency positioning by avoiding the use of MG, and PPW is configured within a BWP. Thus, in order to support PPW with FH as gap-less measurement, BWP switching may be required instead, which prevents low latency positioning. In addition, maximum number of BWPs per serving cell is specified as 4 in maxNrofBWPs in TS 38.133. However, considering supporting up to 100 MHz measurement bandwidth for RedCap UE, at least 5 hops are needed, and if the frequency overlapping is introduced, at least 6 hops are needed. For these reasons, we think MG-less measurement doesn’t applicable for FH, and RAN1 may not need to support PPW with FH.

Proposal 2:

- RAN1 may not need to support PPW with FH.

  - Frequency hopping with partial overlap

Regarding the frequency overlap between hops, following proposals were discussed at the last meeting.

Proposal 2.3a-2: for positioning enhancements for RedCap UEs, regarding the frequency resource overlap (downselect): Alt1: the number of overlapping frequency resources in adjacent hops required for the UE to perform phase offset compensation the overlap between hop for DL PRS Rx frequency hopping is a UE capability - FFS: granularity of the capability, including the case of no overlap required Alt2: the effective measurement bandwidth and the number of hops for DL PRS Rx frequency hopping is a UE capability - FFS: granularity of the capability, including the case of no overlap required Proposal 2.3b-2: for positioning for RedCap UEs, the number of hops for DL PRS Rx frequency hopping is a UE capability - FFS: granularity of the capability

The discussion in the SI phase shows that several factors such as phase offset, time gap, UE speed and timing error affect to positioning accuracy performance. We believe that it is necessary to mitigate the effects of these factors in order to achieve accurate positioning with FH. A key issue is phase shift compensation between adjacent hopping resources. If the phase shift between hops is larger than a tolerable value, it can no longer be considered as a single wider bandwidth. To compensate for the phase discontinuity between hops, we think frequency hopping with partial overlap resources should be considered.

At the last meeting, some companies questioned whether overlap is a UE capability. We believe that the support of overlap is essential for RedCap UEs to compensate for phase error. Therefore, RAN1 should first agree to support the overlap between hops, and then discuss a UE capability related to overlap.

Proposal 3:

- RAN1 should first agree to support the overlap between hops, and then discuss a UE capability related to overlap.

Regarding the capability related to overlap, it seems companies have different intentions in mind. Some companies may intend that the support of measurement on overlapping frequency resources is to be separate capability from the measurement on DL PRS with FH, while some other companies may intend that just number of hops and/or amount of overlapped resources are to be reported by UE as part of the capability for support of measurement on DL PRS with FH. We think the intention of UE capability may need to be clarified first.

Observation 1:

- The intention of UE capability may need to be clarified first.

We discuss the amount of overlapped resources from the Alt.1 perspective in this contribution. The appropriate overlap size should be discussed with following considerations.

- It depends on the scenarios such as Doppler.

- The tradeoff should be considered, i.e., the total bandwidth will be reduced if too much frequency is overlapped while number of hops is limited by MG length.

The number of PRBs allocated for the DL-PRS Resource is described in dl-PRS-ResourceBandwidth in TS 37.355 [3].

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

In Rel-17, the minimum number of PRBs for DL-PRS resource is 24 PRBs and the bandwidth size is defined by the step of 4 PRBs. Therefore, 4 PRBs overlap may be a reasonable starting point from the specification impact perspective. Of course, RAN1 also needs to evaluate the phase shift compensation performance under the parameters assumption e.g., whether the 4 PRBs overlap is sufficient for phase shift compensation. It should be noted that if the overlap size needs to be larger than 4 PRBs, it is preferable that the value is multiple of 4 PRBs.

Observation 2:

- The appropriate overlap size should be discussed with following considerations.

  - It depends on the scenarios such as Doppler.

  - The tradeoff should be considered, i.e., the total bandwidth will be reduced if too much frequency is overlapped.

Proposal 4:

- If the number of overlapping frequency resources in adjacent hops is a UE capability, 4 PRBs overlap may be a reasonable starting point from the specification impact perspective.

  - If the overlap size needs to be larger than 4 PRBs, it should be multiple of 4 PRBs.

- Discussion on UL SRS frequency hopping

  - SRS hopping configuration

Regarding the SRS hopping configuration, the following agreement was made at the last meeting [2].

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

We would like to discuss the FFS issue in this contribution. If the time gap between hops is large, accuracy performance may degrade due to potential large phase and channel varying. Therefore, we think LS reply from RAN4 regarding the switching time values should be taken into account, and considering that RAN1 agreed DL PRS Rx hopping within a DL PRS resource at the last meeting, RAN1 should discuss further to support UL SRS hopping within a SRS resource depending on the switching time values.

Proposal 5:

- LS reply from RAN4 regarding the switching time values should be taken into account, and considering that RAN1 agreed DL PRS Rx hopping within a DL PRS resource at the last meeting, RAN1 should discuss further to support UL SRS hopping within a SRS resource depending on the switching time values.

- Conclusion

In this contribution, we discussed potential enhancements for positioning for RedCap UEs. Based on the discussion, we made following observations and proposals.

Observation 1:

- The intention of UE capability may need to be clarified first.

Observation 2:

- The appropriate overlap size should be discussed with following considerations.

  - It depends on the scenarios such as Doppler.

  - The tradeoff should be considered, i.e., the total bandwidth will be reduced if too much frequency is overlapped.

Proposal 1:

- RAN1 should support a single instance of a MG for DL PRS FH.

Proposal 2:

- RAN1 may not need to support PPW with FH.

Proposal 3:

- RAN1 should first agree to support the overlap between hops, and then discuss a UE capability related to overlap.

Proposal 4:

- If the number of overlapping frequency resources in adjacent hops is a UE capability, 4 PRBs overlap may be a reasonable starting point from the specification impact perspective.

  - If the overlap size needs to be larger than 4 PRBs, it should be multiple of 4 PRBs.

Proposal 5:

- LS reply from RAN4 regarding the switching time values should be taken into account, and considering that RAN1 agreed DL PRS Rx hopping within a DL PRS resource at the last meeting, RAN1 should discuss further to support UL SRS hopping within a SRS resource depending on the switching time values.

References

- RP-223549, 3GPP RAN#98-e, “New WID on Expanded and Improved NR Positioning”, Electronic Meeting, Dec. 2022.

- 3GPP RAN1#112, Chair’s Notes, Feb. 2023.

- 3GPP TS 37.355 V17.3.0 (2022-12), LTE Positioning Protocol (LPP)