3GPP TSG RAN WG1 #112bis-e	 R1-2303821

Online, April 17th – April 26th, 2023

Agenda item:       9.5.2

Source:	IIT Kanpur, CEWiT

Title:	Discussion on NR DL and UL carrier phase positioning

Document for:     Discussion and Decision

# Introduction

In Rel-16 native NR, positioning support was standardized, and in Rel-17, enhancements were made. At RAN#94, a new SI “Study on expanded and improved NR positioning” was approved for enhancements for Rel-18 NR positioning. One of the objectives of this enhancement was to introduce the carrier phase-based positioning method for further accurate positioning realizing in sub meters.

The study was concluded in the meeting RAN1#111, and based on the agreements of the study, the following work item description was endorsed in RAN#98e [1] for normative work from the carrier phase-based positioning perspective.

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

The normative phase of the carrier phase positioning commenced in the last meeting, RAN1#112 [3], following agreements made during the meeting.

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

This contribution provides further views on physical layer measurements and signaling to support NR DL and UL carrier phase positioning for Rel-18 NR positioning considering agreements in the last meeting.

# Discussion

## Carrier-Phase measurement

To support carrier phase positioning, new measurements should be defined. In the last meeting following agreement was made.

Agreement

To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced:

- DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP.

  - FFS: The detailed definition of the DL CP

- DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs

  - FFS: The detailed definition of the DL CPD

To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced:

- UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE.

  - FFS: The detailed definition of the UL CP

Further commenting on FFS, the carrier phase could be measured at different frequency granularities, such as at the subcarrier level, DC carrier level, or different positioning frequency layers. As it is well understood that phase is a sensitive measurement, reporting the carrier phase of multiple subcarriers will provide robustness against receiver noise.

Proposal 1: The carrier phase measurement, at least at the subcarrier level, should be supported for carrier phase positioning.

Observation 1: Multiple subcarriers will provide robustness against receiver noise by performing noise averaging.

Proposal 2: For UE-assisted carrier-phase-based positioning, carrier-phase measurement over multiple subcarriers should be reported to help mitigate the effects of noise.

Proposal 3: The frequency location of the subcarriers and the number of subcarriers used for carrier phase reporting should be conveyed to the TRP and/or to the UE performing the carrier phase (CP) measurement.

## Carrier Phase measurement with respect to Tx and Rx antenna

Multipath and a NLOS-dominant-channel inherently deteriorate the positioning performance. It is clearly observed in the simulations study that the mitigation of the effect of the multipath improves the performance of positioning methods.

Further, the AOD and AOA measurements over PRS, either in DL or UL direction, are of great help in identifying the LOS and NLOS signals and mitigating the NLOS bias in the measurement. Carrier phase measurements across antennas can give the angle information at the receiver.  Further, if the angle of departure w.r.t. to the transmitter is known, using the measured angle of arrival at the receiver can help in defining LOS/NLOS confidence parameter. E.g. in DL-AOD-based positioning measurements, along with RSRP per beam, if the UE is allowed to report carrier phase measurement across each receiving antenna, then it can be used to estimate the DL-AOA with fair accuracy to decide the LOS or NLOS nature of the path experienced by the PRS between gNB/TRP and UE. During the study phase, the benefits of reporting the phase measurements across TRP Rx antennas were discussed. Therefore, reporting of the phase measurements across UE/TRP Rx, and TRP Rx antennas should be supported in Rel 18.

Observation 2: To achieve the desired accuracy in carrier phase-based positioning, multipath mitigation techniques are necessary.

Observation 3: The AOD and AOA measurements with respect to either in DL or UL link will be important to mitigating multipath.

Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported.

Proposal 5: The carrier phase reported should be mapped to the reference point. The reference point shall be the antenna connector of the UE/TRP.

Further, UEs or TRPs should indicate the LOS-NLOS indicator corresponding to the carrier phase measurements. At least reporting of the carrier phase of the first path, and optionally, the additional paths should be supported.

Agreement

NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency.

- FFS: the reference point of the UL RSCP

- FFS: whether/how the measurement timing is defined

- Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion

- Note: The support of MIMO SRS for positioning is transparent to UE

Agreement

Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Proposal 6: For carrier phase-based positioning, reporting of carrier phase measurement corresponding to the first path should be supported. The measurement corresponding to the additional path could be optionally supported.

Proposal 7: The RSRPP should be reported across all the additional paths if the carrier phase of additional paths is reported. The number of paths reported is UE capability.

## Integer Ambiguity Resolution

The carrier phase-based positioning method is viewed as a standalone as well as a non-standalone method.

In the case of the non-standalone method, the carrier phase will be reported along with the UEs/TRPs measurement of legacy positioning methods e.g., DL/UL-TDOA, M-RTT, DL-AOD, UL-AOA, etc. Once we report the carrier phase along with the legacy measurement, it will inherently act as smoothing of legacy measurement. In the last meeting, the following agreement was made as a starting point to enable the CPP.

Agreement

For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF

- FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements

Proposal 8: In Rel 18, for integer ambiguity resolution of carrier phase positioning, at least RSTD and RTOA among the existing timing-based measurements should be supported for UE-assisted and UE-based positioning.

In the case of the standalone method, the integer ambiguity needs to be resolved. For UE-assisted positioning, the LMF could use the virtual carrier method where the measuring TRP or UE could report the phase difference between the phases of two or more than two frequencies, where the phase could be measured at subcarrier level, DC carrier level or different positioning frequency layers.

It is found that for standalone CPP, the difference between the phase of subcarriers helps in the swift resolution of integer ambiguity.

Agreement

To support NR carrier phase positioning, further consider the following options:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

Observation 4: The difference between the carrier phase of multiple subcarriers helps in swift integer ambiguity resolution.

Proposal 9: For carrier-phase-based positioning, the difference between the carrier phase of multiple subcarriers, along with the frequency difference with respect to the subcarriers, should be reported, which could be introduced as a new measurement.

Further, for UE-based positioning, the LMF could explicitly convey the integer ambiguity to the target device. The LMF could convey the exact value of integer ambiguity or may convey a possible range of integer ambiguity.

Proposal 10: For UE-based carrier phase positioning, the integer ambiguity could be conveyed to the UE.

Option 1: Single value of Integer ambiguity.

Option 2: A possible range of integer ambiguity

## UE Capabilities for CPP

For NR carrier phase positioning, the new set of UE capabilities should be defined. This capability information includes information like whether UE supports carrier phase positioning or not. If yes, then standalone carrier phase positioning or non-standalone carrier phase positioning (i.e. in combination with Rel-17 positioning methods).

Observation 5: New UE capabilities are required for NR carrier phase positioning.

Proposal 11: For NR carrier phase positioning, the UE capabilities should include the following:

- Frequency range supported by the UE

- Frequency granularity supported by the UE to enable carrier phase measurement

  - DC carrier

  - Subcarrier

  - Positioning frequency level

- Measurement reference point granularity supported by the UE

  - Single measurement reference point supported

  - Multiple measurement reference points supported

    - Per Rx antenna

    - Per receiver RF chain

    - Group of Rx chain/antenna

- The supported integer ambiguity resolution method (UE-based CPP)

  - Virtual carrier method

  - Psudo range method (in the combination of existing measurements)

- Whether the measuring UE supports LoS/NLoS identification (UE-based CPP)

- The PRS/SRS configuration supported by the UE

## Additional information along with carrier phase measurement

Additional information, apart from the phase measurement(s), should be conveyed to the LMF for an accurate position estimate.

Proposal 12: The additional information apart from the carrier phase measurement should include:

- The specific frequency domain resource(carrier, subcarrier, PFL) corresponding to the carrier phase measurement(s).

- Measurement reference point, for example, an antenna ID or an antenna pannel ID.

- PRS/SRS resource ID and resource set ID.

- The receiver beam ID

- Inter frequency offset if multiple carrier phase measurements are reported, or carrier phase differences are reported.

- LoS/NLoS indication

- RSRP/RSRPP of the paths corresponding to carrier phase measurement

- The error sources corresponding to the measurements, such as Timing Error Group, Phase Error Group.

# Conclusion

This paper provides the following observations and proposals.

Proposal 1: The carrier phase measurement, at least at the subcarrier level, should be supported for carrier phase positioning.

Observation 1: Multiple subcarriers will provide robustness against receiver noise by performing noise averaging.

Proposal 2: For UE-assisted carrier-phase-based positioning, carrier-phase measurement over multiple subcarriers should be reported to help mitigate the effects of noise.

Proposal 3: The frequency location of the subcarriers and the number of subcarriers used for carrier phase reporting should be conveyed to the TRP and/or to the UE performing the carrier phase (CP) measurement.

Observation 2: To achieve the desired accuracy in carrier phase-based positioning, multipath mitigation techniques are necessary.

Observation 3: The AOD and AOA measurements with respect to either in DL or UL link will be important to mitigating multipath.

Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported.

Proposal 5: The carrier phase reported should be mapped to the reference point. The reference point shall be the antenna connector of the UE/TRP.

Proposal 6: For carrier phase-based positioning, reporting of carrier phase measurement corresponding to the first path should be supported. The measurement corresponding to the additional path could be optionally supported.

Proposal 7: The RSRPP should be reported across all the additional paths if the carrier phase of additional paths is reported. The number of paths reported is UE capability.

Proposal 8: In Rel 18, for integer ambiguity resolution of carrier phase positioning, at least RSTD and RTOA among the existing timing-based measurements should be supported for UE-assisted and UE-based positioning.

Observation 4: The difference between the carrier phase of multiple subcarriers helps in swift integer ambiguity resolution.

Proposal 9: For carrier-phase-based positioning, the difference between the carrier phase of multiple subcarriers, along with the frequency difference with respect to the subcarriers, should be reported, which could be introduced as a new measurement.

Proposal 10: For UE-based carrier phase positioning, the integer ambiguity could be conveyed to the UE.

Option 1: Single value of Integer ambiguity.

Option 2: A possible range of integer ambiguity

Observation 5: New UE capabilities are required for NR carrier phase positioning.

Proposal 11: For NR carrier phase positioning, the UE information should include the following.

- Frequency range supported by the UE

- Frequency granularity supported by the UE to enable carrier phase measurement

  - DC carrier

  - Subcarrier

  - Positioning frequency level

- Measurement reference point granularity supported by the UE

  - Single measurement reference point supported

  - Multiple measurement reference points supported

    - Per Rx antenna

    - Per receiver RF chain

    - Group of Rx chain/antenna

- The supported integer ambiguity resolution method (UE-based CPP)

  - Virtual carrier method

  - Psudo range method (in the combination of existing measurements)

- Whether the measuring UE supports LoS/NLoS identification (UE-based CPP)

- The PRS/SRS configuration supported by the UE

Proposal 12: The additional information apart from the carrier phase measurement should include.

- The specific frequency domain resource(carrier, subcarrier, PFL) corresponding to the carrier phase measurement(s).

- Measurement reference point, for example, an antenna ID or an antenna pannel ID.

- PRS/SRS resource ID and resource set ID.

- The receiver beam ID

- Inter frequency offset if multiple carrier phase measurements are reported, or carrier phase differences are reported.

- LoS/NLoS indication

- RSRP/RSRPP of the paths corresponding to carrier phase measurement

- The error sources corresponding to the measurements, such as Timing Error Group, Phase Error Group.

# References

- RP-223549 New WID on Expanded and Improved NR Positioning

- RAN1#111 meeting chairman notes

- RAN1#112 FL summary