3GPP TSG-RAN WG1 Meeting #113	Draft R1-2305957

Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	9.6.1

Title:	FL summary #2 on Rel-18 RedCap UE complexity reduction

Source:	Moderator (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This feature lead (FL) summary (FLS) concerns the Rel-18 work item (WI) on enhanced support of reduced capability (RedCap) NR devices [1, 2]. The final FLS from the previous RAN1 meeting can be found in [3], and a RAN1 agreement summary is available in [4].

The core part of the WI [1] has the following objective and notes related to further reduced UE complexity:

Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

RAN#99 discussed whether UE peak data rate reduction (“PR1”) should be supported as a standalone feature or only in combination with UE BB bandwidth reduction (“BW3/PR3”) and endorsed the following proposal [5], where the different nicknames for the UE complexity reduction features (“PR1” and “BW3/PR3”) originate from TR 38.865 [6].

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is same including unicast and broadcast respectively. Note 2: PRB processing capability of “Rel-18 eRedCap: UE capable of 20MHz + PR1” is not limited to “25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS” and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

This document summarizes contributions [7] – [36] submitted to agenda items 9.6 and 9.6.1, and contributions [37-39] submitted to another agenda item, and the following email discussion:

[113-R18-RedCap] Email discussion on eRedCap – Johan (Ericsson) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, Tdoc number of the moderator summary for online session, etc

The issues in this document are tagged and color coded with High Priority or Medium Priority. The FLS from the initial discussion round can be found in [41]. The issues that are in the focus of this round are furthermore tagged FL3.

Follow the naming convention in this example:

- eRedCapFLS2-v000.docx

- eRedCapFLS2-v001-CompanyA.docx

- eRedCapFLS2-v002-CompanyA-CompanyB.docx

- eRedCapFLS2-v003-CompanyB-CompanyC.docx

If needed, you may “lock” a discussion document for 30 minutes by creating a checkout file, as in this example:

- Assume CompanyC wants to update eRedCapFLS2-v002-CompanyA-CompanyB.docx.

- CompanyC uploads an empty file named eRedCapFLS2-v003-CompanyB-CompanyC.checkout

- CompanyC checks that no one else has created a checkout file simultaneously, and if there is a collision, CompanyC tries to coordinate with the company who made the other checkout (see, e.g., contact list below).

- CompanyC then has 30 minutes to upload eRedCapFLS2-v003-CompanyB-CompanyC.docx

- If no update is uploaded in 30 minutes, other companies can ignore the checkout file.

- Note that the file timestamps on the server are in UTC time.

In file names, please use the hyphen character (not the underline character) and include ‘v’ in front of the version number, as in the examples above and in line with the general recommendation (see slide 11 in R1-2304302), otherwise the sorting of the files will be messed up (which can only be fixed by the RAN1 secretary).

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document. Companies are invited to enter the contact info in the table below.

FL3 Question 1-1a: Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| FUTUREWEI | Vip Desai | vipul.desai@futurewei.con |
| Panasonic | Shotaro Maki | maki.shotaro@jp.panasonic.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| Spreadtrum | Sicong Zhao | Sicong.zhao@unisoc.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| Vivo | Lihui Wang | wanglihui@vivo.com |
| LG Electronics | Seungjin Ahn | Seungjin.ahn@lge.com |
| MediaTek | Chiou-Wei Tsai | cw.tsai@mediatek.com |
| Nokia, NSB | Rapeepat Ratasuk | rapeepat.ratasuk@nokia.com |
| Qualcomm | Yongjun Kwak | yongkwak@qualcomm.com |
| NEC | Takahiro Sasaki | takahiro.sasaki@nec.com |
| Sierra Wireless | Serkan Dost | sdost@sierrawireless.com |
| Ericsson | Sandeep Narayanan Kadan Veedu | sandeep.narayanan.kadan.veedu@ericsson.com |
| CMCC | Tuo Yang, Lijie Hu | yangtuo@chinamobile.com; hulijie@chinamobile.com |
| OPPO | Zhisong Zuo | zuozhisong@oppo.com |

# 2	UE BB bandwidth reduction

## 2.0	Earlier agreements

RAN1 has made the following agreements for UE BB bandwidth reduction [4]:

Initial BWP Agreement: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Number of PRBs Agreement: For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number PUSCH bandwidth Agreement: For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. UE post-FFT buffer size Conclusion: For UE BB complexity reduction, for broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz Unicast PDSCH bandwidth Agreement: - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e SIB1/OSI transmission Conclusion: For UE BB complexity reduction, broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. Agreement: - For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation). The scheduling of SIB1 PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. - For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation). The scheduling of OSI PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Paging bandwidth Agreement: From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. RAR bandwidth, random access timeline, and early indication Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs Agreement: Down-select one among the following options in RAN1#113: - Option 1: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 2: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 3: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - FFS: Whether legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 4: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). Agreement: The potential timeline relaxations for the following cases are FFS: - For 2-step RACH: - Case 2a: Between reception of fallbackRAR and transmission of Msg3 - Case 2b: Between reception of successRAR and transmission of corresponding HARQ-ACK - For 4-step RACH: - Case 4a: Between reception of RAR PDSCH in which UE does not correctly receive the transport block and upcoming transmission of PRACH - Case 4b: Between reception of RAR with RAPID which is not associated with the corresponding PRACH transmission and upcoming transmission of PRACH Msg4 bandwidth Agreement: Confirm the following working assumption by assuming that Msg3 indication is available: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Agreement: Final LS R1-2304262 is endorsed Simultaneous reception Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Conclusion: For UE BB bandwidth reduction, for autonomous SI acquisition, the following paragraph in TS 38.214 clause 5.1 still applies: - “The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.” - FFS: Msg4 PDSCH scheduled by TC-RNTI case

## 2.1	Random access timeline and early indication

### 2.1.1	Msg2-Msg3 timeline and Msg1 indication

RAN1#112bis-e [4] agreed that a down-selection between four options related to Msg2-Msg3 timeline relaxation and potential separate early indication in Msg1 should take place in RAN1#113.

Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Agreement: Down-select one among the following options in RAN1#113: - Option 1: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 2: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported. - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 3: - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - FFS: Whether legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - Option 4: - For the “FFS: value(s) of X”, - X = 0.5/0.25 ms for 15/30 kHz SCS - Note: Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

Among the submitted contributions, most contributions [8, 9, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 23, 25, 26, 27, 31, 32, 34, 35, 36] express support for Option 3 or some slightly modified version of it. However, the contributions express different views on how to resolve the FFS on whether legacy default TDRA table Δ are reused or not.

Furthermore, a significant number of contributions [8, 9, 14, 20, 22, 24, 27, 28, 32, 36] support Option 4, a few contributions [8, 10, 22, 31] support Option 1, and a couple of contributions [17, 33] support Option 2.

FL1 High Priority Question 2.1.1-1a: Companies are invited to give each one of the following options a grade:

- Option 1

- Option 2

- Option 3a = Option 3, and legacy default TDRA table and Δ are reused

- Option 3b = Option 3, and legacy default TDRA table and Δ are not reused (use comment field if needed)

- Option 4

Please use the following grade scale (where there is no restriction on the number of times a grade can be used).

- +1 = preferred

- 0 = neutral/ok

- -1 = not preferred

| Company | Grade (+1/0/-1) for each one of Options 1/2/3a/3b/4 |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3a | 3b | 4 |  |
| Nordic | -1 | -1 | 1 | 0 | -1 |  |
| FUTUREWEI | -1 | -1 | +1 | 0 | +1 | We are open to supporting option 3b |
| Panasonic | -1 | -1 | +1 | 0 | 0 |  |
| DOCOMO | -1 | -1 | 0 | +1 | +1 | Especially for option 3a/3b, to enable Rel-18 eRedCap specific TDRA configuration for Msg3 other than the expansion of legacy default TDRA table and Δ, we suggest to introduce new RRC parameter in pusch-CofingCommon which is specific to Rel-18 eRedCap. |
| Spreadtrum | -1 | -1 | +1 | 0 | -1 |  |
| CATT | +1 | 0 | -1 | -1 | -1 | We may reconsider Msg1 early indication but MsgA PRACH early indication should not be supported. This should be a package. |
| Lenovo | -1 | -1 | +1 | 0 | 0 |  |
| Vivo | +1 | 0 | -1 | -1 | -1 |  |
| LG | -1 | -1 | 0 | +1 | -1 | Besides default TDRA Table and Δ updated, as an alternative, various solutions can be discussed to transmit Message 3 successfully. |
| MediaTek | -1 | +1 | 0 | 0 | -1 | We have provided analysis on why X=1 slot is needed. Option 1 and Option 4, both with X=0.5 slot, are unacceptable to us. |
| Nokia, NSB | 0 | -1 | +1 | +1 | 0 | We think that TDRA extension or separate table can be considered if Msg1 early indication is supported |
| Qualcomm | +1 | 0 | 0 | -1 | +1 | We do not see any motivation to consider the modification of TDRA table or Δ |
| Sharp | -1 | -1 | +1 | +1 | 0 | It can be discussed how to extend the timing for some cases that all rows in default TDRA table cannot meet the minimal timing requirement. |
| NEC | -1 | -1 | +1 | 0 | +1 |  |
| Sierra Wireless | -1 | -1 | +1 | 0 | +1 |  |
| Xiaomi | -1 | -1 | +1 | -1 | +1 | Even with X=1/0.5ms for 15/30 kHz SCS, there is no need to enhance the legacy TDRA table. |
| Ericsson | 0 | -1 | 0 | -1 | +1 | Although our preference is Option 4, Option 1 or Option 3a could be a good compromise. We support introducing PUSCH TDRA in pusch-ConfigCommon that is specific to Rel-18 eRedCap UEs, as also suggested by DOCOMO. |
| SONY | -1 | -1 | +1 | 0 | -1 |  |
| CMCC | +1 | 0 | -1 | -1 | 0 | Our preference is no EI. That is option 1, and option 2 is also OK, we can compromise to option4, don’t support option 3x. |
| OPPO | +1 | +1 | -1 | -1 | -1 |  |

The responses received for Question 2.1.1-1a are summarized in the following table:

|  | Option 1 | Option 2 | Option 3a | Option 3b | Option 4 |
| --- | --- | --- | --- | --- | --- |
| Preferred | 5 | 2 | 11 | 4 | 7 |
| Neutral/ok | 2 | 4 | 5 | 9 | 5 |
| Not preferred | 13 | 14 | 4 | 7 | 8 |
| Sum of grades | -8 | -12 | 7 | -3 | -1 |

Based on the above, the following proposal can be considered.

FL2 High Priority Proposal 2.1.1-1b:

Down-select one among the following options in RAN1#113:

- Option 1:

  - For the “FFS: value(s) of X”,

    - X = 0.5/0.25 ms for 15/30 kHz SCS

    - Note: Legacy default TDRA table and Δ are reused.

  - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported.

    - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

- Option 2:

  - For the “FFS: value(s) of X”,

    - X = 1/0.5 ms for 15/30 kHz SCS

    - Note: Legacy default TDRA table and Δ are reused.

  - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is not supported.

    - When Msg1 indication for Rel-17 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

- Option 3:

  - For the “FFS: value(s) of X”,

    - X = 1/0.5 ms for 15/30 kHz SCS

    - FFS: Whether legacy default TDRA table and Δ are reused.

  - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported.

    - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

- Option 4:

  - For the “FFS: value(s) of X”,

    - X = 0.5/0.25 ms for 15/30 kHz SCS

    - Note: Legacy default TDRA table and Δ are reused.

  - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported.

    - When Msg1 indication for Rel-18 RedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

In the online session on Monday 22nd May, the following proposal seemed to be acceptable as a compromise, except for the fourth main bullet and its sub-bullet (regarding the case when additional separate early indication in Msg1 is not configured for Rel-18 eRedCap UEs), so the proposal can be considered again. Companies are invited to provide constructive comments on how to update the fourth main bullet and its sub-bullet so it can also be acceptable to everyone.

#### FL3 High Priority Proposal 2.1.1-1c:

- For the “FFS: value(s) of X”,

  - X = 1/0.5 ms for 15/30 kHz SCS

- Legacy default TDRA table and Δ are reused.

- A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported.

  - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

- When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs will share the PRACH that is configured for Rel-17 RedCap UEs.

  - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs.

- Additional early indication in MsgA PRACH is not supported.

| Company | Y/N | Comments |
| --- | --- | --- |
| CATT2 | Y | The forth bullet should be kept. Otherwise: 1) It may cause a strange case when R17 RedCap UE uses separate initial BWP, but R18 RedCap UE uses legacy initial BWP, which is against RAN2’2 agreement. 2) If R18 RedCap UE does not use R17 RedCap PRACH, but use PRACH of normal UE instead, then the scheduling of normal UE is seriously impacted – gNB need to consider R18 RedCap UE when scheduling Msg2 and Msg3 for normal UE (DL coverage, BW, timeline of X, …) This is even worse. |
| DOCOMO |  | Considering that at most non-RedCap, Rel-17 RedCap and Rel-18 eRedCap UEs can be supported in a same cell, we should discuss at least the following cases; - Case 1: Neither Rel-17 RedCap specific Msg1 early indication or Rel-18 eRedCap specific Msg1 separate early indications is not configured - All of non-RedCap, Rel-17 RedCap and Rel-18 eRedCap UEs share the same PRACH configuration. - Case 2: Rel-17 RedCap specific Msg1 early indication is configured but Rel-18 eRedCap specific Msg1 separate early indication is not configured - At least Rel-17 RedCap and Rel-18 eRedCap can share PRACH configuration - FFS whether non-RedCap and Rel-18 eRedCap can share the same PRACH configuration - Case 3 (FFS): Rel-17 RedCap specific Msg1 early indication is not configured but Rel-18 eRedCap specific separate early indication is configured - non-RedCap and Rel-17 RedCap can share the same PRACH configuration - Case 4: Both Rel-17 RedCap specific Msg1 early indication and Rel-18 eRedCap specific Msg1 separate early indications are configured - Separate PRACH configuration is indicated for each non-RedCap, Rel-17 RedCap and Rel-18 eRedCap UE In our understanding, at least the above cases other than FFS cases are agreeable, thus we suggest to agree on them. The FFS cases are not discussed well so far, thus we are open to discuss them further or also fine to add as FFS in the proposal. |
| Samsung | Y | We are OK for the proposal 2.1.1-1c. |
| vivo | Y | We share CATT’s views on the Red Bullets. For the cases listed by DCM, our understanding for Case 3, it is supported based on the FL3 High Priority Proposal 2.1.1-1c, the subbullet of the third bullet. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). |
| FUTUREWEI |  | For the 4th bullet, we can accept "can" for flexibility. Also, OK for either “can” or “will” as a working assumption based on majority view. In any case, the detailed signaling is up to RAN2. |

### 2.1.2	UE behavior in the ‘otherwise’ case

Contribution [8] proposes to revise the highlighted bullet in the following earlier RAN1 agreement [4]-

Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH

The contribution proposes that the highlighted case can be treated in the same way as the other cases described in the following paragraph in clause 8.2 in TS 38.213:

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE shall be ready to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N1 assuming SCS configuration μ=0.

Companies are invited to comment on the following question.

#### FL1/FL3 Medium Priority Question 2.1.2-1a:

Should the highlighted bullet in the above agreement be revised to:

- Otherwise, the UE behavior is the same as if the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window (as specified in clause 8.2 in TS 38.213).

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | N | Up to UE implementation is just fine. |
| FUTUREWEI |  | In our understanding of the clause, there are 4 cases (two deal with DCI, one deals with incorrectly decoded PDSCH, and one deals with RAPID identification). The otherwise bullet in the agreement deals with processing. It seems to be unclear how the clause applies except possibly for the incorrectly decoded PDSCH. |
| DOCOMO | Y | We support to discuss the clarification on the highlighted case. |
| Spreadtrum | N | Up to UE implementation |
| CATT | N | Not urgent discussion. |
| vivo | N | No significant issue is found from earlier agreements. Hence no need to revise previous agreement. |
| LG | N | We think it is not needed |
| Nokia, NSB | N | We think it should be left to UE implementation as some UE might be able to still meet the timeline |
| Qualcomm | N | RAN1 already agreed that it is up to UE implementation. We prefer to keep the current agreement. |
| NEC | N |  |
| Xiaomi | N |  |
| Ericsson | Y | We think it is good to have a well-defined UE behavior for the case when Msg2-Msg3 timing is less than NT,1 + NT,2 + 0.5 + X ms. Otherwise, some UEs may follow same behavior as when the UE does not detect the RAR DCI, whereas other UEs may go ahead and transmit Msg3 anyway (if capable to do so). Also, the UEs that follow the former behavior is not required to follow the eRedCap “ready to transmit” timeline. We think such ambiguities can be avoided by having a well-defined UE behavior for the case above. |
| CMCC | N | Prefer original wording, which allows more flexibility of UE behavior. For example, if UE has enough capability and can process RAR with delay less than NT,1 + NT,2 + 0.5 + X ms and before scheduling of Msg3, UE can still transmit Msg3 and the random access may success. If it is changed to wording in clause 8.2 in TS 38.213, UE always need to transmit a PRACH, resulting in large access latency. |
| OPPO | N | They could be same meaning. May not need to clarify. |

### 2.1.3	Timeline in similar cases

RAN1#112bis-e made the following agreement [4] regarding timeline relation for other similar cases as the Msg2-Msg3 case described in previous sections.

Agreement: The potential timeline relaxations for the following cases are FFS: - For 2-step RACH: - Case 2a: Between reception of fallbackRAR and transmission of Msg3 - Case 2b: Between reception of successRAR and transmission of corresponding HARQ-ACK - For 4-step RACH: - Case 4a: Between reception of RAR PDSCH in which UE does not correctly receive the transport gblock and upcoming transmission of PRACH - Case 4b: Between reception of RAR with RAPID which is not associated with the corresponding PRACH transmission and upcoming transmission of PRACH

Contributions [8, 15, 17, 22, 26, 28, 29, 31, 32] express that the same timeline relaxation should be used for these cases as in the Msg2-Msg3 case described in earlier sections.

Contributions [11, 14, 18, 35] express that the same timeline relaxation should be used for Case 2a/4a/4b as in the Msg2-Msg3 the case described in earlier sections. Contributions [14, 35] express that timeline relaxation does not apply to Case 2b.

Contributions [10, 13, 16, 33] express that the same timeline relaxation should be used for Case 4a/4b as in the Msg2-Msg3 case described in earlier sections. Contributions [10, 13, 16, 23, 33] express that Case 2a/2b depends on the outcome of the MsgB PDSCH bandwidth discussion.

#### FL1/FL2/FL3 Medium Priority Proposal 2.1.3-1a:

- For UE BB bandwidth reduction, the same timeline relaxation as for the Msg2-Msg3 timeline applies at least for the following cases:

  - Case 4a: Between reception of RAR PDSCH in which UE does not correctly receive the transport block and upcoming transmission of PRACH

  - Case 4b: Between reception of RAR with RAPID which is not associated with the corresponding PRACH transmission and upcoming transmission of PRACH

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y |  |
| Panasonic | Y |  |
| DOCOMO | Y |  |
| CATT | Y |  |
| vivo | Y |  |
| LG | Y | 2 cases should be considered. |
| MediaTek | Y |  |
| Nokia, NSB | Y |  |
| Qualcomm | Y |  |
| Sharp | Y |  |
| NEC | Y |  |
| Xiaomi | Y |  |
| Ericsson | Y |  |
| CMCC | Y |  |
| OPPO | Y |  |

Contribution [23] expresses that there are some additional similar cases to consider.

#### FL1/FL3 Medium Priority Question 2.1.3-2a:

What (if any) other similar cases should be considered?

| Company | Comments |
| --- | --- |
| DOCOMO | Similar timeline relaxation to case 4a/4 should be applied between MsgB PDSCH and PRACH or MsgA retransmission for 2-step RACH corresponds to the following case in 38.213 section 8.2A. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321].. |
| CATT | N |
| LG | If MsgB PDSCH BW can be allocated over 25 PRBs for 15KHz SCS or 12 PRBs for 30KHz, similar cases can be also applied to 2-step RACH procedure. |
| MediaTek | For 4-step RACH, we don’t identify other cases. For 2-step RACH, the following cases should be considered, if MsgB PDSCH is larger than 25/12 PRBs for 15/30 kHz SCS, - Between reception of fallbackRAR and transmission of Msg3 - Between reception of successRAR and transmission of corresponding HARQ-ACK - Msg1 PRACH or MsgA (PRACH and PUSCH) retransmission after the failure of MsgB reception or decoding |
| QC | Share the view of MediaTek |
| Xiaomi | For 2-step RACH, it is up to the MsgB PDSCH BW. So, we can wait until MsgB PDSCH bandwidth has a conclusion. |
| CATT2 | Regarding 2-step RACH, we should determine MsgB PDSCH BW first. |
| vivo | For case 2a and 2b, it can be discussed after determining the MsgB PDSCH BW. |

### 2.1.4	Early indication in MsgA PRACH

A few contributions [8, 18, 31] propose to support additional separate early indication in MsgA PRACH (at least if it is also supported in Msg1), whereas a couple of other contributions [15, 33] express that it is not needed. This question can be treated with low priority until the Msg1 case has been resolved.

FL1 Low Priority Question 2.1.4-1a: Should additional separate early indication in MsgA PRACH be supported?

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | N |  |
| FUTUREWEI |  | Can wait until Msg1 case is resolved |
| DOCOMO | Y | But we are fine to differ the discussion until the progress on Msg1-based separate early indication. |
| CATT | N |  |
| Lenovo | Y | Same view with DOCOMO |
| vivo | N |  |
| Nokia, NSB | N | Even if separate Msg1 early indication is supported, we do not see the need for MsgA PRACH given PUSCH early indication is already supported |
| Xiaomi | Y | Same view with DOCOMO. |
| CMCC | N |  |
| OPPO | N | Does that earlier indication duplicate PUSCH part, which already have earlier indication? |
| FL | Early indication in Msg3 PRACH is handled in Proposal 2.1.1-1c. |  |

## 2.2	Simultaneous reception

For simultaneous reception of multiple broadcast channels, 38.214 clause 5.1 specifies the following:

The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs.

For simultaneous reception of a unicast channel and a broadcast channel, 38.214 clause 5.1 specifies the following:

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time. The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.

RAN1 has made the following conclusions [4]:

Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Conclusion: For UE BB bandwidth reduction, for autonomous SI acquisition, the following paragraph in TS 38.214 clause 5.1 still applies: - “The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.” - FFS: Msg4 PDSCH scheduled by TC-RNTI case

### Autonomous SI acquisition

The contributions express the following views regarding the FFS for Msg4 during autonomous SI acquisition.

- Contributions [8, 11, 14, 15, 18] propose that Msg4 PDSCH scheduled by TC-RNTI should be treated in the same say as unicast PDSCH (i.e., “The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, CS-RNTI, or TC-RNTI during a process of autonomous SI acquisition”).

- Contributions [11, 13, 22, 33] propose that the UE should decode the Msg4 PDSCH in this case, but that the decoding priority should be specified if the Msg4 PDSCH is scheduled with more than 25 PRBs for 15 kHz or more than 12 PRBs for 30 kHz SCS.

- Contribution [10] proposes that for autonomous SI acquisition, the UE is not required to process a Msg4 PDSCH scheduled with more than 25 PRBs for 15 kHz SCS or more than 12 PRBs for 30 kHz SCS.

- Contribution [12] argues that according to the previous agreement, if Msg4 is wider than 5MHz, the UE is not required to process Msg4 and that no further agreement is needed.

- Contribution [32] proposes to extend the earlier RAN1 conclusion to say that “For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR/PDSCH is scheduled with TC-RNTI”.

- Contributions [9, 26, 28] propose to leave this case up to UE implementation.

- Contribution [20] propose to wait for RAN2’s reply to the LS in [40].

Companies are invited to provide further comments below.

#### FL1/FL3 Medium Priority Question 2.2.1-1a:

Companies are invited to express their preference regarding the UE behavior in case of Msg4 PDSCH decoding during a process of autonomous SI acquisition (considering the arguments provided in the contributions listed above).

| Company | Comments |
| --- | --- |
| Nordic | We support also: Contributions [8, 11, 14, 15, 18] propose that Msg4 PDSCH scheduled by TC-RNTI should be treated in the same say as unicast PDSCH |
| Panasonic | No spec change would be needed. It means that the reception of Msg4 is up to UE implementation.as well as the legacy UE behavior. We do not see the reason why only eRedCap UE with BB BW reduction is mandated to receive Msg4 in this case. |
| CATT | Buffer SI and decode Msg4, and provide HARQ-ACK feedback for Msg4. No spec change. |
| vivo | We do not think additional agreement is needed given following already made agreement: Agreement: Confirm the following working assumption by assuming that Msg3 indication is available: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. |
| LG | We think that no spec change is needed including Msg4 with TC-RNTI. |
| MediaTek | For Rel-18 eRedCap during a process of autonomous SI acquisition, when Msg4 PDSCH with TC-RNTI is scheduled with another PDSCH with SI-RNTI, - If Msg4 PDSCH is not greater than 25/12 PRBs in 15/30kHz SCS, UE is expected to decode the Msg4 PDSCH scheduled by TC-RNTI. Otherwise, UE is expected to decode the PDSCH scheduled by SI-RNTI. |
| Nokia, NSB | We agree Msg4 PDSCH scheduled by TC-RNTI should be treated in the same say as unicast PDSCH and no specification change is needed |
| Qualcomm | In the current spec, there is nothing defined for Msg4 PDSCH decoding during a process of autonomous SI acquisition. There is no big motivation to have different behavior only for Rel-18 eRedCap UE. We propose to remove the FFS from the agreement and just reuse the behavior defined in the current spec. |
| NEC | Share view with Nordic. |
| Xiaomi | Share view with Nordic. |
| Ericsson | Same behavior as for unicast PDSCH should be applied for Msg4 PDSCH scheduled by TC-RNTI |
| CMCC | The handling of Msg4 PDSCH generally follows legacy behavior except when Msg4 is larger than 5MHz. If bandwidth of Msg4 is larger than the UE can process per slot, according to previous agreement, UE is not required to process Msg4, the UE decodes SI PDSCH. |
| OPPO | We think no specification needed. |
| DOCOMO | We tend to agree with Qualcomm that Msg4 PDSCH scheduled with TC-RNTI and autonomous SI acquisition is not expected to be overlapped in time domain for legacy UEs. In that sense, we didn’t get the motivation to treat this case only for Rel-18 eRedCap. Thus, we are fine to just remove FFS. |

### P-RNTI triggered SI acquisition

RAN1#112bis-e discussed this proposal for P-RNTI triggered SI acquisition without reaching a conclusion [3]:

High Priority Proposal 2.5-2a: - Conclusion: For UE BB bandwidth reduction, for P-RNTI triggered SI acquisition, the following paragraph in TS 38.214 clause 5.1 still applies: - “On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.” - FFS: the Msg4 PDSCH case

Contributions [8, 10, 11, 12, 13, 14, 15, 16, 20, 23, 26, 28, 32, 33, 34, 35] present their views on simultaneous reception during P-RNTI SI acquisition. Contribution [10] proposes to consider the following options:

- Option 1: The UE prioritizes reception of unicast PDSCH over SI PDSCH triggered by P-RNTI.

- Option 2: The UE may skip decoding of unicast PDSCH but decodes SI PDSCH triggered by P-RNTI.

- Option 3: The prioritization between reception of unicast and SI PDSCH triggered by P-RNTI is up to the UE implementation.

- Option 4: During a process of P-RNTI triggered SI acquisition, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Companies are invited to comment on the above list of options.

#### FL1/FL3 Medium Priority Question 2.2.2-1a:

Can the above list of options be used as a basis for further discussion and down-selection? If you think the list should be updated somehow, please elaborate in the comment field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Option 5 | For PR3 UE, when UE is scheduled to receive in RRC connected mode a non-overlapping SI and unicast PDSCH (if any) spanning more than 5MHz in a slot n, UE is not required to receive in slot n+1. |
| Panasonic | Y in principle | For options 1-3, “unicast PDSCH” should be replaced with “the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI” as in the TS 38.214. For example, Option 2 can be updated as follows: • Updated Option 2: The UE may skip decoding of the scheduled unicast PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI but decodes SI PDSCH triggered by P-RNTI. Among 4 options, we prefer (updated) Option 2. |
| CATT |  | One clarification question, does ‘prioritize A’ mean ‘drop B’ in the proposal? We think the UE shall be able to decode both, by proper implementation, e.g. buffer SI but decode unicast PDSCH + provide HARQ-ACK at first. |
| vivo | Y | It is good to align companies understanding on above case when the total number of PRBs for the PDSCH scheduled with SI-RNTI and the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI is larger than the maximum number of PRBs that the UE can process per slot. Reply to CATT: my understanding is ‘prioritize A’ means ‘B may or may not be dropped’ |
| LG | N | Our preference is that P-RNTI triggered SI acquisition and autonomous SI acquisition should not be handled differently. |
| MediaTek |  | At least P-RNTI triggered SI acquisition for PWS/CMAS notification should be prioritized. |
| Qualcomm | Y but | Share the view of CATT on option 1. What would be the expected spec impact by option 1. Assuming that SI PDSCH has sufficient decoding time budget, prioritizing unicast PDSCH can be done by UE implementation meaning no spec impact. If it has different intention as vivo mentioned, the option has to be clarified further. |
| NEC |  | Share view with LG |
| Xiaomi |  | Same view as CATT |
| Ericsson | Y | Option 2 is preferred. |
| CMCC | Y | Fine to down-selection. |
| OPPO | N | Autonomous SI acquisition not specifically treated. |
| CATT2 |  | Thanks @vivo for feedback. If UE may or may not do something for the deprioritized behavior, do we need to capture the deprioritized behavior in the specification? It seems equal to ‘up to UE implementation’ and no spec impact? |
| DOCOMO |  | We share the same view with CATT that another option UE can decode both a unicast PDSCH and a SI-PDSCH triggered by P-RNTI should be taken into account. |
| vivo2 |  | Reply to CATT: For R18 eRedCap, in our view, it is needed since current spec requires the UE to decode both P-RNTI triggered SI PDSCH and PDSCH scheduled by C-RNTI, MCS-C-RNTI, or CS-RNTI. |
| Intel | Y | Option 2 is preferred. We share vivo’s view the discussion could focus on the case that the total number of PRBs for the SI PDSCH and the unicast PDSCH is larger than 5MHz. |

### 2.2.3	Unicast transmission and RAR

Contributions [16, 33] propose that decoding of RAR PDSCH should be prioritized over unicast PDSCH. The following proposal from contribution [33] can be considered.

#### FL1/FL3 Medium Priority Question 2.2.3-1a:

If a PDSCH is scheduled with RA-RNTI or MSGB-RNTI in slot n, UE is not expected to decode another PDSCH scheduled with C-RNTI, SI-RNTI, MCS-C-RNTI, G-RNTI for multicast or broadcast, MCCH-RNTI, G-CS-RNTI or CS-RNTI,

- in the same slot (i.e. slot n) if the PDSCH scheduled with RA-RNTI or MSGB-RNTI is not greater than 25/12 PRBs with 15/30kHz SCS;

- in slots n and n+1 if the PDSCH scheduled with RA-RANTI or MSGB-RNTI is greater than 25/12 PRBs with 15/30kHz SCS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y | Similar should apply in our opinion also for SI + unicast |
| CATT |  | This is a very corner case, if not completely impossible: For contention based RACH, the UE receiving RAR does not access the cell so no UE-specific RNTI is allocated to the UE. For contention free RACH, typically the UE is adjusting TA or HO to another cell. In either case it seems no unicast PDSCH will be sent to this UE. |
| vivo |  | There may be issues and we are open to discuss. |
| LG | N | We think that gNodeB can schedule it in order to evade this situation and it seems to be UE implementation. It is reasonable that SI and RAR should be handled similarly. They were not regarded differently in the past. |
| MediaTek | Y | Decoding of RACH messages should be prioritized as per legacy operation. @CATT, RACH can be triggered for a connected UE under multiple events (See TS38.300 for details.). One example that counters your argument is: gNB sends DL via unicast PDSCH to UE while UE has triggered RACH for UL grant request. |
| Qualcomm | N | Too much restriction on NW scheduling if the restriction is given to (n+1) slot |
| Xiaomi |  | We are open to discuss it. |
| CMCC | Y |  |
| CATT2 |  | Thanks @MTK for clarification, but current spec seems enough? The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, G-RNTI for multicast or broadcast, MCCH-RNTI, G-CS-RNTI or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time.. The current spec already precludes overlapping in the same slot n. Or the intension is to discuss slot n+1 only? |
| DOCOMO | N | We share the view with Qualcomm. Regardless of whether RAR/MsgB PDSCH BW is larger or smaller than 5MHz, the following prioritization/dropping rule in the current spec is sufficient. The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time.. |

### 2.2.4	Unicast transmission in HD-FDD

Contribution [39] proposes that a Rel-18 eRedCap HD-FDD UE should be capable of processing one additional unicast DCI scheduling PUSCH (as in TDD).

FL1 Low Priority Question 2.2.4-1a: Should a Rel-18 eRedCap HD-FDD UE be able to process one additional unicast DCI scheduling PUSCH (as in TDD)?

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y | There is a clear benefit for network spectral efficiency |
| Xiaomi | N |  |
| OPPO | N | Not see the requirement for it. |
| DOCOMO | N | This is not supported for Rel-17 RedCap, hence we don’t see the necessity to introduce only for Rel-18 eRedCap. |
| Sierra Wireless | Y | There is a benefit |

## 2.3	Msg3 PUSCH bandwidth

Contribution [16] proposes that the UE can restart the PRACH procedure if Msg3 is scheduled with more than 25 PRBs for 15 kHz SCS or more than 12 PRBs for 30 kHz SCS.

#### FL1/FL3 Medium Priority Proposal 2.3-1a:

- For UE BB bandwidth reduction, if Msg3 PUSCH is scheduled with more than 25 PRBs for 15 kHz or more than 12 PRBs for 30 kHz, the UE can restart the PRACH procedure.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y |  |
| FUTUREWEI |  | Perhaps this question seems be part of the clause cited in question 2.1.2-1a (related to RAPID identification) |
| CATT |  | We already agree that the UE is not expected to be scheduled with >5MHz Msg3. Sounds like a natural outcome. It seems just like a natural result without further conclusion. |
| vivo | N | The proposal contradicts with following agreements: Agreement: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. In addition, we recall that when above agreements for Msg.3 BW was made, all companies seem sharing the understanding that the TBS for Msg.3 is typically small, e.g. 56bits, the number of PRBs corresponding to 5MHz is sufficient even for non-RedCap UEs. |
| LG | Y | In the only case of no configuration on separate early indication for eRedCap, this behavior can be discussed. |
| MediaTek |  | If this is supported, it should apply to both R18 eRedCap UEs, i.e. UE BB bandwidth reduction and UE peak rate reduction, per Note4 in RAN#99 agreements, copied below for reference: Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: • Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1 |
| Qualcomm | N | Not required to be specified in the spec. |
| Sharp | Y | We share similar view with LG. |
| NEC |  | In an initial BWP where Rel-18 RedCap UE is supported, it would be an error case. |
| Xiaomi | N | The network scheduling is restricted by the following agreement: Agreement: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. So, it is an error case for eRedCap to be allocated with more than 25/12 PRBs for 15/30kHz SCS. |
| Ericsson |  | Wouldn’t similar case also exist for Rel-17 RedCap (i.e., Msg3 PUSCH for Rel-17 RedCap scheduled with larger than 20 MHz)? Since we haven’t specified any such behavior for Rel-17 RedCap, there might not be a need to specify the behavior for Rel-18 eRedCap. However, we are open to hearing more views from other companies. |
| CMCC |  | If Msg3 PUSCH is scheduled with more than 5MHz, R18 RedCap UE consider that it fails in access and can follow legacy behavior. |
| OPPO | Y | The behavior is OK, check further if the spec. should take care of it. |
| DOCOMO | Y | We support the proposal. We suggest to clarify whether such case can be happened first. In our view, legacy UE and Rel-18 eRedCap can share the same RA-RNTI and the Rel-18 eRedCap UE may receive RAR which schedules Msg3 PUSCH with larger BW than 5MHz for legacy UE. Thus, we support to clarify the UE behavior for this case, i.e., when the Msg3 PUSCH is scheduled with larger BW than 5MHz. |
| Intel | Y | We support the proposal. This issue is essentially same as msg4. I.e., due to the collision of selected PRACH preamble, it can happen that an eRedCap UE receives a RAR/UL grant that schedules Msg3 with >5MHz, since the Msg3 is targeting a Rel-17 RedCap UE or non-RedCap UE. |

## 2.4	Msg4 PDSCH bandwidth

Contribution [18] proposes to revise the earlier RAN1 agreement [4] on Msg4 PDSCH bandwidth to distinguish Msg4 PDSCH transmissions scheduled by different RNTIs.

#### FL1/FL2/FL3 Medium Priority Proposal 2.4-1a:

- Revise the following RAN1 agreement:

  - Confirm the following working assumption by assuming that Msg3 indication is available:

    - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

      - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

- to:

  - Confirm the following working assumption by assuming that Msg3 indication is available:

    - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH scheduled by TC-RNTI with resource allocation spanning a bandwidth of more than ~5 MHz per slot.

      - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

      - For Msg4 PDSCH scheduled by C-RNTI, limit its bandwidth in the same way as for unicast PDSCH.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y |  |
| FUTUREWEI |  | While the intent seems clear, the sub-bullets should be revised as shown in blue (the second bullet deals with TC-RNTI while the lowest bullet deals with C-RNTI) - Confirm the following working assumption by assuming that Msg3 indication is available: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH scheduled by TC-RNTI with resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. - For UE BB complexity reduction, for Msg4 PDSCH scheduled by C-RNTI, limit its bandwidth in the same way as for unicast PDSCH. |
| Panasonic | Y |  |
| CATT |  | Seems not urgent but OK. |
| LG | N | We think that any modification is not needed on the previous agreement. |
| MediaTek | Y |  |
| Nokia, NSB | N | Similar view as LG |
| Qualcomm | N | Not needed |
| NEC | N | To revise the agreement does not seem essential. |
| Xiaomi | Y | For Msg4 PDSCH scheduled by C-RNTI, its bandwidth is limited in the same way as for unicast PDSCH, so we think further modification is needed. |
| Ericsson | Y |  |
| CMCC | Y |  |
| DOCOMO | N | We don’t see the need. |
| vivo | Y |  |

## 2.5	MsgB PDSCH bandwidth

RAN1#112bis-e made the following agreement regarding the Msg4 PDSCH bandwidth [4]:

Agreement: Confirm the following working assumption by assuming that Msg3 indication is available: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

RAN1#112bis-e also sent an LS in [40] to inform RAN2 about the following case, to consider, if needed, the UE behavior in the RAN2 specifications, and ask RAN2 for feedback if any:

- For UE BB complexity reduction, the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process

RAN1#112bis-e also discussed this proposal on MsgB PDSCH bandwidth without reaching a conclusion [3]:

Medium Priority Proposal 2.9-1b: Assuming that MsgA indication is available, - For UE BB complexity reduction, a UE is able to receive a MsgB PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a MsgB PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Contributions [8, 11, 12, 13, 34] express that the MsgB bandwidth should be limited in a similar way as Msg4 (i.e., as in the above RAN1#112bis-e Proposal 2.9-1b).

Contributions [9, 10, 15, 16, 20, 28, 32] express that the MsgB bandwidth should instead be limited in a similar way as Msg2 (i.e., allowing a larger number of PRBs).

Contributions [14, 18] express that the MsgB successRAR bandwidth should be limited in a similar way as Msg4 but that the MsgB fallbackRAR bandwidth should be limited in a similar way as Msg2.

Contribution [33] expresses that the bandwidth of a MsgB scheduled with MSGB-RNTI should be limited in a similar way as Msg2 but that the bandwidth of a MsgB scheduled with C-RNTI should be limited in a similar way as Msg4.

Contribution [17] argues that the MsgB bandwidth requires further consideration due to its difference compared to both Msg2 and Msg4.

#### FL1/FL3 Medium Priority Question 2.5-1a:

Companies are invited to express their preference regarding the MsgB PDSCH bandwidth (considering the arguments provided in the contributions listed above).

| Company | Comments |
| --- | --- |
| Nordic | 5MHz as all PUSCH so far. |
| FUTUREWEI | The proposal presumed that MsgA indication was available and also is related to section 2.1.4. It seems the behavior for MsgB PDSCH bandwidth when MsgA indication is not available needs to be examined first |
| Panasonic | As MsgB can contain the messages to multiple UEs and is support scaling factor of Ninfo, it would be reasonable that the MsgB is allowed to be scheduled larger than 25/12 PRBs for 15/30 kHz SCS. |
| CATT | Still think it is reasonable to consider different handling methods for successRAR (as for Msg4) or fallbackRAR (as for Msg2), if we consider that MsgA PUSCH may fail. Otherwise, Msg4-like handling directly – the gNB shall be aware with the accessing UE is a Rel-18 RedCap UE. Just as other PDSCH. |
| vivo | As shown in the following MsgB MAC PDU structure, MsgB bandwidth should instead be limited in a similar way as Msg2 (i.e., allowing a larger number of PRBs) MSGB MAC PDU with MAC SDU(s) |
| LG | MsgB-RNTI is different from C-RNTI or TC-RNTI (CS-RNTI, MCS-C-RNTI) which is scheduled for one UE of unicast PDSCH and is almost same with SI-RNTI, P-RNTI or RA-RNIT which can be scheduled for multiple UEs. Multiple UEs’ MAC Control PDU contents can be multiplexed into a Message B according to TS 38.321. Additionally, the proposal seems to be problematic that the number of PRBS for Msg B PDSCH is allocated within 5MHz PRBs for SuccessRAR and the number of PRBS for Msg B PDSCH can be allocated over 5MHz PRBs for FallbackRAR. Then, FallbackRAR and SuccessRAR are not multiplexed in one Message B and two MsgB PDSCHs (one is for SuccessRAR and the other is for FallbackRAR) are operated in the same cell. We don’t know why this overhead and restriction is needed in 2-step RACH process for Rel-18 eRedCap. |
| MediaTek | Similar to Msg2, MsgB PDSCH is multicast containing responses to multiples UEs. Similar to Msg2, MsgB should be allowed to be larger than 25/12 PRBs while not greater than 20MHz as per legacy operation. |
| Nokia, NSB | MsgB bandwidth should be treated in the same way as Msg2, so we support scheduling larger than 25/12 PRBs for 15/30 kHz SCS |
| Qualcomm | MsgB PDSCH bandwidth should be limited in a similar way as Msg2 as multiple messages for more than one UEs can be multiplexed in the same channel. MsgB PDSCH has to be handled as other broadcast PDSCHs including Msg2. Also successRAR and fallbackRAR can be multiplexed together so that no differentiation is needed between those two. |
| NEC | MsgB bandwidth should be handled as Msg2 as MsgB may contain MAC subPDUs for multiple UE. |
| Xiaomi | Share similar view with CATT. |
| CMCC | Since MsgA indication is available, MsgB of R18 RedCap can be scheduled separately, proposal 2.9-1b in RAN1#112bis-e is ok. |
| DOCOMO | At least MsgB PDSCH scheduled with MsgB-RNTI should be treated in the same way as Msg2 PDSCH. |
| FUTUREWEI | Got clarification that MsgA indication refers to MAC CE. Similar understanding as Nokia. |

## 2.6	MBS PDSCH bandwidth

RAN1#112bis-e discussed potential restriction of the bandwidth for broadcast and multicast MBS PDSCH transmissions without reaching a conclusion [3]. Based on contributions [8, 18], the following proposals for broadcast and multicast, respectively, can be considered.

#### FL1/FL2/FL3 Medium Priority Proposal 2.6-1a:

For broadcast MBS PDSCH, allow the scheduling to be larger than 5MHz (as in legacy operation). The PDSCH repetition case is FFS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic | Y | Whether MBS PDSCH is capable or not is not able to be known to the network for IDLE mode as which UE support broadcast MBS PDSCH is not informed. Therefore, if the RedCap UE support MBS PDSCH, it should be same as non-RedCap UE. |
| CATT | Y |  |
| LG | Y | Broadcast MBS PDSCH should be regarded as broadcast SIB PDSCH. |
| Nokia, NSB | Y |  |
| Xiaomi | Y |  |
| Ericsson | Y |  |
| DOCOMO | Y | Basically we don’t see the need to discuss specific to MBS feature but fine with this proposal without any further optimization for MBS. |
| vivo | Y |  |

#### FL1/FL2/FL3 Medium Priority Proposal 2.6-2a:

For multicast MBS PDSCH, the number of PRBs scheduled in DCI is not larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic | Y | Should be restricted as the unicast. The multicast retransmission is UE specific and it should be limited to the capability. |
| CATT | FFS |  |
| LG | Optionally Y | For multicast MBS PDSCH, HARQ feedback for Multicast can be disabled by RRC configuration or DCI. multicast PDSCH is same as unicast PDSCH with HARQ feedback (ACK/NACK or NACK-Only), but multicast PDSCH can be same as Broadcast PDSCH with disabling HARQ feedback. We think that the number of PRBs for multicast can be different whether HARQ feedback is disabled or not. with HARQ feedback disabled, multicast PDSCH is not needed to be processed within one slot in the same as for unicast PDSCH. |
| Nokia, NSB | Y |  |
| Xiaomi | Y |  |
| Ericsson | Y |  |
| OPPO | Y |  |
| DOCOMO | Y | Same comment as Proposal 2.6-1a. |

# 3	UE peak data rate reduction

## 3.0	Earlier agreements

RAN1 has made the following agreements for UE peak data rate reduction [4]:

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement: - The minimum DL peak rate target (for FD-FDD) is 10 Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2.

RAN#99 discussed whether UE peak data rate reduction (“PR1”) should be supported as a standalone feature or only in combination with UE BB bandwidth reduction (“BW3/PR3”) and endorsed the following proposal [5], where the different nicknames for the UE complexity reduction features (“PR1” and “BW3/PR3”) originate from TR 38.865 [6].

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is same including unicast and broadcast respectively. Note 2: PRB processing capability of “Rel-18 eRedCap: UE capable of 20MHz + PR1” is not limited to “25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS” and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

RAN1#112bis-e discussed this proposal on the values of X and Y without reaching a conclusion [3]:

High Priority Proposal 3.1-1h: - For UE peak data rate reduction with UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = 3.2 - For UE peak data rate reduction without UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = 0.8 - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression. - FFS: Whether the 10-Mbps peak rate target is a minimum peak rate or a fixed peak rate.

The reason for the deadlock in RAN1#112bis-e was that companies had two different understandings of the RAN#99 decision listed in the introduction of this contribution. The question is whether the 10-Mbps peak rate target should be understood as a fixed peak rate target for all Rel-18 eRedCap UEs (regardless of what optional features they might support) or a minimum peak rate target (that might be exceeded by UEs that support optional features such as MIMO).

## 3.1	Relaxed constraints

As a potential way forward, companies are invited to consider the following updated proposal. Note that combinations with optional features is treated separately in the next section in this document.

#### FL1/FL2/FL3 High Priority Proposal 3.1-1a:

Agree the following (without any intention to indicate one way or the other whether the 10-Mbps peak rate target is a minimum peak rate or a fixed peak rate):

- For UE peak data rate reduction with UE BB bandwidth reduction,

  - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = 3.2

- For UE peak data rate reduction without UE BB bandwidth reduction,

  - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = [0.75 or 0.8]

  - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression.

- FFS: Whether the 10-Mbps peak rate target is a minimum peak rate or a fixed peak rate.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y | We can compromise to this |
| FUTUREWEI |  | While we are fine with the values of 3.2 and [0.75 or 0.8], due to a different understanding of the target, RAN will need to discuss |
| Panasonic | Y | We can accept the proposal for the progress. If the value of X or Y cannot be agreed, another possibility is to directly discuss the combinations of {vLayers, Qm, f} considering the achievable peak rate as in the table below: vLayers; Qm; BW3/PR3+PR1 peak rate [Mbps] (Minimum f is selected to achieve 10 Mbps); 20MHz+PR1 peak rate [Mbps] (Minimum f is selected to achieve 10 Mbps); Rel-17 RedCap min. peak rate [Mbps] vLayers·Qm·f ≥ 4. Rel-18 eRedCap: Potential capability report; 1; 2; N/A (Cannot achieve 10 Mbps); 11.3/10.9 (f=0.4); -. 1; 4; 10.7/10.3 (f=0.8); 22.7/21.8 (f=0.4); -. 1; 6; 15.0/14.4 (f=0.75); 34.0/34.7 (f=0.4); -. 1; 8; 10.7/10.3 (f=0.4); 45.4/43.7 (f=0.4); -. 2; 4; 10.7/10.3 (f=0.4); 45.4/43.7 (f=0.4); -. 2; 6; 16.1/15.4 (f=0.4); 68.1/65.5 (f=0.4); -. 2; 8; 21.4/20.5 (f=0.4); 90.7/87.3 (f=0.4); -. Rel-17 RedCap: Min. capability report; 1; 6; -; -; 63.8/61.4 (f=0.75). Note: xx/yy [Mbps] corresponds to the peak rate for 15/30 kHz SCS. |
| DOCOMO | Y |  |
| Spreadtrum |  | Fine with the values here. In addition, according to RANP’s conclusion, the 10-Mbps peak rate target is a fixed peak rate. In order to avoid possible ambiguities, the FFS should be changed to the following note: Note: The 10-Mbps peak rate target is a fixed peak rate. |
| CATT | Y | It is a good idea to leave ‘fixed’ or ‘minimum’ to RANP, if we really want RAN1 progress. |
| Lenovo | Y |  |
| vivo | Y |  |
| LG |  | vLayers·Qm·f >= X or Y should be kept according to WID. |
| MediaTek |  | RAN#99 agreements are clear. We don’t agree to the text in the parentheses. In RAN1, we should focus on the supported v*Q*f values. Proposal: Agree the following (without any intention to indicate one way or the other whether the 10-Mbps peak rate target is a minimum peak rate or a fixed peak rate): - For UE peak data rate reduction with UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = 3.2 - For UE peak data rate reduction without UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = [0.75 or 0.8] - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression. |
| Nokia, NSB | Y | We are fine to support the values and leave the discussion on minimum vs fixed peak data rate to RAN plenary |
| Qualcomm | Y | It is suggested that RAN1 decide X and Y to meet the 10bps target. We leave the discussion on whether it is minimum peak rate or fixed peak rate to RAN plenary. |
| NEC | Y | With understanding this is within the scope of WID and would not be intended to change frame work of TS 38.306. |
| Sierra Wireless | Y | We are Ok with the values. Our preference is to remove the parentheses. |
| Ericsson | Y |  |
| SONY |  | We are OK to agree on the two bullets. There is no need to agree to the red text as it would imply that RAN1 agrees that the 10Mbps target peak data rate (from RANP) could be interpreted as a minimum peak data rate. RAN1 doesn’t agree that there is that possible interpretation. We would hence be OK with the updated text from MediaTek. |
| CMCC |  | Generally fine. Regarding minimum or fixed peak rate, we still incline that 10Mbps is a minimum value, there is no need to restrict peak rate of R18 RedCap UEs with optional capabilities. According to the objective of the WID, the peak data rate target 10Mbps is a minimum value, this is aligned with the peak data rate defined for legacy and R17 RedCap UEs,. According to conclusion of RAN#99 meeting, PR1 as standalone feature or PR1 as add on feature is targeted at the same peak date rate 10Mbps, which does not intend to change the WID objective, but only gives a target relaxed peak data rate value. So that for PR1 standalone R18 RedCap UEs, vLayers·Qm·f is also larger than a constraint value X corresponding to 10Mbps. |
| OPPO | N | We see the value 3 is also needed for 5MHz RedCap. Also to soften the problem of pending RAN confirmation. The wording should be like this: - For UE peak data rate reduction with UE BB bandwidth reduction, - The vLayers·Qm·f target 3 or 3.2 for the 10-Mbps peak rate - For UE peak data rate reduction without UE BB bandwidth reduction, - The vLayers·Qm·f target 0.75 or 0.8 for the 10-Mbps peak rate - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression. |
| FUTUREWEI | Y | For progress |

For UE peak data rate reduction without UE BB bandwidth reduction, for the vLayers·Qm·f value corresponding to 10 Mbps peak rate, some contributions [12, 15, 18, 20, 25, 28, 29, 35] propose that the value should be 0.75, whereas other contributions [8, 9, 10, 11, 13, 14, 16, 21, 22, 34] propose that the value should be 0.8. A few contributions [16, 23, 26, 33, 36] express that it should be one of those two values.

FL1 High Priority Question 3.1-2a: For UE peak data rate reduction without UE BB bandwidth reduction, for the vLayers·Qm·f value corresponding to 10 Mbps peak rate, should the value be 0.75 or 0.8?

| Company | Value (0.75 or 0.8) | Comments |
| --- | --- | --- |
| Nordic | 0.75 | because it does fulfil 10Mbits peak rate for both 15 and 30kHz SCS |
| FUTUREWEI |  | We can be ok with either value, as long as only one value as in agreements. Slight preference for 0.8 if two layers. |
| Panasonic |  | Either is fine. |
| DOCOMO | 0.75/0.8 | Given that the difference on complexity reduction is marginal, we are fine with either value. |
| Spreadtrum | 0.75 | 0.8 is also fine for us, if 2Rx is needed. |
| CATT |  | Tend to 0.8 which can avoid new scaling factor (even for 2Rx UE). But anyway either one should work. Difference on cost is marginal. |
| Vivo |  | Finer either value, but slightly prefer 0.8 since it can allow 2Rx/2 layer transmission. |
| LG | 0.75 | We support 0.75. but, we can live with 0.8 |
| MediaTek | 0.75 | As pointed out by Nordic, 0.75 can fulfill 10Mbps for both SCSs. |
| Nokia, NSB | 0.75 or 0.8 | Either value is OK with us |
| Qualcomm | 0.75 (or 0.8) | 0.75 meets 10Mbps more closely but we are also ok with 0.8. |
| NEC | Either would be fine | We slightly prefer 0.75 as it is enough for 10Mbps. |
| Xiaomi | 0.75 | We can live with 0.8. |
| Ericsson | 0.8 | We could also live with 0.75. |
| SONY | 0.75 | 0.75 is our preference as it fulfills 10Mbps. We would be OK with 0.8 as the complexity increase 0.8 is marginal wrt 0.75. |
| CMCC | 0.75 or 0.8 | Fine with 0.75 or 0.8. |
| OPPO | 0.75 | We can go with value supported by majority. |

Based on the responses received for Question 3.1-2a, the following proposal can be considered.

#### FL2/FL3 High Priority Proposal 3.1-2b:

- For UE peak data rate reduction without UE BB bandwidth reduction,

  - The 10-Mbps peak rate target corresponds to vLayers·Qm·f = 0.75

  - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression.

| Company | Y/N | Comments |
| --- | --- | --- |
| CATT2 | Y | OK for progress. |
| Samsung | Y | OK for the proposal. |
| Vivo | Y |  |
| FUTUREWEI | Y | Ok for progress |
| Sierra Wireless | Y |  |

## 3.2	Combinations with optional features

Many contributions [8, 10, 12, 14, 15, 16, 18, 20, 21, 22, 24, 26, 28, 29, 32, 33, 35, 36] discuss the peak rate target for Rel-18 eRedCap UEs that support optional features. Regardless of the peak rate target discussion, now is probably a good time to start discussing what combinations with optional features that Rel-18 eRedCap UEs should be able to support.

#### FL1/FL3 High Priority Question 3.2-1a:

Which ones (if any) of the following features should Rel-18 eRedCap UEs be able to support as optional features?

- Potential optional feature 1: DL MIMO

- Potential optional feature 2: DL 256QAM

- Potential optional feature 3: Peak rate scaling factors (f) higher than the minimum value

| Company | Potential optional feature(s) | Comments |
| --- | --- | --- |
| Nordic |  | F1 can be useful for coverage and spectral efficiency, F2 for improved spectral efficiency, and F3 is controversial 😊 |
| FUTUREWEI | 1,2 | UEs should have the options to support such features. For #3, it is unclear what values need to be supported |
| Panasonic |  | This discussion would strongly be related to the determination whether 10 Mbps is a fix peak rate or minimum peak rate. If 10 Mbps is a fixed peak rate, the motivation to support features 1-2 would be quite low. For feature 3, we are not sure the meaning of “the minimum value” of the f. |
| DOCOMO |  | In principle, Rel-18 eRedCap UE can support these features depending on its UE capability same as legacy UEs. Thus, we don’t see the strong need to restrict the available optional capabilities so far. |
| Spreadtrum |  | This question is not very clear to us, especially we combine this question with proposal 3.1-1a. Is it means that the peak data rate can be higher than 10Mbps, if the features listed here are supported? |
| CATT | 1, 2, | Up to UE vendor’s interest. |
| vivo |  | Share Panasonic’s views. |
| LG | All | By default, we think that all features on Rel-18 eRedCap UEs can be supported optionally. |
| MediaTek |  | We don’t see the value for a further reduced UE to support these capabilities. However, if they are supported, they should not exceed the supported (v*Q*f) values from Proposal 3.1-1a. |
| Nokia, NSB | 1,2,3 | DL MIMO and 256QAM can already be optionally supported as per WID. We think if DL MIMO and 256QAM are supported, UE should be able to support higher peak data rate and therefore larger scaling values. |
| Qualcomm | 1/2/3 |  |
| Sierra Wireless |  | We do not see a need for eRedCap to support any of these features. |
| Xiaomi | 1,2 | Share the same view as CATT. For option 3, it is not clear for us. |
| Ericsson | 1 or 2 (perhaps not both) Defer discussion on 3 | We think it might also be worth discussing whether an eRedCap be able to support 2 Rx branches without supporting DL MIMO. The low peak rates for eRedCap UEs does not require DL MIMO support, but 2 Rx branches could still be beneficial to improve coverage and spectral efficiency. Potential option feature 3 could be discussed once it is clear whether the 10-Mbps peak rate target is a minimum peak rate or a fixed peak rate. |
| SONY | 1,2 | OK with 1,2. Support of these optional features does not imply that the peak data rate is higher than 10Mbps (i.e. proposal 3.1-1a still applies). For proposal 3, is scaling factor a “feature”? We don’t understand the point of this proposal when the peak data rate is 10Mbps in any case. |
| CMCC | 1,2,3 | Similar as legacy UEs, optional feature of R18 RedCap UEs is not restricted. |
| OPPO |  | We think by default they can be supported. Rel-17 works well and all those are not mandated to UE vendors. |
| Samsung |  | Share the same view as Panasonic. |

# 4	Higher-layer parameters

Contribution [7] provides the initial rapporteur input on higher layer signaling for RAN1 aspects of Rel-18 eRedCap. The contribution expresses that the agreements made so far do not result in any new or modified configuration parameters, but that depending on the outcome of the discussion on additional separate early indication in Msg1/MsgA PRACH, there may or may not be a need for Rel-18 eRedCap specific PRACH configuration parameters. The contribution also expresses that it might be relevant to capture the following RAN1 agreements [4] in higher-layer parameter descriptions, either for configuration parameters or capability signaling:

- Initial BWP: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, the Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs.

- Unicast PDSCH bandwidth: For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e

- Msg4 PDSCH bandwidth: For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

- Msg3 PUSCH bandwidth: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

- MsgA PUSCH bandwidth: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

- CG PUSCH bandwidth: For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

Thus, it might be worth discussing in RAN1 whether and how to capture and communicate the above agreements.

#### FL1/FL3 High Priority Question 4-1a:

- Do you agree that the RAN1 agreements made so far do not result in new or modified higher-layer parameters that need to be captured as part of the overall Rel-18 higher-layer parameter list from RAN1?

- If you think that some of the earlier RAN1 agreements need to be captured in the higher-layer parameter list from RAN1, please elaborate in the comment field.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nordic | Y |  |
| DOCOMO | Y |  |
| CATT | Y | So far no new HL parameter is needed. |
| vivo | Y |  |
| LG | Y | If Msg A PUSCH bandwidth is allocated more than 5MHz BW PRBs in SIB, Rel-18 eRedCap cannot utilize 2-step RACH procedure. |
| Nokia, NSB | Y |  |
| Qualcomm | Y |  |
| NEC | Y |  |
| Xiaomi |  | Share the same view as LG that if the MsgA PUSCH bandwidth is configured more than 25/12 PRBs, the 2-step RACH couldn’t be performed by the Rel-18 RedCap. |
| Ericsson | Y | If the RAN1 agreements listed above are not captured in the UE feature list (e.g., as components there), it might be good to communicate the agreements in some other way to RAN2 (e.g., within the higher-layer parameter list activity). |
| CMCC | Y |  |
| CATT2 |  | If the Proposal 2.1.1-1c on early indication in Msg1 is agreed, then we have impact since Rel-18 dedicated PRACH resource RRC configuration is expected. |
| Samsung | Y |  |
| FUTUREWEI |  | If Rel18 Msg1 early indication is agreed, then an RRC parameter is needed |

# 5	Other aspects

The submitted contributions bring up the following other aspects which are not covered in any other section in this FLS.

Initial BWP

- Support an additional separate initial BWP [17, 26].

- Do not support an additional separate initial BWP [14, 15].

FDRA optimization

- In the case where the RBG size for FDRA is large (e.g., 16 RBs in 15 kHz SCS, 8 RBs in 30 kHz SCS), consider enhancements to increase the efficiency in terms of DL/UL frequency resource utilization [23].

- For unicast, the FDRA indications and RBG sizes can be based on 5-MHz sub-bands [30].

- Discuss whether/how to use potential spare bits in FDRA field in RAR UL grant [13].

- There is no need to consider potential optimization of FDRA indications [15].

SRS bandwidth

- The SRS bandwidth does not need to be limited to 5 MHz [15, 17]

- Continue to discuss SRS bandwidth reduction to 5 MHz [26].

Other functionality

- Support 60 kHz SCS [14, 15].

- Consider enhancements of user multiplexing capacity for common PUCCH [23, 32].

- Support frequency hopping at least for unicast PUSCH [30].

- Consider options for support of 5-MHz MsgA PUSCH resource allocation [18].

- Enable UE to buffer none or a partial bandwidth for potential PDSCH within the activated BWP depending on the UE capabilities or circumstances such as other channel scheduling [15].

To be able to focus on more pressing issues, the above aspects could be down-prioritized in this meeting.

#### FL1/FL3 Medium Priority Question 5-1a:

Is there a need to treat any of the issues listed above in this meeting?

| Company | Y/N | Comments |
| --- | --- | --- |
| DOCOMO | Y | Common PUCCH capacity We prefer to consider enhancement on common PUCCH capacity for Rel-18 eRedCap. Given that the number of UEs is expected to be largely increased if NW accommodate both Rel-17 and Rel-18 RedCap, we have a concern on capacity on random access. Thus, we prefer to make it sure common PUCCH would not be a bottleneck for random access. |
| LG | Y | Enhancements of user multiplexing capacity for common PUCCH can be discussed considering the situation that people who carry not only smartphones but only simultaneously wear additional smart watches and VR glasses will demand explosive connections toward gNodeB with a lot of mobile devices at the same time. |
| Xiaomi | Y | We propose to further discuss the issue given by [18] for the MsgA resource allocation. How to deal with it if the MsgA PUSCH is more than 5MHz? |

# References

| [1] | RP-223544 | Revised WID on Enhanced support of reduced capability NR devices | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-2300177 | WI work plan for Rel-18 RedCap | Rapporteur (Ericsson) |
| [3] | R1-2304261 | FL summary #6 on Rel-18 RedCap UE complexity reduction | Moderator (Ericsson) |
| [4] | R1-2303938 | RAN1 agreements for Rel-18 NR RedCap | Rapporteur (Ericsson) |
| [5] | RP-230778 | Proposal for PR1 in eRedCap | Moderator (CMCC) |
| [6] | TR 38.865 V18.0.0 | Study on further NR RedCap UE complexity reduction (Release 18) | RAN1 |
| [7] | R1-2304336 | Initial input on higher layer signalling for Rel-18 eRedCap | Rapporteur (Ericsson) |
| [8] | R1-2304338 | Further RedCap UE complexity reduction | Ericsson |
| [9] | R1-2304359 | Discussion on R18 RedCap complexity | FUTUREWEI |
| [10] | R1-2304491 | Discussion on further UE complexity reduction | Vivo |
| [11] | R1-2304526 | Discussion on further UE complexity reduction | ZTE, Sanechips |
| [12] | R1-2304569 | Discussion on enhanced support of RedCap devices | Spreadtrum Communications |
| [13] | R1-2304629 | Discussion on potential solutions to further reduce UE complexity | Huawei, HiSilicon |
| [14] | R1-2304742 | Discussion on further complexity reduction for Rel-18 RedCap UE | CATT |
| [15] | R1-2304758 | RedCap UE Complexity Reduction | Nokia, Nokia Shanghai Bell |
| [16] | R1-2304802 | Complexity reduction for eRedCap UE | Intel Corporation |
| [17] | R1-2304860 | Discussion on further complexity reduction for eRedCap UEs | China Telecom |
| [18] | R1-2304912 | Discussion on further complexity reduction for eRedCap UEs | Xiaomi |
| [19] | R1-2304974 | UE complexity reduction | Lenovo |
| [20] | R1-2305024 | Discussion on Rel-18 RedCap UE | NEC |
| [21] | R1-2305046 | On eRedCap complexity reduction | Sony |
| [22] | R1-2305105 | Discussion on further reduced UE complexity | CMCC |
| [23] | R1-2305142 | Discussion on further UE complexity reduction for eRedCap | LG Electronics |
| [24] | R1-2305158 | Considerations for further UE complexity reduction | Sierra Wireless. S.A. |
| [25] | R1-2305254 | Further RedCap UE complexity reduction | Apple |
| [26] | R1-2305287 | UE complexity reduction for eRedCap | Panasonic |
| [27] | R1-2305308 | Discussion on complexity reduction for eRedCap UE | Sharp |
| [28] | R1-2305348 | UE complexity reduction for eRedCap | Qualcomm Incorporated |
| [29] | R1-2305449 | Further consideration on reduced UE complexity | OPPO |
| [30] | R1-2305525 | Further UE complexity reduction for eRedCap | Samsung |
| [31] | R1-2305567 | Discussion on UE complexity reduction | DENSO CORPORATION |
| [32] | R1-2305607 | Discussion on further UE complexity reduction for eRedCap | NTT DOCOMO, INC. |
| [33] | R1-2305647 | On eRedCap UE complexity reduction | MediaTek Inc. |
| [34] | R1-2305709 | Discussion on UE complexity reduction | Transsion Holdings |
| [35] | R1-2305853 | On further complexity reduction of NR UE | Nordic Semiconductor ASA |
| [36] | R1-2305868 | Considerations for Rel-18 eRedCap UE complexity reduction | Sequans Communications |
| [37] | R1-2304512 | Discussion on UE features for R18 eRedCap | Vivo |
| [38] | R1-2304795 | On support of legacy features for Rel-18 eRedCap UEs | Ericsson |
| [39] | R1-2305855 | On eRedCap features | Nordic Semiconductor ASA |
| [40] | R1-2304262 | LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs | RAN1, Ericsson |
| [41] | R1-2305957 | FL summary #1 on Rel-18 RedCap UE complexity reduction | Moderator (Ericsson) |