3GPP TSG RAN WG1 #113	R1-23xxxxx

Incheon, Korea, May 22nd – May 26th, 2023

# Title:	[Draft] LS on XR capacity enhancements

# Response to:	-

# Release:	Rel-18

# Work Item:	NR_XR_enh

Source:	Ericsson [RAN1]

To:	RAN2

Cc:	-

Contact Person:

Name:	Sorour Falahati

E-mail Address:	sorour.falahati@ericsson.com

Attachments:	none

# 1. Overall Description:

With respect to the following objective of the Rel-18 WI on XR:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

RAN1 would respectfully inform RAN2 about the following developments regarding the normative work in RAN1:

With respect to design aspect of HARQ process ID determination of multi-PUSCH configured grant, in RAN1#112 the following was concluded:

Conclusion RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG.

The outcome of discussions in RAN1 since RAN1#112, is finally concluded in RAN1#113 and captured in the agreement below.

Agreement

From RAN1 perspective, for determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

      - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

  - The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

    - X= the number of configured PUSCHs in the CG period

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

# 2. Actions:

To RAN2 group:

ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

# 3. Date of Next RAN1 Meetings:

TSG WG RAN1 #114		August 21st – 25th, 2023	Toulouse, France

TSG WG RAN1 #114-bis		October 9th – 13th, 2023		Xiamen, China