3GPP TSG RAN WG1 #113                                                                 R1-2304820

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Intel Corporation

Title:	Evaluation for AI/ML Beam Management

Agenda item:	9.2.3.1

Document for:	Discussion and Decision

# Introduction

In this paper, we present our views on the study of AI/ML applications to physical layer for beam management based on the objectives in the Rel-18 NR study item on AI/ML [1]:

Use cases to focus on: - Initial set of use cases includes: - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

We discuss evaluation methodology for the study and steps needed to align results across participating companies for meaningful comparison. We also present initial evaluation results for spatial and temporal domain beam prediction.

# Evaluation Methodology

The beam management use case for AI/ML study can be broadly divided into two sub-cases i.e., spatial domain beam management and temporal domain beam management. Regardless for the specific use case, a common evaluation methodology can be considered with respect to the AI/ML model training and deployment.

Figure 1: AI/ML Model Training and Deployment

An example is shown in Figure 1, where a supervised machine learning model with offline training/validation is considered. In the first step, a dataset should be generated based on the specific use case being considered. The dataset should contain beam-specific information including inputs for the AI/ML model as well output labels for training, validation, and testing. The dataset is usually split into non-overlapping portions for training and testing.  The next step is data pre-processing and normalization to ensure that the data is in the proper value range for the considered AI/ML model. Note that it may also be necessary to normalize the output labels depending on the problem formulation and type of output layer activation function being used. Based on this normalized input (output) data, the AI/ML model is trained and validated. As part of training to ensure better accuracy hyperparameter tuning is also an important step but care should be taken to ensure that the model is not overfitted to the training data. In this example, the dataset generation as well the model training is assumed to be an offline non-real-time process. Once the model is trained, the held-out test data is used for determining the model accuracy. Once the model accuracy is acceptable, the model can be deployed in a real-time environment to aid in specific beam management tasks.

Although more sophisticated online training methods including reinforcement learning exist in literature, the current study item should first concentrate on the offline training methodology with bigger emphasis on realistic evaluation assumptions and use-cases. Online models, if required, can be studied in future.

In previous RAN1 meetings, the following agreement was made on the selection of Set B of beams or beam-pairs.

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B). Agreement For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering - Various Set B of beam(pairs)

In our view, the Set B should be fixed in cardinality across training and inference. Furthermore, the definition of Set B itself should not depend on L1 measurement and reporting details like periodicity. It can be the case that during inference, different elements of Set B are updated at different times due to availability of L1 report corresponding to the desired beam or beam-pair. But this should not translate to Set B being variable across training and inference.

- The variability of Set B can only be due to updating the L1 measurements corresponding to beams or beam-pairs in Set B at different intervals. The cardinality of the set should not change across training and inference.

## Spatial Domain Beam Prediction

In RAN1#112, the following agreement was reached for DL Tx and DL Tx-Rx beam pair-link prediction.

Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options are not precluded and can be reported by companies. - Note: This is only for evaluation discussion

For consideration on Top-1 genie aided beams, the assumption on specific Rx beams is still FFS. Since the performance benchmark of machine learning should not be too unrealistic, it may be assumed that in both cases, the best UE beam and panel combination is assumed for measurement.

- For DL Tx and DL Tx-Rx beam pair prediction, the definition of top 1 genie aided beams considers the best UE beam on the best panel as the specific Rx beam.

## Temporal Domain Beam Prediction

### UE Rotation Modeling

In our evaluations, we have modeled UE rotation in the following manner:

- The UE is dropped at a random location within a cell with a random boresight orientation.

- The UE is moved along a straight-line trajectory which does not go out of the cell boundary

- The UE starts rotation at a fixed RPM from the starting point of the trajectory where the UE’s panels are assumed to be vertical.

- For training, longer UE trajectories are segmented into smaller trajectories with minimal overlap and different starting points. Each of these segments are used a different training sample. Thus, the UEs initial rotation point at each of these trajectories is different and is a function of the starting point.

In our observation, since the starting point of the rotation is different for different training samples, this methodology models a somewhat non-ideal scenario, and the ML model can learn across many such trajectories to provide beam prediction accuracy which improves on the baseline case. Even without the trajectory segmentation, the rotation modelel in our evaluation is simple and should provide some basis for evaluating BM-Case2 with UE rotation modelling. Therefore, we propose to adopt the above modelling for UE rotation.

- For BM-Case2 with UE rotation, drop UEs with random boresight and begin rotation along an axis with a fixed RPM as the UE traverses the trajectory. A larger trajectory may be segmented into smaller ones to use as different training samples with different initial UE orientations along the rotation axis.

### UE Trajectory and Spatial Consistency Modeling

UE Trajectory Modeling

UE trajectory modeling is an important part of temporal domain beam prediction. In the feMIMO mobility EVM, the trajectories of the UEs along with the starting points and ending points are fixed. Different from the feMIMO EVM, for training AI/ML models more diversity in environment is better. Therefore, we propose a more generic trajectory modeling as shown in Figure 2.

Figure 2: UE Trajectory for travel time of 2sec at 150km/hr

In this model, the UE can be dropped randomly at any location within the deployment and moves in a straight-line trajectory to an end point. The travel time is fixed, and the trajectory is sampled at equal intervals and beam specific data is collected along the trajectory for training the AI/ML model. If the trajectory hits the cell boundary, the trajectory is terminated and if the trajectory observation time is less than a threshold, the trajectory sample is discarded.

- UE trajectories with straight line movement without sharp turns should be considered for evaluation.

Spatial Consistency Modeling

Since the main goal is to capture the temporal variation of beam performance of a UE under motion, the SLS evaluation for this case is fundamentally different from drop based evaluations performed in the past where UEs are dropped into the system once at the beginning of the simulation and their positions are not changed for the duration of the simulation. In these static use cases, spatial consistency does not have a major impact on the evaluation results. However, in the new evaluation setup, where UEs are moving across the cell, it is very important to properly model spatial consistency for both large-scale as well small-scale fading parameters since this has a major impact on the overall simulation results. For example, if not modeled in a spatially consistent manner, the shadow fading can fluctuate widely over the trajectory of the UE and therefore yield a highly unstable channel realization which manifests itself in the fact that the UE is totally incapable of tracking the beams properly. This is not a desirable phenomenon and would adversely impact the training capabilities of the AI/ML models since the data samples across the trajectory may appear uncorrelated. Therefore, it is of utmost importance to ensure that large- and small-scale parameters are generated such that they vary slowly (within a realistic range). While the implementation of spatially consistent large-scale parameter generation is up to companies, care should be taken to ensure that there is approximately the exponential autocorrelation relationship

between different points at distance d m in the trajectory as specified in the WINNER II channel [4], and where is the decorrelation distance. Additionally, there are two spatial consistency models specified in [5]. The spatial consistency model A is meant for a traditional drop-based simulation where UE positions are not changed over the simulation duration. This model is not suitable for the temporal domain beam prediction evaluations, since it is based on extrapolation from the initial dropped location of the UE and does not account for mobility. The spatial consistency model B should be used since this model calculates the small-scale parameters at each point in the trajectory based on a grid of random numbers which is generated once per simulation for each UE and gNB and the random number values are calculated based on the grid and the UE position at each trajectory update.

- Spatially consistent large-scale parameter generation should be used for mobility evaluations. Additionally, only spatial consistency model B in [4] can be used for mobility evaluation.

The trajectory of the UEs can be sampled at different periodicities and companies should report how often the UE locations are updated in their evaluation assumptions. In order to ensure large scale parameters are updated often enough during the evaluation, it is preferable to update the location every 1m or at least at a distance less than the minimum decorrelation distance of the large-scale parameters for the given evaluation scenario.

- The UE trajectory should be sampled at least at the minimum decorrelation distance of the large-scale parameters corresponding to the scenario of evaluation.

To highlight this importance, results for SLS based spatially consistent UE trajectory sampling is provided below. For this case, a single UE moving in a linear trajectory within a cell as shown in Figure 2 is modeled. The trajectory is shown for 2sec of travel time and the preliminary results for beam tracking are provided over 1sec of travel time on this trajectory. The trajectory is sampled every 1m. The UE has two panels facing 180° apart and system is operating at a CF = 30 GHz and 120kHz SCS. An Urban Micro deployment with 200m ISD and an outdoor UE at a speed of 150km/hr is considered. Detailed evaluation assumptions are provided in the Appendix.

In this simulation spatial consistency has been modeled for both large scale parameters and small-scale parameters using spatial consistency model B from [5]. The following figures show how the large-scale parameters vary over the trajectory sample points.

Figure 3: LSP variation over the UE trajectory for 1sec travel time

From Figure 3, it can be seen that the large-scale parameters, especially shadow fading, which has a major impact on RSRP varies slowly over time as the trajectory is sampled. The same holds true for other LSPs. For small-scale fading, the spatial consistency model B is leveraged and the fast-fading RSRP also shows consistent behavior over the UE trajectory. Based on these initial results, proper modeling of parameters for channel modeling is very important for creating a meaningful dataset for AI/ML evaluation for temporal domain beam prediction.

## Model Generalization

Agreement - For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies

On the model generalization aspect with respect to deployment scenarios, one example mentions “same deployment, different cells with different configuration/assumptions”. On this point, we wanted to reiterate that in this study we are using system level simulations and 3GPP based statistical channel models for synthetic data generation. Additionally, the system level deployment models usually follow wrap-around methodology to avoid edge-of-the-world effects. Therefore, different cells within a system level deployment are statistically equivalent and would not in general lend themselves to show any sort of generalization performance. Therefore, if companies report generalization across cells of the same deployment, companies need to model different configurations in each cell e.g., with respect to antenna arrays, gNB/UE heights etc. which can alter the statistics of the channel model thereby making generalization study worthwhile.

- Generalization across different cells of the same deployment in system level simulation should necessarily have different configurations in each cell such that the cells are not statistically identical with respect to generated channels.

# Initial Evaluation Results

In this section, we provide some initial evaluation results for temporal and spatial domain beam prediction use cases with additional details on problem formulation, dataset generation and AI/ML model assumptions.

## Temporal Domain Beam Prediction (BM Case 2)

### Problem Formulation and Evaluation Metrics

In this section, we provide an example of temporal domain beam prediction modeling. The long short-term memory (LSTM) based networks can be used to construct the temporal domain beam prediction model as shown in Figure 4. The model works over an observation window where measurements are performed and then provided to the model which is able to make a prediction of the measurement or indices to be used in the prediction window.

Figure 4. AIML Model for Temporal Domain Beam Prediction

The architecture of the model for training and inferencing respectively are shown in Figure 5 and Figure 6. Let vector sequence x<t>  and {y<t>} represent the input and output of the AIML model, where t=1,…,T. At one time instance t, the RSRP vector x<t> is fed into the CNN/DNN layer(s) and then the LSTM cell. The output of an LSTM cell at time instance t can be regarded as one predicted RSRP vector. The LSTM cell in the figures is the time-unwrapped model with the hidden states a<t> spanning time t=1,…,T.

Figure 5. LSTM architecture for training

In training mode, the output sequence {y<t>} can be a one time-step delayed version of input sequence {x<t>}, where the sequence length is T for both input and output.

Figure 6. LSTM architecture for inferencing with single time-step prediction

In inferencing mode, as shown in Figure 6 and Figure 4, the T1 RSRP vector samples, x<0>  to x<T1-1>, obtained in the observation window, are fed into the AIML model to predict the following T2 RSRP vectors, x<T1> to  x<T1+T2-1>, in the prediction window.

For comparison, a baseline scheme using a sample-and-hold method is also evaluated, as shown in Figure 7.

Figure 7. Baseline Method for Temporal Domain Beam Prediction

Different metrics are used to evaluate the beam prediction results, such as the beam prediction accuracy for Top-1 beam and Top-K beams, mean L1-RSRP difference of Top-1 beam, CDF of L1-RSRP difference for Top-1 beam.

### Dataset Generation and Statistics for Temporal Domain Prediction

For BM-case-2 (temporal domain beam prediction), we assume 32 downlink Tx beams at NW side, 4 downlink Rx beams per UE panel and 2 panels at UE side. We also assume, for simplicity that the best panel at the UE is selected for measurement reports. Thus, there are 32x8 = 256 beam pairs between the BS and the UE assuming panel selection. Different UE speeds, e.g., 30km/h and 90km/h, measurement interval, e.g., 10ms, 50ms, and 100ms are considered. Straight-line trajectories starting from random locations around the center of three hexagonal sectors are generated within the coverage area, as shown in Figure 2, for training and testing.

In the following part of this subsection, we show some data statistics of the temporal domain beam management data collected from the SLS assuming intra-cell mobility i.e., the trajectory is assumed to be contained within a sector/cell boundary and no handover is modeled.

First, we found total number of unique best beams along a given UE trajectory is usually small.  We counted the number of unique best BS beams and the number of unique UE best beams for each trajectory, and the statistics for 1500 trajectories is shown in Figure 8 and Figure 9. As the figures show, most of the trajectories only have 2 different best BS beams and 2 different best UE beams although there are 32 different BS beams and 8 different UE beams. And for 90% of the trajectories, the number of unique best BS/UE beam is less than 5.

- For a large portion of the trajectory samples, the number of unique best BS/UE beam is very limited.

Figure 8. Number of Unique Best BS Beams (UE speed = 30km/h, measurement interval = 10ms)

Figure 9. Number of Unique Best UE Beams (UE speed = 30km/h, measurement interval = 10ms)

Next, the best beams for several randomly selected trajectories are shown in Figure 10 and the corresponding trajectories are shown in Figure 11, where five trajectories with 600 time-domain samples are given.

Figure 10. Best BS beam indexes (left) and UE beam indexes (right) (Five trajectories in different colours, UE speed = 30km/h, measurement interval = 10ms)

Figure 11. Trajectory samples (Five trajectories in different colours matched to the best beam figures)

We can see the number of unique best BS beams and best UE beams are very limited but the switching between different best beams can be frequent if the maximum RSRP of the beams is used to switch at every time instance. In other words, a ping-pong effect is observed for the best beam indices among consecutive time-domain samples. Although the best beam can change frequently in these time stamps, the best RSRP corresponding to the best BS-UE beam pair does not change by a significant margin in most of the cases. In particular, the measured RSRP of two BS-UE beam pairs from the green trajectory in Figure 11 are shown in Figure 12. As Figure 10 shows, the one occasion of ping-pong effect in best BS beam of green trajectory happens from time index 0 to time index 20. In this time, the best BS beam index switches between beam index 26 and beam index 18, and the best UE beam is beam 8. And the measured RSRP in Figure 12 shows, the RSRP difference between the two best beam pairs is very small during that time.

Figure 12. Measured RSRP values for two beam pairs in the green trajectory (UE speed = 30km/h, measurement interval = 10ms).

Figure 13. Filtered Best BS beam indexes (left) and UE beam indexes (right) (Five trajectories in different colours, UE speed = 30km/h, measurement interval = 10ms)

The ping-pong effect may not cause significant problems to RSRP-based beam prediction/KPI. However, for index-based beam prediction/KPI, as the input and output of the NN are beam indexes only, the ping-pong effect could cause a negative impact to the prediction accuracy of the model that may not be practically significant. To avoid the negative impact, RSRP thresholding (or more sophisticated hysteresis) and/or RSRP smoothening can be used for beam selection to reduces the ping-pong effect observed in the best beam index (while not causing beam failure). In the following, the RSRP thresholding method works as follows: Assuming the best beam pair indexes for the whole trajectory are known, and at time stamp t, the best index is Index{t}. From t = 0 to t = end, the best beam pair Index{t+1} may be updated only if the corresponding RSRP difference between time stamp t and t+1 is larger than a certain threshold. If the RSRP difference threshold is set as 3dB, the best BS and UE beam indexes are shown in Figure 13. The thresholding can lead to significant reduction in the ping-pong effect.

- Ping-pong effect is observed with the best beam index selection among close time-domain samples, but the measured RSRP of best beam pairs during ping-pong effect can be very small.

- Ping-pong effect for best beam indices can be reduced by thresholding (hysteresis implementation) and/or smoothening the RSRP for best beam selection.

The effect of UE speed and measurement interval to the number of unique beams is shown in Figure 14 and Figure 15, where all the trajectories have the same physical length. Figure 14 shows that higher UE speed leads to marginally smaller number of unique best beams, and Figure 15 indicates smaller measurement interval results in marginally larger number of unique best beams. Overall, the effect of UE speed and measurement interval to the number of unique beams is not significant.

- The effect of UE speed and measurement interval to the number of unique beams is not significant.

Figure 14. CDF of number of unique best beams (measurement interval = 10ms).

Figure 15. CDF of the number of unique best beams (UE speed = 30km/h)

The effect of UE rotation is also considered. The CDF curves of the number of unique best BS and UE beams per trajectory with different RPM values, e.g., 0rpm, 10rpm and 100rpm, are shown in Figure 16. As the figure shows, UE rotation mainly affects the number of unique best UE beams. However, the total number of the unique best BS beams and UE beams are not affected by the UE rotation. The larger the RPM, the higher the percentage of larger number of unique UE beam values. In particular, we plot the best BS-UE beam pair indexes for 5 trajectories in Figure 17, which shows that higher UE rotation speed leads to more frequent change of the best beam pair index.

Figure 16. CDF of the number of unique best beams (measurement interval = 10ms)

Figure 17. Best BS-UE Beam Pair Indexes (0rpm [left], 10rpm [middle], and 100rpm [right], measurement interval = 10ms)

- The UE rotation mainly affects the distribution of the number of unique best UE beams. Higher UE rotation speed leads to more frequent change of the best beam pair index.

Furthermore, Figure 18 shows the best BS-UE beam pair indexes in the first 60 time instances for different measurement intervals, e.g., the actual time interval between two consecutive time indexes are 10ms, 50ms and 100ms. The data with 50ms and 100ms measurement intervals are down-sampled with respect to trajectory points from the data with 10ms interval. As we can see, very small measurement interval leads to little changes to the best beam index but with larger measurement intervals, the number of beam changes increases. Thus, to train an LSTM-based beam prediction model, a proper measurement interval should be selected. We note that this is an important criterion because choice of proper measurement interval leads to the AI/ML model significantly outperforming the sample-and-hold baseline.

Figure 18. Best BS-UE Beam Pair Indexes (10rpm, measurement interval: 10ms [left], 50ms [middle], and 100ms [right])

### Observations from Preliminary Results – Beam Pair Prediction

As we discussed in previous section, the best beam can switch very frequently between two beams but the number of unique best beams in a trajectory is very limited. Most of the ping-pong effect is because of the difference between the largest RSRP and the second largest RSRP is very small. And the ping-pong effect can be reduced by filtering the RSRP and applying a hysteresis threshold.

The following evaluation results in Figure 19 to Figure 22 are based on the processed data with the filtering and assuming 10rpm UE rotation with 50ms and 100ms measurement periodicity. More simulation results and assumptions are collected in the appended table, Temporal Domain Beam Pair Prediction Results.

- RAN1 should further discuss input sample length and the number of beam changes or beam dwelling time for BM-Case 2 to ensure model performance is not misleading.

Figure 19. Beam Prediction Accuracy for Different Training Window Length (Top-1 [left], Top-3 [right]. Config0: Training window length = 20, Config1: training window length = 30, Config2: training window length = 40)

We first test the impact of training window size to the prediction accuracy of the LSTM-based beam pair prediction. In Figure 19, training window size T = 20, 30, 40 have been tested with the observation window size T1 = [1, 2, 3, 4, 5, 10, 15, 20, 30, 40] and the prediction window size T2 =5. As shown in the figure, the beam prediction accuracy increases rapidly when the observation window size increase from 1 to 5, and then the performances are saturated. Overall using small observation window can have good prediction accuracy gain over the baseline sample and hold method. Furthermore, larger training window size also leads to better accuracy. We note that the number of samples used for training is the same for different training window sizes. This means the actual time duration used for training is longer for the configuration of larger training window.

- Using larger training window length for LSTM model training achieves better performance for beam prediction

Next, the Top-1 and Top-3 beam pair prediction accuracies achieved by different prediction window length T2 for two different measurement intervals, 50ms and 100ms, are shown in Figure 20. The LSTM model is trained with a fixed number of time instances T = 20. Relatively, the observation window length T1 = 5. As shown in Figure 20, increasing the measurement interval can decrease the beam prediction accuracy for both the LSTM model and the baseline scheme because of the reducing of spatial consistency. However, the LSTM model is impacted less significantly than the baseline scheme when the measurement periodicity increases. Besides, as shown in the figure, increasing the prediction window size leads to worse prediction accuracy for both the LSTM model and the baseline scheme. But the performance loss of LSTM mode is less than the baseline when the prediction window increases. Note that the LSTM model in our evaluation predicts one step at a time and the single time-step prediction is provided as input to the next time step until the end of the prediction window.

- The LSTM model is more resilient to the increase of measurement periodicity than baseline sample-and-hold method.

- Increasing the prediction window size leads to worse prediction accuracy for both the LSTM model and the baseline scheme.

Furthermore, the CDF of RSRP difference and the mean RSRP difference are shown in Figure 21 and Figure 22, respectively. As shown in Figure 20 to Figure 22, a larger prediction window corresponds to a larger mean RSRP difference but a lower prediction accuracy. We also found the prediction accuracy for both the LSTM-based and baseline method is related to the RSRP difference of high percentile. For example, comparing the values of RSRP difference of the 95-percentile, the lower the value, the higher the prediction accuracy. Thus, the performance of LSTM model can also be evaluated with the CDF of RSRP difference and the mean RSRP difference.

- The prediction accuracy is related to the RSRP difference from the genie-aided scheme

Figure 20. Beam Prediction Accuracy for Different Prediction Window Length (Top-1 [left], Top-3 [right])

Figure 21. CDF of RSRP Difference (100ms[left] 50ms[right] measurement interval)

Figure 22. Mean RSRP Difference (100ms[left] 50ms[right] measurement interval, T1 = 5)

## Spatial Domain Beam Prediction (BM Case 1)

### Problem Formulation

Traditionally, beam acquisition or tracking would require measurement on all the beams at the UE for any specific gNB transmit beam. In order to select the best beam pair link, there would need to a prohibitive number of measurements at the UE if exhaustive search were used rendering it infeasible. The goal of the AI/ML aided beam acquisition and tracking is to use a sub-set of these measurements to predict the best beam.

Figure 23: ML aided beam prediction at gNB conditioned on single UE beam

A possible problem formulation shown in Figure 23 which assumes that the AI/ML model will predict the RSRP values for all the gNB beams based on the input of RSRP values from only a few measurement beams. This is different from the traditional classification problem which predicts only the best beam index. In this regression-based formulation, the RSRP values for all the beams are predicted which enables the model to identify not only the best beam index but also the top-K (K=2,3,4, …) beam indexes.

Figure 24: Example DNN for UE beam prediction

An example DNN network is shown in Figure 24, where the hidden fully connected layers use ReLU activation, and the final output layer uses a tanh activation for RSRP prediction for all beams. An alternate CNN was also evaluated where the 1D-conv layers were used. The CNN with same depth as the DNN has 10x less trainable parameters than the DNN.  In the next sections, we provide initial evaluation results based on SLS and LLS based data generation.

### SLS Based Evaluations

#### Dataset Generation and Training for Spatial Domain Prediction

For the dataset generation, 5G UMa channel models are used. Different channel conditions, such as 1) 80% indoor + 20% outdoor UE distribution and nLoS + LoS mixed channels, and 2) 100% outdoor UE distribution and nLoS + LoS mixed channels, are considered. The UE is assumed to have 2 panels and each panel has 1x4 cross-polarized antenna elements with DFT beams. The gNB is assumed to have 1 panel which has 8x8=64 cross-polarized antenna elements with DFT beams. The 64 gNB antenna elements form 64 narrow beams and 16 virtualized wide beams at gNB.

In this section, three problems are considered,

- BM-Case-1a: Narrow beam measurement-based BS narrow beam prediction with best UE panel and beam selection.

- BM-Case-1b: Wide beam measurement-based BS narrow beam prediction with best UE panel and beam selection.

- BM-Case-1c: Joint UE-BS beam-pair-link prediction with best UE panel selection.

For the first and second problem, we assume the UE can select the best UE panel and the best UE beam of the panel by comparing the average RSRPs across all narrow/wide gNB-UE beam pairs of two panels. For the third problem, we assume the UE selects the panel and there are 64x4 gNB-UE narrow beam pair links. Next, a certain number of beams are measured and the RSRP is provided as the input of the ML model. For example, 8, 10, 12, and 16 beams are measured in case-1a and case-1b. 16 and 32 beams are measured in case-1c.

250,000 data points are generated for dataset, and each datapoint consists of RSRP values measured on all the beams. The data is partitioned as 70% is used for training, of which 10% is used for validation. In order to limit the input data range within (-1,1), each data point was individually normalized by the max absolute value of the RSRP of the input beams. For training the outputs were also similarly normalized.

A 5-layer 2 dimensional CNN with Leaky ReLU as activation function was trained for 500 epochs with early stopping. The batch size was kept at 200.

Four KPIs of beam prediction accuracy are evaluated for the problems as follows.

- Top-1 beam: Top-1 predicted beam

- Top-1 beam with 1dB margin: percentage of the Top-1 predicted beam whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam

- Top-3 beam (opt-1): The beam prediction accuracy is the percentage of the Top-1 predicted beam is one of the Top-3 genie-aided beams

- Top-3 beam (opt-2): The beam prediction accuracy is the percentage of the Top-1 genie-aided beam is one of the Top-3 predicted beams

#### Performance Results

##### Beam Prediction with Fixed UE Beam

As mentioned above, we consider two cases for beam prediction with fixed UE beam, case-1a, narrow beam measurement-based BS narrow beam prediction with best UE panel and beam selection, and case-1b, wide beam measurement-based BS narrow beam prediction with best UE panel and beam selection.

In case-1a, the 64 input beams are sampled to 8, 10, 12, or 16 beams, according to the sampling pattern shown in Figure 25 - Figure 28, to predict the optimal beam among the 64 beams. The beam prediction accuracies for different number of samples and different metrics are shown in Figure 29 - Figure 31, where different channel conditions are considered.

Figure 25. Case-1a 8 samples from 64 beams

Figure 26. Case-1a 10 samples from 64 beams

Figure 27. Case-1a 12 samples from 64 beams

Figure 28. Case-1a 16 samples from 64 beams

Figure 29. Case-1a NLOS + LOS channel

Figure 30. Case-1a NLOS + LOS channel

Figure 31. Case-1a LOS channel

Figure 32. Case-1a CDF of RSRP difference

From Figure 29 - Figure 31, we can see that the beam prediction accuracy increases with the number of sampled beams. Although the beam prediction accuracies are imperfect under Top-1 beam prediction criteria, the Top-3 beam prediction accuracies are good for different channel scenarios. From Figure 32, it can be seen that even for the error cases, most of the predicted beam indexes may still be good enough since the 90th percentile of the RSRP error CDF is around 5dB.

The soft evaluation metrics such as Top-1 beam prediction accuracy with 1 dB margin and Top-3 beam (option 1 and option 2) lead to a gap between the hard evaluation metric of Top-1 beam. The gaps different for different channel conditions. For example, in outdoor scenario, the gap between Top-1 beam prediction accuracy with 1 dB margin and without margin is about 0.05. And in indoor scenario, the gap is about 0.1. And the Top-3 beam prediction option 1 has slightly higher accuracy then option 2 for the same data set.

Furthermore, a hierarchical beam search scheme is considered as a baseline scheme to compare with the AI/ML based beam prediction. The hierarchical beam search consists of two steps. First, x beams are uniformly sampled from the 64 input beams, and the beam corresponding to the largest RSRP is selected. Second, y beams around the selected beam in step 1 is selected, and the beam corresponding to the largest RSRP is selected as the final beam search output. The accuracy of CNN-based beam prediction with 16 samples and baseline beam search scheme with x=8/y=8 is shown in Table 1, which shows that the CNN-based beam prediction outperforms the baseline scheme for all the evaluation metrics and scenarios.

Table 1. Case-1a Accuracy Comparison of Baseline and CNN-based scheme with 16 samples

|  | Top-1 beam |  | Top-1 beam w/ 1dB margin |  | Top-3 beam (opt-1) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CNN | Baseline | CNN | Baseline | CNN | Baseline |
| NLOS+LOS, 80% indoor | 0.7273 | 0.6639 | 0.8198 | 0.7296 | 0.9630 | 0.9141 |
| NLOS+LOS, outdoor | 0.8704 | 0.8235 | 0.9204 | 0.8484 | 0.9853 | 0.9411 |
| Los, 80% indoor | 0.7580 | 0.6953 | 0.8450 | 0.7567 | 0.9717 | 0.9248 |

Results data and assumptions are also collected in the appended table, Tx Beam Prediction (Case-1a).

In case-1b, the 16 input beams are sampled to 8, 10, 12, or 16 beams, according to uniform sampling the beam indices, to predict the optimal beam among the 64 beams. The beam prediction accuracies for different number of samples and different metrics are shown in Figure 33– Figure 36, where different channel conditions are considered.

Figure 33. Case-1b LOS channel

Figure 34. Case-1b LOS Channel

Figure 35. Case-1b NLOS + LOS channel

Figure 36. Case-1b NLOS + LOS channel

Similar to case-1a, the beam prediction accuracy increases with the number of sampled beams. The Top-3 beam prediction accuracies are good for different channel scenarios.

Similar to case-1a, the soft evaluation metrics such as Top-1 beam prediction accuracy with 1 dB margin and Top-3 beam (option 1 and option 2) lead to a gap between the hard evaluation metric of Top-1 beam. The gaps different for different channel conditions. For example, in outdoor scenario, the gap between Top-1 beam prediction accuracy with 1 dB margin and without margin is about 0.05. And in indoor scenario, the gap is about 0.1. And the option-1 and option-2 of Top-3 beam prediction accuracies are almost the same for the same data set.

For both case-1a and case-1b, it can be seen that NLOS channel models are less accurate than the LOS case, and 80% indoor + 20% outdoor channel models are less accurate than the 100% outdoor channel models. But Top-3 beam prediction accuracies are always acceptable. We also notice that the wide beam-based beam prediction has slightly higher beam prediction accuracy than the narrow beam based one when number of sampled beams is small, e.g., 8. This is because there is some beam correspondence between wide beam and narrow beam.

Furthermore, a hierarchical beam search scheme is considered as a baseline scheme to compare with the AI/ML based beam prediction. The hierarchical beam search consists of two steps. First, 16 beams are uniformly sampled from the 16 input wide beams, and the beam corresponding to the largest RSRP is selected. Second, 4 narrow beams corresponding to the selected wide beam in step 1 is selected, and the beam corresponding to the largest RSRP is selected as the final beam search output. The accuracy of CNN-based beam prediction with 16 samples and baseline beam search scheme is shown in Table 2, which shows that the CNN-based beam prediction outperforms the baseline scheme significantly.

Table 2. Case-1b Accuracy Comparison of Baseline and CNN-based scheme with 16 samples

|  | Top-1 beam |  | Top-1 beam w/ 1dB margin |  | Top-3 beam (opt-1) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CNN | Baseline | CNN | Baseline | CNN | Baseline |
| NLOS+LOS, 80% indoor | 0.6820 | 0.6120 | 0.7727 | 0.6878 | 0.9281 | 0.9541 |
| NLOS+LOS, outdoor | 0.8697 | 0.6467 | 0.9217 | 0.6860 | 0.9830 | 0.9421 |
| Los, 80% indoor | 0.7151 | 0.6199 | 0.8018 | 0.6913 | 0.9424 | 0.9571 |
| Los, outdoor | 0.9399 | 0.6758 | 0.9754 | 0.7080 | 0.9961 | 0.9484 |

Results data and assumptions are also collected in the appended table, Tx Beam Prediction (Case-1b).

##### Joint UE-gNB Beam Pair Link Prediction

As mentioned above, we consider joint UE-gNB beam prediction with best UE panel selection (case-1c). The 256 input beams pairs are sampled to 8, 16, or 32 beams, according to uniform sampling the beam indices, to predict the optimal beam among the 256 beam pairs. The beam prediction accuracies for different number of samples and different metrics are shown in Figure 37 - Figure 38. As shown in the figures, the beam prediction accuracy increases with the number of sampled beams. It can be seen that NLOS channel models are less accurate than the LOS case, and 80% indoor + 20% outdoor channel models are less accurate than the 100% outdoor channel models. Although using 8 samples over 256 beam pairs has acceptable prediction accuracy over 85% for LOS outdoor channel, it is too sparse to be used to predict well in other more complicated channel conditions.

Figure 37. Case-1c LOS channel

Figure 38. Case-1c NLOS + LOS channel

Results data and assumptions are also collected in the appended table, Beam Pair Prediction.

### LLS Based Evaluations

#### Dataset Generation and Training

For the dataset generation, 3GPP CDL channel models were used. The UE panel was assumed to have 8x8 = 64 cross-polarized antenna elements with DFT beams. The fixed gNB beam was assumed to be pointing towards the horizon at 90 degrees from the vertical. Each datapoint consists of RSRP values measured on all the UE beams for a given orientation of the UE antenna array. 1 million data points were generated each with a different UE antenna panel orientation, and the data was partitioned as shown in Figure 39. In order to limit the input data range within (-1,1), each data point was individually normalized by the max absolute value of the RSRP of the input beams. For training the outputs were also similarly normalized. The normalization is performed per data point to ensure there is no cross-UE dependence on the normalization for the case when the model may be deployed in practical networks.

Figure 39: Dataset split for training, validation and testing. The percentages shown are examples.

The DNN was trained for 500 epochs with early stopping. The batch size was kept at 100 and Adam optimizer was used. For this problem, the accuracy of predicting the best beam as well the accuracy for predicting the top K=3 beams were considered. For instance, if the model predicts only one of the best 3 beams, the accuracy is 33.33%, for two of the best beams, the accuracy 66.66% and so on. These hard metrics give an indication of the absolute performance of the network.

#### Performance Results

##### Beam Prediction at the UE with Fixed gNB Beam

The AI/ML model was provided with 8 out of the 64 beams for measurement for different channel models and performance was evaluated with respect to beam prediction accuracy.

Figure 40:ML-aided UE beam prediction with 8 measurement beams and uniform sampling of measurement beams in the index domain

Figure 41: ML-aided UE beam prediction with 8 measurement beams and optimized sampling of measurement beams

From Figure 40 and Figure 41, it can be seen that beam prediction in the spatial domain at the UE side has good accuracy even with only 8 beams out of 64 beams measured. It is also seen that sampling the measurement beams plays a role in model accuracy with the more optimized sampling providing even better performance. Note that despite the 10% error rate in LOS case, the actual RSRP difference even in most of the error cases is quite small as shown by Figure 42 which plots the CDF of the RSRP difference matrix for the error cases for the optimized sampling.

Figure 42. RSRP difference between predicted beams and optimal beams for the error cases in AI/ML model prediction for UE beam prediction conditioned on a fixed gNB beam

Figure 43: ML-aided UE beam prediction with 6 measurement beams and optimized sampling of measurement beams

Figure 43 shows the performance of 6 measurement beams with optimized sampling where the performance is quite close to the case for 8 measurement beams. Note that other sampling methods with 6 beams do not yield good results.

Next, we can consider a larger antenna array at the UE to check if the results from smaller arrays scale for larger number of antennas. In the following, we consider a 16x16 2D planar array at the UE with 256 non-oversampled DFT beams.

Figure 44: ML-aided UE beam prediction with 256 UE beams and 32 measurement measuremt beams

It can be seen that NLOS channel models are less accurate than the LOS case, but the accuracy is still within acceptable levels especially considering the overhead reduction from measurements.

##### Joint UE-gNB Beam Pair Link Prediction

In this problem formulation, joint beam pair link at UE and BS is predicted by the model. For dataset generation, RSRP across all gNB beams and UE beams is considered with only a subset used for input to the model. The implication of using the subset is that specific UE-gNB beam pair links are measured. Considering a gNB array with 32 DFT beams and UE array with 8 DFT beams, 16 measurement beam pair links are considered. The measurement beams at UE and gNB are shown in Figure 45.

Figure 45: Joint BS-UE beam pair link prediction. The rows correspond to UE beams and the columns correspond to gNB beams with the red boxes representing measurement beam pair links.

From the results, it can be seen that performance of AI/ML models for joint beam pair link prediction is very promising both LOS (CDL-E) and NLOS (CDL-A) channels.

# Conclusion

In this paper, beam management for AI/ML applications has been discussed. The main proposals and observations from this paper are outlined here:

Proposals:

- The variability of Set B can only be due to updating the L1 measurements corresponding to beams or beam-pairs in Set B at different intervals. The cardinality of the set should not change across training and inference

- For DL Tx and DL Tx-Rx beam pair prediction, the definition of top 1 genie aided beams considers the best UE beam on the best panel as the specific Rx beam

- For BM-Case2 with UE rotation, drop UEs with random boresight and begin rotation along an axis with a fixed RPM as the UE traverses the trajectory. A larger trajectory may be segmented into smaller ones to use as different training samples with different initial UE orientations along the rotation axis

- UE trajectories with straight line movement without sharp turns should be considered for evaluation.

- Spatially consistent large-scale parameter generation should be used for mobility evaluations. Additionally, only spatial consistency model B in [4] can be used for mobility evaluation

- The UE trajectory should be sampled at least at the minimum decorrelation distance of the large-scale parameters corresponding to the scenario of evaluation

- Generalization across different cells of the same deployment in system level simulation should necessarily have different configurations in each cell such that the cells are not statistically identical with respect to generated channels

- RAN1 should further discuss input sample length and the number of beam changes or beam dwelling time for BM-Case 2 to ensure model performance is not misleading

Observations:

Observation 1: For a large portion of the trajectory samples, the number of unique best BS/UE beam is very limited.

Observation 2: Ping-pong effect is observed with the best beam index selection among close time-domain samples, but the measured RSRP of best beam pairs during ping-pong effect can be very small.

Observation 3: Ping-pong effect for best beam indices can be reduced by thresholding (hysteresis implementation) and/or smoothening the RSRP for best beam selection.

Observation 4: The effect of UE speed and measurement interval to the number of unique beams is not significant.

Observation 5: The UE rotation mainly affects the distribution of the number of unique best UE beams. Higher UE rotation speed leads to more frequent change of the best beam pair index.

Observation 6: Using larger training window length for LSTM model training achieves better performance for beam prediction

Observation 7: The LSTM model is more resilient to the increase of measurement periodicity than baseline sample-and-hold method.

Observation 8: Increasing the prediction window size leads to worse prediction accuracy for both the LSTM model and the baseline scheme.

Observation 9: The prediction accuracy is related to the RSRP difference from the genie-aided scheme

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, 3GPP TSG RAN#94e, December 6 – 17, 2021.

- Chairman’s Notes, 3GPP RAN1#110, Toulouse France, August 2022.

- R1-2007151, “Moderator summary for multi-beam enhancement: EVM”, Samsung, 3GPP RAN1#102-e, August 17-28, 2020.

- IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007 (http://www.ist-winner.org/deliverables.html).

- 3GPP TR 38.901 v16.1.0, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- R1-2302793, “Other aspects on AI/ML for Beam Management”, Intel Corporation, 3GPP RAN1#112bis-e, April 2023.

# Appendix

## Temporal Domain Beam Pair Prediction Results

| Assumptions |  | Number of [beams/beam pairs] in Set A | 256 beams pairs |  |
| --- | --- | --- | --- | --- |
|  |  | Number of [beams/beam pairs] in Set B | 256 beams pairs |  |
|  |  | Speed | 30km/h |  |
|  |  | UE rotation | 10rpm, 50rpm |  |
|  |  | measurement periodicity | 40ms, 50ms, 100ms |  |
|  |  | Observation window T1 | 1, 2, 3, 4, 5, 10 |  |
|  |  | Prediction window T2 | 5, 10 |  |
|  |  | Baseline scheme | Sample and hold (using the best beam of the last time instance in T1 as the predicted best beam in T2) |  |
| AI/ML model |  | AIML Input and Output | L1-RSRPs for different time instances |  |
|  |  | Data size Training | 600 trajectories |  |
|  |  | Data size Testing | 200 trajectories |  |
|  |  | Short model description | CNN + LSTM |  |
| Evaluation results | Beam prediction accuracy (%) | RPM: 10 Sampling interval: 100ms T1: 1, 2, 3, 4, 5, 10 T2: 5 | LSTM Top-1 | (T1=1) 0.635 (T1=2) 0.6536 (T1=3) 0.6672 (T1=4) 0.6801 (T1=5) 0.6811 (T1=10) 0.7002 |
|  |  |  | Baseline Top-1 | 0.5721 |
|  |  |  | LSTM Top-3 | (T1=1) 0.8796 (T1=2) 0.8951 (T1=3) 0.9048 (T1=4) 0.9107 (T1=5) 0.9206 (T1=10) 0.9333 |
|  |  |  | Baseline Top-3 | 0.837 |
|  |  | RPM: 10 Sampling interval: 100ms T1: 1, 2, 5 T2: 10 | LSTM Top-1 | (T1=1) 0.579 (T1=2) 0.596 (T1=5) 0.616 |
|  |  |  | Baseline Top-1 | 0.348 |
|  |  |  | LSTM Top-3 | (T1=1) 0.8535 (T1=2) 0.8656 (T1=5) 0.8848 |
|  |  |  | Baseline Top-3 | 0.6026 |
|  |  | RPM: 10 Sampling interval: 50ms T1: 1, 2, 5 T2: 5 | LSTM Top-1 | (T1=1) 0.7563 (T1=2) 0.765 (T1=5) 0.7796 |
|  |  |  | Baseline Top-1 | 0.7510 |
|  |  |  | LSTM Top-3 | (T1=1) 0.9551 (T1=2) 0.9637 (T1=5) 0.9675 |
|  |  |  | Baseline Top-3 | 0.9544 |
|  |  | RPM: 10 Sampling interval: 50ms T1: 1, 2, 5 T2: 10 | LSTM Top-1 | (T1=1) 0.6990 (T1=2) 0.7144 (T1=5) 0.7236 |
|  |  |  | Baseline Top-1 | 0.5903 |
|  |  |  | LSTM Top-3 | (T1=1) 0.9329 (T1=2) 0.9414 (T1=5) 0.9503 |
|  |  |  | Baseline Top-3 | 0.8506 |
|  |  | RPM: 50 Sampling interval: 100ms T1: 1, 2, 3, 4, 5 T2: 5 | LSTM Top-1 | (T1=1) 0.3295 (T1=2) 0.3045 (T1=3) 0.2987 (T1=4) 0.2936 (T1=5) 0.2951 |
|  |  |  | Baseline Top-1 | 0.1052 |
|  |  |  | LSTM Top-3 | (T1=1) 0.6358 (T1=2) 0.6066 (T1=3) 0.6103 (T1=4) 0.614 (T1=5) 0.6175 |
|  |  |  | Baseline Top-3 | 0.2572 |
|  |  | RPM: 50 Sampling interval: 40ms T1: 1, 2, 3, 4, 5 T2: 5 | LSTM Top-1 | (T1=1) 0.6196 (T1=2) 0.6361 (T1=3) 0.6413 (T1=4) 0.6449 (T1=5) 0.6474 |
|  |  |  | Baseline Top-1 | 0.3169 |
|  |  |  | LSTM Top-3 | (T1=1) 0.8802 (T1=2) 0.9026 (T1=3) 0.9096 (T1=4) 0.9125 (T1=5) 0.9133 |
|  |  |  | Baseline Top-3 | 0.5765 |

## Spatial Domain Beam Prediction Results

### Tx Beam Prediction (Case-1a)

| Assumptions |  | Number of [beams/beam pairs] in Set A | 64 narrow beams |  |
| --- | --- | --- | --- | --- |
|  |  | Number of [beams/beam pairs] in Set B | 8, 10, 12, 16 narrow beams |  |
|  |  | Baseline scheme | 2-step hierarchical beam search based on RSRP. Step-1 search 8 beams and select the best beam. Step-2 search 8 beams around the step-1’s best beam, the best beam is selected as the output. 16 beams are searched in total |  |
| AI/ML model |  | AIML Input and Output | L1-RSRPs |  |
|  |  | Data size Training | 175000 |  |
|  |  | Data size Testing | 50000 |  |
|  |  | Short model description | CNN |  |
|  |  |  |  | Set-B: [8, 10, 12, 16] beams |
| Evaluation results | Beam prediction accuracy (%) | NLOS+LOS, 80% indoor | AIML Top-1 | 0.5586 0.6604 0.6822 0.7273 |
|  |  |  | Baseline Top-1 | 0.6639 |
|  |  |  | AIML Top-1 (1dB margin) | 0.6432 0.7518 0.7739 0.8198 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.7296 |
|  |  |  | AIML Top-3/1 | 0.8476 0.9045 0.9238 0.9500 |
|  |  |  | Baseline Top-1/3 | 0.9141 |
|  |  | NLOS+LOS, outdoor | AIML Top-1 | 0.7818 0.8101 0.8217 0.8704 |
|  |  |  | Baseline Top-1 | 0.8235 |
|  |  |  | AIML Top-1 (1dB margin) | 0.8321 0.8625 0.8766 0.9204 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.8484 |
|  |  |  | AIML Top-3/1 | 0.9189 0.9461 0.9565 0.9773 |
|  |  |  | Baseline Top-1/3 | 0.9411 |
|  |  | Los, 80% indoor | AIML Top-1 | 0.6023 0.6929 0.7102 0.7580 |
|  |  |  | Baseline Top-1 | 0.6953 |
|  |  |  | AIML Top-1 (1dB margin) | 0.6839 0.7780 0.7974 0.845 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.7567 |
|  |  |  | AIML Top-3/1 | 0.8726 0.9198 0.9337 0.9604 |
|  |  |  | Baseline Top-1/3 | 0.9248 |
|  | System performance | RS OH reduction Option-1 |  | 87.5% 84.4% 81.3% 75% |
|  |  | RS OH reduction Option-2 |  | 50% 37.5% 25% 0% |

### Tx Beam Prediction (Case-1b)

| Assumptions |  | Number of [beams/beam pairs] in Set A | 64 narrow beams |  |
| --- | --- | --- | --- | --- |
|  |  | Number of [beams/beam pairs] in Set B | 8, 10, 12, 16 wide beams |  |
|  |  | Baseline scheme | 2-step hierarchical wide/narrow beam search based on RSRP. Step-1 search 16 wide beams and select the best wide beam. Step-2 search 4 beams within the step-1’s best wide beam, the best narrow beam is selected as the output. 20 beams are searched in total |  |
| AI/ML model |  | AIML Input and Output | L1-RSRPs |  |
|  |  | Data size Training | 175000 |  |
|  |  | Data size Testing | 50000 |  |
|  |  | Short model description | CNN |  |
|  |  |  |  | Set-B: [8, 10, 12, 16] wide beams |
| Evaluation results | Beam prediction accuracy (%) | NLOS+LOS, 80% indoor | AIML Top-1 | 0.6077 0.6390 0.6462 0.6820 |
|  |  |  | Baseline Top-1 | 0.6120 |
|  |  |  | AIML Top-1 (1dB margin) | 0.6931 0.7262 0.7368 0.7727 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.6878 |
|  |  |  | AIML Top-3/1 | 0.8805 0.9053 0.9127 0.9351 |
|  |  |  | Baseline Top-1/3 | 0.9541 |
|  |  | NLOS+LOS, outdoor | AIML Top-1 | 0.8135 0.8362 0.8473 0.8697 |
|  |  |  | Baseline Top-1 | 0.6467 |
|  |  |  | AIML Top-1 (1dB margin) | 0.8639 0.8879 0.8986 0.9217 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.6860 |
|  |  |  | AIML Top-3/1 | 0.9475 0.9631 0.9685 0.9845 |
|  |  |  | Baseline Top-1/3 | 0.9421 |
|  |  | LOS, 80% indoor | AIML Top-1 | 0.6437 0.6748 0.6783 0.7151 |
|  |  |  | Baseline Top-1 | 0.6199 |
|  |  |  | AIML Top-1 (1dB margin) | 0.7241 0.7596 0.7619 0.8018 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.6913 |
|  |  |  | AIML Top-3/1 | 0.9005 0.9225 0.9279 0.9491 |
|  |  |  | Baseline Top-1/3 | 0.9571 |
|  |  | LOS, outdoor | AIML Top-1 | 0.9047 0.9130 0.9129 0.9399 |
|  |  |  | Baseline Top-1 | 0.6758 |
|  |  |  | AIML Top-1 (1dB margin) | 0.9437 0.9525 0.9537 0.9754 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.7080 |
|  |  |  | AIML Top-3/1 | 0.9837 0.9888 0.9886 0.9969 |
|  |  |  | Baseline Top-1/3 | 0.9484 |
|  | System performance | RS OH reduction Option-1 |  | 87.5% 84.4% 81.3% 75% |
|  |  | RS OH reduction Option-2 |  | 60% 50% 40% 20% |

### Beam Pair Prediction

| Assumptions |  | Number of [beams/beam pairs] in Set A | 256 beam pairs (64 BS beams and 4 UE beams) |  |
| --- | --- | --- | --- | --- |
|  |  | Number of [beams/beam pairs] in Set B | 8, 16, 32 beam pairs |  |
|  |  | Baseline scheme | 2-step hierarchical beam search based on RSRP. Step-1 search 28 beam pairs and select the best beam. Step-2 search 4 beam pairs around the step-1’s best beam pair, the best beam pair is selected as the output. 32 beam pairs are searched in total |  |
| AI/ML model |  | AIML Input and Output | L1-RSRPs |  |
|  |  | Data size Training | 175000 |  |
|  |  | Data size Testing | 50000 |  |
|  |  | Short model description | CNN |  |
|  |  |  |  | Set-B: [8, 16, 32] beam pairs |
| Evaluation results | Beam prediction accuracy (%) | NLOS+LOS, 80% indoor | AIML Top-1 | 0.3375 0.4900 0.5829 |
|  |  |  | Baseline Top-1 | 0.3477 |
|  |  |  | AIML Top-1 (1dB margin) | 0.4042 0.5763 0.6768 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.4051 |
|  |  |  | AIML Top-3/1 | 0.6139 0.7829 0.8499 |
|  |  |  | Baseline Top-1/3 | 0.6261 |
|  |  | NLOS+LOS, outdoor | AIML Top-1 | 0.4844 0.6596 0.6938 |
|  |  |  | Baseline Top-1 | 0.2548 |
|  |  |  | AIML Top-1 (1dB margin) | 0.5432 0.7343 0.7723 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.2888 |
|  |  |  | AIML Top-3/1 | 0.7077 0.8700 0.9032 |
|  |  |  | Baseline Top-1/3 | 0.4906 |
|  |  | LOS, 80% indoor | AIML Top-1 | 0.4160 0.5861 0.6604 |
|  |  |  | Baseline Top-1 | 0.3688 |
|  |  |  | AIML Top-1 (1dB margin) | 0.4860 0.6734 0.7530 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.4236 |
|  |  |  | AIML Top-3/1 | 0.6916 0.8614 0.9058 |
|  |  |  | Baseline Top-1/3 | 0.6455 |
|  |  | LOS, outdoor | AIML Top-1 | 0.6918 0.8314 0.8553 |
|  |  |  | Baseline Top-1 | 0.2759 |
|  |  |  | AIML Top-1 (1dB margin) | 0.7522 0.8924 0.9170 |
|  |  |  | Baseline Top-1 (1dB margin) | 0.3000 |
|  |  |  | AIML Top-3/1 | 0.8683 0.9638 0.9788 |
|  |  |  | Baseline Top-1/3 | 0.5072 |
|  | System performance | RS OH reduction Option-1 |  | 96.9% 93.8% 87.5% |
|  |  | RS OH reduction Option-2 |  | 75% 50% 0% |

## SLS Dataset Generation Assumptions

| Simulation Parameters | Dense Urban MACRO eMBB |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Simulation BW | 50 MHz |
| Sub-carrier Spacing and Slot Length | 120 kHz |
| Channel Model | UMa, NLOS + LOS channel and LOS channel |
| Inter-Site Distance | 200m |
| BS Antenna Configuration for Spatial Domain Prediction | One panel: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) |
| BS Antenna Configuration for Temporal Domain Prediction | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) |
| UE Antenna Configuration | Two panels, (M, N, P) = (1, 4, 2) |
| BS Height | 25m |
| UE Deployment | Option 1: 80% indoor ,20% outdoor Option 2: 100% outdoor |