3GPP TSG RAN WG1 #113	R1-2305194

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Sharp

Title:	Evaluation of NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

# System-level simulation results for Indoor office FR1

Table 1 shows the resource utilizations, unfinished/dropped packet rate, CDF of average UPT, tail-UPT, median-UPT, packet latency, and UE-average latency for legacy TDD and SBFD. The other evaluation assumptions are in Annex. Based on the evaluation results, we observe the following:

Observation 1: For the Indoor office scenario with small packet size in FR1, -14.12% to 1.65% gain by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in DL.

Observation 2: For the Indoor office scenario with small packet size in FR1, 45.76% to 384.57% gain by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in UL.

Observation 3: For the Indoor office scenario with small packet size in FR1, -1.74% to 13.50% increase by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in DL.

Observation 4: For the Indoor office scenario with small packet size in FR1, 8.38% to 72.29% reduction by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in UL.

Observation 5: For the Indoor office scenario with small packet size in FR1, 15.96% to 58.35% decrease by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in DL.

Observation 6: For the Indoor office scenario with small packet size in FR1, 42.41% to 680.97% gain by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in UL.

Observation 7: For the Indoor office scenario with small packet size in FR1, 22.41% to 110.39% increase by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in DL.

Observation 8: For the Indoor office scenario with small packet size in FR1, 26.84% to 83.53% reduction by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in UL.

Table 1: The resource utilizations, unfinished/dropped packet rate, CDF of average UPT, tail-UPT, median-UPT, packet latency, and UE-average latency for legacy TDD and SBFD for FR1 Indoor office

| Interference Modelling |  |  | gNB self-interference - αSI | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | UE-UE co-channel inter-subband CLI | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| SBFD subband and slot configuration |  |  | SBFD slot configuration | Alt 2 | Alt 2 | Alt 2 | Alt 2 | Alt 2 | Alt 2 |
|  |  |  | SBFD Subband configuration | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | Guard symbols | 0 OS | 0 OS | 0 OS | 0 OS | 0 OS | 0 OS |
|  |  |  | UL resource percentage per TDD period | 19.66% | 19.66% | 19.66% | 19.66% | 19.66% | 19.66% |
|  |  |  | DL resource percentage per TDD period | 78.62% | 78.62% | 78.62% | 78.62% | 78.62% | 78.62% |
| BS transmit power & antenna configuration |  |  | BS transmit power for SBFD | Option1 | Option1 | Option1 | Option1 | Option1 | Option1 |
|  |  |  | BS antenna configuration for legacy TDD | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | BS antenna configuration for SBFD | Twice area&same TxRUs | Twice area&same TxRUs | Twice area&same TxRUs | Twice area&same TxRUs | Twice area&same TxRUs | Twice area&same TxRUs |
|  |  |  | BS antenna radiation pattern | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | UE antenna configuration | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
| Traffic Model |  |  | DL/UL traffic assignment for the same UE | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | DL/UL FTP packet size | Option 1 | Option 1 | Option 1 | Option 2 | Option 2 | Option 2 |
|  |  |  | DL/UL traffic load for legacy TDD | Option 1 | Option 2 | Option 3 | Option 1 | Option 2 | Option 3 |
| Channel model |  |  | gNB-gNB | Option 2 | Option 2 | Option 2 | Option 2 | Option 2 | Option 2 |
|  |  |  | UE-UE | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | UE-UE details | Option 2 | Option 2 | Option 2 | Option 2 | Option 2 | Option 2 |
| Others |  |  | Open loop power control parameters | P0= -60 dBm, alpha = 0.6 | P0= -60 dBm, alpha = 0.6 | P0= -60 dBm, alpha = 0.6 | P0= -60 dBm, alpha = 0.6 | P0= -60 dBm, alpha = 0.6 | P0= -60 dBm, alpha = 0.6 |
|  |  |  | UE receiver | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | Channel estimation | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 | Option 1 |
|  |  |  | Transmission scheme | SU-MIMO | SU-MIMO | SU-MIMO | SU-MIMO | SU-MIMO | SU-MIMO |
|  |  |  | Overhead |  |  |  |  |  |  |
|  |  |  | Additional Comments |  |  |  |  |  |  |
| RU and Unfinished/dropped Packet Rate | Legacy TDD | DL | Type-1 RU | 7.78% | 20.66% | 50.12% | 7.89% | 22.31% | 49.95% |
|  |  |  | Type-2 RU | 9.72% | 25.85% | 62.65% | 9.73% | 27.92% | 62.44% |
|  |  |  | Unfinished/dropped Packet Rate | 0.00% | 0.02% | 0.02% | 0.10% | 0.47% | 0.94% |
|  |  | UL | Type-1 RU | 1.87% | 4.86% | 11.53% | 1.81% | 5.20% | 15.41% |
|  |  |  | Type-2 RU | 9.41% | 24.17% | 57.67% | 9.57% | 25.91% | 77.04% |
|  |  |  | Unfinished/dropped Packet Rate | 0.00% | 0.08% | 0.09% | 0.12% | 0.47% | 0.43% |
|  | SBFD | DL | Type-1 RU | 5.84% | 13.12% | 24.85% | 4.98% | 15.78% | 26.10% |
|  |  |  | Type-2 RU | 9.74% | 26.51% | 62.86% | 10.07% | 39.95% | 76.65% |
|  |  |  | Unfinished/dropped Packet Rate | 0.00% | 0.02% | 0.03% | 0.00% | 0.56% | 1.61% |
|  |  | UL | Type-1 RU | 0.67% | 1.50% | 3.41% | 1.21% | 2.78% | 7.69% |
|  |  |  | Type-2 RU | 8.62% | 11.29% | 18.22% | 9.03% | 15.42% | 38.33% |
|  |  |  | Unfinished/dropped Packet Rate | 0.02% | 0.09% | 0.10% | 0.00% | 0.47% | 0.56% |
| UPT gain of SBFD over legacy TDD (e.g., X%=SBFD UPT/TDD UPT-1) | DL-UPT | Average-UPT CDF | Mean | -0.38% | -0.44% | -1.40% | -21.63% | -30.54% | -36.62% |
|  |  |  | 5% | -1.40% | 0.45% | -1.27% | -28.00% | -47.00% | -49.99% |
|  |  |  | 50% | -0.61% | 0.15% | -1.18% | -22.46% | -34.43% | -45.73% |
|  |  |  | 95% | 0.33% | -0.65% | -1.21% | -20.35% | -18.27% | -23.38% |
|  |  | Tail-UPT CDF | Mean | -0.02% | -1.76% | -3.76% | -21.63% | -35.33% | -43.34% |
|  |  |  | 5% | -0.35% | -3.45% | -14.12% | -20.79% | -57.67% | -49.29% |
|  |  |  | 50% | 0.16% | -4.19% | -1.34% | -28.10% | -43.22% | -58.35% |
|  |  |  | 95% | 1.29% | -2.75% | -5.60% | -18.77% | -21.27% | -29.13% |
|  |  | Median-UPT CDF | Mean | -0.38% | -0.54% | -1.48% | -21.05% | -31.02% | -38.94% |
|  |  |  | 5% | -1.35% | 1.65% | -1.83% | -26.90% | -45.85% | -48.28% |
|  |  |  | 50% | 0.03% | -0.85% | -0.53% | -23.96% | -36.83% | -49.31% |
|  |  |  | 95% | 0.46% | -0.51% | -0.88% | -15.96% | -18.65% | -19.22% |
|  | UL-UPT | Average-UPT CDF | Mean | 53.41% | 55.74% | 68.64% | 67.73% | 79.79% | 150.47% |
|  |  |  | 5% | 59.75% | 72.72% | 87.06% | 115.32% | 146.33% | 366.69% |
|  |  |  | 50% | 54.46% | 54.78% | 69.31% | 64.46% | 82.38% | 159.28% |
|  |  |  | 95% | 46.41% | 45.76% | 52.26% | 42.41% | 63.88% | 95.45% |
|  |  | Tail-UPT CDF | Mean | 83.05% | 107.13% | 203.22% | 69.81% | 100.79% | 284.33% |
|  |  |  | 5% | 77.57% | 171.92% | 384.57% | 127.36% | 213.05% | 680.97% |
|  |  |  | 50% | 81.64% | 82.56% | 222.09% | 71.26% | 109.82% | 423.88% |
|  |  |  | 95% | 87.34% | 90.30% | 82.98% | 51.28% | 58.54% | 200.58% |
|  |  | Median-UPT CDF | Mean | 72.50% | 74.58% | 89.12% | 70.89% | 81.87% | 159.10% |
|  |  |  | 5% | 85.16% | 91.00% | 112.21% | 112.06% | 157.15% | 495.82% |
|  |  |  | 50% | 73.87% | 77.65% | 89.95% | 64.60% | 66.12% | 169.21% |
|  |  |  | 95% | 60.34% | 55.95% | 71.20% | 42.92% | 63.62% | 96.75% |
| Latency gain of SBFD compared to legacy TDD (e.g., X%=SBFD latency/TDD latency - 1) | DL-Latency | Packet-Latency CDF (baseline) | Mean | 5.10% | 2.51% | 13.50% | 27.20% | 63.71% | 95.45% |
|  |  |  | 5% | -0.11% | 0.02% | 0.39% | 23.20% | 22.58% | 24.79% |
|  |  |  | 50% | 0.44% | 0.68% | 1.09% | 26.59% | 53.16% | 68.77% |
|  |  |  | 95% | -0.01% | 1.44% | 6.06% | 25.73% | 77.43% | 77.16% |
|  |  | UE-Average-Latency CDF (optional) | Mean | 0.94% | 0.90% | 3.11% | 28.34% | 65.68% | 99.55% |
|  |  |  | 5% | 0.16% | 1.20% | 0.85% | 23.53% | 22.41% | 38.51% |
|  |  |  | 50% | 1.06% | 1.34% | 3.60% | 30.52% | 58.13% | 110.39% |
|  |  |  | 95% | 0.97% | -1.74% | 9.34% | 32.07% | 87.60% | 79.19% |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean | -40.66% | -44.10% | -55.35% | -44.25% | -52.34% | -74.75% |
|  |  |  | 5% | -9.05% | -8.38% | -10.55% | -26.84% | -34.82% | -46.44% |
|  |  |  | 50% | -42.19% | -42.77% | -47.04% | -41.25% | -43.77% | -60.24% |
|  |  |  | 95% | -44.69% | -53.26% | -72.29% | -50.50% | -59.58% | -82.58% |
|  |  | UE-Average-Latency CDF (optional) | Mean | -40.46% | -43.26% | -53.29% | -45.04% | -51.57% | -77.25% |
|  |  |  | 5% | -38.49% | -37.75% | -41.21% | -29.60% | -39.61% | -55.27% |
|  |  |  | 50% | -40.54% | -42.78% | -53.01% | -38.16% | -48.02% | -72.04% |
|  |  |  | 95% | -41.83% | -49.12% | -63.60% | -53.27% | -61.97% | -83.53% |
| UPT of legacy TDD (absolute value) | DL-UPT | Average-UPT CDF | Mean (Mbps) | 40.04359802 | 37.5816802 | 33.4863604 | 448.9484554 | 346.9902574 | 245.540797 |
|  |  |  | 5% (Mbps) | 38.5714 | 34.0641 | 26.3087 | 338.063 | 181.999 | 69.6628 |
|  |  |  | 50% (Mbps) | 40.1461 | 37.6282 | 33.9275 | 458.978 | 367.791 | 239.851 |
|  |  |  | 95% (Mbps) | 41.3416 | 40.7438 | 38.4563 | 552.564 | 480.683 | 448.788 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 22.5241099 | 18.38058317 | 12.66000168 | 353.460099 | 214.4522941 | 95.33209208 |
|  |  |  | 5% (Mbps) | 21.486 | 13.1599 | 6.49949 | 175.136 | 76.4972 | 17.3026 |
|  |  |  | 50% (Mbps) | 22.4722 | 18.344 | 12.9958 | 381.777 | 195.287 | 73.4008 |
|  |  |  | 95% (Mbps) | 23.7647 | 23.2317 | 20.2079 | 472.32 | 450.879 | 222.041 |
|  |  | Median-UPT CDF | Mean (Mbps) | 38.88310693 | 36.76651287 | 32.48981287 | 446.9382277 | 350.2882871 | 248.3943386 |
|  |  |  | 5% (Mbps) | 37.0606 | 33.3354 | 22.7741 | 305.917 | 160.322 | 50.078 |
|  |  |  | 50% (Mbps) | 38.9163 | 36.7334 | 33.7076 | 466.805 | 365.593 | 226.982 |
|  |  |  | 95% (Mbps) | 40.7131 | 40.0517 | 37.9242 | 514.776 | 483.464 | 474.558 |
|  | UL-UPT | Average-UPT CDF | Mean (Mbps) | 3.988571287 | 3.88028495 | 3.492994752 | 130.659105 | 123.7965723 | 91.20211881 |
|  |  |  | 5% (Mbps) | 3.66626 | 3.2855 | 3.01654 | 64.5413 | 59.5968 | 29.0527 |
|  |  |  | 50% (Mbps) | 3.97808 | 3.91866 | 3.49437 | 129.737 | 117.549 | 86.0923 |
|  |  |  | 95% (Mbps) | 4.31698 | 4.38014 | 4.00801 | 215.508 | 205.547 | 167.411 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 2.362522772 | 2.032454772 | 1.289875743 | 94.04210198 | 71.64046931 | 25.19140549 |
|  |  |  | 5% (Mbps) | 2.29346 | 1.34665 | 0.690152 | 42.5508 | 22.5487 | 4.50048 |
|  |  |  | 50% (Mbps) | 2.3644 | 2.28669 | 1.24595 | 90.5977 | 66.9231 | 17.8063 |
|  |  |  | 95% (Mbps) | 2.5203 | 2.49254 | 2.32944 | 164.142 | 144.456 | 59.6803 |
|  |  | Median-UPT CDF | Mean (Mbps) | 3.538100891 | 3.464869505 | 3.120783663 | 125.2932752 | 119.3451842 | 86.94978861 |
|  |  |  | 5% (Mbps) | 3.11964 | 2.94522 | 2.6613 | 61.6541 | 54.1202 | 20.8631 |
|  |  |  | 50% (Mbps) | 3.516 | 3.41301 | 3.1055 | 125.088 | 123.554 | 80.7255 |
|  |  |  | 95% (Mbps) | 3.98582 | 4.18354 | 3.58818 | 213.999 | 202.644 | 171.441 |
| UPT of SBFD (absolute value) | DL-UPT | Average-UPT CDF | Mean (Mbps) | 39.89085842 | 37.41616733 | 33.01619703 | 351.8410792 | 241.0241149 | 155.6224347 |
|  |  |  | 5% (Mbps) | 38.0305 | 34.2178 | 25.9738 | 243.393 | 96.4549 | 34.8359 |
|  |  |  | 50% (Mbps) | 39.9017 | 37.683 | 33.526 | 355.882 | 241.148 | 130.178 |
|  |  |  | 95% (Mbps) | 41.4795 | 40.4808 | 37.9906 | 440.141 | 392.863 | 343.848 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 22.51986931 | 18.05706832 | 12.18389109 | 277.0113861 | 138.6893307 | 54.01866267 |
|  |  |  | 5% (Mbps) | 21.4102 | 12.7064 | 5.58164 | 138.724 | 32.3825 | 8.77476 |
|  |  |  | 50% (Mbps) | 22.5078 | 17.5762 | 12.8218 | 274.503 | 110.883 | 30.5751 |
|  |  |  | 95% (Mbps) | 24.071 | 22.5921 | 19.077 | 383.647 | 354.968 | 157.353 |
|  |  | Median-UPT CDF | Mean (Mbps) | 38.7361802 | 36.56946832 | 32.00832673 | 352.8734455 | 241.6253901 | 151.658195 |
|  |  |  | 5% (Mbps) | 36.5614 | 33.8849 | 22.3579 | 223.62 | 86.8101 | 25.899 |
|  |  |  | 50% (Mbps) | 38.9277 | 36.4219 | 33.5302 | 354.98 | 230.953 | 115.068 |
|  |  |  | 95% (Mbps) | 40.8987 | 39.8472 | 37.5892 | 432.599 | 393.317 | 383.367 |
|  | UL-UPT | Average-UPT CDF | Mean (Mbps) | 6.118929901 | 6.043181485 | 5.890591386 | 219.1573069 | 222.5770594 | 228.4363168 |
|  |  |  | 5% (Mbps) | 5.85686 | 5.6747 | 5.64274 | 138.968 | 146.805 | 135.587 |
|  |  |  | 50% (Mbps) | 6.14454 | 6.06547 | 5.91643 | 213.371 | 214.39 | 223.224 |
|  |  |  | 95% (Mbps) | 6.32041 | 6.38465 | 6.10275 | 306.898 | 336.854 | 327.198 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 4.32469099 | 4.209822673 | 3.91115495 | 159.692504 | 143.8490188 | 96.81843069 |
|  |  |  | 5% (Mbps) | 4.07258 | 3.66177 | 3.34425 | 96.7452 | 70.5884 | 35.1472 |
|  |  |  | 50% (Mbps) | 4.29465 | 4.17463 | 4.01308 | 155.161 | 140.417 | 93.2836 |
|  |  |  | 95% (Mbps) | 4.72164 | 4.74331 | 4.26233 | 248.309 | 229.021 | 179.385 |
|  |  | Median-UPT CDF | Mean (Mbps) | 6.103223465 | 6.049086436 | 5.901986733 | 214.1191782 | 217.0550495 | 225.2847822 |
|  |  |  | 5% (Mbps) | 5.77638 | 5.62526 | 5.64767 | 130.745 | 139.17 | 124.307 |
|  |  |  | 50% (Mbps) | 6.11327 | 6.06314 | 5.89885 | 205.89 | 205.254 | 217.323 |
|  |  |  | 95% (Mbps) | 6.39077 | 6.52413 | 6.14283 | 305.852 | 331.557 | 337.31 |
| Latency of legacy TDD (absolute value) | DL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 0.926463109 | 1.02653898 | 1.567952099 | 9.40285495 | 14.87121772 | 31.46296604 |
|  |  |  | 5% (ms) | 0.532418 | 0.53718 | 0.544594 | 6.51952 | 8.03402 | 8.12889 |
|  |  |  | 50% (ms) | 0.822036 | 0.867587 | 0.95043 | 8.54079 | 10.9661 | 16.9871 |
|  |  |  | 95% (ms) | 1.42327 | 1.85448 | 2.94074 | 14.7145 | 33.2771 | 104.706 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 0.877316277 | 0.984889733 | 1.270841574 | 9.371747723 | 14.35495337 | 32.14110485 |
|  |  |  | 5% (ms) | 0.837776 | 0.855836 | 0.936505 | 7.37164 | 8.3247 | 9.68201 |
|  |  |  | 50% (ms) | 0.874694 | 0.964165 | 1.18208 | 8.82301 | 12.3351 | 22.4595 |
|  |  |  | 95% (ms) | 0.92878 | 1.16173 | 1.85034 | 12.8724 | 25.4524 | 87.0069 |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 2.284410792 | 2.468508812 | 3.215379208 | 9.065012574 | 10.82342881 | 24.75553139 |
|  |  |  | 5% (ms) | 1.13211 | 1.13119 | 1.15598 | 4.00984 | 4.06068 | 4.39312 |
|  |  |  | 50% (ms) | 2.26294 | 2.31006 | 2.55695 | 8.02139 | 8.25047 | 11.5745 |
|  |  |  | 95% (ms) | 3.38828 | 4.11533 | 7.2053 | 16.2195 | 23.3569 | 77.3116 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 2.263136733 | 2.41259198 | 3.048874257 | 9.138946832 | 10.52276871 | 25.59257089 |
|  |  |  | 5% (ms) | 2.09817 | 2.06534 | 2.31159 | 4.7527 | 5.05202 | 7.43001 |
|  |  |  | 50% (ms) | 2.26383 | 2.39233 | 3.02482 | 7.90156 | 9.32322 | 18.86 |
|  |  |  | 95% (ms) | 2.43271 | 2.87689 | 4.11321 | 15.9349 | 19.4176 | 61.738 |
| Latency of SBFD (absolute value) | DL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 0.973699594 | 1.052346495 | 1.779567436 | 11.96062158 | 24.34582545 | 61.49564267 |
|  |  |  | 5% (ms) | 0.531811 | 0.53728 | 0.546738 | 8.03218 | 9.84789 | 10.1437 |
|  |  |  | 50% (ms) | 0.825642 | 0.873496 | 0.960822 | 10.8115 | 16.7953 | 28.6685 |
|  |  |  | 95% (ms) | 1.4231 | 1.88117 | 3.11907 | 18.5012 | 59.0451 | 185.501 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 0.885535861 | 0.993728426 | 1.31031602 | 12.02741812 | 23.78289307 | 64.13784653 |
|  |  |  | 5% (ms) | 0.83908 | 0.866068 | 0.944458 | 9.106 | 10.1899 | 13.4108 |
|  |  |  | 50% (ms) | 0.883942 | 0.97712 | 1.22466 | 11.5156 | 19.5058 | 47.2524 |
|  |  |  | 95% (ms) | 0.937778 | 1.14154 | 2.02309 | 17.0002 | 47.7481 | 155.909 |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 1.355522485 | 1.379983337 | 1.435512634 | 5.053780099 | 5.158559208 | 6.250639307 |
|  |  |  | 5% (ms) | 1.02962 | 1.03641 | 1.03404 | 2.93344 | 2.64687 | 2.35305 |
|  |  |  | 50% (ms) | 1.30825 | 1.32196 | 1.35428 | 4.71285 | 4.63887 | 4.6022 |
|  |  |  | 95% (ms) | 1.87394 | 1.9235 | 1.99663 | 8.02927 | 9.44053 | 13.4665 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 1.347494653 | 1.368997525 | 1.424014554 | 5.022619208 | 5.096374653 | 5.821094851 |
|  |  |  | 5% (ms) | 1.2905 | 1.28573 | 1.35888 | 3.34586 | 3.05098 | 3.3232 |
|  |  |  | 50% (ms) | 1.34613 | 1.36893 | 1.42135 | 4.88648 | 4.84653 | 5.27235 |
|  |  |  | 95% (ms) | 1.41515 | 1.46381 | 1.49739 | 7.4469 | 7.38388 | 10.1689 |

# System-level simulation results for UMa FR1

Table 2 shows the resource utilizations, unfinished/dropped packet rate, CDF of average UPT, tail-UPT, median-UPT, packet latency, and UE-average latency for legacy TDD and SBFD. The other evaluation assumptions are in Annex.

Table 2: The resource utilizations, unfinished/dropped packet rate, CDF of average UPT, tail-UPT, median-UPT, packet latency, and UE-average latency for legacy TDD and SBFD for FR1 UMa

| Layout |  |  | Macro Layer | Option 1 | Option 1 |
| --- | --- | --- | --- | --- | --- |
| UE distribution |  |  | UE distribution | Option 2 | Option 1 |
| Interference Modelling |  |  | gNB self-interference - αSI | Option 1 | Option 1 |
|  |  |  | Co-site inter-sector co-channel inter-subband CLI | Option 2 | Option 4 |
|  |  |  | UE-UE co-channel inter-subband CLI | 33dBc | 33dBc |
| SBFD subband and slot configuration |  |  | SBFD slot configuration | Alt 2 | Alt 2 |
|  |  |  | SBFD Subband configuration | Option 1 | Option 1 |
|  |  |  | Guard symbols | 0 | 0 |
|  |  |  | UL resource percentage per TDD period | 19.66% | 19.66% |
|  |  |  | DL resource percentage per TDD period | 78.62% | 78.62% |
| BS transmit power & antenna configuration |  |  | BS transmit power for legacy TDD | Option 1 | Option 1 |
|  |  |  | BS transmit power for SBFD | Option 1 | Option 1 |
|  |  |  | BS antenna configuration for legacy TDD | Option 1 | Option 1 |
|  |  |  | BS antenna configuration for SBFD | Twice area&same TxRUs | Twice area&same TxRUs |
|  |  |  | BS antenna radiation pattern | Option 1 | Option 1 |
|  |  |  | UE antenna configuration | Option 1 | Option 2 |
| Traffic Model |  |  | DL/UL traffic assignment for the same UE | Option 1 | Option 1 |
|  |  |  | DL/UL FTP packet size | Option 2 | Option 2 |
|  |  |  | DL/UL traffic load for legacy TDD | Option 2 | Option 3 |
| Channel model |  |  | gNB-gNB | Option 2 | Option 2 |
|  |  |  | UE-UE | Option 2 | Option 2 |
|  |  |  | UE-UE details | Option2 | Option 2 |
| Others |  |  | Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -80 dBm, alpha = 0.8 |
|  |  |  | UE receiver | Option 1 | Option 1 |
|  |  |  | Channel estimation | Option 1 | Option 1 |
|  |  |  | Transmission scheme | SU-MIMO | SU-MIMO |
|  |  |  | Overhead |  |  |
|  |  |  | Additional Comments |  |  |
| RU and Unfinished/dropped Packet Rate | Legacy TDD | DL | Type-1 RU | 16.47% | 42.42% |
|  |  |  | Type-2 RU | 20.64% | 53.03% |
|  |  |  | Unfinished/dropped Packet Rate | 0.26% | 1.02% |
|  |  | UL | Type-1 RU | 4.51% | 11.89% |
|  |  |  | Type-2 RU | 22.30% | 59.46% |
|  |  |  | Unfinished/dropped Packet Rate | 0.53% | 1.41% |
|  | SBFD | DL | Type-1 RU | 11.08% | 26.55% |
|  |  |  | Type-2 RU | 31.79% | 78.40% |
|  |  |  | Unfinished/dropped Packet Rate | 0.82% | 3.37% |
|  |  | UL | Type-1 RU | 2.88% | 8.05% |
|  |  |  | Type-2 RU | 14.40% | 40.12% |
|  |  |  | Unfinished/dropped Packet Rate | 1.57% | 3.85% |
| UPT gain of SBFD over legacy TDD (e.g., X%=SBFD UPT/TDD UPT-1) | DL-UPT | Average-UPT CDF | Mean | -29.66% | -49.55% |
|  |  |  | 5% | -53.15% | -84.44% |
|  |  |  | 50% | -31.33% | -59.89% |
|  |  |  | 95% | -20.03% | -27.44% |
|  |  | Tail-UPT CDF | Mean | -36.50% | -54.31% |
|  |  |  | 5% | -75.11% | -69.72% |
|  |  |  | 50% | -40.33% | -74.30% |
|  |  |  | 95% | -30.66% | -28.22% |
|  |  | Median-UPT CDF | Mean | -30.46% | -52.80% |
|  |  |  | 5% | -61.08% | -84.84% |
|  |  |  | 50% | -30.48% | -68.41% |
|  |  |  | 95% | -18.85% | -26.68% |
|  | UL-UPT | Average-UPT CDF | Mean | 77.32% | 68.01% |
|  |  |  | 5% | 93.72% | 136.62% |
|  |  |  | 50% | 76.54% | 54.98% |
|  |  |  | 95% | 84.44% | 86.54% |
|  |  | Tail-UPT CDF | Mean | 88.89% | 97.01% |
|  |  |  | 5% | 98.02% | 92.63% |
|  |  |  | 50% | 99.15% | 112.98% |
|  |  |  | 95% | 91.94% | 72.09% |
|  |  | Median-UPT CDF | Mean | 80.82% | 65.99% |
|  |  |  | 5% | 122.17% | 80.73% |
|  |  |  | 50% | 81.65% | 88.59% |
|  |  |  | 95% | 77.85% | 78.92% |
| Latency gain of SBFD compared to legacy TDD (e.g., X%=SBFD latency/TDD latency - 1) | DL-Latency | Packet-Latency CDF (baseline) | Mean | 133.77% | 184.12% |
|  |  |  | 5% | 26.23% | 26.49% |
|  |  |  | 50% | 39.62% | 111.31% |
|  |  |  | 95% | 140.54% | 261.93% |
|  |  | UE-Average-Latency CDF (optional) | Mean | 151.07% | 313.76% |
|  |  |  | 5% | 34.35% | 47.81% |
|  |  |  | 50% | 49.73% | 239.88% |
|  |  |  | 95% | 237.24% | 279.04% |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean | -40.94% | -42.45% |
|  |  |  | 5% | -42.23% | -40.14% |
|  |  |  | 50% | -46.67% | -38.80% |
|  |  |  | 95% | -50.08% | -38.17% |
|  |  | UE-Average-Latency CDF (optional) | Mean | -44.99% | -39.12% |
|  |  |  | 5% | -45.57% | -40.80% |
|  |  |  | 50% | -42.30% | -41.20% |
|  |  |  | 95% | -33.68% | -37.16% |
| UPT of legacy TDD (absolute value) | DL-UPT | Average-UPT CDF | Mean (Mbps) | 270.3095 | 219.6332 |
|  |  |  | 5% (Mbps) | 110.263 | 57.62 |
|  |  |  | 50% (Mbps) | 255.956 | 196.451 |
|  |  |  | 95% (Mbps) | 463.305 | 440.575 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 158.3976 | 98.43995 |
|  |  |  | 5% (Mbps) | 36.6623 | 9.19641 |
|  |  |  | 50% (Mbps) | 128.045 | 66.5462 |
|  |  |  | 95% (Mbps) | 388.101 | 304.032 |
|  |  | Median-UPT CDF | Mean (Mbps) | 266.0992 | 212.7908 |
|  |  |  | 5% (Mbps) | 95.0548 | 31.5329 |
|  |  |  | 50% (Mbps) | 244.874 | 180.284 |
|  |  |  | 95% (Mbps) | 467.29 | 466.79 |
|  | UL-UPT | Average-UPT CDF | Mean (Mbps) | 88.10232 | 86.10152 |
|  |  |  | 5% (Mbps) | 7.84045 | 2.65278 |
|  |  |  | 50% (Mbps) | 70.29 | 70.3753 |
|  |  |  | 95% (Mbps) | 233.173 | 208.09 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 61.2977 | 42.52029 |
|  |  |  | 5% (Mbps) | 4.56058 | 1.33971 |
|  |  |  | 50% (Mbps) | 40.8608 | 22.2987 |
|  |  |  | 95% (Mbps) | 206.253 | 168.237 |
|  |  | Median-UPT CDF | Mean (Mbps) | 82.96387 | 83.31172 |
|  |  |  | 5% (Mbps) | 6.57009 | 2.32414 |
|  |  |  | 50% (Mbps) | 67.001 | 55.0592 |
|  |  |  | 95% (Mbps) | 234.304 | 216.357 |
| UPT of SBFD (absolute value) | DL-UPT | Average-UPT CDF | Mean (Mbps) | 190.1292 | 110.7977 |
|  |  |  | 5% (Mbps) | 51.6615 | 8.96353 |
|  |  |  | 50% (Mbps) | 175.776 | 78.803 |
|  |  |  | 95% (Mbps) | 370.518 | 319.674 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 100.5813 | 44.98064 |
|  |  |  | 5% (Mbps) | 9.12433 | 2.78488 |
|  |  |  | 50% (Mbps) | 76.3996 | 17.1002 |
|  |  |  | 95% (Mbps) | 269.091 | 218.248 |
|  |  | Median-UPT CDF | Mean (Mbps) | 185.0354 | 100.4397 |
|  |  |  | 5% (Mbps) | 36.9916 | 4.78111 |
|  |  |  | 50% (Mbps) | 170.23 | 56.9499 |
|  |  |  | 95% (Mbps) | 379.218 | 342.238 |
|  | UL-UPT | Average-UPT CDF | Mean (Mbps) | 156.2251 | 144.6552 |
|  |  |  | 5% (Mbps) | 15.1882 | 6.27688 |
|  |  |  | 50% (Mbps) | 124.087 | 109.068 |
|  |  |  | 95% (Mbps) | 430.058 | 388.168 |
|  |  | Tail-UPT CDF | Mean (Mbps) | 115.7849 | 83.76737 |
|  |  |  | 5% (Mbps) | 9.03078 | 2.58063 |
|  |  |  | 50% (Mbps) | 81.373 | 47.491 |
|  |  |  | 95% (Mbps) | 395.879 | 289.527 |
|  |  | Median-UPT CDF | Mean (Mbps) | 150.0115 | 138.2905 |
|  |  |  | 5% (Mbps) | 14.5966 | 4.2005 |
|  |  |  | 50% (Mbps) | 121.709 | 103.835 |
|  |  |  | 95% (Mbps) | 416.719 | 387.116 |
| Latency of legacy TDD (absolute value) | DL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 22.96274 | 45.97276 |
|  |  |  | 5% (ms) | 8.05875 | 8.22428 |
|  |  |  | 50% (ms) | 15.9937 | 20.0021 |
|  |  |  | 95% (ms) | 54.8208 | 140.063 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 21.92341 | 46.55314 |
|  |  |  | 5% (ms) | 8.89578 | 9.57362 |
|  |  |  | 50% (ms) | 18.1761 | 26.5872 |
|  |  |  | 95% (ms) | 42.9928 | 165.945 |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 37.68883 | 74.41836 |
|  |  |  | 5% (ms) | 4.0942 | 4.00636 |
|  |  |  | 50% (ms) | 14.3235 | 13.686 |
|  |  |  | 95% (ms) | 146.112 | 268.866 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 37.78701 | 86.83339 |
|  |  |  | 5% (ms) | 4.70165 | 4.97408 |
|  |  |  | 50% (ms) | 17.5827 | 20.3094 |
|  |  |  | 95% (ms) | 213.494 | 411.762 |
| Latency of SBFD (absolute value) | DL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 53.67919 | 130.6196 |
|  |  |  | 5% (ms) | 10.1725 | 10.4032 |
|  |  |  | 50% (ms) | 22.3298 | 42.2671 |
|  |  |  | 95% (ms) | 131.867 | 506.924 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 55.04365 | 192.6177 |
|  |  |  | 5% (ms) | 11.9512 | 14.1506 |
|  |  |  | 50% (ms) | 27.2148 | 90.3648 |
|  |  |  | 95% (ms) | 144.991 | 629.004 |
|  | UL-Latency | Packet-Latency CDF (baseline) | Mean (ms) | 22.26088 | 42.83084 |
|  |  |  | 5% (ms) | 2.3652 | 2.39815 |
|  |  |  | 50% (ms) | 7.63811 | 8.37584 |
|  |  |  | 95% (ms) | 72.9439 | 166.253 |
|  |  | UE-Average-Latency CDF (optional) | Mean (ms) | 20.78821 | 52.85998 |
|  |  |  | 5% (ms) | 2.55933 | 2.94452 |
|  |  |  | 50% (ms) | 10.1453 | 11.9427 |
|  |  |  | 95% (ms) | 141.595 | 258.739 |

# Conclusion

In this contribution, we observe the following:

Observation 1: For the Indoor office scenario with small packet size in FR1, -14.12% to 1.65% gain by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in DL.

Observation 2: For the Indoor office scenario with small packet size in FR1, 45.76% to 384.57% gain by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in UL.

Observation 3: For the Indoor office scenario with small packet size in FR1, -1.74% to 13.50% increase by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in DL.

Observation 4: For the Indoor office scenario with small packet size in FR1, 8.38% to 72.29% reduction by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in UL.

Observation 5: For the Indoor office scenario with small packet size in FR1, 15.96% to 58.35% decrease by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in DL.

Observation 6: For the Indoor office scenario with small packet size in FR1, 42.41% to 680.97% gain by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile average-UPT/tail-UPT/median-UPT in UL.

Observation 7: For the Indoor office scenario with small packet size in FR1, 22.41% to 110.39% increase by the SBFD over the legacy TDD is observed in 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in DL.

Observation 8: For the Indoor office scenario with small packet size in FR1, 26.84% to 83.53% reduction by the SBFD over the legacy TDD is observed in terms of 5%-tile, 50%-tile, and 95%-tile packet latency/UE-average latency in UL.

# Annex

Table A1 summarizes system-level simulation assumptions for the Indoor office.

Table A1: Evaluation assumptions for the Indoor office

| SBFD RF configuration |  |
| --- | --- |
| Self-interference | 1 dB sensitivity degradation |
| In channel sensitivity at gNB | 46 dB |
| In channel leakage ratio at gNB | 45 dB |
| In channel sensitivity at UE | 33 dB |
| In channel leakage ratio at UE | Follows ACLR for UE in TS38.101-1 |
| SBFD baseband configuration |  |
| SBFD slot configuration | XXXXU |
| SBFD subband configuration | <ND, NU, NG >=<104, 55, 5> |
| Guard symbols | 0 OFDM symbols |
| UL resource percentage per TDD period | 60.95% |
| DL resource percentage per TDD period | 34.95% |
| Antenna configuration |  |
| BS transmit power | 24 dBm, power scaling is not used |
| UE transmit power | 23 dBm as maximum (P0 = -60 dB, alpha = 1) |
| BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) |
| BS antenna configuration for SBFD | Twice area & same TxRUs |
| BS antenna radiation pattern | Ceiling mount |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2) |
| Traffic model |  |
| DL/UL traffic assignment | Each UE is either assigned UL or DL |
| FTP packet size | 0.004/0.001 MB for DL/UL for small packet size 0.5/0.125 MB for DL/UL for large packet size |
| Channel model |  |
| gNB-UE | Both large and small scale fading |
| gNB-gNB | Only large scale fading |
| UE-UE | Only large scale fading |
| Other parameters |  |
| UE receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Transmission scheme | SU-MIMO |
| Control overhead | 2 OFDM symbols in each DL or SBFD slot |

Table A2 summarizes system-level simulation assumptions for the UMa.

Table A2: Evaluation assumptions for the UMa

| SBFD RF configuration |  |
| --- | --- |
| Self-interference | 1 dB sensitivity degradation |
| In channel sensitivity at gNB | 46 dB |
| In channel leakage ratio at gNB | 45 dB |
| In channel sensitivity at UE | 33 dB |
| In channel leakage ratio at UE | Follows ACLR for UE in TS38.101-1 |
| SBFD baseband configuration |  |
| SBFD slot configuration | XXXXU |
| SBFD subband configuration | <ND, NU, NG >=<104, 55, 5> |
| Guard symbols | 0 OFDM symbols |
| UL resource percentage per TDD period | 60.95% |
| DL resource percentage per TDD period | 34.95% |
| Antenna configuration |  |
| BS transmit power | 50 dBm, power scaling is not used |
| UE transmit power | 23 dBm as maximum (P0 = -60 dB, alpha = 1) |
| BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,88,2,1,1; 2,8) |
| BS antenna configuration for SBFD | Twice area & same TxRUs |
| BS antenna radiation pattern | According to TR38.901 |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2) |
| Traffic model |  |
| DL/UL traffic assignment | Each UE is either assigned UL or DL |
| FTP packet size | 0.004/0.001 MB for DL/UL for small packet size 0.5/0.125 MB for DL/UL for large packet size |
| Channel model |  |
| gNB-UE | Both large and small scale fading |
| gNB-gNB | Only large scale fading |
| UE-UE | Only large scale fading |
| Other parameters |  |
| UE receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Transmission scheme | SU-MIMO |
| Control overhead | 2 OFDM symbols in each DL or SBFD slot |