3GPP TSG RAN WG1 #113			   R1-2305314

Meeting, May 22nd – May 26th, 2023

Title:	[Draft] Reply LS to RAN2 on multicast reception in RRC_INACTIVE

Response to:	R1-2304325(R2-2304330)

Release:	Release 18

Work Item:	NR_MBS_enh-Core

Source:	Qualcomm [RAN WG1]

To:	RAN WG2

Cc:	N/A

Contact Person:

#### Name:	Le Liu

#### E-mail Address:	leliu (at) qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for providing the LS on the progress on multicast reception in RRC_INACTIVE and ask the questions to RAN1. RAN1 has discussed the questions from RAN2 and would like to provide the following response to RAN2.

- Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed?

  - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used.

RAN1 response: RAN1 confirms the above RAN2 assumption is feasible. Compared with DCI format 4_2, DCI format 4_1 with fixed DCI size is simpler and recommended as the basic DCI format for multicast MTCH reception in RRC_INACTIVE.

- Question 2: Is the following RAN2 understanding correct?

  - RAN2 understanding is that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 for multicast MTCH in RRC_CONNECTED as well as for broadcast MTCH).

RAN1 response: RAN1 confirms the above RAN2 understanding is correct. For slot-level repetition for multicast MTCH PDSCH reception in RRC_INACTIVE,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor per G-RNTI for multicast MTCH PDSCH in RRC_INACTIVE

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table for scheduling multicast MTCH PDSCH in RRC_INACTIVE.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same multicast PDSCH.

- Question 3: Is it feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH?

  - 3.1) Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED).

  - 3.2) Separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE.

RAN1 response: From RAN1 perspective, it is feasible to reuse Type3-CSS for multicast MTCH in RRC_INACTIVE and reuse Type0B-CSS for multicast MCCH in RRC_INACTIVE. Therefore, separate CSSes for multicast MCCH and multicast MTCH are configured for UEs for multicast reception in RRC_INACTIVE.

2. Actions:

To RAN1 group.

ACTION: 	RAN1 respectfully requests RAN2 group to take the provided information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN1 Meeting#114		 21th – 25th August 2023	Toulouse

TSG-RAN1 Meeting#114bis	 9th – 13th October 2023	Xiamen