3GPP TSG RAN WG1 #113	R1-2306273

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.9

Source:	WI rapporteur (MediaTek)

Title:	RAN1 agreements for Rel-18 WI on IoT NTN enhancements up to RAN1#113

Document for:	Information

# Content

Content	1

1	Introduction	1

2	WID Objective	2

3	3GPP TSG RAN WG1 Meeting #109-e	4

3.1	Disabling of HARQ feedback for IoT NTN	4

3.2	Improved GNSS operations for IoT NTN	4

4	3GPP TSG RAN WG1 Meeting #110	5

4.1	Disabling of HARQ feedback for IoT NTN	5

4.2	Improved GNSS operations for IoT NTN	6

5	3GPP TSG RAN WG1 Meeting #110-bis	6

5.1	Disabling of HARQ feedback for IoT NTN	6

5.2	Improved GNSS operations for IoT NTN	6

6	3GPP TSG RAN WG1 Meeting #111	6

6.1	Disabling of HARQ feedback for IoT NTN	6

6.2	Improved GNSS operations for IoT NTN	7

7	3GPP TSG RAN WG1 Meeting #112	7

7.1	Disabling of HARQ feedback for IoT NTN	7

7.2	Improved GNSS operations for IoT NTN	8

8	3GPP TSG RAN WG1 Meeting #112bis-e	9

8.1	Disabling of HARQ feedback for IoT NTN	9

8.2	Improved GNSS operations	10

9	3GPP TSG RAN WG1 Meeting #113	10

9.1	Disabling of HARQ feedback for IoT NTN	10

9.2	Improved GNSS operations for IoT NTN	12

10	Reference	14

# Introduction

This document summaries RAN1 agreements made for the Rel-18 WI on NR NTN enhancements.

The Agreements, Working Assumptions and Conclusions achieved in RAN1 meetings up to RAN1#113 are listed in sections 3 to 9.

The WI objective from the WID [1]  is inserted below for convenience.

# WID Objective

The work item aims at specifying enhancements for NG-RAN based NTN (Non-Terrestrial Networks) according to the following assumptions with implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios: - GSO (Geo Synchronous Orbit) and NGSO (Non Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - Both “VSAT” (Very Small Aperture Terminal) devices with directive antenna (including fixed and moving platform mounted devices) and commercial handset terminals (e.g. Power class 3) are supported in FR1 - Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands. Note 1: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). Note 2: The Rel-17 NTN architecture is assumed. This WI is adding functionality in REL-18 to enhance features that were introduced in REL-15/16/17: Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1] NR-NTN deployment in above 10 GHz bands The following assumptions are taken a baseline for this work: - GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered - ESIM scenarios for NGSO in Ka band are not considered in this WI. - Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study. - FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2 - The ITU-R harmonized Ka band will serve as reference - Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification. The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022. - Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4] - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4] - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4] - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4] - For all the above, RAN4 process as agreed for NTN in FR1 (see 3GPP TR 38.863) should be used for coexistence analysis in above 10 GHz bands [RAN4]. - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4] - Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4] - Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed.as follows [RAN4]: - time relationship related enhancement (e.g. K_offset) - subcarrier spacing for different UL/DL signals/channels - PRACH configuration index for FDD above 10 GHz. Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

# 3GPP TSG RAN WG1 Meeting #109-e

## Disabling of HARQ feedback for IoT NTN

Agreement

For IoT NTN, to configure/indicate enabling/disabling on HARQ feedback for downlink transmission, one or more of the following options can be considered:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 2: per HARQ process via SIB signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Option 4: implicitly determined by existing configured/indicated parameter(s) (e.g., repetition number, TBS)

- Option 5: per HARQ process via MAC CE

- Other options or combinations are not excluded

Note: Option(s) for eMTC and NBIoT can be separately discussed.

Agreement

For IoT NTN, further study the potential issues due to enabling/disabling on HARQ feedback for downlink transmission

- Issue A: SPS PDSCH

- Issue B: (N)PDSCH/(N)PDCCH scheduling restriction

- Issue C: HARQ feedback for scheduling multiple TB

- Issue D: HARQ bundling for eMTC HD-FDD

- Issue F: NPRACH capacity

- Issue G: Serving cell/satellite change during data transfer (FFS: for eMTC and/or NB-IoT)

- Other issues are not excluded

Note: The “Issues” in common for eMTC and NB-IoT can be separately discussed.

## Improved GNSS operations for IoT NTN

Conclusion

IoT NTN UE may need to re-acquire a valid GNSS position fix in long connection time.

- FFS: Whether and how to update or reduce the need to update GNSS position fix in long connection time.

Agreement

Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time.

Agreement

At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations:

- Option 1: UE re-acquires GNSS position fix during RLF procedure

- Option 2: UE re-acquires GNSS position fix with a new gap

Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options.

Agreement

UE reports additional GNSS assistance information and further study the detailed GNSS assistance information, including e.g. GNSS position fix measurement time

- Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information.

Agreement

Further study on whether there is a need for potential enhancements on the following for long connection time

- UE triggered GNSS measurement.

- Network triggered GNSS measurement.

# 3GPP TSG RAN WG1 Meeting #110

## Disabling of HARQ feedback for IoT NTN

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling.

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field).

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above.

Agreement

For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above

Agreement

- For a DL HARQ process with disabled HARQ feedback in NB-IoT, at least the following UE behavior(s) can be considered:

- Option 1: UE is not expected to receive another NPDCCH carrying a DCI scheduling a NPDSCH for a given HARQ process that starts until X(ms) after the end of the reception of the last NPDSCH for that HARQ process.

  - X =12

- Option 2: UE is not required to monitor NPDCCH in a period of Y(ms) from the end of reception of the last NPDSCH

  - Y=12

- Note: it may be different UE behaviors for different UE categories (e.g., UE with single/multiple HARQ processes)

## Improved GNSS operations for IoT NTN

Agreement

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement

- GNSS validity duration

Agreement

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix

- FFS details of signalling

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

- Note: further discuss whether a UE is expected to handle all eNB triggers

# 3GPP TSG RAN WG1 Meeting #110-bis

## Disabling of HARQ feedback for IoT NTN

Agreement

For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of Y=12(ms) from the end of reception of the NPDSCH.

Agreement

For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select ONE from the following options at RAN1#111:

- Option 6a-1: Support RRC signaling configured between Option 1 and Option 3

- Option 6a-4: Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission

## Improved GNSS operations for IoT NTN

Agreement

Support eNB to at least aperiodically trigger UE to make GNSS measurement.

Agreement

If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

Agreement

UE reports GNSS position fix time duration for measurement at least during the initial access stage

which message carries this information is up to RAN2

Agreement

In connected mode, UE may report GNSS validation duration with MAC CE.

# 3GPP TSG RAN WG1 Meeting #111

## Disabling of HARQ feedback for IoT NTN

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission

Additional RRC signaling to enable Option 3

If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured.

FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes.

FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

## Improved GNSS operations for IoT NTN

Agreement

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

FFS details of gap configuration

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

FFS based on configured timing

# 3GPP TSG RAN WG1 Meeting #112

## Disabling of HARQ feedback for IoT NTN

Conclusion

For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling.

Agreement

For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process.

Agreement

For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation

- for the first SPS PDSCH after activation,

  - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise.

Conclusion

For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration.

Agreement

For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Agreement

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

## Improved GNSS operations for IoT NTN

Agreement

The following alternatives can be considered to inform eNB the success of GNSS measurement at UE side after GNSS measurement in RRC connected.

- Alt-1: The UE will report the new GNSS validity duration

- Alt-2: The reception of any UL transmission from the UE at eNB after the GNSS measurement

Agreement

On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration.

FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration.

Agreement

On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, RAN1 can down select one of the following alternatives:

- Alt 1: the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot

  - FFS: details of X, e.g. predefined value or configured value

- Alt 2: the start time should be based on the current GNSS validity duration with delay or without delay

Agreement

UE reports only one GNSS position fix time duration for GNSS measurement at least when moving to RRC connected state.

Agreement

At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

- FFS: with legacy closed loop time correction or enhanced closed loop time correction

- This mechanism is enabled/configured by eNB

- FFS: whether such mechanism will be specified depends on the outcome of this study

# 3GPP TSG RAN WG1 Meeting #112bis-e

## Disabling of HARQ feedback for IoT NTN

Agreement

For Option 3 DCI indication:

- Option A: when both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration

    - For single TB scheduled by DCI, the DCI based overridden indication is applied to one of the following options (to be down-selected):

      - Option A-1: only applied to semi-statically HARQ disabled processes

      - Option A-4: applied to both semi-statically HARQ disabled and enabled processes

    - FFS for multiple TBs scheduled by single DCI

- Option B: DCI-based HARQ enabling/disabling direct indication in case DCI solution enabling/disabling signaling is configured and per-HARQ process bitmap signaling is not configured (i.e. no bitmap is configured)

  - DCI-based mechanism is DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission

    - For single TB scheduled by DCI, DCI-based direct indication is applied to the scheduled TB

    - FFS for multiple TBs scheduled by single DCI

Agreement

For single TB scheduled by DCI, for DCI-based direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI (e.g., 1-bit)

  - Note: Other fields in DCI are the same as legacy.

- Option 2: Indication by reusing/reinterpreting existing field in DCI

  - Option 2A: HARQ-ACK related field

    - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

      - FFS: detailed state

    - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

      - FFS: detailed state

  - Option 2B: MCS or repetition number field

    - Reduce 1bit of legacy MCS or repetition number field and add 1bit new field in DCI format 6-1B and N1 to indicate the HARQ feedback enabled/disabled

      - FFS: detailed for interpreting of the reduced MCS or repetition number field

  - Option 2C: HARQ-ACK related field v2

    - For eMTC CE mode B, reduce 1bit of legacy “HARQ-ACK resource offset” field and add 1bit new field in DCI format 6-1B to indicate the HARQ feedback enabled/disabled

      - FFS: detailed for interpreting of the reduced “HARQ-ACK resource offset” field

    - For NBIoT, reduce 1bit of legacy “HARQ-ACK resource” field and add 1bit new field in DCI format N1 to indicate the HARQ feedback enabled/disabled

      - FFS: detailed for interpreting of the reduced “HARQ-ACK resource” field

  - Option 2D: Other indication by reusing/reinterpreting existing field

## Improved GNSS operations

Agreement

For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB.

- The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB.

Conclusion

From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX.

- Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted

- Note: details are up to RAN2

Send an LS to RAN2 for the conclusion.

Agreement

On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot

- FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc

Agreement

Draft LS in R1-2304125 is endorsed. Final LS is approved in R1-2304126.

Agreement

UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including [1,2] seconds as component values.

- FFS: value of N, other component value(s) of GNSS position fix time duration (e.g. N=3, with value in [3,7,13,19,25, X] seconds, and X is FFS).

FFS: whether RAN4 input is needed.

# 3GPP TSG RAN WG1 Meeting #113

## Disabling of HARQ feedback for IoT NTN

Working assumption

For DCI-based direct indication in single TB scheduled by DCI,

- Indication by reusing/reinterpreting HARQ-ACK related field in DCI

  - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

    - FFS: detailed state, and whether this state is different across different UEs

  - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource.

    - FFS: detailed state, and whether this state is different across different UEs

- If reusing/reinterpreting HARQ-ACK related field in DCI is also used for DCI overriding scheme, the interpretation of the state can be different than for DCI-based direct indication.

Agreement

For single TB scheduled by DCI,

- Working assumption 1 DCI based overridden indication is applied to both semi-statically HARQ disabled and enabled processes

  - For DCI based overridden indication, adopt indication by reusing/reinterpreting HARQ-ACK related field in DCI

    - For eMTC CE mode B, “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled.

      - HARQ feedback disabled is reversed to enabled in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is maintained as disabled.

      - HARQ feedback enabled is maintained in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is reversed to disabled.

        - FFS: detailed state A, and whether this state A is different across different UEs

    - For NBIoT, “HARQ-ACK resource” field in DCI format N1 is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled.

      - The same DCI indication functionality as eMTC is adopted.

- Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration).

- Send an LS to RAN2 with the following contents:

  - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact).

Agreement

The draft LS in R1-2306205 is endorsed. Final LS in R1-2306245.

Agreement

For the RRC configuration of DCI solution enabling/disabling of HARQ feedback for NB-IoT and LTE-MTC in CE Mode B, the RRC configuration is UE-specific.

Agreement

for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs.

Agreement

Adopt the response below to RAN2’s question 3:

Understanding 2 is the correct understanding.

Agreement

For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms.

Note: this implies a RAN1 specification change in Rel-18

Agreement

For the response to RAN2’s question 1a: provide the agreement above.

Agreement

Adopt the response below to RAN2’s question 1b:

For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation.

Agreement

Adopt the response below to RAN2’s question 2:

Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

Agreement

For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms.

Note: this implies a RAN1 specification change in Rel-18

Agreement

For the response to RAN2’s question 1a: provide the agreement above.

Agreement

The draft LS in R1-2306134 is endorsed with the following changes:

Add the agreement on NB-IoT UE operating with one HARQ process to the list of RAN1 agreements listed in the LS

Change “kindly” to “respectfully” in the action

Final LS in R1-2306182.

## Improved GNSS operations for IoT NTN

Agreement

From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition.

RAN1 will decide further details of the above.

Agreement

UE reports one GNSS position fix time duration for GNSS measurement via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31]

- FFS: other component values

Agreement

The UE is not required to transmit or receive any channel / signal within the aperiodic GNSS measurement gap duration before the UE reacquires GNSS successfully.

FFS: UE’s behavior within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap.

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, down select one of the alternatives for the start time of the gap:

- Alt 1: should be at n+ X, where n is the end of MAC CE receiving subframe/slot and X>= 12ms for NB-IoT, X>= 3ms for eMTC

  - Note: X is one value regardless of HARQ feedback enabled or disabled for the MAC CE

  - FFS: details, e.g. X is predefined value or configured value

- Alt 2: should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC, or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled

  - FFS: details, e.g. X1 and X2 are predefined value or configured value, including whether X1 and X2 can be the same

- Alt3: should be at p+ X, where p is the end of HARQ feedback transmission subframe/slot, where HARQ feedback for the MAC CE is always enabled

  - FFS: details, e.g. X is predefined value or configured value

Agreement

For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration.

- FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

- Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network.

- Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured

- Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact

# Reference

| - RP-223519, “Revised WID on IoT NN enhancements”, RAN#98-e - R1-2205555, Feature lead summary #3 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#109-e - R1-2205553, Feature lead summary#1 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#109-e - R1-2208011, Feature lead summary #3 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#110-e - R1-2207737, Feature lead summary#2 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#110-e - R1-2210330, Feature lead summary #2 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#110bis-e - R1-2210564, Feature lead summary#3 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#110bis-e - R1-2212555, Feature lead summary #2 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#111 - R1-2212920, Feature lead summary#3 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#111 - R1-2301804, Feature lead summary #2 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#112 - R1-2302120, Feature lead summary#3 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#112 - R1-2303999, Feature lead summary #2 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#112bis-e - R1-2304073, Feature lead summary#3 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#112bis-e - R1-2306021, Feature lead summary #2 on disabling of HARQ feedback for IoT NTN, Moderator (Lenovo), RAN1#113 - R1-2306202, Feature lead summary#3 of AI 9.12.3 on improved GNSS operations Moderator (MediaTek), RAN1#113 |  |
| --- | --- |