3GPP TSG RAN WG1 #114										 	     R1-2306947

Toulouse, France, 21 - 25 August, 2023

Agenda item:	9.10.1

Source:	NEC

Title:              	Discussion on L1 enhancements for inter-cell beam management

Document for:	Discussion and Decision

# Introduction

In RAN1#113 [1], the following agreement was reached for L1 enhancements for inter-cell beam management:

Conclusion

For R18 LTM, in order to activate multiple joint TCI state or/and pair of (DL/UL) TCI states for candidate cell case, do not support TCI state activation together with beam indication of the candidate cell in the same MAC-CE message.

- FFS: UE assumption on the active TCI states other than the indicated TCI state after the reception of the cell switch command.

Agreement

- A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command.

Agreement

- For the configuration of SSB based L1-RSRP measurement,

- periodicity of SSB, SSB position in burst are provided as time domain information for intra- and inter- frequency

Conclusion

For R18 LTM, in order to activate multiple joint TCI state or/and pair of (DL/UL) TCI states for candidate cell case, do not support TCI state activation together with beam indication of the candidate cell in the same MAC-CE message.

- FFS: UE assumption on the active TCI states other than the indicated TCI state after the reception of the cell switch command.

Agreement

A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command.

Agreement

For the configuration of SSB based L1-RSRP measurement,

- periodicity of SSB, SSB position in burst are provided as time domain information for intra- and inter- frequency

Agreement

For the beam application time for Rel-18 LTM,

- Beam application time is supported, and starts after the last symbol of the PUCCH or PUSCH carrying the HARQ-ACK for the PDSCH which carries MAC-CE containing cell switch command with the beam indication for the target cell(s)

  - FFS: reference SCS, i.e. serving cell and/or target cell

- At least the following components are further studied to define the beam application time

  - Whether TCI state activation is received before/together with cell switch command

  - Legacy values, i.e. 3Nslotsubframe,µ and BeamAppTime-r17

  - RF retuning time when inter-frequency switch is performed, which is up to RAN4

  - Whether the target cell is one of the current serving cells

Cell switching time, which is defined by RAN2 and RAN4, may or may not include the potential components of beam application time above.

Agreement

For the beam selection for SSB based L1-RSRP measurement report,

- the inclusion of current SpCell in the L1 measurement report is configurable.

  - New UE capability(ies) are introduced and details can be discussed in UE feature.

Agreement

Send an LS to RAN2,3,4 on the RAN1 agreements in this meeting

- All agreements in AI 9.10.1 and 9.10.2 in RAN1#113 are included

- The following information to RAN2 is included:

  - Whether C-RNTI that is to be used by target cell needs to be included within the MAC-CE containing cell switch command will be left to RAN2 decision.

  - It will be left to RAN2 decision whether the following fields are always present or not in the cell switch command:

    - TA related information

In this contribution, we provided our views on L1 enhancements for L1 measurement, L1 measurement reporting, beam indication and cell switch command.

# Discussion on L1 measurement

The short periodicity of L1 measurements in L1/L2 triggered inter-cell handover may increase the signaling overhead and energy consumption of the UE, due to the amount of measurements and number of reporting performed for the candidate cells. Although these L1 measurements of candidate beams/cells may be activated/deactivated e.g. via MAC CE, the available bits in MAC CE is limited and signalling overhead would be higher. Also, gNB may not be able to deactivate all these L1 measurements without delay. In particular, if a handover cannot happen immediately, e.g. when the UE stops moving or target cell radio link condition does not improve further, large number of active L1 measurements of multiple candidate cells may be running over a prolonged period. Therefore, we propose to specify a default or configurable time duration for active L1 measurements of candidate cells, so that the number of active measurements can be reduced upon timer expiry without signalling overhead. The timer may be started upon the activation of the neighbouring candidate cell’s L1 mobility measurements. If no handover is triggered within a specified time, the UE stops active L1/L2 measurements and reporting for the corresponding candidate cell.

Proposal 1: Specify a default or configurable time duration for active L1 measurements of candidate cells for L1/L2 based inter-cell mobility.

In addition, gNB may be able to start or stop these L1 measurements for inter-cell mobility based on L3 RSRP measurement results. gNB may configure beam measurement thresholds for the start and stop of L1 beam measurements for inter-cell mobility.

gNB may reactivate the L1 measurements for inter-cell mobility dynamically based on e.g. L3 RSRP measurement results. gNB may define measurement thresholds for activating L1 measurements for inter-cell mobility. The beam measurement threshold can be an absolute value of the serving beam, or a delta value for the difference between the beam of the target cell and the beam of the serving cell, below/above which, the measurements for a particular beam/cell can be stopped. L3 measurement threshold may then be used to restart the corresponding L1 beam measurements.

Proposal 2: Specify measurement thresholds for activating/deactivating L1 measurements for inter-cell mobility.

# Discussion on L1 measurement reporting

The L1 measurement reports can be carried via MAC-CE or UCI. Since L3 measurement values are filtered and averaged over a configurable time period, L3 measurement values can be a good indication for whether L1 measurement reporting is required or not for the purpose of LTM, it is beneficial to specify L3 measurement values as the start/stop thresholds of periodic/semi-persistent L1 measurement reporting.

Proposal 3: Specify L3 measurement values for the start/stop thresholds of periodic/semi-persistent L1 measurement reporting.

For the report of SSB based L1-RSRP, it is agreed that L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the remaining RSRP values are indicated with a differential L1-RSRP quantized to a 4-bit value where the differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value. The following option is to be down-selected in RAN1#114:

- Option A: Differential encoding is performed per cell, i.e. for each cell, a beam to apply absolute L1-RSRP is chosen among M beams and the differential L1-RSRP is applied to M-1 beams

- Option B: a beam to apply absolute L1-RSRP is chosen among M *L beams and the differential L1-RSRP is applied to M*L-1 beams

- Option C:  Take the following 2-step approach

  - Step one: Differential encoding is performed per cell, i.e. for each cell, a beam to apply absolute L1-RSRP is chosen among M beams and the differential L1-RSRP is applied to M-1 beams.

  - Step two: a beam to apply absolute L1-RSRP is chosen among L beams applying absolute L1-RSRP in step one and the differential L1-RSRP is applied to L-1 beams.

Give that the measured L1-RSRP values are likely to have a bigger difference among different candidate cells, especially for inter-frequency case, the differential values are likely to be high and accuracy can be lost for a lot of measurement values due to out of range, therefore we propose not to support Option B or C. Differential encoding should be performed per cell.

Proposal 4: Support that differential encoding can be performed per cell for L1 measurement report.

# Discussion on beam indication

# Beam indication mechanism

For beam indication of target cell based on Rel-17 unified TCI framework applied to CORESET#0 and CORESETs (other than CORESET#0) associated with Type 0A/1/2-PDCCH CSS sets where no TCI state activation is provided, follow the indicated TCI state until a new TCI state is configured or activated by the target cell. Note that followUnifiedTCI-state is not provided because UE does not process the RRC configuration of target cell before cell switch. Then, how to determine the corresponding beam needs to be specified in case UE needs to receive the channels after cell switch command.

If the candidate cell supports only Rel-15 TCI framework, the indicated TCI state in cell switch command can be applicable for the SSB beam by default. Further beam indications for CSI-RS, CORESET, PDSCH, PUCCH, PUSCH may be needed after the cell switch command. To reduce latency, it may be beneficial to include TCI states for each channel, such as CSI-RS, CORESET, PDSCH, PUCCH, PUSCH like in Rel-15, also in the cell switch command.

Proposal 5: Rel-15 TCI framework based TCI state indication for each channel may be included in the cell switch command.

# Beam application time

For R18 LTM, a new beam application time should be defined. For the beam application time for R17 ICBM, only the time of beam switch was considered. However, for the beam application time for R18 LTM, in addition to the time of beam switch, the time of cell switch needs to be considered. It means that, the beam application time for R18 LTM should be longer. For example, for Scenario 2, it has been agreed that beam indication for target cell is included in a cell switch command carried by a MAC CE. In this case, the indicated beam for target cell can be applied after NW receives HARQ-ACK corresponding to the MAC CE. Further, considering that the TCI state included in the cell switch command may be a joint, DL or UL unified TCI state, different beam application time may need to be defined.

Proposal 6: Support to define new beam application time for R18 LTM, at least the following components are further studied to define the new beam application time.

- Legacy values, i.e., 3Nslotsubframe,µ and BeamAppTime-r17

Proposal 7: Support to define different beam application time for joint, DL or UL TCI state of target cell indicated by cell switch command carried by MAC CE.

# Conclusion

In this contribution, we provided our views on L1 enhancements for L1 measurement, L1 measurement reporting, beam indication and cell switch command. Specifically, we have the following proposals:

Proposal 1: Specify a default or configurable time duration for active L1 measurements of candidate cells for inter-cell mobility monitoring.

Proposal 2: Specify measurement thresholds for activating/deactivating L1 measurements for inter-cell mobility.

Proposal 3: Specify L3 measurement values for the start/stop thresholds of periodic/semi-persistent L1 measurement reporting.

Proposal 4: Support that differential encoding can be performed per cell for L1 measurement report.

Proposal 5: Rel-15 TCI framework based TCI state indication for each channel may be included in the cell switch command.

Proposal 6: Support to define new beam application time for R18 LTM, at least the following components are further studied to define the new beam application time.

- Legacy values, i.e., 3Nslotsubframe,µ and BeamAppTime-r17

Proposal 7: Support to define different beam application time for joint, DL or UL TCI state of target cell indicated by cell switch command carried by MAC CE.

# References

- Chair's notes RAN1#113