3GPP TSG RAN WG1 #114		                                                       R1-2307550

Toulouse, France, August 21st - August 25th, 2023

Agenda item:	9.16.11

Source: 	OPPO

Title: 	Discussion on UE features for IoT NTN enhancements

Document for:	Discussion and decision

- Introduction

This contribution provides our views on RAN1 UE feature list for the Rel-18 WI of IoT NTN enhancements.

Firstly, UL HARQ mode B is introduced in Rel-18 IoT NTN. One UL HARQ process may be configured with UL HARQ mode A or mode B for uplink transmission. If UL HARQ mode B is configured for an IoT NTN UE, its PDCCH monitoring behaviors should be enhanced, e.g., the minimum time between the end of PUSCH transmission and the start of PDCCH monitoring may need to be defined. This feature should be introduced in UE feature list.

Proposal 1: Introduce UE feature list for PDCCH monitoring restrictions for UL HARQ mode B.

Secondly, when disabling HARQ feedback for downlink transmission is introduced, the corresponding UE PDCCH monitoring behaviors should also be enhanced. And this PDCCH monitoring restrictions should be reflected in UE feature list.

Proposal 2: Reflect PDCCH monitoring restrictions for DL disabling HARQ feedback in UE feature list.

Thirdly, regarding GNSS measurement autonomously triggered by UE itself, we think it can also be configured independent of GNSS measurement triggered by eNB, so FG2-4a and FG2-4b should also refer to FG Rel.17 2-1 and FG Rel.17 2-1b respectively.

Proposal 3: FG2-4a and FG2-4b should also refer to FG Rel.17 2-1 and FG Rel.17 2-1b respectively.

Our views are reflected in the following proposed UE feature list table in blue color based on the endorsed UE feature list table for IoT NTN enhancements in [2].

- Reference

[1] R1-2305447, Discussion on UE features for IoT NTN enhancements, OPPO.

[2] R1-2306164, Session Notes of AI 9.16.11, Ad-Hoc Chair (AT&T).

- Proposed UE feature list for IoT NTN enhancements

| 2. IoT_NTN_enh | 2-1a | Semi-static HARQ feedback disabling for eMTC CE mode B | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process 2. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback 2. UE supports the corresponding PDCCH monitoring restriction when HARQ feedback disabling is configured | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Candidate values: {dynamic, semi-static, dynamic + semi-static} Note: HARQ disabling with Option 1 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1b | Dynamic HARQ feedback disabling for eMTC CE mode B | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback 2. UE supports the corresponding PDCCH monitoring restriction when HARQ feedback disabling is indicated | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to single-TB case FFS: whether to merge 2-1a and 2-1b FFS: whether to have a separate FG for the combination of FGs 2-1a and 2-1b [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d | Semi-static HARQ feedback disabling for eMTC CE mode A | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process 2. UE supports the corresponding PDCCH monitoring restriction when HARQ feedback disabling is configured | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-2 | Semi-static HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | UE reports ACK/NACK for the first SPS PDSCH after activation if enabled, and follow per-process HARQ feedback enabled/disabled configuration otherwise | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE Mode A cannot disable HARQ feedback for SPS PDSCH | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A FFS: whether to merge 2-1d and 2-2 [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

| 2. IoT_NTN_enh | 2-1e | Semi-static HARQ feedback disabling for NB-IoT | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process 2. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback 2. UE supports the corresponding PDCCH monitoring restriction when HARQ feedback disabling is configured | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Candidate values: {dynamic, semi-static, dynamic + semi-static} Note: HARQ disabling with Option 1 Note: this applies to single-TB case FFS: whether to merge 2-1f and 2-1e FFS: whether to have a separate FG for the combination of FGs 2-1e and 2-1f [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1f | Dynamic HARQ feedback disabling for NB-IoT | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback 2. UE supports the corresponding PDCCH monitoring restriction when HARQ feedback disabling is indicated | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

| 2. IoT_NTN_enh | 2-3a | GNSS position fix in RRC Connected state for eMTC—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] Note: This applies to non-DRX | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-4a | GNSS position fix in RRC Connected state for eMTC—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | Rel. 17 2-1, [Rel. 18 2-3a] | Yes | N/A | Release 18 eMTC UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX FFS: merge 2-4a with 2-3a [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

| 2. IoT_NTN_enh | 2-3b | GNSS position fix in RRC Connected state for NB-IoT—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot get triggered GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-4b | GNSS position fix in RRC Connected state for NB-IoT—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | Rel. 17 2-1b, [Rel. 18 2-3b] |  |  | Release 18 NB-IoT UE cannot get autonomous GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] FFS: merge 2-3b with 2-4b | Optional with capability signalling |

| 2. IoT_NTN_enh | 2-5a | Semi-static UL HARQ mode B for eMTC | 1. UE gets RRC configuration for UL HARQ mode B per UE per process 2. UE supports the corresponding PDCCH monitoring restriction when UL HARQ mode B is configured for Type B half-duplex FDD operation | Rel. 17 2-3 | Yes | N/A | Release 18 eMTC UE cannot support UL HARQ mode B | [Per UE/Per band] | No | No | For UEs supporting UL HARQ mode B, it must indicate this FG is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 2. IoT_NTN_enh | 2-5b | Semi-static UL HARQ mode B for NB-IoT | 1. UE gets RRC configuration for UL HARQ mode B per UE per process 2. UE supports the corresponding PDCCH monitoring restriction when UL HARQ mode B is configured | Rel. 17 2-3a | Yes | N/A | Release 18 NB-IoT UE cannot support UL HARQ mode B | [Per UE/Per band] | No | No | For UEs supporting UL HARQ mode B, it must indicate this FG is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |