3GPP TSG RAN WG1 #114			R1-2308366

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.1.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary#2 on Rel-18 CSI enhancements: Round 1

Document for:	Discussion and Decision

## Introduction

The scope given in the Rel-18 NR Evolved MIMO WID pertaining to CSI enhancement is as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

## Summary of companies’ proposals and views

Proposals for online session

Conclusion: For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting the following: - additional dropping rules beyond the current specification for CSI reporting - regarding amplitude quantization, refining the previous agreement by replacing the agreed amplitude quantization entry index “15” (“1111” in binary) with an “invalid” value - regarding channel and interference measurement restriction, supporting TD measurement restriction

Conclusion: For the Rel-18 Type-II codebook refinement for CJT mTRP, there is no consensus on supporting different CSI reporting and dropping behaviour when dynamic TRP selection is configured

Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS, w=14.(KP–1).d, where d denotes the CSI-RS periodicity and KP is {1,2,4} - UE doesn’t expect that Z is larger than 14x32 symbols, i.e. Z=min(legacy_Z+w,14x32) for Capability 1, and Z=min(legacy_Z+w+r,14x32) for Capability 2 FL Note: Failure to reach consensus on this w issue implies the absence of Z2/Z2’ specification for P/SP-CSI-RS in Rel-18 (hence the support for P/SP-CSI-RS for Type-II Doppler/UE-side prediction is incomplete), i.e. only AP-CSI-RS measurement is complete. Support/fine: ZTE, LG, MediaTek, Nokia/NSB, Qualcomm, Samsung (only when w=min(FLP,14x32)), vivo, Huawei/HiSi, OPPO, Xiaomi, Fujitsu Concern:

Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the candidate values of Y are {1,2,3} Support/fine: Ericsson, ZTE, Samsung, Google, OPPO, LG (but add 2/3), Xiaomi Concern:

Proposal: For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, r=legacy Z2’.NTRP/4 FL Note: Alt1 is proposed as it is the common part between Alt1 and ALt2. Note that failure to reach consensus on this issue by Wed/Thu of RAN1#114 implies Capability 2 for Z/Z’ isn’t supported in Rel-18. Support/fine: Huawei/HiSi, vivo, Lenovo/MotM, NEC, Google, LG, CATT, Fujitsu, OPPO, Qualcomm Concern: ZTE (SC dependence), Samsung (14)

Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r= legacy Z2’.N4/8 FL Note: Failure to reach consensus on this r issue by Wed/Thu of RAN1#114 implies that Capability 2 for Z/Z’ is not supported in Rel-18 Support/fine: Huawei/HiSi, ZTE, Sony, LG, CATT, Samsung, Google, Fujitsu, NEC, OPPO, Qualcomm, Xiaomi Concern:

### Issue 1: Type-II codebook refinement for CJT

Table 1 Summary: issue 1

| # | Issue/proposal | Companies’ views |
| --- | --- | --- |
| 1.2 | [113] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’: - For NTRP=1: reuse legacy Z/Z’ values - For NTRP>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Legacy Z/Z’ values + r - The value(s) of r>0 can depend on the configured NTRP value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, decide, in RAN1#114, based on the following alternatives: - Alt1. r is dependent only on the value of NTRP (exact dependence TBD) and reported - Alt2. r is dependent only on the value of NTRP and sub-carrier spacing (exact dependence TBD), reported - Alt3. r is taken from a set of candidate values, e.g. {14, 28, legacy Z2’} Proposal: For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, r=legacy Z2’.NTRP/4 FL Note: Alt1 is proposed as it is the common part between Alt1 and ALt2. Note that failure to reach consensus on this issue by Wed/Thu of RAN1#114 implies Capability 2 for Z/Z’ isn’t supported in Rel-18. Alt1: Huawei/HiSi, vivo, Lenovo, NEC, Google, LG, CATT, Fujitsu, Qualcomm (r=legacy Z2’.N_TRP/4) Alt2: ZTE, Lenovo/MotM, Apple Alt3: MediaTek, Samsung (r=14), Nokia/NSB, Sony, NTT DOCOMO | Support/fine: Huawei/HiSi, vivo, Lenovo/MotM, NEC, Google, LG, CATT, Fujitsu, OPPO, Qualcomm Concern: ZTE (SC dependence), Samsung (14) |
| 1.5 | [114] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the CSI-RS resource set no later than CSI reference resource and drops the report otherwise. - FFS: Whether when dynamic TRP selection is configured has different behaviour Conclusion: For the Rel-18 Type-II codebook refinement for CJT mTRP, there is no consensus on supporting different CSI reporting and dropping behaviour when dynamic TRP selection is configured Q: When dynamic TRP selection is configured, should the associated UE behaviour remain the same? FL Note: No consensus on supporting a different behaviour implies that the same behaviour is assumed. | FFS same behaviour: NTT DOCOMO, Spreadtrum, MediaTek, Ericsson, ZTE, Samsung, Google, OPPO, LG FFS different behaviour: [NEC] |

Table 2 Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 1 |
| NEC | Issue 1.5: As we discussed in Round 0, we would like to clarify the issue, with the following example, in case of a first CSI-RS resource not satisfying the condition (i.e. not receive the first CSI-RS resource no later than CSI reference resource) while a second CSI-RS satisfying the condition (i.e. receive the second CSI-RS resource no later than CSI reference resource), in this case, whether UE can still report valid CSI based on the second CSI-RS resource? We support the CSI can be reported, both gNB and UE are aligned on whether the condition for each CSI-RS resource satisfied or not, so there is no ambiguous, and supporting this, at least valid CSI can be reported which can improve the resource utilization instead of simply dropping any CSI report. So we propose: In case of dynamic TRP selection is configured, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each selected CSI-RS resource in the CSI-RS resource set corresponding to the CSI report no later than CSI reference resource and drops the report otherwise. [Mod: I don’t think this is correct. The TRP selection is performed AFTER CSI-RS reception and measurement. The proposed text presumes the opposite. The current text “each CSI-RS resource” can only imply each configured CSI-RS resource – which is the correct behaviour.] |
| Qualcomm | 1.2: Added our position in the FL note (missed us in Alt3) We failed to see a reason for eliminating SCS as a dependent factor – probably the only reason is to save discussion time (which is understandable, although). We suggest to leverage existing Z2’ to take into account SCS – no need for lengthy discussion on how to depend on SCS. A suggested compromise can be: r=X*NTRP*Z2’, where even a small value of X can be OK e.g. X=1/4 [Mod: We can try] 1.5: Seems a simple rule under TRP-selection case can be: For the Rel-18 Type-II codebook refinement for CJT mTRP, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resources in the CSI-RS resource set no later than CSI reference resource and drops the report otherwise. |
| Lenovo/ MotM | Issue 1.1: Support Issue 1.5: Agree with QC. For dynamic TRP selection, the UE can report a CSI corresponding to the subset of CSI-RS resources from which at least one CSI-RS transmission occasion has been received. Would the following wording help? “When dynamic TRP selection is configured, it suffices that the UE receives at least one CSI-RS transmission occasion for at least one CSI-RS resource in the CSI-RS resource set no later than CSI reference resource.” [Mod: If dynamic TRP selection is ON, for dynamic TRP selection to be useful at all, each configured CSI-RS resource should be received – which is already implied in the current behaviour. But we can discuss the wording better if needed. “At least one” seems to encourage premature reporting. But regardless, it’s not clear if this is an issue especially since we have agreed to reuse legacy constraint that all the NTRP configured resources should be within 2 consecutive slots] |
| AT&T | Issue 1.5: Agree with the Mod in the above comment that Dynamic TRP selection can potentially offer System Performance gain when the UE receives CSI-RS transmission occasion from each CSI-RS resource in the CSI-RS resource set. If the UE tries to make the decision of Dynamic TRP selection before receiving the CSI-RS transmission occasion from all the TRPs in the CSI-RS resource set, this may lead to premature reporting as the UE may miss the TRP associated with the strongest receive signal power. |
| NTT DOCOMO | Issue 1.2: Our first preference is Alt3. Issue 1.5: We donot think a different UE ehaviour is needed. Anyway, UE performs dynamic TRP selection after measuring the valid CSI-RS transmission occasions. |
| Spreadtrum | Issue 1.5: I think this issue is related with how we understand the current spec: ‘After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.’ Since for R15-17 CSI for S-TRP and NCJT, we didn’t specify special behaviour for the case when only a subset of CMR within one CSI-RS transmission occasion is received. We prefer not supporting a different behaviour for this issue as well. |
| MediaTek | Issue 1.5: We are fine with having the same UE behaviour for with and without dynamic TRP selection for CSI dropping rule when not all CSI-RS occasions are received. We believe this is a limited corner case and we should not spend time creating new rules. gNB should be aware that the reliable TRP selection can only be performed by the UE once all CSI-RS resources configured for a CJT report is received by the UE. |
| Ericsson | On Issue 1.5, we don’t see a need for supporting a different behaviour. The same behaviour as to what we agreed in [114] can be applied when dynamic TRP selection is configured. |
| ZTE | Issue 1.2: Technically speaking, r should be based on subcarrier space if reviewing the current spec, considering that the CSI calculation duration is much relevant to absolute time rather than number of symbols. Therefore we still think that Alt2 is a reasonable solution. ‘r=legacy Z2’.NTRP/4’ seems to be alternative of considering subcarrier space issues due to that Z2’ is SCS-specific, but we have a little bit worry about whether the added part is too long, and performance degradation may occur. Issue 1.5: We still prefer to have the same UE behaviour for with/without TRP selection. |
| Samsung | Issue 1.2 One distinct point between Rel-16 Type-II and Rel-18 Type-II is the CSI-RS resource(s) measurement duration when NTRP>1 is configured. The NTRP CSI-RS resources are allowed to be configured within two consecutive slots (i.e., within the same slot or 2 slots), and thus one additional slot can be needed to measure the CSI-RS resources when NTRP>1, compared to Rel-16 Type-II CSI. Therefore, the UE may potentially need one additional slot time for CSI computation time, and we propose r=14. Issue 1.5 Agreed with FL in the above comments. A proper dynamic selection can be performed only after each CSI-RS resource is received/measured (as the agreement stated). |
| Google | Issue 1.5: We do not see the need for different ehaviour. |
| Mod V14 | No revision |
| OPPO | Issue 1.2 We are fine with QC’s suggestion as in the FL proposal. Issue 1.5 We prefer the same behaviour with or without dynamic TRP selection. CSI report based on only part of CSI-RS resources cannot guarantee the performance. |
| LG | Issue 1.2 Our first preference is that r is dependent only on the value of NTRP but we are fine with QC’s suggestion as well. Issue 1.5 We have similar view with OPPO. |
| Mod V20 | Revised 1.2 with Qualcomm’s compromise suggestion Conclusion for 1.5 |
| Samsung | Issue 1.2 We are still not sure why the QC’s compromise proposal is needed. We still prefer r=14. |
| Mod V24 | No revision |

### Issue 2: Type-II codebook refinement for high/medium UE velocities (with time/Doppler-domain compression)

Table 3 Summary: issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation: OCPU = Y.N4 when P/SP-CSI-RS is configured for CMR, or OCPU = Y.K when AP-CSI-RS is configured for CMR - Y≥1 is defined based on UE capabilities and determined by the UE, and can be different between P/SP-CSI-RS and AP-CSI-RS. - FFS: Whether the supported value(s) of Y can depend on codebook parameter values - The legacy specification on CPU pools is fully reused - When N4=1, OCPU =4 - OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the value of Y is reported by the UE (as a part of UE capability reporting) and not dependent on any codebook parameter value - FFS (by RAN1#114): The candidate value(s) of Y Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the candidate values of Y are {1,2,3} Q: Please share your view {1,2,3,4,5}: {1,2,3}: Ericsson, ZTE, Samsung, Google, {2,3}: Add 2/3: LG | Support/fine: Ericsson, ZTE, Samsung, Google, OPPO, LG (but add 2/3), Xiaomi Concern: |
| 2.2 | [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, decide, in RAN1#114, based on the following alternatives: - Alt1: the value of r is dependent only on N4 (exact dependence TBD) - Alt2: the value of r is dependent only on N4 and sub-carrier spacing (exact dependence TBD) - Alt3: r is taken from a set of candidate values, e.g. {14, 28, legacy Z2’} Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r= legacy Z2’.N4/8 FL Note: Failure to reach consensus on this r issue by Wed/Thu of RAN1#114 implies that Capability 2 for Z/Z’ (issue 2.2 and 2.3) is not supported in Rel-18 Alt1: Huawei/HiSi, Sony, LG, CATT, Samsung, Google, Fujitsu, NEC, Qualcomm (also ok with r=legacy Z2’.N4/8) Alt2: Apple, ZTE Alt3: MediaTek, Nokia/NSB, | Support/fine: Huawei/HiSi, ZTE, Sony, LG, CATT, Samsung, Google, Fujitsu, NEC, OPPO, Qualcomm, Xiaomi Concern: |
| 2.3 | [113] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’ [114] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives: - Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity - Alt2: w=14 or 28 (fixed) Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS, w=14.(KP–1).d, where d denotes the CSI-RS periodicity and KP is {1,2,4} - UE doesn’t expect that Z is larger than 14x32 symbols, i.e. Z=min(legacy_Z+w,14x32) for Capability 1, and Z=min(legacy_Z+w+r,14x32) for Capability 2 FL Note: The proposal is based on the majority view and analogous to AP-CSI-RS FL Note: Failure to reach consensus on this w issue implies the absence of Z2/Z2’ specification for P/SP-CSI-RS in Rel-18 (hence the support for P/SP-CSI-RS for Type-II Doppler/UE-side prediction is incomplete), i.e. only AP-CSI-RS measurement is complete. Alt1: ZTE, LG, MediaTek, Nokia/NSB, Qualcomm Alt2: Samsung (also ok with min(FL proposal, 14x32)), Google | Support/fine: ZTE, LG, MediaTek, Nokia/NSB, Qualcomm, Samsung (only when w=min(FLP,14x32)), vivo, Huawei/HiSi, OPPO, Xiaomi, Fujitsu Concern: |
| 2.5 | Proposal: For the Type-II codebook refinement for high/medium velocities, in case of A/SP-CSI reporting repetition towards M-TRP, the start of CSI reporting window is slot l=n+δ, where n is the last uplink slot carrying the CSI report Q: Any company against this proposal having a change of mind? - Support/fine: vivo, Samsung (ok), Qualcomm, Apple, Google, OPPO, CMCC, IDCC, CATT, LG, Ericsson, NTT DOCOMO, Spreadtrum (ok if RAN1 identifies use case), Nokia/NSB (ok), Xiaomi (ok), Fujitsu (ok), - Not support: ZTE (motivation is unclear), Lenovo/MotM, Huawei/HiSi, |  |

Table 4 Additional inputs: issue 2

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 3 |
| NTT DOCOMO | For 2.5, we are fine with the Proposal. |
| Qualcomm | 2.2: Added our position in the FL note (missed us in Alt3) Similarly as CJT 1.2, we failed to see a reason for eliminating SCS as a dependent factor. We suggest to leverage existing Z2’ to take into account SCS, too. A suggested compromise can be: X*N4*Z2’, where small X can be OK e.g. 1/8 [Mod: We can try] 2.3: We strongly suggest to think more about “Kp-1” This P-CSI-RS case is different than AP, since for AP-CSI-RS, K occasions b/w PDCCH and Z’-symbol-priorTo-PUSCH can be ensured by previous agreements below on report dropping ehaviour (note that X=1 below means all K). Proposal: For the Type-II codebook refinement for high/medium velocities, the UE reports a CSI report only if receiving at least X consecutive CSI-RS transmission occasion for each CSI-RS resource in the CSI-RS resource set no later than CSI reference resource, and/or one CSI-IM occasion for interference measurement, else drops the report otherwise. - X=1 for AP-CSI-RS. - X=KP for P/SP-CSI-RS, where KP denotes the scaling factor of active P/SP-CSI-RS resource counting Note: This includes the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration. However, there is no such “promise” for Kp occasions b/w PDCCH and Z’-symbol-priorTo-PUSCH. Therefore, we suggest to use timeline parameter w to ensure this: w=14. KP.d [Mod: Sadly Qualcomm is the only one proposing this … So I am “bound” not to propose this especially since the Alt2 companies have concern about Alt1 being too large] |
| Spreadtrum | Issue 2.5: Our view is that we prefer to specify feature (combination) that have clear use case. Based on our experience, making the spec too flexible will only bring complexity and cause confusion. We are OK if RAN1 identified a use case. |
| Ericsson | Issue 2.1: For candidate value(s) of Y, {1, 2, 3} may be a middle ground and we support that. |
| ZTE | Issue 2.1: We prefer ‘{1,2,3}’; Issue 2.2: Just clarify our preference: we have the same views as in Issue 1.2. So, we support Alt2 rather than Alt1. |
| Samsung | Issue 2.1 We can be OK with Ericsson’s suggestion. Y={1,2,3}. Issue 2.3 We would like to point out some technical issue on Alt1. Currently, the time where CSI reporting happens can be configured 0 to 32 slots (using RRC parameter reportSlotOffsetList) from the slot where DCI is triggered, i.e., up to 32 slots for CSI reporting can be apart from where the DCI is triggered. Under the current form of Alt1, Kp-1d can be larger than 32 depending on Kp and d, and this case should not happen for UE and/or NW to assume a non-conflict operation on it. Hence, we suggest to add the following restriction on the w in order to ensure not to have Kp-1d>32: w=min14Kp-1d,14×32. |
| Google | Issue 2.1: We slightly prefer Y={1,2,3} Issue 2.3: We do not know why we need such a big value of w. Such big value could potentially require higher UE complexity since the UE has to “predict” CSI far from the measured CMRs. |
| Mod V14 | No revision |
| Mod V15 | Revision per Tue offline outcome |
| OPPO | Issue 2.1: Y={1,2,3} is fine to us. Issue 2.2: Prefer Alt.1 Issue 2.3: Fine with the proposal. |
| LG | Issue 2.1: reporting Y=2/3 should be possible. Otherwise, K=12 cannot be supported given that maximum number of simultaneous CSI is 8 according to current spec. Issue 2.2: Our first preference is that r is dependent only on the value of N4 but we are fine with QC’s suggestion as well. Issue 2.3: we are fine with considering upper bound but it should be applied to Z not w since Z is minimum time gap between DCI and PUSCH. In addition, w is needed for Z for Capability 1 as well according to previous agreement. Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 1/2 associated with P/SP-CSI-RS, w=14.(KP–1).d, where d denotes the CSI-RS periodicity and KP is {1,2,4} - UE doesn’t expect that Z for Capability 1/2 is larger than 14x32 symbols [Mod: You are correct. Revised] |
| Xiaomi | Issue2.1~2.3. We are fine with these proposals. Issue2.5. Although we think it is not needed to consider the CSI reporting repetition towards M-TRP, we can live with it if major companies can agree with proposal. |
| Mod V20 | Revised 2.2 based on Qualcomm’s compromise proposal Revised 2.3 based on LG’s input |
| Samsung | Issue 2.3 We are fine with the current proposal. |
| Fujitsu | Issue 2.2 and 2.3 Fine with the update proposal. Issue2.5 We can be flexible for this proposal. |
| LG | Issue 2.3 “PUSCH scheduling trigger offset” is configured by TDRA field and it is already less than 14*32 symbols according to current specification since largest K2 is 32. Therefore, it should be Z. Also, we don’t need min operation since scheduling restriction is sufficient. We suggest the following revision. Proposal: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS, w=14.(KP–1).d, where d denotes the CSI-RS periodicity and KP is {1,2,4} - UE doesn’t expect that Z is larger than 14x32 symbols. |
| Mod V24 | Minor revision 2.3 per LG |

### Issue 3: TRS-based reporting of time-domain channel properties (TDCP)

Table 5 Summary: issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.3+3.12 | Conclusion: For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting additional dropping rules beyond the current specification for CSI reporting Proposal: For the Rel-18 TRS-based TDCP reporting, for the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration, the UE reports a TDCP only if receiving at least [one pair of] CSI-RS transmission occasions [in the configured CSI-RS resource set(s)] no later than CSI reference resource, else [drops the report][reports an “invalid” hypothesis] otherwise Q: Please share your inputs on the above wording – considering 3.14 as well FL assessment: No consensus even after Tue offline session | Support/fine: NTT DOCOMO, Qualcomm, Samsung (soln similar to Doppler can be adopted), OPPO, ZTE, Apple (discuss), vivo, CMCC, Nokia/NSB, MediaTek, IDCC, Xiaomi, Spreadtrum, Not support: Google (more discuss) , Ericsson (more discussion, bracketed depends on D) |
| 3.14 | Conclusion: For the Rel-18 TRS-based TDCP reporting, regarding amplitude quantization, there is no consensus on refining the previous agreement by replacing the agreed amplitude quantization entry index “15” (“1111” in binary) with an “invalid” value Proposal: For the Rel-18 TRS-based TDCP reporting, regarding amplitude quantization, refine the previous agreement by replacing the agreed amplitude quantization entry index “15” (“1111” in binary) with an [“invalid”] value - [“Invalid”] comprises … FL assessment: From x7909 (Qualcomm). | Support/fine: Qualcomm, OPPO, Apple, Google, CMCC, MediaTek, IDCC, CATT, Ericsson, Spreadtrum, Mavenir, NTT DOCOMO (open), Not support: vivo, Nokia/NSB, NEC (what invalid means), Lenovo/MotM, ZTE (also ok with replacing 0000), Samsung (same as ZTE), MediaTek |
| 3.13 | Conclusion: For the Rel-18 TRS-based TDCP reporting, regarding channel and interference measurement restriction, there is no consensus on supporting TD measurement restriction Proposal: For the Rel-18 TRS-based TDCP reporting, regarding channel and interference measurement restriction, - The current specification supports NW-configurable TD measurement restriction for any CSI reporting (via CSI Reporting Setting), which includes TDCP Q: Please share your inputs on the above wording FL assessment: While time-domain averaging restriction via timeRestrictionForChannelMeasurements is not supported for TRS, it is present in CSI Reporting setting. Since TDCP is one of the possible values for ReportQuantity, this seems to imply that TD measurement restriction is supported for TDCP Support/fine: Ericsson, ZTE, Spreadtrum, Not support: Google (current spec says no) |  |

Table 6 Additional inputs: issue 3

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 5 |
| NTT DOCOMO | #3.3 + #3.12: Support the proposal. #3.14: Open #3.15: Our understanding is that the Note itself implies timeRestrcitionForChannelMeasurement configuration is supported for TDCP. We agree with that direction. |
| NEC | Issue 3.3+3.12: Regarding “the UE reports a TDCP associated with a configured D value”, in case of Y delays (corresponding to Y configured D values), does it mean UE can report only one correlation associated with one configured D value? If no, it should be “the UE reports a TDCP associated with configured D values” Issue 3.14: We are open to discuss the issue, while the thing to be clarified is what will lead to invalid amplitude value, does it mean there is no actual TRS transmission on the two symbols separated by Dn symbols? |
| Qualcomm | 3.3+3.12: Editorial suggestion: Proposal: For the Rel-18 TRS-based TDCP reporting, for the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration, the UE reports a TDCP associated with a configured D value only if receiving at least [more than one] CSI-RS transmission occasion [of every CSI-RS resource in the configured CSI-RS resource set(s)] no later than CSI reference resource, else refrains from reporting drops the report otherwise [Mod: Not sure if “refrain from reporting” is currently used in the spec. But yes we can discuss if we should replace “drop” with, e.g. “invalid”]. The reason for using wording “refrains from reporting” to replace “drop report,” is a joint consideration with 3.14. Since UE may be able to report zero-correlation or invalid codepoint, it may not need to drop these bits (benefit is to prevent error propagation in UCI packing) Other wording on how to describe “at least” condition, maybe better to discuss in tomorrow’s offline. [Mod: Yes ] |
| Lenovo/ MotM | Issue 3.14: The motivation behind replacing a quantization value with an invalid value is unclear. We can either keep the quantization value as is or replace with value zero as SS suggested |
| Spreadtrum | Issue 3.13: We know that the endorsed CR implies that timeRestrictionForChannelMeasurements is supported for TDCP reporting. But without an explicit agreement, isn’t it the right way to change the CR rather than supporting something by default? |
| MediaTek | Issue 3.12: With regards to new wording, We are not sure about the newly added red texts. Why do we need to explicitly mention TDCP associated with configured D value(s)? is there is a TDCP report not associated with a D value(s)? Also we are not what is meant by “at least [more than one]”. If we are using at least then we should put definite value, rather than “more than” a limit. Is the intention to imply more than one? In which case then we should remove the word more. . Issue 3.14: Although we don’t have any objections towards having an invalid value but we also need to define the cases where “invalid” value can be reported. So we suggest adding an FFS on what scenarios can lead to an “invalid” value reporting. |
| Ericsson | On Issue 3.3+3.12: I guess the intention is that the UE receives enough CSI-RS transmission occasions needed to compute TDCP associated with configured D value(s). So instead of saying [more than one], could we go with the suggested change below? Proposal: For the Rel-18 TRS-based TDCP reporting, for the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration, the UE reports a TDCP associated with configured D value(s) only if receiving at least [more than one] sufficient CSI-RS transmission occasions needed to compute TDCP associated with configured D value(s) [of every CSI-RS resource in the configured CSI-RS resource set(s)] no later than CSI reference resource, else [drops the report] otherwise As for the [drops the report] part, we are open to the suggestion from Qualcomm. That is, instead of dropping the whole report, those TDCP values the UE could not compute are can be reported with an invalid value. On Issue 3.14: We support the proposal On Issue 3.13: Ok with the conclusion with the understanding that whether or not to configure ‘timeRestrictionforChannelMeasurement’ is up to the network. |
| ZTE | Proposal 3.14: Not support. In our views, entry index “15” (“1111” in binary) is useful codepoint/realistic, and we may be flexible if using the “0” (“0000” in binary) (channel correlation estimates of 1-1/256 seems to be non-reasonable high if considering noise/interference). Otherwise, we think the proposal in 3.3+3.12 of removing ‘more than’ seems sufficient. Issue 3.13: Support |
| Samsung | 3.3+3.12: we need at least one pair of CSI-RS measurement with separation D. Proposal: For the Rel-18 TRS-based TDCP reporting, for the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration, the UE reports a TDCP associated with configured D value(s) only if receiving at least [more than one] one pair of CSI-RS transmission occasions corresponding to (or separated by) configured D value(s) [of every CSI-RS resource in the configured CSI-RS resource set(s)] no later than CSI reference resource, else [drops the report] otherwise P 3.14: if the work ‘invalid’ is specified, then we have to define it in the spec, which is not the intension of this proposal I suppose. Including a value ‘0’ is simple, and conveys the same information to the NW. |
| Google | Issue 3.3/3.12: We think this should depend on the configured value of delays. UE is still able to report TDCP for some delays. It is unnecessary to drop the whole TDCP report. Issue 3.13: We suggest removing the note, since it is not aligned with the following text in spec. A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to ‘configured’. [Mod: You are correct] |
| Mod V14 | Some revisions |
| Mod V15 | Revised per Tuesday offline outcomes. 3.13 is also revised as conclusion of no consensus in light of Google input |
| OPPO | Issue 3.13: Support the conclusion. |
| Xiaomi | Issue 3.12/3.13/3.14: Support these conclusions. |
| Fujitsu | Issue 3.13 & 3.14: Support the conclusions. |
| Mod V23 | No revision |

# References