3GPP TSG RAN WG1#114															         R1-238596

Toulouse, France, Aug 21th – Aug 25th, 2023

Source:	Moderator (Apple)

Title:	draft TP for LP-WUR architecture analysis for TR38.869

Agenda Item:	9.11.2

Document for:	Discussion and Decision

# Introduction

This document provides an updated TP based on the endorsed version for the analysis of LP-WUR architecture. The update is based on the updated input from companies in “LP WUR architecture analysis results collection_v014_SS_vivo_QC.xlsx”, plus some other corrections

____________________________Start of Text Proposal for TR38.869_______________________________

# 7	LP-WUR and LP-WUS Design

## 7.1	LP-WUS receiver architectures

### 7.1.1a Analysis of LP-WUR architectures

For the analysis of different LP-WUR receiver types, at least the following aspects have been considered, and the corresponding power consumption and noise figure have been estimated.

- Assumed modulation/waveform/coding

- Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any

- Local oscillator

  - Type of oscillator and the corresponding frequency accuracy/drifting

- Handling of time/frequency impairments (e.g. synchronization signal processing and time/frequency synchronization for LP-WUR)

- Presence of PLL or FLL

- ADC: sampling rate, bit-width

- Assumed signal bandwidth and guard band, and frequency location within a carrier (including whether it is fixed or can be flexible)

- RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies), if any

- Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any)

- Assumed frequency band(s) and the support of band and/or carrier tuning

- Duty cycle handling of WUS and other signals (if any)

- Interference rejection capability (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

- Handling of inter-cell interference

- Whether there is any mobility support function, e.g. measurement capability

The results are summarized for each receiver architecture considering different waveform candidates (more details can be found in Annex xxx and the references). The analysis is very high level, and the actual power consumption and noise figure are very implementation specific. Note that the power consumption is provided in relative unit, relative to the deep sleep state of the MR for a non-RedCap UE.

For the analysis,

FR1 is considered.

Most sources assumed RF LNA is present for improved sensitivity.

For LP-WUS bandwidth, most sources consider at least 5MHz. Some sources think the analysis is also applicable to bandwidth up to 20MHz.

#### 7.1.1a.1	OOK-1/2/4

#### 7.1.1a.1.1	RF envelope detection

For OOK-1/2/4 with RF envelope detection, some sources ([7.1-1] [7.1-2] [7.1-3] [7.1-4] [7.1-5] [7.1-6]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-1.

- The relative power consumption for ON state is in the range of 0.01~0.2.

- The noise figure is in the range of 12~22 dB.

Table 7.1.1a-1 Relative power consumption and noise figure for OOK-1/2/4 with RF envelope detection

| Source reference | [7.1-1] | [7.1-2] | [7.1-3] | [7.1-4] | [7.1-5] | [7.1-6] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.05 for single-branch, 0.01 for each additional branch | 0.01 | 0.01~0.1 | 0.01 | 0.01~0.1 | 0.05~0.2 |
| Noise figure (dB) | 20 | 17~22 | [12-18] | 20 | 15 | 20 |

#### 7.1.1a.1.2	Heterodyne architecture with IF envelope detection

For OOK-1/2/4 with heterodyne architecture with IF envelope detection, some sources ([7.1-1] [7.1-2] [7.1-3] [7.1-4] [7.1-5] [7.1-6] [7.1-7] [7.1-8]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-2.

- The relative power consumption for ON state is in the range of 0.1~4.

- The noise figure is in the range of 9~15 dB.

Table 7.1.1a-2 Relative power consumption and noise figure for OOK-1/2/4 with heterodyne architecture with IF envelope detection

| Source reference | [7.1-1] | [7.1-2] | [7.1-3] | [7.1-4] | [7.1-5] | [7.1-6] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.1 for single-branch, 0.01 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.1~1 | 1~4 |
| Noise figure (dB) | 15 | 10~15 | [9-15] | 15 | 12 | 12~15 |

#### 7.1.1a.1.3	Homodyne/zero-IF architecture with baseband envelope detection

For OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection, some sources ([7.1-1] [7.1-2] [7.1-3] [7.1-4] [7.1-5] [7.1-6] [7.1-7] [7.1-8] [7.1-9] [7.1-10]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-3.

- The relative power consumption for ON state is in the range of 0.05~4.

- The noise figure is in the range of 10~16 dB.

Table 7.1.1a-3 Relative power consumption and noise figure for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection

| Source reference | [7.1-1] | [7.1-2] | [7.1-3] | [7.1-4] | [7.1-5] | [7.1-6] | [7.1-7] | [7.1-8] | [7.1-9] | [7.1-10] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.05~ 0.5 | 0.5~1 | 0.1~0.5 | 4 | ~1 | 0.1~0.5 |
| Noise figure (dB) | 15 | 10~15 | [10-16] | 15 | 12 | 15 | 12~15 | 15 | ~15 | 12 |

#### 7.1.1a.2	FSK

#### 7.1.1a.2.1	Parallel RF envelope detection

For FSK with parallel RF envelope detection, some sources ([7.1-1] [7.1-4]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-4.

Table 7.1.1a-4 Relative power consumption and noise figure for FSK with parallel RF envelope detection

| Source reference | [7.1-1] | [7.1-4] |
| --- | --- | --- |
| Power consumption (ON state) | 0.05 for single-branch, 0.01 for each additional branch | 0.01 |
| Noise figure (dB) | 20 | 20 |

#### 7.1.1a.2.1	Parallel heterodyne architecture

For FSK with parallel heterodyne architecture, some sources ([7.1-1] [7.1-2] [7.1-3] [7.1-4]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-5.

- The relative power consumption for ON state is in the range of 0.1~1.

- The noise figure is in the range of 9~15 dB.

Table 7.1.1a-5 Relative power consumption and noise figure for FSK with parallel heterodyne architecture

| Source reference | [7.1-1] | [7.1-2] | [7.1-3] | [7.1-4] |
| --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.1 for single-branch, 0.01 for each additional branch | 0.5 | 0.1~1 | 0.1 |
| Noise figure (dB) | 15 | 10~15 | [9~15] | 15 |

#### 7.1.1a.2.2	Parallel homodyne architecture

For FSK with parallel homodyne architecture, some sources ([7.1-1] [7.1-2] [7.1-3] [7.1-4]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-6.

- The relative power consumption for ON state is in the range of 0.1~1.

- The noise figure is in the range of 10~16 dB.

Table 7.1.1a-6 Relative power consumption and noise figure for FSK with parallel homodyne architecture

| Source reference | [7.1-1] | [7.1-2] | [7.1-3] | [7.1-4] |
| --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 |
| Noise figure (dB) | 15 | 10~15 | [10~16] | 15 |

#### 7.1.1a.2.3	Heterodyne architecture with frequency to amplitude conversion

For FSK with heterodyne architecture with frequency to amplitude conversion, some sources ([7.1-1] [7.1-3]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-7.

Table 7.1.1a-7 Relative power consumption and noise figure for FSK with heterodyne architecture with frequency to amplitude conversion

| Source reference | [7.1-1] (for SC-FSK) | [7.1-3] |
| --- | --- | --- |
| Power consumption (ON state) | 0.1 | 1 |
| Noise figure (dB) | 15 | TBD |

#### 7.1.1a.2.4	Homodyne architecture with frequency to amplitude conversion

For FSK with homodyne architecture with frequency to amplitude conversion, 1 source ([7.1-1]) estimated that for SC-FSK, the relative power consumption for ON state is 0.1, and the noise figure is 15dB.

#### 7.1.1a.3	OFDMA-based signal

For OFDMA-based signal, companies have considered sequence-based signal, where the LP WUR performs either time-domain correlation without FFT or frequency-domain correlation after FFT.

#### 7.1.1a.3.1	Time-domain correlation (without FFT)

For OFDM-based signal with time-domain correlation, some sources ([7.1-1] [7.1-3] [7.1-5] [7.1-6] [7.1-7] [7.1-8] [7.1-9]) provided analysis, with relative power consumption and noise figure summarized in in Table 7.1.1a-8.

- The relative power consumption for ON state is in the range of 0.15~30.

- The noise figure is in the range of 7~25 dB.

Table 7.1.1a-8 Relative power consumption and noise figure for OFDM-based signal with time-domain correlation

| Source reference | [7.1-1] | [7.1-3] | [7.1-5] | [7.1-6] | [7.1-7] | [7.1-8] | [7.1-9] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.15~0.2 | 10 | 10~20 | 10~30 | 1~5 | 10~20 | ~5 |
| Noise figure (dB) | 15 | 9.5 | 9.5 or 12 | 9 | 7~10 | 9 | 15~25 |

#### 7.1.1a.3.2	Frequency-domain correlation (with FFT)

For OFDM-based signal with frequency-domain correlation, some sources ([7.1-2] [7.1-3] [7.1-5] [7.1-7] [7.1-10]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-9.

- The relative power consumption for ON state is in the range of 1~30.

- The noise figure is in the range of 7~12 dB.

Table 7.1.1a-9 Relative power consumption and noise figure for OFDM-based signal with frequency-domain correlation

| Source reference | [7.1-2] | [7.1-3] | [7.1-5] | [7.1-7] | [7.1-10] | [7.1-10] |
| --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 10 | 30 | 20~30 | 1~5 | 10 | 4 |
| Noise figure (dB) | 7~12 | 7 | 9.5 or 12 | 7~10 | 9 | 12 |

### 7.1.3 References

[7.1-1]	3GPP R1-2306549: “Discussion on architecture of LP-WUS receiver”, Huawei, HiSilicon.

[7.1-2]	3GPP R1-2306692: " Discussion on LP-WUS receiver architectures", InterDigital, Inc.

[7.1-3]	3GPP R1-2308472: "Remaining issues on low power wake-up receiver architecture", vivo.

[7.1-4]	3GPP R1-2307063: "Low-power WUS receiver architectures and its performance", CATT.

[7.1-5]	3GPP R1-2307143: "LP-WUS receiver architectures", ZTE, Sanechips.

[7.1-6]	3GPP R1-2307700: "Receiver architecture for LP-WUS", Samsung.

[7.1-7]	3GPP R1-2307990: "Low power WUS receiver architectures", Ericsson.

[7.1-8]	3GPP R1-2307949: "Receiver architecture for LP-WUS", Qualcomm Incorporated.

[7.1-9]	3GPP R1-2307300: "On low power wake-up receiver architectures", Apple, Inc.

[7.1-10]	3GPP R1-2308068: "Low power WUS receiver architectures", MediaTek, Inc.