3GPP TSG-RAN WG1 Meeting #114	  R1-2306516

Toulouse, France, August 21 - 25, 2023

Agenda Item:	9.2.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

# Introduction

In the RAN1 #111 meeting direct and assisted AI/ML based positioning were agreed to be selected as representative sub-use cases and in RAN1#112 and RAN1#112bis-e more details were identified related to data collection, model inference and model monitoring. These aspects are discussed further in this paper including both the AI/ML operation modes and potential specification impact needed for AI/ML based positioning accuracy enhancements.

This contribution is organized case-by-case based on the following mapping between the agreed sub use cases (direct and assisted positioning) and different positioning frameworks:

| Direct AI/ML positioning | AI/ML assisted positioning |
| --- | --- |
| Case 1: UE-based positioning with UE-side model, downlink positioning | Case 1: UE-based positioning with UE-side model, downlink positioning |
| Case 2b: UE-assisted/LMF-based positioning with LMF-side model, downlink positioning | Case 2a: UE-assisted/LMF-based positioning with UE-side model, downlink positioning |
| Case 3b: NG-RAN node assisted positioning with LMF-side model, uplink positioning | Case 3a: NG-RAN node assisted positioning with gNB-side model, uplink positioning |

# Discussion

## Discussion of Working Assumption from RAN1#112bis-e

In RAN1#112bis-e, the following WA was made:

Working Assumption Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified. - Ground truth label - At least for model training - Report from the label data generation entity - Measurement (corresponding to model input) - At least for model training - Report from the measurement data generation entity - Quality indicator - For and/or associated with ground truth label and/or measurement at least for model training - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity - RS configuration(s) - At least for deriving measurement - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement - Time stamp - At least for and/or associated with training data for model training - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities - Report from data generation entity together with training data and/or as LMF assistance signaling - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement - FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection - Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed - Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

It is our view that not all of the Cases 1-3b do have specification impact. According to offline discussions during RAN1#113, this is also the view of most (maybe all) companies. For example for training in Case 1/2a, when PRU/UE would deliver labels and/or measurements to the UE-side training entity, this should be done transparent to RAN. Therefore, we prefer to add the following note to the Working Assumption.

- : The Working Assumption from RAN1#112bis-e does not have spec impact in all cases. This should be clarified and the following note should be added:

- Note: Whether data collection has spec impact depends on per Case 1/2a/2b/3a/3b.

## Functionality Identification

For LCM, model based and functionality based identification have both been discussed mainly in 9.2.1. In RAN1#113, some companies also raised these aspects also for the use case of positioning.

Functionality based LCM:

For the positioning use case, it is our view that the functionality may at least correspond to the UE-side operations for Case 1 and Case 2a and that the UE should and could indicate their general requirement and support including RS configurations or inference output (e.g., LOS indicator or RSTD for Case 2a) via UE capability signaling.

Model based LCM:

Model based LCM may be at least suitable for two-sided AI/ML models due to the need of model pairing via model ID. For the UE-sided model (Case 1 and Case 2a), it is at least our assessment, that if the UE supports one or multiple models, it may update, fine-tune or switch between them without a need to notify the network (LMF to be specific). Introducing model identification for one-sided models may bring additional effort at the LMF for UE-side model management.

- : Functionality based LCM is appropriate for UE-side model of Case 1/2a.

- Note: Whether model based LCM is applicable to UE-side model of Case 1/2a can be further clarified.

Regarding the conditions for functionality based LCM, following proposal was up for discussion in RAN1#113 meeting [1]:

##### Proposal 1-3b Regarding potential specification impact for LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), the following aspects on information related to the conditions are identified if the corresponding type of LCM is supported - For functionality-based LCM - information related to the conditions of a Feature/FG available for functionality is reported as UE capability - Candidate information related to the conditions to be considered for potential down-selection - Applicable configuration(s) for measurement of a Feature/FG - Measurement and report of a Feature/FG - Applicable (valid) area/scenario/site of a Feature/FG - Generalization of a Feature/FG - Information of input/output/complexity for a model which provide the functionality - monitoring functionality

The candidate information related to the conditions as given in Proposal 1-3b from last meeting is discussed briefly below:

- Measurement and report of a Feature/FG (1st and 2nd sub-sub-bullets) are needed, as the network needs to be aware of the required measurement capability for Case 1/2a, and also needs to be aware of the report for Case 2a, to make the corresponding configurations to the UE.

- The motivation to provide area/scenario/site information (3rd sub-sub-bullet) is not clear in our view, and requires further clarification. Firstly, the UE-side can train a generalized model by dataset mixing over the IIoT scenarios, for which the channel characteristics are very limited compared to the macro network, so the necessity of a scenario specific model is not clear. Secondly, it should be clarified that even for training a scenario specific model, what additional information would be needed to be aligned with the network given that the UE already is itself able to sense the scenario and location for training/selecting an appropriate model. A third issue is that for the purpose of scenario specific model training, a common understanding on the categorization of the scenarios/areas needs to be achieved between the network and UE vendor, so that the interpretation of the physical meaning of the scenario/area is inevitable; this may disclose proprietary information of e.g. the network planning, RF module type, etc.

- The motivation for the alignment of the generalization of a Feature/FG (4th sub-sub-bullet) is not clear since the UE can trigger the model/functionality switching by itself, or request the LMF to grant the model/functionality switching. How to specify the generalization capability of a functionality or a model is not clear; aligning the generalization over scenarios/areas may also disclose the interpretation of the physical meaning of the scenario/area as analyzed above.

- Input/output/complexity (5th sub-sub-bullet). Input (corresponding to measurement)/output (corresponding to report) have been analyzed previously and this discussion should not be repeated. There is no need to provide this information since the complexity of the model is UE proprietary information and should not be reported.

- Conditions for the monitoring functionality (6th sub-sub-bullet) has lower priority in our view, since the potential spec impact of monitoring itself requires further discussion (as will be analyzed in later sections).

- : For the conditions of functionality based LCM for UE-side model of Case 1/2a, consider the applicable configuration(s) for measurement and report of a Feature/FG with higher priority.

## Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

### Model deployment

In the RAN1 #111 meeting the following agreement for model deployment during inference was achieved:

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

For the one-sided model, the inference is performed at either network-side or UE-side, but it is not agreed where the training and updating procedures take place. When the training and inference happen in different sides, an AI/ML model exchange would face the issues of model transfer. In our view, discussing model transfer in AI/ML positioning would introduce unnecessary complications at this late stage, it should be kept in AI 9.2.1 where it already is addressed. Therefore, we only discuss the cases when model training and model inference are performed by the same side and we are making the following proposal:

- : For Case 1 (UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning), the model training/updating and inference are performed all at UE-side.

### Data collection for model training and model updating

As illustrated in Figure 1, for the UE-side positioning model’s training and updating, the UE needs to collect channel measurements of PRS, e.g., the enhanced delay profile and also ground truth coordinates as the model training inputs. These channel measurements can be obtained with the help of PRUs and UEs. And the ground truth coordinates can be obtained with the help of PRUs as agreed in RAN1#112 (“At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1)”).

Direct/Assisted positioning model training/updating phase

Figure 1 – Case 1 model training

In RAN1#113 it has been agreed to study the necessity and benefits whether UEs could be used to generate ground truth labels for the use case, i.e. when the required positioning accuracy cannot be achieved only by using PRUs. If sufficient data can be generated from PRUs there is no need to additionally consider other entities. In our view this should be the case in (almost) all situations, since PRUs could be mounted on AGVs to be mobile within the service area and to autonomously collect measurements and labels from different locations. If it still is deemed that no sufficient numbers of labels generated from PRUs are available, then the additionally from the UE approximated labels should have a higher accuracy than the accuracy requirements of the use case. It is not clear to us how the UE can assess the accuracy of it self-generated ground truth labels, and whether this estimation needs to introduce RAN4 requirement.

- : In the majority of situations, sufficient ground truth labels generated from PRUs are available. For ground truth labels approximated by UEs, it is not clear how the UE accuracy can be known and guaranteed.

In light of the above observation, the following discussions are based on only using PRUs to generate the ground truth labels. Then, the following sources and destination for the generation and signaling of measurements and labels would be applicable for training data collection. As the training entity and the sources to generate labels/measurement are both located at the UE-side, there is no motivation for any network-side entity to be involved in the data collection procedure. The data collection for Case 1 training could be handled entirely in a network transparent manner.

Table 1 – Case 1 (direct and assisted positioning). Signaling of measurements and labels

|  | Generation source | Signaled to | Potential spec impact |
| --- | --- | --- | --- |
| Measurements | PRU/UE based on PRS | UE-side training entity | None |
| Labels | PRU | UE-side training entity | None |

Based on the above discussion and made agreements/working assumptions, we are making the following observation:

- : For Case 1, data collection for training can be achieved at least with a spec transparent method.

### Model inference

For both UE-based direct and assisted AI/ML positioning with a UE-side model, as shown in Figure 2, the inference is directly performed at the UE itself with low latency for downlink positioning.

| - Direct positioning model inference phase | - Assisted positioning model inference phase |
| --- | --- |

Figure 2 - Case 1 model inference

We can make the following observation for the signaling of measurements during inference for direct positioning:

- : For the model inference of Case 1 for direct positioning,

- The gNB sends PRS to the UE that contains the AI/ML model.

- The UE infers the position based on the channel measurements obtained from PRS.

For UE-based AI/ML assisted positioning with UE-side model, as shown in Figure 2(b), the inference and final positioning are performed at the UE itself. We can make the following observation for the signaling of measurements during inference:

- : For the model inference of Case 1 for assisted positioning,

- The gNB sends PRS to the UE that contains the AI/ML model.

- The UE infers the information needed for final positioning (e.g. LOS/NLOS states, RSTD).

- The UE performs the final positioning.

We therefore observe that inference for Case 1 can be performed in a spec transparent manner.

- : For Case 1, inference can be at least achieved with a spec transparent method.

### Model monitoring

In the RAN1 #112bis e-meeting, the following agreement for model monitoring with UE-side model was achieved:

Agreement Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model)

According to above agreement for Case 1/2a, the entity that derives monitoring metric can at least be UE. To be more generic, it can be extended to the UE-side.

- As one candidate, the UE generates the metric as the distribution of the measurement (model input) and make monitoring decision.

- As another candidate, the PRU generates the metrics for the UE-side decision.

- As a third candidate, the UE-side server can generate the metrics by implementation based on the inputs from PRU.

Over all of the above candidates, the entity to generate metrics and make monitoring decision is the same, i.e., the UE-side can define the metric and monitor its own performance with implementation.

- : For the monitoring of Case 1, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For UE-side model monitoring, the metric is derived in the same UE-side entity which also performs the monitoring.

Another candidate entity to derive the monitoring metric is the PRU (which was agreed as FFS in RAN1#112):

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a

The PRU can generate the ground truth label. E.g., it can be used to derive metrics based on ground truth labels and output in implementation manner. Alternatively, PRU can also generate the metric based on the distribution of the measurements without ground truth.

- : For the monitoring of Case 1/2a, PRUs can be additionally used to derive the monitoring metric with spec transparent manner.

## Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

### Model deployment

As discussed in Section 2.3.1, we only address the cases when model training and model inference are performed by the same side and we are making the following proposal:

- : For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at UE-side.

### Data collection for model training and model updating

As illustrated in Figure 3, for the UE-side positioning model’s training and updating, the UE needs to collect channel measurements of PRS, e.g., the enhanced delay profile and also ground truth coordinates as the model training inputs. These channel measurements can be obtained with the help of PRUs and UEs. And the ground truth coordinates can be obtained with the help of PRUs (“At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)”).

Assisted positioning model training/updating phase

Figure 3 - Case 2a model training

The following sources and destination for the generation and signaling of measurements and labels would be applicable for training data collection. Similar to Case 1, as the training entity and the sources to generate labels/measurement are both subject to UE-side, there seems to be no motivation for the network-side to be involved in the data collection procedure. The data collection for Case 2a training could be handled entirely in a network transparent manner.

Table 2 – Case 2a. Signaling of measurements and labels

|  | Generation source | Signaled to | Potential spec impact |
| --- | --- | --- | --- |
| Measurements | PRU/UE based on PRS | UE-side training entity | None |
| Labels | PRU | UE-side training entity | None |

Based on the above discussion and made agreements/working assumptions, we are making the following observation:

- : For Case 2a, data collection for training can be achieved at least with a spec transparent method.

### Model inference

For inference, as shown in Figure 4, the UE measures PRS from the gNBs and uses the results as the input to the AI/ML model and then transmits the inference results (such as RSTD or LOS indicator) to the LMF where the UE coordinates are calculated.

Assisted positioning model inference phase

Figure 4 - Case 2a model inference

Based on the above discussion, we can make the following observation for the signaling of measurements during inference:

- : For the model inference of Case 2a,

- The gNB sends PRS to the UE that performs the channel measurements.

- The UE uses its channel measurements to infer the model output (e.g., LOS/NLOS states, RSTD).

- The UE transmits the model output to the LMF that performs the final positioning.

In RAN1#113 the following agreement was mad for Case 2a:

Agreement For assisted AI/ML positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), at least the following types of model inference output are identified as candidates providing performance benefits - Timing estimation - FFS potential specification impact including details of report to LMF, e.g., time difference relative to a reference time, soft information report - FFS applicability for DL-TDOA, UE/gNB RTT and UL-RTOA - Note: the report to LMF is derived based on and maybe different from the model inference output - LOS/NLOS indicator - FFS potential specification impact (if any w.r.t. existing measurement report) - FFS RSRPP

The above agreement contains FFS whether spec impact is needed for AI/ML model output “timing information” and “LOS/NLOS indicator”. Both functionalities are already supported in the existing specification and corresponding evaluations have shown that they result into good performance. Whether/how these report can be further enhanced can be discussed, maybe based on the evaluations. The agreements also contain a FFS on the RSRPP. In Rel-17, the RSRPPs (with delay and power) can be provided for up to 9 paths, but according to our understanding of evaluations the RSRPP has not been considered as inference output, only as inference input. We do not see any justification to consider RSRPP as model output for assisted positioning.

- : For the model inference of Case 2a, the report of the model output(s) timing estimation, LOS/NLOS indication may be further studied.

### Model monitoring

As analyzed for Case 1, also for Case 2a the entity to generate metrics can be the same as the entity to perform monitoring. Similar to Case 1, UE-side monitoring can be considered with higher priority. Moreover, the PRU could also be considered to generate the monitoring metrics.

In RAN1#113 the LMF was agreed to be considered as a candidate to derive the monitoring for Case 2a:

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, the following entities are identified as candidates to derive monitoring metric in addition to entities from previous agreement - LMF for Case 2a (with UE-side model) and Case 3a (with gNB-side model) at least when monitoring is based on provided ground truth label (or its approximation)

For LMF, on the other hand, before going into the generation of a metric, it needs to be discussed whether it is eligible to monitor the UE-side model.

- Firstly, as the LMF has no UE-side model nor any of its knowledge, it is most likely not eligible to monitor the performance. The network deployer can consider relying on the UE or UE-side server to monitor the positioning performance under this situation.

- Second, if there is need that LMF has to do the monitoring, it may have an implementation manner to obtain the label and the model output, e.g. from the PRU or UE-side server, or request the UE to report the measurement (which is the same as for Case 2b).

- : For the monitoring of Case 2a, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For UE-side model monitoring, the metric is derived in the same entity which also performs the monitoring.

- For LMF-side monitoring (if eligible), the LMF can request ground truth label/measurements from PRU or request measurements from UE to derive the metric.

For LMF-side monitoring of Case 2a, if eligible, it can be performed based on the report of the measurements/model output from UE or the report of ground truth label/measurement from PRU. There is no strong need to consider additional spec efforts on supporting LMF based monitoring.

- : For the monitoring of Case 2a, there seems no strong need to spend additional standard effort on LMF-side monitoring (if eligible).

The approach to let the UE derive the metric and to let it monitor itself is simpler and straightforward as opposed to LMF-side monitoring. For the LMF-side monitoring, there seems to be no strong need to spend additional efforts on specified manner to derive the metrics (other than the measurement report). We are therefore making the following proposal for Case 2a:

- : For the monitoring of Case 2a, UE-side monitoring can be considered with higher priority.

- UE-side entity collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

## Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

### Model deployment

As discussed in Section 2.3.1, we only discuss the cases when model training and model inference are performed by the same side as starting point and we are making the following proposal:

- : For Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF-side.

### Data collection for model training and model updating

During the model training/updating phase, as shown in Figure 5, PRUs or other UEs measure PRS from gNBs and transmit the results, e.g., the enhanced delay profile to the network. For the labels, LMF is assumed to generate the ground truth label of the PRU. After measurements have been received and labels have been generated by the LMF, it can perform AI/ML model training/updating for positioning.

Direct positioning model training/updating phase

Figure 5 - Case 2b model training

Based on the agreements from previous meeting, the following sources and destination for the generation and signaling of measurements and labels would be applicable for training data collection.

Table 3 – Case 2b. Signaling of measurements and labels

|  | Generation source | Signaled to | Potential spec impact |
| --- | --- | --- | --- |
| Measurements | PRU/UE generates model input based on PRS | LMF | FFS on the enhancement of RSRPP |
| Labels | LMF is using the known PRU location | Available inside LMF | None |

The data collection for Case 2b requires signaling of measurements as model input over the air interface. The overhead and performance should be evaluated and if deemed as feasible, enhanced reporting of measurements could be discussed from PRU/UE to LMF.

From our evaluations in [2], DP or RSRPP (i.e., DP with power) with up to 9 paths can be a good candidate to achieve good performance with much less overhead than CIR/PDP. For example it can be seen that for 18 TRPs as model input dimension, already the RSRPP would result in very good accuracy. On top of that, reporting the properly increasing the number of paths beyond 9 may bring additional benefits to the positioning accuracy.

Based on the above discussion, we are making the following proposal:

- : For the data collection for training of Case 2b, consider the report of RSRPP from PRU to LMF as the model input which can achieve good performance but with much less overhead than CIR/PDP.

### Model inference

For inference, as shown in Figure 6, the UEs measure PRS from the gNBs and transmit the results to the LMF where the UE coordinates are inferred.

Direct positioning model inference phase

Figure 6 - Case 2b model inference

It should be noted that, for the inference, the reporting of the measurement as model input is similar to the training phase, except that the report is from UE to LMF rather than PRU. Therefore, we are making the following proposal:

- : For the model inference of Case 2b, consider the report of RSRPP from UE to LMF as the model input which can achieve good performance but with much less overhead than CIR/PDP.

### Model monitoring

In the RAN1 #112bis e-meeting, the following agreement for model monitoring with LMF-side model was achieved:

Agreement Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model)

Monitoring the LMF-side model can be entirely performed at LMF-side. The LMF can collect the ground truth labels obtained by using the PRU information as monitoring inputs and calculates the metric. Then it makes monitoring decisions according to the metric. As an alternative, the LMF can also request the UE to report the measurement (which has similar spec impact as inference) and perform input distribution based monitoring by implementation.

- : For the monitoring of Case 2b, the monitoring metric can be derived at the LMF without additional spec impact on top of inference.

- LMF collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

## Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

### Model deployment

As discussed in Section 2.3.1, we only address the cases when model training and model inference are performed by the same node as starting point and we are making the following proposal:

- : For Case 3a (NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at gNB side.

### Data collection for model training and model updating

During the model training/updating phase, as shown in Figure 7 for uplink positioning, PRUs are used to collect the model training inputs. They send SRS as configured. And the gNB measures the SRS and uses the results together with the known labels to train the AI/ML model (from RAN1#112: “At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)”). This operation can be performed without specification impact.

Assisted positioning model training/updating phase

Figure 7 - Case 3a model training

Based on the agreements from previous meeting, the following sources and destination for the generation and signaling of measurements and labels would be applicable for training data collection.

Table 4 – Case 3a. Signaling of measurements and labels

|  | Generation source | Signaled to | Potential spec impact |
| --- | --- | --- | --- |
| Measurements | gNB generates model input (e.g. DP/CIR/ PDP) based on SRS | Kept in gNB | None |
| Labels | gNB generates/obtains the ground truth label by implementation | Available inside gNB | None |

Based on the above discussion and agreements, we are making the following observation:

- : For Case 3a, data collection for training can be achieved at least with a spec transparent method.

### Model inference

As shown in Figure 8, during inference, the gNBs can measure SRS from PRUs or UEs, using the measurements as the input to the AI/ML model and then transmit the inference results to the LMF where the UE coordinates are calculated.

Assisted positioning model inference phase

Figure 8 - Case 3a model inference

Based on the above discussion, we can make the following observation for the signaling of measurements during inference:

- : For the model inference of Case 3a,

- The UE transmits SRS to the gNB.

- The gNB performs channel measurements based on SRS and delivers them as input to the AI/ML-model to infer the model output (e.g., LOS/NLOS states, RSTD).

- The gNB transmits the results of the inference to the LMF where the final positioning is performed.

Similar to our discussion about Case 2a, it is our view that the legacy mechanisms to obtain LOS, RSTD achieve sufficient performance. Whether/how to perform enhancements on top of these can be further studied.

- : For the model inference of Case 3a, the report of the model output(s) timing estimation, LOS/NLOS indication can be studied.

### Model monitoring

In the RAN1 #112bis e-meeting, the following agreement for model deployment with gNB-side model was achieved:

Agreement Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model)

In addition to the entity from the agreement above, according to the progress in RAN1#113, the LMF was agreed to be considered as a candidate to derive the monitoring for Case 3a:

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, the following entities are identified as candidates to derive monitoring metric in addition to entities from previous agreement - LMF for Case 2a (with UE-side model) and Case 3a (with gNB-side model) at least when monitoring is based on provided ground truth label (or its approximation)

Similar to our analysis to Case 2a, the entity that derives monitoring metric can at least be gNB.

- As one candidate, the gNB generates the metric as the distribution of the measurement (model input) and makes the monitoring decision.

- As another candidate, the gNB generates both the ground truth label (refer to the analysis for training) and the model output to calculate the metrics. Then the gNB can make monitoring decision.

For LMF, if eligible, how it derives the metric for monitoring is discussed below.

- As one candidate, LMF can request the PRU to deliver the ground truth label, and request the gNB to deliver the inference output (which is the same as inference for Case 3a) of the same PRU; the LMF then compares the ground truth label with the output to derive the metric.

- As another candidate, the gNB can deliver the measurement (model input, which is the same as inference for Case 3b) to the LMF, and the LMF generates the metric as the distribution of measurement.

Similar to Case 2a, the approach to let the gNB derive the metric and to let it monitor itself is straightforward. For the LMF-side monitoring, if eligible, there seems to be no strong need to spend additional efforts on a specified manner to derive the metrics (other than the report of measurement/model output as for inference).

Based on the above discussion, we can make the following proposal for the signaling of model monitoring:

- : For the monitoring of Case 3a, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For gNB-side model monitoring, the metric is derived in the same entity which also performs the monitoring.

- For LMF-side monitoring (if eligible), the LMF can request ground truth label from PRU and request inference output from gNB to derive the metric.

- : For the monitoring of Case 3a, gNB-side monitoring can be considered with higher priority.

- gNB side collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

- : For the monitoring of Case 3a, there seems no strong need to spend additional spec effort on LMF-side monitoring (if eligible).

## Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

### Model deployment

As mentioned in Section 2.3.1, we only discuss the cases when model training and model inference are performed by the same node as starting point and we are making the following proposal:

- : For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF-side.

### Data collection for model training and model updating

During the model training/updating phase as shown in Figure 9, PRUs/UEs send SRS as being configured which then is measured at the gNB. Regarding the ground truth labels, the PRU positions are already known by the LMF and will not be signaled. The gNBs will then measure SRS from PRUs and transmit the results, e.g., the enhanced delay profile and corresponding labels to the LMF.

Direct positioning model training/updating phase

Figure 9 - Case 3b model training

Based on the agreements from previous meeting, the following sources and destination for the generation and signaling of measurements and labels would be applicable for training data collection.

Table 5 – Case 3b. Signaling of measurements and labels

|  | Generation source | Signaled to | Potential spec impact |
| --- | --- | --- | --- |
| Measurements | gNB generates model input based on SRS | LMF | FFS on the enhancement of RSRPP |
| Labels | LMF | Available inside LMF | None |

Similar to Case 2b, the data collection for Case 3b requires signaling of measurement as model input from gNB to LMF.

For the type of the measurement, RSRPP with up to 9 paths can be a good candidate. Though the overhead between gNB and LMF is not big burden, RSRPP with proper enhancement on, e.g., number of paths, can already provide similar performance as CIR/PDP as per our evaluation [2]. So there seems no strong need to introduce new measurement types.

Therefore, we are making the following proposal:

- : For the data collection for training of Case 3b, consider the report of RSRPP from gNB to LMF as the model input which can achieve good performance but with smaller spec effort than CIR/PDP.

### Model inference

As shown in Figure 10, for inference, gNBs can measure SRS from the UEs and transmit the results to the LMF where the UE coordinates are inferred.

Assisted positioning model inference phase

Figure 10 - Case 3b model inference

For the inference, the reporting of the gNB measurement as model input is similar to the training phase. Therefore, we are making the following proposal:

- : For the model inference of Case 3b, consider the report of RSRPP from gNB to LMF as the model input which can achieve good performance but with smaller spec effort than CIR/PDP.

### Model monitoring

Similar to Case 2b, we are making the following proposal for Case 3b.

- : For the monitoring of Case 3b, the monitoring metric can be derived at the LMF without additional spec impact on top of inference.

- LMF collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

# Conclusions

In this contribution, we discussed the potential specification impact and the consideration of sub use cases for positioning accuracy enhancements. Based on the discussion, we have the following proposals and observations:

- : Functionality based LCM is appropriate for UE-side model of Case 1/2a.

- Note: Whether model based LCM is applicable to UE-side model of Case 1/2a can be further clarified.

- : In the majority of situations, sufficient ground truth labels generated from PRUs are available. For ground truth labels approximated by UEs, it is not clear how the UE accuracy can be known and guaranteed.

- : For Case 1, data collection for training can be achieved at least with a spec transparent method.

- : For the model inference of Case 1 for direct positioning,

- The gNB sends PRS to the UE that contains the AI/ML model.

- The UE infers the position based on the channel measurements obtained from PRS.

- : For the model inference of Case 1 for assisted positioning,

- The gNB sends PRS to the UE that contains the AI/ML model.

- The UE infers the information needed for final positioning (e.g. LOS/NLOS states, RSTD).

- The UE performs the final positioning.

- : For Case 1, inference can be at least achieved with a spec transparent method.

- : For the monitoring of Case 1, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For UE-side model monitoring, the metric is derived in the same UE-side entity which also performs the monitoring.

- : For Case 2a, data collection for training can be achieved at least with a spec transparent method.

- : For the model inference of Case 2a,

- The gNB sends PRS to the UE that performs the channel measurements.

- The UE uses its channel measurements to infer the model output (e.g., LOS/NLOS states, RSTD).

- The UE transmits the model output to the LMF that performs the final positioning.

- : For the monitoring of Case 2a, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For UE-side model monitoring, the metric is derived in the same entity which also performs the monitoring.

- For LMF-side monitoring (if eligible), the LMF can request ground truth label/measurements from PRU or request measurements from UE to derive the metric.

- : For the monitoring of Case 2a, there seems no strong need to spend additional spec effort on LMF-side monitoring (if eligible).

- : For Case 3a, data collection for training can be achieved at least with a spec transparent method.

- : For the model inference of Case 3a,

- The UE transmits SRS to the gNB.

- The gNB performs channel measurements based on SRS and delivers them as input to the AI/ML-model to infer the model output (e.g., LOS/NLOS states, RSTD).

- The gNB transmits the results of the inference to the LMF where the final positioning is performed.

- : For the monitoring of Case 3a, there seems to be no need to request a metric as it can be derived at the monitoring entity

- For gNB-side model monitoring, the metric is derived in the same entity which also performs the monitoring.

- For LMF-side monitoring (if eligible), the LMF can request ground truth label from PRU and request inference output from gNB to derive the metric.

- : For the monitoring of Case 3a, there seems no strong need to spend additional spec effort on LMF-side monitoring (if eligible).

- : The Working Assumption from RAN1#112bis-e does not have spec impact in all cases. This should be clarified and the following note should be added:

- Note: Whether data collection has spec impact depends on per Case 1/2a/2b/3a/3b.

- : For the conditions of functionality based LCM for UE-side model of Case 1/2a, consider the applicable configuration(s) for measurement and report of a Feature/FG with higher priority.

- : For Case 1 (UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning), the model training/updating and inference are performed all at UE-side.

- : For the monitoring of Case 1/2a, PRUs can be additionally used to derive the monitoring metric with spec transparent manner.

- : For Case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at UE-side.

- : For the model inference of Case 2a, the report of the model output(s) timing estimation, LOS/NLOS indication may be further studied.

- : For the monitoring of Case 2a, UE-side monitoring can be considered with higher priority.

- UE-side entity collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

- : For Case 2b (UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF-side.

- : For the data collection for training of Case 2b, consider the report of RSRPP from PRU to LMF as the model input which can achieve good performance but with much less overhead than CIR/PDP.

- : For the model inference of Case 2b, consider the report of RSRPP from UE to LMF as the model input which can achieve good performance but with much less overhead than CIR/PDP.

- : For the monitoring of Case 2b, the monitoring metric can be derived at the LMF without additional spec impact on top of inference.

- LMF collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

- : For Case 3a (NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning), the model training/updating and inference are performed all at gNB side.

- : For the model inference of Case 3a, the report of the model output(s) timing estimation, LOS/NLOS indication can be studied.

- : For the monitoring of Case 3a, gNB-side monitoring can be considered with higher priority.

- gNB side collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

- : For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), the model training/updating and inference are performed all at LMF-side.

- : For the data collection for training of Case 3b, consider the report of RSRPP from gNB to LMF as the model input which can achieve good performance but with smaller spec effort than CIR/PDP.

- : For the model inference of Case 3b, consider the report of RSRPP from gNB to LMF as the model input which can achieve good performance but with smaller spec effort than CIR/PDP.

- : For the monitoring of Case 3b, the monitoring metric can be derived at the LMF without additional spec impact on top of inference.

- LMF collects inputs for monitoring, calculates monitoring metric, and makes monitoring decision.

# References

- R1-2306206, “FL summary #5 of other aspects on AI/ML for positioning accuracy enhancement”, vivo, RAN1#113, Incheon, Korea, May 22 - 26, 2023.

- R1-2306515, “Evaluation on AI/ML for positioning accuracy enhancement”, RAN1#114, Huawei, HiSilicon, Toulouse, France, August 21 - 25, 2023.