3GPP TSG RAN WG1 #114	R1-2306685

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.4.3
Source:	Toyota InfoTechnology Center
Title:	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Document for:	Discussion and Decision

# Introduction

In RAN #98-e meeting [1], the revised WID for Rel-18 NR sidelink evolution was approved [2]. In this revision, it was agreed to start the study of NR sidelink beam management on the FR2 licensed spectrum as shown below. This study is limited to the support of NR sidelink beam management for unicast communication by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution, we provide our views on NR sidelink beam management on FR2 licensed spectrum.

# Discussion on Sidelink Communication in FR2 Bands

In this section, we discuss aspects related to NR sidelink beam management in FR2 bands, which includes sidelink beam initial beam pairing, beam maintenance, beam failure recovery, and sensing/resource selection/resource reservation.

## Sidelink Initial Beam Pairing

### General Procedure

In the RAN1 #113 meeting [3], the following agreements were reached for initial beam pairing before/during/after unicast link establishment.

Agreement In the candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, - In step 1, - the applicable reference signal is selected based on - Alt 1-1: S-SSB or its modified format - Alt 1-2: standalone SL CSI-RS or its modified format - Alt 1-3: non-standalone SL CSI-RS - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. - Alt 1-4: PSCCH/PSSCH DMRS - the reference signals are sent - Alt 2-0: aperiodically - Alt 2-1: periodically - Alt 2-2: semi-persistent with activation and deactivation - FFS details of activation/deactivation - FFS resources and resource allocation of reference signal - FFS: if CSI-RS is used, whether UE1 transmits other information associated with the CSI-RS - In step 2, - UE1’s transmit beam and UE2’s receive beam are determined by UE2 as the pair with the RSRP measurement satisfying certain condition(s) - FFS details of condition(s) - FFS explicit or implicit determination of UE1 transmit beam by UE2 - UE2’s transmit beam is at least determined as the one corresponding to determined UE2’s receive beam, at least if beam correspondence is assumed - FFS other scheme - In step 3, - UE2’s beam reporting is associated with determined UE1’s transmit beam. - FFS details of association - FFS details of beam reporting - Note: this does not preclude beam reporting in the link establishment message. - FFS: how UE1 determines its transmit beam if it receives different beam reporting from different UEs - FFS: whether/how to avoid unnecessary beam measurement and reporting from multiple UEs; Agreement In the candidate procedure where initial beam pairing is performed during sidelink unicast link establishment, - In step 1, the candidate reference signal which is transmitted together with unicast link establishment message is selected based on one of the following alternatives - Alt 1-1: SL CSI-RS - Alt 1-2: PSCCH/PSSCH DMRS - In step 2, UE2 determines UE1’s transmit beam(s) and UE2’s receive beam(s) as the pair with the RSRP measurement satisfying certain condition(s). - UE2’s transmit beam is at least determined as the one corresponding to determined UE2’s receive beam, at least if beam correspondence is assumed - FFS the format of UE1’s transmit beam determined by UE2, e.g. implicit or explicit - FFS details of condition(s) - In step 2, UE2 indicates UE1’s transmit beam(s). - FFS details of beam indication, including contents (e.g., ACK/NACK, beam ID, RSRP measurement), container (e.g., PSCCH/PSSCH, PSFCH) and association (e.g. resources) - In step 3, UE1 determines UE1’s transmit beam based on one or more of the following alternatives - Alt 2-1: the latest beam indication - Alt 2-2: beam indication contents (e.g., RSRP measurement) - Alt 2-3: measurement/detection of beam indication signal Agreement In the candidate procedure where initial beam pairing starts after sidelink unicast link establishment (if feasible), the initial beam pairing follows a similar procedure as beam maintenance.

In our view, initial beam pairing should be performed during unicast link establishment. This is because initial beam pairing before and after unicast link establishment have issues and limitations:

- Initial beam pairing before unicast link establishment: there is an issue of unnecessary beam pairing because each UE needs to perform initial beam pairing with every surrounding UE regardless of whether they perform unicast communication or not after initial beam pairing. This leads to a waste of resources and increased energy consumption. Also, this is problematic particularly when there are a lot of UEs in proximity;

- Initial beam pairing after unicast link establishment: there is an issue of a limited communication range because paired beams cannot be used for unicast link establishment. Therefore, this procedure is not suitable for use cases that require a long communication range, e.g., V2X. Also, it is not clear how to establish unicast links without initial beam pairing.

Initial beam pairing during unicast link establishment can mitigate these issues. Therefore, we propose to prioritize the study of initial beam pairing during unicast link establishment over initial beam pairing before/after unicast link establishment. In addition, if initial beam pairing before unicast link establishment is further studied, RAN1 should study solutions to avoid unnecessary beam measurement and reporting from multiple UEs.

Observation 1: Initial beam pairing before unicast link establishment has an issue of unnecessary beam pairing because each UE needs to perform initial beam pairing with every surrounding UE regardless of whether they perform unicast communication or not after initial beam pairing. This leads to a waste of resources and power consumption. This is problematic particularly when there are a lot of UEs in proximity.

Observation 2: Initial beam pairing after unicast link establishment has an issue of a limited communication range because paired beams cannot be used for unicast link establishment. Therefore, this procedure is not suitable for use cases that require a long communication range, e.g., V2X. It is not clear how to establish unicast links without initial beam pairing.

Observation 3: Initial beam pairing during unicast link establishment can mitigate the issues of unnecessary beam pairing and limited communication range.

Proposal 1: Prioritize the study of initial beam pairing during unicast link establishment over initial beam pairing before/after unicast link establishment.

Proposal 2: For initial beam pairing before unicast link establishment, RAN1 to study solutions to avoid unnecessary beam measurement and reporting from multiple UEs.

## Sidelink Beam Maintenance

### Sidelink CSI-RS for Sidelink Beam Maintenance

In the RAN1 #113 meeting [3], the following agreements were reached for sidelink CSI-RS transmissions for sidelink beam maintenance.

Agreement For beam maintenance, consider non-standalone sidelink CSI-RS transmissions. - FFS physical layer structure, including enhancements of existing non-standalone sidelink CSI-RS - FFS signaling details - FFS timing details - FFS whether non-standalone sidelink CSI-RS transmissions use the same or different transmit beam as accompanying data - FFS whether/how to support multiple transmit beams for non-standalone sidelink CSI-RS in the same slot Agreement For beam maintenance, consider standalone sidelink CSI-RS transmissions. - FFS physical layer structure - FFS timing details - FFS whether/how the same or different transmit beams are used in the same slot of standalone sidelink CSI-RS transmissions. - FFS resources, resource allocation and resource indication - FFS signaling details - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. Agreement For beam maintenance, consider design details for sidelink CSI-RS transmissions with or without repetition on transmit beams.

In Rel-16/17 NR sidelink, only non-standalone SL CSI-RS transmission is supported, i.e., SL CSI-RS transmission is always accompanied with sidelink data in the same slot. One open question is whether to support standalone SL CSI-RS, which does not accompany sidelink data in the same slot. In our view, standalone SL CSI-RS is useful to accommodate a greater number of symbols for CSI-RS, which enables to search for a greater number of beams within a slot. Therefore, we propose to study standalone SL CSI-RS transmission.

Observation 4: Standalone SL CSI-RS transmission for sidelink beam maintenance is useful to search a greater number of beams within a slot.

Proposal 3: Study standalone SL CSI-RS transmissions for sidelink beam maintenance.

For sidelink beam maintenance, sidelink CSI-RS transmissions with and without repetition on transmit beams within a slot should be considered. Sidelink CSI-RS transmissions with repetition on the same transmit beam within a slot is beneficial for fast Rx beam training within the slot. In addition, multiple sidelink CSI-RS transmissions of different Tx beams without repetition on transmit beams within a slot is beneficial for fast Tx beam training within the slot.

Observation 5: For sidelink beam maintenance, sidelink CSI-RS transmissions with repetition on transmit beams within a slot is beneficial for fast Rx beam training within the slot. In addition, multiple sidelink CSI-RS transmissions of different Tx beams without repetition on transmit beams within a slot is beneficial for fast Tx beam training within the slot.

Proposal 4: Study sidelink CSI-RS transmissions with and without repetition on transmit beams within a slot.

Proposal 5: Study multiple sidelink CSI-RS transmissions of different Tx beams without repetition on transmit beams within a slot.

In the RAN1 #113 meeting [3], multiple sidelink CSI-RS transmissions within a slot for sidelink beam maintenance was discussed. The following was proposed by FL but not agreed.

FL proposal in RAN1 #113 Proposal 2-4-b: For beam maintenance, support consider multiple sidelink CSI-RS transmissions in a slot. - FFS whether/how to avoid/mitigate AGC issues in case the multiple sidelink CSI-RS transmissions are transmitted using different transmit beams - FFS applicable scenarios

When Tx UE performs Tx beam sweeping based on multiple sidelink CSI-RS transmissions using different Tx beams within a slot, it may cause an AGC issue at surrounding Rx UEs due to a sudden, unexpected change of received powers within the slot. Therefore, if Tx beam sweeping within a slot is further studied, this AGC issue needs to be addressed.

Observation 6: For sidelink beam maintenance, Tx beam sweeping within a slot may cause an AGC issue at surrounding Rx UEs.

Proposal 6: Study solution(s) to mitigate the AGC issue due to Tx beam sweeping within a slot.

In the RAN1 #113 meeting [3], aperiodic, periodic, and/or semi-persistent sidelink CSI-RS transmissions for sidelink beam maintenance was discussed. The following was proposed by FL but not agreed.

FL proposal in RAN1 #113 Proposal 2-3-b: For beam maintenance, supportconsider aperiodic, periodic and/or semi-persistent sidelink CSI-RS transmissions. - FFS triggering conditions for aperiodic sidelink CSI-RS transmissions - FFS activation/deactivation of semi-persistent sidelink CSI-RS transmissions

Rel-16/17 NR sidelink supports only aperiodic CSI measurement/reporting, which is triggered by a 1-bit CSI request in the 2nd SCI. For sidelink beam measurement and reporting, one open question is the periodicity of CSI-RS transmissions, i.e., whether to support periodic/semi-persistent and/or aperiodic schemes for CSI-RS transmission. In our view, both periodic/semi-persistent and aperiodic schemes should be supported. The periodic/semi-persistent scheme is beneficial for regular monitoring, while the aperiodic scheme is beneficial for event-triggered monitoring (e.g., based on a change of UE position/speed). Also, if aperiodic CSI-RS transmission is supported, it is important to study triggering mechanisms for aperiodic CSI-RS transmission.

Observation 7: Periodic/semi-persistent CSI-RS transmission is beneficial for regular monitoring, while aperiodic CSI-RS transmission is beneficial for event-triggered monitoring (e.g., based on change of UE position/speed).

Proposal 7: Study both periodic/semi-persistent and aperiodic SL CSI-RS transmissions for beam maintenance.

Proposal 8: If aperiodic CSI-RS transmission is supported, study triggering mechanisms for aperiodic CSI-RS transmission.

### Beam Reporting in Sidelink Beam Maintenance

In the RAN1 #113 meeting [3], the following agreements were reached for beam reporting using PSFCH and SL MAC CE in beam maintenance.

Agreement For beam reporting using PSFCH (if supported) in beam maintenance, study - Whether/how there is an association rule between PSFCH for beam reporting and sidelink CSI-RS (either standalone or non-standalone) - FFS beam reporting content, - Whether/how PSFCH carries multiple beam reporting bits - Alt 1: New PSFCH format - Alt 2: PSFCH format 0 by exploring the relationship with frequency and/or code domain resources - Other alternatives are not precluded - FFS: PSFCH carries one beam reporting bit - FFS: whether beam reporting and sidelink HARQ ACK can be reported together, e.g., same or different PSFCH Agreement For beam reporting using sidelink MAC CE (if supported) in beam maintenance, study - beam reporting content - timeline, e.g., - whether/how to reuse Rel-16 sidelink CSI reporting window as baseline for the association between sidelink beam reporting and sidelink CSI-RS resources. - Periodic, aperiodic and/or semi-persistent reporting timeline

In Rel-16/17 NR sidelink, CSI reporting is used for sidelink link adaptation and rank adaptation in sidelink unicast communication. The contents of sidelink CSI reporting are CQI and RI, which are always reported together. The CSI reporting container is MAC CE.

For sidelink beam reporting containers, using PSFCH and SL MAC CE were proposed. Each option has pros and cons as follows:

- Beam reporting using PSFCH enables reporting of beam indication with low latency and low reporting overhead. However, reporting of L1-RSRP (and other L1 measurements) would not be possible due to a small signaling capacity.

- Beam reporting using SL MAC CE enables reporting of beam indication as well as L1-RSRP (and other L1 measurements). However, it would increase the latency and reporting overhead.

We propose to support both beam reporting using PSFCH and beam reporting using SL MAC CE for sidelink beam maintenance.

Observation 8: Beam reporting using PSFCH is useful for reporting only beam indication (e.g., CRI) with low latency and low reporting overhead, with the cost of lacking L1 measurement(s) reporting.

Observation 9: Beam reporting using SL MAC CE is useful for reporting more contents such as beam indication (e.g., CRI) and L1-RSRP (and other L1 measurements), with the cost of increased latency and reporting overhead.

Proposal 9: Support both beam reporting using PSFCH and beam reporting using SL MAC CE for sidelink beam maintenance.

Regarding beam reporting using PSFCH, one open issue is whether/how there is an association rule between PSFCH for beam reporting and sidelink CSI-RS (either standalone or non-standalone). It is beneficial to have an association rule between PSFCH for beam reporting and sidelink CSI-RS to reduce latency and signaling overhead for beam reporting. Therefore, we propose to study an association rule between PSFCH for beam reporting and sidelink CSI-RS.

Observation 10: For beam reporting using PSFCH in beam maintenance, it is beneficial to have an association rule between PSFCH for beam reporting and sidelink CSI-RS to reduce latency and signaling overhead for beam reporting.

Proposal 10: For beam reporting using PSFCH in beam maintenance, RAN1 to study an association rule between PSFCH for beam reporting and sidelink CSI-RS.

Regarding beam reporting using SL MAC CE, one open issue is the beam reporting content. Reporting at least beam indication (e.g., CRI) and L1-RSRP is beneficial to assess the quality of each Tx beam at Tx UE. Based on this information, Tx UE can determine a set of better Tx beam candidates for sidelink beam maintenance. Therefore, we propose to consider at least beam indication (e.g., CRI) and L1-RSRP for beam reporting in sidelink beam maintenance.

Observation 11: For beam reporting using sidelink MAC CE in beam maintenance, reporting beam indication (e.g., CRI) and L1-RSRP is beneficial to assess the quality of each Tx beam at Tx UE, which enables Tx UE to determine a set of better Tx beam candidates for beam maintenance.

Proposal 11: For beam reporting using sidelink MAC CE in beam maintenance, RAN1 to consider at least beam indication (e.g., CRI) and L1-RSRP as beam reporting content.

### Sidelink Beam Indication and Switching in Sidelink Beam Maintenance

In the RAN1 #113 meeting [3], the following agreement was also reached for sidelink beam indication and switching schemes in sidelink beam maintenance.

Agreement For beam indication in sidelink beam maintenance, study sidelink TCI state mechanism: - Details of sidelink TCI state at least including/indicating e.g. sidelink TCI state ID, sidelink CSI-RS resource and Tx/Rx spatial filter related information - FFS: QCL types for sidelink - FFS: whether PSCCH and associated PSSCH always have the same TCI state - FFS: whether the beam indication is via sidelink CSI-RS resource - Study the following optional beam indication candidate container - SCI - sidelink MAC CE - PC5-RRC - FFS: activation time of indicated beam - FFS: whether to reuse the Rel-17 unified TCI framework - Study whether/how beam indication is sent by transmitter UE or receiver UE. - FFS study beam indication in mode 1 on Uu interface

Regarding sidelink beam indication and switching, one open question is the timing relationship between sidelink beam indication and beam switching. This is because Rx UE needs a certain time to decode the Tx beam indication and apply an appropriate Rx beam. In NR Uu, the UE reports to the gNB its capability on the minimum number of OFDM symbols required to perform PDCCH/DCI reception and apply the spatial QCL information received in DCI, via timeDurationForQCL. A similar aspect should be considered for SL beam management.

Proposal 12: Study the timing relationship between sidelink beam indication and beam switching by taking into account the signal processing time and beam switching time at Rx UE.

Another open question is the relationship of Tx beams (TCI/QCL relationship) between the PSCCH and corresponding PSSCH in a slot. In Rel-16/17 NR sidelink packet structures, PSCCH and the corresponding PSSCH are multiplexed in the FDM+TDM manner in the same slot. So, there is not enough time for Rx UE to switch Rx beams between the PSCCH reception and the corresponding PSSCH reception. Therefore, if we reuse the Rel-16/17 NR sidelink packet structures, the same Tx beam needs to be used for transmission of the PSCCH and corresponding PSSCH. Also, this suggests that the Tx beam of PSCCH and corresponding PSSCH needs to be indicated in advance so that Rx UE can have enough time to decode the Tx beam indication and apply an appropriate Rx beam.

Further, Rel-16/17 NR sidelink supports SPS transmission and one-shot transmission, as well as retransmissions based on HARQ ACK/NACK feedback for unicast. This leads to another open question on when and how Tx UE indicates Tx beam(s) for the current PSCCH/PSSCH and subsequent PSCCH/PSSCH for the same TB and different TB (i.e., initial transmission and retransmissions for the same TB and different TB).

Observation 12: Since PSCCH and corresponding PSSCH are multiplexed in the FDM+TDM manner in the same slot, there is not enough time for Rx UE to switch the Rx beam between PSCCH and the corresponding PSSCH.

Proposal 13: If the Rel-16/17 NR sidelink packet structures are reused, Tx UE uses the same Tx beam for PSCCH and corresponding PSSCH.

Proposal 14: The Tx beam of PSCCH and corresponding PSSCH is indicated in advance so that Rx UE can have enough time to decode the Tx beam indication and apply an appropriate Rx beam.

Proposal 15: Study when and how Tx UE indicates Tx beam(s) for the current PSCCH/PSSCH and subsequent PSCCH/PSSCH for the same TB and different TB (i.e., initial transmission and retransmissions for the same TB and different TB).

## Sidelink Beam Failure Recovery

### Sidelink Beam Failure Detection

In the RAN1 #113 meeting [3], the following agreement was reached for sidelink beam failure instance (BFI) and beam failure detection (BFD).

Agreement RAN1 can study the following two schemes to trigger sidelink beam failure instance (BFI) that PHY layer provides to MAC layer. - Scheme 1: Sidelink BFI is triggered based on PSFCH carrying sidelink HARQ feedback - Note: this scheme follows the principle of sidelink RLF. - FFS any other enhancements - FFS whether/how to support candidate beam identification in case of BFD - FFS criteria of triggering sidelink BFI - FFS whether there is RAN1 impact - Scheme 2: Sidelink BFI is triggered based on the measurement of reference signal for BFD - Note: this scheme follows the principle of Uu BFR. - FFS any other enhancements - Consider the following reference signals for BFD - Periodic and/or semi-persistent sidelink CSI-RS - S-SSB or its modified format - FFS criteria of triggering of sidelink BFI - FFS whether/how to trigger the transmission of reference signal for BFD - Other options are not precluded.

Rel-16/17 NR sidelink supports HARQ-based sidelink RLF detection. HARQ-based sidelink RLF is detected if the number of consecutive instances that PSFCH reception is absent on the PSFCH reception occasion reaches a threshold.

In NR Uu, beam failure is detected by counting the number of BFIs, where a BFI is indicated from the PHY layer to the MAC layer when an L1 measurement (e.g., L1-RSRP) reaches a threshold. If the number of consecutive BFIs exceeds a threshold, it triggers a beam failure recovery (BFR) procedure.

In the RAN1 #113 meeting, for sidelink BFI, it was agreed to study Scheme 1 and Scheme 2 as shown above. Scheme 1 is based on the principle of sidelink RLF, in which BFI is triggered at Tx UE. On the other hand, Scheme 2 is based on the principle of NR Uu BFR, in which BFI is triggered at Rx UE. In our view, both schemes are complementary. Therefore, we propose to support both Schemes 1 and 2.

Observation 13: For sidelink BFI, Scheme 1 (based on PSFCH carrying sidelink HARQ feedback) and Scheme 2 (based on measurement of reference signal for BFD) are complementary. This is because sidelink BFI is triggered at Tx UE in Scheme 1, whereas sidelink BFI is triggered at Rx UE in Scheme 2.

Proposal 16: For sidelink BFI, support both Scheme 1 (based on PSFCH carrying sidelink HARQ feedback) and Scheme 2 (based on measurement of reference signal for BFD). Other options are not precluded.

Regarding Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), one open issue is the criteria of triggering sidelink BFI. Similar to Rel-16/17 NR sidelink RLF detection, we can consider consecutive instances in the PHY layer that PSFCH reception is absent on the PSFCH reception occasion. However, if only absence of PSFCH reception is considered, beam failure is the same as RLF. It would be better to make use of other information to get early indication for beam failure. So, in addition to monitoring absent PSFCH reception, we can consider the number of HARQ retransmission attempts. For example, the HARQ process indicates a BFI when the number of HARQ retransmission attempts reaches a threshold.

For example, we can consider the following procedure:

- If the PHY layer experiences that PSFCH reception is absent on the PSFCH reception occasion, it indicates a BFI to the MAC layer. If the number of HARQ retransmission attempts reaches a threshold, the HARQ process indicates a BFI to the MAC layer.

- Upon reception of BFI indication by the MAC layer, either from the PHY layer or from the HARQ process, the MAC layer starts a timer and increases the BFI count by one for every BFI.

- If the BFI count exceeds a threshold prior to expiry of the timer, beam failure is declared. If the timer expires, the BFI count is reset to zero.

Therefore, we have the following proposal.

Proposal 17: For sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), sidelink BFI is triggered at Tx UE when PSFCH/HARQ feedback reception is absent on the PSFCH reception occasion and/or when the number of HARQ retransmission attempts reaches a threshold.

Regarding Scheme 2 (based on the measurement of reference signal for BFD), one open issue is the criteria of triggering sidelink BFI. Similar to NR Uu criteria, we can consider a L1 metric (e.g., L1-RSRP, hypothetical BLER) based on the measurement of BFD reference signals for triggering sidelink BFI. In addition, we can consider issued HARQ NACKs at Rx UE for triggering sidelink BFI. Taking into account issued HARQ NACKs is beneficial when BFD reference signals may not be always transmitted. It should be noted that the HARQ process in Rx UE issues NACKs even if the HARQ feedback cannot be sent due to some reasons (e.g., half duplex, beam conflict).

For example, we can consider the following procedure:

- If the PHY layer experiences a degraded L1 metric, it indicates a BFI to the MAC layer. If a HARQ retransmission is needed, i.e., a NACK is issued, the HARQ process indicates a BFI to the MAC layer.

- Upon reception of BFI indication by the MAC layer, either from the PHY layer or from the HARQ process, the MAC layer starts a timer and increases the BFI count by one for every BFI.

- If the BFI count exceeds a threshold prior to expiry of the timer, beam failure is declared. If the timer expires, the BFI count is reset to zero.

Therefore, we have the following proposal.

Observation 14: For sidelink BFI Scheme 2 (based on the measurement of reference signal for BFD), taking into account issued HARQ NACKs is beneficial when BFD reference signals may not be always transmitted.

Proposal 18: For sidelink BFI Scheme 2 (based on the measurement of reference signal for BFD), sidelink BFI is triggered at Rx UE when a L1 metric (e.g., L1-RSRP, hypothetical BLER) reaches a threshold and/or based on the number of issued NACKs.

### Candidate Beam Determination in Sidelink Beam Failure Recovery

Regarding sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), one open issue is how to support candidate beam determination (CBD) in case of BFD. In sidelink BFI Scheme 1, Tx UE detects beam failure and thus trigger a beam failure recovery procedure. On the other hand, Rx UE may not know that the beam failure recovery procedure is triggered by Tx UE. So, Tx UE needs to send an indication/signal to Rx UE to inform that the beam failure recovery procedure is triggered. In addition, CBD reference signals should be transmitted upon beam failure detection to reduce the resource overhead. Taking into account these aspects, we can consider the following procedure for CBD in sidelink BFI Scheme 1, where beam correspondence is assumed.

- Upon beam failure detection by Tx UE, Tx UE sends a transmission request of CBD reference signals (e.g., using a wide beam) to Rx UE.

- Based on the received request, Rx UE transmits CBD reference signals to Tx UE.

- Tx UE identifies a new candidate beam from the measurement of CBD reference signals.

- Tx UE sends the candidate beam information to the Rx UE.

Therefore, we have the following proposal:

Observation 15: In sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), Tx UE detects beam failure and thus trigger a beam failure recovery procedure. On the other hand, Rx UE may not know that the beam failure recovery procedure is triggered by Tx UE. So, Tx UE needs to send an indication/signal to Rx UE to inform that the beam failure recovery procedure is triggered.

Observation 16: For beam failure recovery, CBD reference signals should be transmitted upon beam failure detection to reduce the resource overhead.

Proposal 19: For sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), RAN1 to study solutions for candidate beam determination (e.g., upon beam failure detection by Tx UE, Tx UE sends a transmission request of CBD reference signals to Rx UE, and based on the received request, Rx UE transmits CBD reference signals to Tx UE.).

### Sidelink Beam Failure Recovery Request (BFRQ)

In the RAN1 #113 meeting [3], the following agreement was reached for sidelink beam failure recovery request (BFRQ) for the scheme where sidelink BFI is triggered on measurement of BFD reference signal.

Agreement At least for the scheme where sidelink BFI is triggered on measurement of BFD reference signal (if supported), for sidelink BFRQ, study - Resources - Option 1: associated with CBD reference signal, i.e. identified candidate beam - Option 2: within a responding window after CBD reference signal transmission - Option 3: Resources determined based on sensing and resource allocated after identification of a new beam. - Option 4: associated with BFD reference signal - Other options are not precluded - Container - Option 1: PSFCH or modified PSFCH - Option 2: SCI - Option 3: sidelink MAC CE - Option 4: new channel associated with beam reporting in initial beam pairing phase - Other options are not precluded - Transmit beam - Option 1: widest attainable beam - FFS up to UE implementation - Option 2: beam sweeping - Option 3: corresponding to the receive beam of identified candidate beam - Other options are not precluded - Content can be the BFD and/or new beam indication - FFS: whether/how to support sidelink BFRQ in mode 1 including signaling to/from the network.

Regarding the resources and container of sidelink BFRQ, using PSFCH resources associated with CBD reference signal corresponding to the identified candidate beam is beneficial to reduce latency and signaling overhead.

Regarding the Tx beam of sidelink BFRQ, a suitable Tx beam depends on the assumption of beam correspondence. If beam correspondence is not assumed, the widest attainable beam should be used because a suitable Tx beam is not identified yet. If beam correspondence is assumed, the Tx beam corresponding to the Rx beam of the identified candidate beam should be used. Therefore, we propose the following options.

Observation 17: For sidelink BFRQ for the scheme where sidelink BFI is triggered on measurement of BFD reference signal:
  - Resources: Option 1 (associated with CBD reference signal, i.e., identified candidate beam) is beneficial to reduce latency and signaling overhead. 
  - Container: Option 1 (PSFCH or modified PSFCH) is beneficial to reduce latency and signaling overhead.
  - Transmit beam: A suitable Tx beam depends on the assumption of beam correspondence. If beam correspondence is not assumed, Option 1 (widest attainable beam) should be used. If beam correspondence is assumed, Option 3 (corresponding to the receive beam of identified candidate beam) should be used.

Proposal 20: For sidelink BFRQ for the scheme where sidelink BFI is triggered on measurement of BFD reference signal, RAN1 to support the following options:
  - Resources: Option 1 (associated with CBD reference signal, i.e., identified candidate beam)
  - Container: Option 1 (PSFCH or modified PSFCH)
  - Transmit beam: If beam correspondence is not assumed, Option 1 (widest attainable beam) is used. If beam correspondence is assumed, Option 3 (corresponding to the receive beam of identified candidate beam) is used.

### Sidelink Beam Failure Recovery Response (BFRR)

In the RAN1 #113 meeting [3], the following agreement was reached for sidelink beam failure recovery response (BFRR) for the scheme where sidelink BFI is triggered on measurement of BFD reference signal.

Agreement At least for the scheme where sidelink BFI is triggered on measurement of BFD reference signal (if supported), for sidelink BFRR, study - Container - Option 1: SCI - Option 2: sidelink MAC CE - Option 3: PSFCH - Option 4: PC5-RRC - Other options are not precluded - Transmit beam - Option 1: widest attainable beam - FFS up to UE implementation - Option 2: beam sweeping - Option 3: corresponding to the candidate beam indicated by the received BFRQ - Other options are not precluded - FFS: new beam indication (e.g., SL CRI, SL SSBRI) - FFS: activation time of new beam - FFS: whether/how to support sidelink BFRR in mode 1 including signaling to/from the network

For sidelink BFRR, a suitable container depends on the container of sidelink BFRQ. For example, if the container of sidelink BFRQ is PSFCH or modified PSFCH, it is reasonable to use SCI. On the other hand, if the container of sidelink BFRQ is SCI or SL MAC CE, it is beneficial to use PSFCH for low latency and low signaling overhead.

For the Tx beam of sidelink BFRR, the Tx beam indicated by the received BFRQ should be used because the Tx beam is already identified. Regarding the other options (i.e., widest attainable beam and beam sweeping), the necessity and benefit are not clear. Therefore, we propose the following.

Observation 18: For sidelink BFRR for the scheme where sidelink BFI is triggered on measurement of BFD reference signal:
  - Container: A suitable container depends on the container of BFRQ. For example, if the container of sidelink BFRQ is PSFCH or modified PSFCH, Option 1(SCI) should be used. If the container of sidelink BFRQ is SCI or SL MAC CE, Option 3 (PSFCH) should be used for low latency and low signaling overhead.
  - Transmit beam: The Tx beam indicated by the received BFRQ should be used because the Tx beam is already identified.

Proposal 21: For sidelink BFRR for the scheme where sidelink BFI is triggered on measurement of BFD reference signal, RAN1 to support the following options: 
  - Container: If the container of sidelink BFRQ is PSFCH or modified PSFCH, Option 1 (SCI) is used. If the container of sidelink BFRQ is SCI or SL MAC CE, Option 3 (PSFCH) is used.
  - Transmit beam: Option 3 (corresponding to the candidate beam indicated by the received BFRQ).

### Fallback to Wide Beams for Sidelink Beam Failure Recovery

Beam failure can occur due to blockage by surrounding objects (e.g., vehicles, building, and other objects), beam misalignment caused by UE mobility, etc. As shown in Figure 1, one way to mitigate this issue is to use a coordinated beam fallback mechanism, where Tx UE and/or Rx UE fall back to wide beams to keep FR2 sidelink commination when beam failure occurs. For example, after Tx UE and Rx UE establish a beam-based unicast link, Tx UE and Rx UE agree on which resources Rx UE will apply a wide Rx beam for beam fallback. Then, when Tx UE detects beam failure, Tx UE use pre-agreed resources on which Rx UE will apply a wide Rx beam to recover from beam failure. Therefore, we propose to study fallback mechanisms from a narrow beam to a wide beam.

Figure 1. Examples of beam fallback from narrow beams to wide beams when beam failure occurs.

Observation 19: When beam failure occurs due to beam misalignment or blockage, falling back from a narrow beam to a wide beam is beneficial to recover from beam failure.

Proposal 22: For sidelink beam failure recover, RAN1 to study fallback mechanisms from a narrow beam to a wide beam, e.g., when Tx UE detects beam failure, Tx UE use pre-agreed resources on which Rx UE will apply a wide Rx beam.

Regarding how to agree on the resources on which Rx UE will apply a wide Rx beam, we can reuse and enhance Rel-17 NR sidelink DRX feature. In Rel-17 sidelink DRX for unicast, Rx UE may send the sidelink DRX assistance information to Tx UE to assist the determination of the sidelink DRX configuration. Then, Tx UE sends the sidelink DRX configuration to Rx UE, and Rx UE may accept or reject the sidelink DRX configuration via PC5-RRC signaling. For a coordinated beam fallback mechanism, we can reuse and enhance Rel-17 NR sidelink DRX procedure so that Tx UE and Rx UE agree on the resources on which Rx UE will apply a wide Rx beam.

Observation 20: Rel-17 NR sidelink DRX procedure can be reused and enhanced to configure resources on which Rx UE will apply a wide Rx beam.

Proposal 23: For the coordinated beam fallback, RAN1 to reuse and enhance Rel-17 NR sidelink DRX procedure so that Tx UE and Rx UE agree on which resources Rx UE will apply a wide Rx beam.

## PSFCH Tx/Rx Beam Determination

In the RAN1 #112bis-e meeting [4], the following agreement was reached for PSFCH Tx/Rx beam determination.

Agreement Consider the following two options for determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot. - Option 1: PSFCH transmit beam is derived from the corresponding PSCCH/PSSCH receive beam; PSFCH receive beam is derived from the corresponding PSCCH/PSSCH transmit beam. - Note this is based on sidelink beam correspondence - Option 2: PSFCH transmit beam is derived from the PSCCH/PSSCH transmit beam for reverse data transmission; PSFCH receive beam is derived from the PSCCH/PSSCH receive beam for reverse data transmission. - Note this is based on beam training for reverse data transmission - Note: The PSFCH transmit/receive beam can be the same as PSCCH/PSSCH transmit/receive beam used for the reverse data transmission. - FFS: support both options or down select one option.

Also, in the RAN1 #113 meeting [3], the following was proposed by FL but not agreed.

FL proposal in RAN1 #113 Proposal 4-1-a: For determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot, down-select one of the two alternatives - Alt 1: Only Option 1 is supported - Alt 2: Both Option 1 and Option 2 are supported - FFS how to select between Option 1 and Option 2 - Note: - Option 1: PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam. - Option 2: PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission.

We prefer Alt 2 (both Option 1 and Option 2 are supported) to support both cases that beam correspondence is assumed and not assumed. More specifically, if beam correspondence is assumed, Option 1 (PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam) should be used. On the other hand, if beam correspondence is not assumed, Option 2 (PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission) should be used. Therefore, we have the following proposal.

Observation 21: For determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot, if beam correspondence is assumed, Option 1 (PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam) should be used. On the other hand, if beam correspondence is not assumed, Option 2 (PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission) should be used.

Proposal 24: For determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot, both Option 1 (PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam) and Option 2 (PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission) are supported (i.e., Alt 2).

## Beam Conflict

In the RAN1 #112bis-e meeting [4], the following conclusion was made regarding simultaneous transmission and reception using different beams.

Conclusion RAN1 to prioritize the case where a UE is incapable of simultaneous transmitting or receiving PSCCH/PSSCH/PSFCH using different beams in Rel-18.

In addition, the FL summary Clause 3.5.1.1 in RAN1 #112bis-e [5] described the issues of PSFCH reception/transmission beam conflict and PSCCH/PSSCH reception beam conflict.

In NR sidelink, the simultaneous transmission or reception of multiple PSFCHs is possible. In legacy NR sidelink, these transmissions or receptions are in omni-directional beam. However, in SL-FR2, the PSFCH transmit or receive beam is separately trained for each sidelink unicast session (cf. Section 3.4). If a UE supports multiple sidelink unicast sessions, the trained PSFCH beams for pair UEs in different sidelink unicast sessions may be different. Similarly, a UE may have simultaneous PSCCH/PSSCH receptions. If a UE supports multiple sidelink unicast sessions, the trained PSCCH/PSSCH reception beams for pair UEs in different sidelink unicast sessions may be different. On the other hand, in practice, UE’s capability may be limited so that UE can only support simultaneous transmissions/receptions based on a single beam. … Under the assumption that a UE is incapable of making simultaneous transmissions or simultaneous receptions using different beams, the issue of PSFCH transmit/receive beams conflict or PSCCH/PSSCH receive beams conflict should be addressed.

For UE incapable of simultaneous transmission and reception using different beams, Figure 2 depicts examples of PSFCH reception/transmission beam conflict when UE is involved with multiple unicast links.

- Figure 2(a) shows an example of PSFCH reception beam conflict. UE 1 is supposed to receive multiple PSFCH transmissions from UE 2, UE 3, and UE 4 in the same slot. However, since UE 1 cannot use different Rx beams for PSFCH receptions simultaneously, PSFCH reception beam conflict occurs at UE 1.

- Figure 2(a) shows an example of PSFCH transmission beam conflict. UE 1 is supposed to transmit multiple PSFCH transmissions to UE 2, UE 3, and UE 4 in the same slot. However, since UE 1 cannot use different Tx beams for PSFCH transmissions simultaneously, PSFCH transmission beam conflict occurs at UE 1.

Figure 2. Examples of PSFCH Tx/Rx beam conflict.

We propose to study solution(s) to avoid/mitigate the beam conflict issues. For example, we can consider the following approaches:

- Enhancing resource (re-)selection (e.g., resource (re-)selection taking into account the single beam constraint)

- Enhancing PSSCH-PSFCH resource mapping (e.g., introducing multiple PSFCH slot occasions, flexible/dynamic PSFCH resource indication)

- Enhancing inter-UE coordination

- Re-evaluation of resources and beams

- Using a single common beam (e.g., wide beam) for multiple links

Observation 22: For UE incapable of simultaneous transmitting or receiving using different beams, beam conflict issues would occur.

Proposal 25: Study solution(s) to avoid/mitigate the beam conflict issues (e.g., enhancing resource (re-)selection, enhancing PSSCH-PSFCH resource mapping, enhancing inter-UE coordination, re-evaluation of resources and beams, using a single common beam for multiple links).

## Sensing, Resource Selection, and Resource Reservation

The use of directional beams would affect sensing, resource selection, and resource reservation. This is because Rel-16/17 NR sidelink in FR1 bands assumes omni-directional transmission and reception and thus may not be suitable for beam-based sidelink communication in FR2 bands. For example, if UEs use directional beams, they can transmit and receive signals in only limited directions. It would affect sensing and SCI transmission/reception because successful SCI reception depends on whether the Tx beam and Rx beam are properly aligned or not. As shown in Figure 3, if Tx/Rx beams are not aligned among Tx UE transmitting resource reservation/SCI and Rx UEs, Rx UEs miss resource reservation/SCI from Tx UE, and the existing Rel-16/17 sidelink sensing mechanism would not work properly. This leads to inappropriate resource selection.

Figure 3. SCI transmission/reception with directional beams: when Tx UE transmits SCI, Rx UE 1 successfully receives the SCI as the beams of Tx UE and Rx UE 1 are aligned, while Rx UE 2 fails to receive the SCI due to beam misalignment between Tx UE and Rx UE 2.

Observation 23: Rel-16/17 NR sidelink resource selection and reservation assume omni-directional transmission and reception and may not be suitable for beam-based sidelink communication in FR2 bands.

Observation 24: If directional Tx/Rx beams are used, Rx UEs may miss resource reservation/SCI from other Tx UEs, and the existing Rel-16/17 sidelink sensing mechanism would not work properly. This leads to inappropriate resource selection.

In addition, the directionality of beam-based communication can be utilized to improve the spatial reuse of time/frequency resources. For example, as shown in Figure 4, the two beam links (Tx UE 1 – Rx UE 1 and Tx UE 2 – Rx UE 2) do not interfere with each other even though they are close to each other, while Tx UE 3 should avoid transmission as its Tx beam will cause interference to Rx UE 1. This would suggest that resource (re-)selection should take into account Tx/Rx beams for better spatial reuse. Therefore, appropriate sensing, resource selection, and resource reservation/SCI mechanisms tailored for beam-based sidelink communication should be considered.

Figure 4. Spatial reuse of time/frequency resources by utilizing directional communication, where the two beam links (Tx UE 1 – Rx UE 1 and Tx UE 2 – Rx UE 2) do not interfere with each other even though they are close to each other, while Tx UE 3 should avoid transmission as its Tx beam will cause interference to Rx UE 1.

Observation 25: The directionality of beam-based communication can be utilized to improve the spatial reuse of time/frequency resources.

Proposal 26: Study enhancements of sensing, resource selection, and resource reservation/SCI to take into account directional Tx/Rx beams and UE mobility.

# Conclusion

In this contribution, we discussed aspects that need to be considered for NR sidelink beam management on the FR2 licensed spectrum. Our observations and proposals are summarized as follows:

Observation 1: Initial beam pairing before unicast link establishment has an issue of unnecessary beam pairing because each UE needs to perform initial beam pairing with every surrounding UE regardless of whether they perform unicast communication or not after initial beam pairing. This leads to a waste of resources and power consumption. This is problematic particularly when there are a lot of UEs in proximity.

Observation 2: Initial beam pairing after unicast link establishment has an issue of a limited communication range because paired beams cannot be used for unicast link establishment. Therefore, this procedure is not suitable for use cases that require a long communication range, e.g., V2X. It is not clear how to establish unicast links without initial beam pairing.

Observation 3: Initial beam pairing during unicast link establishment can mitigate the issues of unnecessary beam pairing and limited communication range.

Observation 4: Standalone SL CSI-RS transmission for sidelink beam maintenance is useful to search a greater number of beams within a slot.

Observation 5: For sidelink beam maintenance, sidelink CSI-RS transmissions with repetition on transmit beams within a slot is beneficial for fast Rx beam training within the slot. In addition, multiple sidelink CSI-RS transmissions of different Tx beams without repetition on transmit beams within a slot is beneficial for fast Tx beam training within the slot.

Observation 6: For sidelink beam maintenance, Tx beam sweeping within a slot may cause an AGC issue at surrounding Rx UEs.

Observation 7: Periodic/semi-persistent CSI-RS transmission is beneficial for regular monitoring, while aperiodic CSI-RS transmission is beneficial for event-triggered monitoring (e.g., based on change of UE position/speed).

Observation 8: Beam reporting using PSFCH is useful for reporting only beam indication (e.g., CRI) with low latency and low reporting overhead, with the cost of lacking L1 measurement(s) reporting.

Observation 9: Beam reporting using SL MAC CE is useful for reporting more contents such as beam indication (e.g., CRI) and L1-RSRP (and other L1 measurements), with the cost of increased latency and reporting overhead.

Observation 10: For beam reporting using PSFCH in beam maintenance, it is beneficial to have an association rule between PSFCH for beam reporting and sidelink CSI-RS to reduce latency and signaling overhead for beam reporting.

Observation 11: For beam reporting using sidelink MAC CE in beam maintenance, reporting beam indication (e.g., CRI) and L1-RSRP is beneficial to assess the quality of each Tx beam at Tx UE, which enables Tx UE to determine a set of better Tx beam candidates for beam maintenance.

Observation 12: Since PSCCH and corresponding PSSCH are multiplexed in the FDM+TDM manner in the same slot, there is not enough time for Rx UE to switch the Rx beam between PSCCH and the corresponding PSSCH.

Observation 13: For sidelink BFI, Scheme 1 (based on PSFCH carrying sidelink HARQ feedback) and Scheme 2 (based on measurement of reference signal for BFD) are complementary. This is because sidelink BFI is triggered at Tx UE in Scheme 1, whereas sidelink BFI is triggered at Rx UE in Scheme 2.

Observation 14: For sidelink BFI Scheme 2 (based on the measurement of reference signal for BFD), taking into account issued HARQ NACKs is beneficial when BFD reference signals may not be always transmitted.

Observation 15: In sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), Tx UE detects beam failure and thus trigger a beam failure recovery procedure. On the other hand, Rx UE may not know that the beam failure recovery procedure is triggered by Tx UE. So, Tx UE needs to send an indication/signal to Rx UE to inform that the beam failure recovery procedure is triggered.

Observation 16: For beam failure recovery, CBD reference signals should be transmitted upon beam failure detection to reduce the resource overhead.

Observation 17: For sidelink BFRQ for the scheme where sidelink BFI is triggered on measurement of BFD reference signal:
  - Resources: Option 1 (associated with CBD reference signal, i.e., identified candidate beam) is beneficial to reduce latency and signaling overhead. 
  - Container: Option 1 (PSFCH or modified PSFCH) is beneficial to reduce latency and signaling overhead.
  - Transmit beam: A suitable Tx beam depends on the assumption of beam correspondence. If beam correspondence is not assumed, Option 1 (widest attainable beam) should be used. If beam correspondence is assumed, Option 3 (corresponding to the receive beam of identified candidate beam) should be used.

Observation 18: For sidelink BFRR for the scheme where sidelink BFI is triggered on measurement of BFD reference signal:
  - Container: A suitable container depends on the container of BFRQ. For example, if the container of sidelink BFRQ is PSFCH or modified PSFCH, Option 1(SCI) should be used. If the container of sidelink BFRQ is SCI or SL MAC CE, Option 3 (PSFCH) should be used for low latency and low signaling overhead.
  - Transmit beam: The Tx beam indicated by the received BFRQ should be used because the Tx beam is already identified.

Observation 19: When beam failure occurs due to beam misalignment or blockage, falling back from a narrow beam to a wide beam is beneficial to recover from beam failure.

Observation 20: Rel-17 NR sidelink DRX procedure can be reused and enhanced to configure resources on which Rx UE will apply a wide Rx beam.

Observation 21: For determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot, if beam correspondence is assumed, Option 1 (PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam) should be used. On the other hand, if beam correspondence is not assumed, Option 2 (PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission) should be used.

Observation 22: For UE incapable of simultaneous transmitting or receiving using different beams, beam conflict issues would occur.

Observation 23: Rel-16/17 NR sidelink resource selection and reservation assume omni-directional transmission and reception and may not be suitable for beam-based sidelink communication in FR2 bands.

Observation 24: If directional Tx/Rx beams are used, Rx UEs may miss resource reservation/SCI from other Tx UEs, and the existing Rel-16/17 sidelink sensing mechanism would not work properly. This leads to inappropriate resource selection.

Observation 25: The directionality of beam-based communication can be utilized to improve the spatial reuse of time/frequency resources.

Proposal 1: Prioritize the study of initial beam pairing during unicast link establishment over initial beam pairing before/after unicast link establishment.

Proposal 2: For initial beam pairing before unicast link establishment, RAN1 to study solutions to avoid unnecessary beam measurement and reporting from multiple UEs.

Proposal 3: Study standalone SL CSI-RS transmissions for sidelink beam maintenance.

Proposal 4: Study sidelink CSI-RS transmissions with and without repetition on transmit beams within a slot.

Proposal 5: Study multiple sidelink CSI-RS transmissions of different Tx beams without repetition on transmit beams within a slot.

Proposal 6: Study solution(s) to mitigate the AGC issue due to Tx beam sweeping within a slot.

Proposal 7: Study both periodic/semi-persistent and aperiodic SL CSI-RS transmissions for beam maintenance.

Proposal 8: If aperiodic CSI-RS transmission is supported, study triggering mechanisms for aperiodic CSI-RS transmission.

Proposal 9: Support both beam reporting using PSFCH and beam reporting using SL MAC CE for sidelink beam maintenance.

Proposal 10: For beam reporting using PSFCH in beam maintenance, RAN1 to study an association rule between PSFCH for beam reporting and sidelink CSI-RS.

Proposal 11: For beam reporting using sidelink MAC CE in beam maintenance, RAN1 to consider at least beam indication (e.g., CRI) and L1-RSRP as beam reporting content.

Proposal 12: Study the timing relationship between sidelink beam indication and beam switching by taking into account the signal processing time and beam switching time at Rx UE.

Proposal 13: If the Rel-16/17 NR sidelink packet structures are reused, Tx UE uses the same Tx beam for PSCCH and corresponding PSSCH.

Proposal 14: The Tx beam of PSCCH and corresponding PSSCH is indicated in advance so that Rx UE can have enough time to decode the Tx beam indication and apply an appropriate Rx beam.

Proposal 15: Study when and how Tx UE indicates Tx beam(s) for the current PSCCH/PSSCH and subsequent PSCCH/PSSCH for the same TB and different TB (i.e., initial transmission and retransmissions for the same TB and different TB).

Proposal 16: For sidelink BFI, support both Scheme 1 (based on PSFCH carrying sidelink HARQ feedback) and Scheme 2 (based on measurement of reference signal for BFD). Other options are not precluded.

Proposal 17: For sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), sidelink BFI is triggered at Tx UE when PSFCH/HARQ feedback reception is absent on the PSFCH reception occasion and/or when the number of HARQ retransmission attempts reaches a threshold.

Proposal 18: For sidelink BFI Scheme 2 (based on the measurement of reference signal for BFD), sidelink BFI is triggered at Rx UE when a L1 metric (e.g., L1-RSRP, hypothetical BLER) reaches a threshold and/or based on the number of issued NACKs.

Proposal 19: For sidelink BFI Scheme 1 (based on PSFCH carrying sidelink HARQ feedback), RAN1 to study solutions for candidate beam determination (e.g., upon beam failure detection by Tx UE, Tx UE sends a transmission request of CBD reference signals to Rx UE, and based on the received request, Rx UE transmits CBD reference signals to Tx UE.).

Proposal 20: For sidelink BFRQ for the scheme where sidelink BFI is triggered on measurement of BFD reference signal, RAN1 to support the following options:
  - Resources: Option 1 (associated with CBD reference signal, i.e., identified candidate beam)
  - Container: Option 1 (PSFCH or modified PSFCH)
  - Transmit beam: If beam correspondence is not assumed, Option 1 (widest attainable beam) is used. If beam correspondence is assumed, Option 3 (corresponding to the receive beam of identified candidate beam) is used.

Proposal 21: For sidelink BFRR for the scheme where sidelink BFI is triggered on measurement of BFD reference signal, RAN1 to support the following options: 
  - Container: If the container of sidelink BFRQ is PSFCH or modified PSFCH, Option 1 (SCI) is used. If the container of sidelink BFRQ is SCI or SL MAC CE, Option 3 (PSFCH) is used.
  - Transmit beam: Option 3 (corresponding to the candidate beam indicated by the received BFRQ).

Proposal 22: For sidelink beam failure recover, RAN1 to study fallback mechanisms from a narrow beam to a wide beam, e.g., when Tx UE detects beam failure, Tx UE use pre-agreed resources on which Rx UE will apply a wide Rx beam.

Proposal 23: For the coordinated beam fallback, RAN1 to reuse and enhance Rel-17 NR sidelink DRX procedure so that Tx UE and Rx UE agree on which resources Rx UE will apply a wide Rx beam.

Proposal 24: For determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot, both Option 1 (PSFCH transmit/receive beam is derived from the corresponding PSCCH/PSSCH receive/transmit beam) and Option 2 (PSFCH transmit/receive beam is derived from PSCCH/PSSCH transmit/receive beam for the reverse data transmission) are supported (i.e., Alt 2).

Proposal 25: Study solution(s) to avoid/mitigate the beam conflict issues (e.g., enhancing resource (re-)selection, enhancing PSSCH-PSFCH resource mapping, enhancing inter-UE coordination, re-evaluation of resources and beams, using a single common beam for multiple links).

Proposal 26: Study enhancements of sensing, resource selection, and resource reservation/SCI to take into account directional Tx/Rx beams and UE mobility.

# Reference

- RP-230052, “Meeting Report for TSG RAN meeting #98-e”, RAN #98-e, Dec. 2022.

- RP-222806, “WID revision: NR sidelink evolution”, RAN #98-e, Dec. 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #113, Incheon, Korea, May 2023.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #112bis-e, e-meeting, Apr. 2023.

- R1-2303905, “FL summary #3 for AI 9.4.3 Enhanced sidelink operation on FR2 licensed spectrum”, RAN1 #112bis-e, e-meeting, Apr. 2023.