3GPP TSG RAN WG1 #114                                                       R1-2306869

Toulouse, France, August 21st – August 25th, 2023

Source:	ZTE

Title:	Discussion on UE feature for Rel-18 NR positioning

Agenda item:	9.16.2

Document for:	Discussion and Decision

Introduction

In recent 3GPP TSG RAN WG1 meetings, the enhancements for Rel-18 positioning have been comprehensively discussed in RAN1 aspect. Most enhancements have been specified to improve the performance of NR positioning. As the discussion on positioning in Rel-18 is drawing to the end, we need to define new UE features for the enhancements that are approved in Rel-18 NR positioning.

In this contribution, we provide our further views towards the UE feature on Rel-18 NR positioning to support the agreed enhancement for positioning sub-agendas, including Sidelink positioning, NR DL and UL carrier phase positioning, LPHAP, Bandwidth aggregation for positioning measurements and positioning for RedCap UEs. All the suggestions are based on the consensus made in RAN1#113 meeting as below [1].

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-1 | Common SL PRS Processing Capability [across all resource pools] | FFS: separate rows for [for SL-TDOA DL type/ SL-TDOA UL type /SL-AOA\RTT-type solutions using SL single sided/ RTT-type solutions using SL single sided double-sided] incl SL-PRS resources |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4a | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS scheme 2 in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-5 | SL-PRS congestion control [in a dedicated/shared resource pool] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-7 | SL PRS measurement for [SL-TDOA DL type/ SL-TDOA UL type /SL-AOA\RTT-type solutions using SL single sided/ RTT-type solutions using SL single sided double-sided] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-8 | Support of random selection [in a dedicated resource pool] | FFS: merge with 41-1-4 |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-10 | Support of [full\partial] sensing [in a dedicated resource pool] | FFS: merge with 41-1-4 |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-11 | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-12 | Comb-based multiplexing for SL-PRS [transmission/reception] from different UEs in the same slot |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-13 | Reporting the additional paths for SL positioning |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-14 | LoS/NLoS indicator for SL positioning |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-17 | Open loop SL power control and SL RSRP report [in dedicated resource pool] | FFS: merge with 41-1-4 for SL PC and 41-1-3 for RSRP report |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-19 | UE can provide ARP location for sidelink as assistance data |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-19a | Report of ARP-ID with SL positioning measurements |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-1 | DL RSCP reporting based on DL PRS in RRC_CONNECTED | FFS: separate rows for DL PRS processing capabilities\measurement for CPP measurements in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-1a | DL RSCPD reporting based on DL PRS in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-2 | DL RSCP reporting based on DL PRS in RRC_INACTIVE | FFS: separate rows for DL PRS processing capabilities\measurement for CPP measurements in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-2a | DL RSCPD reporting based on DL PRS in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-3 | Measurement on indicated DL PRS resource sets [within the indicated time window(s) for UE based and UE assisted] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-4 | UE-based Carrier Phase Positioning | Components are 1. Support of carrier phase measurement 2. Support of Assistance data for UE-based Carrier Phase Positioning |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-1 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state for initial UL BWP |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-2 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state configured outside initial UL BWP |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-3 | [Processing capability for/Support of] PRS in RRC_IDLE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-1 | DL PRS processing capabilities for aggregated PRS processing [of 2 PFLs] in intra-band contiguous [within a MG] [for RRC_CONNECTED sate, RRC_INACTIVE state, RRC_IDLE] | FFS: separate row for number of PFLs FFS: total aggregated BW and BW combinations |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-3 | Support of PRS bandwidth aggregation in RRC_CONNECTED — DL-TDOA |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-3a | Support of PRS bandwidth aggregation in RRC_CONNECTED —Multi-RTT |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-4 | Support of PRS bandwidth aggregation in RRC_ INACTIVE — DL-TDOA |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-4a | Support of PRS bandwidth aggregation in RRC_ INACTIVE —Multi-RTT |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-5 | Support of PRS bandwidth aggregation in RRC_IDLE — [DL-TDOA and Multi-RTT] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-6 | Support of positioning SRS bandwidth aggregation in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-7 | Support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-8 | Support of positioning SRS bandwidth aggregation in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-5-1 | Support of PRS with Rx frequency hopping [and measurement report] [within a MG] [in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-5-2 | Support of positioning SRS with Tx frequency hopping [in RRC_CONNECTED/RRC_INACTIVE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability |  |  |  |  |  |  |  |  |  |  |

UE feature on sidelink positioning

In RAN1#113 meeting, the basic FG rows have been agreed on positioning for sidelink positioning as shown in section 1. On top of the agreements, we further provide our views.

Proposal 1: FG 41-1-4a should be common for scheme 1 and scheme 2 for shared resource pool, the prerequisite is legacy scheme 1 or 2.

Proposal 2: For FG 41-1-8, random selection should be only for dedicated resource pool. That’s because the UE behavior is the same as legacy for shared resource pool.

Proposal 3: FG 41-1-10 should be combined with FG 41-4-4c because full sensing should be the basic function of SL-PRS transmission for scheme 2.

Proposal 4: FG 41-1-11 and FG 41-1-12 should be only for reception. For transmission side for a single UE, the UE behavior has no difference regardless of TDMed or comb-based multiplexing of SL-PRS reception from other UEs

Our views on the detailed table can be found as follows where the suggested part is marked in red.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-1-1 | Common SL PRS Processing Capability across all resource pools | - Maximum SL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - SL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of SL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of SL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz | 15-1 | Yes |  | SL PRS reception and processing is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-1-2 | Receiving SL-PRS in a shared resource pool | Support of SL PRS reception in a shared resource pool | 41-1-1 | Yes |  | Receiving SL-PRS in a shared resource pool is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-1-3 | Receiving SL-PRS in a dedicated resource pool | Support of SL PRS reception in a dedicated resource pool | 41-1-1 | Yes |  | Receiving SL-PRS in a dedicated resource pool is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-1-4a | Transmitting SL-PRS scheme 1 and scheme 2 in a shared resource pool | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Support downlink pathloss based open loop power control | 15-2 or 15-3 | Yes |  | Transmitting SL-PRS scheme 1 and scheme 2 in a shared resource pool is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool | - Support of SL-PRS transmission scheme 1 in a dedicated resource pool - Max number of SL-PRS Resources - Support downlink pathloss based open loop power control |  | Yes |  | SL-PRS transmission scheme 1 in a dedicated resource pool is not supported | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-4c | Transmitting SL-PRS scheme 2 in a dedicated resource pool | - Support of SL-PRS transmission scheme 2 in a dedicated resource pool - Max number of SL-PRS Resources - Support downlink pathloss based open loop power control - Support full sensing |  | Yes |  | SL-PRS transmission scheme 2 in a dedicated resource pool is not supported | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-5 | SL-PRS congestion control in a dedicated resource pool | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates |  | Yes |  | SL Positioning congestion control is not supported | Per band |  |  |  | Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signaling |
| 41-1-7 | SL PRS measurement | 1. Support of SL PRS measurement report for SL-TDOA DL type - Maximum number of SL PRS RSTD per pair of UEs. Values = {1, 2, 3, 4} 2. Support of SL PRS measurement report for SL-TDOA UL type - Maximum number of SL PRS RTOA per pair of UEs. Values = {1, 2, 3, 4} 3.Support of SL PRS measurement report for single side SL RTT - Max number of UE Rx–Tx time difference measurements corresponding to a single SL PRS resource with each measurement corresponding to a single SL PRS resource from another UE. Value for component 1: {1,2,3,4} 3.Support of SL PRS measurement report for double side SL RTT - Max number of UE Rx–Tx time difference measurements corresponding to a single SL PRS resource with each measurement corresponding to a single SL PRS resource from another UE. Value for component 1: {1,2,3,4} 4. Support of SL PRS measurement report for SL AoA - Max number of SL AoA measurements on different SL PRS resources from the same UE supported by the UE. Values = {1, 2, 3, 4} 4. Support of SL PRS RSRP and SL PRS RSRPP measurement - Max number of SL PRS RSRP measurements on different SL PRS resources from the same UE supported by the UE. Values = {1, 2, 3, 4, 5, 6, 7, 8} - Max number of SL PRS RSRPP measurements on different SL PRS resources from the same UE supported by the UE. Values = {1, 2, 3, 4, 5, 6, 7, 8} |  |  |  | SL PRS measurement is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-1-8 | Support of random selection in a dedicated resource pool | Support of random selection in a dedicated resource pool | 41-1-4c | Yes |  | Random selection in a dedicated resource pool is not supported | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-11 | TDM-based multiplexing of SL-PRS reception from different UEs in the same slot for a dedicated resource pool | Support TDMed multiplexing of SL-PRS reception from different UEs in the same slot for a dedicated resource pool | 41-1-3 | Yes |  | TDMed multiplexing of SL-PRS reception from different UEs in the same slot for a dedicated resource pool is not supported | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-12 | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot for a dedicated resource pool | Support Comb-based multiplexing of SL-PRS reception from different UEs in the same slot for a dedicated resource pool | 41-1-3 | Yes |  | Comb-based multiplexing of SL-PRS reception from different UEs in the same slot for a dedicated resource pool | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-13 | Reporting the additional paths for SL positioning | 1. Support of additional detected path timing reporting for K additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths |  | No |  | Additional path reporting for SL-PRS measurement is not supported | Per UE |  |  |  | Component 1 candidate values: {4, 6, 8} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 41-1-14 | LoS/NLoS indicator for SL positioning | 1. Support reporting LoS/NLoS indicator type Note: the loS/NLos indicator granularity is per UE pair |  | No |  | LOS/NLOS Indicator is not supported | Per UE |  |  |  | Component 1 candidate values: {hard value, hard+soft value} Need for location server to know if the feature is supported | Optional with capability signaling |
| 41-1-17 | Open loop SL power control and SL RSRP report [in dedicated resource pool] | - Support sidelink pathloss based open loop power control for SL-PRS and RSRP reporting |  | Yes |  | Open loop SL power control and SL RSRP report is not supported | Per band |  |  |  |  | Optional with capability signaling |
| 41-1-19 | UE can provide ARP location for sidelink as assistance data | - Support providing ARP location for sidelink as assistance data |  | No |  | Providing ARP location for sidelink as assistance data is not supported | Per UE |  |  |  | Need for location server to know if the feature is supported | Optional with capability signaling |
| 41-1-19a | Report of ARP-ID with SL positioning measurements | - Report of ARP-ID with SL positioning measurements |  | No |  | Report of ARP-ID with SL positioning measurements is not supported | Per UE |  |  |  | Need for location server to know if the feature is supported | Optional with capability signaling |

UE feature on NR DL and UL carrier phase positioning

For FG 41-2-1 and FG 41-2-2, a new UE processing capability is needed in our view as UE need to additionally abstract the carrier phase on top of RSTD or UE Rx-Tx time difference detection. For PRU assisted positioning procedure, UE may further do double differential operation to mitigate the initial phase error among multiple TRPs.

For FG 41-2-3 and FG 41-2-4, we provide detailed UE feature on measurements on indicated DL PRS resource sets and UE-based carrier phase positioning.

Proposal 5: For FG 41-2-1 and FG 41-2-1a:

- Report the PRS processing time {NCPM, TCPM} for carrier phase measurement in RRC_CONNECTED.

  - Compared with {N, T} for PRS measurement in FG 13-1, more UE processing time is needed for carrier phase measurement. Hence, NCPM ≤ N or TCPM≥ T.

- Report the maximum number of DL PRS resources MCPM that UE can process in a slot for carrier phase measurement.

  - Compared with current PRS measurement, more UE memory/complexity is needed for carrier phase measurement, so less resources may be processed. Hence, MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 13-1.

Proposal 6: For FG 41-2-2 and FG 41-2-2a:

- Report the PRS processing time {NCPM, TCPM} for carrier phase measurement in RRC_INACTIVE.

  - Compared with {N, T} for PRS measurement in FG 13-1, more UE processing time is needed for carrier phase measurement. Hence, NCPM ≤ N or TCPM≥ T.

- Report the maximum number of DL PRS resources MCPM that UE can process in a slot for carrier phase measurement.

  - Compared with current PRS measurement, more UE memory/complexity is needed for carrier phase measurement, so less resources may be processed. Hence, MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 13-1.

Proposal 7: For FG 41-2-3 concerning DL PRS measurement within the indicated time window(s):

- Report the PRS processing capability {N3, TCPM} for DL PRS measurement within the indicated time window(s).

  - N3 is the number of time windows in a period TCPM.

  - Report the maximum duration N4 of each time window.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-2-1 | DL RSCP reporting based on DL PRS in RRC_CONNECTED | - Duration of DL PRS symbols NCPM in units of ms a UE can process every TCPM ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - TCPM : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NCPM : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note1: NCPM ≤ N or TCPM≥ T, where {N, T} is reported by FG 13-1 - Max number of DL PRS resources MCPM that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note2: MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 13-1 Note3: RSCP is reported by UE together with UE Rx–Tx time difference measurement in Multi-RTT positioning. | 13-1, 13-4 | No |  | DL RSCP measurement in RRC_CONNECTED is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-2-1a | DL RSCPD reporting based on DL PRS in RRC_CONNECTED | - Duration of DL PRS symbols NCPM in units of ms a UE can process every TCPM ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - TCPM : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NCPM : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note1: NCPM ≤ N or TCPM≥ T, where {N, T} is reported by FG 13-1 - Max number of DL PRS resources MCPM that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note2: MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 13-1 Note3: RSCPD is reported by UE together with RSTD measurement in DL TDOA | 13-1,13-3 | No |  | DL RSCPD measurement in RRC_CONNECTED is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-2-2 | DL RSCP reporting based on DL PRS in RRC_INACTIVE | - Duration of DL PRS symbols NCPM in units of ms a UE can process every TCPM ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - TCPM : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NCPM : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note1: NCPM ≤ N or TCPM≥ T, where {N, T} is reported by FG 27-6 - Max number of DL PRS resources MCPM that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note2: MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 27-6 Note3: RSCP is reported by UE together with UE Rx–Tx time difference measurement in Multi-RTT positioning. | 27-6, 27-18c | No |  | DL RSCP measurement in RRC inactive state is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-2-2a | DL RSCPD reporting based on DL PRS in RRC_INACTIVE | - Duration of DL PRS symbols NCPM in units of ms a UE can process every TCPM ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - TCPM : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NCPM : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note1: NCPM ≤ N or TCPM≥ T, where {N, T} is reported by FG 27-6 - Max number of DL PRS resources MCPM that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note2: MCPM ≤ M, where M is maximum number PRS resources for PRS measurement reported by FG 27-6 Note3: RSCPD is reported by UE together with RSTD measurement in DL TDOA | 27-6, 27-18a | No |  | DL RSCPD measurement in RRC inactive state is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-2-3 | Measurement on indicated DL PRS resource sets within the indicated time window(s) for UE based and UE assisted | - Maximum number of indicated time window(s) N3 a UE can process in every TCPM ms, which is supported and reported by UE. - TCPM : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N3: {1,2,...,N} FFS: The value of N - Maximum duration of DL PRS symbols N4 in units of ms in each time window. - N4: {1,2,...K} ms FFS: The value of K | 13-1 | No |  | - Measurement on DL PRS within the indicated time window(s) is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-2-4 | UE-based Carrier Phase Positioning | 1. Support of carrier phase measurement 2. Support of Assistance data for UE-based Carrier Phase Positioning | 41-2-1 | No |  | UE-based Carrier Phase Positioning not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |

UE feature on LPHAP

For FG 41-3-3, we provide our views on detailed processing capability for PRS in RRC_DILE as below:

Proposal 8: For FG 41-3-3 concerning the processing capability for PRS in RRC_DILE:

- Report the DL PRS buffer capability in RRC_IDLE state.

- Report the DL PRS processing capability in RRC_IDLE state.

- Report the maximum number of DL PRS resources that UE can process in a slot.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-3-3 | Processing capabilities of PRS in RRC idle | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms (in RRC_IDLE) assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} - Max number of DL PRS resources that UE can process in a slot FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  | Yes |  | Processing capabilities of PRS in RRC idle is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |

UE feature on bandwidth aggregation for positioning measurements

In RAN1#113 meeting, the basic FG rows have been agreed for PRS/SRS bandwidth aggregation as shown in section 1. Here we provide our views for further details.

For FG 41-4-1, we think separate FGs should be introduced for PRS processing in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE modes as UE may have different processing capabilities in different RRC states.  Because UE is able to process 2 or 3 PFLs simultaneously, so larger PRS bandwidth processing is needed and more complexity is caused. Therefore, UE may not be able to process PRS as much as Rel-16/17 which is only for single PF

For FG 41-4-5, we think Multi-RTT method can be supported where UE can detect PRS in RRC idle state and report measurement results in RRC connected state. So this FG can be split into two rows for DL-TDOA and Multi-RTT.

For FG 41-4-6 and 41-4-8, it is sufficient to only report the support number of contiguous carriers as the maximum number of SRS resource sets, resources, resources per slot, etc. per BWP can refer to legacy FG 13-8 and 27-15b.

For FG 41-4-7, the switching time before and after the aggregation SRS transmission should be reported based on the following agreement made in RAN1#113 meeting.

Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed.

Proposal 9: Split FG 41-4-1 into 41-4-1a, 1b and 1c for RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE modes respectively. For each one of 41-4-1a, 1b and 1c

- Report the maximum number of aggregated PFLs F.

  - The maximum aggregated bandwidth can be derived from F and the maximum bandwidth reported by legacy FG 13-1.

- Report the PRS processing time {NAgg, TAgg} for aggregation.

  - Compared with {N, T} for single PFL, more UE processing time is needed for aggregation measurement. Hence, NAgg ≤ N or TAgg≥ T.

- Report the maximum number of DL PRS resources MAgg that UE can process in a slot for PFL aggregation

  - Compared with single PFL measurement, more UE memory/complexity is needed, so less resources may be processed. Hence, MAgg ≤ M, where M is maximum number PRS resources for single PFL measurement reported by FG 13-1

Proposal 10: FG 41-4-5 should be split into two rows for DL-TDOA and Multi-RTT respectively.

Proposal 11: For FG 41-4-6 and 41-4-8, it is sufficient to only report the support number of contiguous carriers as the maximum number of SRS resource sets, resources, resources per slot, etc. per BWP can refer to legacy FG 13-8 and 27-15b

Proposal 12: For FG 41-4-7, the switching time before and after the aggregation SRS transmission should be reported.

Our views on the detailed table can be found as follows where the suggested part is marked in red.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-4-1a | DL PRS processing capability for PRS bandwidth aggregation measurement in MG | - Maximum number F of PFLs for aggregation, which is supported and reported by UE. F: {2, 3}, where F < the value reported by FG13-1a Note1: the maximum bandwidth for aggregation is F×B where B is the bandwidth for single PFL reported by FG 13-1; - Duration of DL PRS symbols NAgg in units of ms a UE can process every TAgg ms assuming maximum DL PRS bandwidth after F PFLs aggregation in MHz, which is supported and reported by UE. - TAgg : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NAgg : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note2: NAgg ≤ N or TAgg≥ T, where {N, T} is reported by FG 13-1 - Max number of DL PRS resources MAgg that UE can process in a slot under it for PFL aggregation measurement a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note3: MAgg ≤ M, where M is maximum number PRS resources for single PFL measurement reported by FG 13-1 | 13-1, 13-1a, | No |  | DL PRS bandwidth aggregation measurement is not supported in MG | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-1b | DL PRS processing capabilities for bandwidth aggregation measurement in RRC inactive state | - Maximum number F of PFLs for aggregation, which is supported and reported by UE. F: {2, 3}, where F < the value reported by FG13-1a Note1: the maximum bandwidth for aggregation is F×B where B is the bandwidth for single PFL reported by FG 13-1 - Duration of DL PRS symbols NAgg in units of ms a UE can process every TAgg ms assuming maximum DL PRS bandwidth after F PFLs aggregation in MHz, which is supported and reported by UE. - TAgg : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NAgg : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note2: NAgg ≤ N or TAgg≥ T, where {N, T} is reported by FG 27-6 - Max number of DL PRS resources MAgg that UE can process in a slot under it for PFL aggregation measurement a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note3: MAgg ≤ M, where M is maximum number PRS resources for single PFL measurement reported by FG 27-6 | 27-6 | No |  | DL PRS bandwidth aggregation measurement is not supported in RRC inactive state | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-1c | DL PRS processing capabilities for bandwidth aggregation measurement in RRC idle state | - Maximum number F of PFLs for aggregation, which is supported and reported by UE. F: {2, 3}, where F < the value reported by FG13-1a Note1: the maximum bandwidth for aggregation is F×B where B is the bandwidth for single PFL reported by FG 13-1 - Duration of DL PRS symbols NAgg in units of ms a UE can process every TAgg ms assuming maximum DL PRS bandwidth after F PFLs aggregation in MHz, which is supported and reported by UE. - TAgg : {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - NAgg : {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of DL PRS resources MAgg that UE can process in a slot under it for PFL aggregation measurement a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  | No |  | DL PRS bandwidth aggregation measurement is not supported in RRC idle state | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-3 | Support of PRS bandwidth aggregation in RRC_CONNECTED — DL-TDOA | Support DL-TDOA positioning method with PRS bandwidth aggregation in RRC_CONNECTED | 13-3 | Yes |  | DL-TDOA positioning method with PRS bandwidth aggregation is not supported in RRC_CONNECTED | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-3a | Support of PRS bandwidth aggregation in RRC_CONNECTED —Multi-RTT | Support Multi-RTT positioning method with PRS bandwidth aggregation in RRC_CONNECTED | 13-4 | Yes |  | Multi-RTT positioning method with PRS bandwidth aggregation is not supported in RRC_CONNECTED | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-4 | Support of PRS bandwidth aggregation in RRC_ INACTIVE — DL-TDOA | Support DL-TDOA positioning method with PRS bandwidth aggregation in RRC_ INACTIVE | 27-18a | Yes |  | DL-TDOA positioning method with PRS bandwidth aggregation is not supported in RRC_ INACTIVE | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-4a | Support of PRS bandwidth aggregation in RRC_ INACTIVE —Multi-RTT | Support Multi-RTT positioning method with PRS bandwidth aggregation in RRC_ INACTIVE | 27-18c | Yes |  | Multi-RTT positioning method with PRS bandwidth aggregation is not supported in RRC_ INACTIVE | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-5 | Support of PRS bandwidth aggregation in RRC_IDLE — [DL-TDOA and Multi-RTT] | Support DL-TDOA positioning method with PRS bandwidth aggregation in RRC_ IDLE | 41-4-1c | Yes |  | DL-TDOA positioning method with PRS bandwidth aggregation is not supported in RRC_ IDLE | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-5a | Support of PRS bandwidth aggregation in RRC_IDLE — [DL-TDOA and Multi-RTT] | Support Multi-RTT positioning method with PRS bandwidth aggregation in RRC_ IDLE | 41-4-1c | Yes |  | Multi-RTT positioning method with PRS bandwidth aggregation is not supported in RRC_ IDLE | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-6 | Support of positioning SRS bandwidth aggregation in RRC_CONNECTED | 1. The number of contiguous carriers for SRS aggregation Candidate values {2, 3} | 13-8 | Yes |  | Positioning SRS bandwidth aggregation is not supported for UE in RRC_CONNECTED state | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-7 | Support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | 1. The switching time before and after the aggregated SRS transmissions when an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed | 41-4-6 | Yes |  | Positioning SRS bandwidth aggregation independent from UL communication CA is not supported in RRC_CONNECTED | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-4-8 | Support of positioning SRS bandwidth aggregation in RRC_INACTIVE | 1. The number of SRS resources for positioning on a symbol across contiguous carriers Candidate values {2, 3} | 27-15b | Yes |  | Positioning SRS bandwidth aggregation is not supported for UE in RRC_INACTIVE state | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |

UE feature on positioning for RedCap UEs

In RAN1#113 meeting, the basic FG rows have been agreed on positioning for RedCap UEs as shown in section 1. On top of the agreements, we further provide our views.

Proposal 13: For FG 41-5-1 for DL PRS Rx hopping

- Report the maximum number of hops

  - The bandwidth of each hop refers to legacy FG 13-1

- Report capability of overlapping PRB(s) between adjacent hops

- Report the RF Rx retune times between consecutive hops as the previous agreement

Proposal 14: Split the FG 41-5-2 into two rows for RRC_CONNECTED and RRC_INACTIVE state:

- Report the maximum number of hops

  - The bandwidth of each hop refers to FG 28-1

- Report capability of overlapping PRB(s) between adjacent hops

- Report the RF Rx retune times between consecutive hops as the previous agreement.

- Report the support number of SRS resource or resource sets similar as Rel-16/17.

Our views on the detailed table can be found as follows where the suggested part is marked in red.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-5-1 | Support of PRS with Rx frequency hopping within a MG for RedCap UEs | 1. Maximum number H of hops for PRS Rx frequency hopping, which is supported and reported by UE. a): {2, 3, 4, 5, 6} Note1: the bandwidth of each hop refers to FG 13-1 - Overlapping PRB(s) between adjacent hops a): {0, 1, 2, 4} Note2: The maximum equivalent bandwidth after combing all hops can be derived from the number of hops H multiplies the maximum bandwidth per hop 3. RF Rx retune times between consecutive hops which is supported and reported by the UE a) FR1 bands: {70 usec, 140 usec, 210 usec} b) FR2 bands: {35 usec, 70 usec, 140usec} | 13-1 | No |  | DL PRS with Rx frequency hopping in MG is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-5-2 | Support of positioning SRS with Tx frequency hopping in RRC_CONNECTEDfor RedCap UEs | 1. Maximum number H of hops for SRS Tx frequency hopping, which is supported and reported by UE. H: {2, 3, 4, 5, 6} Note1: the bandwidth of each hop refers to FG 28-1 2. Overlapping PRB(s) between adjacent hops a): {0, 1, 2, 4} Note2: The maximum equivalent bandwidth after combing all hops can be derived from the number of hops H multiplies the maximum bandwidth per hop 3. RF Tx retune times between consecutive hops which is supported and reported by the UE a) FR1 bands: {70 usec, 140 usec, 210 usec} b) FR2 bands: {35 usec, 70 usec, 140 usec} 4. Max number of SRS Resource Sets for positioning supported by UE 5. Max number of periodic SRS Resources for positioning 6. Max number of periodic SRS Resources for positioning per slot 7. Max number of P/SP SRS Resources for positioning 8. Max number of P/SP SRS Resources for positioning per slot | 28-1 | Yes |  | RedCap UE SRS frequency hopping is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41-5-2 | Support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for RedCap UEs | 1. Maximum number H of hops for SRS Tx frequency hopping, which is supported and reported by UE. H: {2, 3, 4, 5, 6} Note1: the bandwidth of each hop refers to FG 28-1 2. Overlapping PRB(s) between adjacent hops a): {0, 1, 2, 4} Note2: The maximum equivalent bandwidth after combing all hops can be derived from the number of hops H multiplies the maximum bandwidth per hop 3. RF Tx retune times between consecutive hops which is supported and reported by the UE a) FR1 bands: {70 usec, 140 usec, 210 usec} b) FR2 bands: {35 usec, 70 usec, 140 usec} 4. Max number of SRS Resource Sets for positioning supported by UE 5. Max number of periodic SRS Resources for positioning 6. Max number of periodic SRS Resources for positioning per slot 7. Max number of P/SP SRS Resources for positioning 8. Max number of P/SP SRS Resources for positioning per slot | 28-1 |  |  | RedCap UE SRS frequency hopping is not supported | Per band |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |

Reference

[1] R1-2306223	Updated RAN1 UE features list for Rel-18 NR after RAN1#113	Moderators (AT&T, NTT DOCOMO, INC.)