3GPP TSG RAN WG1 #114			R1-2307057

Toulouse, France, August 21st – August 25th, 2023

Source: 	CATT

Title:	SRS enhancement for CJT and 8Tx operation

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

- Introduction

In previous meetings, it was agreed to specify SRS enhancements for TDD CJT and UL 8Tx, and several agreements have been reached. Specifically, for TDD CJT, both cyclic shift hopping and comb offset hopping are supported. For UL 8Tx, both 8-port SRS resource with the 8 ports mapped onto one or more OFDM symbols using legacy schemes and 8-port SRS resource with the resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot are supported. In RAN1 #113 meeting, draft CRs on Rel-18 NR MIMO was discussed and endorsed[1]-[6].

In this contribution, we present our views on remaining issues of SRS for TDD CJT and UL 8Tx operation, and provide some TPs on draft CRs on Rel-18 SRS enhancements.

- Discussion

  - SRS enhancement targeting TDD CJT

    - Calculation of comb offset and cyclic shift

In TS 38.211[7], the legacy SRS resources without comb offset hopping and/or cyclic shift hopping configured are determined as follows.

For a legacy SRS resource without comb offset hopping configured, the frequency-domain starting position k0(pi) for port pi is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC
kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSmSRS,bNscRBnb
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

where nshift is a threshold, if NBWPstart≤nshift, the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP, where NBWPstart is the start of bandwidth. NscRB is the number of subcarriers per RB. koffsetl' is the offset value of the SRS. For positioning, the value of koffsetl' is shown in Table 6.4.1.4.3-2[7]. kTC∈0,1,…,KTC-1 is contained in the higher layer parameter transmissionComb.

Without comb offset hopping configured, when two SRSs are allocated on the same comb offset(s) on an OFDM symbol and causing strong interference to each other, they may continue to interfere each other on other OFDM symbols. To improve the interference randomization across different SRSs sent by different UEs, comb offset hopping over time is supported in Rel-18. When an SRS resource is configured with comb offset hopping, the SRS hops to different comb offsets on different OFDM symbols based on a pseudo random sequence. To achieve this, an additional comb offset kcomb, offsetl' can be added while determining k0(pi) of the SRS.

For an SRS resource with comb offset hopping configured, k0(pi) can be modified as follows:

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+kcomb, offsetl' mod KTC

where  kcomb, offsetl' is the additional hopping value of comb offset hopping：

kcomb, offsetl'=m=07c8*t+m*2mmod KTC.

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition.SFN is the System Frame Number of current radio frame, N is the reinitialization periodicity of comb offset hopping/cyclic shift hopping and N is determined as a fix value of 128 in the last meeting[8]. Nslotframe,μ is the number of slot in a radio frame, Nsymbslot is the number of OFDM symbols in a slot. Nslotframe,μNsymbslot is the number of OFDM symbols in a radio frame. ns,fμ indicates the current slot number, Nsymbslot indicates the number of OFDM symbols in a slot. ns,fμNsymbslot is the number of OFDM symbols before the current slot in current radio frame, and l0 indicates the starting OFDM symbol position of the SRS in a slot. l' is the index of the OFDM symbol, l'∈0,1,…,NsymbSRS-1, where NsymbSRS is the number of OFDM symbols of the SRS resource. c∙ is a pseudo-random sequence. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

For a legacy SRS resource without cyclic shift hopping configured, the cyclic shift αi for port pi is defined by

αi=2πnSRScs,inSRScs,max
nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise,

where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1[7].

To improve interference randomization across different SRSs sent by different UEs, cyclic shift hopping over time is supported in Rel-18. When an SRS resource is configured with cyclic shift hopping, the SRS hops to different cyclic shifts on different OFDM symbols based on a pseudo random sequence. To achieve this, an additional offset value nSRScs,offset can be added in the formula on calculating cyclic shift αi for port pi.

For an SRS resource with cyclic shift hopping configured, the cyclic shift αi for port pi can be determined by

αi=2πnSRScs,i+nSRScs,offsetmod nSRScs,maxnSRScs,max

where  nSRScs,offset is the additional hopping value of cyclic shift hopping：

nSRScs,offset=m=07c8*t+m*2mmod nSRScs,max.

where

t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition.

c∙ is a pseudo-random sequence, the additional hopping value of cyclic shift hopping is calculated by converting 8-bit binary numbers to decimal integers within  nSRScs,max. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

Proposal 1: For an SRS resource with comb offset hopping configured, k0(pi) can be modified as follows:

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+kcomb, offsetl' mod KTC

where

kcomb, offsetl'=m=07c8*t+m*2mmod KTC

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 2: For an SRS resource with cyclic shift hopping configured, the cyclic shift αi for port pi can be determined by

αi=2πnSRScs,i+nSRScs,offsetmod nSRScs,maxnSRScs,max

where

nSRScs,offset=m=07c8*t+m*2mmod nSRScs,max

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

    - The subset of comb offsets / cyclic shifts for hopping

In some cases, UEs supporting comb offset hopping and/or cyclic shift hopping may be multiplexed on the same OFDM symbol with legacy UEs not supporting these hopping, and the hopping may lead to collisions of comb offset (REs) and/or cyclic shift between these UEs. In order to avoid potential collision, it was agreed that gNB can configure a subset of comb offsets/cyclic shifts for the UE. If a Rel-18 SRS resource configured with comb offset hopping and/or cyclic shifts hopping is FDMed/CDMed with a legacy SRS resource, the hopping is performed within the subset, thus avoiding potential collisions.

In RAN1 #113 meeting, the following agreement on a subset of cyclic shifts/comb offsets was reached:

Agreement Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. - This is a UE-optional feature.

For an SRS resource configured with a subset of offsets for comb offset hopping/cyclic shift hopping, the configured subset shall be considered when calculating frequency domain position/cyclic shift of the SRS.

For an SRS resource with a subset of comb offsets configured, the frequency-domain starting position k0(pi) for port pi can be determined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+s(kTC(pi)+koffsetl'+kcomb, offsetsubset modNcombsubset).

kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSmSRS,bNscRBnb
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

Ncombsubset is the number of comb offsets in the configured subset, Ncombsubset≤KTC. s(x) is a function that maps x to a value in the subset. For example, s(x) can be defined as the xth value in the subset, xϵ {0,1,…,Ncombsubset-1}.

kcomb, offsetsubset can be calculated as follows:

kcomb, offsetsubset=m=07c8*t+m*2mmod Ncombsubset

For an SRS resource with a subset of cyclic shifts configured, the cyclic shift αi for port pi can be determined by

αi=2πs(nSRScs,i+ncs, offsetsubsetmod Ncssubset)nSRScs,max

nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

Ncssubset is the number of cyclic shifts in the configured subset, Ncssubset≤nSRScs,max. s(x) is a function that maps x to a value in the subset. For example, s(x) is defined as the xth value in the subset,xϵ{0,1,…,Ncssubset-1}.

ncs, offsetsubset is calculated based on the following formula:

ncs, offsetsubset=m=07c8*t+m*2mmodNcssubset

Proposal 3:

For an SRS resource with subset of comb offsets configured, the frequency-domain starting position k0(pi) for port pi is determined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+s(kTC(pi)+koffsetl'+kcomb, offsetsubset modNcombsubset).

kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSmSRS,bNscRBnb
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

kcomb, offsetsubset=m=07c8*t+m*2mmodNcombsubset

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 4:

For an SRS resource with subset of cyclic shifts configured, the cyclic shift αi for port pi is determined by

αi=2πs(nSRScs,i+ncs, offsetsubsetmod Ncssubset)nSRScs,max

nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

ncs, offsetsubset=m=07c8*t+m*2mmodNcssubset

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

The subset of comb offsets/cyclic shifts needs to be configured by the gNB. In the last meeting, the configuration of the subset was also discussed, and the following proposal was proposed:

Proposal 2.2-4: When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, down select from the following options: - Option 1: The subset is configured using a new RRC parameter which includes one or more integer values for hopping offsets. - Option 2: The subset is explicitly configured by legacy parameters cyclicShift and combOffset for cyclic shift hopping and comb offset hopping, respectively. - Option 3: The number of subsets is fixed as 2, and a subset is defined as half of all possible hop locations.

In Option 3, the allocation of resources for hopping is rigid, always set at half of all resource locations. This lack of flexibility becomes problematic when there exists a substantial disparity in numbers between Rel-18 UEs and legacy UEs. To address this, it is crucial to flexibly configure the proportion of resources that the subset of legacy UEs and Rel-18 UEs can utilize, based on the specific scenario at hand. Consequently, Option 3 is ruled out first.

If the gNB configures the subset based on Option 2, the gNB uses legacy parameter combOffset and cyclicShift, which is contained in the higher-layer parameter transmissionComb, to configure the legacy parameter(kTC and nSRScs) and the subset of the comb offset/cyclic shifts. Linking the subset configuration to an existing legacy parameter will impose limitation on the usage of legacy parameters. It is not desirable from the configuration flexibility perspective.

Option 1 is a natural selection that gNB configures a subset of comb offsets/cyclic shifts explicitly with a new RRC parameter (one or multiple integers). Option 1 is flexible in the configuration of the subset without impacting functionality of legacy parameters.

Proposal 5: The subset is configured using a new RRC parameter which includes one or more integer values for hopping offsets.

  - SRS enhancement for UL 8Tx

    - 8-port SRS resource based on TDMed schemes

Port splitting

According to the agreement in RAN1 #112 meeting, for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the 8 ports are equally partitioned into s subsets with each subset having 8/s different ports. Each of the m OFDM symbols has only one subset. And the existing resource mapping designed for 8/s ports on each OFDM symbol is reused. There is no agreement on how to split the 8 ports to the two subsets yet. For an SRS resource used for codebook based PUSCH transmission, SRS ports for the same coherent group mapped to the same OFDM symbol(s) (if possible) or consecutive OFDM symbols (if mapped to the same OFDM symbols is not allowed) are desired. Therefore for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s = 2, each subset can be the set of SRS ports for a coherent group of the partially-coherent precoding for an 8Tx UEs with Ng=2.

In RAN1 #113 meeting, it has been agreed that the two coherent groups are {1000, 1001, 1004, 1005} and {1002, 1003, 1006, 1007} for the codebook design of an 8Tx partial-coherent UE with Ng = 2. Then for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s = 2, the two subsets mapped to the two different OFDM symbols shall be {1000, 1001, 1004, 1005} and {1002, 1003, 1006, 1007}, respectively.

Proposal 6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s, the two subsets mapped to the two different OFDM symbols shall be {1000, 1001, 1004, 1005} and {1002, 1003, 1006, 1007}, respectively.

According to the agreement in RAN1 #112 meeting, for a TDM based 8-port SRS resource, the existing resource mapping designed for 4-port SRS resource is reused, including frequency-domain resource allocation and mapping to cyclic shifts. Since for an 8-port SRS resource with resource mapping based on TDM with TDM factor s = 2, {1000, 1001, 1004, 1005} are mapped to the same OFDM symbols and {1002, 1003, 1006, 1007} are mapped to the same OFDM symbols, the same comb offsets and cyclic shifts as 1000, 1001, 1002, 1003 for an 4-port SRS resource can be applied to ports {1000, 1002}, {1001, 1003}, {1004, 1006}, {1005, 1007} respectively.

Proposal 7: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s, the same comb offsets and cyclic shifts as 1000, 1001, 1002, 1003 for an 4-port SRS resource are applied to ports {1000, 1002}, {1001, 1003}, {1004, 1006}, {1005, 1007}, respectively.

Power control

In Rel-17, the transmit power for an SRS resource is calculated with the following formula, and the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for the SRS.

[dBm]

For a TDM based 8-port SRS resource, each OFDM symbol is mapped with part of SRS ports. Power control enhancement for SRS can be considered to improve the SRS coverage and transmission reliability.

For a UE capable of transmitting at PCMAX(i)(i.e., the maximum transmission power specified in the current specifications, PCMAX,f,c(i)) per OFDM symbol with part of SRS ports, the linear value PSRS (i.e., PSRS,b,f,c(i,qs,l) in current specifications) of the transmit power PSRS,b,f,c(i,qs,l) can be split equally across the SRS antenna ports mapped onto one OFDM symbol.

There are various PA combinations of UEs that not capable of transmitting at PCMAX(i) per OFDM symbol with part of SRS ports. For those UEs, power control enhancement for SRS based on TDM-ed schemes also can be considered. No matter which PA combination does a UE have, the transmit power of SRS antenna ports onthe same OFDM symbols should be the same. Therefore, one of the following alternatives on power control for a TDM based 8-port SRS resource with TDM factor s should be considered:

- Alt 1: The linear value PSRS of the transmit power PSRS is split equally across all antenna ports of the SRS resource;

- Alt 2: The linear value PSRS(l) of the transmit power PSRS(l) is split equally across the SRS antenna ports mapped onto one OFDM symbol, where l is the index of the OFDM symbol, and PSRS(l) denotes a transmit power for symbol l.

For Alt 1, all SRS antenna ports have the same transmit power. For Alt 2, all SRS antenna ports mapped onto the same OFDM symbols have the same transmit power, and SRS ports mapped onto different OFDM symbols can have same or different transmit power.

For PSRS in Alt 1, several options can be considered:

- Option 1:

PSRS = minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi is a new maximum transmission power for SRS.

- Option 2:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α is a splitting factor depends on UE’s PA combinations. α can be determined by the PA with the lowest maximum output power. For example, if s = 2, the PAs of a UE are 20+20+20+20+17+17+17+17 dBm, α can be 2. If s = 2, the PAs of a UE are 20+20+20+20+14+14+14+14 dBm, α=1. Specifically, for α=1, PSRS equals to  PSRS,b,f,c(i,qs,l) specified in the current specifications.

- Option 3:

PSRS= minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi is a new maximum transmission power for SRS, s is the TDM factor of the SRS resource.

- Option 4:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α is a splitting factor depends on UE’s PA combinations, s is the TDM factor of the SRS resource.

- Option 5:

PSRS=10log10α+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α is a splitting factor depends on UE’s PA combinations.

For option 1, by specifying a new maximum transmission power for SRS larger than PCMAX(i) (i.e., the maximum transmission power specified in the current specifications), the transmit power of SRS can be boosted when the total transmit power of 8 SRS antenna ports is larger than PCMAX(i). Then the SRS coverage and transmission reliability can be improved.

For option 2, by specifying a scaling factor α>1, the transmit power of SRS also can be boosted when the total transmit power of 8 SRS antenna ports is larger than PCMAX(i). Option 1 and option 2 can achieve the same improvement. The difference of the two options is whether a new definition for maximum transmission power for SRS is introduced or the maximum transmission power for SRS is introduced by scaling the linear value of the maximum transmission power in current specifications by α.

For option 3, besides improving the SRS coverage and transmission reliability when the total transmit power of 8 SRS antenna ports is larger than PCMAX(i), the transmit power of SRS antenna ports also can be boosted when the total transmit power of 8 SRS antenna ports is no larger than PCMAX(i).

For option 4, similar as that for option 3, the transmit power of SRS can be increased when the total transmit power of 8 SRS antenna ports is larger than PCMAXi or is no larger than PCMAX(i). Option 3 and option 4 can achieve the same improvement. The difference of the two options is whether a new definition for maximum transmission power for SRS is introduced or the maximum transmission power for SRS is introduced by scaling the linear value of the maximum transmission power in current specifications by α.

For option 5, the power boosting value of SRS antenna ports is constant.

For PSRS(l) in Alt 2, the following two alternatives can be considered:

- Alt 1: PSRS(l) is determined based on PSRS,b,f,c(i,qs,l), by multiplexing a symbol specific scaling factor αl to the linear value of the transmit power PSRS,b,f,c(i,qs,l) specified in the current specifications.

- Alt 2: PSRS(l) is determined based on the power control parameters for the SRS resource set, with one of the following options adopted:

  - Option 1:

PSRS(l) = minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l.

  - Option 2:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l. α(l) can be determined based on the PA with the lowest maximum output power in the OFDM symbol.

  - Option 3:

PSRS= minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l, s is the TDM factor of the SRS resource.

  - Option 4:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l, s is the TDM factor of the SRS resource.

  - Option 5:

PSRS=10log10α(l)+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l.

Proposal 8: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s for a UE not capable of transmitting at PCMAXi (i.e., the maximum transmission power specified in the current specifications) per OFDM symbol, down selection one of the followings:

- Alt 1: The linear value PSRS of the transmit power PSRS is split equally across all antenna ports of the SRS resource, where PSRS is determined based on one of the following options:

  - Option 1:

PSRS = minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi is a new maximum transmission power for SRS.

  - Option 2:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α is a splitting factor depends on UE’s PA combinations. α can be determined by the PA with the lowest maximum output power. Specifically, for α=1, PSRS equals to  PSRS,b,f,c(i,qs,l) specified in the current specifications.

  - Option 3:

PSRS= minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi is a new maximum transmission power for SRS, s is the TDM factor of the SRS resource.

  - Option 4:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α is a splitting factor depends on UE’s PA combinations, s is the TDM factor of the SRS resource.

  - Option 5:

PSRS=10log10α+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α is a splitting factor depends on UE’s PA combinations.

- Alt 2: The linear value PSRS(l) of the transmit power PSRS(l) is split equally across the SRS antenna ports mapped onto one OFDM symbol, where l is the index of the OFDM symbol, PSRS(l) is determined based one of the following alternatives:

  - Alt 1: PSRS(l) is determined based on PSRS,b,f,c(i,qs,l), by multiplexing a symbol specific scaling factor αl to the linear value of the transmit power PSRS,b,f,c(i,qs,l) specified in the current specifications

  - Alt 2: PSRS(l) is determined based on the power control parameters for the SRS resource set, with one of the following options adopted:

    - Option 1:

PSRS(l) = minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l.

    - Option 2:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l. α(l) can be determined based on the PA with the lowest maximum output power in the OFDM symbol.

    - Option 3:

PSRS= minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l,  s is the TDM factor of the SRS resource.

    - Option 4:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l, s is the TDM factor of the SRS resource.

    - Option 5:

PSRS=10log10α(l)+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l.

Collision handling

In RAN1 #112bis-e meeting, the following agreement was achieved on collision handling of SRS and other signals with higher priority than the SRS:

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions: - Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration. - Whether or not a UE changes the transmission order of the subsets of ports.

It is our view that legacy collision handling rule can be reused, i.e., only the SRS transmission on the collision symbols are dropped. Changing the transmission order of the subsets of ports would increase the complexity of UEs and gNB, therefore after dropping the SRS transmission on collision symbols, the UE shall not change the transmission order of the subsets of ports.

Proposal 9: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped due to collision with a signal with higher priority than the SRS, only the SRS transmission on the collision symbols are dropped.

Proposal 10: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE does not change the transmission order of the subsets of ports.

    - TPs for UL 8Tx SRS enhancement

In draft CR of TS 38.211 for Rel-18 UL 8Tx SRS enhancement, higher layer parameter enhanced-dmrs-Type_r18 is used to identify whether an SRS resource is configured to be a TDM based SRS resource. However, according to the meeting agreement in RAN1 #113 meeting, the higher layer parameter should be tdm.

Proposal 11: Adopt the following TP for draft CR of TS 38.211 for Rel-18 UL 8Tx SRS:

----------------Start of TP for TS38.211---------------------

##### 6.4.1.4.2	Sequence generation

<unrelated part omitted>

The sounding reference signal sequence for an SRS resource shall be generated according to

r(pi)n,l'=wTDM(pi)l'ru,v(αi,δ)n

0≤n≤Msc,bSRS-1

l'∈0,1,…,NsymbSRS-1

where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb.

The quantity wTDMpil' is given by

-	if the higher-layer parameter tdm is configured

wTDMpil'=1if l'∈0,2,…,NsymbSRS-2 and pi∈{1000, 1001, 1002, 1003}1if l'∈1,3,…,NsymbSRS-1 and pi∈{1004, 1005, 1006, 1007}0otherwise

-	otherwise

wTDMpil'=1

The cyclic shift αi for antenna port pi is given as

αi=2πnSRScs,inSRScs,max

where, if the higher-layer parameter tdm is not configured,

nSRScs,i=nSRScs+nSRScs,maxpi-10004NapSRS/4 mod nSRScs,maxif NapSRS=8 and nSRScs,max=6nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6; or if NapSRS=8 and nSRScs,max=12nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max is given by Table 6.4.1.4.2-1.

<unrelated part omitted>

##### 6.4.1.4.3	Mapping to physical resources

<unrelated part omitted>

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sRR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter tdm is configured, otherwise s=1. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

<unrelated part omitted>

----------------End of TP for TS38.211---------------------

- Conclusions

In this contribution, we provided our views on remaining issues for enhancement of SRS for Rel-18 TDD CJT and UL 8Tx operation. We have the following proposals:

Proposal 1:

For an SRS resource with comb offset hopping configured, k0(pi) can be modified as follows:

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+kcomb, offsetl' mod KTC

where

kcomb, offsetl'=m=07c8*t+m*2mmod KTC.

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 2:

For an SRS resource with cyclic shift hopping configured, the cyclic shift αi for port pi can be determined by

αi=2πnSRScs,i+nSRScs,offsetmod nSRScs,maxnSRScs,max

where

nSRScs,offset=m=07c8*t+m*2mmod nSRScs,max.

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 3:

For an SRS resource with subset of comb offsets configured, the frequency-domain starting position k0(pi) for port pi is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+s(kTC(pi)+koffsetl'+kcomb, offsetsubset modNcombsubset).

kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSmSRS,bNscRBnb
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

kcomb, offsetsubset=m=07c8*t+m*2mmodNcombsubset

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 4:

For an SRS resource with subset of cyclic shifts configured, the cyclic shift αi for port pi is defined by

αi=2πs(nSRScs,i+ncs, offsetsubsetmod Ncssubset)nSRScs,max

nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

ncs, offsetsubset=m=07c8*t+m*2mmodNcssubset

where t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',if combOffsetHoppingWithRepetition is set to Per-symbol modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'RR,if combOffsetHoppingWithRepetition is set to per-R-Repetition

Proposal 5: The subset is configured using a new RRC parameter which includes one or more integer values for hopping offsets.

Proposal 6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s, the two subsets mapped to the two different OFDM symbols shall be {1000, 1001, 1004, 1005} and {1002, 1003, 1006, 1007}, respectively.

Proposal 7: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s, the same comb offsets and cyclic shifts as 1000, 1001, 1002, 1003 for an 4-port SRS resource are applied to ports {1000, 1002}, {1001, 1003}, {1004, 1006}, {1005, 1007}, respectively.

Proposal 8: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s for a UE not capable of transmitting at PCMAXi (i.e., the maximum transmission power specified in the current specifications) per OFDM symbol, down selection one of the followings:

- Alt 1: The linear value PSRS of the transmit power PSRS is split equally across all antenna ports of the SRS resource, where PSRS is determined based on one of the following options:

  - Option 1:

PSRS = minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi is a new maximum transmission power for SRS.

  - Option 2:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α is a splitting factor depends on UE’s PA combinations. α can be determined by the PA with the lowest maximum output power. Specifically, for α=1, PSRS equals to  PSRS,b,f,c(i,qs,l) specified in the current specifications.

  - Option 3:

PSRS= minPCMAX,SRS(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi is a new maximum transmission power for SRS, s is the TDM factor of the SRS resource.

  - Option 4:

PSRS = minPCMAXi+10log10αPOSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α is a splitting factor depends on UE’s PA combinations, s is the TDM factor of the SRS resource.

  - Option 5:

PSRS=10log10α+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α is a splitting factor depends on UE’s PA combinations.

- Alt 2: The linear value PSRS(l) of the transmit power PSRS(l) is split equally across the SRS antenna ports mapped onto one OFDM symbol, where l is the index of the OFDM symbol, PSRS(l) is determined based one of the following alternatives:

  - Alt 1: PSRS(l) is determined based on PSRS,b,f,c(i,qs,l), by multiplexing a symbol specific scaling factor αl to the linear value of the transmit power PSRS,b,f,c(i,qs,l) specified in the current specifications

  - Alt 2: PSRS(l) is determined based on the power control parameters for the SRS resource set, with one of the following options adopted:

    - Option 1:

PSRS(l) = minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l.

    - Option 2:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where PCMAX(i) is the maximum transmission power specified in the current specifications, and α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l. α(l) can be determined based on the PA with the lowest maximum output power in the OFDM symbol.

    - Option 3:

PSRS= minPCMAX,SRS(i,l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX,SRSi,l is a new maximum transmission power for SRS for OFDM symbol l,  s is the TDM factor of the SRS resource.

    - Option 4:

PSRS = minPCMAXi+10log10α(l)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l+10log10s,

where PCMAX(i) is the maximum transmission power specified in the current specifications, α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l, s is the TDM factor of the SRS resource.

    - Option 5:

PSRS=10log10α(l)+minPCMAX(i)POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,ci∙+PLb,f,cqs+hb,f,ci,l,

where α(l) is a splitting factor depends on UE’s PA combinations for OFDM symbol l.

Proposal 9: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped due to collision with a signal with higher priority than the SRS, only the SRS transmission on the collision symbols are dropped.

Proposal 10: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE does not change the transmission order of the subsets of ports.

Proposal 11: Adopt the following TP for draft CR of TS 38.211 for Rel-18 UL 8Tx SRS:

----------------Start of TP for TS38.211---------------------

##### 6.4.1.4.2	Sequence generation

<unrelated part omitted>

The sounding reference signal sequence for an SRS resource shall be generated according to

r(pi)n,l'=wTDM(pi)l'ru,v(αi,δ)n

0≤n≤Msc,bSRS-1

l'∈0,1,…,NsymbSRS-1

where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb.

The quantity wTDMpil' is given by

-	if the higher-layer parameter tdm is configured

wTDMpil'=1if l'∈0,2,…,NsymbSRS-2 and pi∈{1000, 1001, 1002, 1003}1if l'∈1,3,…,NsymbSRS-1 and pi∈{1004, 1005, 1006, 1007}0otherwise

-	otherwise

wTDMpil'=1

The cyclic shift αi for antenna port pi is given as

αi=2πnSRScs,inSRScs,max

where, if the higher-layer parameter tdm is not configured,

nSRScs,i=nSRScs+nSRScs,maxpi-10004NapSRS/4 mod nSRScs,maxif NapSRS=8 and nSRScs,max=6nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6; or if NapSRS=8 and nSRScs,max=12nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise

where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max is given by Table 6.4.1.4.2-1.

<unrelated part omitted>

##### 6.4.1.4.3	Mapping to physical resources

<unrelated part omitted>

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sRR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter tdm is configured, otherwise s=1. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

<unrelated part omitted>

----------------End of TP for TS38.211---------------------

- References

- R1-2306274, Release 18 TS38.215 Editor CR for MIMO, Intel Corporation, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- R1-2306292, Introduction of MIMO Evolution for Downlink and Uplink, Samsung, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- R1-2306309, Introduction of MIMO evolution for downlink and uplink, Ericsson, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- R1-2306315, Introduction of Rel-18 MIMO Evolution for Downlink and Uplink, Huawei, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- R1-2306326, Release 18 TS38.202 Editor CR for MIMO, Qualcomm, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- R1-2306330, Introduction of specification support for MIMO enhancements on CSI, Nokia, RAN1#113, Incheon, Korea, May 22nd – May 26th, 2023

- 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; 3GPP TS 38.211 V17.2.0 (2022-06).

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #113, May 22nd – May 26th, 2023.