3GPP TSG RAN WG1 #114	                          			   		R1-2307253

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.12.3

Source:	InterDigital, Inc.

Title:	Dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

# Introduction

RAN approved a WI on further coverage enhancements for NR [1]. The WI includes the following objective:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

The WI also includes corresponding justification point:

- DFT-S-OFDM waveform is beneficial for UL coverage limited scenario because of its lower PAPR compared with CP-OFDM waveform. Currently, UL waveform is configured via RRC and this limitation imposes a large barrier to switch over to DFT-S-OFDM waveform for cell-edge UEs practically.

This contribution discusses what enhancements should be specified to support this functionality.

# Applicability of dynamic waveform switching

In RAN1#110bis-e, RAN1 started discussions on the types of PUSCH transmissions to which dynamic waveform switching is applicable. RAN1 agreed that dynamic waveform switching (DWS) is applicable at least to PUSCH dynamically scheduled by DCI format 0_1/0_2.

In RAN1#111, RAN1 agreed to support DWS indication from UL scheduling DCI for DCI-based solution. RAN1 further took a working assumption to support a new 1-bit field for DWS indication from UL scheduling DCI.

In RAN1#112, RAN1 agreed that DWS is not applicable to PUSCH transmissions with a Type 1 configured grant. In addition, there is no consensus to support DWS for PUSCH transmissions with a Type 2 configured grant.

In RAN1#112bis-e, RAN1 concluded that for PUSCH transmission scheduled by C-RNTI with DCI format 0_0, UE considers transform precoding enabled or disabled according to msg3-transformPrecoder as in legacy. No conclusion was made for the case of msg3 PUSCH initial transmission or re-transmission.

Msg3 PUSCH and PUSCH scheduled by TC-RNTI

Since configuring DFT-S-OFDM semi-statically at cell level is already possible, the potential benefit for supporting switching for the RACH case is that the network would not be forced to configure msg3 PUSCH and its retransmissions assuming the worst-case channel conditions for all UEs. Some UEs that are in good channel conditions could transmit using CP-OFDM instead of DFT-S-OFDM. However, considering that msg3 payload is typically relatively small, the resulting gain in resource utilization from system perspective may not be very big.

One hurdle for applying switching to msg3 PUSCH or its retransmissions is that it requires early indication of UE capability and thus potentially additional PRACH partitioning. To avoid this, several companies proposed to tie the functionality to that of PRACH repetition which will be supported in R18. Even if this approach is adopted, supporting DWS indication from RAR or DCI format 0_0 (by TC-RNTI) would have large impact considering the benefit. However, one could still consider a fixed association of DFT-S-OFDM waveform to the use of PRACH repetition without indication from network.

Proposal 1: DCI format 0_0 does not support dynamic waveform switching indication in R18.

Proposal 2: Random access response does not support dynamic waveform switching indication in R18.

Uplink carrier aggregation

Applicability to uplink carrier aggregation was briefly discussed during RAN1#113, but no agreement was reached. Benefits of supporting the functionality are foreseen for this case at least when the two carriers are in different bands or when the UE does not simultaneously transmit on more than one carrier. It is suggested to agree that the functionality is supported while subject to UE capability.

Proposal 3: Dynamic waveform switching is supported for a UE configured with multiple UL carriers, subject to UE capability.

# Indication of dynamic waveform switching

Interpretation of 1-bit indication and BWP switching

RAN1 agreed to support 1-bit DWS indication but the interpretation of the bit is still open. A straightforward solution is to directly map each value to a specific waveform. For example, mapping 0 to DFT-S-OFDM and 1 to CP-OFDM would be consistent with the order used by the transformPrecoder parameter ({enabled, disabled}).

In case of bandwidth part switching from a bandwidth part in which DWS field is not configured (size 0 bit) to a bandwidth part in which DWS is configured (size 1 bit), the UE interprets the DWS field for the indicated bandwidth part by prepending zero (TS38.213, section 12) which means that the waveform corresponding to value “0” is indicated in this scenario. This general principle has been followed since R15 for DCI fields and does not seem to present any special difficulty in the case of the DWS field. In case each value of the DWS field is directly mapped to a waveform, the initial transmission on the bandwidth part configured with DWS from the bandwidth part not configured with DWS always follows that waveform (for example, DFT-S-OFDM). In RAN1#113, it has been suggested to interpret the value of the DWS field based on the setting of an RRC parameter (such as transformPrecoder) to allow the network to configure the waveform of the initial transmission in that scenario. However, the resulting increase of complexity does not seem warranted considering that the scenario does not occur frequently.

Proposal 4: Interpretation of DWS field upon bandwidth part switching follows legacy procedure.

Proposal 5: DWS indication values “0” and “1” are mapped to DFT-S-OFDM and CP-OFDM, respectively.

Handling of FDRA type

In R17, uplink resource allocation in frequency domain for PUSCH can be type 0, type 1 or type 2. Type 1 and type 2 are supported for both CP-OFDM and DFT-S-OFDM while type 0 is supported for CP-OFDM only [38.214 section 6.1.2.2]. When DWS field is configured for a DCI format, the parameter resourceAllocation and/or the FDRA field can potentially indicate resource allocation type 0 which is not supported for DFT-S-OFDM. The following agreement from RAN1#112bis-e identifies two options for handling this:

Agreement For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, downselect between following options: - Option 1 (configuration restriction with error case handling): - UE does not expect resourceAllocation set to resourceAllocationType0. - If DFT-S-OFDM is indicated and resourceAllocation set to dynamicSwitch, UE does not expect MSB of FDRA field set to 0. - Option 2 (UE only uses resourceAllocation if CP-OFDM is indicated): - If DFT-S-OFDM is indicated, UE applies type 1 resource allocation. - If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE. - Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

The two options are to either restrict the network from configuring or indicating type 0 when DWS field is configured for the DCI format (Option 1), or to specify that the configuration/indication of resource allocation type is not applicable when DFT-S-OFDM is indicated (Option 2).

With Option 1, there is some penalty if resource allocation type 0 with CP-OFDM is desired. Since setting the value of resourceAllocation to type 0 is not possible, the network configures resourceAllocation to dynamicSwitch. When resourceAllocation is set to dynamicSwitch, the size of the FDRA field is set to the maximum between the sizes required for type 0 and type 1 allocations, plus one bit to dynamically indicate the type as per R17 specification (TS38.212, 7.3.1.1.2).

With Option 2, the network can set the value of resourceAllocation to type 0. The size of the FDRA field is then the maximum between sizes required for type 0 and type 1 without the additional bit that dynamically indicates the resource allocation type. In this scenario, Option 2 saves one bit compared to Option 1.

In case resource allocation type 1 is desired for both CP-OFDM and DFT-S-OFDM, both Options are equivalent from the perspective of DCI payload since in both Options the FDRA field size would be set based on type 1 allocation.

Observation: For scheduling CP-OFDM with Type 0 resource allocation, Option 1 (Restricting resourceAllocation parameter) adds 1 bit of DCI overhead compared to Option 2 (Not restricting).

Thus, there seems to be some performance benefit for Option 2 compared to Option 1 if one wants to keep the possibility of using type 0 resource allocation when CP-OFDM is indicated. On the other hand, Option 2 has some specification impact since it would be necessary to add the case of resourceAllocation set to type 0 and DWS field configured in the determination of the number of bits for FDRA field. In addition, in section 6.1.2.2 of 38.214 a sentence would be added to specify that the uplink frequency resource allocation type is set to type 1 regardless of the parameter resourceAllocation, when DFT-S-OFDM is indicated. Given that the overall impact seems limited, Option 2 which does not suffer from any performance penalty is slightly preferred. Option 1 may be acceptable if type 0 resource allocation is not widely used in practical implementations.

Proposal 6: For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, the following applies (Option 2):

- If DFT-S-OFDM is indicated, UE applies type 1 resource allocation.

- If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE.

- Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

Handling of DMRS type

Another case of potential incompatible configuration is indicating DFT-S-OFDM when dmrs-Type is set to type 2. The following agreement from RAN1#112bis-e identifies two options for handling this:

Agreement For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, downselect between following options: - Option 1 (configuration restriction with error case handling): - UE does not expect dmrs-Type to be set to type2. - Option 2 (UE only uses dmrs-Type if CP-OFDM is indicated): - If DFT-S-OFDM is indicated, UE applies DMRS type 1. - If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

The two Options are essentially same as for the issue of FDRA type. One difference, however, is that Option 1 is more restrictive than for the case of FDRA type since it is not possible to dynamically indicate the DMRS type. This means that Option 1 does not allow using DMRS type 2 for CP-OFDM at all when DWS field is configured. Option 2 overcomes this restriction with very limited specification impact.

Proposal 7: For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, the following applies (Option 2):

- If DFT-S-OFDM is indicated, UE applies DMRS type 1.

- If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

# Power headroom reporting enhancement

In RAN1#113, RAN1 made the following agreement:

Agreement For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. - Details FFS. - Note: Any MAC CE related decision is up to RAN2 - Option 4: No enhancement.

Subsequent discussions took place during RAN1#113 to define details of Option 1. The following proposal [2] was presented but not agreed:

FL proposal 3-2r5 (R1-2306255) For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select one from the following options: - Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - FFS: If actual PUSCH transmission is not scheduled by a DCI with DWS field, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Power headroom information for assumed PUSCH is supported at least if multiplePHR=False and twoPHRmode is not enabled. FFS: other cases. - For a serving cell with power headroom information of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only power headroom information for assumed PUSCH is reported. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) if Option 1 is selected. - Option 4: No enhancement.

In RAN#100, RAN discussed PHR enhancements for dynamic waveform switching and endorsed the following proposal [3]:

- Proposal #2 - RAN provide guidance to RAN1/2 on dynamic waveform switching objective as below - RAN1 will decide whether to define any PHR enhancement for dynamic waveform switching and to provide the details to RAN2 by August meeting - RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI

In addition, according to guidance from RAN1 chairman, RAN1 will make decision on whether to define any PHR enhancement for Rel-18 on the first day of RAN1#114.

The issue of whether to define any PHR enhancement has been analyzed extensively in previous meetings. In summary, without PHR enhancement (Option 4) the gNB determines whether to switch waveform by a process of trial and error and/or by scheduling transmissions with parameters purposefully set to discover or infer PHR situation. An obvious drawback of this approach is that it can result in a substantial number of unsuccessful transmissions to probe the performance with an alternate waveform. With PHR enhancement (Option 1), the gNB receives PHR for both waveforms at the same time and thus under same conditions. Any difference reported between the PHR of current waveform and PHR of target waveform for the transmission is attributable to the waveform only, therefore there is minimum uncertainty for the resulting impact of the waveform switch.

Proposal 8: Support Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH (PH information for assumed waveform).

Definition of PH information for assumed waveform

The definition of the power headroom information in Option 1 can be based on FL proposal 3-2r5 of R1-2306255.

Proposal 9: Support reporting of power headroom information for assumed waveform as per the following:

- Power headroom information for assumed waveform is based on an actual PUSCH transmission.

  - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed waveform is not supported.

- If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform.

  - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH.

  - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported.

- Power headroom information for assumed waveform includes the following:

  - PCMAX,f,c(i) of assumed PUSCH

    - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH.

Reporting of power headroom information for assumed waveform

According to RAN decision, RAN1 is to provide details on PHR enhancement for dynamic waveform switching (if agreed) to RAN2 by the end of RAN1#114. The following propose to agree on details of reporting that can be provided to RAN2.

As explained in the above, reporting power headroom information for assumed PUSCH is beneficial when reported together with the legacy power headroom information since this allows the gNB to directly assess the impact of changing waveform.

Proposal 10: When UE reports PH information for assumed waveform, the report also includes legacy PHR.

A benefit of reporting the information along with legacy PHR is that the existing PHR triggers can be reused. Such triggers ensure that the information is available in most situations where the gNB might consider a change of waveform (e.g. periodically, change of path loss). Additional triggers could be considered to capture other situations where waveform switch would potentially improve performance, such as when (legacy) PHR becomes lower/higher than a threshold. However, considering limited time remaining for completion of the work item and the decision from RAN#100 that RAN2 will not work on PHR triggering procedure, it is sufficient if UE reports PH information for assumed PUSCH only if a PHR is reported according to an existing PHR trigger.

Proposal 11: UE includes PH information for assumed waveform only in a PHR that is reported according to an existing PHR trigger.

The MAC specification already includes multiple PHR MAC CE’s, i.e.:

- Single entry PHR MAC CE

- Multiple entry PHR MAC CE

- Enhanced single entry PHR MAC CE

- Enhanced multiple entry PHR MAC CE

- Enhanced single entry PHR for multiple TRP MAC CE

- Enhanced multiple entry PHR for multiple TRP MAC CE

To avoid multiplication of additional PHR MAC CE’s, reporting of PH information for assumed waveform can be supported only for certain cases. For example, it is likely not necessary to include the information in a MPE P-MPR report for a MAC entity configured with mpe-Reporting-FR2 or mpe-Reporting-FR2-r17. In addition, inclusion of PH information for assumed waveform only when twoPHRmode is not configured may be an acceptable restriction.

Proposal 12: PH information for assumed waveform is not included in a MPE P-MPR report.

Proposal 13: Reporting PH information for assumed waveform is only supported if twoPHRMode is not configured.

With the above restrictions, at most two additional PHR MAC CE’s would need to be specified, one for single entry PHR and a second for multiple entry PHR.

Proposal 14: Instruct RAN2 to specify one additional MAC CE for single entry PHR and one additional MAC CE for multiple entry PHR for inclusion of PH information for assumed waveform.

To avoid unnecessary overhead, the PH information for assumed waveform can be included in a PH report only if there is significant difference between the PCMAX,f,c(i) of assumed waveform compared to the actual waveform.

Proposal 15: UE only includes PH information for assumed waveform (PCMAX,f,c(i)) in a PH report if the difference between PCMAX,f,c(i) of assumed waveform and PCMAX,f,c(i) of actual PUSCH (in absolute value) is above a threshold.

# Configuration aspects

In R17, waveform applicable to different cases is configured using parameters msg3-transformPrecoder, msgA-transformPrecoder and transformPrecoder. The latter parameter configures the waveform for PUSCH scheduled by DCI format 0_1 and 0_2.

In RAN1#113, RAN1 agreed that configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2. Given this agreement, it seems straightforward to introduce, for each DCI format, one new RRC parameter indicating the presence of DWS field for this DCI format.

Proposal 16: Introduce new RRC parameter (e.g. dynamicWaveformSwitchingDCI0-1) indicating if dynamic waveform switching field is present in DCI format 0_1.

Proposal 17: Introduce new RRC parameter (e.g. dynamicWaveformSwitchingDCI0-2) indicating if dynamic waveform switching field is present in DCI format 0_2.

# Conclusion

This contribution discussed what enhancements should be specified to support dynamic switching between DFT-S-OFDM and CP-OFDM, and proposed the following:

Proposal 1: DCI format 0_0 does not support dynamic waveform switching indication in R18.

Proposal 2: Random access response does not support dynamic waveform switching indication in R18.

Proposal 3: Dynamic waveform switching is supported for a UE configured with multiple UL carriers, subject to UE capability.

Proposal 4: Interpretation of DWS field upon bandwidth part switching follows legacy procedure.

Proposal 5: DWS indication values “0” and “1” are mapped to DFT-S-OFDM and CP-OFDM, respectively.

Proposal 6: For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, the following applies (Option 2):

- If DFT-S-OFDM is indicated, UE applies type 1 resource allocation.

- If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE.

- Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

Proposal 7: For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, the following applies (Option 2):

- If DFT-S-OFDM is indicated, UE applies DMRS type 1.

- If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

Proposal 8: Support Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH (PH information for assumed waveform).

Proposal 9: Support reporting of power headroom information for assumed waveform as per the following:

- Power headroom information for assumed waveform is based on an actual PUSCH transmission.

  - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed waveform is not supported.

- If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform.

  - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH.

  - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported.

- Power headroom information for assumed waveform includes the following:

  - PCMAX,f,c(i) of assumed PUSCH

    - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH.

Proposal 10: When UE reports PH information for assumed waveform, the report also includes legacy PHR.

Proposal 11: UE includes PH information for assumed waveform only in a PHR that is reported according to an existing PHR trigger.

Proposal 12: PH information for assumed waveform is not included in a MPE P-MPR report.

Proposal 13: Reporting PH information for assumed waveform is only supported if twoPHRMode is not configured.

Proposal 14: Instruct RAN2 to specify one additional MAC CE for single entry PHR and one additional MAC CE for multiple entry PHR for inclusion of PH information for assumed waveform.

Proposal 15: UE only includes PH information for assumed waveform (PCMAX,f,c(i)) in a PH report if the difference between PCMAX,f,c(i) of assumed waveform and PCMAX,f,c(i) of actual PUSCH (in absolute value) is above a threshold.

Proposal 16: Introduce new RRC parameter (e.g. dynamicWaveformSwitchingDCI0-1) indicating if dynamic waveform switching field is present in DCI format 0_1.

Proposal 17: Introduce new RRC parameter (e.g. dynamicWaveformSwitchingDCI0-2) indicating if dynamic waveform switching field is present in DCI format 0_2.

# References

RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom.

R1-2306255, “Summary #5 on dynamic switching between DFT-S-OFDM and CP-OFDM”, Moderator (InterDigital, Inc.)

RP-231498, “Summary for discussion [RAN1-UL-Coverage]”, Moderator (NTT DOCOMO, INC.)

# Appendix: Previous agreements

RAN1#113

Agreement

Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

Agreement

For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options:

- Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH.

- Details FFS.

- Note: Any MAC CE related decision is up to RAN2

- Option 4: No enhancement.

RAN1#112b-e

Agreement

For DCI format 0_1/0_2 containing dynamic waveform indication, bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, if different.

- If, for the waveform indicated in the DCI, the bit width N of a field would be smaller than the bit width of the field set as per the above, UE decodes the field using N least significant bits. If N=0, the UE ignores the field for the indicated waveform.

Agreement

For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options:

- Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH.

  - Details FFS.

  - Note: reporting PH information for both waveforms is not precluded.

  - Note: additional trigger for PH for reference PUSCH is not precluded.

- Option 2: New trigger of power headroom report based on waveform switching event.

  - Details FFS.

- Option 3: Both Option 1 and Option 2.

  - Details FFS.

- Option 4: No enhancement.

Conclusion

For PUSCH transmission scheduled by C-RNTI with DCI format 0_0, UE considers transform precoding enabled or disabled according to msg3-transformPrecoder as in legacy.

Agreement

Dynamic waveform switching is configured separately for each BWP, within PUSCH-Config.

Agreement

For UE configured with multi-PUSCH scheduling in time domain in a carrier (i.e. pusch-TimeDomainAllocationListForMultiPUSCH), DCI format 0_1 supports 1-bit field for dynamic waveform switching indication.

- When configured, 1-bit field indicates waveform for all scheduled PUSCH transmissions.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

- UE does not expect resourceAllocation set to resourceAllocationType0.

- If DFT-S-OFDM is indicated and resourceAllocation set to dynamicSwitch, UE does not expect MSB of FDRA field set to 0.

- Option 2 (UE only uses resourceAllocation if CP-OFDM is indicated):

- If DFT-S-OFDM is indicated, UE applies type 1 resource allocation.

- If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE.

- Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

- UE does not expect dmrs-Type to be set to type2.

- Option 2 (UE only uses dmrs-Type if CP-OFDM is indicated):

- If DFT-S-OFDM is indicated, UE applies DMRS type 1.

- If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

Agreement

For configuration of 1-bit dynamic waveform switching indication in DCI format 0_1/0_2 per a carrier, downselect between following options:

- Option 1: Separate configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2.

- Option 2: Common configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2.

RAN1#112

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

RAN1#111

Agreement

For DCI based solution,

- For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI

Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion

Note: It does not imply that the waveform switching indication applies to other transmission or not

- Indication from non-UL scheduling DCI is not supported.

Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed

Working Assumption

Support new 1-bit field for dynamic waveform indication from UL scheduling DCI

- Note: no change of the current size alignment procedure between UL DCI and DL DCI

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

RAN1#110bis-e

Agreement

Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel.

Working Assumption

Support at least one of the following options for the dynamic waveform indication in R18:

Alt 1: Indication from an UL scheduling DCI

- Alt 1-A: New field in scheduling DCI

- Alt 1-B: Reuse existing field in scheduling DCI

  - Alt 1-B-1: Explicit indication by repurposing field, e.g.

    - Add one column to TDRA table

    - Add one column to MCS table(s)

    - Other solutions not precluded

  - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g.

    - RA type, MSB of RA

    - Number of RBs (below threshold or multiple of 2,3,5)

    - Location of RB allocation within carrier and the associated MPR

    - MCS below threshold

    - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS

    - Number of DMRS CDM group(s) without data

    - Precoding information and number of layers

    - SRI

    - Condition over multiple types of scheduling information

    - Other types of scheduling information not precluded

- Indicated waveform applies at least to the scheduled PUSCH transmission

  - FFS: Whether it also applies to subsequent transmissions, and of which type

- FFS: DCI formats can contain the indication

- FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.)

Alt 2: Indication from a non-UL scheduling DCI

- FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI)

- FFS: Types of subsequent transmissions to which indication is applicable

Agreement

To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as:

- Reporting power headroom related information

- Other solutions are not precluded

Agreement

Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1.

- Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0).

R1-2210749	Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)