3GPP TSG RAN WG1 #114			     R1-2307496

Toulouse, FR, August 21st – August 25th, 2023

Source:	NTT DOCOMO, INC.

Title:	Calibration results and peak data rate analysis for NR NTN

Agenda Item:	9.14.2

Document for: 	Discussion

- Introduction

In RAN#99 meeting, a new Study item on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology was approved [1]. The aim of the SI is to provide the description of the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514.

In RAN1#113 meeting, the evaluation methodologies and simulation assumptions for IMT-2020 satellite were discussed, and the proposals and conclusions in Section 1 of [2] are endorsed, except for proposal 2.4 (which is already included in the proposed conclusion). It is agreed that companies are encouraged to provide calibration curves aligned with TR 38.821 calibration case 9 or case 10 (depending on whether frequency reuse factor one or three is used) for system-level simulation. Then there is no need for additional cross-company calibration.

In this contribution, as a starting point, we focus on case 9 for calibration, in which frequency reuse factor one is used. We share our calibration results for both downlink and uplink, and then calculate the peak data rate and peak spectral efficiency for NR NTN based on the agreed parameters of [2].

- Calibration

We developed our simulator for NTN self-evaluation, which comprises the NTN channel model, wrap around mechanism, satellite antenna pattern, beam layout and so on. For related implementation methods, we referred to TR 38.821 [3] and TR 38.811 [4].

In this section, we firstly provide the beam layout and UE dropping for single satellite simulation, which is a critical point for calibration. Then we provide the calibration results for both DL and UL for case9.

2.1 Beam layout and UE dropping

In [3], the beam layout definition for single satellite simulation is described as hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. The UV plane is illustrated as figure 2-1 [3], in which θ is the beam boresight direction. The 19-beam layout considering wrap-around mechanism with FRF = 1 can be illustrated as Figure 2-2. Two additional tiers of beams are considered in the simulation surrounding the 19-beam layout. There are totally 61 beams considered for FRF = 1. We only obtain the data from the 19 central beams for related statistics, which are numbered from 0 to 18 in Figure 2-2. All remaining beams are treated as interference since they share the same frequency band for FRF = 1.

Figure 2-1: UV plane illustration

To get the beam layout and UE dropping, we use the following steps:

- Define the satellite location in ECEF coordinates.

Suppose the radius of the earth is 6371km, at this time for LEO-600, the satellite location in ECEF coordinates is (X,Y,Z) = (6371+600, 0, 0).

- Define the central beam boresight direction based on elevation angle target in UV coordinates.

The elevation angle target is illustrated as Figure 2-3. For the case of elevation angle target with 90 degree, which means that the central beam boresight direction is considered at nadir point, the θ is 0°. So the central beam boresight direction coordinates on UV plane is (0,0). For the case of elevation angle target with 45 degree, we can calculate that θ is 40.26 degree for LEO-600. Then assuming φ is 0°, the central beam bore sight direction coordinates on UV plane can be calculated according to the formula in Figure 2-1. The results is (0.6462,0).

- Calculate the adjacent beam spacing (ABS), which is related to the 3dB beamwidth. Then using ABS to define the beam layout.

For LEO-600 Set-1 S-band, the 3dB beamwidth HPBW is 4.4127 degree, then the adjacent beam spacing on UV plane is ABS=3×sin⁡(HPBW/2). Finally, we can get the 61-beam layout defined on the UV-plane as Figure 2-2.

ABS

Figure 2-2: Beam layout considering wrap-around mechanism with FRF = 1

Figure 2-3: Elevation angle target illustration

- Drop UE to the UV coordinate system in the form of a regular hexagon for each beam. Then translate it into ECEF coordinates.

Figure 2-4 and Figure 2-5 (a)~(c) show the figures of 37-beam layout, UE dropping for 0-degree and 45-degree elevation angle target, in which (a) is the beam layout on UV plane, (b) is the UE dropping on UV plane in form of a regular hexagon for each beam, (c) is the UE location in ECEF coordinates which is translated from UV coordinates. We can find that as the elevation angle becomes larger, the coverage of the beam far away from the satellite is larger.

- Beam layout on UV plane

(b) drop UE on UV coordinates.

(c) translate UE location to ECEF coordinates.

Figure 2-4: Beam layout and UE dropping for 0-degree elevation angle target.

- Beam layout on UV plane

(b) drop UE on UV coordinates.

(c) translate UE location to ECEF coordinates.

Figure 2-5: Beam layout and UE dropping for 45-degree elevation angle target.

According to the position of the UE and the satellite in the ECEF coordinate system, we can calculate the distance between UE and satellite, the angle between UE and beam boresight, and antenna gain, etc. Then we can obtain the calibration results.

2.2 Calibration results for Downlink

In TR 38.821 [3], the list of calibration study cases is shown in table 6.1.1.1-9. The calibration case 9 is for LEO-600 with Set-1 satellite parameter, the detail information is shown in Table 2-1. For the detail simulation assumptions, we follow table 6.1.1.1-1, table 6.1.1.1-3 and table 6.1.1.1-5 in [3] for satellite parameter, UE characteristics and system level simulation assumptions for calibration, respectively. Based on the assumptions, the results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions are reported by the different companies in [5][6].

Table 2-1 calibration case 9

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 1 |

For handheld terminal, the antenna pattern is omni-directional. For the satellite antenna pattern, it is based on Bessel function [4]. The normalized gain pattern for the satellite with 2m equivalent satellite antenna aperture (aperture radius of 1m) and 2GHz frequency is shown in Figure 2-6.

Figure 2-6: Satellite antenna gain pattern for aperture radius 1m at 2GHz frequency

We deploy 61 beams for FRF = 1 as figure 2-2, and we only obtain the data from the 19 central beams for related statistics, which are numbered from 0 to 18. All remaining beams are treated as interference since they share the same frequency band for FRF = 1. Each UE will select one beam with the strongest signal as serving beam, and the other beams are treated as interfering beam. The interference for this UE depends on the angle between the UE location and the interfering beam boresight. Summing the interference from all interfering beams, we can obtain the interference received by this UE. Then we can calculate SIR and SINR. Figure 2-7 illustrates the DL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR. They are aligned with the results of other companies in [5].

Figure 2-7: DL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR.

Observation 1: DL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR are aligned with other companies.

2.3 Calibration results for Uplink

For uplink calibration, the biggest difference compared to DL is that the interference is not defined by a simple geometry, since the interfering signals are received at the satellite, the active UEs in different locations will cause different interference level. In this case, for each beam, we select one UE randomly for transmission, which means a random scheduling is considered. Then we perform the random selection many times to obtain stable and sufficient results for uplink calibration. Figure 2-8 illustrates the UL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR. They are aligned with the results of other companies in [6]. The bandwidth is assumed to be 0.36MHz for UL.

Figure 2-8: UL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR.

Observation 2: UL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR are aligned with other companies.

- Peak data rate and peak spectral efficiency analysis

In RAN1#113 meeting, the following proposals related to peak data rate and peak spectral efficiency are agreed [2].

Proposal 1.1: The following working assumption is confirmed:

For peak spectral efficiency and peak data rate parameters:

- The parameters are chosen based on “ideal conditions”: 90degree elevation angle, 0dB atmospheric loss, 0dB shadow fading margin, 0dB scintillation loss, 0dB polarization loss, 0dB additional losses.

- Companies to provide in RAN1#113 realistic parameters, declaring the assumptions and evaluations leading to those parameters.

Proposal 1.2: For uplink peak spectral efficiency and peak data rate:

- A bandwidth of 8 RBs is used.

- The peak data rate is [2.22-2.65]Mpbs

- The peak spectral efficiency is [1.54-1.84]bps/Hz

- This value is achieved with an uplink SNR of [7.05-7.08]dB

- FFS: Other assumptions to be included in the TR.

Proposal 1.3: For downlink peak spectral efficiency and peak data rate:

A transmission bandwidth of 160 PRBs out of a channel bandwidth of 30 MHz is used. Channel bandwidth of 30 MHz is used as denominator for calculation of spectrum efficiency.

- The peak data rate is [104.58-118.9]Mbps

- The peak spectral efficiency is [3.67-4.13]bps/Hz

- This value is achieved with an uplink SNR of [16.91-16.95]dB

- FFS: Other assumptions to be included in the TR.

Based on these agreements and the parameters in [3], we analyze the peak data rate and peak spectral efficiency as the following steps, and the results are shown in Table 2-2.

- Step1: Calculate the maximum SNR according to link budget;

- Step2: Obtain MCS from 64QAM table based on the SNR;

- Step3: Assuming that DL overhead is 0.15 and UL is 0.0875, calculate peak spectral efficiency, then calculate peak data rate.

Table 2-2 Peak data rate and peak spectral efficiency analysis

| Transmission mode | DL | UL |
| --- | --- | --- |
| Target elevation angle [deg] | 90 | 90 |
| Distance [km] | 600 | 600 |
| Frequency [GHz] | 2.00 | 2.00 |
| TX: EIRP [dBm] | 78.77 | 23.00 |
| RX: G/T [dB/T] | -31.62 | 1.10 |
| Bandwidth [MHz] | 30.00 | 1.44 |
| Free space path loss [dB] | 154.03 | 154.03 |
| Atmospheric loss [dB] | 0.00 | 0.00 |
| Shadow fading margin [dB] | 0.00 | 0.00 |
| Scintillation loss [dB] | 0.00 | 0.00 |
| Polarization loss [dB] | 0.00 | 0.00 |
| Additional losses [dB] | 0.00 | 0.00 |
| SNR [dB] | 16.95 | 7.08 |
| MCS | 25 | 14 |
| SE w/o overhead [bit/s/Hz] | 4.495 | 2.016 |
| SE w/ overhead [bit/s/Hz] | 3.821 | 1.84 |
| Peak data rate [Mbps] | 114.63 | 2.649 |

The requirement of DL and UL peak spectral efficiency and peak data rate are described in section 7.2.2 and 7.2.1 of Report ITU-R M.2514 [7], which are shown in the following Table 2-3 and Table 2-4. We can observe that DL and UL peak spectral efficiency and peak data rate can fulfil Requirements.

Table 2-3 The requirement of DL and UL peak spectral efficiency

| Peak spectral efficiency (DL) | 3 bit/s/Hz (1) |
| --- | --- |
| Peak spectral efficiency (UL) | 1.5 bit/s/Hz (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

Table 2-4 The requirement of DL and UL peak data rate

| Peak data rate (DL) | 70 Mbit/s (1) |
| --- | --- |
| Peak data rate (UL) | 2 Mbit/s (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

Observation 3: The SNR for peak spectral efficiency and peak data rate are 16.95dB for DL with 30MHz, 7.08 dB for UL with 1.44MHz, respectively.

Observation 4: DL and UL peak spectral efficiency and peak data rate can fulfil Requirements.

Conclusion

This contribution provides our calibration results, and peak data rate analysis for NR NTN, and our observations are listed as following:

Observation 1: DL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR are aligned with other companies.

Observation 2: UL calibration results of case 9 including coupling loss, geometry SIR and geometry SINR are aligned with other companies.

Observation 3: The SNR for peak spectral efficiency and peak data rate are 16.95dB for DL with 30MHz, 7.08 dB for UL with 1.44MHz, respectively.

Observation 4: DL and UL peak spectral efficiency and peak data rate can fulfil Requirements.

References

- RP-230736, “New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology”, RAN#99, Ericsson, Qualcomm, Thales, MediaTek, Mar. 20 - 23, 2023.

- R1-2306083, “Feature lead summary #1 on evaluation methodology for IMT-2020 Satellite”, Moderator (Qualcomm), RAN WG1 #113, May 22nd – 26th, 2023.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”.

- TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

- R1-1913404 "System Level Calibration Results for NTN on DL transmissions", Thales, submitted to RAN1#99

- R1-1913405 "System Level Calibration Results for NTN on UL transmissions", Thales, submitted to RAN1#99

- ITU-R M.2514, "Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020"