3GPP TSG RAN WG1 #114	R1-2307696

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.9.4

Source:	Samsung

Title:	Improved GNSS operation for IOT NTN

Document for:	Discussion and decision

- Introduction

A work item on enabling NB-IoT and eMTC to support Non-Terrestrial Network (NTN) was approved in RAN#94e. One possible enhancement related to RAN1 is as following [1]:

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

Regarding the possible enhancement, it has been discussed in previous several RAN1 meetings, and following agreements were achieved in the last meeting [2]:

Agreement

From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition.

Agreement

UE reports one GNSS position fix time duration for GNSS measurement via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31]

- FFS: other component values

Agreement

The UE is not required to transmit or receive any channel / signal within the aperiodic GNSS measurement gap duration before the UE reacquires GNSS successfully.

FFS: UE’s behavior within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap.

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, down select one of the alternatives for the start time of the gap:

- Alt 1: should be at n+ X, where n is the end of MAC CE receiving subframe/slot and X>= 12ms for NB-IoT, X>= 3ms for eMTC

  - Note: X is one value regardless of HARQ feedback enabled or disabled for the MAC CE

  - FFS: details, e.g. X is predefined value or configured value

- Alt 2: should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC, or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled

  - FFS: details, e.g. X1 and X2 are predefined value or configured value, including whether X1 and X2 can be the same

- Alt3: should be at p+ X, where p is the end of HARQ feedback transmission subframe/slot, where HARQ feedback for the MAC CE is always enabled

  - FFS: details, e.g. X is predefined value or configured value

Agreement

For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration.

- FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

- Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network.

- Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured

- Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact

In this contribution, remaining issues on GNSS operation are discussed.

- Discussion

In the last RAN1 meeting, one agreement is that UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. The pre-condition of allowing UL transmission is that frequency error and timing error are within frequency and timing error requirements. UE can use TA timer (i.e., timeAlignmentTimer) to judge whether or not timing/ frequency error are within timing/frequency error requirements. If timeAlignmentTimer is running, timing/frequency error can be considered to be within timing/frequency error requirements. If timerAlignmentTimer is expired, timing/frequency error can be considered to be out of timing/ frequency error requirements. So, autonomous GNSS re-acquisition can be triggered by UE after timerAlignmentTimer is expired. Moreover, the value of X can be configured by eNB by UE specific RRC signaling, and the configuration can be optional. If the value of X is not configured by eNB, UL transmission shouldn’t be allowed after GNSS validity duration is expired. Thus, UE can autonomously trigger GNSS re-acquisition.

Proposal 1: Autonomous GNSS re-acquisition can be triggered by UE in following cases:

- If the value of X is not configured by eNB, UE can autonomously trigger GNSS re-acquisition after GNSS validity duration is expired.

- If the value of X is configured by eNB, UE can autonomously trigger GNSS re-acquisition after the duration X is expired.

  - Within the duration X, UE can autonomously trigger GNSS re-acquisition after timerAlignmentTimer is expired.

Note: X is the duration wherein UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

One open issue is about the UE’s behavior within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap. Since eNB cannot know when the UE finishes the GNSS measurement, it is straightforward to specify that UE’s behavior within the duration is the same as that before the UE reacquires GNSS successfully. Some company proposes that the UE can initiate RACH procedure to tell eNB it has finished GNSS measurement. It can early terminate the GNSS measurement gap. However, RACH procedure will consume time and power. Comparing with the effort on RACH procedure, it may not be economic to early terminate the GNSS measurement gap. So, a unified UE behavior is preferred within GNSS measurement gap, i.e., no transmission/reception of any channel/signal.

Proposal 2: UE is not required to transmit or receive any channel/signal within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap.

Another open issue is about the start time of the aperiodic GNSS measurement gap triggered by eNB with MAC CE. There are three alternatives for down selection:

- Alt 1: should be at n+ X, where n is the end of MAC CE receiving subframe/slot and X>= 12ms for NB-IoT, X>= 3ms for eMTC

  - Note: X is one value regardless of HARQ feedback enabled or disabled for the MAC CE

  - FFS: details, e.g. X is predefined value or configured value

- Alt 2: should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC, or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled

  - FFS: details, e.g. X1 and X2 are predefined value or configured value, including whether X1 and X2 can be the same

- Alt3: should be at p+ X, where p is the end of HARQ feedback transmission subframe/slot, where HARQ feedback for the MAC CE is always enabled

  - FFS: details, e.g. X is predefined value or configured value

If HARQ feedback for the MAC CE triggering aperiodic GNSS measurement gap can be disabled, some misunderstanding on GNSS measurement gap can be caused between eNB and UE. Considering the UE’s behavior is not expected to transmit/receive any channel/signal within GNSS measurement gap, the misunderstanding will seriously impact UE experience. So, HARQ feedback should be always enabled for the MAC CE. Alt3 is preferred for the start time of aperiodic GNSS measurement gap triggered by MAC CE.

Proposal 3: Alt3 is preferred for the start time of aperiodic GNSS measurement gap triggered by MAC CE

Although MAC CE triggering aperiodic GNSS re-acquisition has been agreed, other triggering signaling can be considered as supplement. For example, considering aperiodic GNSS measurement may be triggered under the case UL synchronization is lost. In this case, the UE cannot send HARQ-ACK for MAC CE triggering signaling. Thus, eNB cannot sure whether the MAC CE is successfully received by the UE. Therefore, DCI based triggering signaling should be supported for this case. In addition, after GNSS measurement is completed and new GNSS position fix is obtained, PRACH can be transmitted for the purpose of UL resynchronization. So, the DCI used for indication of PDCCH order RACH can also be used to trigger a new GNSS measurement, e.g., implicit indication without any change for all of the DCI fields, or using some reserved bit(s) in the DCI. And, the PDCCH order RACH can be initiated after completing GNSS measurement by the UE.

Proposal 4: Besides MAC CE based triggering signaling, PDCCH order/DCI based triggering signaling can be considered.

In previous RAN1 meeting, one agreement is that “Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time”. Currently, closed loop time correction is supported, and no specification impact is expected if no potential enhancement. However, closed loop frequency correction is not supported in current specification. Only supporting closed loop time correction cannot guarantee time/frequency error within time/frequency error requirements for a long time. Frequent GNSS re-acquisition cannot be avoided. From the perspective of reducing the times of GNSS re-acquisition, support of closed loop frequency control is needed. Similar to current TA command, using a MAC CE to indicate frequency command can be considered. And, absolute frequency command and/or frequency adjustment command can be considered.

Proposal 5: Support closed loop correction for pre-compensated frequency offset, e.g., absolute frequency command and/or frequency adjustment command can be considered.

- Conclusions

This contribution discusses remaining issues on GNSS operations and the proposals are summarized as following:

Proposal 1: Autonomous GNSS re-acquisition can be triggered by UE in following cases:

- If the value of X is not configured by eNB, UE can autonomously trigger GNSS re-acquisition after GNSS validity duration is expired.

- If the value of X is configured by eNB, UE can autonomously trigger GNSS re-acquisition after the duration X is expired.

  - Within the duration X, UE can autonomously trigger GNSS re-acquisition after timerAlignmentTimer is expired.

Note: X is the duration wherein UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

Proposal 2: UE is not required to transmit or receive any channel/signal within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap.

Proposal 3: Alt3 is preferred for the start time of aperiodic GNSS measurement gap triggered by MAC CE.

Proposal 4: Besides MAC CE based triggering signaling, PDCCH order/DCI based triggering signaling can be considered.

Proposal 5: Support closed loop correction for pre-compensated frequency offset, e.g., absolute frequency command and/or frequency adjustment command can be considered.

- References:

- RP-213596, New WID on IOT NTN enhancements, MediaTek Inc.

- Chairman’s notes for RAN1#113 meeting.