3GPP TSG RAN WG1 #114			R1-2307712

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.16.4

Source:	Samsung

Title:	UE features for NR NCR

Document for:	Discussion and Decision

- Introduction

In this contribution, our views on capabilities for NR network-controlled repeater (NCR) are presented.

- Discussion

In RAN1#112bis and RAN1#113 [1-2], the framework for NR NCR feature was agreed. Further change / revision based on the framework is discussed as follows.

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: Component 2 candidate values: - 15 kHz: {[0,] 1 [, 2,3,4]} - 30 kHz: {[0,]1 [, 2,3,4]} - 60 kHz: {[0,]1, 2 [ ,3,4]} - 120 kHz: {[0,]1, 2, [,3,4]} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In RAN1#113, FG 43-3 was agreed as above. One FFS point is the candidate value set for k subject to different SCSs. In our view, the offset k is used for simplified implementation of NCR and/or to incorporate beam switching delay of the NCR-Fwd. The value of k should be larger than that of a similar UE capability for DCI processing (e.g., parameter N_pdsch) in order to support (e.g., parameters timeDurationForQCL or beamAppTime). In this sense, the value of 0 is not preferrable since it will cause further issues. For example, if value 0 is used, a new timeline is required to be defined to ensure the sufficient processing time for NCR. This extra standardization effort can be avoided if the value 0 of k is removed. Hence, the proposed modification for FG 43-3 is provided as below.

Proposal 1: Adopt FG 43-3 with the following modification (i.e., removing the value of zero and removing the brackets for other values).

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {[0,] 1,2,3,4} - 30 kHz: {[0,] 1,2,3,4} - 60 kHz: {[0,] 1,2,3,4} - 120 kHz: {[0,] 1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As agreed in RAN1#113, FG 43-4 is agreed as follows.

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 43-4, the only leftover issue is whether to keep the word of “temporary”. In our view, it is preferred to remove the word of ‘temporary’. In the case of MAC-CE update of RRC configured beam index(es), the behavior of NCR is already captured in 38.213 (i.e., the beam index(es) provided by the MAC-CE is for a single indication). The adding of ‘temporary’ will cause ambiguity to NCR behavior since the wording of ‘temporary’ itself is not well defined. Hence, the proposed modification for FG 43-4 is provided as below.

Proposal 2: Adopt FG 43-4 with the following modification (i.e., removing “temporary” in component 3).

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In RAN1#112bis-e, FG 43-6 and FG 43-8 were agreed as follows.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

The remaining discussion point for FG 43-6 and FG 43-8 is the corresponding component candidate values. For FG 43-8, it is preferred to confirm the FFS part since it is straightforward to use explicit indication rather than implicit determination of the supported beam indication framework from legacy MIMO FGs. For FG 43-6, it is preferred to confirm the FFS part similar as FG 43-8. In addition, a note can be added to FG 43-6 that the reported candidate values of FG 43-6 should be same as or a subset of the reported candidate values of FG 43-8. Hence, we propose the following modifications.

Proposal 3: Adopt FG 43-6 and FG 43-8 with the following modification.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} Note: the reported value should be same as or a subset of reported value of FG 43-8. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

As agreed in RAN1#110b-e, it is subject to NCR capability to support simultaneous transmission of the UL of C-link and UL of backhaul link. In addition, TDMed UL transmission of C-link and backhaul link was agreed as default capability. For an NCR that supports simultaneous transmission of the UL of C-link and UL of backhaul link, an additional capability for NCR power needs to be considered. The NCR power capability is to indicate whether the NCR: (i) is not subject to a joint power limit across the C-link and BH-link (e.g., using separate PAs / power resources for the C-link and BH-link), or (ii) is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited, or (iii) is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited (e.g., transmit only one of C-link or BH-link when power limited). In our view, NCR capability (i), herein referred to as Type-1 NCR, should not be considered as the only/baseline implementation for an NCR with simultaneous transmission of the UL of C-link, as it is not economical to allocate a dedicated PA / power resource for NCR-MT which is infrequently used for control signaling only. A more reasonable NCR implementation can use a shared PA / power resource for both NCR-MT and NCR-Fwd with basic power sharing capability, such as NCR capabilities (ii) and (iii), herein referred to as Type-2 and Type-3 NCR, respectively. Corresponding NCR behavior can be further discussed.

Proposal 4: Introduce a new component in FG 43-5 to indicate the supported power allocation / sharing method between the C-link and the backhaul link.

| 43. NR_netcon_repeater | 43-5 | 1. Simultaneous UL transmission of backhaul link and C-link 2. Power allocation/sharing between C-link and B-link of an NCR | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes | Component candidate values: {Type 1, Type 2, Type 3} Note: For Type 1, NCR is not subject to a joint power limit across the C-link and BH-link. For Type 2, is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited. For Type 3, is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

The observations and proposals made in this contribution are summarized below.

Proposal 1: Adopt FG 43-3 with the following modification (i.e., removing the value of zero and removing the brackets for other values).

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {[0,] 1,2,3,4} - 30 kHz: {[0,] 1,2,3,4} - 60 kHz: {[0,] 1,2,3,4} - 120 kHz: {[0,] 1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Adopt FG 43-4 with the following modification (i.e., removing “temporary” in component 3).

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Adopt FG 43-6 and FG 43-8 with the following modification.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} Note: the reported value should be same as or a subset of reported value of FG 43-8. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

Proposal 4: Introduce a new component in FG 43-5 to indicate the supported power allocation / sharing method between the C-link and the backhaul link.

| 43. NR_netcon_repeater | 43-5 | 1. Simultaneous UL transmission of backhaul link and C-link 2. Power allocation/sharing between C-link and B-link of an NCR | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes | Component candidate values: {Type 1, Type 2, Type 3} Note: For Type 1, NCR is not subject to a joint power limit across the C-link and BH-link. For Type 2, is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited. For Type 3, is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- Chair notes, RAN1#112bis.

- Chair notes, RAN1#113.