3GPP TSG RAN WG1#114                              R1- 2307761

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.4.1.1

Source:	Transsion Holdings

Title:	Discussion of channel access mechanism for sidelink in unlicensed spectrum

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, a new WID had been approved. And in RAN#95e meeting, this WID had been revised as the following:[1]

- Objective of SI or Core part WI or Testing part WI To check in RAN#97 for objectives 1 and 3, taking into account the progress on objectives 2 and 4, aiming to have specification work for both objective 1 and 3. ...... - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - ......

In this contribution, we focus on the channel access mechanism for sidelink in unlicensed spectrum, including EDT determination, channel access procedures, CP extension, COT sharing, multi-channel access and FBE operation.

Discussion

EDT determination

Energy detection is the only criterion to determine whether the channel is busy or not, so the setting of the energy detection threshold (EDT) will greatly affect the probability of successful channel access. In NR-U/LAA, the setting of the threshold value depends on several parameters, such as channel bandwidth, transmit power, etc.. Another factor is whether carrier frequencies are shared with other wireless access technologies (e.g., WiFi) for long-term, or whether the network deployment can guarantee that the carrier frequency are only used by NR. When NR coexists with other wireless access technologies on the same carrier frequency, the maximum threshold for a carrier with a 20 MHz bandwidth is -72 dBm. When NR is the only technology using carrier frequency, for a carrier with 20 MHz bandwidth, the threshold is set to -62 dBm unless a lower threshold is required by the regulatory requirements. For uplink transmissions of NR-U/LAA, RRC signaling can also be used to configure thresholds to meet regulatory requirements.

For sidelink unlicensed access system, the transmitting power and operating bandwidth of both transmitting and receiving UE are similar to the UE of NR system. Therefore, the EDT determination method for NR-U/LAA uplink can be used as a starting point for the study of EDT determination method for sidelink unlicensed access system.

In RAN1#112bis-e meeting, the following agreement was agreed: [2]

Agreement The existing NR-U EDT procedures for uplink transmissions is taken as the baseline for SL-U in Rel-18. - FFS: details for S-SSB and PSFCH transmissions (e.g., EDT determination based on PC,MAX and/or network configured EDT, value for TA), if needed

Considering that S-SSB transmission is more like DL transmission rather than UL transmission, different EDT determination procedure should be taken for S-SSB. Since type 2A channel access procedure is performed for S-SSB transmission, the value of TA should be set to 5dB which is used in NR-U SSB transmission. Similarly, if type 2A channel access procedure is allowed to be performed for PSFCH transmission, then the value of TA should be set to 5dB.

Proposal 1: The value of TA should be set to 5dB for S-SSB transmission.

Proposal 2: The value of TA can be set to 5dB for PSFCH transmission if type 2A channel access procedure is allowed to be performed for PSFCH transmission.

Channel access procedures

Multiple nodes (eNB/gNB/UE/WiFi AP) using different radio access technologies need to coexist on unlicensed spectrum. And the coordination of resource scheduling among these nodes is not possible. To share the unlicensed spectrum fairly with LAA/WiFi, new channel access mechanisms are introduced in NR-U to assist in resource scheduling. Two channel access procedures are defined in NR-U specification: dynamic channel access procedure and semi-static channel access procedure.

The key technology for the dynamic channel access procedure is listen-before-talk (LBT), in which the transmitter needs to evaluate the channel availability prior to transmission and uses random back-off time in case of conflict. Two types of dynamic channel access procedure are defined in the NR specification: Type 1 DL/UL channel access procedure and Type 2 DL/UL channel access procedure. The former is used to start DL or UL transmission at the beginning of the COT, while the later is used for COT sharing and DRS transmission. According to the duration of the transmission gap in the COT, Type 2 channel access procedures has three sub-types:

- Type 2A, used when the transmission gap in the COT is 25us or greater (FBE), or for DRS transmission;

- Type 2B, used when the transmission gap in the COT is 16us;

- Type 2C, used when the transmission gap in the COT is up to 16us;

As analyzed above, in order to fairly co-exist with NR-U/LAA/WiFi system, channel access procedure should be introduced for the sidelink unlicensed access system. It is beneficial to use the channel access procedure defined in NR-U as a starting point for the study of channel access procedures in sidelink unlicensed access system. This is because it saves the workload of standardization.

It had been agreed at RAN1#109-e meeting that Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel. A typical application scenario for Type 1 channel access procedure is to initiate a sidelink transmission at the beginning of the COT. Similarly, for Type 2 (2A/2B/2C) channel access procedure, the typical application scenario is applied to sidelink transmission within the COT.

According to the European regulation (ETSI EN 301 893) [3], short control signaling can be used with the following limitations:

“Within an observation period of 50ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50;

and the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2 500 µs within said observation period.”

According to the limitation of short control signaling, only some relatively sparse signals/channels can use the short control signaling transmission mechanism. Considering the short transmission time of PSFCH, e.g., less than one slot, their use of short control signaling mechanism is expected to have little impact on the performance of other RATs.

In RAN1#111 meeting, RAN1 made the following agreement [4] to support Type 2A channel access procedure for S-SSB transmissions from a UE without a shared channel occupancy with a time duration and a duty cycle constraint. But the applicability of using the same channel access mechanism (Type 2A without a shared COT) for PSFCH transmission is left for further study.

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

In NR-U, the short control signaling mechanism is used for discovery burst transmission (SSB) from the gNB. However, in NR-U, the gNB still needs to perform a Type 2A channel access procedure to ensure that the channel is idle before the discovery bust transmission. It is understood to be particularly friendly to transmissions from other RATs coexisting in the same shared channel. In addition, in order to fairly co-exist with NR-U in the same shared channel, the short control signaling (i.e. PSFCH) should use the same transmission mechanism used in NR-U, i.e., perform Type 2A channel access procedure prior to PSFCH transmission.

Proposal 3: Type 2A channel access procedure is used for PSFCH transmission.

CP extension

It can be found that in a sidelink slot, there is a guard symbol at the end of the PSSCH channel or PSFCH channel. And for 15kHz/30kHz SCS, the guard symbol can be 70.3us/35.2us respectively, which is greater than 25us. In unlicensed spectrum, when the gap is greater than 25us, the next transmission will need to use a Type 1 channel access procedure, which increases the probability of losing the channel. In order to use the Type 2 channel access procedure, it is necessary to ensure that the gap between two consecutive transmissions is small enough, even less than the duration of a symbol. This can be solved by CP extension that makes the transmission start earlier than the symbol boundary.

In RAN1#113 meeting, the following agreement was made [5]:

Agreement A set of one or more candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT (for the case of sharing a COT) and outside a COT (for the case of initiating a COT) is separately (pre-)configured per resource pool based on the pre-defined set of all candidate CPE starting positions. - Note: for the case of sharing a COT, the CPE occurs after LBT gap for type 2A/2B/2C - FFS whether a subset of candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT is indicated by SCI carrying COT sharing information - FFS whether default starting position is included in each set

For the case of sharing a COT, in NR-U, different CP extension duration is applied to generate different gaps between consecutive transmissions. These CP extension duration are indicated by DCI from gNB. In SL-U, the COT-initiating UE may share the COT to multiple responding UEs. To avoid conflicts, the transmissions from different responding UEs should be aligned in time domain. In this case, same CPE starting position can be dynamically indicated by the COT-initiating UE for multiple responding UEs to access the shared COT.

Proposal 4: A subset of candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT is indicated by SCI carrying COT sharing information.

COT sharing

In RAN1#111 meeting, the following agreement was made [4]:

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

Regarding PSFCH transmission, when a UE needs to transmit multiple SL HARQ feedback in a PSFCH occasion, the SL HARQ feedback may be for different PSSCH transmissions from different UEs. In this case, some SL HARQ feedback may be within the same COT as the PSSCH transmitting UE, while some may be outside the COT. Therefore PSFCH transmitting UEs may need to perform different types of LBTs that have different sense lengths and may result in using different starting positions of CPEs to access the channel. In this sense, it is not fair for the PSFCH transmitting UE to self-block its own transmission. Therefore, a better approach is that when a responding UE is transmitting at least one PSFCH intended for the COT initiating UE, the responding UE is able to use the COT to send SL HARQ feedback to other UEs in the same PSFCH occasion.

In last meeting the following proposal was discussed but no consensus was achieved: [6]

Proposal 5-2 (III): - When receiving a grant/indication to use a PSFCH occasion in a shared COT, a responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator. - FFS: details on the grant/indication to use a PSFCH occasion in a shared COT

Since more than one PSFCH occasion can be configured, and these PSFCH occasions can be dynamically indicated as available or unavailable by the UE initiating the COT during the COT. For example, if there is a slot with a PSFCH occasion on the COT and the COT initiator neither expects to receive a PSFCH nor has a PSFCH to transmit, then it can indicate the PSFCH occasion as unavailable. In other hand, if the COT initiator UE indicate a PSFCH occasion as available, then the responding UE can transmit the PSFCH to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator. Because more transmission ensures continuity of transmission on the COT.

Proposal 5: When receiving a grant/indication to use a PSFCH occasion in a shared COT, a responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator.

In RAN1#112bis-e meeting, the following agreement was made [2]:

Agreement At least the following information should be used as part of COT sharing information from the COT initiator UE. - CAPC used for initiating the COT - Existing / legacy R16/17 L1 source and destination IDs - FFS additional ID(s) - Time domain information of the shared COT - FFS: starting offset, number of slots, [remaining or total] COT duration, or a combination of them - Frequency domain information of the shared COT - FFS applicable RB set(s), FRIV, and any other(s) - FFS: how each of the above is indicated. - Note, other information is not precluded.

In RAN1#113 meeting, the following agreement was made [5]:

Agreement For the time-domain information to be included as part of COT sharing information, at least the following is included: - Remaining COT duration - FFS it is an absolute time length in ms or in number of slots, and payload size - FFS: how to determine the shared slots and the starting time of the shared slots, e.g. if some slots are only intended for the COT-initiating UE and not to be shared with other UEs

The existing 1st-stage-SCI is used for scheduling of PSSCH and 2nd-stage-SCI on PSSCH, while the 2nd-stage-SCI is used for the decoding of PSSCH or providing inter-UE coordination information or requesting inter-UE coordination information. Therefore, in order to carry COT sharing information, a new SCI format needs to be designed for SL-U.

In mode 2, the sidelink UE requires sensing the resources and therefore needs to blind decode the 1st-stage-SCI. If a new 1st-stage-SCI with different payload size is defined, then the complexity of blind decoding increases for the sensing UE, which is not desirable. Therefore, it is preferable to reuse the 1st-stage-SCI. It is important to note that the transmission of a responding UE is limited by the CAPC of the COT-initiating UE’s transmission. It is therefore advantageous to notify the CAPC value at 1st-stage-SCI, so as to avoid allowing responding UEs whose CAPC values of the transmission are greater than the indicated CAPC value to continue decoding the 2nd-stage-SCI.

Proposal 6: Reuse the 1st-stage-SCI, which also includes CAPC value from the COT sharing information, and other COT sharing information should be included in 2nd-stage-SCI.

In most cases, for a COT-initiating UE, it has a need to transmit PSSCH, so it needs to transmit PSCCH at the beginning of the COT. If COT-initiating UE transmits a PSCCH carrying the COT sharing information, the responding UE will need to monitor both SCI formats, which will increase the complexity of blind decoding. A simple solution to the issue of blind decoding complexity is to use a single 2nd-stage-SCI to provide both the COT sharing information and the decoding information of PSSCH. Therefore, a new 2nd-stage-SCI named SCI format 2-D can be defined and indicated in 1st-stage-SCI. In other cases, there may be no need for COT-initiating UE to transmit PSSCH, e.g., it needs to transmit PSFCH or S-SSB. In this case, it is not necessary for the 2nd-stage-SCI to provide the decoding information for the PSSCH, only the COT sharing information. In order to satisfy both of the above cases, a field in the 2nd-stage-SCI is used to indicate whether the decoding information of the PSSCH is included in the 2nd-stage-SCI.

Proposal 7: A new SCI format 2-D is used for providing the COT sharing information and/or the decoding of PSSCH.

Proposal 8: A field in the SCI format 2-D is used to indicate whether the decoding information of the PSSCH is included in the SCI format 2-D.

Regarding the remaining COT duration, the sidelink slots within a resource pool may be not consecutive, because they may be separated by the S-SSB slots. In this sense, if the remaining COT duration is interpreted in number of slots, it may exceed the maximum COT duration, which is an absolute time. Therefore, to simplify the design, the remaining COT duration should be expressed as an absolute time length in milliseconds.

Proposal 9: The remaining COT duration should be expressed as an absolute time length in milliseconds.

In sidelink COT sharing, the COT-initiating UE cannot schedule other sidelink UE (eg. responding UE), which is similar to the UL-to-DL COT sharing in NR-U. In this case, the responding UE should know when to use the shared COT, otherwise the responding UE needs to keep sensing the channel until the channel is available. Furthermore, in order to enable simultaneous transmission of multiple responding UEs, the transmission of the responding UEs should be aligned in the time domain. Therefore, the starting time of the shared slots and the duration of the shared slots should be indicated to the responding UEs. With these information, the responding UE can determine the starting time of the shared slots and perform channel access type 2A/2B/2C before that time. The responding UE can also determine the length of duration that can be used for transmission. Therefore, these information can be included as part of COT sharing information.

Proposal 10: The starting time of the shared slots and the duration of the shared slots should be included as part of COT sharing information and indicated in SCI.

Regarding the frequency domain indication, when the COT-initiating UE does not need to transmit PSSCH, e.g., it needs to transmit PSFCH or S-SSB, then an explicit frequency domain indication is required to provide the responding UE with information of available RB set(s).

Regarding additional ID, it is agreed that not only the PSSCH intended UE is allowed to share the COT, but also the responding UEs that can receive the COT sharing information. Then the additional ID can be used to determine which UEs are allowed to share the COT. For example, UEs with source IDs within the indicated set of additional IDs may share COT.

Proposal 11: Applicable RB set(s) and additional ID(s) should be included in the COT sharing information and indicated in SCI.

Multi-channel access

In RAN1#111 meeting, the following agreement was made [4]:

Agreement - For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

In NR-U, for the DL multi-channel access procedure, the gNB is required to perform LBT independently on each shared channel or RB set that the gNB intends to transmit. gNB transmits on the channel on which the channel access procedure succeeds. On the other hand, for the UL multi-channel access procedure, the UE needs to perform LBT on all shared channels or RB sets that it intends to transmit. The UE can transmit only if the channel access procedure is successful for all channels. Otherwise, the UE does not transmit on any channel.

DL multi-channel access procedure can also be applied if S-SSB can be configured within each RB set and multiple S-SSBs are supported within multiple RB sets.

Proposal 12: The NR-U DL multi-channel access procedure should be used as baseline for the transmission of S-SSB for multiple SSB transmissions on multiple channels.

FBE operation

FBE operation can be applied with the guarantee that the LBE nodes are absent on a long term basis (e.g., by level of regulation). Compared to LBE operation, FBE operation achieves lower channel access complexity because there is no need to perform random fallback. And if FBE nodes are synchronized, the network can achieve the following: Ability to use frequency reuse factor 1. Considering the benefits of lower channel access complexity and higher spectrum efficiency, FBE operation should be studied in sidelink unlicensed access system.

Proposal 13: FBE operation should be studied in sidelink unlicensed access system.

Conclusion

In this contribution, we focus on the channel access mechanism for sidelink in unlicensed spectrum, including EDT determination, channel access procedures, CP extension, COT sharing, multi-channel access and FBE operation. Based on the discussion in section 2, we provide the following proposals:

Proposal 1: The value of TA should be set to 5dB for S-SSB transmission.

Proposal 2: The value of TA can be set to 5dB for PSFCH transmission if type 2A channel access procedure is allowed to be performed for PSFCH transmission.

Proposal 3: Type 2A channel access procedure is used for PSFCH transmission.

Proposal 4: A subset of candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT is indicated by SCI carrying COT sharing information.

Proposal 5: When receiving a grant/indication to use a PSFCH occasion in a shared COT, a responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator.

Proposal 6: Reuse the 1st-stage-SCI, which also includes CAPC value from the COT sharing information, and other COT sharing information should be included in 2nd-stage-SCI.

Proposal 7: A new SCI format 2-D is used for providing the COT sharing information and/or the decoding of PSSCH.

Proposal 8: A field in the SCI format 2-D is used to indicate whether the decoding information of the PSSCH is included in the SCI format 2-D.

Proposal 9: The remaining COT duration should be expressed as an absolute time length in milliseconds.

Proposal 10: The starting time of the shared slots and the duration of the shared slots should be included as part of COT sharing information and indicated in SCI.

Proposal 11: Applicable RB set(s) and additional ID(s) should be included in the COT sharing information and indicated in SCI.

Proposal 12: The NR-U DL multi-channel access procedure should be used as baseline for the transmission of S-SSB for multiple SSB transmissions on multiple channels.

Proposal 13: FBE operation should be studied in sidelink unlicensed access system.

Reference

- RP-220300		WID revision: NR sidelink evolution, 	OPPO, 	2022-03

- Draft_Minutes_report_RAN1#112bis-e_v010, 2023-04

- ETSI EN 301 893

- Draft_Minutes_report_RAN1#111_v020, 2022-11

- Draft_Minutes_report_RAN1#113_v010, 2023-05

- R1-2303975	FL summary for AI 9.4.1.1: SL-U channel access mechanism (EOM),	OPPO