3GPP TSG RAN WG1 #114						R1-2307868

Toulouse (France), 21 – 25 Aug 2023

Source:		Continental Automotive

Title:	Considerations on Resource Allocation and Congestion Control for Sidelink Positioning

Agenda item:	9.5.1.3

Document for:	Discussion and decision

# Introduction

The scope of the work item on Expanded and Improved NR Positioning [1] includes specifying solutions for sidelink (SL) positioning. One of the objectives with respect to SL positioning is to specify support of resource allocation for SL positioning reference signal (PRS) as given in the below excerpt from the objectives of the work item description (WID) [1]:

- Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

In this contribution we wish to highlight some issues that could arise in meeting the above objectives, and solutions to help address them.

# Discussion

## Mapping between SCI and SL PRS resources

In the RAN WG1 #113 meeting the following was agreed regarding SCI for SL PRS for the dedicated resource pool (RP):

Agreement In the dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a - SL-PRS-resource-based allocation - SCI for SL-PRS should at least indicate the following values: - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - Cast type - With regards to the SL-PRS configuration and/or SL-PRS time assignment information, select one alternative at RAN1#114: - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and an associated SL-PRS resource in the same slot. - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot - Note: Same number of PSCCH resource(s) and SL-PRS resource(s) - Alt. 3.2: explicit signaling of SL PRS resource in the same slot - Alt. 3.3: support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot and explicit signaling of SL PRS resource - Only a one-to-one mapping is used between a PSCCH resource and an associated SL-PRS resource in the same slot if explicit signalling is not used - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped - FFS: details, including (pre)configuration - FFS: Whether and how to indicate SCI resource(s) or SL-PRS resource (s) for a future slot - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods

Similarly, in the same meeting, the following was agreed regarding SCI for SL PRS for the shared RP:

Agreement With regards to the SCI signaling in a shared resource pool, - Support a new format for 2nd stage SCI. - FFS how to indicate the new 2nd stage SCI format - FFS: If a 2nd stage SCI indicates both SL-PRS and SL-SCH, the cast type, destination ID, source ID are shared. Agreement In shared resource pools, - With regards to PSCCH and SL-PRS multiplexing, support Alt. B.1. from previous agreement (i.e., Only TDMing is supported) Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported for the already agreed comb sizes 1, 2, 4 Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

Note that while for dedicated RPs, the agreement includes Alt 3.3 where “a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot” is possible, there has been no such consideration for the shared RP so far.

For SL communication there is a one-to-one mapping between the PSCCH and the PSSCH, since there is only one PSSCH to be indicated. However, for SL positioning, there can be multiple SL PRS resources from the same UE within the same slot, by virtue of comb-based multiplexing of SL PRS resources even if with the same (M, N) values, where N denotes the comb-size and M denotes the number of OFDM symbols in the time domain occupied by the SL PRS resource, which is outlined by the following agreements from previous meetings.

RAN1 #112:

Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M, N) value is possible. - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M, N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

RAN1 #113:

Agreement Comb-based multiplexing of SL PRS resources from different UEs in a slot is NOT supported for shared resource pools. Conclusion TDM-ed SL PRS resources within a slot from a single UE in a dedicated/shared resource pool is not supported in Rel-18. Agreement Multiple (M,N) pairs within a slot in a dedicated resource pool is supported only when the different (M, N) pairs are always multiplexed via TDM to different sets of symbols in a slot. Only a single (M,N) value can be mapped within one TDM duration (i.e. one set of symbols).

Based on the above agreements, we believe that a one-to-many mapping relationship between the SCI and SL PRS resource(s) is both necessary and beneficial, since in this case, multiple PSCCHs or SCIs do not need to be transmitted for indicating the multiple SL PRSs in the same slot.

Observation 1.1: It is beneficial to have a one-to-many mapping between one SCI and multiple SL PRS resources multiplexed in the same slot by the same UE.

In the study item [2], it has been agreed to use the existing PRS/SRS design as a starting point for the SL PRS design, as quoted below:

New reference signal designs for SL positioning/ranging, referred to as SL PRS, are studied using the existing PRS/SRS design and SL design framework as starting points.

Moreover, in RAN WG1 #112, the following agreement to consider the RE-offset sequences of DL PRS as a starting point was made:

Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences

Following the spirit of the above directions, we believe that the SL PRS resources by the same UE in the same slot can be multiplexed in terms of either the RE-offset or the first symbol of the SL PRS resources in the slot. To recall from TS 38.211 [3], for a DL PRS resource, the mapping of the sequence rm, m=0,1,… to resource elements (REs) k,lp,μ, where k is the index of the RE in the frequency domain and l in the time domain, is according to the following rules:

k=mKcombPRS+koffsetPRS+k'mod KcombPRS

l=lstartPRS,lstartPRS+1,…,lstartPRS+LPRS-1

In the above, lstartPRS  is the first symbol of the DL PRS within a slot and given by the higher-layer parameter dl-PRS-ResourceSymbolOffset, LPRS∈2,4,6,12 is the size of the DL PRS resource in the time domain, given by the higher-layer parameter dl-PRS-NumSymbols, KcombPRS∈2,4,6,12 is the comb size given by the higher-layer parameter dl-PRS-CombSizeN with additional restrictions on the combination of {LPRS,KcombPRS}, and koffsetPRS∈0,1,…,KcombPRS-1 is the resource-element offset obtained from the higher-layer parameter dl-PRS-CombSizeN-AndReOffset. An example of a DL PRS resource is shown in Fig. 1.

Figure 1: Example of DL PRS resource.

DL PRS Resource:

Let KcombPRS=6,koffsetPRS=5, lstartPRS=0, LPRS=6.

Then r0, r1,… are at: k=5,11,… for l=0, k=2,8,… for l=1, k=0,6,… for l=2, and so on.

l=

0

1

2

3

k=

0

1

2

3

4

11

11

We propose that multiple SL PRS resources transmitted within a slot by the same UE should be indicated by a single associated SCI, which may be carried in the PSCCH for a dedicated RP, and in the 2nd-stage SCI for a shared RP. The information on the locations of the multiple SL PRS resources within the slot can be encoded using a relationship between the locations of the multiple SL PRS resources. The relationship between the locations of the multiple SL PRS resources may be indicated in terms of the parameters that determine the SL PRS pattern within the slot.

Fig. 2 shows an example where three SL PRS resources are multiplexed with M=N=6 for all three, but with different RE offsets for each SL PRS resource, given by koffsetPRS=1, koffsetPRS=3 and koffsetPRS=5. In this case the RE offsets of the three SL PRS resources are increments of 2 from the previous SL PRS resource. Thus, multiple SL PRS resources may be indicated by a single PSCCH in the dedicated RP or a single 2nd-stage SCI in the shared RP by indicating the following:

- The type of relationship between the SL PRS resources in the slot (where the PSCCH or part of the SCI is transmitted); in the example of Fig. 2, this is the RE offset. This maybe indicated by one bit, if there are only two types of relation (e.g., the RE offset and the starting symbol). If there is only one type of relation, no indication is necessary. The maximum number of bits depend on the number of possible types of relationships between the SL PRS resources in the slot.

- The total number of SL PRS resources in the slot. In Fig. 2, this value is 3. This is determined by the values of M and N and the type of relationship. E.g., with M=N=6 there are possibly 6 SL PRS resources that can be multiplexed based on the difference in RE-offsets, and hence 3 bits are required. In the worst case, for the relation in terms of RE offset, this would need log2N bits.

- The value of the related parameter for the first SL PRS resource (which may be defined as the resource with the lowest value of the related parameter) in the slot. In Fig. 2, koffsetPRS=1 for SL PRS resource 1. This is again determined by M and N and the type of relationship. In the worst case, for the relationship in terms of RE offset, this would need log2N bits.

- The increment to get the value of the related parameter for the next SL PRS resource in the slot. In Fig. 2, this is 2, and in general the number of bits required to indicate this would again depend on M and N, the type of relationship, and the total number of SL PRS resources in the slot. In the worst case, for the relation in terms of RE offset, it would need log2N bits.

Figure 2: Example of one-to-many mapping of a single SCI to multiple SL PRS resources in the same slot.

PSCCH (or part of SCI)

SL PRS Resource 3

l=

0

1

2

3

0

1

2

3

4

11

11

SL PRS Resource 1

SL PRS Resource 2

k=

If we were to indicate these SL PRS resources separately, or even by explicit time-frequency indication, it would consume more PSCCH/SCI resources than this scheme we suggest. Thus, we conclude that it is possible and beneficial to have a one-to-many mapping between a single SCI and multiple SL PRS resources in a slot, with an indication in terms of the relationship between the parameters of the multiple SL PRS resources. This brings us to the following proposal.

Proposal 1.1: Support a mapping relationship between the PSCCH/SCI and SL PRS, whereby multiple SL PRS resources may be indicated by a single PSCCH (in the dedicated RP) or 2nd-stage SCI (in the shared RP), i.e., support a one-to-many mapping relationship.

Proposal 1.2: The mapping can exploit a relationship in terms of a parameter between the multiple SL PRS resources to implicitly signal all the parameters necessary to establish the location of the multiple SL PRS resources.

## Congestion control for SL positioning

In the RAN WG1 #112-bis-e meeting the following agreement was reached on congestion control mechanisms [4].

Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Further in RAN WG1 #113, the following proposal was put forward for congestion control in a dedicated RP based on the discussions [5]:

[MEDIUM] Feature Lead Proposal 3.5.3.2-v1 For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, using the congestion control mechanisms from Rel.16 NR SL as a starting point, consider the following modifications: - Modification 1: For the definition of SL PRS CR and CBR: - Alt. 1: Reuse the NR SL legacy definition (i.e., based on the number of subchannels in a slot) - Suport: Continental, Futurewei, CATT, Qualcomm - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration - FFS: Details, including: - Whether the definition should be different depending on whether comb-based or TDM-based multiplexing is possible in a resource pool - whether there is a need to define a “SL-PRS allocation unit” - whether to define a separate CBR for different comb-offsets in a slot - Support: ZTE, OPPO, Nokia, NSB - Modification 2: For the evaluation of RSSI used in the CBR definition: - RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS - Support: ZTE, Continental, FUturewei, CATT, OPPO, Nokia, NSB - Modification 3: For the CR and CBR measurement time window size, - it can be separately configured for a dedicated resource pool and could take the values: - Alt. 1: 100ms or 100·2^µ slots - ZTE, Continental, Futurewei, CATT, OPPO, Nokia, NSB - Alt. 2: other (Please provide a different option) - Modification 4: For the congestion control processing time, - It should be based on both SCS and a new UE capability-(ies) - FFS: Details - Note: other potential modifications are not precluded

Regarding modification 1, our proposal is to reuse the NR SL legacy definition based on number of subchannels in a slot.

Proposal 2.1: In a dedicated RP, the UE could follow a similar congestion control mechanism as used for SL communication, but with corresponding measurements SL-P RSSI, SL-P CR and SL-P CBR performed on resources used for SL positioning.

For congestion control in a shared RP, the following proposal was put forward in RAN WG1 #113 [5]:

[MEDIUM] Feature Lead Proposal 3.5.3.1-v1 For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, - CBR and CR mechanisms from Rel.16 NR SL are reused, with the following potential changes: - Modification 1: with regards to the evaluation of the CR and CBR - One CR/CBR shall be evaluated for both SL-PRS and PSSCH. - Modification 2: with regards to the RSSI evaluation for CBR - Alt. 1: Keep existing SL RSSI definition in 38.215: - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol. - Alt. 2: Explicitly include SL-PRS in the SL RSSI definition in 38.215 as follows: - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH or additionally for SL-PRS, starting from the 2nd OFDM symbol.

Since a shared RP is used for both SL communication and SL positioning, if using the conventional method, the UE would need to perform congestion control for communication and positioning separately. However, this would imply that separate CBR tables be maintained for communication and positioning in a shared RP and separate indications about these be provided to the UE by the network.

Observation 2.1: In a shared RP, using separate measurements for SL positioning and SL communication congestion control creates additional overhead and signaling requirements.

To avoid this additional signaling and computation in the same RP, we propose to use a new set of shared measurements and shared priority values, which may be computed using an appropriate weighting between SL communication and positioning indicated by the network.

In essence we envision that all the shared measurements and the shared priority value may be computed using a new weight parameter that may be indicated by the network for each RP. If 0≤ω≤1 is a weighting parameter for the shared RP, the UE can compute a shared priority value as ishared=ω⋅ipos+1-ω⋅icomm, where ipos is the SL positioning priority value in the slot and icomm is the SL communication priority value in the slot. E.g., if ω=0.5, ipos=2 and icomm=3, then ishared=1+1.5=3 .

The new measurements in the shared RP are the following:

- SL shared received signal strength indicator (SL-Sh RSSI)

- SL shared channel occupancy ratio (SL-Sh CR) which may be computed for each SL shared priority value

- SL shared channel busy ratio (SL-Sh CBR)

More details on the definitions of the shared measurements are in our companion contribution to RAN WG1 #114 [5]. Essentially, they are constructed by weighting the measurements for congestion control on the resources used for SL positioning and SL communication using the weighting parameter ω provided by the network.

Proposal 2.2: Use shared measurements and a shared priority value for congestion control for both SL positioning and SL communication in a shared RP.

Proposal 2.3: The shared measurements in the shared RP may be computed by the UE by appropriately weighting and combining the measurements on SL positioning and SL communication resources using a weighting parameter provided by the network.

## RP selection for exchange of SL PRS by UEs (pre-)configured with multiple RPs

In RAN1#112-bis-e it was agreed that a UE could be (pre-)configured with multiple dedicated and/or shared RPs.

Agreement For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier. - A UE can be (pre-)configured with one or more dedicated SL resource pools. - A UE can be (pre-)configured with one or more shared SL resource pools.

However, in this case, it is not clear which RP a UE should choose for SL PRS transmission. If the decision of which RP should be used for transmission is left to UE implementation, UEs may always choose dedicated RP(s) (if available), since there is more bandwidth available, eventually causing excessive congestion in those, while the shared RP(s) remain unused. Or if multiple RPs of the same kind (either dedicated or shared) are (pre-)configured for the same UE, it is not clear which RP a UE should select for a SL positioning operation.

Observation 3.1: When dedicated and/or shared RPs are (pre-)configured for a UE, it is not clear which RP a UE should select for a particular SL positioning session.

Therefore, we need methods to enable a UE to select between multiple dedicated and/or shared RP(s). To this end we propose that whenever a UE is (pre-)configured with more than one dedicated and/or shared RP(s), it is provided an indication by the network to use dedicated and/or shared RP(s) for SL positioning, where the network determines the indication based on the priority/ QoS of SL positioning and/or other parameters (e.g., geographical zone ID the UE is in).

The priority/ QoS of an SL positioning service may be mapped to a physical layer priority/QoS indication and the physical layer priority/ QoS indication for SL positioning must be transmitted as part of the control information in dedicated and/or shared RP(s), like the priority indication in SL communication.

The UE may be provided a mapping between the physical layer priority/ QoS value and the type and identification (ID) of the RP(s) to be used for the SL positioning transmission corresponding to this priority/ QoS value by the network; this maybe provided via system information or RRC (pre-)configuration.

Proposal 3.1: When dedicated and/or shared RPs are (pre-)configured for a UE, the network indicates to the UE which RP to select.

Proposal 3.2: The UE selects the RP based on parameters related to the SL positioning operation (e.g., priority).

## RP selection for SL positioning measurement reporting

In the RAN WG1 #114 meeting, it was agreed that a dedicated RP would not contain the PSSCH.

Agreement PSSCH is not included in dedicated resource pool for SL positioning.

This implies an underlying assumption that when a dedicated RP is assigned to a UE for SL positioning, at least one shared RP or SL communication RP should also be assigned to the UE. However, if a UE has been assigned more than one shared RP and/or SL communication RP, it is not clear which RP should be used for exchange of SL positioning-related data with another UE. If left to implementation, each UE could choose a different shared RP or SL communication RP, which would make it impossible to exchange SL positioning related data among them. On the other hand, if multiple UEs choose the same share RP or SL communication RP, it could cause unnecessary congestion on that RP.

Observation 4: When a UE is assigned a dedicated RP and more than one shared RP and/or SL communication RP, it is not clear which shared RP or SL communication RP it should choose to exchange SL-positioning related data with other UEs.

To avoid the above problem, we suggest that whenever a UE is assigned at least one dedicated RP along with multiple shared RP(s) and/or SL communication RP(s), it should be provided an indication of the shared RP and/or SL communication RP associated with each dedicated RP. This indication may be provided by the network (pre-)configuration together with the dedicated RP configuration.

Proposal 4.1: When a UE is assigned at least one dedicated RP along with multiple shared RP(s) and/or SL communication RP(s), it should be provided an indication of the shared RP and/or SL communication RP associated with each dedicated RP. The indication may be provided by the network together with the dedicated RP (pre-)configuration.

# Conclusion

In this contribution we have highlighted some issues and potential solutions related to resource allocation for SL positioning and made the below observations and proposals:

Observation 1.1: It is beneficial to have a one-to-many mapping between one SCI and multiple SL PRS resources multiplexed in the same slot by the same UE.

Proposal 1.1: Support a mapping relationship between the PSCCH/SCI and SL PRS, whereby multiple SL PRS resources may be indicated by a single PSCCH (in the dedicated RP) or 2nd-stage SCI (in the shared RP), i.e., support a one-to-many mapping relationship.

Proposal 1.2: The mapping can exploit a relationship in terms of a parameter between the multiple SL PRS resources to implicitly signal all the parameters necessary to establish the location of the multiple SL PRS resources.

Proposal 2.1: In a dedicated RP, the UE could follow a similar congestion control mechanism as used for SL communication, but with corresponding measurements SL-P RSSI, SL-P CR and SL-P CBR performed on resources used for SL positioning.

Observation 2.1: In a shared RP, using separate measurements for SL positioning and SL communication congestion control creates additional overhead and signaling requirements.

Proposal 2.2: Use shared measurements and a shared priority value for congestion control for both SL positioning and SL communication in a shared RP.

Proposal 2.3: The shared measurements in the shared RP may be computed by the UE by appropriately weighting and combining the measurements on SL positioning and SL communication resources using a weighting parameter provided by the network.

Observation 3.1: When dedicated and/or shared RPs are (pre-)configured for a UE, it is not clear which RP a UE should select for a particular SL positioning session.

Proposal 3.1: When dedicated and/or shared RPs are (pre-)configured for a UE, the network indicates to the UE which RP to select.

Proposal 3.2: The UE selects the RP based on parameters related to the SL positioning operation (e.g., priority).

Observation 4: When a UE is assigned a dedicated RP and more than one shared RP and/or SL communication RP, it is not clear which shared RP or SL communication RP it should choose to exchange SL-positioning related data with other UEs.

Proposal 4.1: When a UE is assigned at least one dedicated RP along with multiple shared RP(s) and/or SL communication RP(s), it should be provided an indication of the shared RP and/or SL communication RP associated with each dedicated RP. The indication may be provided by the network together with the dedicated RP (pre-)configuration.

# References

- RP-223549, New WID on Expanded and Improved NR Positioning, 3GPP TSG RAN Meeting #98-e; Intel Corporation, CATT, Ericsson.

- 3GPP TR 38.859 V18.0.0 (2022-12); Technical Specification Group Radio Access Network; Study on expanded and improved NR positioning; (Release 18).

- 3GPP TS 38.211 V17.5.0 (2023-06); NR; Physical channels and modulation (Release 17).

- R1-2306238; 3GPP TSG RAN WG1 Meeting #113; RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning; Rapporteur (Intel Corporation).

- R1-2306171; 3GPP TSG RAN WG1 #113; Moderator Summary #3 on resource allocation for SL PRS; Moderator (Qualcomm)

- R1-2307870; 3GPP TSG RAN WG1 #114; On Measurements for SL Positioning Congestion Control; Continental Automotive.