3GPP TSG RAN WG1 #114	        	R1-2308007

Toulouse, France, Aug 21st – Aug 25th, 2023

Source: 	CEWiT

Agenda:	9.5.1.3

Title:	On SL positioning resource allocation for SL positioning

Document for:	Discussion & Decision

# Introduction

Rel. 18 is continuing the enhancement of positioning support in NR and a study on expanded and improved NR positioning is concluded with a TR 38.859 [1] in RAN#98. Study was focussing on improving the accuracy of the positioning methods and extending the positioning to different verticals including sidelink. The positioning framework is being extended over sidelink to guarantee more accurate localization for many critical sidelink applications like V2X, IIoT public safety and commercial sidelink (D2D) in in-coverage, partial coverage, and out-of-coverage NR positioning use cases as emphasized in TR 38.845.

In the last three RAN 1 meeting, the normative work has been undertaken [2]. This contribution is targeting the resource allocation for sidelink positioning reference signal (SL-PRS) agenda considering dedicated SL positioning resource pool and legacy SL resource pool shared for SL-PRS transmission for network assisted resource configuration (scheme 1) and autonomous resource configuration by SL UEs (scheme 2). The present contribution provides the further details on positioning resource allocation schemes for SL-PRS resource configuration, triggering and reporting considering previous agreements.

# SL resource pool

For sidelink positioning, two type of resource pool will be supported. A dedicated and shared legacy resource pool can be used for SL-PRS resource configuration and measurement reporting in Rel. 18. Dedicated resource pool will be important to have more flexibility to manage SL-PRS resource bandwidth to achieve desired accuracy and latency constraints. The following agreement is drawn in RAN1#112 meeting regarding the SL resource pool.

Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

Further it is agreed that SL BWP can include both dedicated and shared resource configurations simultaneously. Only one SL-BWP will be active in the UE at any point in time and UE might be doing SL legacy operation as well as positioning measurement/estimation simultaneously. Therefore, it is necessary to configure the dedicated and shared resources in the same SL-BWP. Following agreement is drawn in the last meeting regarding this.

Agreement

For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier.

- A UE can be (pre-)configured with one or more dedicated SL resource pools.

- A UE can be (pre-)configured with one or more shared SL resource pools.

Some open issues on resource pool are remaining to decide. The detailed discussion on dedicated and legacy resource pool (RP) is in the following section focusing on open issues.

## Dedicated Resource Pool:

The reason for introducing dedicated resource pool is that the bandwidth requirement to achieve the SL positioning accuracy for various use cases which may not be satisfied by legacy resource pool or may be difficult to control the sub-channel configuration for positioning purpose in presence of legacy sidelink channels. So, it will be beneficial to have dedicate resource pool for positioning purpose. Additionally, it will avoid the backward compatibility issues.

Configuration parameters for Dedicated Resource Pool

In the last meetings, design of slot structure for dedicated resource pool is discussion with following agreement on multiplexing of SL-PRS with other channel and decided that dedicated resource pool will have only SL-PRS and associated PSCCH. The following are the agreements regarding this.

Agreement

For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot.

- FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Agreement

For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH.

Agreement

PSSCH is not included in dedicated resource pool for SL positioning.

Further it is agreed that the set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool. Further, it is agreed that the bandwidth will be same as configured BWP. It is important to configure the SL-PRS resources associated with dedicated resource pool. Multiple SL-PRS can be configured per D-RP by higher layers (SLPP or Tx UE higher layer). The mapping of these SL-PRS will be conveyed by lower layer signalling i.e., in SCI-1B.

Proposal 1: For dedicated resource pool, multiple SL-PRS resources should be configured per resource pool. Maximum SL-PRS resource per resource pool can be 8.

SCI for SL-PRS

For dedicated resource pool, it is agreed to use new 1st stage SCI format i.e., SCI-1B. Following agreement is drawn in the last meeting regarding this.

Agreement

In the dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - SL-PRS-resource-based allocation

- SCI for SL-PRS should at least indicate the following values:

  - Source ID

  - Destination ID

  - Resource reservation period

  - SL-PRS Priority

  - Cast type

  - With regards to the SL-PRS configuration and/or SL-PRS time assignment information, select one alternative at RAN1#114:

    - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and an associated SL-PRS resource in the same slot.

      - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot

      - Note: Same number of PSCCH resource(s) and SL-PRS resource(s)

    - Alt. 3.2: explicit signaling of SL PRS resource in the same slot

    - Alt. 3.3: support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot and explicit signaling of SL PRS resource

      - Only a one-to-one mapping is used between a PSCCH resource and an associated SL-PRS resource in the same slot if explicit signalling is not used

        - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped

        - FFS: details, including (pre)configuration

    - FFS: Whether and how to indicate SCI resource(s) or SL-PRS resource (s) for a future slot

  - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods

Dedicated resource pool can be configured or pre-configured with one or more than one SL-PS resource. SL PRS resource will be indicated by the resource ID in the new 1st stage SCI (SCI-1B). Maximum 4 or 8 SL PRS should be able to configure using SCI. Each SL-PRS resource can be configured with the same or different parameters like comb size, repetition, sequence ID etc.  If only one SL PRS is configured, then no need of explicit signaling in SCI. Therefore, we are supporting of the Alt 3.3 from the above agreement.

Proposal 2: With regards to the SL-PRS configuration and/or SL-PRS time assignment information in first stage SCI, support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot. Explicit signaling of SL PRS resource should be supported in dedicated resource pool.

- If explicit signalling is not used, then a one-to-one mapping is assumed between a PSCCH resource and an associated SL-PRS resource in the same slot.

  - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped

- Resource ID should be included in SCI. Maximum 8 SL-PRS can be configured. This indication is signalled using bit map.

Further, it is agreed in the previous meeting that UE-A can trigger UE-B to transmit the SL-PRS using lower-layer signalling using SCI. So, SCI should include an SL PRS trigger for Rx UE. This can be one bit trigger or multiple bit trigger. Multiple bit will select the SL-PRS to trigger.

Positioning session ID is not necessary to convey in lower layer as we do not see benefit to convey it to the Rx UE. Whereas it will take care by higher layer configuration like SLPP message which is being discussed in RAN 2.

Proposal 3: In dedicated resource pool, the new 1st stage SCI should include SL PRS trigger for Rx UE. The SL-PRS trigger can be one bit or multiple bits.

Proposal 4: In dedicated resource pool, the new 1st stage SCI should not include positioning session ID.

PSCCH resources:

Regarding the PSCCH mapping in dedicated resource pool following agreement is drawn in the last meeting,

Agreement

In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects:

- The first PSCCH symbol is mapped to the 2nd symbol available for SL transmissions in a slot

  - Note: 1st symbol available for SL transmissions in a slot is for PSCCH AGC similar to legacy

- PSCCH DM-RS in the slot is being reused from legacy

- The number of PSCCH symbol(s) is (pre-)configured to (down-select at RAN1#114):

  - Alt. 1: 2 or 3 symbols (same as legacy)

  - Alt. 3: 1, or 2 or 3 symbols

- The number of PRBs is (pre-)configured using the legacy values.

  - FFS: reconsider if 1-symbol PSCCH is supported

Open issue in this is the number of PSCCH symbols to be (pre)-configured in dedicated resource pool. Regarding D-RP, it is possible that the configured bandwidth is larger than the subchannel BW in the legacy SL resource pool. Therefore, in some cases one symbol itself will be sufficient. Therefore, it should be possible to configure 1 symbol PSCCH unlike legacy shared RP. This will be efficient to optimise the dedicated resource pool resource usage.

For frequency domain resources in legacy sidelink specification, the number of PRBs is configured by sl-FreqResourcePSCCH parameter and takes value {n10, n12, n15, n20, n25}and is limited by subchannel size. Here the BW of dedicated-RP can be wider than the sub-channel in the shared resource pool, so it is possible to configure more PRBs and if 1 symbol PSCCH is supported larger PRB is necessary. So, values should be extended. Additionally {n30, n40, n50} should be supported.

Proposal 5: In a dedicated resource pool, with regards to the PSCCH time domain configuration, 1 or 2 or 3 symbols should be (pre-)configured by higher layers.

Proposal 6: In a dedicated resource pool, with regards to the PSCCH frequency domain configuration, additional PRB values should be specified.

- Additional values could be {30, 40, 50} RBs. Other values are not precluded.

## Shared Resource Pool (legacy SL resource pool):

SCI for SL-PRS

In legacy resource pool, The SL-PRS configuration will be indicated using first stage SCI and second stage SCI to the receiving UE. In the last meeting following is agreed for shared resource pool

Agreement

In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot:

- With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported for the already agreed comb sizes 1, 2, 4

Agreement

In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot:

- The PSSCH is used for 2nd SCI and SL-SCH

  - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

Agreement

For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2.

- SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges.

If the slot is configured for SL-PRS transmission, then first stage SCI can be the presence of SL-PRS in the slot/subchannel. A new second stage SCI can be designed to convey both PSCCH and SL-PRS. SL-PRS resource trigger, cast type indicator and PSSCH presence indicator should be configured in 2nd stage SCI. If PSSCH indicator is set, then only PSSCH related bit will be set otherwise is will not be filled in 2nd stage SCI. In other way, new second stage SCI and old second stage SCI should be multiplexed together to indicate SL-PRS and PSSCH related parameters respectively.

Proposal 7: For legacy SL resource configuration for SL positioning, SL-PRS will be indicated using first stage SCI and new second stage SCI. First stage SCI will include SL-PRS presence indicator. It will be of one bit.

Proposal 8: For legacy SL resource configuration for SL positioning, chose one of the following options.

- Option 1: a new second stage SCI should convey both PSSCH and SL-PRS.

  - It will include PSSCH presence indicator. If it is set, then PSSCH is multiplexed in the slot otherwise not.

  - New 2nd stage SCI should include SL-PRS resource trigger for Rx UE SL-PRS triggering, cast type indicator and SL PRS resource ID if multiple SL-PRS resources are configured.

- Option 2: new second stage SCI and old second stage SCI should be multiplexed together to indicate SL-PRS and PSSCH related parameters respectively.

  - If PSSCH is absent the corresponding 2nd stage SCI will not be multiplexed.

  - New 2nd stage SCI should include SL-PRS resource trigger for Rx UE SL-PRS triggering, cast type indicator and SL PRS resource ID if multiple SL-PRS resources are configured.

SL-PRS multiplexing within the S-RP

If the bandwidth of configured subchannel is less than the required, then subchannel aggregation should be supported where adjacent subchannel are aggregated for SL-PRS transmission to match the positioning bandwidth requirement. This aggregation should be provided by higher layer configuration.

Proposal 9: For legacy SL resource configuration for SL positioning, subchannel aggregation should be supported where adjacent subchannel are aggregated for SL-PRS transmission to match the positioning bandwidth requirement.

- The aggregation information should be provided by higher layer.

Meaning of backward compatibility

Further regarding the backward compatibility following agreement is drawn,

Agreement: With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

Here according to us the backward compatibility means legacy UEs should be able to avoid the subchannel used for the Rel 18 positioning and no multiplexing or channel from multiple Tx UE should be allowed for better performance.

Proposal 10: In a shared resource pool for positioning, with regards to the meaning of backward compatibility requirement, when SL-PRS is transmitted in a resource, legacy UEs should avoid such a resource at least in mode 2 type of resource allocation.

# Resource Allocation Schemes for SL positioning:

As positioning and ranging solutions should support the in/partial coverage and OOC scenarios, the network centric (scheme 1) and UE autonomous resource selection (scheme 2) schemes of SL-PRS resources are being introduced for SL positioning and ranging. It is agreed that both schemes are important and should be supported in the specification. Scheme 1 is suitable at least in in-coverage and partial coverage scenarios whereas the scheme 2 is mainly targeting OOC scenario. Considering both the schemes some initial agreements are derived.

## Scheme 1:

In this type of resource allocation, anchor UEs and target UEs will be assisted with SL-PRS resource configurations and reporting configurations by the gNB(s) or LMF. In sidelink communication such resource configurations can be configured using dynamic grant and configured grant signalling from gNB to the participating UEs. In the last meetings following agreement is drawn on scheme 1.

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e., Option 1 is not pursued further).

Agreement

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - the SL-PRS transmission(s) follows the higher layer configuration

- Configured grant type 2

  - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats)

- The above mechanisms use NR Rel-16 mode-1 signaling as a starting point

- FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool.

- FFS: Further details

Agreement

In dynamic grant type resource allocation in scheme 1,

- For shared resource pool, DCI format 3_0 is being used as a starting point, down-select between the two alternatives below:

  - Alt. 1: Indication SL-PRS specific information is explicitly included in DCI

    - FFS: Which SL-PRS specific information

  - Alt. 2: Indication SL-PRS specific information is not explicitly included in DCI

- FFS: Dedicated resource pool

Agreement mentioned above indicate that gNB/TRP will report the resource pool (legacy or dedicated) dynamically using dynamic grant or semi statically using configured grant type 1 and type 2. Further gNB/TRP can report this resource configurations to the LMF at least in when the final positioning is being estimated by the LMF. It is decided in the last meeting, the DCI for dynamic grant is DCI 3_0 same can be used in the configured grant type 2 for activation/releasing of SL-PRS resources. For Dedicated pool also 3_0 should be sufficient. Dedicated resource pool will be defined by time domain, frequency domain resource and possible SL-PRS resources which will be configured by “sl-TxPoolScheduling” configured by higher layer and for DCI we can reuse the sl-TxPoolScheduling field IDs in DCI 3_0 to convey the dedicated resource pool. No need of indication of SL-PRS specific information explicitly included in DCI. Further SL-PRS and configured resource pool association can be provided directly in the resource configuration or via SLPP message. It is upto RAN 2 to decide this.

Proposal 11: In dynamic grant type resource allocation in scheme 1,

  - Indication of SL-PRS specific information is not explicitly included in DCI.

Proposal 12: In configured grant type 2, DCI 3_0 should be considered to support activating and releasing the configured grant.

Observation 1: In case of dedicated resource pool, the parameters of resource pool like the time domain, frequency domain resource and possible SL-PRS resources can be configured by “sl-TxPoolScheduling” by higher layer and in DCI the sl-TxPoolScheduling field IDs in DCI 3_0 can be reused to convey the dedicated resource pool.

Proposal 13: In dedicated resource pool dynamic grant type and activation/deactivation in configured grant type 2 resource allocation in scheme 1, DCI 3_0 can be reused.

Proposal 14: In scheme 1 type of resource allocation, gNB may provide configured resource configuration information to the positioning server/LMF.

## Scheme 2:

In the last meetings, the discussion related to resource selection methods in scheme 2 including sensing based and random selection, exact sensing mechanism for shared and dedicated resource pool, congestion control and inter-UE coordination for SL-PRS is carried out with some agreement on them.

Regarding the scheme 2 resource selection, sensing based, and random resource selection is agreed to support. The following agreement further drawn related to the sensing-based resource allocation mainly for dedicated resource pool,

Agreement

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

  - Option 1: SL-PRS

  - Option 2: PSCCH DMRS

  - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget.

  - Option 2: the selection window is provided by higher layers.

- Modification 3: For the SL-PRS priority:

  - Option 1: A single L1 SL-PRS priority is allowed in a resource pool.

  - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool.

- Modification 4: For the definition of a candidate resource within the resource selection window:

  - Options TBD

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length ():

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD.

- Modification 7: For the initial S-RSRP threshold & step size, target resource ratio X (%):

  - Options TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Options TBD

- Note 1: Other potential modifications and/or other options within each modification are not precluded.

- Note 2: Multiple options for each potential modification may be supported.

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools.

- With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission

  - FFS: whether this is enabled by (pre)configuration

- FFS: to support also SL-PRS

Agreement

For Scheme 2, in a dedicated resource pool,

- Multiple L1 SL-PRS priority are allowed in a resource pool

- A SL PRS resource within the resource selection window is used as a candidate resource

- with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

  - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point.

- with regards to the resource (re)-selection procedure

  - support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

For the resource exclusion in scheme 2 sensing-based resource selection RSRP need to be estimated. For dedicated resource pool, at least one of SL-PRS, and PSCCH DMRS should be used based on availability. Relative power variations should be considered for calculation of RSRP.

Proposal 15: In scheme 2 sensing-based resource selection for dedicated resource pool, at least one of SL-PRS, and PSCCH DMRS should be used for sensing RSRP calculation for resource exclusion.

- Relative power variations should be considered for calculation of RSRP.

For resource window selection two approaches are discussed, one on packet delay budget based and another higher layer configured. Higher layer configured approach will bring more dependence on coordinating node (e.g., server node or V2X or Prose layer) which may not optimally control the physical layer channel conditions. So, it will be better to use location delay budget approach similar to packet delay budget. This delay budget will be configured by higher layer or application layer.

Proposal 16: In scheme 2 for the derivation of the selection window, using the legacy approach as a starting point (option 1).

- Location delay budge (LDB) should be specified by higher layer.

Regarding the sensing window length (T0), it will be efficient to configure it based on largest reservation interval. This will bring more flexibility based on the number of SL-PRS resource repetition and expected latency.

Proposal 17: For scheme 2 sensing window length (T0), window should be is equal to or larger than the largest reservation interval.

- Window should be configured by higher layer.

Further regarding the congestion control following agreement drawn,

Agreement

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point.

- Study at least the following aspects on potential changes over the existing congestion control mechanisms:

  - CBR and CR definition for SL-PRS

  - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

  - CR and CBR measurement time window

  - Congestion control processing time

  - Number of CBR ranges

  - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Agreement

In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. Consider further the following parameter(s):

- Option 1: SL PRS transmission power

- Option 2: Periodicity of SL PRS

- Option 3: Number of occupied subchannels of SL-PRS (for shared resource pool)

- Option 4: Number of SL PRS resources in a slot

- Option 5: comb-size of a SL PRS resource in a slot

- Option 7: Number of OFDM symbols of a SL PRS resource in a slot

- Option 8: Number of SL PRS (re-)transmissions

- FFS: Other options are not precluded

For shared resource pool, it is discussed that legacy congestion control mechanism can be adopted but some modifications are needed. First modification was regarding separate CBR for SL-PRS and PSSCH. We feel it is not necessary to evaluate separate CBR value as resource selection mechanism should not be different for PSSCH and SL-PRS. Therefore, one CBR and CR value should be evaluated for SL-PRS and PSSCH. Another modification that was being discussed is on the change in the definition of SL RSSI in 38.215. For positioning and ranging solution, the definition should include the SL_PRS measurement in calculation of RSSI for CBR/CR estimation. Therefore, we support he explicitly include the SL-PRS in definition of RSSI in 38.215.

Proposal 18: For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, CBR and CR mechanisms from Rel.16 NR SL are reused,

- One CR/CBR shall be evaluated for both SL-PRS and PSSCH

- With regards to the RSSI evaluation for CBR explicitly include SL-PRS in the SL RSSI definition in 38.215 as follows:

  - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH and/or for SL-PRS, starting from the 2nd OFDM symbol.

On the other hand, for dedicated resource pool, we should redefine the CR/CBR definition and the RSSI definition. For CR/CBR, the definition should be considering SL-PRS resources and RSSI should include slot rather than subchannel to calculate average received power. Measurement window can be reused same as legacy i.e., 100ms or 100·2µ slots.

Proposal 19: For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, using the congestion control mechanisms from Rel.16 NR SL as a starting point, consider the following modifications,

- Redefine CBR/CR by considering the SL-PRS resource allocation/configuration.

- RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS

- The CR and CBR measurement time window size, can be separately configured for a dedicated resource pool and could take the values: 100ms or 100·2µ slots.

# Conclusion

The contribution provides the following proposals towards SL positioning and ranging resource allocation and coordination:

Proposal 1: For dedicated resource pool, multiple SL-PRS resources should be configured per resource pool. Maximum SL-PRS resource per resource pool can be 8.

Proposal 2: With regards to the SL-PRS configuration and/or SL-PRS time assignment information in first stage SCI, support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot. Explicit signaling of SL PRS resource should be supported in dedicated resource pool.

- If explicit signalling is not used, then a one-to-one mapping is assumed between a PSCCH resource and an associated SL-PRS resource in the same slot.

  - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped

- Resource ID should be included in SCI. Maximum 8 SL-PRS can be configured. This indication is signalled using bit map.

Proposal 3: In dedicated resource pool, the new 1st stage SCI should include SL PRS trigger for Rx UE. The SL-PRS trigger can be one bit or multiple bits.

Proposal 4: In dedicated resource pool, the new 1st stage SCI should not include positioning session ID.

Proposal 5: In a dedicated resource pool, with regards to the PSCCH time domain configuration, 1 or 2 or 3 symbols should be (pre-)configured by higher layers.

Proposal 6: In a dedicated resource pool, with regards to the PSCCH frequency domain configuration, additional PRB values should be specified.

- Additional values could be {30, 40, 50} RBs. Other values are not precluded.

Proposal 7: For legacy SL resource configuration for SL positioning, SL-PRS will be indicated using first stage SCI and new second stage SCI. First stage SCI will include SL-PRS presence indicator. It will be of one bit.

Proposal 8: For legacy SL resource configuration for SL positioning, chose one of the following options.

- Option 1: a new second stage SCI should convey both PSSCH and SL-PRS.

  - It will include PSSCH presence indicator. If it is set, then PSSCH is multiplexed in the slot otherwise not.

  - New 2nd stage SCI should include SL-PRS resource trigger for Rx UE SL-PRS triggering, cast type indicator and SL PRS resource ID if multiple SL-PRS resources are configured.

- Option 2: new second stage SCI and old second stage SCI should be multiplexed together to indicate SL-PRS and PSSCH related parameters respectively.

  - If PSSCH is absent the corresponding 2nd stage SCI will not be multiplexed.

  - New 2nd stage SCI should include SL-PRS resource trigger for Rx UE SL-PRS triggering, cast type indicator and SL PRS resource ID if multiple SL-PRS resources are configured.

Proposal 9: For legacy SL resource configuration for SL positioning, subchannel aggregation should be supported where adjacent subchannel are aggregated for SL-PRS transmission to match the positioning bandwidth requirement.

- The aggregation information should be provided by higher layer.

Proposal 10: In a shared resource pool for positioning, with regards to the meaning of backward compatibility requirement, when SL-PRS is transmitted in a resource, legacy UEs should avoid such a resource at least in mode 2 type of resource allocation.

Proposal 11: In dynamic grant type resource allocation in scheme 1,

  - Indication of SL-PRS specific information is not explicitly included in DCI.

Proposal 12: In configured grant type 2, DCI 3_0 should be considered to support activating and releasing the configured grant.

Observation 1: In case of dedicated resource pool, the parameters of resource pool like the time domain, frequency domain resource and possible SL-PRS resources can be configured by “sl-TxPoolScheduling” by higher layer and in DCI the sl-TxPoolScheduling field IDs in DCI 3_0 can be reused to convey the dedicated resource pool.

Proposal 13: In dedicated resource pool dynamic grant type and activation/deactivation in configured grant type 2 resource allocation in scheme 1, DCI 3_0 can be reused.

Proposal 14: In scheme 1 type of resource allocation, gNB may provide configured resource configuration information to the positioning server/LMF.

Proposal 15: In scheme 2 sensing-based resource selection for dedicated resource pool, at least one of SL-PRS, and PSCCH DMRS should be used for sensing RSRP calculation for resource exclusion.

- Relative power variations should be considered for calculation of RSRP.

Proposal 16: In scheme 2 for the derivation of the selection window, using the legacy approach as a starting point (option 1).

- Location delay budge (LDB) should be specified by higher layer.

Proposal 17: For scheme 2 sensing window length (T0), window should be is equal to or larger than the largest reservation interval.

- Window should be configured by higher layer.

Proposal 18: For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, CBR and CR mechanisms from Rel.16 NR SL are reused,

- One CR/CBR shall be evaluated for both SL-PRS and PSSCH

- With regards to the RSSI evaluation for CBR explicitly include SL-PRS in the SL RSSI definition in 38.215 as follows:

  - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH and/or for SL-PRS, starting from the 2nd OFDM symbol.

Proposal 19: For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, using the congestion control mechanisms from Rel.16 NR SL as a starting point, consider the following modifications,

- Redefine CBR/CR by considering the SL-PRS resource allocation/configuration.

- RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS

- The CR and CBR measurement time window size, can be separately configured for a dedicated resource pool and could take the values: 100 ms or 100·2µ ms.

# References

- TR 38.859 V0.3.0 (2022-11) Study on Expanded and Improved NR Positioning; (Release 18)

- RP-231460 New WID on Expanded and Improved NR Positioning

- R1-2306171 Moderator Summary #3 on resource allocation for SL PRS (RAN 1#113)

- TR 23.700-86 V1.3.0 Study on Architecture Enhancement to support Ranging based services and sidelink positioning (Release 18)

- R1-2308005 Further discussion on SL positioning reference signal design

- R1-2308006 Discussion on measurements and reporting for SL positioning methods