B.2	System level simulation results for semi-static SBFD

B.2.1	SBFD Deployment Case 1 (FR1)

B.2.1.1	Indoor office (FR1)

B.2.1.1.1	SBFD#1_InH_FR1_Sub#5

For sub-case SBFD#1_InH_FR1_Sub#5, assuming RSI based on 1dB desense, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.06% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.85% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 106.99% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 109.86% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.77% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.66% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 98.38% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 79.28% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -14.55% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -88.16% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 53.93% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -69.72% for SBFD

B.2.1.1.2	SBFD#1_InH_FR1_Sub#6

For sub-case SBFD#1_InH_FR1_Sub#6, assuming RSI based on 1dB desense, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.26% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.28% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.02% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -7.06% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.34% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -47.92% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.02% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.02% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -65.01% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.91% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -56.41% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -70.49% for SBFD

B.2.1.1.3	SBFD#1_InH_FR1_Sub#7

For sub-case SBFD#1_InH_FR1_Sub#7, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.12% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.22% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 99.34% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 101.26% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -8.88% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -35.28% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 96.82% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.88% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -34.51% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.98% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 96.58% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 118.43% for SBFD

B.2.1.1.4	SBFD#1_InH_FR1_Sub#8

For sub-case SBFD#1_InH_FR1_Sub#8, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.35% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -33.40% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.14% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 73.46% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -66.13% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -85.90% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.91% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 111.93% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.45% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -91.21% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 141.73% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 473.00% for SBFD

B.2.1.1.5	SBFD#1_InH_FR1_Sub#9

For sub-case SBFD#1_InH_FR1_Sub#9, assuming RSI based on 1dB desense, SBFD slot configuration Alt-1 ({DDDSU} vs. {DXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -0.56% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -0.38% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 73.07% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 74.00% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -0.56% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -1.81% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 69.25% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 68.49% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -3.12% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation of -16.65% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 64.83% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement of 57.21% for SBFD

B.2.1.1.6	SBFD#1_InH_FR1_Sub#10

For sub-case SBFD#1_InH_FR1_Sub#10, assuming RSI based on 1dB desense, SBFD slot configuration Alt-1 ({DDDSU} vs. {DXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-12.10%~-21.02%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-14.56%~-32.97%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {58.36%~67.15%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {57.19%~68.98%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-13.65%~-20.01%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-7.32%~-18.42%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {64.33%~103.54%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {82.46%~127.07%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-19.38%~-28.25%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-29.45%~-37.67%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {74.49%~168.03%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {111.59%~205.06%} for SBFD

B.2.1.1.7	SBFD#1_InH_FR1_Sub#11

For sub-case SBFD#1_InH_FR1_Sub#11, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.33% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.11% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 77.95% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 79.21% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 7.83% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.08% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 62.90% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 46.41% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.96% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.24% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 58.34% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 10.03% for SBFD

B.2.1.1.8	SBFD#1_InH_FR1_Sub#12

For sub-case SBFD#1_InH_FR1_Sub#12, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.08% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.08% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 14.64% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 10.72% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.95% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -11.09% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.40% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.45% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -23.62% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -42.40% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -22.57% for SBFD

B.2.1.1.9	SBFD#1_InH_FR1_Sub#13

For sub-case SBFD#1_InH_FR1_Sub#13, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.29% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.82% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 77.87% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 79.14% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.91% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -50.44% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 38.01% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -75.99% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -17.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -96.07% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 30.35% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -90.42% for SBFD

B.2.1.1.10	SBFD#1_InH_FR1_Sub#14

For sub-case SBFD#1_InH_FR1_Sub#14, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -7.13% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.61% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.35% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.99% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -37.85% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -56.81% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -21.11% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -40.38% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -72.59% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -86.25% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -58.69% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -80.22% for SBFD

B.2.1.2	Urban Macro (FR1)

B.2.1.2.1	SBFD#1_UMA_FR1_Sub#6

For sub-case SBFD#1_UMA_FR1_Sub#6, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-23.44%~-38.51%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-74.19%~-94.08%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Intel]) reported an improvement of 50.51% for SBFD, and one source ([Nokia]) reported a degradation in the range of {-40.23%~-44.04%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Intel]) reported an improvement of 110.55% for SBFD, and one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-35.03%~-66.08%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-90.63%~-98.47%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Intel]) reported an improvement of 26.14% for SBFD, and one source ([Nokia]) reported a degradation in the range of {-90.54%~-92.38%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Intel]) reported an improvement of 72.75% for SBFD, and one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-32.92%~-72.20%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Intel], [Nokia]) reported a degradation in the range of {-94.69%~-97.64%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Intel]) reported an improvement of 52.68% for SBFD, and one source ([Nokia]) reported a degradation in the range of {-99.79%~-99.85%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Intel]) reported an improvement of 72.43% for SBFD, and one source ([Nokia]) reported a degradation of -100.00% for SBFD

B.2.1.2.2	SBFD#1_UMA_FR1_Sub#13

For sub-case SBFD#1_UMA_FR1_Sub#13, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & same TxRUs (Option 1), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {3.28%~4.55%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-5.13%~-6.15%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {32.43%~37.66%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {51.92%~55.77%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {3.34%~3.63%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-32.82%~-36.70%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {47.43%~49.39%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-7.69%~-9.62%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-2.75%~-3.52%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-32.86%~-34.29%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {55.97%~60.51%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-16.00%~-24.00%} for SBFD

B.2.1.2.3	SBFD#1_UMA_FR1_Sub#14

For sub-case SBFD#1_UMA_FR1_Sub#14, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & same TxRUs (Option 1), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-16.85%~-17.39%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-31.15%~-33.98%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 0.66% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-3.81%~-5.71%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-18.18%~-21.81%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-38.45%~-47.73%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 2.35% for SBFD, and one source ([Samsung]) reported a degradation of -3.04% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {10.29%~11.76%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-21.80%~-23.31%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-41.33%~-42.92%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-15.82%~-29.74%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-6.78%~-8.47%} for SBFD

B.2.1.2.4	SBFD#1_UMA_FR1_Sub#15

For sub-case SBFD#1_UMA_FR1_Sub#15, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-21.16%~-22.71%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-94.69%~-96.57%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-15.82%~-35.94%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-51.58%~-77.00%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-25.95%~-26.42%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.79% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-56.32%~-72.15%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-86.59%~-95.16%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-31.47%~-32.12%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-99.86%~-99.87%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-84.60%~-90.52%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-91.76%~-98.88%} for SBFD

B.2.1.2.5	SBFD#1_UMA_FR1_Sub#16

For sub-case SBFD#1_UMA_FR1_Sub#16, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-8.81%~-31.16%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-18.31%~-85.51%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-6.28%~-27.41%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 51.92% for SBFD, and one source ([Nokia]) reported a degradation in the range of {-25.96%~-48.99%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-24.41%~-43.67%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-49.15%~-96.20%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-58.90%~-70.77%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-65.90%~-94.19%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-44.06%~-46.73%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-75.30%~-99.77%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-88.32%~-92.76%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Nokia]) reported a degradation in the range of {-93.21%~-100.00%} for SBFD

B.2.1.2.6	SBFD#1_UMA_FR1_Sub#17

For sub-case SBFD#1_UMA_FR1_Sub#17, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & same TxRUs (Option 1), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-18.34%~-19.72%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-22.71%~-23.50%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {50.63%~56.90%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {105.77%~119.23%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-22.18%~-22.88%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-29.52%~-31.29%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {69.68%~72.62%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {44.23%~50.00%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-23.72%~-24.14%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-41.20%~-41.46%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {53.12%~54.83%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {32.00%~40.00%} for SBFD

B.2.1.2.7	SBFD#1_UMA_FR1_Sub#18

For sub-case SBFD#1_UMA_FR1_Sub#18, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & same TxRUs (Option 1), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-32.64%~-32.71%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-35.17%~-40.41%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {50.76%~60.25%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {63.81%~67.62%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-38.98%~-40.99%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-42.12%~-44.51%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {75.32%~81.29%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {48.53%~66.18%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-36.80%~-38.96%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-48.13%~-54.98%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {41.96%~47.45%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {35.59%~37.29%} for SBFD

B.2.1.2.8	SBFD#1_UMA_FR1_Sub#19

For sub-case SBFD#1_UMA_FR1_Sub#19, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.40% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -41.34% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 52.70% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 57.19% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.56% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -70.25% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 18.33% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -44.05% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -71.29% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -85.46% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 10.80% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.58% for SBFD

B.2.1.2.9	SBFD#1_UMA_FR1_Sub#20

For sub-case SBFD#1_UMA_FR1_Sub#20, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-1 ({DDDSU} vs. {DXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-17.29%~-33.00%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-36.76%~-49.20%} for SBFD, and one source ([Huawei]) reported no change for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {46.73%~87.49%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Xiaomi]) reported an improvement of 126.00% for SBFD, and one source ([Huawei]) reported no change for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-26.73%~-44.05%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-50.11%~-90.70%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {9.08%~74.91%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Xiaomi]) reported an improvement of 90.91% for SBFD, and one source ([Huawei]) reported no change for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-18.73%~-56.94%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported a degradation in the range of {-66.33%~-96.85%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Huawei], [Xiaomi]) reported an improvement in the range of {4.28%~74.23%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Xiaomi]) reported an improvement of 55.81% for SBFD, and one source ([Huawei]) reported no change for SBFD

B.2.1.2.10	SBFD#1_UMA_FR1_Sub#21

For sub-case SBFD#1_UMA_FR1_Sub#21, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.04% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.46% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 6.07% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 50.99% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -19.57% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -22.78% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -40.82% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.35% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -50.47% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -53.08% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -74.52% for SBFD

B.2.1.2.11	SBFD#1_UMA_FR1_Sub#22

For sub-case SBFD#1_UMA_FR1_Sub#22, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.90% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.44% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -7.03% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.27% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -25.51% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.79% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -39.72% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -75.03% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -51.28% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -74.36% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -65.29% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -89.98% for SBFD

B.2.1.2.12	SBFD#1_UMA_FR1_Sub#23

For sub-case SBFD#1_UMA_FR1_Sub#23, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-18.08%~-18.09%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-85.39%~-86.28%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-99.16%~-99.61%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-18.58%~-18.82%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.58% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-21.59%~-22.27%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.70% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

B.2.1.2.13	SBFD#1_UMA_FR1_Sub#24

For sub-case SBFD#1_UMA_FR1_Sub#24, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-33.96%~-34.96%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-85.57%~-86.99%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-63.80%~-68.17%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-41.71%~-41.77%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-95.00%~-95.35%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-96.28%~-97.04%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-39.84%~-40.19%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-99.12%~-99.17%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation in the range of {-99.91%~-99.94%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -100.00% for SBFD

B.2.1.3	Dense Urban Macro layer (FR1)

B.2.1.3.1	SBFD#1_DUMacro_FR1_Sub#3

For sub-case SBFD#1_DUMacro_FR1_Sub#3, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 0.02% for SBFD, and one source ([CMCC]) reported a degradation of -2.96% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported a degradation in the range of {-0.31%~-3.11%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {50.96%~102.17%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {68.38%~115.37%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported a degradation in the range of {-2.64%~-4.71%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported a degradation in the range of {-7.75%~-8.30%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {38.17%~103.30%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {50.18%~108.38%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported a degradation in the range of {-10.56%~-12.52%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported a degradation in the range of {-22.09%~-57.22%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {26.54%~101.51%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([CMCC], [ZTE]) reported an improvement in the range of {39.84%~83.27%} for SBFD

B.2.1.3.2	SBFD#1_DUMacro_FR1_Sub#5

For sub-case SBFD#1_DUMacro_FR1_Sub#5, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported an improvement in the range of {2.34%~18.62%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported an improvement in the range of {2.61%~16.98%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported an improvement in the range of {32.87%~90.46%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported an improvement in the range of {27.49%~77.63%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -4.97% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -34.52% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -58.08% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -98.95% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -8.04% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -98.65% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -93.98% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -93.67% for SBFD

B.2.1.3.3	SBFD#1_DUMacro_FR1_Sub#6

For sub-case SBFD#1_DUMacro_FR1_Sub#6, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([OPPO]) reported an improvement of 3.01% for SBFD, and one source ([Nokia]) reported a degradation of -0.44% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported a degradation in the range of {-7.67%~-11.84%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported an improvement in the range of {2.74%~44.92%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 218.13% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported a degradation in the range of {-3.38%~-9.73%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported a degradation in the range of {-42.45%~-45.10%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 51.18% for SBFD, and one source ([Nokia]) reported a degradation of -48.72% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -2.78% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported a degradation in the range of {-0.40%~-18.48%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [OPPO]) reported a degradation in the range of {-15.89%~-72.06%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 41.46% for SBFD, and one source ([Nokia]) reported a degradation of -92.91% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -98.15% for SBFD

B.2.1.3.4	SBFD#1_DUMacro_FR1_Sub#7

For sub-case SBFD#1_DUMacro_FR1_Sub#7, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([OPPO]) reported an improvement of 10.06% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([OPPO]) reported an improvement of 7.52% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 90.66% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 75.51% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF,

-	Regarding 5%-tile of DL average-UPT CDF,

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF,

-	Regarding 5%-tile of UL average-UPT CDF,

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF,

-	Regarding 5%-tile of DL average-UPT CDF,

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF,

-	Regarding 5%-tile of UL average-UPT CDF,

B.2.1.3.5	SBFD#1_DUMacro_FR1_Sub#8

For sub-case SBFD#1_DUMacro_FR1_Sub#8, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -17.60% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -31.07% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 103.52% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF,

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -27.16% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -59.90% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 114.71% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF,

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -26.08% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([OPPO]) reported a degradation of -39.40% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([OPPO]) reported an improvement of 85.64% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF,

B.2.1.3.6	SBFD#1_DUMacro_FR1_Sub#9

For sub-case SBFD#1_DUMacro_FR1_Sub#9, assuming RSI based on 1dB desense,  93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported an improvement in the range of {0.84%~3.68%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 4.55% for SBFD, and one source ([ZTE]) reported a degradation of -0.54% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported an improvement in the range of {22.83%~54.57%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 93.23% for SBFD, and one source ([ZTE]) reported a degradation of -1.28% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 0.05% for SBFD, and one source ([Nokia]) reported a degradation of -3.71% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported a degradation in the range of {-1.31%~-28.37%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported an improvement in the range of {23.61%~23.83%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 12.44% for SBFD, and one source ([ZTE]) reported a degradation of -1.49% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported a degradation in the range of {-3.40%~-9.15%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported a degradation in the range of {-19.79%~-98.84%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 26.74% for SBFD, and one source ([Nokia]) reported a degradation of -23.87% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Nokia], [ZTE]) reported a degradation in the range of {-0.60%~-78.76%} for SBFD

B.2.1.3.7	SBFD#1_DUMacro_FR1_Sub#11

For sub-case SBFD#1_DUMacro_FR1_Sub#11, assuming RSI based on 1dB desense,  93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 0.02% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -0.31% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 102.17% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 115.37% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -2.64% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -8.30% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 103.30% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 108.38% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -12.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -57.22% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 101.51% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 83.27% for SBFD

B.2.1.3.8	SBFD#1_DUMacro_FR1_Sub#13

For sub-case SBFD#1_DUMacro_FR1_Sub#13, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 2.33% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 3.14% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 47.30% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 76.21% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -10.01% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -62.19% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -4.00% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -92.35% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -19.95% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.32% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -66.35% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -87.94% for SBFD

B.2.1.3.9	SBFD#1_DUMacro_FR1_Sub#14

For sub-case SBFD#1_DUMacro_FR1_Sub#14, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -3.58% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -23.19% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 6.83% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 235.36% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -15.02% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -48.84% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -25.62% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 92.06% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -29.97% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -87.47% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -71.49% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -92.57% for SBFD

B.2.1.3.10	SBFD#1_DUMacro_FR1_Sub#15

For sub-case SBFD#1_DUMacro_FR1_Sub#15, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-1 ({DDDSU} vs. {DXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-17.95%~-28.21%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-17.16%~-26.22%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {51.80%~61.11%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {43.92%~90.52%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-25.03%~-38.35%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-33.09%~-56.26%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {43.60%~59.17%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {38.07%~192.27%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-29.72%~-45.79%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Huawei]) reported a degradation in the range of {-40.87%~-72.48%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {29.69%~62.07%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Huawei]) reported an improvement in the range of {70.45%~546.21%} for SBFD, and one source ([Huawei]) reported a degradation of -100.00% for SBFD

B.2.1.3.11	SBFD#1_DUMacro_FR1_Sub#16

For sub-case SBFD#1_DUMacro_FR1_Sub#16, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 0.10% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -0.02% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 20.04% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -10.89% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -9.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -52.96% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -84.66% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.57% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -18.78% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.23% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -96.92% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -94.41% for SBFD

B.2.1.3.12	SBFD#1_DUMacro_FR1_Sub#17

For sub-case SBFD#1_DUMacro_FR1_Sub#17, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -4.31% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -24.92% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -16.79% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 137.56% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -21.39% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -60.56% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -70.48% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -84.13% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -30.25% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -88.66% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -97.03% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -99.51% for SBFD

B.2.1.4	Dense Urban with 2-layer (FR1)

B.2.1.4.1	SBFD#1_DU2Layer_FR1_Sub#1

For sub-case SBFD#1_DU2Layer_FR1_Sub#1, assuming RSI based on 1dB desense,  93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 0.95% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 1.94% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 7.23% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported no change for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 0.89% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported an improvement of 1.50% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 11.51% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported no change for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -4.62% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -28.72% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 7.85% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported no change for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -5.05% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -71.48% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 13.00% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported no change for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -11.66% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -27.50% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 22.27% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 25.00% for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -8.19% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -8.70% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 27.80% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 152.83% for SBFD

B.2.1.4.2	SBFD#1_DU2Layer_FR1_Sub#2

For sub-case SBFD#1_DU2Layer_FR1_Sub#2, assuming RSI based on 1dB desense,  93dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -30.39% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -38.60% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 10.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 18.09% for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -29.87% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -33.60% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 14.54% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 23.62% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -32.29% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -37.43% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 10.74% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 5.24% for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -30.82% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -30.19% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 13.25% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 10.50% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	For Layer-1:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -38.93% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -56.48% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 10.93% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 6.34% for SBFD

-	For Layer-2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -37.65% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([ZTE]) reported a degradation of -52.08% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 13.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([ZTE]) reported an improvement of 3.22% for SBFD

B.2.2	SBFD Deployment Case 1 (FR2-1)

B.2.2.1	Indoor office (FR2-1)

B.2.2.1.1	SBFD#1_InH_FR2_Sub#3

For sub-case SBFD#1_InH_FR2_Sub#3, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported a degradation in the range of {-4.40%~-21.75%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported a degradation in the range of {-22.33%~-48.91%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported an improvement in the range of {64.79%~79.80%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported an improvement in the range of {21.30%~84.98%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported a degradation in the range of {-6.90%~-25.99%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported a degradation in the range of {-30.02%~-71.43%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported an improvement in the range of {29.90%~91.25%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [ZTE]) reported an improvement in the range of {41.70%~102.86%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -38.57% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.46% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 131.14% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 260.20% for SBFD

B.2.2.1.2	SBFD#1_InH_FR2_Sub#4

For sub-case SBFD#1_InH_FR2_Sub#4, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-18.65%~-21.69%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-13.40%~-21.73%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {79.84%~92.49%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {85.31%~140.58%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-18.16%~-25.91%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-18.59%~-30.44%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {80.97%~91.37%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {102.83%~116.91%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-24.45%~-38.46%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-26.77%~-44.17%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {97.88%~131.10%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {198.37%~257.17%} for SBFD

B.2.2.1.3	SBFD#1_InH_FR2_Sub#5

For sub-case SBFD#1_InH_FR2_Sub#5, assuming RSI based on 1dB desense, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.95% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.39% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 105.27% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 107.80% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.48% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.44% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 100.82% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 94.52% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.68% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.85% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 81.60% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 24.38% for SBFD

B.2.2.1.4	SBFD#1_InH_FR2_Sub#6

For sub-case SBFD#1_InH_FR2_Sub#6, assuming RSI based on 1dB desense, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.37% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.26% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.48% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.12% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.07% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.62% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.92% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.40% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.74% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.44% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -16.18% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -37.67% for SBFD

B.2.2.1.5	SBFD#1_InH_FR2_Sub#7

For sub-case SBFD#1_InH_FR2_Sub#7, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -22.39% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -23.45% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.49% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.30% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.83% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -35.88% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 85.45% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.89% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.61% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -57.00% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 121.18% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 236.14% for SBFD

B.2.2.1.6	SBFD#1_InH_FR2_Sub#8

For sub-case SBFD#1_InH_FR2_Sub#8, assuming RSI based on 1dB desense, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -22.44% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -24.05% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.58% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.55% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.75% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -36.28% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 85.55% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 90.43% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.44% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -56.80% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 121.09% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 231.26% for SBFD

B.2.2.1.7	SBFD#1_InH_FR2_Sub#9

For sub-case SBFD#1_InH_FR2_Sub#9, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.31% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.40% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.66% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 77.73% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 8.65% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.41% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 67.85% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 58.91% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 7.22% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.02% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 57.04% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 38.33% for SBFD

B.2.2.1.8	SBFD#1_InH_FR2_Sub#10

For sub-case SBFD#1_InH_FR2_Sub#10, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.01% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.16% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.91% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.03% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.08% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.21% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.04% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.13% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.53% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.74% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.08% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -15.87% for SBFD

B.2.2.1.9	SBFD#1_InH_FR2_Sub#11

For sub-case SBFD#1_InH_FR2_Sub#11, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.34% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 10.84% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.60% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 77.12% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 8.50% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.14% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 67.59% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 57.57% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.43% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -8.55% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 55.14% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 22.73% for SBFD

B.2.2.1.10	SBFD#1_InH_FR2_Sub#12

For sub-case SBFD#1_InH_FR2_Sub#12, assuming RSI based on 1dB desense, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.14% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.21% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.06% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.48% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.00% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.74% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.15% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.04% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.19% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.04% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.36% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.69% for SBFD

B.2.2.2	Dense Urban Macro layer (FR2-1)

B.2.2.2.1	SBFD#1_DUMacro_FR2_Sub#3

For sub-case SBFD#1_DUMacro_FR2_Sub#3, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -0.39% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -6.73% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 87.97% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 281.48% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -0.70% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -11.41% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 73.84% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 227.27% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -2.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -12.91% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 86.64% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 201.89% for SBFD

B.2.2.2.2	SBFD#1_DUMacro_FR2_Sub#4

For sub-case SBFD#1_DUMacro_FR2_Sub#4, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-15.95%~-21.91%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-5.54%~-22.69%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {75.58%~109.24%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {66.67%~80.11%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-21.19%~-29.27%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-26.51%~-47.90%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {80.27%~142.84%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {92.78%~164.50%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-30.93%~-38.75%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-28.55%~-52.11%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {87.00%~134.26%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {67.65%~236.80%} for SBFD

B.2.2.2.3	SBFD#1_DUMacro_FR2_Sub#5

For sub-case SBFD#1_DUMacro_FR2_Sub#5, assuming RSI based on 1dB desense, less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 4.60% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 4.95% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 47.36% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 143.55% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 1.76% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -1.62% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -12.79% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported an improvement of 511.72% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -0.62% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported a degradation of -10.72% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -93.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Nokia]) reported a degradation of -76.65% for SBFD

B.2.2.2.4	SBFD#1_DUMacro_FR2_Sub#7

For sub-case SBFD#1_DUMacro_FR2_Sub#7, assuming RSI based on 1dB desense,  98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {2.92%~4.69%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 5.07% for SBFD, and one source ([Qualcomm]) reported a degradation of -32.44% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {32.58%~57.78%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {65.51%~202.15%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {1.88%~2.57%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Nokia]) reported an improvement of 1.27% for SBFD, and one source ([Qualcomm]) reported a degradation of -33.97% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {34.88%~59.09%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {63.03%~4165.62%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {0.31%~0.90%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported a degradation in the range of {-7.41%~-35.11%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {38.85%~41.22%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Nokia], [Qualcomm]) reported an improvement in the range of {96.07%~131.63%} for SBFD

B.2.2.2.5	SBFD#1_DUMacro_FR2_Sub#9

For sub-case SBFD#1_DUMacro_FR2_Sub#9, assuming RSI based on 1dB desense,  98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -0.39% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -6.73% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 87.97% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 281.48% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -0.70% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -11.41% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 73.84% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 227.27% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -2.52% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -12.91% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 86.64% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 201.89% for SBFD

B.2.2.2.6	SBFD#1_DUMacro_FR2_Sub#10

For sub-case SBFD#1_DUMacro_FR2_Sub#10, assuming RSI based on 1dB desense,  98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-15.95%~-21.91%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-5.54%~-22.69%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {75.58%~108.73%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {66.67%~80.11%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-21.28%~-29.27%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-28.00%~-47.90%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {80.27%~140.96%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {92.78%~164.50%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-31.39%~-38.75%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported a degradation in the range of {-36.43%~-52.11%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {87.00%~130.83%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, 2 sources ([Ericsson], [Samsung]) reported an improvement in the range of {67.65%~236.80%} for SBFD

B.2.2.2.7	SBFD#1_DUMacro_FR2_Sub#11

For sub-case SBFD#1_DUMacro_FR2_Sub#11, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & same TxRUs (Option 1), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported an improvement of 9.37% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -6.30% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 52.08% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 74.07% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported an improvement of 8.98% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -23.35% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 52.06% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 27.27% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported an improvement of 15.93% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -19.29% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 64.93% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 362.26% for SBFD

B.2.2.2.8	SBFD#1_DUMacro_FR2_Sub#12

For sub-case SBFD#1_DUMacro_FR2_Sub#12, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & same TxRUs (Option 1), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-2.89%~-2.97%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-15.73%~-15.84%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 49.08% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {25.93%~44.44%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-33.87%~-33.98%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-69.57%~-71.05%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {44.66%~45.25%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {41.24%~42.27%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-39.86%~-39.88%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-51.38%~-52.80%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {32.09%~32.85%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {56.86%~58.82%} for SBFD

B.2.2.2.9	SBFD#1_DUMacro_FR2_Sub#13

For sub-case SBFD#1_DUMacro_FR2_Sub#13, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.25%~-1.32%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-9.45%~-15.39%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-4.58%~-9.19%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-6.01%~-10.74%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-1.84%~-4.25%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-6.23%~-25.67%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-17.00%~-38.73%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-43.88%~-63.49%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-6.36%~-9.33%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-19.62%~-33.61%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-32.60%~-67.05%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-48.50%~-79.95%} for SBFD

B.2.2.2.10	SBFD#1_DUMacro_FR2_Sub#14

For sub-case SBFD#1_DUMacro_FR2_Sub#14, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & same TxRUs (Option 1), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -2.65% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -14.48% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 65.41% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 192.59% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -2.85% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -18.60% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 51.06% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 130.30% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -6.26% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation of -20.99% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 58.96% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement of 50.94% for SBFD

B.2.2.2.11	SBFD#1_DUMacro_FR2_Sub#15

For sub-case SBFD#1_DUMacro_FR2_Sub#15, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & same TxRUs (Option 1), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-22.99%~-23.05%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-27.53%~-28.00%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {67.09%~69.38%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {14.81%~21.30%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-36.12%~-36.33%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-50.16%~-50.69%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {85.65%~86.33%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {41.24%~43.30%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-46.70%~-46.90%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Samsung]) reported a degradation in the range of {-51.67%~-52.04%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {67.68%~71.30%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Samsung]) reported an improvement in the range of {26.47%~28.43%} for SBFD

B.2.2.2.12	SBFD#1_DUMacro_FR2_Sub#16

For sub-case SBFD#1_DUMacro_FR2_Sub#16, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-23.03%~-24.07%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-29.10%~-35.66%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {70.36%~74.56%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {70.65%~73.03%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-31.51%~-34.08%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-52.80%~-60.13%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {71.61%~89.31%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {129.90%~149.90%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-43.18%~-46.05%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-62.44%~-66.31%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {74.83%~113.28%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {68.91%~201.57%} for SBFD

B.2.2.2.13	SBFD#1_DUMacro_FR2_Sub#17

For sub-case SBFD#1_DUMacro_FR2_Sub#17, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {1.64%~1.94%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.08% for SBFD, and one source ([Ericsson]) reported a degradation of -0.51% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.21% for SBFD, and one source ([Ericsson]) reported a degradation of -0.93% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.15% for SBFD, and one source ([Ericsson]) reported a degradation of -3.12% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {0.13%~0.66%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.57%~-5.93%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-3.06%~-12.70%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-19.23%~-40.85%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.85%~-1.75%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-5.18%~-8.22%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-10.43%~-30.32%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-21.02%~-43.32%} for SBFD

B.2.2.2.14	SBFD#1_DUMacro_FR2_Sub#18

For sub-case SBFD#1_DUMacro_FR2_Sub#18, assuming RSI based on 1dB desense, no less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.48% for SBFD, and one source ([Ericsson]) reported a degradation of -1.22% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-10.81%~-17.64%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-1.61%~-3.94%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-2.77%~-6.46%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-2.92%~-6.03%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-11.51%~-31.33%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-7.49%~-17.98%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-28.89%~-47.31%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-8.90%~-12.38%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-21.97%~-36.42%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-16.94%~-37.41%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-26.98%~-50.01%} for SBFD

B.2.2.2.15	SBFD#1_DUMacro_FR2_Sub#19

For sub-case SBFD#1_DUMacro_FR2_Sub#19, assuming RSI based on 1dB desense, less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.29%~-1.33%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-8.67%~-14.96%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-10.14%~-21.45%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-13.62%~-27.46%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-2.21%~-4.73%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-8.04%~-27.51%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-42.02%~-77.57%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-69.06%~-97.33%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-6.53%~-10.03%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-20.54%~-35.08%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-67.91%~-95.48%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-79.22%~-99.73%} for SBFD

B.2.2.2.16	SBFD#1_DUMacro_FR2_Sub#20

For sub-case SBFD#1_DUMacro_FR2_Sub#20, assuming RSI based on 1dB desense, less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {1.63%~1.86%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.07%~-0.15%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-1.49%~-6.89%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-3.16%~-9.53%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement in the range of {0.09%~0.57%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-1.19%~-5.03%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-14.73%~-36.17%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-46.66%~-73.81%} for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-0.94%~-2.27%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-2.48%~-11.01%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-32.70%~-63.58%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-46.43%~-82.79%} for SBFD

B.2.2.2.17	SBFD#1_DUMacro_FR2_Sub#21

For sub-case SBFD#1_DUMacro_FR2_Sub#21, assuming RSI based on 1dB desense, less than 98dB for spatial isolation and digital isolation for co-site inter-sector CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.42% for SBFD, and one source ([Ericsson]) reported a degradation of -1.27% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-10.44%~-16.64%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-4.33%~-9.83%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-6.22%~-12.97%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.18% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.25% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -19.47% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -51.65% for SBFD

-	Traffic load with {DL,UL} = {High, High}:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-9.11%~-12.80%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-20.62%~-35.67%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-38.26%~-68.26%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation in the range of {-49.19%~-88.15%} for SBFD

B.2.3	SBFD Deployment Case 3-2 (FR1)

B.2.3.1	2-layer Scenario B (FR1)

B.2.3.1.1	SBFD#3-2_ScenarioB_FR1_Sub#5

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#5, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.01% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.57% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 80.17% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 68.09% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -11.38% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -43.00% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 57.19% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -81.13% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.72% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.56% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 23.14% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.84% for SBFD

B.2.3.1.2	SBFD#3-2_ScenarioB_FR1_Sub#6

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#6, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -32.38% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -39.86% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.38% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.89% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -66.56% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -82.02% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -62.19% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -95.47% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.49% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -93.01% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -75.57% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -96.98% for SBFD

B.2.3.1.3	SBFD#3-2_ScenarioB_FR1_Sub#7

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#7, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.00% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.26% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 96.59% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.54% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -13.54% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -53.24% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.33% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 60.84% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -33.96% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.78% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 100.19% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 361.46% for SBFD

B.2.3.1.4	SBFD#3-2_ScenarioB_FR1_Sub#8

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#8, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -35.08% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -42.90% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 75.31% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 69.76% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -67.59% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -82.61% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 82.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 50.07% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.50% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -92.95% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 108.56% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 249.80% for SBFD

B.2.3.1.5	SBFD#3-2_ScenarioB_FR1_Sub#9

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#9, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.01% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.05% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 97.92% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 97.44% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.43% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.98% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 95.33% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 64.08% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -20.55% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -90.02% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 112.45% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 390.09% for SBFD

B.2.3.1.6	SBFD#3-2_ScenarioB_FR1_Sub#10

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#10, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -29.99% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -34.28% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 106.57% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 107.83% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -45.57% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -53.32% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 136.02% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 121.56% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -66.40% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -82.87% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 214.26% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 595.02% for SBFD

B.2.3.1.7	SBFD#3-2_ScenarioB_FR1_Sub#11

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#11, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.05% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.15% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 97.31% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 91.21% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -17.79% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -64.68% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 93.16% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 62.62% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -37.35% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.25% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 105.90% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 379.10% for SBFD

B.2.3.1.8	SBFD#3-2_ScenarioB_FR1_Sub#12

For sub-case SBFD#3-2_ScenarioB_FR1_Sub#12, assuming RSI based on 1dB desense or better, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -40.55% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.05% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 90.35% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 84.63% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -75.09% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -88.96% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 101.55% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 69.72% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Layer 1 and Layer 2:

-	DL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -86.87% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -94.10% for SBFD

-	UL performance comparison between SBFD and legacy TDD for Layer 2:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 139.85% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 321.99% for SBFD

B.2.4	SBFD Deployment Case 4 (FR1)

B.2.4.1	Urban Macro (0% grid shift)

B.2.4.1.1	SBFD#4_UMA_FR1_0%_Sub#1

For sub-case SBFD#4_UMA_FR1_0%_Sub#1, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, no less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {0.36%~0.38%} for Target case, and one source ([Qualcomm]) reported a degradation in the range of {-3.17%~-3.31%} for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {0.24%~0.33%} for Target case, and one source ([Qualcomm]) reported a degradation in the range of {-16.46%~-20.38%} for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-2.90%~-8.56%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-44.41%~-99.89%} for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {3.98%~9.46%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {9.39%~9.40%} for SBFD, and one source ([Qualcomm]) reported a degradation in the range of {-33.88%~-34.98%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {150.07%~156.58%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {99.39%~116.15%} for SBFD, and one source ([Qualcomm]) reported a degradation in the range of {-49.63%~-67.97%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-0.44%~-5.58%} for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-0.95%~-69.26%} for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-7.90%~-15.99%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-99.71%~-100.00%} for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {1.74%~7.98%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {8.19%~8.23%} for SBFD, and one source ([Qualcomm]) reported a degradation in the range of {-75.69%~-77.26%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {132.54%~144.68%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement of 5.19% for SBFD, and one source ([Qualcomm]) reported a degradation in the range of {-8.98%~-99.14%} for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-2.37%~-7.45%} for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-14.29%~-99.95%} for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-11.15%~-19.62%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation of -100.00% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {0.16%~6.89%} for SBFD, and one source ([Qualcomm]) reported a degradation of -0.10% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-4.96%~-99.94%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {124.22%~138.82%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-97.86%~-100.00%} for SBFD

B.2.4.1.2	SBFD#4_UMA_FR1_0%_Sub#4

For sub-case SBFD#4_UMA_FR1_0%_Sub#4, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.49% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.92% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -14.55% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.51% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.23% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.66% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -29.52% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 71.75% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.56% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.58% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -58.85% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -90.20% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.85% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -7.37% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -97.10% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -97.98% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.01% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.75% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -78.07% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.85% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.51% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -9.28% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.90% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.1.3	SBFD#4_UMA_FR1_0%_Sub#5

For sub-case SBFD#4_UMA_FR1_0%_Sub#5, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.28% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.32% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.05% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.63% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -22.51% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -26.69% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 45.47% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 113.30% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.35% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.67% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.10% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.30% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -36.59% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.65% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 7.39% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.45% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.41% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.70% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.10% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.88% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -52.81% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -65.54% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.34% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.71% for SBFD

B.2.4.1.4	SBFD#4_UMA_FR1_0%_Sub#8

For sub-case SBFD#4_UMA_FR1_0%_Sub#8, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, no less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.69% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.09% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -17.62% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -37.98% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.17% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.57% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -26.66% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -11.10% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.06% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.65% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -66.78% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -96.77% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.19% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -54.57% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.96% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.51% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.86% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.53% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -81.96% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.36% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -70.80% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -95.28% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.1.5	SBFD#4_UMA_FR1_0%_Sub#9

For sub-case SBFD#4_UMA_FR1_0%_Sub#9, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, no less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.43% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.96% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.03% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.17% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -29.62% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.06% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 37.55% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 8.07% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.65% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.78% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.08% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.69% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -54.61% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -78.60% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.32% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -42.63% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.22% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.64% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.04% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.05% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -71.00% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -83.92% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.31% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.99% for SBFD

B.2.4.1.6	SBFD#4_UMA_FR1_0%_Sub#10

For sub-case SBFD#4_UMA_FR1_0%_Sub#10, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.58% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.63% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -17.39% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -38.18% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -9.28% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -25.68% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -42.07% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -15.62% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.47% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.45% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -64.89% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -95.56% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -21.14% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -42.57% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.64% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.60% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.14% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.06% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -80.91% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -35.37% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -57.74% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.97% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.1.7	SBFD#4_UMA_FR1_0%_Sub#11

For sub-case SBFD#4_UMA_FR1_0%_Sub#11, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.59% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.43% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.24% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 19.56% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -29.43% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.38% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 25.53% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.73% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.27% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.46% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.54% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.11% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -53.90% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -77.83% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -9.40% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.86% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.25% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.18% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.30% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.01% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -70.16% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -81.84% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -17.01% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.94% for SBFD

B.2.4.1.8	SBFD#4_UMA_FR1_0%_Sub#12

For sub-case SBFD#4_UMA_FR1_0%_Sub#12, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.27% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.40% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.07% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.07% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.03% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -31.10% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 51.15% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 109.81% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.50% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.04% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.23% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.35% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -43.97% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -60.37% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 7.13% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.59% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.52% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.88% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.02% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 2.41% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -60.54% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -72.49% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 4.09% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 3.93% for SBFD

B.2.4.1.9	SBFD#4_UMA_FR1_0%_Sub#13

For sub-case SBFD#4_UMA_FR1_0%_Sub#13, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, less than 93dB for spatial isolation for co-site adjacent-channel CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.21% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.42% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.12% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.97% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -33.45% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.00% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 28.53% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.51% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.27% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.64% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.24% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.57% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -60.15% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -82.06% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.38% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -43.42% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.02% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.73% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.22% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.55% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -74.54% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -85.64% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.47% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.26% for SBFD

B.2.4.2	Urban Macro (100% grid shift)

B.2.4.2.1	SBFD#4_UMA_FR1_100%_Sub#1

For sub-case SBFD#4_UMA_FR1_100%_Sub#1, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 4Kbytes for DL and 1Kbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement of 0.27% for Target case, and one source ([Qualcomm]) reported a degradation of -3.54% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement of 0.46% for Target case, and one source ([Qualcomm]) reported a degradation of -31.78% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-11.47%~-12.75%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-63.65%~-99.50%} for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {5.46%~10.83%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement of 11.20% for SBFD, and one source ([Qualcomm]) reported a degradation of -40.51% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {149.36%~152.60%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {17.22%~46.74%} for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-1.31%~-4.74%} for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-4.03%~-77.47%} for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-17.41%~-19.12%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-99.99%~-100.00%} for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {4.40%~9.00%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement of 7.23% for SBFD, and one source ([Qualcomm]) reported a degradation of -74.82% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {125.63%~137.64%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-61.99%~-62.93%} for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-4.73%~-5.23%} for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-35.05%~-99.93%} for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-19.97%~-27.43%} for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation of -100.00% for Target case, and one source ([Qualcomm]) reported no change for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {3.88%~7.14%} for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Qualcomm]) reported a degradation in the range of {-7.83%~-99.88%} for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Qualcomm]) reported an improvement in the range of {97.38%~128.94%} for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Qualcomm]) reported a degradation of -99.95% for SBFD, and one source ([Qualcomm]) reported no change for SBFD

B.2.4.2.2	SBFD#4_UMA_FR1_100%_Sub#4

For sub-case SBFD#4_UMA_FR1_100%_Sub#4, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.57% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.36% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.52% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.69% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.02% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.86% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -14.91% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 118.16% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.82% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.83% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -51.61% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -86.04% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.42% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.07% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -89.10% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -94.81% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.47% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.80% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -73.56% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.04% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.75% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.26% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.26% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.2.3	SBFD#4_UMA_FR1_100%_Sub#5

For sub-case SBFD#4_UMA_FR1_100%_Sub#5, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.27% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.10% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.25% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 12.81% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -21.93% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -24.74% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 57.13% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 141.70% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.23% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.47% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.22% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.23% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -32.89% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -41.14% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 14.58% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 10.73% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.14% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.10% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.99% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 6.14% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.16% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -67.55% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.58% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.43% for SBFD

B.2.4.2.4	SBFD#4_UMA_FR1_100%_Sub#8

For sub-case SBFD#4_UMA_FR1_100%_Sub#8, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.85% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.09% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -15.39% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -35.58% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -9.17% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.31% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.90% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 8.58% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.86% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -8.37% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -60.53% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -94.72% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -24.55% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.85% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -78.70% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.48% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.78% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.32% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -80.17% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -41.86% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -71.47% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -93.51% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.2.5	SBFD#4_UMA_FR1_100%_Sub#9

For sub-case SBFD#4_UMA_FR1_100%_Sub#9, assuming RSI based on 1dB desense, no less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.68% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.65% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.11% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.03% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.40% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -43.04% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 44.21% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 20.49% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.41% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -8.91% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.25% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.19% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.73% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -71.00% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.13% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -44.46% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.31% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -9.32% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.46% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.52% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -68.63% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -84.97% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.53% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -51.86% for SBFD

B.2.4.2.6	SBFD#4_UMA_FR1_100%_Sub#10

For sub-case SBFD#4_UMA_FR1_100%_Sub#10, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-4 ({DDDSU} vs. {XXXXX}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.81% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.88% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -15.01% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -34.63% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -8.49% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -23.98% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -29.41% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.27% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.97% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.26% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -58.84% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -93.05% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -19.54% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -41.04% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -94.71% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -98.95% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.44% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.50% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -78.87% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.99% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -32.15% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -61.01% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -99.78% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -100.00% for SBFD

B.2.4.2.7	SBFD#4_UMA_FR1_100%_Sub#11

For sub-case SBFD#4_UMA_FR1_100%_Sub#11, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-2 ({DDDSU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.72% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.26% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.12% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.08% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -27.94% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -41.25% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 34.62% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.73% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.02% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.91% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.25% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.43% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -48.65% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -69.41% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.35% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -44.50% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.03% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -10.86% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.60% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.34% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -67.26% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -84.37% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.07% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -52.72% for SBFD

B.2.4.2.8	SBFD#4_UMA_FR1_100%_Sub#12

For sub-case SBFD#4_UMA_FR1_100%_Sub#12, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Twice area & same TxRUs (Option 2), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.21% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported an improvement of 1.77% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.36% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.81% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -26.35% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -28.62% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 64.82% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 134.95% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -1.55% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.81% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.60% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 15.27% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -39.58% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -49.07% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 14.42% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 8.36% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.84% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -5.98% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -4.04% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 11.05% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -56.22% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -72.83% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 5.10% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.66% for SBFD

B.2.4.2.9	SBFD#4_UMA_FR1_100%_Sub#13

For sub-case SBFD#4_UMA_FR1_100%_Sub#13, assuming RSI based on 1dB desense, less than 93dB for spatial isolation and digital isolation for co-site co-channel CLI, SBFD slot configuration Alt-3 ({DDSUU} vs. {XXXXU}), Same area & half TxRUs (Option 3), Packet Size with 0.5Mbytes for DL and 0.125Mbyte for UL, key findings are summarized below:

-	Traffic load with {DL,UL} = {Low, Low} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.68% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.18% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 0.21% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.49% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -32.32% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -46.51% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 40.17% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported an improvement of 9.80% for SBFD

-	Traffic load with {DL,UL} = {Medium, Medium} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -2.12% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.56% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.22% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.14% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -55.11% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -76.17% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -6.72% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -45.00% for SBFD

-	Traffic load with {DL,UL} = {High, High} for both Operator#1 and Operator#2:

-	For Operator#1:

-	DL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -3.27% for Target case

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -12.27% for Target case

-	UL performance comparison between legacy TDD coexisting with SBFD in Operator#2 (Target case) and legacy TDD coexisting with legacy TDD in Operator#2 (Baseline):

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.60% for Target case

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -0.09% for Target case

-	For Operator#2:

-	DL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -72.61% for SBFD

-	Regarding 5%-tile of DL average-UPT CDF, one source ([Ericsson]) reported a degradation of -86.84% for SBFD

-	UL performance comparison between SBFD and legacy TDD:

-	Regarding mean value of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -18.29% for SBFD

-	Regarding 5%-tile of UL average-UPT CDF, one source ([Ericsson]) reported a degradation of -52.48% for SBFD