3GPP TSG RAN WG1 #114bis			R1-231nnnn

Xiamen, China, October 9th – October 13th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.12

Agenda Item:		8.16.12

Document for: 	Endorsement

### UE features for MC enhancements

Agreement

- Following is removed from in FG 49-1/49-2

  - [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3]

- Following is removed from FG 49-1b/49-2b

  - [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b]

Agreement

- Component 6 in FG 49-1/49-1b/49-2/49-2b is confirmed as: Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}

  - “FFS whether this component is reported per reported value in component 3” is removed from component 6 in FG 49-1/49-2

  - “FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b” is removed from component 6 in FG 49-1b/49-2b

Agreement

For 49-1:

- [At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 according to 49-1;]

  - If the scheduling cell is in the set and is the reference cell;

    - One unicast SC-DCI (for self scheduling) or one DCI 1_3 for the set of cells that schedules at least the scheduling cell per slot of scheduling cell, or;

    - One unicast SC-DCI (for self scheduling) and one DCI format 1_3 for the set of cells that schedules other than the scheduling cell per slot of scheduling cell

  - If the scheduling cell is NOT in the set;

    - One DCI 1_3 per slot of scheduling cell for the set of cells

  - FFS when the scheduling cell is in the set but is NOT the reference cell;

    - FFS one DCI 1_3 per slot of scheduling cell for the set of cells

      - FFS No SC-DCI is configured for the scheduling cell

R1-2308857	On UE features for MC enhancements	Nokia, Nokia Shanghai Bell

R1-2308918	Discussion on UE features for MC enhancements	Huawei, HiSilicon

R1-2309011	Discussion on UE features for multi-carrier enhancement	Spreadtrum Communications

R1-2309105	Discussion on Rel-18 Multi-carrier enhancements UE features	vivo

R1-2309141	Discussion on UE feature for MC enhancements	ZTE

R1-2309310	Discussion on UE features for MC enhancements	LG Electronics

R1-2309411	UE features for multi-carrier enhancements	Samsung

R1-2309442	Discussion on UE features for MC enhancements	xiaomi

R1-2309512	Further discussion on UE features for MC enhancements	CATT

R1-2309627	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2309865	Views on UE features for Rel-18 MC enhancements	Apple

R1-2309973	On UE feature discussion for Rel-18 multi-carrier enhancement	MediaTek Inc.

R1-2310066	Discussion on UE features for Multi-carrier enhancements	NTT DOCOMO, INC.

R1-2310100	UE features for Rel-18 multi-carrier enhancements	Ericsson

R1-2310178	UE features for MC enhancements	Qualcomm Incorporated