3GPP TSG RAN WG1 #114bis			R1-231nnnn

Xiamen, China, October 9th – October 13th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.16

Agenda Item:		8.16.16

Document for: 	Endorsement

### 8.16.16	Others

Including UE features for endorsed TEI proposals.

Agreement

- The column of “Prerequisite feature groups” in FGs 55-4d and 55-4e is “one of {FG 55-4a, 55-4b, 55-4c}”

- Remove “FFS dependency between 55-4d/55-4e” from FGs 55-4a, 55-4b, and 55-4c

- Add following notes to FG 55-4a/b/c

  - UE does not expect to determine a different codebook size in a PUCCH slot from the codebook size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant that schedules a PUSCH in a slot overlapping with the PUCCH slot.

  - UE does not expect to determine a different [PUCCH resource] in a slot from the [PUCCH resource] determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant that schedules a PUSCH in that slot.

Agreement

- Introduce following Rel-18 FGs that indicate support of UE features defined in FG11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f, 11-2g, and 23-2-1e for a UE supporting FG 55-6 for (2, 2) span-based PDCCH monitoring

| 55. TEI18 | 55-6a | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells | 1.Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells - Candidate value for the component: {2, 3, …, 16) 2.Supported span arrangement for CA -Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} | FG11-2 for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6 for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-6b | Mix of Rel-16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers | Support Rel-15 monitoring capability and Rel-16 PDCCH monitoring capability on different serving cells | FG11-2 for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6 for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6c | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | 1.Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 2. Supported span arrangement for CA - Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} | FG11-2b for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6b for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6d | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring on all the serving cells | Supported combination of (pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) | FG11-2 for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6 for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6e | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) | FG11-2b for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6b for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6f | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells with restriction for non-aligned span case | 1.Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells -Candidate value for the component: {2, 3, …, 16} 2.UE supports aligned span and non-aligned span In case of non-aligned span when the configured number of cells with Rel-16 PDCCH monitoring is larger than the UE reported value, PDCCH monitoring occasion(s) should be configured only on same symbol(s) every slot | FG11-2 for (7, 3) or (4, 3) span based PDCCH monitoring; FG55-6 for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6g | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers with restriction for non-aligned span case | 1.Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) -Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 -Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 2.UE supports aligned span and non-aligned span In case of non-aligned span when the configured number of cells with Rel-16 PDCCH monitoring is larger than the UE reported value, PDCCH monitoring occasion(s) should be configured only on same symbol(s) every slot | FG11-2b for (7, 3) or (4, 4) span based PDCCH monitoring; FG55-6b for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 55. TEI18 | 55-6h | PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | FG23-2-1, and; FG11-2 for (7, 3) or (4, 4) span based PDCCH monitoring; FG55-6 for (2, 2) span based PDCCH monitoring with additional restriction(s) | Yes | N/A |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |

Agreement

  - Introduce following FG

| 55. TEI18 | 55-7 | Two QCL TypeD for CORESET monitoring in multi-DCI based multi-TRP | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA associated with coresetPoolIndex value 0 and 1 | 16-2a | Yes | N/A |  | Per FSPC | N/A | FR2 only | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2308861	On UE features for TEI18	Nokia, Nokia Shanghai Bell

R1-2308888	UE features for endorsed Rel-18 TEI on HARQ multiplexing on PUSCH	Huawei, HiSilicon

R1-2309142	Discussion on UE feature for Rel-18 TEI	ZTE

R1-2309415	UE features for endorsed Rel-18 TEI	Samsung

R1-2310181	Other UE features including TEI and span-based PDCCH monitoring with additional restrictions	Qualcomm Incorporated