3GPP TSG RAN WG1 #114bis			R1-231nnnn

Xiamen, China, October 9th – October 13th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.2

Agenda Item:		8.16.2

Document for: 	Endorsement

### 8.16.2	UE features for NR sidelink evolution

Agreement

- FG 47-s1 is updated as follows

| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink PSCCH/PSSCH/ [PSFCH] TXs and RXs in a resource pool in 15kHz and/or 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. Candidate value sets: {[15KHz, 30kHz, both]} FFS Rx capability including supported SCS | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | [Per band] | N/A | N/A |  | [UE supports EUTRA mode 4 SL in the same band] FFS whether to introduce some restriction on the SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-k1 and 47-k2 are updated as follows

| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS [8. CP extension up to 2 symbols if the UE supports 60kHz SCS] [9. Switching Type 1 to Type 2 channel access when UE is able to use a shared COT] FFS whether to merge with 47-k3 | At least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support channel access for NR sidelink operation in unlicensed shared spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported. [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSSCH transmission in multiple RB sets 2. UE supports Type A and Type B multi-channel access procedures for concurrent PSFCH transmissions in multiple RB sets [3. UE supports Type A and Type B multi-channel access procedures for concurrent S-SSB transmissions in multiple RB sets] 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} FFS whether to introduce FG(s) for transmission on multiple RB sets | 47-k1, 15-11, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in unlicensed shared spectrum. | [Per band / Per UE or Per FS (for 20MHz BW only UE)] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with or without capability signalling] |

Agreement

- FG 47-k3 and 47-k4 are updated as follows

| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | [1. UE supports monitoring SCI to read COT sharing information] 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1, 15-1, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support receiving UE-to-UE COT sharing for NR sidelink operation in unlicensed shared spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported. [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating in SCI COT sharing information | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support transmitting UE-to-UE COT sharing for NR sidelink operation in unlicensed shared spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional without capability signalling] |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: FFS candidate values for X FFS whether to report following - Maximum number of simultaneous PSCCH/PSSCH TX/RX - Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | [Per band] | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA with single or multiple carrier S-SSB transmission/reception | 1) UE supports transmitting/receiving S-SSB on [one selected or] all SL synchronization carrier with the same sync reference from Set-B FFS whether to report followings - Maximum number X of simultaneous S-SSB TX - Maximum number of simultaneous S-SSB RX 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] | Yes | No |  | [Per band] | N/A | N/A |  |  | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving up to X PSFCH resources in a slot over multiple SL carriers 2) UE supports transmitting up to Y PSFCH resources in a slot over multiple SL carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | [Per band] | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} | Optional with capability signalling |

Agreement

- Add following note in FG 47-s1

  - Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP

Agreement

- FG 47-k6 and 47-k7 are updated as follows

| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. [may] avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. [may] avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 1 | [Per UE] band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource], UE [may] prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 2 | [Per UE] band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

Agreement

- FG 47-m1 is updated as follows

| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive FFS whether to separate this FG for Tx/Rx and for PSCCH/PSSCH/PSFCH | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed shared spectrum, where PSD and/or OCB requirements are defined by regulation. ] | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-m3 and 47-m4 are updated as follows

| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | [Optional without capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol [2. UE can monitor a total up to [A] X PSCCHs in a slot in the 1st and 2nd starting symbols] | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The value X is the same as the reported value in FG 15-1 | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |

Agreement

- FG 47-m5 is updated as follows

| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH [with shared spectrum channel access] | 1. UE supports PSFCH transmission/reception on [N] PSFCH occasion(s) per PSCCH/PSSCH [with shared spectrum channel access] FFS whether to separate this FG for Tx and Rx capabilities FFS whether N is same for Tx and Rx | 15-11, 47-k1 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-m6 is updated as follows

| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed shared spectrum, where PSD and/or OCB requirements are defined by regulation.] [It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied] [The signaling is only expected for a band where shared spectrum channel access must be used.] | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must support this FG. indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2308847	On UE features for NR sidelink evolution	Nokia, Nokia Shanghai Bell

R1-2308887	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2309005	Discussion on UE features for NR sidelink evolution	Spreadtrum Communications

R1-2309097	Discussion on Rel-18 sidelink UE features	vivo

R1-2309116	Discussion on UE features for NR sidelink evolution	ZTE, Sanechips

R1-2309402	UE features for NR sidelink evolution	Samsung

R1-2309445	Discussion on UE features for NR sidelink evolution	xiaomi

R1-2309513	Further discussion on UE features for NR sidelink evolution	CATT, CICTCI

R1-2309626	UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2309779	Discussion on Rel-18 NR sidelink evolution UE features	TOYOTA InfoTechnology Center, Hyundai Motor Company, Continental Automotive, Fraunhofer HHI, Fraunhofer IIS

R1-2309857	On UE Features for Rel-18 NR Sidelink Evolution	Apple

R1-2309996	Discussion on UE feature list for NR Sidelink Evolution	MediaTek Inc.

R1-2310056	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2310169	UE features for NR Rel-18 Sidelink	Qualcomm Incorporated